3GPP TSG-RAN4 Meeting #109 	R4-2319457

# Chicago, USA, 13th November 2023 - 17th November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 1903 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [NR_newRAT-Core] CR to remove the word "capable" in "power class 3 capable UE" - TS38.101-1, Rel-18, Cat-A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Limited |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2023-11-17 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Though we studied an applicability of power boosting to the power class 2 UE during the Rel-17 SI (FS_NR_Opt_pi2BPSK), an outcome of the SI has not been reflected to the current specification. Thus, power boosting does not apply to PC2 UE at this moment. But in 38.101-1, the use of the word "capable" as in "power class 3 capable UE" may lead to misinterpretation as a PC2 UE is capable of outputing more power than a PC3, so "power class 3 capable UE" can be misunderstood as applying to PC2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace "power class 3 capable UE" by "power class 3 UE" i.e. remove the word "capable". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity remains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4 and 6.2B.4.1 and 6.4.2.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

<<Start of change>>

### 6.2.4	Configured transmitted power

The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

PCMAX_H,f,c = MIN {PEMAX,c,  PPowerClass – ΔPPowerClass }

where

PEMAX,c is the value given by either the p-Max IE or the field additionalPmax of the NR-NS-PmaxList IE, whichever is applicable according to TS 38.331[7];

PPowerClass is the maximum UE power specified in Table 6.2.1-1 and in Table 6.2F.1-1 for shared spectrum access operation, without taking into account the tolerance specified in the Table 6.2.1-1 and in Table 6.2F.1-1 for shared spectrum access operation;

When  the IE powerBoostPi2BPSK is set to 1, PEMAX,c is increased by +3 dB for a power class 3 UE operating in TDD bands n40, n41, n77, n78, and n79 with PI/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and 40% or less symbols in certain evaluation period are used for UL transmission when PEMAX,c ≥ 20 dBm (The exact evaluation period is no less than one radio frame).

When the IE powerBoostPi2BPSK is set to 1, ΔPPowerClass = -3 dB for a power class 3 UE operating in TDD bands n40, n41, n77, n78, and n79 with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and 40% or less slots in radio frame are used for UL transmission.

ΔPPowerClass =

-	3 dB for a power class 2 capable UE or 6 dB for a power class 1.5 UE when P-max of 23 dBm or lower is indicated; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50%; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or when the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and half the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame).

-	3 dB for a power class 1.5 capable UE when P-max of between 23 dBm and 26 dB is indicated; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is between 25% and 50%; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is between maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC2-FR1/2 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or when the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame).

-	3dB when the UE is configured with SUL configurations and the requirements of default power class are applied as specified in sub-clause 6.2C.1 on the band where UE indicates power class 2;

-	3dB is applied during SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’ with configured SRS resources in each SRS resource set(s) consisting of one SRS port when PC2 capable UE with txDiversity-r16 capability or PC1.5 capable UE further indicates SRS-TxSwitch capability ‘t1r2’ or ‘t1r4’ or ‘t1r1-t1r2’ or ‘t1r1-t1r2-t1r4’;

-	0 dB otherwise;

∆TIB,c is the additional tolerance for serving cell c as specified in clause 6.2A.4.2 for NR CA, clause 6.2C.2 for SUL, or TS 38.101-3 clause  6.2B.4.2 for EN-DC; ∆TIB,c = 0 dB otherwise; In case the UE supports more than one of band combinations for V2X operating bands for concurrent operation, CA, SUL or DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ∆TIB,c shall be the average value for all band combinations defined in clause 6.2A.4.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [3], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ∆TIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ∆TIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [3] for the applicable operating bands.

∆TC,c = 1.5dB when NOTE 3 in Table 6.2.1-1 in 38.101-1 applies for a serving cell c, otherwise ∆TC,c = 0 dB ;

MPRc and A-MPRc for serving cell c are specified in clause 6.2.2 and clause 6.2.3, respectively and in clause 6.2F.2 and clause 6.2F.3 respectively for shared spectrum access operation;

∆MPRc for serving cell c is specified in clause 6.2.2 and in clause 6.2F.2 for shared spectrum access operation.

∆TRxSRS is applied during SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’ when

a)	UE transmits SRS on the second SRS resource in every configured SRS resource set when the SRS-TxSwitch capability is indicated as 't1r2' or 't1r1-t1r2'

b)	UE transmits SRS on the second, third and fourth SRS resources of the total 4 SRS resources from all configured SRS resource set(s) consisting of one SRS port when the SRS-TxSwitch capability is indicated as 't1r4' or, 't1r4-t2r4' or 't1r1-t1r2-t1r4' or, 't1r1-t1r2-t2r2-t1r4-t2r4'

c)	UE transmits SRS from the second SRS port pair on the second SRS resource in every configured SRS resource set consisting of two SRS ports when the SRS-TxSwitch capability is indicated as ' t2r4' or ' t1r4-t2r4', or 't1r1-t1r2-t2r2-t2r4' or 't1r1-t1r2-t2r2-t1r4-t2r4', or

d)	UE transmits SRS to a DL-only carrier

The value of ∆TRxSRS is 4.5dB for bands whose FUL_high is higher than the FUL_low of n79 and 3 dB for bands whose FUL_high is lower than the FUL_low of n79 when the device is capable of power class 3 or power class 5 or power class 1.5 in the band, or when the device is capable of power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating txDiversity-r16..

The value of ∆TRxSRS is 7.5dB for bands whose FUL_high is higher than the FUL_low of n79 and 6 dB for bands whose FUL_high is lower than the FUL_low of n79 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is capable of power class 2 in the band and ΔPPowerClass = 0 dB and not indicating txDiversity-r16.

For other SRS transmissions ∆TRxSRS is zero;

P-MPRc is the power management maximum power reduction for

a)	ensuring compliance with applicable electromagnetic energy absorption requirements and addressing unwanted emissions / self desense requirements in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications;

b)	ensuring compliance with applicable electromagnetic energy absorption requirements in case of proximity detection is used to address such requirements that require a lower maximum output power.

The UE shall apply P-MPRc for serving cell c only for the above cases. For UE conducted conformance testing P-MPRc shall be 0 dB

NOTE 1:	P-MPRc was introduced in the PCMAX,f,c equation such that the UE can report to the gNB the available maximum output transmit power. This information can be used by the gNB for scheduling decisions.

NOTE 2:	P-MPRc may impact the maximum uplink performance for the selected UL transmission path.

TREF and Teval are specified in Table 6.2.4-1. For each TREF, the PCMAX,L,c for serving cell c are evaluated per Teval and given by the minimum  value taken over the transmission(s) within the Teval; the minimum PCMAX_L,f,c over one or more Teval is then applied for the entire TREF

Table 6.2.4-1: Evaluation and reference periods for Pcmax

| TREF | Teval | Teval with frequency hopping |
| --- | --- | --- |
| Physical channel length | Physical channel length | Min(Tno_hopping, Physical Channel Length) |

The measured configured maximum output power PUMAX,f,c shall be within the following bounds:

PCMAX_L,f,c  –  MAX{TL,c, T(PCMAX_L,f,c)}  ≤  PUMAX,f,c  ≤  PCMAX_H,f,c  +  T(PCMAX_H,f,c).

where the tolerance T(PCMAX,f,c) for applicable values of PCMAX,f,c is specified in Table 6.2.4-1. The tolerance TL,c is the absolute value of the lower tolerance for the applicable operating band as specified in Table 6.2.1-1 and in Table 6.2F.1-1 for shared spectrum access operation.

Table 6.2.4-1: PCMAX tolerance

| PCMAX,f,c (dBm) | Tolerance T(PCMAX,f,c) (dB) |
| --- | --- |
| 23 < PCMAX,c ≤ 33 | 2.0 |
| 21 ≤ PCMAX,c ≤ 23 | 2.0 |
| 20 ≤ PCMAX,c < 21 | 2.5 |
| 19 ≤ PCMAX,c < 20 | 3.5 |
| 18 ≤ PCMAX,c < 19 | 4.0 |
| 13 ≤ PCMAX,c < 18 | 5.0 |
| 8 ≤ PCMAX,c < 13 | 6.0 |
| -40 ≤ PCMAX,c < 8 | 7.0 |

<<Unchanged sections skipped>>

#### 6.2B.4.1	Configured transmitted power level for NR-DC

The UE is allowed to set its configured maximum output power PCMAX,f,c,MCG and PCMAX,f,c,SCG for the respective MCG and SCG and its total configured maximum output power for NR-DC operation  PTotalNR-DC=10log10(PTotalNR-DC) with PTotalNR-DC as specified in clause 7.6.2 of [8]. The UE is configured with an inter-CG power sharing mode by NR-DC-PC-mode.The requirements apply for one uplink serving cell configured per CG and for asynchronous and synchronous NR-DC if not otherwise stated.

Unless otherwise stated, the configured maximum output power PCMAX,f,c,MCG (q) in physical-channel q for carrier f of serving cell c shall be set within the bounds if contained in the MCG,

PCMAX_L,f,c,MCG (q) ≤  PCMAX,f,c,MCG (q) ≤  PCMAX_H,f,c,MCG (q)

and the corresponding PCMAX_L,f,c,SCG (q) for a serving cell contained in the SCG,

PCMAX_L,f,c,SCG (q) ≤  PCMAX,f,c,SCG (q) ≤  PCMAX_H,f,c,SCG (q)

where PCMAX_L,f,c,MCG, PCMAX_H,f,c,MCG, PCMAX_L,f,c,SCG and PCMAX_H,f,c,SCG are the limits for a serving cell c as specified in clause 6.2.4 modified as follows:

PCMAX_L,f,c,MCG = MIN{MIN(PEMAX,c , PEMAX,NR-DC, PNR) – ∆TC,c, (PPowerClass,NR-DC – ΔPPowerClass,NR-DC) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc)}

PCMAX_H,f,c,MCG = MIN{PEMAX,c, PEMAX,NR-DC, PNR, PPowerClass, NR-DC – ΔPPowerClass,NR-DC}

for the MCG and

PCMAX_L,f,c,SCG = MIN{MIN(PEMAX,c , PEMAX,NR-DC, PNR) – ∆TC,c, (PPowerClass,NR-DC – ΔPPowerClass,NR-DC) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc)}

PCMAX_H,f,c,SCG = MIN{PEMAX,c, PEMAX,NR-DC, PNR, PPowerClass,NR-DC – ΔPPowerClass,NR-DC}

for the SCG, where

-	PEMAX,NR-DC is the value given by the field p-UE-FR1 of the PhysicalCellGroupConfig IE for the MCG as defined in [7];

-	PNR is the value given by the field p-NR-FR1 of the PhysicalCellGroupConfig IE as defined in [7];

-	PPowerClass,NR-DC is the maximum UE power specified in Table 6.2B.1.3-1 without taking into account the tolerance specified in the Table 6.2B.1.3-1;

-	∆TIB,c is the additional tolerance for serving cell c as specified in clause 6.2B.4.2 for NR-DC; ∆TIB,c = 0 dB otherwise;

-	∆TC,c = 1.5dB when NOTE 2 in Table 6.2B.1.3-1 applies for a serving cell c, otherwise ∆TC,c = 0 dB ;

-	∆MPRc for serving cell c is specified in clause 6.2.2.

-	ΔPPowerClass,NR-DC = 0 dB for a power class 3 UE.

When MSG or SCG are configured with intra-band contiguous carrier aggregation, then intra-band carrier aggregation PCMAX,CA,MCG (q) and/or PCMAX,CA,SCG (q) in physical-channel q shall be set within the bounds:

PCMAX_L,CA, MCG (q) ≤  PCMAX,CA,MCG (q) ≤  PCMAX_H,CA,MCG (q)

for MSG, and/or

PCMAX_L,CA,SCG (q) ≤  PCMAX,CA,SCG (q) ≤  PCMAX_H,CA,SCG (q)

for SCG, where PCMAX_L,CA,MCG, PCMAX_H,CA,MCG, PCMAX,CA,SCG and PCMAX_H,CA,SCG are the limits for a carrier aggregation uplink as specified in clause 6.2A.4.1.1 modified as follows:

PCMAX_L,CA,MCG = MIN{10 log10 ∑ pEMAX,c  - TC , PEMAX,CA, PEMAX,NR-DC, PNR,MCG, (PPowerClass,NR-DC – ΔPPowerClass,NR-DC)  – MAX(MAX(MPR, A-MPR) + ΔTIB,c + TC + TRxSRS, P-MPRc ) }

PCMAX_H,CA,MCG  = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA , PEMAX,NR-DC, PNR,MCG, PPowerClass,NR-DC – ΔPPowerClass,NR-DC }

for the MCG, and

PCMAX_L,CA,SCG = MIN{10 log10 ∑ pEMAX,c  - TC , PEMAX,CA, PEMAX,NR-DC, PNR,SCG, (PPowerClass,NR-DC – ΔPPowerClass,NR-DC)  – MAX(MAX(MPR, A-MPR) + ΔTIB,c + TC + TRxSRS, P-MPRc ) }

PCMAX_H,CA,SCG  = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA , PEMAX,NR-DC, PNR,SCG, PPowerClass,NR-DC – ΔPPowerClass,NR-DC}

for SCG.

For a UE provided with NR-DC-PC-mode = Semi-static-mode1,

PTotalNR-DC = MIN{PEMAX, NR-DC, PPowerClass,NR-DC} + 0.3 dB

with PPowerClass,NR-DC set to power class 3 in case the UE indicates a higher power class in any CG. The UE determines the maximum transmission power for the MCG and the SCG using the respective configured maximum power PCMAX,f,c,MCG and PCMAX,f,c,SCG.

If for synchronous NR-DC operation a UE is provided NR-DC-PC-mode = Semi-static-mode2, the PTotalNR-DC is determined as above and

-	if at least one symbol of slot i1 of the MCG/SCG is indicated as uplink or flexible to a UE by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated, if provided, overlaps with a symbol for any ongoing transmission overlapping with slot i2 of the SCG/MCG, the UE determines a maximum power for the transmission on the SCG/MCG overlapping with slot i2 using the configured maximum power PCMAX,f,c,SCG or PCMAX,f,c,MCG for the SCG or MSG, respectively,

-	otherwise (i.e. an ongoing transmission overlapping with slot i2 of the SCG/MCG overlaps with only semi-static downlink symbols within slot i1 of the MCG/SCG), the UE determines a maximum power for the transmission on MCG or the SCG overlapping with slot i2 using the configured maximum power as specified in clause 6.2.4.

If a UE indicates a capability for dynamic power sharing between the MCG and the SCG and is provided with NR-DC-PC-mode = Dynamic,

PTotalNR-DC = MIN{PEMAX, NR-DC, PPowerClass,NR-DC}

with PPowerClass,NR-DC set to power class 3 in case the UE indicates a higher power class in any CG. The UE determines the maximum transmission power for the MCG and the SCG using the respective configured maximum power PCMAX,f,c,MCG and PCMAX,f,c,SCG except

-	if UE transmission(s) in slot i1 of the MCG or in slot i2 of the SCG do not overlap in time with any UE transmission(s) on the SCG or the MCG, respectively, the UE determines a maximum transmission power in slot i1 of the MCG or in slot i2 of the SCG using the configured maximum power as specified in clause 6.2.4.

If a UE indicates a capability to determine a total transmission power on the SCG at a first symbol of a transmission occasion on the SCG by determining transmissions on the MCG as specified in clause 7.6.2 of [8], and is provided with NR-DC-PC-mode = Dynamic,

PTotalNR-DC = MIN{PEMAX, NR-DC, PPowerClass,NR-DC}

with PPowerClass,NR-DC set to power class 3 in case the UE indicates a higher power class in any CG. The UE determines the maximum transmission power for the MCG and the SCG using the respective configured maximum power PCMAX,f,c,MCG and PCMAX,f,c,SCG.

The measured total maximum output power PUMAX over both CGs measured over the transmission reference time duration is

PUMAX = 10 log10 (pUMAX,c,MCG + pUMAX,c,SCG),

where pUMAX,c,MSG and pUMAX,c,SCG denote the measured output power of serving cells c contained in the respective MSG and SCG expressed in linear scale.

The measured total configured maximum output power PUMAX shall be within the following bounds:

PCMAX_L -TLOW (PCMAX_L)  ≤  PUMAX  ≤  PCMAX_H + THIGH (PCMAX_H)

with the tolerances TLOW(PCMAX_H) and THIGH(PCMAX_H) for applicable values of PCMAX specified in Table 6.2B.4.1.3-2.

When a subframe p on the MSG overlap with a physical-channel q on the SCG, then for PUMAX evaluation, the subframe p on the MCG is taken as reference period TREF and always considered as the reference measurement duration and the following rules are applicable.

TREF and Teval are specified in Table 6.2B.4.1.3-1 when same or different subframe and physical-channel durations are used on the carriers. The PPowerClass shall not be exceeded by the UE during any evaluation period of time.

Table 6.2B.4.1.3-1: PCMAX evaluation window

| Transmission duration | TREF | Teval |
| --- | --- | --- |
| Different transmission duration in different CG carriers | MCG subframe | MIN(Tno_hopping, Physical Channel Length) |

For each TREF, the PCMAX_H is evaluated per Teval and given by the maximum value over the transmission(s) within the Teval as follows:

PCMAX_H  = MAX{PCMAX_NR-DC_H(p,q), PCMAX_NR-DC_H(p,q+1), … , PCMAX_NR-DC_H(p,q+n)}

where PCMAX_NR-DC_H entries are the applicable upper limits for each overlapping scheduling unit pairs (p,q), (p, q+1), up to (p, q+n) for each applicable Teval duration, where q+n is the last physical-channel on the SCG overlapping with subframe p on the MCG, while PCMAX_L is computed as follows:

PCMAX_L = MIN{PCMAX_NR-DC_L(p,q), PCMAX_NR-DC_L(p,q+1), … , PCMAX_NR-DC_L(p,q+n)}

where PCMAX_NR-DC_L entries are the applicable lower limits for each overlapping scheduling unit pairs (p,q), (p, q+1) up to (p, q+n) for each applicable Teval duration, where q+n is the last physical-channel on the SCG overlapping with subframe p on the MCG.

For a UE provided with NR-DC-PC-mode = Semi-static-mode1 and configured with pNR,MCG + pNR,SCG ≤ PTotalNR-DC with pNR,MCG and pNR,SCG the values of the PNR for the respective MCG and SCG expressed in linear scale

PCMAX_NR-DC_L(p,q) = 10 log10 [pCMAX_L, MCG (p) + pCMAX_L, SCG (q)]

PCMAX_NR-DC_H(p,q) = 10 log10 [pCMAX_H, MCG (p) + pCMAX_H, SCG (q)]

where

pCMAX_L, MCG, pCMAX_L, SCG, pCMAX_H, MCG, pCMAX_H, SCG  can be  pCMAX_L,f,c,MCG, pCMAX_H,f,c,MCG, pCMAX_L,f,c,SCG, and pCMAX_H,f,c,SCG the values of the respective PCMAX_L,f,c,MCG, PCMAX_H,f,c,MCG, PCMAX_L,f,c,MCG, and PCMAX_H,f,c,SCG expressed in linear scale, or pCMAX_L,CA,MCG, pCMAX_H,CA,MCG, pCMAX_L,CA,SCG, and pCMAX_H,CA,SCG the values of the respective PCMAX_L,CA,MCG, PCMAX_H,CA,MCG, PCMAX_L,CA,SCG, and PCMAX_H,CA,SCG expressed in linear scale if the contiguous carrier aggregation is configured in MCG and/or SCG or a combinations of single cell and carrier aggregation while the measured configured maximum power PUMAX  for each CG shall meet the requirements as specified in clause 6.2.4 but with bounds for PCMAX,f,c,MCG (p) and PCMAX,f,c,SCG  as specified in this clause or 6.2A.4.1.1 as modified in this clause for contiguous carrier aggregation configured cell group.

If for synchronized NR-DC a UE is provided with NR-DC-PC-mode = Semi-static-mode2 and configured with pNR,MCG + pNR,SCG ≤ PTotalNR-DC with pNR,MCG and pNR,SCG the linear-scale values of the PNR for the respective MCG and SCG

PCMAX_NR-DC_L(p,q) = 10 log10 [pCMAX_L, MCG (p) + pCMAX_L, SCG (q)]

PCMAX_NR-DC_H(p,q) = 10 log10 [pCMAX_H, MCG (p) + pCMAX_H, SCG (q)]

while the measured configured maximum power PUMAX for each CG shall meet the requirements specified in Table 6.2.4-2 but with bounds for PCMAX,f,c,MCG (p) and PCMAX,f,c,SCG(q)  as specified in this clause or 6.2A.4.1.1-1when intra-band carrier aggregation contiguous is configured in the MCG and/or SCG with the bounds PCMAX,CA,MCG (p) and PCMAX,CA,SCG  defined in this clause except

-	if an ongoing transmission overlapping with physical channel q of the SCG or subframe p of the MCG overlaps with only semi-static downlink symbols within the respective subframe p of the MCG or physical channel q of the SCG as indicated to a UE by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated, if provided,

then the measured configured maximum power PUMAX for the transmission subframe p on the MCG or physical channel q on the SCG shall meet the requirements as specified in clause 6.2.4 and with bounds for PCMAX,f,c,MCG (p) or PCMAX,f,c,SCG  as specified in this clause  or Table 6.2A.4.1.1-1when intra-band carrier aggregation contiguous is configured in the MCG and/or SCG with  bounds for PCMAX,CA,MCG (p) and PCMAX,CA,SCG  defined in this clause For a UE provided with NR-DC-PC-mode = Dynamic,

PCMAX_NR-DC_L(p,q) = MIN{10 log10 [pCMAX_L, MCG (p) + pCMAX_L, SCG (q)], PTotalNR-DC}

PCMAX_NR-DC_H(p,q) = MIN{10 log10 [pCMAX_H, MCG (p) + pCMAX_H, SCG (q)], PTotalNR-DC}

while the measured configured maximum power PUMAX on the MCG shall meet the requirements as specified in clause 6.2.4-2 but with bounds for PCMAX,f,c,MCG (p) as specified in this clause, or as specified in Table 6.2A.4.1.1-1 when intra-band carrier aggregation contiguous is configured in the MCG with the bounds for PCMAX,CA,MCG (p) as specified in this clause and the PUMAX on the SCG shall be within

PCMAX_L,  –  MAX{TL,c, T(PCMAX_L,)}  ≤  PUMAX  ≤  PCMAX_H  +  T(PCMAX_H,f,c)

where for single uplink cell SCG

PCMAX_L = MIN{PCMAX_L,f,c,SCG (p), 10 log10 (PTotalNR-DC – pNR,MSG)}

PCMAX_H = MIN{PCMAX_H,f,c,SCG (p), 10 log10 (PTotalNR-DC – pNR,MSG)}

and for intra-band carrier aggregation configured SCG

PCMAX_L = MIN{PCMAX_L,CA,SCG (p), 10 log10 (PTotalNR-DC – pNR,MSG)}

PCMAX_H = MIN{PCMAX_H,CA,SCG (p), 10 log10 (PTotalNR-DC – pNR,MSG)}

where PCMAX_L,CA,SCG and PCMAX_H,CA,SCG bounds are defined in this clause,

with limits as specified in Table 6.2.4-2 or as specified in Table 6.2A.4.1.1-1 when intra-band carrier aggregation contiguous is configured in the MCG and pNR,MCG the value of the PNR for the MCG expressed in linear scale.

Table 6.2B.4.1.3-2: PCMAX tolerance for NR-DC

| PCMAX(dBm) | Tolerance TLOW (PCMAX_L) (dB) | Tolerance THIGH (PCMAX_H) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX ≤ 33 | 3.0 | 2.0 |
| 22 ≤ PCMAX < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX< 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX < 20 | 5.0 |  |
| 11 ≤ PCMAX < 16 | 6.0 |  |
| -40 ≤ PCMAX < 11 | 7.0 |  |
| NOTE 1: For UEs provided with NR-DC-PC-mode = Semi-static-mode1 or with NR-DC-PC-mode = Semi-static-mode2, the upper tolerance Thigh shall be reduced by 0.3 dB for P ≥ 20 dBm. |  |  |

<<Unchanged sections skipped>>

##### 6.4.2.4.1	Requirements for Pi/2 BPSK modulation

These requirements apply if the IE powerBoostPi2BPSK is set to 1 for power class 3 UE operating in TDD bands n40, n41, n77, n78 and n79 with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and 40 % or less slots in radio frame are used for UL transmission. These requirements also apply if the IE dmrs-UplinkTransformPrecoding-r16 is configured and UE indicates support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. Otherwise the requirements for EVM equalizer spectrum flatness defined in clause 6.4.2.4 apply

The EVM equalizer coefficients across the allocated uplink block shall be modified to fit inside the mask specified in Table 6.4.2.4.1-1 for normal conditions, prior to the calculation of EVM. The limiting mask shall be placed to minimize the change in equalizer coefficients in a sum of squares sense.

Table 6.4.2.4.1-1: Mask for EVM equalizer coefficients for Pi/2 BPSK, normal conditions

| Frequency range | Parameter | Maximum ripple (dB) |
| --- | --- | --- |
| ¦FUL_Meas – Fcenter¦ ≤ X MHz (Range 1) | X1 | 6 (p-p) |
| ¦FUL_Meas – Fcenter¦ > X MHz (Range 2) | X2 | 14 (p-p) |
| NOTE 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated NOTE 2: Fcenter refers to the center frequency of an allocated block of PRBs NOTE 3: X, in MHz, is equal to 25% of the bandwidth of the PRB allocation NOTE 4: See Figure 6.4.2.4.1-1 for description of X1, X2 |  |  |

Figure 6.4.2.4.1-1: The limits for EVM equalizer spectral flatness with the maximum allowed variation.  .

For Pi/2 BPSK modulation the UE shall be allowed to employ spectral shaping and the shaping filter shall be restricted so that the impulse response of the shaping filter itself shall meet

│ãt(t,0)│ ≥ │ãt(t, τ)│    ∀τ ≠ 0

20log10│ãt(t,τ)│< -15 dB    1< τ < M - 1,

where│ãt(t, τ)│=IDFT{│ãt(t,f)│ejφ (t,f)},   f  is the frequency of the M allocated subcarriers , ã(t,f) and φ(t,f) are the amplitude and phase response.

0 dB reference is defined as 20log10│ãt(t,0)│.

<<End of change>>