3GPP TSG-RAN4 Meeting #109		R4-2320681

# Chicago, USA, November 13-17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.174 | CR | 0091 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [NR_IAB_enh] Correction to IAB-MT TA adjustment accuracy requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Core |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To correct applicable UL SCSs for which the TA requirements for IAB-MT apply. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The existing IAB-MT TA adjustement accuracy requirements (clause 12.2.3) refer to the corresponding TA adjustement accuracy requirements for the UE (clause 7.3) in TS 38.133. The latter also covers TA adjustement accuracy requirements for UL SCS > 120 kHz (e.g. SCS 480 kHz and 960 kHz). However, IAB-MT requirements including the TA adjustment are applicable for FR2-1 i.e. up to the UL SCS= 120 kHz. - Secondly, the clause 7.3 in TS 38.133 also covers requirements for NR-U i.e. it states that, “the same requirement applies also when the UE is not able to transmit a configured uplink transmission due to the channel assessment procedure.”. But IAB-MT requirements apply only to the licensed band. It is therefore not appropriate to refer to clause 7.3 in TS 38.133 rather the clause 12.2.3 should be standalone. The table containing the applicable UL SCSs and corresponding TA accuracies for the IAB-MT are defined in clause 12.2.3. The changes do not alter the core requirements which should be supported by the IAB-MT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IAB-MT will be required to meet TA accuracy for SCSs which are not supported by the IAB-MT i.e. at frequencies above 52.6 GHz. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.2.3, 12.2.3.1, 12.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE----------------------------

### 12.2.3	IAB-MT timing advance

#### 12.2.3.1	Timing Advance adjustment delay

The IAB-MT shall adjust the timing of its uplink transmission timing at time slot n+ k+1 for a timing advance command received in time slot n, and the value of k is defined in clause 4.2 in TS 38.213 [10]. The requirements in this clause apply for IAB-MT, for ‘Case 1’ transmission timing mode specified in clause 14 of TS 38.213 [10].

#### 12.2.3.2	Timing Advance adjustment accuracy

The IAB-MT shall adjust the timing of its transmissions with a relative accuracy better than or equal to the IAB-MT Timing Advance adjustment accuracy requirement in Table 12.2.3-1, to the signalled timing advance value compared to the timing of preceding uplink transmission. The timing advance command step is defined in TS 38.213 [10].

Table 12.2.3.2-1: UE Timing Advance adjustment accuracy

| UL Sub Carrier Spacing(kHz) | 15 | 30 | 60 | 120 |
| --- | --- | --- | --- | --- |
| UE Timing Advance adjustment accuracy | ±256 Tc | ±256 Tc | ±128 Tc | ±32 Tc |

----------------------END OF CHANGE----------------------------