3GPP TSG RAN meeting #102										RP-233385

Edinburgh, GB, December 11 – 15th 2023

## Status Report to TSG

Agenda item:			9.3.2.6

| WI / SI Name | WI on NR Support for UAV (Uncrewed Aerial Vehicles) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_UAV |  |  |  |  |
| Unique ID | 941105 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-230782 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2023 | Performance part: | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 100% | Performance Part: | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Jedrzej Stanczak |
|  | Company | Nokia, Nokia Shanghai Bell |
|  | Email | jedrzej.stanczak@nokia.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

No agreements.

#### 2.1.2	Remaining Open issues

No remaining open issues.

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2#123bis:

Agreements: - RAN2 will adopt Option2 - Specify a new Information Element. This new IE should contain the list of NS values applicable for UAV UEs only for the considered bands. When attaching to the cell, the UAV shall then support both the list of legacy NS values (from “NR-NS-PmaxList” in SIB4), and the list of “UAV specific” NS values (from the new IE element). - As of now, there is no explicit UAV UE capability but we will discuss which capability will implicitly be linked to a UAV UE capability. Tell RAN4 that we will let them know which capability they can use, once RAN2 agrees on capabilities. - The altitude from -420m to 1k above mt. Everest (i.e. ~10km) with 2m granularity. - The maximum number of height ranges can be 8 - Hysteresis is 1m granularity. Value range for h1-Hysteresis, h2-Hystereris, heightHyst is 1m to 64m. - It is up to network implementation how to configure height range and hysteresis - The height range configurations in ssb-toMeasure and AxHy are independent. - FFS UE behavior with respect to cell list is already clear when it switches to a new height range in either SSB to Measure or in eventAxHy (in rapporteur CR email discussion) - Regardless of whether the measurement object is associated with report configuration including numberOfTriggeringCells, UE measures height dependent SSBs for the current height (If any SSB-ToMeasure is not configured, UE measures all SS blocks as a legacy operation) - height based ssb-toMeasure configuration for IDLE/INACTIVE will not be support in Rel-18 - Prohibit timer will not be supported - RSRP/RSRQ measurements will always be reported with height reporting (as currently it is mandatory) - Similar to all other event triggers in NR, allow reportOnLeave and timeToTrigger configuration for all newly introduced events (already captured in the running CR) - Distance and time thresholds for FP update can be (re)configured in RRCReconfiguration, but not in other messages like RRCResume, RRCSetup and RRCReestablisment. - When UE moves to RRC_IDLE the thresholds for triggering FP update, if configured, are released. - When the UE is in INACTIVE it is released upon RRC Connection Resume procedure (rapporteur will check where it is the best place to capture) - For RRCSetupComplete and RRCResumeComplete, UE doesn't check for the threshold(s) configuration for indicating FP availability (i.e. it is always like new FP available indication). - Capture in the spec that the UE can send an empty flight path to the network to indicate the flight path is no longer valid - Configure the separate resource pools one-level higher on SL-BWPPoolConfigCommon/SL-BWPPoolConfig. Adopt the TP for 38.331. - Add additional flag to indicate whether the pool is for BRID, DAA, or both (check if this introduces a problem).

RAN2#124:

Agreements: - UAV WI is considered complete from RAN2 point of view - NR SIB5 includes aerial specific EUTRA NS values and SIB24 includes aerial specific NR NS values - Check if RAN4 if RAN2 needs to add additionaPmax-r18 in IE NR-NS-PmaxValueAerial-r18 - Introduce a UE capability indication for NR to indicate that the UE supports Rel-18 UAV enhancements - Define the following RAN2 capabilities (names to be discussed offline0: - altitude-meas-r18 and make it conditionally mandatory (CY) - multipleCellsMeasExtension-r18 - This field defines whether the UE supports measurement reporting triggered based on a number of cells (CY) - new capability to support AxHy that also means you support altitudeBasedNumberOfTriggeringCells (feature) (Optional) - - altitudeBasedSSB-ToMeasure-r18 (Optional) - for flight path reporting, we will introduce two capabilities flightPathReporting-r18 and flightPathAvailabilityIndication-r18 for UAI. (optional) - Understanding is that a UE that doesn’t support any frequency band that requires a aerial specific NS value, doesn’t need to implement the procedure for NS value. FFS whether a capability need is needed. - - sl-A2X capability, with BRID, DAA, and both granularity, that also means that it supports dedicated A2X pools. FFS if it is per UE or FS (as working assumption for CR we implement per UE) - No new agreements or spec impact is expected to address previous FFS: UE behavior with respect to cell list is already clear when it switches to a new height range in either SSB to Measure or in eventAxHy. - RAN2 understands if the UE is already in the corresponding altitude range at the time of configuration, similar to legacy behavior for other events, the UE triggers measurement reporting based on the configured event after the TTT expiry (assuming everything else remaining the same). No spec impact is expected. - As currently captured in the running CR: Use single IE (Altitude-r18) for both configuration and reporting inline with agreement from RAN2#123. Granularity for both reporting and configuration would be 1m. - RAN2 understands the existing procedure “include the concerned cell(s) in the cellsTriggeredList” means adding only the cells not already in the list. Revmove related Editor’s Note. No spec change is needed. - altitudeHyst-r18 is not considered in the entering condition of the altitude range configured by ssb-ToMeasureAltitudeBased-r1. - the ssb-ToMeasure-r18 field in SSB-ToMeasureAltitudeBased-r18 should be optional field, and it should be clarified in field description that if the field is not configured, the UE measures on all SS blocks within the corresponding altitude range. - When multiple events are configured simultaneously, network explicitly configures whether the UAV reports all triggered measurement reports or chooses the MR configuration corresponding to the triggered event with the smallest value between the altitude of the UAV and the altitude threshold. This flag applies for all events of the same type (Hx and AxHy) and MO (AxHy). This will be a separate capability - For FlightPathUpdateTimeThr value/range is (0..16383) with 1s granularity. - For FlightPathUpdateDistanceThr value/range is (0..1023) with 5m granularity (i.e. actual value is 5x field value in meters). - Include FlightPathInfoReport in AS-Context within HandoverPreparationInformation (as already captured in the running CR). - UE initiate transmission of UAI again to the target cell if a UAI transmission was initiated during the last 1 second before receiving the reconfigurationWithSync. Rapporteur to check if there are spec changes. - RAN2 can confirm the value zero can be configured for flightPathUpdateDistanceThr and flightPathUpdateTimeThr, and if there is any location/time change in waypoints, the UE should to indicate the flight path update when the value zero is configured - Flightpath update indication can be triggered due to adding a single waypoint. - Flightpath update indication can be triggered due to removal of a single future waypoint, except if it is removing an outdated waypoints - Confirm that the UAV uses the dedicated resource pool for A2X service if the A2X dedicated resource pool is configured. Otherwise, the UAV can use the “normal” resource pool for A2X service. - UE behaviour to select a resource pool from separately configured A2X communication resource pool i.e., sl-BWP-PoolConfigA2X or sl-BWP-PoolConfigCommonA2X can be specified in clause 5.22.1.1 of TS 38.321. - Samsung will produce a running CR and propose solutions - MAC layer knows the logical channel carrying data for BRID or DAA via PQI. Rapporteur will capture it in 38.321

#### 2.2.2	Remaining Open issues

No remaining open issues.

## 2.3	RAN3

#### 2.3.1	Agreements

RAN3#122:

Endorsed as BL CR: - R3-237064 Introduction of Aerial authorization information (Ericsson, AT&T, NTT DOCOMO INC, Qualcomm Incorporated, Intel Corporation, Samsung, NEC, Nokia, Nokia Shanghai Bell, Huawei, ZTE, CATT) - R3-237065 (BLCR to 38.423) NR support for UAV over Xn (Huawei, China Unicom, China Telecom, CATT, Intel Corporation, ZTE, Ericsson, Qualcomm, Nokia, Nokia Shanghai Bell, Samsung) - R3-237210 Draft CR to 38.300 on NR support for UAV (Nokia, Nokia Shanghai Bell, Huawei, Intel Corporation, ZTE, Ericsson, Samsung, CATT, Qualcomm, Deutsche Telekom, NEC) Agreed TPs: - R3-237999 TP to TS 38.423 BLCR for A2X service supporting and flightpath information modification (ZTE, CMCC, CATT, China Telecom, Nokia, Nokia Shanghai Bell, Huawei, Ericsson) - R3-238012 TP (BL CR TS 38.300) Stage 2 corrections for NR support for UAV (Nokia, Nokia Shanghai Bell, ZTE, Huawei) - R3-237991 (TP for TS 38.473) A2X service support in F1AP (Huawei, Nokia, Nokia Shanghai Bell, ZTE, Ericsson) - R3-237998 (TP for BL CR 38.413) Support of A2X communication (CATT, Nokia, Nokia Shanghai Bell, Huawei, ZTE, Ericsson) LS-out: - R3-238019 LS on RAN3 progress for UAV flight path information handling and A2X service support, Source: RAN3, To: RAN2, SA2

#### 2.3.2	Remaining Open issues

No remaining open issues.

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4#108bis:

Documents Approved: R4-2317629 WF on LTE and NR UAV RF requirements Nokia CRs endorsed: R4-2317627 Running draft CR to TS 38.101-1 - Introduction of Aerial UEs support Ericsson, Nokia, Huawei

RAN4#109:

Agreed CRs: R4-2322011 Running CR to TS 38.101-1 - Introduction of Aerial UEs support Ericsson, Nokia, Huawei, Murata, Qualcomm

#### 2.4.2	Remaining Open issues

No remaining open issues.

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SA2

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

TDocs submitted to RAN2:

- R2-2309467	LS on the handling of additional regulatory requirements for UAV UEs (R4-2314700; contact: Ericsson)

- R2-2309611	RRC Running CR (Introduction of Aerial Support)	Qualcomm Incorporated

- R2-2309921	Discussion on flight path reporting	Samsung

- R2-2309969	SL configuration for BRID and DAA	Samsung

- R2-2310069	Remaining Issues on Measurement Reports Enhancements	NEC

- R2-2310070	Remaining Issues on Flight Path Reporting	NEC

- R2-2310248	[Draft] LS to RAN1 on UL power control for NR UAV	CMCC

- R2-2310249	Considerations on measurement reporting	CMCC

- R2-2310250	Discussion on flight path reporting	CMCC

- R2-2310251	Considerations on left issues for UAV identification broadcast	CMCC

- R2-2310461	Proposed resolutions for Editor’s Notes and FFSes in the RRC running CR	Qualcomm Incorporated

- R2-2310462	Altitude-dependent SSB-ToMeasure for RRC_IDLE/RRC_INACTIVE mode measurements

- R2-2310472	Remaining aspects of PC5-based BRID and DAA support	Qualcomm Incorporated

- R2-2310531	Discussion on measurement reporting enhancement for NR UAV	vivo

- R2-2310532	Discussion on Height-dependent measurement configuration enhancement for NR UAV	vivo

- R2-2310533	Discussion on the remaining issue on flight path reporting	vivo

- R2-2310607	On height-dependent SSB-ToMeasure configuration	ZTE Corporation, Sanechips

- R2-2310608	On measurement reporting enhancement	ZTE Corporation, Sanechips

- R2-2310609	On flight path reporting	ZTE Corporation, Sanechips

- R2-2310622	Discussion on measurement reporting enhancements for NR UAV	Xiaomi

- R2-2310623	Discussion on flight path reporting for NR UAV	Xiaomi

- R2-2310639	Uncrewed Aerial Vehicles in Rel-18 - Updated Workplan	Nokia, Nokia Shanghai Bell

- R2-2310640	38.300 Running CR for Rel-18 UAVs	Nokia, Nokia Shanghai Bell

- R2-2310641	36.300 Running CR for Rel-18 UAVs	Nokia, Nokia Shanghai Bell

- R2-2310642	On NS Value Signalling for Rel-18 UAVs	Nokia, Nokia Shanghai Bell

- R2-2310643	On UE Capabilities for Rel-18 UAVs	Nokia, Nokia Shanghai Bell

- R2-2310846	Flightpath update notification for UAV	InterDigital

- R2-2310931	Measurement report enhancement for NR UAV	Huawei, HiSilicon

- R2-2310933	Further discussion on flight path reporting	Huawei, HiSilicon

- R2-2310934	Further discussion on UAV remote identification broadcast	Huawei, HiSilicon

- R2-2310935	Discussion on UE capabilities for UAV	Huawei, HiSilicon

- R2-2310936	Introduction of capabilities for Rel-18 NR Support for UAV WI	Huawei, HiSilicon

- R2-2310973	Discussion on and DRAFT reply to the RAN4 LS on the handling of additional regulatory requirements for UAV UEs	Qualcomm Incorporated

- R2-2311055	On UAV Measurement Reporting Behaviour	Nokia, Nokia Shanghai Bell

- R2-2311074	Remaining issues on measurement reporting enhancements in NR UAV	Samsung

- R2-2311102	Measurement Report Enhancement	LG Electronics

- R2-2311103	Consideration on flight path reporting for NR UAV	DENSO CORPORATION

- R2-2311154	Prohibit timer for mobility measurement reporting for UAVs	Ericsson, Intel, Docomo, China Telecom, Denso

- R2-2311157	UAV remote ID and related items	Beijing Xiaomi Mobile Software

- R2-2311170	Signalling of NS values for Aerial UEs	Ericsson

- R2-2311287	Reply LS on the handling of additional regulatory requirements for UAV Ues

- R2-2312230	Introduction of NR Support for UAV (Uncrewed Aerial Vehicles)	Qualcomm Incorporated

- R2-2312234	[DRAFT] Reply LS on flightpath information forwarding for UAV	ZTE Corporation, Sanechips

- R2-2312240	UE capabilities for NR Support for UAV (Uncrewed Aerial Vehicles)	Qualcomm Incorporated

- R2-2312242	Introduction of Enhanced LTE Support for UAV (Uncrewed Aerial Vehicles)	Qualcomm Incorporated

- R2-2312245	Report of [POST123bis][025][UAV] 38.331 Running CR (Qualcomm) including remaining open issues	Qualcomm Incorporated

- R2-2312833	Discussion on UE capabilities for UAV	Huawei, HiSilicon

- R2-2312851	Introduction of NR Support for UAV	Huawei, HiSilicon

- R2-2312852	Introduction of Enhanced LTE Support for UAV	Huawei, HiSilicon

- R2-2313053	Uncrewed Aerial Vehicles in Rel-18 - Updated Workplan	Nokia, Nokia Shanghai Bell

- R2-2313054	Work Item Agreements for Uncrewed Aerial Vehicles in Rel-18	Nokia, Nokia Shanghai Bell

- R2-2313055	Introduction of NR Support for Uncrewed Aerial Vehicles	Nokia, Nokia Shanghai Bell

- R2-2313056	Introduction of Enhanced LTE Support for Uncrewed Aerial Vehicles	Nokia, Nokia Shanghai Bell

- R2-2313057	On UE Capabilities for Rel-18 UAVs	Nokia, Nokia Shanghai Bell

- R2-2313089	Discussion on RAN4 question on UE capability and DRAFT reply	Qualcomm Incorporated

- R2-2313404	Discussion on UE capability for UAV features	vivo	discussion	NR_UAV-Core

- R2-2312197	Remaining Issues on Measurement Reports Enhancements	NEC	discussion

- R2-2312231	Remaining issues for altitude-based SSB-ToMeasure	ZTE Corporation, Sanechips

- R2-2312232	Remaining issues for event AxHy	ZTE Corporation, Sanechips	discussion

- R2-2312647	Remaining aspects for UAV measurement reports, NS values and capabilities 	Ericsson

- R2-2312652	Discussion on measurement reporting	CMCC	discussion	Rel-18	NR_UAV-Core

- R2-2312822	Measurement report enhancement for NR UAV	Huawei, HiSilicon

- R2-2313084	Discussion on measurement reporting for event AxHy	Sharp	discussion

- R2-2313171	Remaining issues for measurement reporting enhancements	Xiaomi

- R2-2313314	On UAV Measurement Reporting	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_UAV-Core

- R2-2313341	Further discussion on measurement reporting for NR UAV	China Telecom	discussion

- R2-2313358	On UE capability related to measurement enhancements	Qualcomm Incorporated	discussion	Rel-18

- R2-2313405	Remaining issue on measurement reporting for mobility and interference control	vivo	discussion

- R2-2313436	Remaining issues on measurement reporting enhancements in NR UAV	Samsung	discussion

- R2-2312198	Remaining Issues on Flight Path Reporting	NEC	discussion	Rel-18	NR_UAV-Core

- R2-2312233	Remaining issues for flightpath reporting	ZTE Corporation, Sanechips	discussion	Rel-18

- R2-2312448	Remaining consideration on flight path reporting for NR UAV	DENSO CORPORATION	discussion

- R2-2312823	Further discussion on flight path reporting	Huawei, HiSilicon	discussion	Rel-18

- R2-2312922	UAV Flight Path Reporting	Ericsson	discussion	Rel-18

- R2-2313160	Flight Path reporting	LG Electronics	discussion	Rel-18	NR_UAV-Core

- R2-2313172	Remaining issues for flight path reporting	Xiaomi	discussion	Rel-18	NR_UAV-Core

- R2-2313248	Remaining issues on flight path reporting	Samsung	discussion	Rel-18	NR_UAV-Core

- R2-2313347	Further discussion on flight path reporting for NR UAV	China Telecom	discussion

- R2-2313406	Remaining issue on flight path reporting	vivo	discussion	NR_UAV-Core

- R2-2311761	LS on New PQI values for A2X communication over PC5 reference point (S2-2311556; contact: LGE)

- R2-2312457	Discussion on broadcasting remote id for UAV	Lenovo	discussion	Rel-18

- R2-2312653	Discussion on UAV identification broadcast	CMCC	discussion	Rel-18	NR_UAV-Core

- R2-2312694	SL resource pool handling for BRID and DAA	Samsung	discussion	Rel-18	NR_UAV-Core

- R2-2312834	Further discussion on UAV remote identification broadcast	Huawei, HiSilicon

- R2-2312923	UAV Broadcast Identification	Ericsson	discussion	Rel-18

- R2-2313058	On A2X-related LS from SA2 and its Implications	Nokia, Nokia Shanghai Bell

- R2-2313091	Remaining aspects of PC5-based BRID and DAA support	Qualcomm Incorporated

- R2-2313360	Remaining aspects of PC5-based BRID and DAA support and UE capabilities	Qualcomm

- R2-2313355	Support of PQI Values for DAA/BRID	Beijing Xiaomi Mobile Software

TDocs submitted to RAN3:

- R3-237064 Introduction of Aerial authorization information (Ericsson, AT&T, NTT DOCOMO INC, Qualcomm Incorporated, Intel Corporation, Samsung, NEC, Nokia, Nokia Shanghai Bell, Huawei, ZTE, CATT)

- R3-237065 (BLCR to 38.423) NR support for UAV over Xn (Huawei, China Unicom, China Telecom, CATT, Intel Corporation, ZTE, Ericsson, Qualcomm, Nokia, Nokia Shanghai Bell, Samsung)

- R3-237210 Draft CR to 38.300 on NR support for UAV  (Nokia, Nokia Shanghai Bell, Huawei, Intel Corporation, ZTE, Ericsson, Samsung, CATT, Qualcomm, Deutsche Telekom, NEC)

- R3-237713 Discussion on UAV left issues (ZTE, CMCC, China Telecom)

- R3-237714 TP to TS 38.413 BLCR for A2X communication supporting and flightpath modification (ZTE, CMCC, China Telecom)

- R3-237715 TP to TS 38.423 BLCR for A2X communication supporting and flightpath information modification (ZTE, CMCC,CATT, China Telecom)

- R3-237178 TP (BL CR TS 38.300) Stage 2 corrections for NR support for UAV (Nokia, Nokia Shanghai Bell)

- R3-237382 (TPs for UAV BL CRs for TS 38.423, TS 38.413 and TS 38.473) Remaining open issues for NR UAV (Huawei)

- R3-237519 Text Proposal to NGAP and XnAP Baseline CRs on NR support for UAV (Ericsson)

- R3-237177 (TP to BL CR for TS 38.413)Support of A2X communication (CATT)

- R3-237958 Reply LS on flightpath information forwarding for UAV (RAN2 (ZTE))

TDocs submitted to RAN4:

- R4-2315072       n3 and n38 UAV back off for emissions for EU      Qualcomm Incorporated

- R4-2315774       Running draft CR to TS 38.101-1 - Introduction of Aerial UEs support         Ericsson

- R4-2316091       Proposal for power back off for aerial NR UEs      Nokia, Nokia Shanghai Bell

- R4-2316251       On the applicability of NS signalling for aerial Ues              Nokia, Nokia Shanghai Bell

- R4-2316253       DraftCR on NS signalling for UAVs (38.101-1)       Nokia, Nokia Shanghai Bell

- R4-2316867       UAV AMPR comparison Qualcomm Incorporated

- R4-2317270       Topic summary for [108-bis][147] NR_LTE_UAV Moderator (Nokia)

- R4-2317627       Running draft CR to TS 38.101-1 - Introduction of Aerial UEs support         Ericsson, Nokia,

- R4-2317629       WF on LTE and NR UAV RF requirements                Nokia

- R4-2318154       Topic summary for [109][148] NR_LTE_UAV        Moderator (Nokia)

- R4-2318810       UAV AMPR comparison CP-OFDMA         Qualcomm Incorporated

- R4-2319587       Running CR to TS 38.101-1 - Introduction of Aerial UEs support   Ericsson, Nokia, Huawei

- R4-2319588       Aerial UE - NR definition update Ericsson

- R4-2320027       On power back off for aerial NR UEs         Nokia, Nokia Shanghai Bell

- R4-2320028       draftCR for remaining open issues for power back off for aerial NR UEs     Nokia (revised)

- R4-2320734       Clarification of the applicability of NS_UAV_01 for aerial UEs        Nokia,

- R4-2320836       Discussion on UAV operating bands         Huawei, HiSilicon

- R4-2320837       Draft CR to TS 38.101-1: harmonized MFCN bands for UAV operation in ECC countries,

- R4-2320839       draft LS on the UAV OOBE implementation for MFCN harmonized bands Huawei,

- R4-2321742       Running CR to TS 38.101-1 - Introduction of Aerial UEs support   Ericsson, Nokia, Huawei,

- R4-2321744       draftCR for remaining open issues for power back off for aerial NR UEs     Nokia, Nokia

- R4-2321991       Running CR to TS 38.101-1 - Introduction of Aerial UEs support   Ericsson, Nokia, Huawei

- R4-2322011       Running CR to TS 38.101-1 - Introduction of Aerial UEs support   Ericsson, Nokia, Huawei,

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template