3GPP TSG RAN meeting #102										RP-233392

Edinburgh, Scotland, December 11-15, 2023

## Status Report to TSG

Agenda item:			9.3.4.15

| WI / SI Name | Channel raster enhancement |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_channel_raster_enh |  |  |  |  |
| Unique ID | 991045 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-231469 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 03/2024 (changed from 12/2023) | Performance part: | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 95%: | Performance Part: | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | R4 |
| --- | --- | --- |
| Rapporteur | Name | Christian Bergljung |
|  | Company | Ericsson |
|  | Email | Christian.Bergljung@ericsson.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

TU allocated for remaining RAN4 work (see 2.4.2). No TU needed for RAN2 (see 2.2.2).

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

Implementation of the feature group FG 28-1 [127] in 38.306 and 38.331, see 2.4.1.

Exception for the Rel-18 work item is requested to complete specification of a UE capability in TS 38.306 and TS 38.331 as per the agreed feature group FG 28-1 [127], expected completion date 03/2024.

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4#106bis

The work plan was agreed in R4-2306599.

In the work plan for this meeting:

- Discussion of necessary changes of the channel raster for the BS and UE and the applicability of the channel raster entries for support of functionality specified in RAN1 and RAN2 specifications

- Discussion of the need for UE capability to indicate support of configuration of a narrower UE channel BW inside a wider gNB channel BW in all cases

Agreements:

A WF listing candidate solutions was agreed in R4-2306598.

RAN4#107

In the work plan for this meeting:

- the applicability of the more flexible frequency location also to [the carrier resource grid indicated in] SIB1 or not

  - the need for an additional channel raster and if so, its granularity,

- Decision on the need for a UE capability to indicate support of configuration of a narrower UE channel BW inside a wider gNB channel BW in all cases and channel raster enhancements

- LS to RAN2 on a UE capability if needed

Agreements:

A WF was agreed in R4-2310269 listing the following agreement

- There is no backwards compatibility issue, the carrierBandwidth advertised in SIB1 does not have to be placed on the 100kHz raster.

and the following candidate solutions for further study (a shorter list than that in R4-2306598)

1- 	Approach 1: Specify a new channel raster

1-	New channel raster step size:

Option 1: 5 kHz

Option 2: 10 kHz

Option 3: 50kHz

2-	The new channel raster should be specified for:

both UE and gNB.

3-	For which bands this new channel raster should be specified:

All FR1 bands below 3GHz that that currently have 100 kHz channel raster

2-	Approach 2: Do not specify new channel raster entries

- Alternative 1

1-	Clarify in clause 5.4.2.2 of both the BS and UE specifications that the “RF channel” is mapped to the channel raster at the centre of a carrier grid of a serving cell for at least one numerology as advertised in SIB1.

2-	The network should be able to use the RRC specification for configuring the UE with locations of the UE-specific channel BW within a wider cell-specific bandwidth;

- Alternative 3:

1-	For operating bands with a 100 kHz channel raster, the UE can signal a capability to support a UE specific channel BW that

• consists of a contiguous subset of RBs from SCS-SpecificCarrier in SIB1 and

• is a maximum transmission BW configuration

• but need not be centered on the channel raster.

2-	For UEs with the capability to support a	 UE specific channel BW off the 100 kHz raster in corresponding operating bands, the natural raster for the UE specific channel BW is the RB grid of the carrier bandwidth in SIB1. (For a given numerology and location of the SIB1 carrier bandwidth, its RB grid is considerably sparser than the proposed channel rasters and it includes only valid frequency locations, hence rather the RB grid of the carrier bandwidth in SIB1 should be specified as raster for the UE specific channel BW than a new channel raster.)

Way forward:

For the next meeting, companies are encouraged to detail the expected specification’ updates of their preferred approach.

RAN4#108

A WF was agreed in R4-2314681 listing the following approaches

One of the following approaches/alternatives to be chosen:

1-	Approach 1: Specify a new channel raster

The new channel raster step size: 10 kHz

2-	Approach 2: Do not specify new channel raster entries

- Alternative 1 [the second sub-bullet modified as compared to the WF from RAN4#107]

- Clarify in clause 5.4.2.2 of both the BS and UE specifications that the “RF channel” is mapped to the channel raster at the centre of a carrier grid of a serving cell for at least one numerology as advertised in SIB1.

- The network should be able to use the RRC specification for configuring the UE with locations of the UE-specific channel BW within a wider cell-specific bandwidth subject to UE capability; a subset of requirements applies for the UE-specific CHBW within a wider carrier

- Alternative 3: [from RAN4#107]

- For operating bands with a 100 kHz channel raster, the UE can signal a capability to support a UE specific channel BW that

•	consists of a contiguous subset of RBs from SCS-SpecificCarrier in SIB1 and

•	is a maximum transmission BW configuration

•	but need not be centered on the channel raster.

- For UEs with the capability to support a UE specific channel BW off the 100 kHz raster in corresponding operating bands, the natural raster for the UE specific channel BW is the RB grid of the carrier bandwidth in SIB1. (For a given numerology and location of the SIB1 carrier bandwidth, its RB grid is considerably sparser than the proposed channel rasters and it includes only valid frequency locations, hence rather the RB grid of the carrier bandwidth in SIB1 should be specified as raster for the UE specific channel BW than a new channel raster.)

the same as for RAN#107 but with only one raster candidate granularity for Approach 1 and the following agreement on a UE capability (notwithstanding the approach selected):

Agreement:

A new UE capability will be specified to support the WI objectives

Open issues:

- The new UE capability should be per band.

- FFS from which release should the UE capability be applicable.

- Whether the capability should (at least for some bands) be mandatory from Rel-18 onwards.

Way forward

For the next meeting, companies are encouraged to comment on open issues and/or to detail the expected specification updates of their preferred approach.

The progress is almost in line with the original work plan for RAN4#107.

RAN4#108bis

A WF was agreed in R4-2314681 listing the following agreed approach:

Agreement:

•	Approach 1: Specify a new channel raster for both BS and UE.

The new channel raster step size: 10 kHz

•	The new channel raster should be applied to bands below 3 GHz that currently have 100 kHz channel raster:

-	Whether the support of new channel raster for each band is mandatory or optional depends on operators’ input.

•	The full set of UE Tx and Rx requirements shall be applicable for the channel raster which UE supports.

and for the UE capability:

Agreement:

•	Per-band UE capability will be introduced for the bands below 3GHz with 100KHz channel raster specified.

•	The UE capability is applicable for each band from Rel-16.

An LS to RAN2 on a capability for channel raster enhancement was sent [105].

Introduction of the enhanced raster in affected CRs and the description of the UE capability (feature) the items remaining.

RAN4#109

A WF was agreed in R4-2321752 listing the following

Agreement:

- Definition of the channel enhancement capability:

The capability indicates that the UE supports the RAN4 requirements for UE channel bandwidths located on the enhanced channel raster of a band as specified in 38.101-1 and 38.101-5

- UE behaviour when the channel enhancement capability is absent.

How the network reacts when the capability is absent is up to network implementation.

- Channel spacing definition

The definition of the mapping of an RF channel to the channel raster in sub-clause 5.4.2.2 shall not be changed

The enhanced channel raster is not applicable to intra-band contiguous CA.

Affected RAN4 specifications were completed: CRs were agreed in [116] against 38.101-1, [118] against 38.104, [120] against 38.101-5 and in [122] against 38.108. The 38.307 is also listed as an affected RAN4 specification in the WID but will be removed from this list.

The UE capability: the channel raster capability was subsequently included in the RAN4 feature list sent to RAN2 [127][128] as FG 28-1 with the following

component (per band): the UE supports the requirements for UE channel bandwidths located on the enhanced channel raster of a band as specified in TS 38.101-1, 38.101-4, TS 38.101-5 [and in TS38.133]

consequence of not supported: [N/A (not specified)]

The UE capability is applicable from Rel-16 (early implementation) as per the RAN4#108bis agreement.

#### 2.4.2	Remaining Open issues

Discussions on mandated/optional support of the feature for applicable bands remain (also expected to be part of subsequent Rel-18 maintenance).

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## References

RAN4#106bis

[1]	R4-2304462, Work plan for the WI on channel raster enhancement, Ericsson

[2]	R4-2306599, Work plan for the WI on channel raster enhancement, Ericsson

[3]	R4-2304262, Views on the UE channel raster for FR1, Intel Corporation

[4]	R4-2304263, Clarification of UE capability enhancement of channel raster in bands below 3GHz, Intel Corporation

[5]	R4-2304264, Clarification of UE capability enhancement of channel raster in bands below 3GHz, Intel Corporation

[6]	R4-2304265, Clarification of UE capability enhancement of channel raster in bands below 3GHz, Intel Corporation

[7]	R4-2304266, Clarification of UE capability enhancement of channel raster in bands below 3GHz, Intel Corporation

[8]	R4-2304334, Initial considerations for the flexible channel raster, Apple

[9]	R4-2304335, 	Flexible channel raster for the NTN bands, Apple

[10]	R4-2304463, On changing the BS and UE channel raster, Ericsson

[11]	R4-2304653, Discussion on UE and BS channel raster, CMCC

[12]	R4-2304664, Discussion on channel raster for SIB1 CBW and UE-specific CBW, China Telecom

[13]	R4-2304876, Discussion on channel raster enhancement, Nokia, Nokia Shanghai Bell

[14]	R4-2305308, Discussion on Channel raster enhancement, Huawei, HiSilicon

[15]	R4-2305353, Discussion on channel raster enhancement, ZTE Corporation

[16]	R4-2305517, Channel Raster Enhancements, Qualcomm Incorporated

[17]	R4-2305585, Channel raster enhancement, Samsung Electronics Co., Ltd

[18]	R4-2305702, Consideration on channel raster enhancement, MediaTek Inc.

[19]	R4-2305828, 10 kHz raster for bands that currently specify the 100 kHz raster, T-Mobile USA

[20]	R4-2304336, UE capability for the flexible channel raster, Apple

[21]	R4-2304464, On a UE capability for channel raster enhancement, Ericsson

[22]	R4-2304654, Discussion on UE capability of new channel raster, CMCC

[23]	R4-2304877, Discussion on UE capability for channel raster enhancement, Nokia, Nokia Shanghai Bell

[24]	R4-2305560, UE capability for channel raster enhancement, Huawei, HiSilicon

[25]	R4-2305703, Consideration on enhanced channel raster capability, MediaTek Inc.

[26]	R4-2306205, Topic summary for [106-bis-e][126] NR_channel_raster_enh, Moderator (Ericsson)

[27]	R4-2306288, Topic summary for [106-bis-e][126] NR_channel_raster_enh, Moderator (Ericsson)

[28]	R4-2306598, WF on channel raster enhancement, Ericsson

RAN4#107

[29]	R4-2308786, Channel Raster Enhancements Scope, Qualcomm Incorporated

[30]	R4-2307501, Discussion on channel raster enhancement, Nokia, Nokia Shanghai Bell

[31]	R4-2307580, Discussion on UE and BS channel raster, CMCC

[32]	R4-2307737, Options for enhancing the channel raster, Ericsson

[33]	R4-2307763, Discussion on Channel raster enhancement, Huawei, HiSilicon

[34]	R4-2307847, Discussion on UE and BS channel raster, ZTE Corporation

[35]	R4-2307976, Discussion on channel raster for SIB1 CBW and UE-specific CBW, China Telecom

[36]	R4-2308385, Further discussion on NR channel raster enhancement, MediaTek Inc.

[37]	R4-2308787, Channel Raster Enhancements, Qualcomm Incorporated

[38]	R4-2309049, Further considerations for the flexible channel raster, Apple

[39]	R4-2309711, Proposal for the 100 kHz raster issue, T-Mobile USA, TELUS

[40]	R4-2309712, Input on the Way Forward on channel raster enhancements, T-Mobile USA

[41]	R4-2307738, Clarification of the channel raster and capability for UE-specific channel bandwidth, Ericsson

[42]	R4-2307739, Clarification of the channel raster, Ericsson

[43]	R4-2307740, Clarification of the channel raster, Ericsson

[44]	R4-2307502, Discussion on UE capability for channel raster enhancement, Nokia, Nokia Shanghai Bell

[45]	R4-2307581, Discussion on UE capability of new channel raster, CMCC

[46]	R4-2307764, UE capability for channel raster enhancement, Huawei, HiSilicon

[47]	R4-2307848, Discussion on UE capability, ZTE Corporation

[48]	R4-2308386, Further discussion on UE capability for channel raster enhancement, MediaTek Inc.

[49]	R4-2309050, UE capability for the flexible channel raster, Apple

[50]	R4-2307741, Draft LS on a capability for UE-specific channel bandwidth location, Ericsson

[51]	R4-2310012, Topic summary for [107][129] NR_channel_raster_enh, Moderator (Ericsson)

[52]	R4-2310269, WF on NR channel raster enhancement, Ericsson

RAN4#108

[53]	R4-2313503, Discussion on NR channel raster enhancement. Google Inc.

[54]	R4-2311067, Discussion on channel raster for SIB1 CBW and UE-specific CBW, China Telecom

[55]	R4-2311220, Further considerations for the flexible channel raster, Apple

[56]	R4-2311819, Discussion on UE and BS channel raster, CMCC

[57]	R4-2312020, Discussion on UE and BS channel raster, ZTE Corporation

[58]	R4-2312163, More on the channel raster enhancement, Ericsson

[59]	R4-2312314, Discussion on channel raster enhancement, Huawei, HiSilicon

[60]	R4-2312439, Channel raster enhancements, Qualcomm Incorporated

[61]	R4-2312525, Discussion on channel raster enhancement, Nokia, Nokia Shanghai Bell

[62]	R4-2313616, On channel raster enhancement for TN, MediaTek Inc.

[63]	R4-2312164, Clarification of the channel raster and capability for UE-specific channel bandwidth, Ericsson

[64]	R4-2312165, Clarification of the channel raster, Ericsson

[65]	R4-2312166, Clarification of the channel raster, Ericsson

[66]	R4-2312315, Draft CR to introduce 10 kHz channel raster in Rel-18, Huawei, HiSilicon

[67]	R4-2311221, Flexible channel raster for NTN, Apple, Globalstar, Ligado Networks, Inmarsat, Hughes/Echostar

[68]	R4-2311820, Discussion on NTN channel raster, CMCC

[69]	R4-2313617, On channel raster enhancement for NTN, MediaTek Inc.

[70]	R4-2312167, Clarification of the channel raster and capability for UE-specific channel bandwidth, Ericsson

[71]	R4-2311821, Discussion on UE capability of new channel raster, CMCC

[72]	R4-2312021, Discussion on UE capability, ZTE Corporation

[73]	R4-2312316, UE capability for channel raster enhancement, Huawei, HiSilicon

[74]	R4-2312440, Channel raster enhancement capability, Qualcomm Incorporated

[75]	R4-2312526, Discussion on UE capability for channel raster enhancement, Nokia, Nokia Shanghai Bell

[76]	R4-2313618, UE capability for channel raster enhancement, MediaTek Inc.

[77]	R4-2312168, Draft LS to RAN2 on a capability for UE-specific channel bandwidth location, Ericsson

[78]	R4-2314211, Topic summary for [108][129] NR_channel_raster_enh, Moderator (Ericsson)

[79]	R4-2314681, WF on channel raster enhancement, Ericsson

RAN4#108bis

[80]	R4-2316380, Discussion on Channel raster enhancement, Huawei, HiSilicon

[81]	R4-2315125, Discussion on channel raster enhancement, CATT

[82]	R4-2315179, Discussion on TN channel raster, CMCC

[83]	R4-2315370, On design options for enhanced channel raster, Apple

[84]	R4-2315511, Discussion on channel raster enhancement, Nokia, Nokia Shanghai Bell

[85]	R4-2315582, Channel Raster Enhancements, Qualcomm Incorporated

[86]	R4-2315629, Discussion on UE and BS channel raster, ZTE Corporation

[87]	R4-2315779, Channel raster enhancement without a new channel raster, Ericsson

[88]	R4-2316661, Further discussion on channel raster enhancement for TN, MediaTek Inc.

[89]	R4-2316802, Discussion on NR channel raster enhancement

[90]	R4-2315780, Enhancement of the channel raster: etries for UE-specific channel bandwidths, Ericsson

[91]	R4-2315781, Enhancement of the channel raster: correction of mapping to the RF channel, Ericsson

[92]	R4-2315782, Enhancement of the channel raster: informative note on shifted carrier frequencies, Ericsson

[93]	R4-2316381, Draft CR to introduce 10 kHz channel raster in Rel-18, Huawei, HiSilicon

[94]	R4-2316662, Further discussion on channel raster enhancement for NTN, MediaTek Inc.

[95]	R4-2315783, Enhancement of the channel raster: entries for UE-specific channel bandwidths, Ericsson

[96]	R4-2315180, Discussion on UE capability of new channel raster, CMCC[98]

[97]	R4-2315371, On UE capability for enhanced channel raster, Apple

[98]	R4-2315512, Discussion on UE capability for channel raster enhancement, Nokia, Nokia Shanghai Bell

[99]	R4-2315581, Channel Raster Enhancement Capability, Qualcomm Incorporated

[100]	R4-2315630, Discussion on UE capability, ZTE Corporation

[101]	R4-2316382, UE capability for channel raster enhancement, Huawei, HiSilicon

[102]	R4-2316663, On UE capability on channel raster enhancement, MediaTek Inc.

[103]	R4-2315784, Draft LS to RAN2 on a capability for UE-specific channel bandwidth location, Ericsson

[104]	R4-2317606, Draft LS to RAN2 on a capability for UE-specific channel bandwidth location, Ericsson

[105]	R4-2317773, Draft LS to RAN2 on a capability for channel raster enhancement, Ericsson

[106]	R4-2317248, Topic summary for [108-bis][124] NR_channel_raster_enh, Moderator (Ericsson)

[107]	R4-2317607	WF on NR channel raster enhancement

RAN#109

[108]	R4-2318411, On specification options for enhanced channel raster, Apple

[109]	R4-2318710, Discussion on optionality of enhanced channel raster, MediaTek Inc.

[110]	R4-2318823, Channel Raster Enhancements, Qualcomm Incorporated

[111]	R4-2318929, Discussion on the bands that mandatory support of new channel raster, CMCC

[112]	R4-2319433, Specification changes for support of the enhanced channel raster, Ericsson

[113]	R4-2319676, Necessary specification changes for introduction of 10 KHz channel raster, Huawei, HiSilicon

[114]	R4-2318711, DraftCR to TS 38.101-1 on system parameters for supporting enhanced channel raster, MediaTek Inc.

[115]	R4-2319165, Introduction of an enhanced channel raster, Nokia, Nokia Shanghai Bell

[116]	R4-2321749, Introduction of an enhanced channel raster, Nokia, Nokia Shanghai Bell

[117]	R4-2319196, CR to TS38.104: Introduction of an enhanced channel raster, ZTE Corporation

[118]	R4-2321750, CR to TS38.104: Introduction of an enhanced channel raster, ZTE Corporation

[119]	R4-2318412, Enhanced channel raster for NTN bands, Apple, Ligado Networks, Inmarsat, Viasat, Globalstar, Thales, Hughes/Echostar

[120]	R4-2318413, Introduction of the enhanced channel raster to TS 38.101-5, Apple Inc., Inmarsat, Viasat, Globalstar

[121]	R4-2319677, CR to TS 38.108: Introduction of an enhanced channel raster, Huawei, HiSilicon

[122]	R4-2321751, CR to TS 38.108: Introduction of an enhanced channel raster, Huawei, HiSilicon

[123]	R4-2319197, Discussion on UE capability for enhanced channel raster, ZTE Corporation

[124]	R4-2319434, The UE capability for support of the enhanced channel raster, Ericsson

[125]	R4-2318133, Topic summary for [109][127] NR_channel_raster_enh, Moderator (Ericsson)

[126]	R4-2321752, WF on NR_channel_raster_enh, Ericsson

[127]	R4-2321993, RAN4 UE feature list for Rel-18 (version 2)

[128]	R4-2321823, LS on RAN4 UE feature list for Rel-18 (version 2)

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template