3GPP TSG RAN Meeting #102	RP-233403

Edinburgh, Scotland, December 11-15, 2023

Agenda Item:	9.1.2.1

Source:	InterDigital, Inc.

Title:	Mobility Enhancements for Rel-19

Document for:	Discussion, Decision

# Introduction

In Rel-15, the mobility support was limited to basic L3 handover and basic beam level mobility (L1/2 intra-cell).

The goal of conditional handover (CHO) in Rel-16 was to improve mobility robustness by reducing the radio link failure (RLF) and handover failure (HOF) rates through RRC pre-configuration of target cells, and configuration of a UE trigger (conditional trigger) by means of RRC measurement event. In Rel-17, additional scenarios were addressed (inter-SN, CPC, CPA) and in Rel-18 further enhancements were introduced (selective activation of SCG, CHO including MCG and target SCG for CPAC) A main limitation of CHO/CPAC is that handover interruption time is not reduced compared to basic L3 reconfiguration.

The goal of dual active protocol stack (DAPs) in Rel-16 was to reduce the handover interruption time by maintaining the source gNB connection for the DL for some selected bearers after the reception of the handover command (RRC Reconfiguration) until an RRC reconfiguration indicating the release of the source connection is received from the target (i.e., after the handover is completed). Limitations of DAPs include only being applicable to the DL, PCell handover, and no support for FR2-FR2.

The goal of inter-cell beam management (ICBM) in Rel-17 was to enable the UE to receive or transmit dedicated channels via a TRP associated with a PCI different from the PCI of a serving cell. A main limitation of ICBM is not supporting cell switch.

The goal of L1/L2 mobility enhancements in Rel-18 [1] is to enable a serving cell change via L1/L2 signalling in order to reduce the latency, overhead and interruption time by pre-configuration of multiple candidate cells by RRC, early L1 measurement reporting of target cells/beams, early DL synchronization (TCI state activation), early UL synchronization (TA acquisition), and handover (reconfiguration) trigger using MAC CE. Limitations of LTM in rel-18 include only being applicable to intra-gNB handover, mobility robustness (due to not supporting CHO), throughput degradation (due to not supporting early beam refinement, TRS tracking and CSI acquisition), and data loss during/after handover (due to not supporting HARQ continuation).

While each of the mobility enhancements introduced in previous releases provides improvements in one or multiple areas, each also has its own limitations. In Rel-19, the handover interruption time, latency, overhead can be addressed along with mobility robustness (RLF, HOF KPIs) and LTM can be supported in inter-gNB scenarios.

# Discussion

  - Summary of proposals to the Rel-19 workshop

A number of tdocs were submitted to the Rel-19 workshop, motivating further work for mobility enhancement, which can be broadly categorized as follows:

- Inter-CU LTM

  - Security update and L2 reset.

  - Xn/NG enhancement

- LTM Measurement enhancements:

  - L1 measurement Dynamic update / overhead reduction / efficiency

  - Event triggered L1 report

  - CSI-RS support

  - Inter-Freq support

  - SINR support

  - Introduction of L1 Filtering

- CHO:

  - Conditional LTM (UE based LTM trigger)

  - RACH-less CHO

  - LTM/CHO co-existence

  - Inter-RAT CHO

- Other

  - Selective activation of MCG

  - Daps enhancement

  - Group handover

  - L3 measurement prioritisation

  - L3 measurement reporting enhancement

The following was noted in the Chair’s summary [2]:

- Strong desire to further enhance mobility management in Rel-19 - One area raised by many companies is to further evolve Layer 1/2 triggered mobility (LTM), e.g., inter-CU (central unit) LTM - Need to focus on the critical needs for commercial deployments and maintain a reasonable workload - Especially considering it’s a continuation from previous releases

  - Summary progress at RAN#101

After the offline discussion at RAN#101 the following was summarized by the moderator [3]:

Based on the RAN#101 offline discussion the following objectives should be agreeable: - Specify support for inter -CU Layer 2 Mobility (LTM) - Next level questions still to be discussed/decided: - Whether to support cases where CU is acting as MN, and when CU is acting SN? - For case where CU is acting as SN, whether it is support for EN-DC and/or NR-DC? - In answering the above questions, focus should be practical deployments. - Measurements related enhancements for purpose of supporting LTM: - Specify event triggered L1 measurement reporting for triggering LTM - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management and/or other physical layer operations on candidate cells before LTM During the RAN#101 offline discussion there was strong interest in the following - Support of conditional mobility (including set of candidate cells, UE evaluated mobility conditions, ability for UE to perform subsequent mobility procedures without need for RRC configuration) along with short mobility interruption time (similar to that achievable with LTM). - Next Step question to consider the most appropriate approach to achieve this aim. For example, by specifying conditional triggering of LTM, or specifying support of Early TA or RACHless (as defined for LTM) to be used as part of layer 3 CHO, or possibly other approaches. If this questions cannot be concluded during the Rel-19 scoping discussion then it may be possible to start with a RAN2 study phase. During RAN#101 offline discussion the following were agreed to be reviewed again at RAN#102 with consideration to the outcome of the Rel-18 work item. - NR-DC cases supported by LTM. Note the following common understanding from the offline: LTM is important for MCG only case. In case of NR-DC then LTM on the SCG is the important case (e.g. in FR2), followed by LTM in MCG with SCG configured (SCG unchanged by MCG LTM) During on the RAN#101 offline discussion the following were not discussed - LTM related: - L1 measurements filtering enhancement for LTM [CATT, ZTE] - Dynamic L1 meas/reporting configuration update (e.g. via MAC CEs, or UE autonomous based on configured criteria) [Qcom, InterDigital, LGE, CATT] - L3 measurement enhancements to support LTM [Samsung, Ericsson] - Support LTM, CHO enhancements for other features, e.g. UAV (height-dependent CHO), NTN, IAB, NES, etc. [ZTE] - UL measurement based LTM [vivo] - Failure handling enhancements [CMCC] - Group LTM (expanding group HO principles to LTM) [Fujitsu] - Activation/deactivation of LTM candidates [LGE] - Enhancements of unified TCI state activation/indication and TA indication for the LTM candidate cell in scenarios where the UE is configured with ICBM/inter-cell mTRP at the same time [Huawei] - Latency/interruption improvements when source and target cells are both serving cells (for CA) [Apple] - Integrity of MAC CE control message [Apple] - L3 HO enhancements - Make Before Break (baseline LTE MBB) [Apple] - Group HO (for UEs with the same mobility properties) [DT] - HO with CPAC configured for in source and target [Apple] - Avoid data rate degradation during DAPS handover (e.g. DAPS with CA/DC) [Huawei,China Unicom] - DAPs for FR2 [China Unicom] - General measurement enhancements (not specific to LTM) - RRM measurement based on network-configured weighting (RAN4 RRM objective?) [DT, MediaTek] - Conditional L1 and L3 measurements based on network configured criteria [DT, MediaTek] - Inter-frequency measurement priority [Ericsson] - Beam specific parameters for handover measurements (e.g. TTT, CIO) [Nokia] - Other - Further enhancements to Early Measurement and Reporting [Nokia] - CA enhancements for FR2 (e.g. fast activation fast failure detection and recovery)[China Unicom]

According to the RAN Chair’s draft summary for the R19 package in [4], the following potential objectives are suggested

- Potential objectives: - Specify support for inter-CU Layer 2 Mobility (LTM) - Prioritize the case when CU is acting as MN when DC is not configured - Whether to additionally support e.g. when CU is acting SN? (focus on practical deployments) - In this case, it is supported NR-DC only - Measurements related enhancements for purpose of supporting LTM: - Specify event triggered L1 measurement reporting for triggering LTM depending on the detailed discussion in RAN#102, resolve how to avoid overlapping with similar objective in MIMO WI - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management and/or other physical layer operations on candidate cells before LTM - Support of conditional mobility (including set of candidate cells, UE evaluated mobility conditions, ability for UE to perform subsequent mobility procedures without need for RRC configuration) along with short mobility interruption time (similar to that achievable with LTM). - Next Step question to consider the most appropriate approach to achieve this aim. For example, by specifying conditional triggering of LTM, or specifying support of Early TA or RACHless (as defined for LTM) to be used as part of layer 3 CHO, or possibly other approaches. If this questions cannot be concluded during the Rel-19 scoping discussion then it may be possible to start with a RAN2 study phase. - Anything else (e.g., RAN4 related)?

  - Inter-CU LTM

[Motivation]

Rel-18 LTM is limited to the intra-CU scenario. While DAPs could be used to reduce interruption in the inter-CU scenario, this has other limitations which could require significant effort to overcome. Rather, extending LTM to cover the inter-CU scenario would be more attractive to provide the reduced latency and interruption time.

[Proposed work]

To support LTM for inter-CU, at least the security update needs to be supported in LTM cell switch and hence the security key information needs to be provided in some way in the LTM candidate configuration. Other L2 enhancements are likely to be needed, for example PDCP re-establishment is not performed for R18 intra-gNB cell switch.

One of the main benefits of LTM is reduced interruption time due to performing DL and UL synchronization to a target cell ahead of the cell switch to allow a RACH-less handover to occur when the L1 measurement reports indicate a suitable candidate cell. Therefore, the early synchronization procedures introduced in Rel-18, as well as the L1 reporting framework need to be extended to cover the inter-CU scenario.

In addition, some enhancement to Xn and/or NG signalling is necessary to exchange the candidate configurations between gNBs.

While it is assumed that support needs to be introduced for SpCell change on MCG only, one of the primary use-cases of FR2 is the NR-DC case where MCG is on FR1 and SCG is on FR2. Since LTM improves latency and interruption it is particularly beneficial for SCG change in FR2. Another NR-DC case which would be desirable to support is the SpCell on MCG change (SCG not changed).

Hence, inter-CU LTM should be supported in at least the cases of:

- SpCell change on MCG only

- SpCell change on SCG (MCG unchanged).

- SpCell change on MCG (SCG unchanged)

While other cases could be possible to support, such as MCG/SCG switch, this will involve additional work to support and therefore should be sufficiently justified.

For inter-CU we propose the following objective:

- Proposal 1: A Rel-19 work item on NR mobility enhancements includes the following objective:

Specify support for inter-CU Layer 2 Mobility (LTM) in scenarios SpCell change for MCG only, SpCell change on SCG (MCG unchanged), SpCell change on MCG (SCG unchanged) [RAN2, RAN1, RAN3]

- Signalling support for configuration of inter-CU candidate cells, including Xn/NG interface signalling [RAN2, RAN3]

- Layer 2 enhancement including security update and PDCP anchor relocation. [RAN2]

- Support for inter-CU TA management and early DL sync. [RAN2, RAN1, RAN3]

- Enhanced L1 measurement and reporting framework [RAN1, RAN2, RAN3]

  - RACH-less CHO

[Motivation]

While LTM provides improved latency and handover interruption time, support for CHO improves robustness. Like in L3 mobility, LTM can suffer from RLF due to late L1 measurement reports or HOF due to late LTM command. Thus, by combining LTM with CHO, we allow to get the best of both worlds, with a handover mechanism that is both robust (in terms of RLF, HOF reduction) and with a reduced latency and interruption.

[Proposed work]

By utilizing the LTM TA acquisition and early DL sync ahead of a conditional (UE triggered) cell switch, we can improve the handover robustness, while allowing for low handover interruption by executing a RACH-less handover.

Since R18 LTM introduced early UL sync (TA acquisition) and RACH-less cell switch, it is natural to re-use this mechanism as much as possible.

The main question is whether to (re)use a L3 based conditional reconfiguration trigger or whether to introduce a L1 based conditional trigger. As this is a technical matter, this will need to be evaluated in WGs. Some initial study phase would therefore be beneficial to allow for comparison of the pros and cons, and the performance, for each approach.

Another question is whether to re-use the LTM candidate cell framework for handover execution, or to use the CHO framework. In case it is decided to use a L1 based conditional trigger (E.g. using L1 based event triggers defined in R19 as part of another objective) then the obvious choice would be to re-use the LTM framework since this also includes early TA acquisition. In case it is decided that a L3 based measurement trigger is used, then it needs to be discussed whether to introduce a L3 based measurement trigger to the LTM framework, or to introduce early UL sync as defined for LTM into the CHO framework. Again, this is a technical matter that needs to be discussed in WGs.

The overall objective should be to enable a RACH-less CHO by re-using the early DL/UL sync procedures for LTM and executing mobility based on a UE evaluated (conditional) trigger. The question of whether to use a L1 or L3 based trigger, and the question of whether to re-use LTM or CHO signalling framework, can be considered in an initial study phase during 1Q.

For CHO with LTM we propose the following objective, with an initial study phase of 1 quarter to decide on the overall solution direction:

- Proposal 2: A Rel-19 work item on NR mobility enhancements includes the following objective:

Study and specify mechanisms and procedures for RACH-less conditional mobility [RAN2, RAN1, RAN3]

- Support for conditional (UE evaluated) trigger for RACH-less handover. [RAN2, RAN1, RAN3]

- L1 based measurement and L3 based measurement trigger should be considered in the initial study.

- Re-use of the LTM framework and re-use of the CHO framework for handover execution should be considered in the initial study.

Note: Early UL/DL sync for TA acquisition introduced in Rel-18 should be the re-used as much as possible.

The initial study could be part of an initial study phase before normative work begins.

  - LTM measurement enhancements

[Motivation]

LTM in Rel-18 relies on a relatively static configuration for L1 measurements, which results in significant overhead and/or limits the amount of pre-configuration that can be done. During Rel-18, enhancements were discussed to allow for a more efficient L1 measurement and reporting mechanism but due to lack of time/too large scope these have not been specified in Rel-18, for example L1 event triggered reporting and dynamic activation of L1 measurements was discussed in order to reduce overhead from frequent RRC reconfiguration to update handover/measurement parameters in different areas of the cell or group of LTM cells.

In addition, the Rel-18 LTM does not support CSI-RS measurements and therefore beam refinement ahead of LTM trigger is not possible, resulting in a reduced throughput after cell switch (i.e., beam refinement has to be done after the cell switch).

[Proposed work]

We propose to work on improving the L1 measurements and reporting procedure by introducing L1 event triggered measurement reporting. The main benefit of event triggered L1 reporting is the reduction of reporting overhead, therefore we think that this objective is coupled somewhat with dynamic measurement activation. In addition, beam refinement before LTM (including CHO) should be made possible by introducing CSI-RS measurements, TRS tracking, and CSI acquisition for LTM. For example, event triggered L1 reporting may be performed using SSB resources, enabling the dynamic activation of CSI-RS measurements for beam refinement ahead of LTM execution.

For improving LTM measurement procedures we propose the following objective:

- Proposal 3: A Rel-19 work item on NR mobility enhancements includes the following objective:

Specify enhancements to L1 measurement and reporting procedures [RAN1, RAN2]

- Support for L1 event triggered reporting. [RAN1, RAN2]

- Support for dynamic measurement activation/deactivation for overhead reduction [RAN1, RAN2]

- Support for CSI-RS, TRS tracking, and CSI acquisition for beam refinement before LTM (including CHO) [RAN1, RAN2]

# Conclusion

In this contribution, R19 scope for mobility enhancements is discussed. The following proposals are provided:

- Proposal 1: A Rel-19 work item on NR mobility enhancements includes the following objective:

Specify support for inter-CU Layer 2 Mobility (LTM) in scenarios SpCell change for MCG only, SpCell change on SCG (MCG unchanged), SpCell change on MCG (SCG unchanged) [RAN2, RAN1, RAN3]

- Signalling support for configuration of inter-CU candidate cells, including Xn/NG interface signalling [RAN2, RAN3]

- Layer 2 enhancement including security update and PDCP anchor relocation. [RAN2]

- Support for inter-CU TA management and early DL sync. [RAN2, RAN1, RAN3]

- Enhanced L1 measurement and reporting framework [RAN1, RAN2, RAN3]

- Proposal 2: A Rel-19 work item on NR mobility enhancements includes the following objective:

Study and specify mechanisms and procedures for RACH-less conditional mobility [RAN2, RAN1, RAN3]

- Support for conditional (UE evaluated) trigger for RACH-less handover. [RAN2, RAN1, RAN3]

- L1 based measurement and L3 based measurement trigger should be considered in the initial study.

- Re-use of the LTM framework and re-use of the CHO framework for handover execution should be considered in the initial study.

Note: Early UL/DL sync for TA acquisition introduced in Rel-18 should be the re-used as much as possible.

- Proposal 3: A Rel-19 work item on NR mobility enhancements includes the following objective:

Specify enhancements to L1 measurement and reporting procedures [RAN1, RAN2]

- Support for L1 event triggered reporting. [RAN1, RAN2]

- Support for dynamic measurement activation/deactivation for overhead reduction [RAN1, RAN2]

- Support for CSI-RS, TRS tracking, and CSI acquisition for beam refinement before LTM (including CHO) [RAN1, RAN2]

# References

RP-223520 “Revised WID on Further NR mobility enhancements” MediaTek Inc., Apple.

RWS-230488 “RAN Chair’s Summary of Rel-19 Workshop”, RAN Chair.

RP-232618, “Moderator's summary on Rel-19 Mobility Enhancements [08_R19_R2_mobility]”, Intel Corporation

RP-232745, “Summary for RAN Rel-19 Package: RAN1/2/3-led”, RAN Chair