3GPP TSG RAN Meeting #102									RP-233570

Edinburgh, Scotland, December 11-15, 2023

Agenda item:	9.1.2.1

Source: 	China Telecom

Title: 	Considerations on Mobility enhancements in Rel-19

Document for:	Discussion/Decision

- Introduction

In the summary for RAN Rel-19 Package [1], the potential objectives for mobility enhancements were captured as below:

In this contribution, we provide our views on the scope of Mobility enhancements in Rel-19, including inter-CU LTM, conditional LTM and early TA management for CHO.

- Discussion

Inter-CU LTM

During the RAN-plenary meeting in September, almost all companies support to specify the inter-CU LTM mechanism in Rel-19, what has not been reached as consensus is the applicable scenarios for inter-CU LTM. During the offline discussion in the last RAN-plenary meeting, the common understanding among companies is that the LTM is important for standalone with MCG only scenario, the remaining issue toward whether to support LTM in NRDC scenario.

Figure 2: Applicable scenarios for inter-CU LTM

Form our understanding, the LTM mechanism is mainly used to guarantee the emerging high requirements services (such as immersive XR), which requires handover with short interruption and high throughput. In order to meet higher service requirements, the network needs to apply wireless resources in higher frequency bands, which means NRDC may be deployed, especially for SCGs to be deployed in FR2. Therefore, the LTM mechanism should be supported in NRDC scenario.

Proposal 1: Support to specify inter-CU LTM in NR standalone and NR-DC scenario for Rel-19.

In Rel-18, the subsequent LTM for intra-CU scenario is supported, we think the similar mechanism should also be introduced for inter-CU LTM. For inter-CU LTM, the handover parameters exchanged between gNBs, and then needs to be transmitted over Uu interface. Frequent update of handover parameters results in high inter-node singling overhead. By supporting subsequent LTM, the signaling overhead for over Xn and Uu interface can be reduced to a certain extent.

Proposal 2: Support to specify subsequent inter-CU LTM procedures without RRC reconfiguration in Rel-19.

Conditional LTM

Figure 1: Flow chart for conditional handover in intra-CU scenario

In Rel-16, the conditional handover was introduced to enhance the handover robustness, similar mechanism can also be introduced for LTM. In conditional LTM, the UE could trigger the cell switch execution based on pre-configured conditions, in which the signaling overhead and interaction delay for candidate cell configurations over Uu, F1 and Xn interface can be reduced. In addition, the early TA management (such as PDCCH-ordered RACH or RACH-less solution) can also be introduced for conditional LTM to reduce the latency duo to random access procedure.

Proposal 3: Support to specify conditional LTM for both intra-CU and inter-CU scenarios in Rel-19.

In Rel-18, we support subsequent LTM execution towards pre-configured LTM candidate cells without RRC reconfiguration. This mechanism could also be supported in conditional LTM to further reduce signaling overhead and data interruption time, at least for intra-CU scenarios.

Proposal 4: Support to specify subsequent conditional LTM at least for intra-CU scenarios in Rel-19.

Early TA management for CHO

In Rel-18, the early TA management mechanism was introduced to LTM to reduce the data interruption time, in which the UE could perform DL synchronization and TA acquisition toward candidate target cell(s) before receiving the LTM cell switch command. Similar early TA management mechanism can also be supported to CHO.

Compared with LTM, CHO is based on L3 signaling, which may perform better handover performance in low-frequency scenarios. By introducing early TA management into CHO, the data interruption time can be further reduced, reaching a better handover performance. In addition, the early TA management for LTM has already been specified, which can be considered as the baseline for CHO, the specification work to support early TA management for CHO is not too complex.

Moreover, from the perspective of standard evolution, the technology needs to be more refined and integrated, especially in the R19 stage, the feature of the previous release should be further strengthened and improved to form a more harmonious and unified technical system, which is of great significance for both 5G-A and Pre-6G.

Proposal 5: Support to specify the early TA management for CHO in Rel-19.

- Conclusion

In this paper, we provide our views on Rel-19 Mobility enhancements, and provide the following proposals:

Proposal 1: Support to specify inter-CU LTM in NR standalone and NR-DC scenario for Rel-19.

Proposal 2: Support to specify subsequent inter-CU LTM procedures without RRC reconfiguration in Rel-19.

Proposal 3: Support to specify conditional LTM for both intra-CU and inter-CU scenarios in Rel-19.

Proposal 4: Support to specify subsequent conditional LTM at least for intra-CU scenarios in Rel-19.

Proposal 5: Support to specify the early TA management for CHO in Rel-19.

- References

RP-232745, Summary for RAN Rel-19 Package: RAN1/2/3-led, MediaTek, RAN Chair.

RP-236180, Moderator's summary on Rel-19 Mobility Enhancements [08_R19_R2_mobility], Moderator (Intel Corporation)

RP-231590, Deutsche Telekom’s Mobility Enhancements objectives, Deutsche Telekom

RP-231776, Discussion on further enhancement for mobility in Rel19, OPPO

RP-231794, Rel-19 Mobility Enhancements (RAN2-led), vivo

RP-231855, Views on scope for NR Mobility enhancements in Rel-19, Qualcomm Incorporated

RP-231871, Motivations for further mobility enhancements, NEC

RP-231906, Mobility enhancements for Rel-19,	Spreadtrum Communications

RP-231960, Rel-19 Mobility Enhancements, Nokia, Nokia Shanghai Bell

RP-231987, Mobility Enhancements for Rel-19, InterDigital, Inc.

RP-232028, Scope of Mobility Enhancements in Rel-19, Samsung

RP-232044, Proposal on LTM enhancement in Rel-19, LG Electronics Inc

RP-232053, On the scope of R19 mobility enhancements,	CATT

RP-232077, Views on Rel-19 Mobility enhancements, NTT DOCOMO, INC

RP-232128, Mobility Enhancements in Rel-19, Fujitsu Limited

RP-232149, Mobility Enhancements in Rel-19, Xiaomi EV Technology

RP-232150, Further Mobility Enhancement in Rel-19, Lenovo

RP-232174, Views on Mobility enhancement in Rel-19, ZTE, Sanechips

RP-232209, Considerations on mobility enhancements for Rel-19, China Mobile International Ltd

RP-232323, [RAN2-led] Mobility Enhancements, MediaTek Inc.

RP-232339, Mobility enhancements in Rel-19, Huawei, HiSilicon

RP-232398, Further enhancement on mobility in R19, China Unicom

RP-232428, Scoping Rel-19 Mobility Enhancements Objectives, Apple Inc

RP-232433, Rel-19 Mobility enhancements, Ericsson

RP-232553, Discussion on Rel-19 Mobility enhancements	, Intel Corporation

RP-232581, Views on Mobility Enhancements,	CEWiT