3GPP TSG-RAN Meeting 102	RP-233600

Edinburgh, Scotland, December 11-15, 2023

Agenda Item:	9.1.2.4

Source:	China Unicom

Title:	Consideration on AI/ML for mobility

Document for:	Discussion

1	Introduction

At the RAN#101 meeting, most companies showed interests on supporting AI/ML functionality for mobility scenario, and the motivations and draft objectives are also summarized by the moderator in [1]. In [2], RAN chair also propose some guidelines for the potential objectives on AI/ML mobility topic, some issues still need further discussing, e.g. how and whether to down scope AI/ML for mobility SI from L3 mobility and LTM, whether to take LCM framework as baseline, etc.

In this paper, we will provide our considerations on AI/ML for mobility.

2	Discussion for the AI/ML based mobility

2.1 Type of mobility

L3 based mobility include basic handover, CHO mechanisms, etc. For basic handover related features, it’s detailed discussed and commercially used for both FR1 and FR2 bands. However, due to the multi-beamforming mechanism of FR2 band and FR2 band may be easily blocked by obstacles, it still has a lot of room for improvement in the mobility experience of the FR2 network. Comparing with the traditional basic handover mechanism, AI/ML is a potential useful tool to help network and UE select the best cells and beams for FR2 users quickly. Thus, it is foreseeable that discussing the use of AI/ML to optimize L3 Handover will bring better gains, especially for FR2 bands.

For LTM mechanism optimization, it’s observed that intra-CU LTM are specified during Rel-18 phase, and interrupt latency of less than 10ms can be achieved, which already makes the mobility experience of UEs in ToC scenarios excellent. Thus, the computing power and energy consumption brought by using AI/ML and the low latency performance that can be further improved in this scenario may need to be carefully studied and balanced. For inter-CU LTM, it will be discussed and in parallel with AI/ML for mobility SI during Rel-19 phase, it will be very complicated to discuss and optimize inter-CU LTM in two different work items. Thus, it’s suggested to discussed the potential benefits when using AI/ML for LTM scenario, but it can be deprioritized in Rel-19 until L3-based AI/ML mobility is fully studied.

L3 based HO supported with AI/ML can be studied in Rel-19, especially in basic handover for FR2 bands.

Intra-CU LTM supported with AI/ML can be deprioritized in Rel-19.

2.2 LCM framework and others

In the Rel-18 AI/ML for air interface SI, a general framework is defined for LCM and some detailed procedures are also defined, e.g., model transfer/delivery, data collection, functionalities mapping, etc. The framework can be reused in AI/ML for mobility SI due to it has taken beam related optimization into account.

The LCM framework for AI/ML for air Interface in Rel-18 should be taken as baseline for Rel-19 AI/ML for Air Interface (mobility).

3. Conclusions

In this paper, the below considerations for AI/ML based mobility are proposed:

- L3 based HO supported with AI/ML can be studied in Rel-19, especially in basic handover for FR2 bands.

- Intra-CU LTM supported with AI/ML can be deprioritized in Rel-19.

- The LCM framework for AI/ML for air Interface in Rel-18 should be taken as baseline for Rel-19 AI/ML for Air Interface (mobility).

References

RP-232622	Moderator's summary for REL-19 RAN2 topic AI/ML for Air Interface SI (Mobility)

RP-232745	Summary for RAN Rel-19 Package RAN123-led