3GPP TSG-RAN WG1 Meeting #114bis	R1-2310561

Xiamen, China, October 9th – 13th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.213 | CR | 0048 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on energy detection threshold formula for shared spectrum channel access for Rel-17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Samsung, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2023-10-12 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For operation with shared spectrum channel access, the energy detectiuon threshold (EDT) was previously based on ETSI Harmonized Standard for 5 GHz EN 301 893 v2.1.1. However, the recent draft of the EN 301 893 v 2.1.52 has a higher EDT for devices operating in the band. It's crucial that these modifications are incorporated into the 3GPP specs, ensuring 3GPP devices can effectively leverage the EDT as outlined in the harmonized standards. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct 37.213 Sections 4.1.5 and 4.2.3.1 so that the EDT is calculated band-wise for 5 GHz and 6 GHz. To accomplish this, we refer to the band definitions provided in Section 5.2, Table 5.2-1 of TS 38.104. - Change relevant parameters in the EDT formula to achieve the EDT described in the Harmonized standards. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Potential disadvantage to 3GPP devices operating in the band as they would operate with a lower EDT threshold. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.5 and 4.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Draft CR to 37.213 Section 4.1.5 and 4.2.3.1

*** Unchanged text omitted ***

### 4.1.5	Energy detection threshold adaptation procedures

An eNB/gNB accessing a channel on which transmission(s) are performed, shall set the energy detection threshold (XThresh) to be less than or equal to the maximum energy detection threshold XThresh_max.

XThresh_max is determined as follows:

-	If the absence of any other technology sharing the channel can be guaranteed on a long-term basis (e.g. by level of regulation) then:

-	XThresh_max=min&Tmax+10dB,&Xr

-	Xr is maximum energy detection threshold defined by regulatory requirements in dBm when such requirements are defined, otherwise Xr=Tmax+10dB;

-	otherwise,

-	XThresh_max=max&Xreg-72+10⋅log10(BWMHz /20MHz) dBm,&min&Tmax,&Tmax-TA+PH+10⋅log10(BWMHz /20MHz)-PTX

where:

- In regulatory regions and bands where it is allowed,

  - Xreg = -67+10⋅log10BWMHz /20MHz dBm;

  - TA=5dB for all transmissions;

  - PH=23dBm;

Otherwise,

  - Xreg=-72+10⋅log10BWMHz /20MHzdBm;

  - TA = 5dB for transmissions including discovery burst(s) as described in clause 4.1.2, and TA=10dB otherwise;

  - PH=23dBm or in regions and bands where regulations allow, 24dBm;

- TATA=10dB

- PH=23dBm

- PTX is set to the maximum eNB/gNB output power in dBm for the channel;

  - eNB/gNB uses the set maximum transmission power over a single channel irrespective of whether single channel or multi-channel transmission is employed

- Tmax(dBm)=10⋅log103.16228⋅10-8(mW/MHz) ⋅ BWMHz (MHz);

- BWMHz is the single channel bandwidth in MHz.

*** Unchanged text omitted ***

#### 4.2.3.1	Default maximum energy detection threshold computation procedure

If the higher layer parameter absenceOfAnyOtherTechnology-r14 or absenceOfAnyOtherTechnology-r16 is provided

-	X'Thresh_max=min&Tmax+10dB&Xr where

-	Xr is Maximum energy detection threshold defined by regulatory requirements in dBm when such requirements are defined, otherwise Xr=Tmax+10dB

Otherwise

- X'Thresh_max=max&Xreg-72+10⋅log10(BWMHz /20MHz) dBm,&min&Tmax,&Tmax-TA+PH+10⋅log10(BWMHz /20MHz)-PTX

where

  - In regulatory regions and bands where it is allowed,

    - Xreg= -67+10⋅log10BWMHz /20MHz dBm;

    - TA = 5dB;

    - PH=23dBm;

Otherwise,

    - Xreg=-72+10⋅log10BWMHz /20MHzdBm;

    - TA = 10dB;

    - PH=23dBm or in regions and bands where regulations allow, 24dBm;

  - PTX is set to the value of PCMAX_H,c as defined in [3];

  - Tmax(dBm)=10⋅log103.16228⋅10-8(mW/MHz) ⋅ BWMHz (MHz);

  - BWMHz is the single channel bandwidth in MHz.

*** Unchanged text omitted ***