3GPP TSG-RAN WG1 Meeting #115	R1-2312741

# Chicago, USA, November 13-17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0167 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on Rel-18 MIMO Evolution for Downlink and Uplink in 38.212 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core |  |  |  |  |  | Date: |  |  | 2023-11-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Capture the agreements and endorsed text proposals for Rel-18 MIMO evoluiation from RAN1#114bis and RAN1#115 meetings. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Multi-TRP enhancement - Clause 7.3.1.2.1: Add a new field 'PRACH association indicator' in DCI format 1_0 to reflect agreements from RAN1#114bis and RAN1#115 meetings. - Clause 7.3.1.2.2 & 7.3.1.2.3: Update the field of antenna port(s) in DCI format 1_1 and DCI format 1_2 to reflect the agreed TP from RAN1#115 meeting. Reference signal enhancement - Clause 7.3.1.1.2: Update Antenna ports tables to reflect the agreements on DMRS from RAN1#114bis meeting; - Clause 7.3.1.2.2: Update Antenna ports tables to reflect the agreements on DMRS from RAN1#115 meeting; Enhanced uplink transmission - Clause 5.4.2.1: Update this clause to limit the maximum number of layers for one TB for UL-SCH up to 4, per the endorsed text proposal from RAN1#114bis meeting. - Clause 7.3.1.1.2: Alignment of DCI fields for TB#2 for BWP switching; Update of precoding information and number of layers for CB based PUSCH for 8TX; Use of table for TPMI indication when fullpowermode2 is configured, configured rank is larger than 4 and an 4-port SRS resource is indicated; CSI only PUSCH is not indicated when indicate number of layer is larger than 4. CSI enhancement - Clause 6.3.2.1.2: Update Table 6.3.2.1.2-5E/5G to clarify the priority function considering the definition in 38.214; - Clause 6.3.2.1.2: Update Table 6.3.2.1.2-1B/2C/3C/5F to reflect agreements from RAN1#115 meeting; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification for Rel-18 MIMO Evolution for Downlink and Uplink is incomplete or incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.1, 6.3.2.1.2, 7.3.1.1.2, 7.3.1.1.3, 7.3.1.2.1, 7.3.1.2.2, 7.3.1.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.211, TS 38.213, TS 38.214 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged parts are omitted >

#### 5.4.2.1	Bit selection

< Unchanged parts are omitted >

For one TB for UL-SCH, or for one TB for DL-SCH/PCH except for DL-SCH with PDSCH scheduled by DCI format 4_0/4_1/4_2:

-	maximum number of layers for one TB for UL-SCH is given by the minimum of X and 4, where:

-	if the higher layer parameter maxMIMO-Layers of PUSCH-ServingCellConfig of the serving cell is configured, X is given by that parameter;

-	elseif the higher layer parameter maxRank of pusch-Config of the serving cell is configured, X is given by the maximum value of maxRank across all BWPs of the serving cell;

-	otherwise, X is given by the maximum number of layers for PUSCH supported by the UE for the serving cell;

-	maximum number of layers for one TB for DL-SCH/PCH is given by the minimum of X and 4, where:

-	if the higher layer parameter maxMIMO-Layers of PDSCH-ServingCellConfig of the serving cell is configured, X is given by that parameter;

-	otherwise, X is given by the maximum number of layers for PDSCH supported by the UE for the serving cell;

< Unchanged parts are omitted >

#### 6.3.2.1	UCI bit sequence generation

< Unchanged parts are omitted >

##### 6.3.2.1.2	CSI

If cqi-BitsPerSubband is configured, this Clause 6.3.2.1.2 applies by taking Subband CQI as Subband differential CQI and replacing the corresponding number of bits 2 by 4.

< Unchanged parts are omitted >

Table 6.3.2.1.2-1B: PMI of codebookType= typeII-CJT

|  | X1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | i1,1 |  | i1,2 |  | i1,8,1 | i1,8,2 |  | i1,8,3 |  | i1,8,4 |
| N3≤19 | N0log2(O1O2) |  | n=0N0-1log2N1N2Lσ(n) |  | log2KNZ |  |  |  |  |  |
| N3≤19 | N0log2(O1O2) |  | n=0N0-1log2N1N2Lσ(n) |  | log2(n=0N0-12Lσ(n)) | log2(n=0N0-12Lσ(n)) |  |  |  |  |
| N3≤19 | N0log2(O1O2) |  | n=0N0-1log2N1N2Lσ(n) |  | log2(n=0N0-12Lσ(n)) | log2(n=0N0-12Lσ(n)) |  | log2(n=0N0-12Lσ(n)) |  |  |
| N3≤19 | N0log2(O1O2) |  | n=0N0-1log2N1N2Lσ(n) |  | log2(n=0N0-12Lσ(n)) | log2(n=0N0-12Lσ(n)) |  | log2(n=0N0-12Lσ(n)) |  | log2(n=0N0-12Lσ(n)) |
| N3>19 | N0log2(O1O2) |  | n=0N0-1log2N1N2Lσ(n) |  | log2KNZ |  |  |  |  |  |
| N3>19 | N0log2(O1O2) |  | n=0N0-1log2N1N2Lσ(n) |  | log2(n=0N0-12Lσ(n)) | log2(n=0N0-12Lσ(n)) |  |  |  |  |
| N3>19 | N0log2(O1O2) |  | n=0N0-1log2N1N2Lσ(n) |  | log2(n=0N0-12Lσ(n)) | log2(n=0N0-12Lσ(n)) |  | log2(n=0N0-12Lσ(n)) |  |  |
| N3>19 | N0log2(O1O2) |  | n=0N0-1log2N1N2Lσ(n) |  | log2(n=0N0-12Lσ(n)) | log2(n=0N0-12Lσ(n)) |  | log2(n=0N0-12Lσ(n)) |  | log2(n=0N0-12Lσ(n)) |
|  | X2 |  |  |  |  |  |  |  |  |  |
|  | i2,3,1 | i2,3,2 |  | i2,3,3 | i2,3,4 |  | i1,5 |  | i1,6,1 | i1,6,2 |
| N3≤19 |  |  |  |  |  |  |  |  | log2N3-1M1-1 |  |
| N3≤19 |  |  |  |  |  |  |  |  | log2N3-1M2-1 | log2N3-1M2-1 |
| N3≤19 |  |  |  |  |  |  |  |  | log2N3-1M3-1 | log2N3-1M3-1 |
| N3≤19 |  |  |  |  |  |  |  |  | log2N3-1M4-1 | log2N3-1M4-1 |
| N3>19 |  |  |  |  |  |  | log2(2M1) |  | log22M1-1M1-1 |  |
| N3>19 |  |  |  |  |  |  | log2(2M2) |  | log22M2-1M2-1 | log22M2-1M2-1 |
| N3>19 |  |  |  |  |  |  | log2(2M3) |  | log22M3-1M3-1 | log22M3-1M3-1 |
| N3>19 |  |  |  |  |  |  | log2(2M4) |  | log22M4-1M4-1 | log22M4-1M4-1 |
|  | X2 |  |  |  |  |  |  |  |  |  |
|  | i1,6,3 |  |  | i1,6,4 | i1,9 |  | {i2,4,l}l=1,…,υ |  | {i2,5,l}l=1,…,υ | {i1,7,l}l=1,…,υ |
| N3≤19 |  |  |  |  | (N0-1)log2O3N3 |  | 3(KNZ-1) |  | 4(KNZ-1) | 2∙n=0N0-1Lσ(n)M1 |
| N3≤19 |  |  |  |  | (N0-1)log2O3N3 |  | 3(KNZ-2) |  | 4(KNZ-2) | 4∙n=0N0-1Lσ(n)M2 |
| N3≤19 | log2N3-1M3-1 |  |  |  | (N0-1)log2O3N3 |  | 3(KNZ-3) |  | 4(KNZ-3) | 6∙n=0N0-1Lσ(n)M3 |
| N3≤19 | log2N3-1M4-1 |  |  | log2N3-1M4-1 | (N0-1)log2O3N3 |  | 3(KNZ-4) |  | 4(KNZ-4) | 8∙n=0N0-1Lσ(n)M4 |
| N3>19 |  |  |  |  | N0-1log2O3N3 |  | 3(KNZ-1) |  | 4(KNZ-1) | 2∙n=0N0-1Lσ(n)M1 |
| N3>19 |  |  |  |  | (N0-1)log2O3N3 |  | 3(KNZ-2) |  | 4(KNZ-2) | 4∙n=0N0-1Lσ(n)M2 |
| N3>19 | log22M3-1M3-1 |  |  |  | (N0-1)log2O3N3 |  | 3(KNZ-3) |  | 4(KNZ-3) | 6∙n=0N0-1Lσ(n)M3 |
| N3>19 | log22M4-1M4-1 |  |  | log22M4-1M4-1 | (N0-1)log2O3N3 |  | 3(KNZ-4) |  | 4(KNZ-4) | 8∙n=0N0-1Lσ(n)M4 |
| {i1,7,l}l=1,…,υ{i2,4,l}l=1,…,υ{i2,5,l}l=1,…,υ{i1,7,l}{i2,4,l}{i2,5,l}υ2n=0N0-1Lσ(n)Mυ3KlNZ-1KlNZ-1kl,i,f,j(3)kl,i,f,j(2)cl,i,f,jKlNZlKNZ=l=1υKlNZ |  |  |  |  |  |  |  |  |  |  |
|  | Information fields X1 |  |  |  |  |  |  |  |  |  |
|  | i1,1 |  | i1,2 |  | i1,8,1 | i1,8,2 |  | i1,8,3 |  | i1,8,4 |
| Rank=1 N3≤19 | N0log2(O1O2) |  | n=1N0log2N1N2Lσ(n) |  | log2KNZ | N/A |  | N/A |  | N/A |
| Rank=2 N3≤19 | N0log2(O1O2) |  | n=1N0log2N1N2Lσ(n) |  | log2(n=1N02Lσ(n)) | log2(n=1N02Lσ(n)) |  | N/A |  | N/A |
| Rank=3 N3≤19 | N0log2(O1O2) |  | n=1N0log2N1N2Lσ(n) |  | log2(n=1N02Lσ(n)) | log2(n=1N02Lσ(n)) |  | log2(n=1N02Lσ(n)) |  | N/A |
| Rank=4 N3≤19 | N0log2(O1O2) |  | n=1N0log2N1N2Lσ(n) |  | log2(n=1N02Lσ(n)) | log2(n=1N02Lσ(n)) |  | log2(n=1N02Lσ(n)) |  | log2(n=1N02Lσ(n)) |
| Rank=1 N3>19 | N0log2(O1O2) |  | n=1N0log2N1N2Lσ(n) |  | log2KNZ | N/A |  | N/A |  | N/A |
| Rank=2 N3>19 | N0log2(O1O2) |  | n=1N0log2N1N2Lσ(n) |  | log2(n=1N02Lσ(n)) | log2(n=1N02Lσ(n)) |  | N/A |  | N/A |
| Rank=3 N3>19 | N0log2(O1O2) |  | n=1N0log2N1N2Lσ(n) |  | log2(n=1N02Lσ(n)) | log2(n=1N02Lσ(n)) |  | log2(n=1N02Lσ(n)) |  | N/A |
| Rank=4 N3>19 | N0log2(O1O2) |  | n=1N0log2N1N2Lσ(n) |  | log2(n=1N02Lσ(n)) | log2(n=1N02Lσ(n)) |  | log2(n=1N02Lσ(n)) |  | log2(n=1N02Lσ(n)) |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |
|  | i2,3,1 | i2,3,2 |  | i2,3,3 | i2,3,4 |  | i1,5 |  | i1,6,1 | i1,6,2 |
| Rank=1 N3≤19 | 4 | N/A |  | N/A | N/A |  | N/A |  | log2N3-1M1-1 | N/A |
| Rank=2 N3≤19 | 4 | 4 |  | N/A | N/A |  | N/A |  | log2N3-1M2-1 | log2N3-1M2-1 |
| Rank=3 N3≤19 | 4 | 4 |  | 4 | N/A |  | N/A |  | log2N3-1M3-1 | log2N3-1M3-1 |
| Rank=4 N3≤19 | 4 | 4 |  | 4 | 4 |  | N/A |  | log2N3-1M4-1 | log2N3-1M4-1 |
| Rank=1 N3>19 | 4 | N/A |  | N/A | N/A |  | log2(2M1) |  | log22M1-1M1-1 | N/A |
| Rank=2 N3>19 | 4 | 4 |  | N/A | N/A |  | log2(2M2) |  | log22M2-1M2-1 | log22M2-1M2-1 |
| Rank=3 N3>19 | 4 | 4 |  | 4 | N/A |  | log2(2M3) |  | log22M3-1M3-1 | log22M3-1M3-1 |
| Rank=4 N3>19 | 4 | 4 |  | 4 | 4 |  | log2(2M4) |  | log22M4-1M4-1 | log22M4-1M4-1 |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |
|  | i1,6,3 |  |  | i1,6,4 | i1,9 |  | {i2,4,l}l=1,…,υ |  | {i2,5,l}l=1,…,υ | {i1,7,l}l=1,…,υ |
| Rank=1 N3≤19 | N/A |  |  | N/A | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | 3(KNZ-1) |  | 4(KNZ-1) | 2∙n=1N0Lσ(n)M1 |
| Rank=2 N3≤19 | N/A |  |  | N/A | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | 3(KNZ-2) |  | 4(KNZ-2) | 4∙n=1N0Lσ(n)M2 |
| Rank=3 N3≤19 | log2N3-1M3-1 |  |  | N/A | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | 3(KNZ-3) |  | 4(KNZ-3) | 6∙n=1N0Lσ(n)M3 |
| Rank=4 N3≤19 | log2N3-1M4-1 |  |  | log2N3-1M4-1 | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | 3(KNZ-4) |  | 4(KNZ-4) | 8∙n=1N0Lσ(n)M4 |
| Rank=1 N3>19 | N/A |  |  | N/A | N0-1log2O3N3 if Mode1 is configured, NA otherwise |  | 3(KNZ-1) |  | 4(KNZ-1) | 2∙n=1N0Lσ(n)M1 |
| Rank=2 N3>19 | N/A |  |  | N/A | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | 3(KNZ-2) |  | 4(KNZ-2) | 4∙n=1N0Lσ(n)M2 |
| Rank=3 N3>19 | log22M3-1M3-1 |  |  | N/A | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | 3(KNZ-3) |  | 4(KNZ-3) | 6∙n=1N0Lσ(n)M3 |
| Rank=4 N3>19 | log22M4-1M4-1 |  |  | log22M4-1M4-1 | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | 3(KNZ-4) |  | 4(KNZ-4) | 8∙n=1N0Lσ(n)M4 |
| NOTE: the bitwidth for {i1,7,l}l=1,…,υ, {i2,4,l}l=1,…,υ and {i2,5,l}l=1,…,υ shown in Table 6.3.2.1.2-1B is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = υ, respectively, and the corresponding per layer bitwidths are 2n=1N0Lσ(n)Mυ, 3KlNZ-1, and 4KlNZ-1, (i.e., 1, 3, and 4 bits for each respective indicator elements kl,i,f,j(3), kl,i,f,j(2), and cl,i,f,j, respectively), where KlNZ as defined in Clause 5.2.2.2.8 in [6, TS 38.214] is the number of nonzero coefficients for layer l such that KNZ=l=1υKlNZ. |  |  |  |  |  |  |  |  |  |  |

The bitwidth for PMI of codebookType=typeII-Doppler is provided in Tables 6.3.2.1.2-1C, where the values of (N1, N2), (O1, O2), L, KNZ, N3, N4, Q and Mll=1,…, υ are given by Clause 5.2.2.2.10 in [6, TS 38.214].

Table 6.3.2.1.2-1C: PMI of codebookType=typeII-Doppler

|  |  | Information fields X1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | i1,1 |  |  |  | i1,2 |  |  |  | i1,8,1 |  |  | i1,8,2 |  |  | i1,8,3 |  |  | i1,8,4 |  |
| Rank=1 N3≤19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2KNZ |  |  | N/A |  |  | N/A |  |  | N/A |  |
| Rank=2 N3≤19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | N/A |  |  | N/A |  |
| Rank=3 N3≤19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | N/A |  |
| Rank=4 N3≤19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | log2(2LQ) |  |
| Rank=1 N3>19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2KNZ |  |  | N/A |  |  | N/A |  |  | N/A |  |
| Rank=2 N3>19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | N/A |  |  | N/A |  |
| Rank=3 N3>19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | N/A |  |
| Rank=4 N3>19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | log2(2LQ) |  |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | i2,3,1 | i2,3,2 | i2,3,3 |  | i2,3,4 |  | i1,5 |  | i1,6,1 |  |  | i1,6,2 |  |  | i1,6,3 |  |  | i1,6,4 |  |  |
| Rank=1 N3≤19 | 4 | N/A | N/A |  | N/A |  | N/A |  | log2N3-1M1-1 |  |  | N/A |  |  | N/A |  |  | N/A |  |  |
| Rank=2 N3≤19 | 4 | 4 | N/A |  | N/A |  | N/A |  | log2N3-1M2-1 |  |  | log2N3-1M2-1 |  |  | N/A |  |  | N/A |  |  |
| Rank=3 N3≤19 | 4 | 4 | 4 |  | N/A |  | N/A |  | log2N3-1M3-1 |  |  | log2N3-1M3-1 |  |  | log2N3-1M3-1 |  |  | N/A |  |  |
| Rank=4 N3≤19 | 4 | 4 | 4 |  | 4 |  | N/A |  | log2N3-1M4-1 |  |  | log2N3-1M4-1 |  |  | log2N3-1M4-1 |  |  | log2N3-1M4-1 |  |  |
| Rank=1 N3>19 | 4 | N/A | N/A |  | N/A |  | log2(2M1) |  | log22M1-1M1-1 |  |  | N/A |  |  | N/A |  |  | N/A |  |  |
| Rank=2 N3>19 | 4 | 4 | N/A |  | N/A |  | log2(2M2) |  | log22M2-1M2-1 |  |  | log22M2-1M2-1 |  |  | N/A |  |  | N/A |  |  |
| Rank=3 N3>19 | 4 | 4 | 4 |  | N/A |  | log2(2M3) |  | log22M3-1M3-1 |  |  | log22M3-1M3-1 |  |  | log22M3-1M3-1 |  |  | N/A |  |  |
| Rank=4 N3>19 | 4 | 4 | 4 |  | 4 |  | log2(2M4) |  | log22M4-1M4-1 |  |  | log22M4-1M4-1 |  |  | log22M4-1M4-1 |  |  | log22M4-1M4-1 |  |  |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | i1,10,1 |  |  | i1,10,2 |  |  |  | i1,10,3 |  |  | i1,10,4 |  |  | {i2,4,l}l=1,…,υ |  |  | {i2,5,l}l=1,…,υ |  |  | {i1,7,l}l=1,…,υ |
| Rank=1 N3≤19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | NA |  |  |  | NA |  |  | NA |  |  | 3(KNZ-1) |  |  | 4(KNZ-1) |  |  | 2LM1Q |
| Rank=2 N3≤19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  |  | NA |  |  | NA |  |  | 3(KNZ-2) |  |  | 4(KNZ-2) |  |  | 4LM2Q |
| Rank=3 N3≤19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | NA |  |  | 3(KNZ-3) |  |  | 4(KNZ-3) |  |  | 6LM3Q |
| Rank=4 N3≤19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | 3(KNZ-4) |  |  | 4(KNZ-4) |  |  | 8LM4Q |
| Rank=1 N3>19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | NA |  |  |  | NA |  |  | NA |  |  | 3(KNZ-1) |  |  | 4(KNZ-1) |  |  | 2LM1Q |
| Rank=2 N3>19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  |  | NA |  |  | NA |  |  | 3(KNZ-2) |  |  | 4(KNZ-2) |  |  | 4LM2Q |
| Rank=3 N3>19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | NA |  |  | 3(KNZ-3) |  |  | 4(KNZ-3) |  |  | 6LM3Q |
| Rank=4 N3>19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | 3(KNZ-4) |  |  | 4(KNZ-4) |  |  | 8LM4Q |
| NOTE: the bitwidth for {i1,7,l}l=1,…,υ, {i2,4,l}l=1,…,υ and {i2,5,l}l=1,…,υ shown in Table 6.3.2.1.2-1C is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = υ, respectively, and the corresponding per layer bitwidths are 2LMυQ, 3KlNZ-1, and 4KlNZ-1, (i.e., 1, 3, and 4 bits for each respective indicator elements kl,i,f,τ(3), kl,i,f,τ(2), and cl,i,f,τ, respectively), where KlNZ as defined in Clause 5.2.2.2.10 in [6, TS 38.214] is the number of nonzero coefficients for layer l such that KNZ=l=1υKlNZ. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The bitwidth for PMI of codebookType= typeII-PortSelection is provided in Tables 6.3.2.1.2-2, where the values of , , , , , , and  are given by Clause 5.2.2.2.4 in [6, TS 38.214].

Table 6.3.2.1.2-2: PMI of codebookType= typeII-PortSelection

|  | Information fields for wideband PMI |  |  |  |  | Information fields for wideband PMI or per subband PMI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rank=1 SBAmp off |  |  |  | N/A | N/A |  | N/A | N/A | N/A |
| Rank=2 SBAmp off |  |  |  |  |  |  |  | N/A | N/A |
| Rank=1 SBAmp on |  |  |  | N/A | N/A |  | N/A |  | N/A |
| Rank=2 SBAmp on |  |  |  |  |  |  |  |  |  |

The bitwidth for PMI of codebookType=typeII-PortSelection-r16 is provided in Tables 6.3.2.1.2-2A, where the values of PCSI-RS, d, L, KNZ, N3, and  Mll=1,…, υ are given by Clause 5.2.2.2.6 in [6, TS 38.214].

Table 6.3.2.1.2-2A: PMI of codebookType= typeII-PortSelection-r16

|  |  |  |  | Information fields X1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | i1,1 |  |  |  | i1,8,1 |  | i1,8,2 |  |  | i1,8,3 |  | i1,8,4 |  |  |
| Rank=1 N3≤19 |  |  |  | log2PCSI-RS2d |  |  |  | log2KNZ |  | N/A |  |  | N/A |  | N/A |  |  |
| Rank=2 N3≤19 |  |  |  | log2PCSI-RS2d |  |  |  | log2(2L) |  | log2(2L) |  |  | N/A |  | N/A |  |  |
| Rank=3 N3≤19 |  |  |  | log2PCSI-RS2d |  |  |  | log2(2L) |  | log2(2L) |  |  | log2(2L) |  | N/A |  |  |
| Rank=4 N3≤19 |  |  |  | log2PCSI-RS2d |  |  |  | log2(2L) |  | log2(2L) |  |  | log2(2L) |  | log2(2L) |  |  |
| Rank=1 N3>19 |  |  |  | log2PCSI-RS2d |  |  |  | log2KNZ |  | N/A |  |  | N/A |  | N/A |  |  |
| Rank=2 N3>19 |  |  |  | log2PCSI-RS2d |  |  |  | log2(2L) |  | log2(2L) |  |  | N/A |  | N/A |  |  |
| Rank=3 N3>19 |  |  |  | log2PCSI-RS2d |  |  |  | log2(2L) |  | log2(2L) |  |  | log2(2L) |  | N/A |  |  |
| Rank=4 N3>19 |  |  |  | log2PCSI-RS2d |  |  |  | log2(2L) |  | log2(2L) |  |  | log2(2L) |  | log2(2L) |  |  |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | i2,3,1 | i2,3,2 | i2,3,3 |  | i2,3,4 | i1,5 | i1,6,1 |  | i1,6,2 |  | i1,6,3 | i1,6,4 |  | {i2,4,l}l=1,…,υ |  | {i2,5,l}l=1,…,υ | {i1,7,l}l=1,…,υ |
| Rank=1 N3≤19 | 4 | N/A | N/A |  | N/A | N/A | log2N3-1M1-1 |  | N/A |  | N/A | N/A |  | 3(KNZ-1) |  | 4(KNZ-1) | 2LM1 |
| Rank=2 N3≤19 | 4 | 4 | N/A |  | N/A | N/A | log2N3-1M2-1 |  | log2N3-1M2-1 |  | N/A | N/A |  | 3(KNZ-2) |  | 4(KNZ-2) | 4LM2 |
| Rank=3 N3≤19 | 4 | 4 | 4 |  | N/A | N/A | log2N3-1M3-1 |  | log2N3-1M3-1 |  | log2N3-1M3-1 | N/A |  | 3(KNZ-3) |  | 4(KNZ-3) | 6LM3 |
| Rank=4 N3≤19 | 4 | 4 | 4 |  | 4 | N/A | log2N3-1M4-1 |  | log2N3-1M4-1 |  | log2N3-1M4-1 | log2N3-1M4-1 |  | 3(KNZ-4) |  | 4(KNZ-4) | 8LM4 |
| Rank=1 N3>19 | 4 | N/A | N/A |  | N/A | log2(2M1) | log22M1-1M1-1 |  | N/A |  | N/A | N/A |  | 3(KNZ-1) |  | 4(KNZ-1) | 2LM1 |
| Rank=2 N3>19 | 4 | 4 | N/A |  | N/A | log2(2M2) | log22M2-1M2-1 |  | log22M2-1M2-1 |  | N/A | N/A |  | 3(KNZ-2) |  | 4(KNZ-2) | 4LM2 |
| Rank=3 N3>19 | 4 | 4 | 4 |  | N/A | log2(2M3) | log22M3-1M3-1 |  | log22M3-1M3-1 |  | log22M3-1M3-1 | N/A |  | 3(KNZ-3) |  | 4(KNZ-3) | 6LM3 |
| Rank=4 N3>19 | 4 | 4 | 4 |  | 4 | log2(2M4) | log22M4-1M4-1 |  | log22M4-1M4-1 |  | log22M4-1M4-1 | log22M4-1M4-1 |  | 3(KNZ-4) |  | 4(KNZ-4) | 8LM4 |
| NOTE: the bitwidth for {i1,7,l}l=1,…,υ, {i2,4,l}l=1,…,υ and {i2,5,l}l=1,…,υ shown in Table 6.3.2.1.2-2A is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = υ, respectively, and the corresponding per layer bitwidths are 2LMυ, 3KlNZ-1, and 4KlNZ-1, (i.e., 1, 3, and 4 bits for each respective indicator elements kl,i,f(3), kl,i,f(2), and cl,i,f, respectively), where KlNZ as defined in Clause 5.2.2.2.5 in TS 38.214 [6] is the number of nonzero coefficients for layer l such that KNZ=l=1υKlNZ. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The bitwidth for PMI of codebookType=typeII-PortSelection-r17 is provided in Tables 6.3.2.1.2-2B, where the values of PCSI-RS, K1, KNZ, N3, N and M are given by Clause 5.2.2.2.7 in [6, TS 38.214].

Table 6.3.2.1.2-2B: PMI of codebookType= typeII-PortSelection-r17

|  | Information fields X1 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | i1,2 |  | i1,6 |  | i1,8,1 |  | i1,8,2 |  |  | i1,8,3 |  | i1,8,4 |
| Rank=1 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | N/A |  |  | N/A |  | N/A |
| Rank=2 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | log2(K1M) |  |  | N/A |  | N/A |
| Rank=3 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | log2(K1M) |  |  | log2(K1M) |  | N/A |
| Rank=4 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | log2(K1M) |  |  | log2(K1M) |  | log2(K1M) |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |
|  | i2,3,1 | i2,3,2 |  | i2,3,3 |  | i2,3,4 |  | {i2,4,l}l=1,…,υ | {i2,5,l}l=1,…,υ |  | {i1,7,l}l=1,…,υ |  |
| Rank=1 | 4 | N/A |  | N/A |  | N/A |  | 3(KNZ-1) | 4(KNZ-1) |  | N/A if KNZ=K1M; K1M otherwise |  |
| Rank=2 | 4 | 4 |  | N/A |  | N/A |  | 3(KNZ-2) | 4(KNZ-2) |  | N/A if KNZ=2K1M; 2K1M otherwise |  |
| Rank=3 | 4 | 4 |  | 4 |  | N/A |  | 3(KNZ-3) | 4(KNZ-3) |  | 3K1M |  |
| Rank=4 | 4 | 4 |  | 4 |  | 4 |  | 3(KNZ-4) | 4(KNZ-4) |  | 4K1M |  |
| NOTE: the bitwidth for {i1,7,l}l=1,…,υ, {i2,4,l}l=1,…,υ and {i2,5,l}l=1,…,υ shown in Table 6.3.2.1.2-2B is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = υ, respectively, and the corresponding per layer bitwidths are K1M, 3KlNZ-1, and 4KlNZ-1, (i.e., 1, 3, and 4 bits for each respective indicator elements kl,i,f(3), kl,i,f(2), and cl,i,f, respectively), where KlNZ as defined in Clause 5.2.2.2.7 in TS 38.214 [6] is the number of nonzero coefficients for layer l such that KNZ=l=1υKlNZ. |  |  |  |  |  |  |  |  |  |  |  |  |

If csi-ReportSubConfig is configured, for a corresponding CSI sub-report, the mapping order of CSI fields of one CSI sub-report is determined following the procedure in this clause 6.3.2.1.2, by replacing CSI report #n in the following Tables X, Y, Z with CSI sub-report #n.

The bitwidth for PMI of codebookType=typeII-CJT-PortSelection is provided in Tables 6.3.2.1.2-2C, where the values of PCSI-RS, K1,n, KNZ, N3, N, M and σn are given by Clause 5.2.2.2.9 in [6, TS 38.214].

Table 6.3.2.1.2-2C: PMI of codebookType= typeII-CJT-PortSelection

|  | X1 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | i1,2 |  |  | i1,6 |  | i1,8,1 |  |  | i1,8,2 |  | i1,8,3 |  | i1,8,4 |
|  | n=0N0-1log2PCSI-RS/2K1,σ(n)/2 |  |  | log2(N-1) |  | log2n=0N0-1K1,σ(n)M |  |  |  |  |  |  |  |
|  | n=0N0-1log2PCSI-RS/2K1,σ(n)/2 |  |  | log2(N-1) |  | log2n=0N0-1K1,σ(n)M |  |  | log2n=0N0-1K1,σ(n)M |  |  |  |  |
|  | n=0N0-1log2PCSI-RS/2K1,σ(n)/2 |  |  | log2(N-1) |  | log2n=0N0-1K1,σ(n)M |  |  | log2n=0N0-1K1,σ(n)M |  | log2n=0N0-1K1,σ(n)M |  |  |
|  | n=0N0-1log2PCSI-RS/2K1,σ(n)/2 |  |  | log2(N-1) |  | log2n=0N0-1K1,σ(n)M |  |  | log2n=0N0-1K1,σ(n)M |  | log2n=0N0-1K1,σ(n)M |  | log2n=0N0-1K1,σ(n)M |
|  | X2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | i2,3,1 | i2,3,2 | i2,3,3 |  | i2,3,4 |  | {i2,4,l}l=1,…,υ | {i2,5,l}l=1,…,υ |  | i1,9 |  | {i1,7,l}l=1,…,υ |  |
|  |  |  |  |  |  |  | 3(KNZ-1) | 4(KNZ-1) |  | (N0-1)log2O3N3 |  | KNZ=n=0N0-1K1,σ(n)M n=0N0-1K1,σ(n)M |  |
|  |  |  |  |  |  |  | 3(KNZ-2) | 4(KNZ-2) |  | (N0-1)log2O3N3 |  | KNZ=2n=0N0-1K1,σ(n)M 2n=0N0-1K1,σ(n)M |  |
|  |  |  |  |  |  |  | 3(KNZ-3) | 4(KNZ-3) |  | (N0-1)log2O3N3 |  | 3∙n=0N0-1K1,σ(n)M |  |
|  |  |  |  |  |  |  | 3(KNZ-4) | 4(KNZ-4) |  | (N0-1)log2O3N3 |  | 4∙n=0N0-1K1,σ(n)M |  |
| {i1,7,l}l=1,…,υ{i2,4,l}l=1,…,υ{i2,5,l}l=1,…,υ{i1,7,l}{i2,4,l}{i2,5,l}υn=0N0-1K1,σ(n)M3KlNZ-1KlNZ-1kl,i,f,j(3)kl,i,f,j(2)cl,i,f,jKlNZlKNZ=l=1υKlNZ |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Information fields X1 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | i1,2 |  |  | i1,6 |  | i1,8,1 |  |  | i1,8,2 |  | i1,8,3 |  | i1,8,4 |
| Rank=1 | n=1N0log2PCSI-RS/2K1,σ(n)/2 |  |  | log2(N-1) if N > M=2, N/A otherwise |  | log2n=1N0K1,σ(n)M |  |  | N/A |  | N/A |  | N/A |
| Rank=2 | n=1N0log2PCSI-RS/2K1,σ(n)/2 |  |  | log2(N-1) if N > M=2, N/A otherwise |  | log2n=1N0K1,σ(n)M |  |  | log2n=1N0K1,σ(n)M |  | N/A |  | N/A |
| Rank=3 | n=1N0log2PCSI-RS/2K1,σ(n)/2 |  |  | log2(N-1) if N > M=2, N/A otherwise |  | log2n=1N0K1,σ(n)M |  |  | log2n=1N0K1,σ(n)M |  | log2n=1N0K1,σ(n)M |  | N/A |
| Rank=4 | n=1N0log2PCSI-RS/2K1,σ(n)/2 |  |  | log2(N-1) if N > M=2, N/A otherwise |  | log2n=1N0K1,σ(n)M |  |  | log2n=1N0K1,σ(n)M |  | log2n=1N0K1,σ(n)M |  | log2n=1N0K1,σ(n)M |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | i2,3,1 | i2,3,2 | i2,3,3 |  | i2,3,4 |  | {i2,4,l}l=1,…,υ | {i2,5,l}l=1,…,υ |  | i1,9 |  | {i1,7,l}l=1,…,υ |  |
| Rank=1 | 4 | N/A | N/A |  | N/A |  | 3(KNZ-1) | 4(KNZ-1) |  | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | N/A if KNZ=n=1N0K1,σ(n)M; n=1N0K1,σ(n)M otherwise |  |
| Rank=2 | 4 | 4 | N/A |  | N/A |  | 3(KNZ-2) | 4(KNZ-2) |  | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | N/A if KNZ=2n=1N0K1,σ(n)M; 2n=1N0K1,σ(n)M otherwise |  |
| Rank=3 | 4 | 4 | 4 |  | N/A |  | 3(KNZ-3) | 4(KNZ-3) |  | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | 3∙n=1N0K1,σ(n)M |  |
| Rank=4 | 4 | 4 | 4 |  | 4 |  | 3(KNZ-4) | 4(KNZ-4) |  | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | 4∙n=1N0K1,σ(n)M |  |
| NOTE: the bitwidth for {i1,7,l}l=1,…,υ, {i2,4,l}l=1,…,υ and {i2,5,l}l=1,…,υ shown in Table 6.3.2.1.2-2C is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = υ, respectively, and the corresponding per layer bitwidths are n=1N0K1,σ(n)M, 3KlNZ-1, and 4KlNZ-1, (i.e., 1, 3, and 4 bits for each respective indicator elements kl,i,f,j(3), kl,i,f,j(2), and cl,i,f,j, respectively), where KlNZ as defined in Clause 5.2.2.2.9 in [6, TS 38.214] is the number of nonzero coefficients for layer l such that KNZ=l=1υKlNZ. |  |  |  |  |  |  |  |  |  |  |  |  |  |

The bitwidth for PMI of codebookType=typeII-Doppler-PortSelection is provided in Tables 6.3.2.1.2-2D, where the values of PCSI-RS, K1, KNZ, N3, N and M are given by Clause 5.2.2.2.11 in [6, TS 38.214].

Table 6.3.2.1.2-2D: PMI of codebookType= typeII-Doppler-PortSelection

|  | Information fields X1 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | i1,2 |  | i1,6 |  | i1,8,1 |  | i1,8,2 |  |  | i1,8,3 |  | i1,8,4 |
| Rank=1 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | N/A |  |  | N/A |  | N/A |
| Rank=2 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | log2(K1M) |  |  | N/A |  | N/A |
| Rank=3 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | log2(K1M) |  |  | log2(K1M) |  | N/A |
| Rank=4 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | log2(K1M) |  |  | log2(K1M) |  | log2(K1M) |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |
|  | i2,3,1 | i2,3,2 |  | i2,3,3 |  | i2,3,4 |  | {i2,4,l}l=1,…,υ | {i2,5,l}l=1,…,υ |  | {i1,7,l}l=1,…,υ |  |
| Rank=1 | 4 | N/A |  | N/A |  | N/A |  | 3(KNZ-1) | 4(KNZ-1) |  | N/A if KNZ=K1M; K1M otherwise |  |
| Rank=2 | 4 | 4 |  | N/A |  | N/A |  | 3(KNZ-2) | 4(KNZ-2) |  | N/A if KNZ=2K1M; 2K1M otherwise |  |
| Rank=3 | 4 | 4 |  | 4 |  | N/A |  | 3(KNZ-3) | 4(KNZ-3) |  | 3K1M |  |
| Rank=4 | 4 | 4 |  | 4 |  | 4 |  | 3(KNZ-4) | 4(KNZ-4) |  | 4K1M |  |
| NOTE: the bitwidth for {i1,7,l}l=1,…,υ, {i2,4,l}l=1,…,υ and {i2,5,l}l=1,…,υ shown in Table 6.3.2.1.2-2D is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = υ, respectively, and the corresponding per layer bitwidths are K1M, 3KlNZ-1, and 4KlNZ-1, (i.e., 1, 3, and 4 bits for each respective indicator elements kl,i,f(3), kl,i,f(2), and cl,i,f, respectively), where KlNZ as defined in Clause 5.2.2.2.11 in [6, TS 38.214] is the number of nonzero coefficients for layer l such that KNZ=l=1υKlNZ. |  |  |  |  |  |  |  |  |  |  |  |  |

For CSI on PUSCH, two UCI bit sequences are generated,  and . The CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.2.1.2-6, are mapped to the UCI bit sequence  starting with . The CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.2.1.2-7, are mapped to the UCI bit sequence  starting with .

The mapping order of CSI fields of one report for CRI/RSRP or SSBRI/RSRP or CRI/RSRP/CapabilityIndex or SSBRI/RSRP/CapabilityIndex reporting is provided in Table 6.3.1.1.2-8. The mapping order of CSI fields of one report for inter-cell SSBRI/RSRP reporting is provided in Table 6.3.1.1.2-8. The mapping order of CSI fields of one report for CRI/SINR or SSBRI/SINR or CRI/SINR/CapabilityIndex or SSBRI/SINR/CapabilityIndex reporting is provided in Table 6.3.1.1.2-8A. The mapping order of CSI fields of one report for group-based CRI/RSRP or SSBRI/RSRP reporting is provided in Table 6.3.1.1.2-8B. The mapping order of CSI fields of one report for TDCP reporting is provided in Table 6.3.2.1.2-3C. The mapping order of CSI fields of one report for SSBRI/RSRP reporting for L1/L2-triggered mobility is provided in Table 6.3.1.1.2-8C. The procedure in clause 6.3.2 described for CSI part 1 is also applicable for one report for CRI/RSRP, SSBRI/RSRP, CRI/SINR, SSBRI/SINR reporting, or TDCP reporting.

Table 6.3.2.1.2-3: Mapping order of CSI fields of one CSI report, CSI part 1

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 1 | CRI as in Tables 6.3.1.1.2-3/4/6, if reported |
|  | Rank Indicator as in Tables 6.3.1.1.2-3/4/5 or 6.3.2.1.2-8/8A/8B/9/9A, if reported |
|  | Wideband CQI for the first TB as in Tables 6.3.1.1.2-3/4/5 or 6.3.2.1.2-8/8A/8B/9/9A, if reported |
|  | Subband differential CQI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3/4/5 or 6.3.2.1.2-8/8A/8B/9/9A, if reported |
|  | Indicator of the number of non-zero wideband amplitude coefficients M0 for layer 0 as in Table 6.3.1.1.2-5, if reported |
|  | Indicator of the number of non-zero wideband amplitude coefficients M1 for layer 1 as in Table 6.3.1.1.2-5 (if the rank according to the reported RI is equal to one, this field is set to all zeros), if 2-layer PMI reporting is allowed according to the rank restriction in Clauses 5.2.2.2.3 and 5.2.2.2.4 [6, TS 38.214] and if reported |
|  | Indicator of the N0 selected CSI-RS resources by a bitmap with NTRP bits, this field is present only if NTRP>1 and restrictedCMR-Selection is configured to OFF |
|  | Indicator of selected Ln value combination or αn value combination with bitwidth of log2(NL), this field is present only if NL>1 |
|  | Indicator of the total number of non-zero coefficients summed across all layers KNZ as in Tables 6.3.2.1.2-8/8A/8B/9/9A, if reported |
| NOTE: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand as subband 0. |  |

Table 6.3.2.1.2-3A: Mapping order of CSI fields of one CSI report, CSI part 1, csi-ReportMode= Mode 1

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 1 | CRI as in Table 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported |
|  | Rank Combination Indicator as in Table 6.3.1.1.2-3A, if reported |
|  | Wideband CQI for the first TB as in Table 6.3.1.1.2-3A, if reported |
|  | Subband differential CQI for the first TB with increasing order of subband number as in Table 6.3.1.1.2-3A, if reported |
|  | CRI as in Tables 6.3.1.1.2-3B, if associated with one CSI-RS resource, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; First CRI as in Table 6.3.1.1.2-3B, if associated with one CSI-RS resource, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Rank Indicator associated with CRI as in Table 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Rank Indicator associated with the first CRI as in Table 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Wideband CQI associated with CRI for the first TB as in Table 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Wideband CQI associated with the first CRI for the first TB as in Table 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Subband differential CQI associated with CRI for the first TB with increasing order of subband number as in Table 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 1 if reported; Subband differential CQI associated with the first CRI for the first TB with increasing order of subband number as in Table 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Second CRI as in Table 6.3.1.1.2-3B, if associated with one CSI-RS resource, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Rank Indicator associated with the second CRI as in Table 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Wideband CQI associated with the second CRI for the first TB as in Table 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Subband differential CQI associated with the second CRI for the first TB with increasing order of subband number as in Table 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
| NOTE: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand as subband 0. |  |

Table 6.3.2.1.2-3B: Mapping order of CSI fields of one CSI report, CSI part 1, csi-ReportMode= Mode 2

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 1 | CRI as in Table 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported; CRI as in Table 6.3.1.1.2-3B, if associated with one CSI-RS resource and if reported |
|  | Rank Combination Indicator as in Table 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported; Rank Indicator as in Table 6.3.1.1.2-3B, if associated with one CSI-RS resource and if reported; Zero padding bits OP, if needed |
|  | Wideband CQI for the first TB as in Table 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported; Wideband CQI for the first TB as in Table 6.3.1.1.2-3B, if associated with one CSI-RS resource and if reported |
|  | Subband differential CQI for the first TB with increasing order of subband number as in Table 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported; Subband differential CQI for the first TB with increasing order of subband number as in Table 6.3.1.1.2-3B, if associated with one CSI-RS resource and if reported |
| NOTE: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand as subband 0. |  |

Table 6.3.2.1.2-3C: Mapping order of CSI fields of one CSI report for reportQuantity=tdcp

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | Amplitude value for the configured delay values as in Table 6.3.2.1.2-10 based on the order from the first configured delay to the last configured delay |
|  | Phase value for the configured delay values as in Table 6.3.2.1.2-10 based on the order from the first configured delay to the last configured delay, if reported |

< Unchanged parts are omitted >

Table 6.3.2.1.2-5E: Mapping order of CSI fields of one CSI report, CSI part 2 of
codebookType=typeII-CJT

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2, group 0 | PMI fields X1, from left to right as in Table 6.3.2.1.2-1B, if reported; |
| CSI report #n CSI part 2, group 1 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-1B: i2,3,l:l=1,…,υ, i1,5, i1,6,l:l=1,…,υ, i1,9, and max⁡(0,KNZ2-υ)×3 highest priority bits of i2,4,l:l=1,…,υ,max⁡(0,KNZ2-υ)×4 highest priority bits of {i2,5,l:l=1,…,υ} and ν*2∙n=10N0-1Lσ(n)Mυ-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril, ij,f,j Pril,n, i,fdefined in clause 5.2.3 of TS 38.214 [6] where j is equivalent to n, if present and if reported; |
| CSI report #n CSI part 2, group 2 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-1B: minKNZ-v, KNZ2×3 lowest priority bits of i2,4,l:l=1,…,υ,minKNZ-v, KNZ2×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril, ij,f,j Pril,n, i,fdefined in clause 5.2.3 of TS 38.214 [6] where j is equivalent to n, if reported; |

Table 6.3.2.1.2-5F: Mapping order of CSI fields of one CSI report, CSI part 2 of
codebookType= typeII-Doppler

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2, group 0 | PMI fields X1, from left to right as in Table 6.3.2.1.2-1C, if reported; The second time-domain wideband CQI as in Table 6.3.1.1.2-8B, if present and reported |
| CSI report #n CSI part 2, group 1 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-1C: i2,3,l:l=1,…,υ, i1,5, i1,6,l:l=1,…,υ, i1,10,l:l=1,…,υ, and max⁡(0,KNZ2-υ)×3 highest priority bits of i2,4,l:l=1,…,υ,max⁡(0,KNZ2-υ)×4 highest priority bits of {i2,5,l:l=1,…,υ} and ν*2LMυQ-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f for N4=1 or Pril,i,f,j for N4>1 Pril,i,f, qdefined in clause 5.2.3 of TS 38.214 [6], if present and if reported; The second time-domain subband differential CQI of all even subbands with increasing order of subband number, as in Table 6.3.1.1.2-8B, if present and if reported |
| CSI report #n CSI part 2, group 2 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-1C: minKNZ-v, KNZ2×3 lowest priority bits of i2,4,l:l=1,…,υ,minKNZ-v, KNZ2×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f for N4=1 or Pril,i,f,j for N4>1 Pril,i,f, qdefined in clause 5.2.3 of TS 38.214 [6], if reported; The second time-domain subband differential CQI of all odd subbands with increasing order of subband number, as in Table 6.3.1.1.2-8B, if present and if reported |

Table 6.3.2.1.2-5G: Mapping order of CSI fields of one CSI report, CSI part 2 of
codebookType=typeII-CJT-PortSelection

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2, group 0 | PMI fields X1, from left to right as in Table 6.3.2.1.2-2C, if reported |
| CSI report #n CSI part 2, group 1 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-2C: i2,3,l:l=1,…,υ (max⁡(0,KNZ2-υ))×3 highest priority bits of i2,4,l:l=1,…,υ,(max⁡(0,KNZ2-υ))×4 highest priority bits of {i2,5,l:l=1,…,υ}, i1,9 and ν*n=10N0N-1K1,σ(n)nM-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril, ij,f,j Pril,n, i,fdefined in clause 5.2.3 of TS 38.214 [6] where j is equivalent to n, if present and if reported |
| CSI report #n CSI part 2, group 2 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-2C: (minKNZ-v, KNZ2)×3 lowest priority bits of i2,4,l:l=1,…,υ,(minKNZ-v, KNZ2)×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril, ij,f,j Pril,n, i,fdefined in clause 5.2.3 of TS 38.214 [6] where j is equivalent to n, if reported |

< Unchanged parts are omitted >

#### 7.3.1.1	DCI formats for scheduling of PUSCH

< Unchanged parts are omitted >

##### 7.3.1.1.2	Format 0_1

DCI format 0_1 is used for the scheduling of one or multiple PUSCH in one cell, or indicating CG downlink feedback information (CG-DFI) to a UE.

< Unchanged parts are omitted >

-	Frequency hopping flag - 0 or 1 bit:

-	0 bit if only resource allocation type 0 is configured, or if the higher layer parameter frequencyHopping is not configured and the higher layer parameter pusch-RepTypeIndicatorDCI-0-1 is not configured to pusch-RepTypeB, or if the higher layer parameter frequencyHoppingDCI-0-1 is not configured and pusch-RepTypeIndicatorDCI-0-1 is configured to pusch-RepTypeB, or if only resource allocation type 2 is configured;

-	1 bit according to Table 7.3.1.1.1-3 otherwise, only applicable to resource allocation type 1, as defined in Clause 6.3 of [6, TS 38.214].

For transport block 1:

-	Modulation and coding scheme - 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214]

-	New data indicator - 1 bit if the number of scheduled PUSCH indicated by the Time domain resource assignment field is 1; otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined based on the maximum number of schedulable PUSCH among all entries in the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH, where each bit corresponds to one scheduled PUSCH as defined in clause 6.1.4 in [6, TS 38.214].

-	Redundancy version - - number of bits determined by the following:

-	2 bits as defined in Table 7.3.1.1.1-2 if the number of scheduled PUSCH indicated by the Time domain resource assignment field is 1;

-	otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined by the maximum number of schedulable PUSCHs among all entries in the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH, where each bit corresponds to one scheduled PUSCH as defined in clause 6.1.4 in [6, TS 38.214] and redundancy version is determined according to Table 7.3.1.1.2-34.

For transport block 2 (only present if maxRank > 4 or maxMIMO-Layers > 4):

-	Modulation and coding scheme - 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214]

-	New data indicator - 1 bit

-	Redundancy version - 2 bits as defined in Table 7.3.1.1.1-2

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part, the value of maxRank or maxMIMO-Layers for the indicated bandwidth part is larger than 4 and the value of maxRank or maxMIMO-Layers for the active bandwidth part is no more than 4, the UE assumes zeros are padded when interpreting the "Modulation and coding scheme", "New data indicator", and "Redundancy version" fields for transport block 2 according to Clause 12 of [5, TS38.213], and the UE ignores the "Modulation and coding scheme", "New data indicator", and "Redundancy version" fields of transport block 2 for the indicated bandwidth part.

-	Transform precoder indicator - 0 or 1 bit

-	1 bit if the higher layer parameter dynamicTransformPrecoderIndicationDCI-0-1 is configured to 'enabled ' and if the UE is configured to monitor DCI format 0_1 with CRC scrambled by C-RNTI or MCS-C-RNTI, where the bit value of 0 indicates that transform precoder is enabled and the bit value of 1 indicates that transform precoder is disabled.

-	0 bit otherwise.

-	HARQ process number - 5 bits if higher layer parameter harq-ProcessNumberSizeDCI-0-1 is configured; otherwise 4 bits

< Unchanged parts are omitted >

-	Second SRS resource indicator - number of bits determined by the following:

-	log2(maxk∈{1,2,…,min⁡{Lmax,NSRS}}NSRSk) bits according to Tables 7.3.1.1.2-28/29A/30A/31A with the same number of layers indicated by SRS resource indicator field if the higher layer parameter txConfig = nonCodebook, the higher layer paramtere maxMIMO-LayersforSdm is not configured, and SRS resource set indicator field is present, where NSRS is the number of configured SRS resources in the second SRS resource set, and

-	if UE supports operation with maxMIMO-Layers and the higher layer parameter maxMIMO-Layers of PUSCH-ServingCellConfig of the serving cell is configured,

-	Lmax is given by maxMIMO-LayersforSfn if maxMIMO-LayersforSfn is configured

-	Lmax is given by maxMIMO-Layers otherwise

-	otherwise, Lmax is given by the maximum number of layers for PUSCH supported by the UE for the serving cell for non-codebook based operation.

-	 bits according to Tables 7.3.1.1.2-28/29 if the higher layer parameter txConfig = nonCodebook, the higher layer paramtere maxMIMO-LayersforSdm is configured and SRS resource set indicator field is present, where NSRS is the number of configured SRS resources in the second SRS resource set, and Lmax is given by maxMIMO-LayersforSdm.

-	log2(NSRS) bits according to Tables 7.3.1.1.2-32, 7.3.1.1.2-32A and 7.3.1.1.2-32B if the higher layer parameter txConfig = codebook and SRS resource set indicator field is present, where NSRS is the number of configured SRS resources in the second SRS resource set.

-	0 bit otherwise.

-	Precoding information and number of layers - number of bits determined by the following:

-	0 bits if the higher layer parameter txConfig = nonCodeBook;

-	0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook;

-	4, 5, or 6 bits according to Table 7.3.1.1.2-2 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank if multipanelScheme is not configured or max{maxRank, maxRankSfn} if multipanelScheme = sfnScheme or max{maxRank, maxRankSdm} if multipanelScheme = sdmScheme, and codebookSubset;

-	4 or 5 bits according to Table 7.3.1.1.2-2A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=2 if multipanelScheme is not configured or max{maxRank, maxRankSfn} = 2 if multipanelScheme = sfnScheme or max{maxRank, maxRankSdm} = 2 if multipanelScheme = sdmScheme, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset;

-	4 or 6 bits according to Table 7.3.1.1.2-2B for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=3 or 4, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and maxRank=1 if multipanelScheme is not configured or max{maxRank, maxRankSfn} = 1 if multipanelScheme = sfnScheme or max{maxRank, maxRankSdm} = 1 if multipanelScheme = sdmScheme, and codebookSubset;

-	3 or 4 bits according to Table 7.3.1.1.2-3A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=1 if multipanelScheme is not configured or max{maxRank, maxRankSfn} = 1 if multipanelScheme = sfnScheme or max{maxRank, maxRankSdm} = 1 if multipanelScheme = sdmScheme, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset;

-	2 or 4 bits according to Table7.3.1.1.2-4 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank if multipanelScheme is not configured or max{maxRank, maxRankSfn} if multipanelScheme = sfnScheme or max{maxRank, maxRankSdm} if multipanelScheme = sdmScheme, and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-4A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, transform precoder is disabled, maxRank=2 if multipanelScheme is not configured or max{maxRank, maxRankSfn} = 2 if multipanelScheme = sfnScheme or max{maxRank, maxRankSdm} = 2 if multipanelScheme = sdmScheme, and codebookSubset=nonCoherent;

-	1 or 3 bits according to Table7.3.1.1.2-5 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and maxRank=1 if multipanelScheme is not configured or max{maxRank, maxRankSfn}=1 if multipanelScheme = sfnScheme or max{maxRank, maxRankSdm}=1 if multipanelScheme = sdmScheme,  and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-5A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=1 if multipanelScheme is not configured or max{maxRank, maxRankSfn} = 1 if multipanelScheme = sfnScheme or max{maxRank, maxRankSdm} = 1 if multipanelScheme = sdmScheme, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset;

-	7 bits according to Table 7.3.1.1.2-5B for 8 antenna ports, if CodebookType=Codebook1, transform precoder is disabled, maxRank = 8, and according to ULcodebookFC-N1N2;

-	7 bits according to Table 7.3.1.1.2-5C for 8 antenna ports, if CodebookType=Codebook1, transform precoder is disabled, maxRank =7, and according to ULcodebookFC-N1N2;

-	7 bits according to Table 7.3.1.1.2-5D for 8 antenna ports, if CodebookType=Codebook1, transform precoder is disabled, maxRank =4, 5 or 6, and according to maxRank;

-	4, 6 or 7 bits according to Table 7.3.1.1.2-5E for 8 antenna ports, if CodebookType=Codebook1, transform precoder is enabled or maxRank =1, 2 or 3 if transform precoder is disabled, and according to transform precoder and maxRank;

-	8 bits according to Table 7.3.1.1.2-5F for 8 antenna ports, if CodebookType=Codebook4, transform precoder is disabled, maxRank=5, 6, 7 or 8, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to maxRank;

-	6 or 7 or 8 bits according to Table 7.3.1.1.2-5G for 8 antenna ports, if CodebookType=Codebook4, transform precoder is disabled, maxRank=2, 3 or 4, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to maxRank;

-	3 bits according to Table 7.3.1.1.2-5H for 8 antenna ports, if CodebookType=Codebook4, transform precoder is enabled or maxRank=1 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower.

-	10 bits according to Table 7.3.1.1.2-5I for 8 antenna ports, if CodebookType=Codebook2, transform precoder is disabled, maxRank=5, 6, 7 or 8, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to maxRank;

-	5, 9 or 10 bits according to Table 7.3.1.1.2-5J for 8 antenna ports, if CodebookType=Codebook2, transform precoder is enabled or maxRank =1, 2, 3 or 4 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to transform precoder and maxRank;

-	10 bits according to Table 7.3.1.1.2-5K for 8 antenna ports, if CodebookType=Codebook3, transform precoder is disabled, maxRank=5, 6, 7 or 8, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to maxRank;

-	4, 7, 9 or 10 bits according to Table 7.3.1.1.2-5L for 8 antenna ports, if CodebookType=Codebook3, transform precoder is enabled or maxRank =1, 2, 3 or 4 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to transform precoder and maxRank;

-	6 or 7 or 8 bits according to Table 7.3.1.1.2-5M for 8 antenna ports, if CodebookType=Codebook4, transform precoder is disabled, maxRank=2, 3 or 4, ul-FullPowerTransmission is configured to fullpowerMode1, and according to maxRank;

-	4 bits according to Table 7.3.1.1.2-5N for 8 antenna ports, if CodebookType=Codebook4, transform precoder is enabled or maxRank=1 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1.

-	6, 9 or 10 bits according to Table 7.3.1.1.2-5O for 8 antenna ports, if CodebookType=Codebook2, transform precoder is enabled or maxRank =1, 2, 3 or 4 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1, and according to transform precoder and maxRank;

-	5, 7, 9 or 10 bits according to Table 7.3.1.1.2-5P for 8 antenna ports, if CodebookType=Codebook3, transform precoder is enabled or maxRank =1, 2, 3, or 4 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1, and according to transform precoder and maxRank;

-	8 or 9 bits according to Table 7.3.1.1.2-5Q for 8 antenna ports, if CodebookType=Codebook4, transform precoder is disabled, maxRank=5, 6, 7 or 8, ul-FullPowerTransmission is configured to fullpowerMode1, and according to maxRank;

-	10 bits according to Table 7.3.1.1.2-5R for 8 antenna ports, if CodebookType=Codebook2, transform precoder is disabled, maxRank=5, 6, 7 or 8, ul-FullPowerTransmission is configured to fullpowerMode1, and according to maxRank;

-	10 bits according to Table 7.3.1.1.2-5S for 8 antenna ports, if CodebookType=Codebook3, transform precoder is disabled, maxRank =5, 6, 7, or 8, ul-FullPowerTransmission is configured to fullpowerMode1, and according to maxRank;

For the higher layer parameter txConfig=codebook, if ul-FullPowerTransmission is configured to fullpowerMode2, maxRank is configured to be larger than 2, and at least one SRS resource with 4 antenna ports or 8 antenna ports is configured in the SRS resource set indicated by SRS resource set indicator field if present, otherwise in an SRS resource set with usage set to 'codebook', and an SRS resource with 2 antenna ports is indicated via SRI in the same SRS resource set, then Table 7.3.1.1.2-4 is used.

For the higher layer parameter txConfig=codebook, if ul-FullPowerTransmission is configured to fullpowerMode2, maxRank is configured to be larger than 4, and at least one SRS resource with 8 antenna ports is configured in the SRS resource set with usage set to 'codebook', and an SRS resource with 4 antenna ports is indicated via SRI in the same SRS resource set, then Table 7.3.1.1.2-2 is used.

For the higher layer parameter txConfig = codebook, if different SRS resources with different number of antenna ports are configured, the bitwidth is determined according to the maximum number of ports in an SRS resource among the configured SRS resources in all SRS resource set(s) with usage set to 'codebook'. If the number of ports for a configured SRS resource in the set is less than the maximum number of ports in an SRS resource among the configured SRS resources, a number of most significant bits with value set to '0' are inserted to the field.

When the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs, and is provided enableSTx2PofmDCI, and there are two SRS resource sets configured by srs-ResourceSetToAddModList and associated with usage of value 'codebook' or 'nonCodeBook', the Precoding information and number of layers field is associated with the SRS resource set that is associated with the coresetPoolIndex value for the CORESET used for the PDCCH carrying the DCI format 0_1.

For the higher layer parameter txConfig = codebook, when the Transform precoder indicator field is present, if the bit width of the Precoding information and number of layers field for the case with transform precoder enabled is not equal to that for the case with transform precoder disabled, a number of most significant bits with value set to '0' are inserted to the Precoding information and number of layers field for the case with smaller bit width until the bit width of the Precoding information and number of layers field for the two cases are the same.

-	Second Precoding information - number of bits determined by the following:

-	0 bits if SRS resource set indicator field is not present;

-	0 bits if the higher layer parameter txConfig = nonCodeBook;

-	0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook;

-	3, 4, or 5 bits according to Table 7.3.1.1.2-2C with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank if multipanelScheme is not configured or maxRankSfn if multipanelScheme = sfnScheme, and codebookSubset;

-	3 or 4 bits according to Table 7.3.1.1.2-2D with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=2 if multipanelScheme is not configured or maxRankSfn=2 if multipanelScheme = sfnScheme, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset;

-	3 or 4 bits according to Table 7.3.1.1.2-2E with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=3 or 4, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank if multipanelScheme is not configured or maxRankSfn if multipanelScheme = sfnScheme, and codebookSubset;

-	3 or 4 bits according to Table 7.3.1.1.2-3A with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=1 if multipanelScheme is not configured or maxRankSfn=1 if multipanelScheme = sfnScheme, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset;

-	1 or 3 bits according to Table7.3.1.1.2-4B with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank if multipanelScheme is not configured or maxRankSfn if multipanelScheme = sfnScheme, and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-4C with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, transform precoder is disabled, maxRank=2 if multipanelScheme is not configured or maxRankSfn=2 if multipanelScheme = sfnScheme, and codebookSubset=nonCoherent;

-	1 or 3 bits according to Table7.3.1.1.2-5 with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank if multipanelScheme is not configured or maxRankSfn if multipanelScheme = sfnScheme, and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-5A with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=1 if multipanelScheme is not configured or maxRankSfn=1 if multipanelScheme = sfnScheme, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset;

-	4, 5, or 6 bits according to Table 7.3.1.1.2-2 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRankSdm if multipanelScheme = sdmScheme, and codebookSubset;

-	4 or 5 bits according to Table 7.3.1.1.2-2A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRankSdm = 2 if multipanelScheme = sdmScheme, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRankSdm if multipanelScheme = sdmScheme, and codebookSubset;

-	3 or 4 bits according to Table 7.3.1.1.2-3A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRankSdm = 1 if multipanelScheme = sdmScheme, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset;

-	2 or 4 bits according to Table7.3.1.1.2-4 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRankSdm if multipanelScheme = sdmScheme, and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-4A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, transform precoder is disabled, maxRankSdm = 2 if multipanelScheme = sdmScheme, and codebookSubset=nonCoherent;

-	1 or 3 bits according to Table7.3.1.1.2-5 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and maxRankSdm= 1 if multipanelScheme = sdmScheme, and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-5A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRankSdm = 1 if multipanelScheme = sdmScheme, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset;

For the higher layer parameter txConfig=codebook, if ul-FullPowerTransmission is configured to fullpowerMode2, maxRank is configured to be larger than 2, and at least one SRS resource with 4 antenna ports is configured in the SRS resource set indicated by SRS resource set indicator field, and an SRS resource with 2 antenna ports is indicated via Second SRS resource indicator field in the same SRS resource set, then Table 7.3.1.1.2-4B is used.

For the higher layer parameter txConfig = codebook, if different SRS resources with different number of antenna ports are configured, the bitwidth is determined according to the maximum number of ports in an SRS resource among the configured SRS resources in the second SRS resource set with usage set to 'codebook' as defined in Table 7.3.1.1.2-36. If the number of ports for a configured SRS resource in the set is less than the maximum number of ports in an SRS resource among the configured SRS resources, a number of most significant bits with value set to '0' are inserted to the field.

For the higher layer parameter txConfig = codebook, when the Transform precoder indicator field is present, if the bit width of the Second Precoding information field for the case with transform precoder enabled is not equal to that for the case with transform precoder disabled, a number of most significant bits with value set to '0' are inserted to the Second Precoding information field for the case with smaller bit width until the bit width of the Second Precoding information field for the two cases are the same.

< Unchanged parts are omitted >

-	DMRS sequence initialization - 0 bit if transform precoder is enabled; 1 bit if transform precoder is disabled or if the Transform precoder indicator field is present.

-	UL-SCH indicator - 0 or 1 bit as follows

-	0 bit if the number of scheduled PUSCH indicated by the Time domain resource assignment field is larger than 1;

-	1 bit otherwise. A value of "1" indicates UL-SCH shall be transmitted on the PUSCH and a value of "0" indicates UL-SCH shall not be transmitted on the PUSCH. If a UE does not support triggering SRS only in DCI, except for DCI format 0_1 with CRC scrambled by SP-CSI-RNTI, the UE is not expected to receive a DCI format 0_1 with UL-SCH indicator of "0" and CSI request of all zero(s). If a UE supports triggering SRS only in DCI, except for DCI format 0_1 with CRC scrambled by SP-CSI-RNTI, the UE is not expected to receive a DCI format 0_1 with UL-SCH indicator of "0", CSI request of all zero(s) and SRS request of all zero(s). The UE is not expected to receive a DCI format 0_1 with UL-SCH indicator of "0", when the indicated number of layers is larger than 4.

-	ChannelAccess-CPext-CAPC - 0, 1, 2, 3, 4, 5 or 6 bits. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter ul-AccessConfigListDCI-0-1 or in Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access in frequency range 1, or for operation in frequency range 2-2 if ChannelAccessMode2-r17 is provided; otherwise 0 bit. One or more entries from Table 7.3.1.1.2-35 or Table 7.3.1.1.2-35A are configured by the higher layer parameter ul-AccessConfigListDCI-0-1.

< Unchanged parts are omitted >

Table 7.3.1.1.2-5E: Precoding information and number of layers, for 8 antenna ports, if transform precoder is enabled or maxRank=1 or 2 or 3 if transform precoder is disabled, CodebookType=Codebook1, ULcodebookFC-N1N2 = (4,1) or (2,2)

| Bit field mapped to index | transform precoder is enabled, or maxRank=1 if transform precoder is disabled | Bit field mapped to index | transform precoder is disabled, and maxRank=2 | Bit field mapped to index | transform precoder is disabled, and maxRank=3 |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … | … | … |
| 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 |
|  |  | 16 | 2 layer2: TPMI=0 | 16 | 2 layer2: TPMI=0 |
|  |  | 17 | 2 layer2: TPMI=1 | 17 | 2 layer2: TPMI=1 |
|  |  | … | … | … | … |
|  |  | 47 | 2 layers: TPMI=31 | 47 | 2 layers: TPMI=31 |
|  |  | 48-63 | reserved | 48 | 3 layers: TPMI=0 |
|  |  |  |  | 49 | 3 layers: TPMI=1 |
|  |  |  |  | … | … |
|  |  |  |  | 71 | 3 layers: TPMI=23 |
|  |  |  |  | 72-127 | reserved |

< Unchanged parts are omitted >

Table 7.3.1.1.2-5M: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 2, 3 or 4, CodebookType=Codebook4, and ul-FullPowerTransmission configured to fullpowerMode1

| Bit field mapped to index | maxRank = 2 | Bit field mapped to index | maxRank = 3 | Bit field mapped to index | maxRank = 4 |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … | … | … |
| 7 | 1 layer: TPMI=7 | 7 | 1 layer: TPMI=7 | 7 | 1 layer: TPMI=7 |
| 8 | 2 layers: TPMI=8 | 8 | 2 layers: TPMI=8 | 8 | 2 layers: TPMI=8 |
| … | … | … | … | … | … |
| 35 | 2 layers: TPMI=35 | 35 | 2 layers: TPMI=35 | 35 | 2 layers: TPMI=35 |
| 36 | 1 layer: TPMI=255 | 36 | 3 layers: TPMI=36 | 36 | 3 layers: TPMI=36 |
| 37 | 2 layers: TPMI=256 | … | … | … | … |
| 38-63 | reserved | 91 | 3 layers: TPMI=91 | 91 | 3 layers: TPMI=91 |
|  |  | 92 | 1 layer: TPMI=255 | 92 | 4 layers: TPMI=92 |
|  |  | 93 | 2 layers: TPMI=256 | … | … |
|  |  | 94 | 3 layers: TPMI=257 | 161 | 4 layers: TPMI=161 |
|  |  | 95-127 | reserved | 162 | 1 layer: TPMI=255 |
|  |  |  |  | 163 | 2 layers: TPMI=256 |
|  |  |  |  | 164 | 3 layers: TPMI=257 |
|  |  |  |  | 165 | 4 layers: 258 |
|  |  |  |  | 166-255 | reserved |

Table 7.3.1.1.2-5N: Precoding information and number of layers, for 8 antenna ports, if transform precoder is enabled or maxRank=1 if transform is disabled, CodebookType=Codebook4, and ul-FullPowerTransmission configured to fullpowerMode1

| Bit field mapped to index | Precoding information and number of layers |
| --- | --- |
| 0 | 1 layer: TPMI=0 |
| … | … |
| 7 | 1 layer: TPMI=7 |
| 8 | 1 layer: TPMI=255 |
| 9-15 | reserved |

Table 7.3.1.1.2-5O: Precoding information and number of layers, for 8 antenna ports, if transform precoder is enabled, or maxRank = 1, 2, 3 or 4 if transform precoder is disabled, CodebookType=Codebook2, and ul-FullPowerTransmission configured to fullpowerMode1

| Bit field mapped to index | Transform precoder is enabled, or maxRank = 1 if transform precoder is disabled | Bit field mapped to index | Transform precoder is disabled and maxRank = 2 | Bit field mapped to index | Transform precoder is disabled and maxRank = 3 | Bit field mapped to index | Transform precoder is disabled and maxRank = 4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 31 | 1 layer: TPMI=31 | 31 | 1 layer: TPMI=31 | 31 | 1 layer: TPMI=31 | 31 | 1 layer: TPMI=31 |
| 32 | 1 layer: TPMI=32 | 32 | 2 layers: TPMI=0 | 32 | 2 layers: TPMI=0 | 32 | 2 layers: TPMI=0 |
| 33-63 | reserved | … | … | … | … | … | … |
|  |  | 303 | 2 layers: TPMI=271 | 303 | 2 layers: TPMI=271 | 303 | 2 layers: TPMI=271 |
|  |  | 304 | 1 layer: TPMI=32 | 304 | 3 layers: TPMI=0 | 304 | 3 layers: TPMI=0 |
|  |  | 305-511 | reserved | … | … | … | … |
|  |  |  |  | 567 | 3 layers: TPMI=263 | 567 | 3 layers: TPMI=263 |
|  |  |  |  | 568 | 1 layer: TPMI=32 | 568 | 4 layers: TPMI=0 |
|  |  |  |  | 569-1023 | reserved | … | … |
|  |  |  |  |  |  | 635 | 4 layers: TPMI=67 |
|  |  |  |  |  |  | 636 | 1 layer: TPMI=32 |
|  |  |  |  |  |  | 637-1023 | reserved |

Table 7.3.1.1.2-5P: Precoding information and number of layers, for 8 antenna ports, if transform precoder is enabled, or maxRank = 1, 2, 3 or 4 if transform precoder is disabled, CodebookType=Codebook3, and ul-FullPowerTransmission is configured to fullpowerMode1

| Bit field mapped to index | Transform precoder is enabled, or maxRank = 1 if transform precoder is disabled | Bit field mapped to index | transform precoder is disabled and maxRank = 2 | Bit field mapped to index | transform precoder is disabled and maxRank = 3 | Bit field mapped to index | transform precoder is disabled and maxRank = 4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 |
| 16 | 1 layer: TPMI=16 | 16 | 2 layers: TPMI=0 | 16 | 2 layers: TPMI=0 | 16 | 2 layers: TPMI=0 |
| 17-31 | reserved | … | … | … | … | … | … |
|  |  | 119 | 2 layers: TPMI=103 | 119 | 2 layers: TPMI=103 | 119 | 2 layers: TPMI=103 |
|  |  | 120 | 1 layer: TPMI=16 | 120 | 3 layers: TPMI=0 | 120 | 3 layers: TPMI=0 |
|  |  | 121 | 2 layers: TPMI=104 | … | … | … | … |
|  |  | 122-127 | reserved | 423 | 3 layers: TPMI=303 | 423 | 3 layers: TPMI=303 |
|  |  |  |  | 424 | 1 layer: TPMI=16 | 424 | 4 layers: TPMI=0 |
|  |  |  |  | 425 | 2 layers: TPMI=104 | … | … |
|  |  |  |  | 426 | 3 layers: 304 | 703 | 4 layers: TPMI=279 |
|  |  |  |  | 427-511 | reserved | 704 | 1 layer: TPMI=16 |
|  |  |  |  |  |  | 705 | 2 layers: TPMI=104 |
|  |  |  |  |  |  | 706 | 3 layers: TPMI=304 |
|  |  |  |  |  |  | 707-1023 | reserved |

Table 7.3.1.1.2-5Q: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 5, 6, 7, 8, CodebookType=Codebook4, and ul-FullPowerTransmission is configured to fullpowerMode1

| Bit field mapped to index | maxRank = 5 | Bit field mapped to index | maxRank = 6 | Bit field mapped to index | maxRank = 7 | Bit field mapped to index | maxRank = 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 7 | 1 layer: TPMI=7 | 7 | 1 layer: TPMI=7 | 7 | 1 layer: TPMI=7 | 7 | 1 layer: TPMI=7 |
| 8 | 2 layers: TPMI=8 | 8 | 2 layers: TPMI=8 | 8 | 2 layers: TPMI=8 | 8 | 2 layers: TPMI=8 |
| … | … | … | … | … | … | … | … |
| 35 | 2 layers: TPMI=35 | 35 | 2 layers: TPMI=35 | 35 | 2 layers: TPMI=35 | 35 | 2 layers: TPMI=35 |
| 36 | 3 layers: TPMI=36 | 36 | 3 layers: TPMI=36 | 36 | 3 layers: TPMI=36 | 36 | 3 layers: TPMI=36 |
| … | … | … | … | … | … | … | … |
| 91 | 3 layers: TPMI=91 | 91 | 3 layers: TPMI=91 | 91 | 3 layers: TPMI=91 | 91 | 3 layers: TPMI=91 |
| 92 | 4 layers: TPMI=92 | 92 | 4 layers: TPMI=92 | 92 | 4 layers: TPMI=92 | 92 | 4 layers: TPMI=92 |
| … | … | … | … | … | … | … | … |
| 161 | 4 layers: TPMI=161 | 161 | 4 layers: TPMI=161 | 161 | 4 layers: TPMI=161 | 161 | 4 layers: TPMI=161 |
| 162 | 5 layers: TPMI=162 | 162 | 5 layers: TPMI=162 | 162 | 5 layers: TPMI=162 | 162 | 5 layers: TPMI=162 |
| … | … | … | … | … | … | … | … |
| 217 | 5 layers: TPMI=217 | 217 | 5 layers: TPMI=217 | 217 | 5 layers: TPMI=217 | 217 | 5 layers: TPMI=217 |
| 218 | 1 layer: TPMI=255 | 218 | 6 layers: TPMI=218 | 218 | 6 layers: TPMI=218 | 218 | 6 layers: TPMI=218 |
| 219 | 2 layers: TPMI=256 | … | … | … | … | … | … |
| 220 | 3 layers: TPMI=257 | 245 | 6 layers: TPMI=245 | 245 | 6 layers: TPMI=245 | 245 | 6 layers: TPMI=245 |
| 221 | 4 layers: TPMI=258 | 246 | 1 layer: TPMI=255 | 246 | 7 layers: TPMI=246 | 246 | 7 layers: TPMI=246 |
| 222-255 | reserved | 247 | 2 layers: TPMI=256 | … | … | … | … |
|  |  | 248 | 3 layers: TPMI=257 | 253 | 7 layers: TPMI=253 | 253 | 7 layers: TPMI=253 |
|  |  | 249 | 4 layers: TPMI=258 | 254 | 1 layer: TPMI=255 | 254 | 8 layers: TPMI=254 |
|  |  | 250-255 | reserved | 255 | 2 layers: TPMI=256 | 255 | 1 layer: TPMI=255 |
|  |  |  |  | 256 | 3 layers: TPMI=257 | 256 | 2 layers: TPMI=256 |
|  |  |  |  | 257 | 4 layers: TPMI=258 | 257 | 3 layers: TPMI=257 |
|  |  |  |  | 258-511 | reserved | 258 | 4 layers: TPMI=258 |
|  |  |  |  |  |  | 259-511 | reserved |

Table 7.3.1.1.2-5R: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 5, 6, 7, 8, CodebookType=Codebook2, and ul-FullPowerTransmission is configured to fullpowerMode1

| Bit field mapped to index | maxRank = 5 | Bit field mapped to index | maxRank = 6 | Bit field mapped to index | maxRank = 7 | Bit field mapped to index | maxRank = 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 31 | 1 layer: TPMI=31 | 31 | 1 layer: TPMI=31 | 31 | 1 layer: TPMI=31 | 31 | 1 layer: TPMI=31 |
| 32 | 2 layers: TPMI=0 | 32 | 2 layers: TPMI=0 | 32 | 2 layers: TPMI=0 | 32 | 2 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 303 | 2 layers: TPMI=271 | 303 | 2 layers: TPMI=271 | 303 | 2 layers: TPMI=271 | 303 | 2 layers: TPMI=271 |
| 304 | 3 layers: TPMI=0 | 304 | 3 layers: TPMI=0 | 304 | 3 layers: TPMI=0 | 304 | 3 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 567 | 3 layers: TPMI=263 | 567 | 3 layers: TPMI=263 | 567 | 3 layers: TPMI=263 | 567 | 3 layers: TPMI=263 |
| 568 | 4 layers: TPMI=0 | 568 | 4 layers: TPMI=0 | 568 | 4 layers: TPMI=0 | 568 | 4 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 635 | 4 layers: TPMI=67 | 635 | 4 layers: TPMI=67 | 635 | 4 layers: TPMI=67 | 635 | 4 layers: TPMI=67 |
| 636 | 5 layers: TPMI=0 | 636 | 5 layers: TPMI=0 | 636 | 5 layers: TPMI=0 | 636 | 5 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 667 | 5 layers: TPMI=31 | 667 | 5 layers: TPMI=31 | 667 | 5 layers: TPMI=31 | 667 | 5 layers: TPMI=31 |
| 668 | 1 layer: TPMI=32 | 668 | 6 layers: TPMI=0 | 668 | 6 layers: TPMI=0 | 668 | 6 layers: TPMI=0 |
| 669-1023 | reserved | … | … | … | … | … | … |
|  |  | 683 | 6 layers: TPMI=15 | 683 | 6 layers: TPMI=15 | 683 | 6 layers: TPMI=15 |
|  |  | 684 | 1 layer: TPMI=32 | 684 | 7 layers: TPMI=0 | 684 | 7 layers: TPMI=0 |
|  |  | 685-1023 | reserved | … | … | … | … |
|  |  |  |  | 691 | 7 layers: TPMI=7 | 691 | 7 layers: TPMI=7 |
|  |  |  |  | 692 | 1 layer: TPMI=32 | 692 | 8 layers: TPMI=0 |
|  |  |  |  | 693-1023 | reserved | … | … |
|  |  |  |  |  |  | 695 | 8 layers: TPMI=3 |
|  |  |  |  |  |  | 696 | 1 layer: TPMI=32 |
|  |  |  |  |  |  | 697-1023 | reserved |

Table 7.3.1.1.2-5S: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 5, 6, 7, 8, CodebookType=Codebook3, and ul-FullPowerTransmission is configured to fullpowerMode1

| Bit field mapped to index | maxRank = 5 | Bit field mapped to index | maxRank = 6 | Bit field mapped to index | maxRank = 7 | Bit field mapped to index | maxRank = 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 |
| 16 | 2 layers: TPMI=0 | 16 | 2 layers: TPMI=0 | 16 | 2 layers: TPMI=0 | 16 | 2 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 119 | 2 layers: TPMI=103 | 119 | 2 layers: TPMI=103 | 119 | 2 layers: TPMI=103 | 119 | 2 layers: TPMI=103 |
| 120 | 3 layers: TPMI=0 | 120 | 3 layers: TPMI=0 | 120 | 3 layers: TPMI=0 | 120 | 3 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 423 | 3 layers: TPMI=303 | 423 | 3 layers: TPMI=303 | 423 | 3 layers: TPMI=303 | 423 | 3 layers: TPMI=303 |
| 424 | 4 layers: TPMI=0 | 424 | 4 layers: TPMI=0 | 424 | 4 layers: TPMI=0 | 424 | 4 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 703 | 4 layers: TPMI=279 | 703 | 4 layers: TPMI=279 | 703 | 4 layers: TPMI=279 | 703 | 4 layers: TPMI=279 |
| 704 | 5 layers: TPMI=0 | 704 | 5 layers: TPMI=0 | 704 | 5 layers: TPMI=0 | 704 | 5 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 863 | 5 layers: TPMI=159 | 863 | 5 layers: TPMI=159 | 863 | 5 layers: TPMI=159 | 863 | 5 layers: TPMI=159 |
| 864 | 1 layer: TPMI=16 | 864 | 6 layers: TPMI=0 | 864 | 6 layers: TPMI=0 | 864 | 6 layers: TPMI=0 |
| 865 | 2 layers: TPMI=104 | … | … | … | … | … | … |
| 866 | 3 layers: TPMI=304 | 943 | 6 layers: TPMI=79 | 943 | 6 layers: TPMI=79 | 943 | 6 layers: TPMI=79 |
| 867-1023 | reserved | 944 | 1 layer: TPMI=16 | 944 | 7 layers: TPMI=0 | 944 | 7 layers: TPMI=0 |
|  |  | 945 | 2 layers: TPMI=104 | … | … | … | … |
|  |  | 946 | 3 layers: TPMI=304 | 975 | 7 layers: TPMI=31 | 975 | 7 layers: TPMI=31 |
|  |  | 944-1023 | reserved | 976 | 1 layer: TPMI=16 | 976 | 8 layers: TPMI=0 |
|  |  |  |  | 977 | 2 layers: TPMI=104 | … | … |
|  |  |  |  | 978 | 3 layers: TPMI=304 | 991 | 8 layers: TPMI=15 |
|  |  |  |  | 979-1023 | reserved | 992 | 1 layer: TPMI=16 |
|  |  |  |  |  |  | 993 | 2 layers: TPMI=104 |
|  |  |  |  |  |  | 994 | 3 layers: TPMI=304 |
|  |  |  |  |  |  | 995-1023 | reserved |

< Unchanged parts are omitted >

Table 7.3.1.1.2-47: Antenna port(s), transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is configured, maxLength=2, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0,1 | 1 |
| 1 | 2 | 0,1 | 1 |
| 2 | 2 | 2,3 | 1 |
| 3 | 2 | 0,2 | 1 |
| 4 | 2 | 0,1 | 2 |
| 5 | 2 | 2,3 | 2 |
| 6 | 2 | 4,5 | 2 |
| 7 | 2 | 6,7 | 2 |
| 8 | 2 | 0,4 | 2 |
| 9 | 2 | 2,6 | 2 |
| 10 | 1 | 8,9 | 1 |
| 11 | 2 | 8,9 | 1 |
| 12 | 2 | 10,11 | 1 |
| 13 | 2 | 8,9 | 2 |
| 14 | 2 | 10,11 | 2 |
| 15 | 2 | 12,13 | 2 |
| 16 | 2 | 14,15 | 2 |
| 17 | 2 | 9,11 | 1 |
| 18 | 2 | 1,3 | 1 |
| 19 | 2 | 0,2 | 2 |
| 20 | 2 | 1,3 | 2 |
| 21 | 2 | 4,6 | 2 |
| 22 | 2 | 5,7 | 2 |
| 23 | 2 | 8,10 | 2 |
| 24 | 2 | 9,11 | 2 |
| 25 | 2 | 12,14 | 2 |
| 26 | 2 | 13,15 | 2 |
| 27 | 1 | 0,1 | 2 |
| 28 | 1 | 8,9 | 2 |
| 29 | 1 | 4,5 | 2 |
| 30 | 1 | 12,13 | 2 |
| 31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-48: Antenna port(s), transform precoder is disabled, dmrs-Type=1, multipanelScheme is not configured to sdmScheme, enhanced-dmrs-Type is configured, maxLength=2, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 2 | 0,1,4 | 2 |
| 2 | 2 | 2,3,6 | 2 |
| 3 | 2 | 9-11 | 1 |
| 4 | 2 | 8,9,12 | 2 |
| 5 | 2 | 10,11,14 | 2 |
| 6 | 1 | 0,1,8 | 1 |
| 7 | 2 | 0,1,8 | 1 |
| 8 | 2 | 2,3,10 | 1 |
| 9 | 2 | 0,1,8 | 2 |
| 10 | 2 | 4,5,12 | 2 |
| 11 | 2 | 2,3,10 | 2 |
| 12 | 2 | 6,7,14 | 2 |
| 13 | 2 | 5,8,9 | 2 |
| 14 | 2 | 7,10,11 | 2 |
| 15 | 2 | 7,12,13 | 2 |
| 16-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-48A: Antenna port(s), transform precoder is disabled, dmrs-Type=1, multipanelScheme is configured to sdmScheme, enhanced-dmrs-Type is configured, maxLength=2, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 2 | 0,1,4 | 2 |
| 2 | 2 | 2,3,6 | 2 |
| 3 | 2 | 9-11 | 1 |
| 4 | 2 | 8,9,12 | 2 |
| 5 | 2 | 10,11,14 | 2 |
| 6 | 1 | 0,1,8 | 1 |
| 7 | 2 | 0,1,8 | 1 |
| 8 | 2 | 2,3,10 | 1 |
| 9 | 2 | 0,1,8 | 2 |
| 10 | 2 | 4,5,12 | 2 |
| 11 | 2 | 2,3,10 | 2 |
| 12 | 2 | 6,7,14 | 2 |
| 13 | 2 | 5,8,9 | 2 |
| 14 | 2 | 7,10,11 | 2 |
| 15 | 2 | 7,12,13 | 2 |
| 16 | 2 | 0,2,3 | 1 |
| 17-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-49: Antenna port(s), transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is configured, maxLength=2, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-3 | 1 |
| 1 | 2 | 0,1,4,5 | 2 |
| 2 | 2 | 2,3,6,7 | 2 |
| 3 | 2 | 0,2,4,6 | 2 |
| 4 | 2 | 8-11 | 1 |
| 5 | 2 | 8,9,12,13 | 2 |
| 6 | 2 | 10,11,14,15 | 2 |
| 7 | 2 | 1,3,5,7 | 2 |
| 8 | 1 | 0,1,8,9 | 1 |
| 9 | 2 | 0,1,8,9 | 1 |
| 10 | 2 | 2,3,10,11 | 1 |
| 11 | 1 | 0,1,8,9 | 2 |
| 12 | 1 | 4,5,12,13 | 2 |
| 13 | 2 | 0,1,8,9 | 2 |
| 14 | 2 | 4,5,12,13 | 2 |
| 15 | 2 | 2,3,10,11 | 2 |
| 16 | 2 | 6,7,14,15 | 2 |
| 17-31 | Reserved | Reserved | Reserved |

< Unchanged parts are omitted >

##### 7.3.1.1.3	Format 0_2

DCI format 0_2 is used for the scheduling of PUSCH in one cell.

< Unchanged parts are omitted >

-	Second SRS resource indicator - number of bits determined by the following:

-	log2(maxk∈{1,2,…,min⁡{Lmax,NSRS,0_2}}NSRS,0_2k)  bits according to Tables 7.3.1.1.2-28/29A/30A/31A with the same number of layers indicated by SRS resource indicator field if the higher layer parameter txConfig = nonCodebook, the higher layer parameter maxMIMO-LayersforSdmDCI-0-2 is not configured, and SRS resource set indicator field is present, where NSRS, 0_2 is the number of configured SRS resources in the second SRS resource set, and

-	if UE supports operation with maxMIMO-LayersDCI-0-2 and the higher layer parameter maxMIMO-LayersDCI-0-2 of PUSCH-ServingCellConfig of the serving cell is configured,

-	Lmax is given by maxMIMO-LayersforSfnDCI-0-2 if maxMIMO-LayersforSfnDCI-0-2 is configured

-	Lmax is given by maxMIMO-LayersDCI-0-2 otherwise

-	otherwise, Lmax is given by the maximum number of layers for PUSCH supported by the UE for the serving cell for non-codebook based operation.

-	log2k=1minLmax, NSRS,0_2NSRS,0_2k bits according to Tables 7.3.1.1.2-28/29 if the higher layer parameter txConfig = nonCodebook, the higher layer parameter maxMIMO-LayersforSdmDCI-0-2 is configured and SRS resource set indicator field is present, where NSRS, 0_2 is the number of configured SRS resources in the second SRS resource set, where the second SRS resource set is composed of the first NSRS, 0_2 SRS resources together with other configurations in the SRS resource set, or in the SRS resource set with higher srs-ResourceSetId of two SRS resources sets, configured by higher layer parameter srs-ResourceSetToAddModList, if any, and associated with the higher layer parameter usage of value 'nonCodeBook', except for the higher layer parameters 'srs-ResourceSetId' and 'srs-ResourceIdList', and Lmax is given by maxMIMO-LayersforSdmDCI-0-2.

-	log2NSRS, 0_2  bits according to Tables 7.3.1.1.2-32 if the higher layer parameter txConfig = codebook and SRS resource set indicator field is present, where  NSRS, 0_2 is the number of configured SRS resources in the second SRS resource set.

-	0 bit otherwise.

-	Precoding information and number of layers - number of bits determined by the following:

-	0 bits if the higher layer parameter txConfig = nonCodeBook;

-	0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook;

-	4, 5, or 6 bits according to Table 7.3.1.1.2-2 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRankDCI-0-2 if multipanelScheme is not configured or max{maxRankDCI-0-2, maxRankSfnDCI-0-2} if multipanelScheme = sfnScheme or max{maxRankDCI-0-2, maxRankSdmDCI-0-2} if multipanelScheme = sdmScheme, and codebookSubsetDCI-0-2;

-	4 or 5 bits according to Table 7.3.1.1.2-2A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, the values of higher layer parameters maxRankDCI-0-2=2 if multipanelScheme is not configured or max{maxRankDCI-0-2, maxRankSfnDCI-0-2} = 2 if multipanelScheme = sfnScheme or max{maxRanDCI-0-2k, maxRankSdmDCI-0-2} = 2 if multipanelScheme = sdmScheme, transform precoder is disabled, and according to the value of higher layer parameter codebookSubsetDCI-0-2;

-	4 or 6 bits according to Table 7.3.1.1.2-2B for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, the values of higher layer parameters maxRankDCI-0-2=3 or 4, transform precoder is disabled, and according to the value of higher layer parameter codebookSubsetDCI-0-2;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and maxRankDCI-0-2=1 if multipanelScheme is not configured or max{maxRankDCI-0-2, maxRankSfnDCI-0-2} = 1 if multipanelScheme = sfnScheme or max{maxRankDCI-0-2, maxRankSdmDCI-0-2} = 1 if multipanelScheme = sdmScheme, and codebookSubsetDCI-0-2;

-	3 or 4 bits according to Table 7.3.1.1.2-3A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, maxRankDCI-0-2=1 if multipanelScheme is not configured or max{maxRankDCI-0-2, maxRankSfnDCI-0-2} = 1 if multipanelScheme = sfnScheme or max{maxRankDCI-0-2, maxRankSdmDCI-0-2} = 1 if multipanelScheme = sdmScheme, and according to whether transform precoder is enabled or disabled, and the value of higher layer parameter codebookSubsetDCI-0-2;

-	2 or 4 bits according to Table7.3.1.1.2-4 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRankDCI-0-2 if multipanelScheme is not configured or max{maxRankDCI-0-2, maxRankSfnDCI-0-2} if multipanelScheme = sfnScheme or max{maxRankDCI-0-2, maxRankSdmDCI-0-2} if multipanelScheme = sdmScheme, and codebookSubsetDCI-0-2;

-	2 bits according to Table 7.3.1.1.2-4A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, transform precoder is disabled, the maxRankDCI-0-2=2 if multipanelScheme is not configured or max{maxRankDCI-0-2, maxRankSfnDCI-0-2} = 2 if multipanelScheme = sfnScheme or max{maxRankDCI-0-2, maxRankSdmDCI-0-2} = 2 if multipanelScheme = sdmScheme, and codebookSubsetDCI-0-2=nonCoherent;

-	1 or 3 bits according to Table7.3.1.1.2-5 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and maxRankDCI-0-2=1 if multipanelScheme is not configured or max{maxRankDCI-0-2, maxRankSfnDCI-0-2}=1 if multipanelScheme = sfnScheme or max{maxRankDCI-0-2, maxRankSdmDCI-0-2}=1 if multipanelScheme = sdmScheme, and codebookSubsetDCI-0-2;

-	2 bits according to Table 7.3.1.1.2-5A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, maxRankDCI-0-2=1 if multipanelScheme is not configured or max{maxRankDCI-0-2, maxRankSfnDCI-0-2} = 1 if multipanelScheme = sfnScheme or max{maxRankDCI-0-2, maxRankSdmDCI-0-2} = 1 if multipanelScheme = sdmScheme, and according to whether transform precoder is enabled or disabled, and the value of higher layer parameter codebookSubsetDCI-0-2.

-	7 bits according to Table 7.3.1.1.2-5D for 8 antenna ports by replacing maxRank with maxRankDCI-0-2, if CodebookType=Codebook1, transform precoder is disabled, maxRankDCI-0-2 =4, and according to maxRankDCI-0-2;

-	4, 6 or 7 bits according to Table 7.3.1.1.2-5E for 8 antenna ports by replacing maxRank with maxRankDCI-0-2, if CodebookType=Codebook1, transform precoder is enabled or maxRankDCI-0-2 =1, 2 or 3 if transform precoder is disabled, and according to transform precoder and maxRankDCI-0-2;

-	6 or 7 or 8 bits according to Table 7.3.1.1.2-5G for 8 antenna ports by replacing maxRank with maxRankDCI-0-2, if CodebookType=Codebook4, transform precoder is disabled, maxRankDCI-0-2=2, 3 or 4, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to maxRankDCI-0-2;

-	3 bits according to Table 7.3.1.1.2-5H for 8 antenna ports by replacing maxRank with maxRankDCI-0-2, if CodebookType=Codebook4, transform precoder is enabled or maxRankDCI-0-2=1 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower.

-	5, 9 or 10 bits according to Table 7.3.1.1.2-5J for 8 antenna ports by replacing maxRank with maxRankDCI-0-2, if CodebookType=Codebook2, transform precoder is enabled or maxRankDCI-0-2 =1, 2, 3 or 4 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to transform precoder and maxRankDCI-0-2;

-	4, 7, 9 or 10 bits according to Table 7.3.1.1.2-5L for 8 antenna ports by replacing maxRank with maxRankDCI-0-2, if CodebookType=Codebook3, transform precoder is enabled or maxRankDCI-0-2 =1, 2, 3 or 4 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to transform precoder and maxRankDCI-0-2;

-	6 or 7 or 8 bits according to Table 7.3.1.1.2-5M for 8 antenna ports by replacing maxRank with maxRankDCI-0-2, if CodebookType=Codebook4, transform precoder is disabled, maxRankDCI-0-2=2, 3 or 4, ul-FullPowerTransmission is configured to fullpowerMode1, and according to maxRankDCI-0-2;

-	4 bits according to Table 7.3.1.1.2-5N for 8 antenna ports, if CodebookType=Codebook4, transform precoder is enabled or maxRankDCI-0-2=1 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1.

-	6, 9 or 10 bits according to Table 7.3.1.1.2-5O for 8 antenna ports by replacing maxRank with maxRankDCI-0-2, if CodebookType=Codebook2, transform precoder is enabled or maxRankDCI-0-2 =1, 2, 3 or 4 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1, and according to transform precoder and maxRankDCI-0-2;

-	5, 7, 9 or 10 bits according to Table 7.3.1.1.2-5P for 8 antenna ports by replacing maxRank with maxRankDCI-0-2, if CodebookType=Codebook3, transform precoder is enabled or maxRankDCI-0-2 =1, 2, 3 or 4 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1, and according to transform precoder and maxRankDCI-0-2;

For the higher layer parameter txConfig=codebook, if ul-FullPowerTransmission is configured to fullpowerMode2, the values of higher layer parameters maxRankDCI-0-2 is configured to be larger than 2, and at least one SRS resource with 4 antenna ports is configured in the SRS resource set indicated by SRS resource set indicator field if present, otherwise in an SRS resource set with usage set to 'codebook', and an SRS resource with 2 antenna ports is indicated via SRI in the same SRS resource set, then Table 7.3.1.1.2-4 is used.

For the higher layer parameter txConfig = codebook, if different SRS resources with different number of antenna ports are configured, the bitwidth is determined according to the maximum number of ports in an SRS resource among the configured SRS resources in all SRS resource set(s) with usage set to 'codebook'. If the number of ports for a configured SRS resource in the set is less than the maximum number of ports in an SRS resource among the configured SRS resources, a number of most significant bits with value set to '0' are inserted to the field.

For the higher layer parameter txConfig = codebook, when the Transform precoder indicator field is present, if the bit width of the Precoding information and number of layers field for the case with transform precoder enabled is not equal to that for the case with transform precoder disabled, a number of most significant bits with value set to '0' are inserted to the Precoding information and number of layers field for the case with smaller bit width until the bit width of the Precoding information and number of layers field for the two cases are the same.

When the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs, and is provided enableSTx2PofmDCI, and there are two SRS resource sets configured by srs-ResourceSetToAddModListDCI-0-2 and associated with usage of value 'codebook' or 'nonCodeBook', the Precoding information and number of layers field is associated with the SRS resource set that is associated with the coresetPoolIndex value for the CORESET used for the PDCCH carrying the DCI format 0_2.

< Unchanged parts are omitted >

#### 7.3.1.2	DCI formats for scheduling of PDSCH

##### 7.3.1.2.1	Format 1_0

DCI format 1_0 is used for the scheduling of PDSCH in one DL cell.

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats - 1 bits

-	The value of this bit field is always set to 1, indicating a DL DCI format

-	Frequency domain resource assignment -  bits where  is given by Clause 7.3.1.0

If the CRC of the DCI format 1_0 is scrambled by C-RNTI and the "Frequency domain resource assignment" field are of all ones, the DCI format 1_0 is for random access procedure initiated by a PDCCH order, with all remaining fields set as follows:

-	Random Access Preamble index - 6 bits according to ra-PreambleIndex in Clause 5.1.2 of [8, TS38.321]

-	UL/SUL indicator - 1 bit. If the value of the "Random Access Preamble index" is not all zeros and if the UE is configured with supplementaryUplink in ServingCellConfig in the cell, this field indicates which UL carrier in the cell to transmit the PRACH according to Table 7.3.1.1.1-1; otherwise, this field is reserved

-	SS/PBCH index - 6 bits. If the value of the "Random Access Preamble index" is not all zeros, this field indicates the SS/PBCH that shall be used to determine the RACH occasion for the PRACH transmission; otherwise, this field is reserved.

-	PRACH Mask index - 4 bits. If the value of the "Random Access Preamble index" is not all zeros, this field indicates the RACH occasion associated with the SS/PBCH indicated by "SS/PBCH index" for the PRACH transmission, according to Clause 5.1.1 of [8, TS38.321]; otherwise, this field is reserved

-	Cell indicator - log2C+1 bits indicating the cell for the corresponding PRACH transmission if the UE is configured with higher layer parameter EarlyUlSyncConfig, where C is the number of candidate cells configured with higher layer parameter EarlyUlSyncConfig; 0 bit otherwise.

-	PRACH association indicator - 0 or 1 bit

-	1bit if the UE is provided with tag-Id2, and the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs.

-	This field indicates the PCI associated with the PRACH transmission if the UE is provided SSB-MTC-AddtionalPCI. The bit field index 0 of this field is mapped to the PCI of the serving cell, and the bit field index 1 of this field is mapped to the active additional PCI.

-	This field indicates the PL-RS for the PRACH transmission if the UE is not provided SSB-MTC-AddtionalPCI. The bit field index 0 of this field is mapped to the DL RS that the DM-RS of the PDCCH order is quasi-collocated with, and the bit field index 1 of this field is mapped to the SS/PBCH indicated by the SS/PBCH index field in this DCI format.

-	0 bit otherwise.

-	PRACH retransmission indicator - 0 or 1 bit

-	1bit if the UE is configured with higher layer parameter EarlyUlSyncConfig. This field indicates initial transmission or retransmission of PRACH according to Table 7.3.1.2.1-3 if the cell indicated by Cell indicator field is a candidate cell, and this field is reserved if the cell indicated by Cell indicator field is a serving cell but not a candidate cell.

-	0 bit otherwise.

-	Reserved bits - a number of bits as determined by the following:

-	12 bits for operation in a cell with shared spectrum channel access in frequency range 1 or when the DCI format is monitored in common search space for operation in a cell in frequency range 2-2, and if the UE is not configured with higher layer parameter EarlyUlSyncConfig;

-	11-log2C+1 bits for operation in a cell with shared spectrum channel access in frequency range 1 or when the DCI format is monitored in common search space for operation in a cell in frequency range 2-2, and if the UE is configured with higher layer parameter EarlyUlSyncConfig;

-	9-log2C+1 bits for operation in a cell without shared spectrum channel access in frequency range 1 or for operation in a cell in frequency range 2-1 or when the DCI format is monitored in UE-specific search space for operation in a cell in frequency range 2-2, and if the UE is configured with higher layer parameter EarlyUlSyncConfig;

-	10 bits otherwise.

Otherwise, all remaining fields are set as follows:

-	Time domain resource assignment - 4 bits as defined in Clause 5.1.2.1 of [6, TS 38.214]

-	VRB-to-PRB mapping - 1 bit according to Table 7.3.1.2.2-5

-	Modulation and coding scheme - 5 bits as defined in Clause 5.1.3 of [6, TS 38.214]

-	New data indicator - 1 bit

-	Redundancy version - 2 bits as defined in Table 7.3.1.1.1-2

-	HARQ process number - 4 bits

-	Downlink assignment index - 2 bits as defined in Clause 9.1.3 of [5, TS 38.213], as counter DAI

-	TPC command for scheduled PUCCH - 2 bits as defined in Clause 7.2.1 of [5, TS 38.213]

-	PUCCH resource indicator - 3 bits as defined in Clause 9.2.3 of [5, TS 38.213]

-	PDSCH-to-HARQ_feedback timing indicator - 3 bits as defined in Clause 9.2.3 of [5, TS38.213]

-	ChannelAccess-CPext - 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access in frequency range 1; 2 bits indicating channel access type as defined in Table 7.3.1.1.1-4B if ChannelAccessMode2-r17 is provided for operation in a cell in frequency range 2-2; 0 bits otherwise

-	Reserved bits - 2 bits when the DCI format is monitored in common search space for operation in a cell in frequency range 2-2 and the number of bits for the field of 'ChannelAccess-CPext' is 0; 0 bits otherwise

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by P-RNTI:

-	Short Messages Indicator - 2 bits according to Table 7.3.1.2.1-1.

-	Short Messages - 8 bits, according to Clause 6.5 of [9, TS38.331]. If only the scheduling information for Paging, and TRS availability indication if trs-ResourceSetConfig is configured, are carried, this bit field is reserved.

-	Frequency domain resource assignment - bits. If only the short message, and TRS availability indication if trs-ResourceSetConfig is configured, are carried, this bit field is reserved.

-	 is the size of CORESET 0

-	Time domain resource assignment - 4 bits as defined in Clause 5.1.2.1 of [6, TS38.214]. If only the short message, and TRS availability indication if trs-ResourceSetConfig is configured, are carried, this bit field is reserved.

-	VRB-to-PRB mapping - 1 bit according to Table 7.3.1.2.2-5. If only the short message, and TRS availability indication if trs-ResourceSetConfig is configured, are carried, this bit field is reserved.

-	Modulation and coding scheme - 5 bits as defined in Clause 5.1.3 of [6, TS38.214], using Table 5.1.3.1-1. If only the short message, and TRS availability indication if trs-ResourceSetConfig is configured, are carried, this bit field is reserved.

-	TB scaling - 2 bits as defined in Clause 5.1.3.2 of [6, TS38.214]. If only the short message, and TRS availability indication if trs-ResourceSetConfig is configured, are carried, this bit field is reserved.

-	TRS availability indication - 1, 2, 3, 4, 5, or 6 bits, where the number of bits is equal to one plus the highest value of all the indBitID(s) provided by the trs-ResourceSetConfig if configured; 0 bits otherwise.

-	Reserved bits - (8 - M) bits for operation in a cell with shared spectrum channel access in frequency range 1 or for operation in a cell in frequency range 2-2; (6 - M) bits for operation in a cell without shared spectrum channel access, where the value of M is the number of bits for the field of 'TRS availability indication' as defined above.

< Unchanged parts are omitted >

##### 7.3.1.2.2	Format 1_1

DCI format 1_1 is used for the scheduling of one or multiple PDSCH in one cell.

The following information is transmitted by means of the DCI format 1_1 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

< Unchanged parts are omitted >

-	HARQ-ACK retransmission indicator - 0 or 1 bit.

-	1 bit if higher layer parameter pdsch-HARQ-ACK-Retx is configured.

-	0 bit otherwise.

If the UE is configured with a PUCCH-SCell, pdsch-HARQ-ACK-Retx is replaced by pdsch-HARQ-ACK-RetxSecondaryPUCCHgroup for the secondary PUCCH group.

-	Antenna port(s) - 4, 5, 6, 7 or 8 bits as defined by Tables 7.3.1.2.2-1/2/3/4/7/8/9/10 and Tables 7.3.1.2.2-1A/2A/3A/4A/7A/8A/9A/10A, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. The antenna ports  shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4/7/8/9/10 or Tables 7.3.1.2.2-1A/2A/3A/4A/7A/8A/9A/10A. When a UE not configured with dl-OrJointTCI-StateList receives an activation command that maps at least one codepoint of DCI field 'Transmission Configuration Indication' to two TCI states, or when a UE configured with dl-OrJointTCI-StateList is having two indicated TCI states, the UE shall use Table 7.3.1.2.2-1A/2A/3A/4A/7A/8A/9A/10A; otherwise, it shall use Tables 7.3.1.2.2-1/2/3/4/7/8/9/10. The UE can receive an entry with DMRS ports equals to 1000, 1002, 1003 when the UE is not configured with dl-OrJointTCI-StateList and two TCI states are indicated in a codepoint of DCI field 'Transmission Configuration Indication', or when the UE configured with dl-OrJointTCI-StateList is having two indicated TCI states to be applied to PDSCH.

If a UE is configured with both dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB, the bitwidth of this field equals , where  is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeA and  is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeB. A number of  zeros are padded in the MSB of this field, if the mapping type of the PDSCH corresponds to the smaller value of  and .

-	Transmission configuration indication - 0 bit if higher layer parameter tci-PresentInDCI is not enabled; otherwise 3 bits as defined in Clause 5.1.5 of [6, TS38.214].

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part,

-	if the higher layer parameter tci-PresentInDCI is not enabled for the CORESET used for the PDCCH carrying the DCI format 1_1,

-	the UE assumes tci-PresentInDCI is not enabled for all CORESETs in the indicated bandwidth part;

-	otherwise,

-	the UE assumes tci-PresentInDCI is enabled for all CORESETs in the indicated bandwidth part.

< Unchanged parts are omitted >

Table 7.3.1.2.2-7: Antenna port(s) (1000 + DMRS port), dmrs-Type=1,
enhanced-dmrs-Type is configured, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 | 0 | 2 | 0,1,2,3,8 |
| 1 | 1 | 1 | 1 | 2 | 0,1,2,3,8,10 |
| 2 | 1 | 0,1 | 2 | 2 | 0,1,2,3,8,9,10 |
| 3 | 2 | 0 | 3 | 2 | 0,1,2,3,8,9,10,11 |
| 4 | 2 | 1 | 4-31 | Reserved | Reserved |
| 5 | 2 | 2 |  |  |  |
| 6 | 2 | 3 |  |  |  |
| 7 | 2 | 0,1 |  |  |  |
| 8 | 2 | 2,3 |  |  |  |
| 9 | 2 | 0-2 |  |  |  |
| 10 | 2 | 0-3 |  |  |  |
| 11 | 2 | 0,2 |  |  |  |
| 12 | 1 | 8 |  |  |  |
| 13 | 1 | 9 |  |  |  |
| 14 | 1 | 8,9 |  |  |  |
| 15 | 2 | 8 |  |  |  |
| 16 | 2 | 9 |  |  |  |
| 17 | 2 | 10 |  |  |  |
| 18 | 2 | 11 |  |  |  |
| 19 | 2 | 8,9 |  |  |  |
| 20 | 2 | 10,11 |  |  |  |
| 21 | 1 | 0,1,8 |  |  |  |
| 22 | 1 | 0,1,8,9 |  |  |  |
| 23 | 2 | 0,1,8 |  |  |  |
| 24 | 2 | 0,1,8,9 |  |  |  |
| 25 | 2 | 2,3,10 |  |  |  |
| 26 | 2 | 2,3,10,11 |  |  |  |
| 27-31 | Reserved | Reserved |  |  |  |

Table 7.3.1.2.2-7A: Antenna port(s) (1000 + DMRS port), dmrs-Type=1,
enhanced-dmrs-Type is configured, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 | 0 | 2 | 0,1,2,3,8 |
| 1 | 1 | 1 | 1 | 2 | 0,1,2,3,8,10 |
| 2 | 1 | 0,1 | 2 | 2 | 0,1,2,3,8,9,10 |
| 3 | 2 | 0 | 3 | 2 | 0,1,2,3,8,9,10,11 |
| 4 | 2 | 1 | 4-31 | Reserved | Reserved |
| 5 | 2 | 2 |  |  |  |
| 6 | 2 | 3 |  |  |  |
| 7 | 2 | 0,1 |  |  |  |
| 8 | 2 | 2,3 |  |  |  |
| 9 | 2 | 0-2 |  |  |  |
| 10 | 2 | 0-3 |  |  |  |
| 11 | 2 | 0,2 |  |  |  |
| 12 | 1 | 8 |  |  |  |
| 13 | 1 | 9 |  |  |  |
| 14 | 1 | 8,9 |  |  |  |
| 15 | 2 | 8 |  |  |  |
| 16 | 2 | 9 |  |  |  |
| 17 | 2 | 10 |  |  |  |
| 18 | 2 | 11 |  |  |  |
| 19 | 2 | 8,9 |  |  |  |
| 20 | 2 | 10,11 |  |  |  |
| 21 | 1 | 0,1,8 |  |  |  |
| 22 | 1 | 0,1,8,9 |  |  |  |
| 23 | 2 | 0,1,8 |  |  |  |
| 24 | 2 | 0,1,8,9 |  |  |  |
| 25 | 2 | 2,3,10 |  |  |  |
| 26 | 2 | 2,3,10,11 |  |  |  |
| 27 | 2 | 0,2,3 |  |  |  |
| 28-31 | Reserved | Reserved |  |  |  |

Table 7.3.1.2.2-8: Antenna port(s) (1000 + DMRS port), dmrs-Type=1,
enhanced-dmrs-Type is configured, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | 0 | 2 | 0,1,2,3,8 | 1 |
| 1 | 1 | 1 | 1 | 1 | 2 | 0,1,2,3,8,10 | 1 |
| 2 | 1 | 0,1 | 1 | 2 | 2 | 0,1,2,3,8,9,10 | 1 |
| 3 | 2 | 0 | 1 | 3 | 2 | 0,1,2,3,8,9,10,11 | 1 |
| 4 | 2 | 1 | 1 | 4 | 2 | 0-4 | 2 |
| 5 | 2 | 2 | 1 | 5 | 2 | 0,1,2,3,4,6 | 2 |
| 6 | 2 | 3 | 1 | 6 | 2 | 0,1,2,3,4,5,6 | 2 |
| 7 | 2 | 0,1 | 1 | 7 | 2 | 0,1,2,3,4,5,6,7 | 2 |
| 8 | 2 | 2,3 | 1 | 8 | 1 | 0,1,4,5,8 | 2 |
| 9 | 2 | 0-2 | 1 | 9 | 1 | 0,1,4,5,8,12 | 2 |
| 10 | 2 | 0-3 | 1 | 10 | 1 | 0,1,4,5,8,9,12 | 2 |
| 11 | 2 | 0,2 | 1 | 11 | 1 | 0,1,4,5,8,9,12,13 | 2 |
| 12 | 1 | 8 | 1 | 12 | 2 | 0,1,4,5,8 | 2 |
| 13 | 1 | 9 | 1 | 13 | 2 | 0,1,4,5,8,12 | 2 |
| 14 | 1 | 8,9 | 1 | 14 | 2 | 0,1,4,5,8,9,12 | 2 |
| 15 | 2 | 8 | 1 | 15 | 2 | 0,1,4,5,8,9,12,13 | 2 |
| 16 | 2 | 9 | 1 | 16~127 | Reserved | Reserved | Reserved |
| 17 | 2 | 10 | 1 |  |  |  |  |
| 18 | 2 | 11 | 1 |  |  |  |  |
| 19 | 2 | 8,9 | 1 |  |  |  |  |
| 20 | 2 | 10,11 | 1 |  |  |  |  |
| 21 | 1 | 0,1,8 | 1 |  |  |  |  |
| 22 | 1 | 0,1,8,9 | 1 |  |  |  |  |
| 23 | 2 | 0,1,8 | 1 |  |  |  |  |
| 24 | 2 | 0,1,8,9 | 1 |  |  |  |  |
| 25 | 2 | 2,3,10 | 1 |  |  |  |  |
| 26 | 2 | 2,3,10,11 | 1 |  |  |  |  |
| 27 | 2 | 0 | 2 |  |  |  |  |
| 28 | 2 | 1 | 2 |  |  |  |  |
| 29 | 2 | 2 | 2 |  |  |  |  |
| 30 | 2 | 3 | 2 |  |  |  |  |
| 31 | 2 | 4 | 2 |  |  |  |  |
| 32 | 2 | 5 | 2 |  |  |  |  |
| 33 | 2 | 6 | 2 |  |  |  |  |
| 34 | 2 | 7 | 2 |  |  |  |  |
| 35 | 2 | 0,1 | 2 |  |  |  |  |
| 36 | 2 | 2,3 | 2 |  |  |  |  |
| 37 | 2 | 4,5 | 2 |  |  |  |  |
| 38 | 2 | 6,7 | 2 |  |  |  |  |
| 39 | 2 | 0,4 | 2 |  |  |  |  |
| 40 | 2 | 2,6 | 2 |  |  |  |  |
| 41 | 2 | 0,1,4 | 2 |  |  |  |  |
| 42 | 2 | 2,3,6 | 2 |  |  |  |  |
| 43 | 2 | 0,1,4,5 | 2 |  |  |  |  |
| 44 | 2 | 2,3,6,7 | 2 |  |  |  |  |
| 45 | 2 | 0,2,4,6 | 2 |  |  |  |  |
| 46 | 2 | 8 | 2 |  |  |  |  |
| 47 | 2 | 9 | 2 |  |  |  |  |
| 48 | 2 | 10 | 2 |  |  |  |  |
| 49 | 2 | 11 | 2 |  |  |  |  |
| 50 | 2 | 12 | 2 |  |  |  |  |
| 51 | 2 | 13 | 2 |  |  |  |  |
| 52 | 2 | 14 | 2 |  |  |  |  |
| 53 | 2 | 15 | 2 |  |  |  |  |
| 54 | 2 | 8,9 | 2 |  |  |  |  |
| 55 | 2 | 10,11 | 2 |  |  |  |  |
| 56 | 2 | 12,13 | 2 |  |  |  |  |
| 57 | 2 | 14,15 | 2 |  |  |  |  |
| 58 | 2 | 0,1,8 | 2 |  |  |  |  |
| 59 | 2 | 0,1,8,9 | 2 |  |  |  |  |
| 60 | 2 | 4,5,12 | 2 |  |  |  |  |
| 61 | 2 | 4,5,12,13 | 2 |  |  |  |  |
| 62 | 2 | 2,3,10 | 2 |  |  |  |  |
| 63 | 2 | 2,3,10,11 | 2 |  |  |  |  |
| 64 | 2 | 6,7,14 | 2 |  |  |  |  |
| 65 | 2 | 6,7,14,15 | 2 |  |  |  |  |
| 66-127 | Reserved | Reserved | Reserved |  |  |  |  |

Table 7.3.1.2.2-8A: Antenna port(s) (1000 + DMRS port), dmrs-Type=1,
enhanced-dmrs-Type is configured, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | 0 | 2 | 0,1,2,3,8 | 1 |
| 1 | 1 | 1 | 1 | 1 | 2 | 0,1,2,3,8,10 | 1 |
| 2 | 1 | 0,1 | 1 | 2 | 2 | 0,1,2,3,8,9,10 | 1 |
| 3 | 2 | 0 | 1 | 3 | 2 | 0,1,2,3,8,9,10,11 | 1 |
| 4 | 2 | 1 | 1 | 4 | 2 | 0-4 | 2 |
| 5 | 2 | 2 | 1 | 5 | 2 | 0,1,2,3,4,6 | 2 |
| 6 | 2 | 3 | 1 | 6 | 2 | 0,1,2,3,4,5,6 | 2 |
| 7 | 2 | 0,1 | 1 | 7 | 2 | 0,1,2,3,4,5,6,7 | 2 |
| 8 | 2 | 2,3 | 1 | 8 | 1 | 0,1,4,5,8 | 2 |
| 9 | 2 | 0-2 | 1 | 9 | 1 | 0,1,4,5,8,12 | 2 |
| 10 | 2 | 0-3 | 1 | 10 | 1 | 0,1,4,5,8,9,12 | 2 |
| 11 | 2 | 0,2 | 1 | 11 | 1 | 0,1,4,5,8,9,12,13 | 2 |
| 12 | 1 | 8 | 1 | 12 | 2 | 0,1,4,5,8 | 2 |
| 13 | 1 | 9 | 1 | 13 | 2 | 0,1,4,5,8,12 | 2 |
| 14 | 1 | 8,9 | 1 | 14 | 2 | 0,1,4,5,8,9,12 | 2 |
| 15 | 2 | 8 | 1 | 15 | 2 | 0,1,4,5,8,9,12,13 | 2 |
| 16 | 2 | 9 | 1 | 16~127 | Reserved | Reserved | Reserved |
| 17 | 2 | 10 | 1 |  |  |  |  |
| 18 | 2 | 11 | 1 |  |  |  |  |
| 19 | 2 | 8,9 | 1 |  |  |  |  |
| 20 | 2 | 10,11 | 1 |  |  |  |  |
| 21 | 1 | 0,1,8 | 1 |  |  |  |  |
| 22 | 1 | 0,1,8,9 | 1 |  |  |  |  |
| 23 | 2 | 0,1,8 | 1 |  |  |  |  |
| 24 | 2 | 0,1,8,9 | 1 |  |  |  |  |
| 25 | 2 | 2,3,10 | 1 |  |  |  |  |
| 26 | 2 | 2,3,10,11 | 1 |  |  |  |  |
| 27 | 2 | 0 | 2 |  |  |  |  |
| 28 | 2 | 1 | 2 |  |  |  |  |
| 29 | 2 | 2 | 2 |  |  |  |  |
| 30 | 2 | 3 | 2 |  |  |  |  |
| 31 | 2 | 4 | 2 |  |  |  |  |
| 32 | 2 | 5 | 2 |  |  |  |  |
| 33 | 2 | 6 | 2 |  |  |  |  |
| 34 | 2 | 7 | 2 |  |  |  |  |
| 35 | 2 | 0,1 | 2 |  |  |  |  |
| 36 | 2 | 2,3 | 2 |  |  |  |  |
| 37 | 2 | 4,5 | 2 |  |  |  |  |
| 38 | 2 | 6,7 | 2 |  |  |  |  |
| 39 | 2 | 0,4 | 2 |  |  |  |  |
| 40 | 2 | 2,6 | 2 |  |  |  |  |
| 41 | 2 | 0,1,4 | 2 |  |  |  |  |
| 42 | 2 | 2,3,6 | 2 |  |  |  |  |
| 43 | 2 | 0,1,4,5 | 2 |  |  |  |  |
| 44 | 2 | 2,3,6,7 | 2 |  |  |  |  |
| 45 | 2 | 0,2,4,6 | 2 |  |  |  |  |
| 46 | 2 | 8 | 2 |  |  |  |  |
| 47 | 2 | 9 | 2 |  |  |  |  |
| 48 | 2 | 10 | 2 |  |  |  |  |
| 49 | 2 | 11 | 2 |  |  |  |  |
| 50 | 2 | 12 | 2 |  |  |  |  |
| 51 | 2 | 13 | 2 |  |  |  |  |
| 52 | 2 | 14 | 2 |  |  |  |  |
| 53 | 2 | 15 | 2 |  |  |  |  |
| 54 | 2 | 8,9 | 2 |  |  |  |  |
| 55 | 2 | 10,11 | 2 |  |  |  |  |
| 56 | 2 | 12,13 | 2 |  |  |  |  |
| 57 | 2 | 14,15 | 2 |  |  |  |  |
| 58 | 2 | 0,1,8 | 2 |  |  |  |  |
| 59 | 2 | 0,1,8,9 | 2 |  |  |  |  |
| 60 | 2 | 4,5,12 | 2 |  |  |  |  |
| 61 | 2 | 4,5,12,13 | 2 |  |  |  |  |
| 62 | 2 | 2,3,10 | 2 |  |  |  |  |
| 63 | 2 | 2,3,10,11 | 2 |  |  |  |  |
| 64 | 2 | 6,7,14 | 2 |  |  |  |  |
| 65 | 2 | 6,7,14,15 | 2 |  |  |  |  |
| 66 | 2 | 0,2,3 | 1 |  |  |  |  |
| 67-127 | Reserved | Reserved | Reserved |  |  |  |  |

Table 7.3.1.2.2-9: Antenna port(s) (1000 + DMRS port), dmrs-Type=2,
enhanced-dmrs-Type is configured, maxLength=1

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  | Two codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 | 0 | 3 | 0-4 |
| 1 | 1 | 1 | 1 | 3 | 0-5 |
| 2 | 1 | 0,1 | 2 | 2 | 0,1,2,3,12 |
| 3 | 2 | 0 | 3 | 2 | 0,1,2,3,12,14 |
| 4 | 2 | 1 | 4 | 2 | 0-3,12-14 |
| 5 | 2 | 2 | 5 | 2 | 0-3,12-15 |
| 6 | 2 | 3 | 6 | 3 | 0,1,2,3,12 |
| 7 | 2 | 0,1 | 7 | 3 | 0,1,2,3,12,14 |
| 8 | 2 | 2,3 | 8 | 3 | 0-3,12-14 |
| 9 | 2 | 0-2 | 9 | 3 | 0-3,12-15 |
| 10 | 2 | 0-3 | 10~63 | Reserved | Reserved |
| 11 | 3 | 0 |  |  |  |
| 12 | 3 | 1 |  |  |  |
| 13 | 3 | 2 |  |  |  |
| 14 | 3 | 3 |  |  |  |
| 15 | 3 | 4 |  |  |  |
| 16 | 3 | 5 |  |  |  |
| 17 | 3 | 0,1 |  |  |  |
| 18 | 3 | 2,3 |  |  |  |
| 19 | 3 | 4,5 |  |  |  |
| 20 | 3 | 0-2 |  |  |  |
| 21 | 3 | 3-5 |  |  |  |
| 22 | 3 | 0-3 |  |  |  |
| 23 | 2 | 0,2 |  |  |  |
| 24 | 1 | 12 |  |  |  |
| 25 | 1 | 13 |  |  |  |
| 26 | 1 | 12,13 |  |  |  |
| 27 | 2 | 12 |  |  |  |
| 28 | 2 | 13 |  |  |  |
| 29 | 2 | 14 |  |  |  |
| 30 | 2 | 15 |  |  |  |
| 31 | 2 | 12,13 |  |  |  |
| 32 | 2 | 14,15 |  |  |  |
| 33 | 3 | 12 |  |  |  |
| 34 | 3 | 13 |  |  |  |
| 35 | 3 | 14 |  |  |  |
| 36 | 3 | 15 |  |  |  |
| 37 | 3 | 16 |  |  |  |
| 38 | 3 | 17 |  |  |  |
| 39 | 3 | 12,13 |  |  |  |
| 40 | 3 | 14,15 |  |  |  |
| 41 | 3 | 16,17 |  |  |  |
| 42 | 1 | 0,1,12 |  |  |  |
| 43 | 1 | 0,1,12,13 |  |  |  |
| 44 | 2 | 0,1,12 |  |  |  |
| 45 | 2 | 0,1,12,13 |  |  |  |
| 46 | 2 | 2,3,14 |  |  |  |
| 47 | 2 | 2,3,14,15 |  |  |  |
| 48 | 3 | 0,1,12 |  |  |  |
| 49 | 3 | 0,1,12,13 |  |  |  |
| 50 | 3 | 2,3,14 |  |  |  |
| 51 | 3 | 2,3,14,15 |  |  |  |
| 52 | 3 | 4,5,16 |  |  |  |
| 53 | 3 | 4,5,16,17 |  |  |  |
| 54-63 | Reserved | Reserved |  |  |  |

Table 7.3.1.2.2-9A: Antenna port(s) (1000 + DMRS port), dmrs-Type=2,
enhanced-dmrs-Type is configured, maxLength=1

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  | Two codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 | 0 | 3 | 0-4 |
| 1 | 1 | 1 | 1 | 3 | 0-5 |
| 2 | 1 | 0,1 | 2 | 2 | 0,1,2,3,12 |
| 3 | 2 | 0 | 3 | 2 | 0,1,2,3,12,14 |
| 4 | 2 | 1 | 4 | 2 | 0-3,12-14 |
| 5 | 2 | 2 | 5 | 2 | 0-3,12-15 |
| 6 | 2 | 3 | 6 | 3 | 0,1,2,3,12 |
| 7 | 2 | 0,1 | 7 | 3 | 0,1,2,3,12,14 |
| 8 | 2 | 2,3 | 8 | 3 | 0-3,12-14 |
| 9 | 2 | 0-2 | 9 | 3 | 0-3,12-15 |
| 10 | 2 | 0-3 | 10~63 | Reserved | Reserved |
| 11 | 3 | 0 |  |  |  |
| 12 | 3 | 1 |  |  |  |
| 13 | 3 | 2 |  |  |  |
| 14 | 3 | 3 |  |  |  |
| 15 | 3 | 4 |  |  |  |
| 16 | 3 | 5 |  |  |  |
| 17 | 3 | 0,1 |  |  |  |
| 18 | 3 | 2,3 |  |  |  |
| 19 | 3 | 4,5 |  |  |  |
| 20 | 3 | 0-2 |  |  |  |
| 21 | 3 | 3-5 |  |  |  |
| 22 | 3 | 0-3 |  |  |  |
| 23 | 2 | 0,2 |  |  |  |
| 24 | 1 | 12 |  |  |  |
| 25 | 1 | 13 |  |  |  |
| 26 | 1 | 12,13 |  |  |  |
| 27 | 2 | 12 |  |  |  |
| 28 | 2 | 13 |  |  |  |
| 29 | 2 | 14 |  |  |  |
| 30 | 2 | 15 |  |  |  |
| 31 | 2 | 12,13 |  |  |  |
| 32 | 2 | 14,15 |  |  |  |
| 33 | 3 | 12 |  |  |  |
| 34 | 3 | 13 |  |  |  |
| 35 | 3 | 14 |  |  |  |
| 36 | 3 | 15 |  |  |  |
| 37 | 3 | 16 |  |  |  |
| 38 | 3 | 17 |  |  |  |
| 39 | 3 | 12,13 |  |  |  |
| 40 | 3 | 14,15 |  |  |  |
| 41 | 3 | 16,17 |  |  |  |
| 42 | 1 | 0,1,12 |  |  |  |
| 43 | 1 | 0,1,12,13 |  |  |  |
| 44 | 2 | 0,1,12 |  |  |  |
| 45 | 2 | 0,1,12,13 |  |  |  |
| 46 | 2 | 2,3,14 |  |  |  |
| 47 | 2 | 2,3,14,15 |  |  |  |
| 48 | 3 | 0,1,12 |  |  |  |
| 49 | 3 | 0,1,12,13 |  |  |  |
| 50 | 3 | 2,3,14 |  |  |  |
| 51 | 3 | 2,3,14,15 |  |  |  |
| 52 | 3 | 4,5,16 |  |  |  |
| 53 | 3 | 4,5,16,17 |  |  |  |
| 54 | 2 | 0,2,3 |  |  |  |
| 55-63 | Reserved | Reserved |  |  |  |

Table 7.3.1.2.2-10: Antenna port(s) (1000 + DMRS port), dmrs-Type=2,
enhanced-dmrs-Type is configured, maxLength=2

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | 0 | 3 | 0-4 | 1 |
| 1 | 1 | 1 | 1 | 1 | 3 | 0-5 | 1 |
| 2 | 1 | 0,1 | 1 | 2 | 2 | 0,1,2,3,12 | 1 |
| 3 | 2 | 0 | 1 | 3 | 2 | 0-3,12,14 | 1 |
| 4 | 2 | 1 | 1 | 4 | 2 | 0-3,12-14 | 1 |
| 5 | 2 | 2 | 1 | 5 | 2 | 0-3,12-15 | 1 |
| 6 | 2 | 3 | 1 | 6 | 3 | 0,1,2,3,12 | 1 |
| 7 | 2 | 0,1 | 1 | 7 | 3 | 0-3,12,14 | 1 |
| 8 | 2 | 2,3 | 1 | 8 | 3 | 0-3,12-14 | 1 |
| 9 | 2 | 0-2 | 1 | 9 | 3 | 0-3,12-15 | 1 |
| 10 | 2 | 0-3 | 1 | 10 | 2 | 0,1,2,3,6 | 2 |
| 11 | 3 | 0 | 1 | 11 | 2 | 0,1,2,3,6,8 | 2 |
| 12 | 3 | 1 | 1 | 12 | 2 | 0,1,2,3,6,7,8 | 2 |
| 13 | 3 | 2 | 1 | 13 | 2 | 0,1,2,3,6,7,8,9 | 2 |
| 14 | 3 | 3 | 1 | 14 | 1 | 0,1,6,7,12 | 2 |
| 15 | 3 | 4 | 1 | 15 | 1 | 0,1,6,7,12,18 | 2 |
| 16 | 3 | 5 | 1 | 16 | 1 | 0,1,6,7,12,13,18 | 2 |
| 17 | 3 | 0,1 | 1 | 17 | 1 | 0,1,6,7,12,13,18,19 | 2 |
| 18 | 3 | 2,3 | 1 | 18 | 2 | 0,1,6,7,12 | 2 |
| 19 | 3 | 4,5 | 1 | 19 | 2 | 0,1,6,7,12,18 | 2 |
| 20 | 3 | 0-2 | 1 | 20 | 2 | 0,1,6,7,12,13,18 | 2 |
| 21 | 3 | 3-5 | 1 | 21 | 2 | 0,1,6,7,12,13,18,19 | 2 |
| 22 | 3 | 0-3 | 1 | 22 | 3 | 0,1,6,7,12 | 2 |
| 23 | 2 | 0,2 | 1 | 23 | 3 | 0,1,6,7,12,18 | 2 |
| 24 | 1 | 12 | 1 | 24 | 3 | 0,1,6,7,12,13,18 | 2 |
| 25 | 1 | 13 | 1 | 25 | 3 | 0,1,6,7,12,13,18,19 | 2 |
| 26 | 1 | 12,13 | 1 | 26~255 | Reserved | Reserved | Reserved |
| 27 | 2 | 12 | 1 |  |  |  |  |
| 28 | 2 | 13 | 1 |  |  |  |  |
| 29 | 2 | 14 | 1 |  |  |  |  |
| 30 | 2 | 15 | 1 |  |  |  |  |
| 31 | 2 | 12,13 | 1 |  |  |  |  |
| 32 | 2 | 14,15 | 1 |  |  |  |  |
| 33 | 3 | 12 | 1 |  |  |  |  |
| 34 | 3 | 13 | 1 |  |  |  |  |
| 35 | 3 | 14 | 1 |  |  |  |  |
| 36 | 3 | 15 | 1 |  |  |  |  |
| 37 | 3 | 16 | 1 |  |  |  |  |
| 38 | 3 | 17 | 1 |  |  |  |  |
| 39 | 3 | 12,13 | 1 |  |  |  |  |
| 40 | 3 | 14,15 | 1 |  |  |  |  |
| 41 | 3 | 16,17 | 1 |  |  |  |  |
| 42 | 1 | 0,1,12 | 1 |  |  |  |  |
| 43 | 1 | 0,1,12,13 | 1 |  |  |  |  |
| 44 | 2 | 0,1,12 | 1 |  |  |  |  |
| 45 | 2 | 0,1,12,13 | 1 |  |  |  |  |
| 46 | 2 | 2,3,14 | 1 |  |  |  |  |
| 47 | 2 | 2,3,14,15 | 1 |  |  |  |  |
| 48 | 3 | 0,1,12 | 1 |  |  |  |  |
| 49 | 3 | 0,1,12,13 | 1 |  |  |  |  |
| 50 | 3 | 2,3,14 | 1 |  |  |  |  |
| 51 | 3 | 2,3,14,15 | 1 |  |  |  |  |
| 52 | 3 | 4,5,16 | 1 |  |  |  |  |
| 53 | 3 | 4,5,16,17 | 1 |  |  |  |  |
| 54 | 3 | 0 | 2 |  |  |  |  |
| 55 | 3 | 1 | 2 |  |  |  |  |
| 56 | 3 | 2 | 2 |  |  |  |  |
| 57 | 3 | 3 | 2 |  |  |  |  |
| 58 | 3 | 4 | 2 |  |  |  |  |
| 59 | 3 | 5 | 2 |  |  |  |  |
| 60 | 3 | 6 | 2 |  |  |  |  |
| 61 | 3 | 7 | 2 |  |  |  |  |
| 62 | 3 | 8 | 2 |  |  |  |  |
| 63 | 3 | 9 | 2 |  |  |  |  |
| 64 | 3 | 10 | 2 |  |  |  |  |
| 65 | 3 | 11 | 2 |  |  |  |  |
| 66 | 3 | 0,1 | 2 |  |  |  |  |
| 67 | 3 | 2,3 | 2 |  |  |  |  |
| 68 | 3 | 4,5 | 2 |  |  |  |  |
| 69 | 3 | 6,7 | 2 |  |  |  |  |
| 70 | 3 | 8,9 | 2 |  |  |  |  |
| 71 | 3 | 10,11 | 2 |  |  |  |  |
| 72 | 3 | 0,1,6 | 2 |  |  |  |  |
| 73 | 3 | 2,3,8 | 2 |  |  |  |  |
| 74 | 3 | 4,5,10 | 2 |  |  |  |  |
| 75 | 3 | 0,1,6,7 | 2 |  |  |  |  |
| 76 | 3 | 2,3,8,9 | 2 |  |  |  |  |
| 77 | 3 | 4,5,10,11 | 2 |  |  |  |  |
| 78 | 1 | 0 | 2 |  |  |  |  |
| 79 | 1 | 1 | 2 |  |  |  |  |
| 80 | 1 | 6 | 2 |  |  |  |  |
| 81 | 1 | 7 | 2 |  |  |  |  |
| 82 | 1 | 0,1 | 2 |  |  |  |  |
| 83 | 1 | 6,7 | 2 |  |  |  |  |
| 84 | 2 | 0,1 | 2 |  |  |  |  |
| 85 | 2 | 2,3 | 2 |  |  |  |  |
| 86 | 2 | 6,7 | 2 |  |  |  |  |
| 87 | 2 | 8,9 | 2 |  |  |  |  |
| 88 | 3 | 12 | 2 |  |  |  |  |
| 89 | 3 | 13 | 2 |  |  |  |  |
| 90 | 3 | 14 | 2 |  |  |  |  |
| 91 | 3 | 15 | 2 |  |  |  |  |
| 92 | 3 | 16 | 2 |  |  |  |  |
| 93 | 3 | 17 | 2 |  |  |  |  |
| 94 | 3 | 18 | 2 |  |  |  |  |
| 95 | 3 | 19 | 2 |  |  |  |  |
| 96 | 3 | 20 | 2 |  |  |  |  |
| 97 | 3 | 21 | 2 |  |  |  |  |
| 98 | 3 | 22 | 2 |  |  |  |  |
| 99 | 3 | 23 | 2 |  |  |  |  |
| 100 | 3 | 12,13 | 2 |  |  |  |  |
| 101 | 3 | 14,15 | 2 |  |  |  |  |
| 102 | 3 | 16,17 | 2 |  |  |  |  |
| 103 | 3 | 18,19 | 2 |  |  |  |  |
| 104 | 3 | 20,21 | 2 |  |  |  |  |
| 105 | 3 | 22,23 | 2 |  |  |  |  |
| 106 | 1 | 12 | 2 |  |  |  |  |
| 107 | 1 | 13 | 2 |  |  |  |  |
| 108 | 1 | 18 | 2 |  |  |  |  |
| 109 | 1 | 19 | 2 |  |  |  |  |
| 110 | 1 | 12,13 | 2 |  |  |  |  |
| 111 | 1 | 18,19 | 2 |  |  |  |  |
| 112 | 2 | 12,13 | 2 |  |  |  |  |
| 113 | 2 | 14,15 | 2 |  |  |  |  |
| 114 | 2 | 18,19 | 2 |  |  |  |  |
| 115 | 2 | 20,21 | 2 |  |  |  |  |
| 116 | 2 | 0,1,12 | 2 |  |  |  |  |
| 117 | 2 | 0,1,12,13 | 2 |  |  |  |  |
| 118 | 2 | 6,7,18 | 2 |  |  |  |  |
| 119 | 2 | 6,7,18,19 | 2 |  |  |  |  |
| 120 | 2 | 2,3,14 | 2 |  |  |  |  |
| 121 | 2 | 2,3,14,15 | 2 |  |  |  |  |
| 122 | 2 | 8,9,20 | 2 |  |  |  |  |
| 123 | 2 | 8,9,20,21 | 2 |  |  |  |  |
| 124 | 3 | 0,1,12 | 2 |  |  |  |  |
| 125 | 3 | 0,1,12,13 | 2 |  |  |  |  |
| 126 | 3 | 6,7,18 | 2 |  |  |  |  |
| 127 | 3 | 6,7,18,19 | 2 |  |  |  |  |
| 128 | 3 | 2,3,14 | 2 |  |  |  |  |
| 129 | 3 | 2,3,14,15 | 2 |  |  |  |  |
| 130 | 3 | 8,9,20 | 2 |  |  |  |  |
| 131 | 3 | 8,9,20,21 | 2 |  |  |  |  |
| 132 | 3 | 4,5,16 | 2 |  |  |  |  |
| 133 | 3 | 4,5,16,17 | 2 |  |  |  |  |
| 134 | 3 | 10,11,22 | 2 |  |  |  |  |
| 135 | 3 | 10,11,22,23 | 2 |  |  |  |  |
| 136-255 | Reserved | Reserved | Reserved |  |  |  |  |

Table 7.3.1.2.2-10A: Antenna port(s) (1000 + DMRS port), dmrs-Type=2,
enhanced-dmrs-Type is configured, maxLength=2

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data |  | DMRS port(s) |  | Number of front-load symbols |  | Value |  | Number of DMRS CDM group(s) without data |  | DMRS port(s) |  | Number of front-load symbols |  |
| 0 | 1 |  | 0 |  | 1 |  | 0 |  | 3 |  | 0-4 |  | 1 |  |
| 1 | 1 |  | 1 |  | 1 |  | 1 |  | 3 |  | 0-5 |  | 1 |  |
| 2 | 1 |  | 0,1 |  | 1 |  | 2 |  | 2 |  | 0,1,2,3,12 |  | 1 |  |
| 3 | 2 |  | 0 |  | 1 |  | 3 |  | 2 |  | 0-3,12,14 |  | 1 |  |
| 4 | 2 |  | 1 |  | 1 |  | 4 |  | 2 |  | 0-3,12-14 |  | 1 |  |
| 5 | 2 |  | 2 |  | 1 |  | 5 |  | 2 |  | 0-3,12-15 |  | 1 |  |
| 6 | 2 |  | 3 |  | 1 |  | 6 |  | 3 |  | 0,1,2,3,12 |  | 1 |  |
| 7 | 2 |  | 0,1 |  | 1 |  | 7 |  | 3 |  | 0-3,12,14 |  | 1 |  |
| 8 | 2 |  | 2,3 |  | 1 |  | 8 |  | 3 |  | 0-3,12-14 |  | 1 |  |
| 9 | 2 |  | 0-2 |  | 1 |  | 9 |  | 3 |  | 0-3,12-15 |  | 1 |  |
| 10 | 2 |  | 0-3 |  | 1 |  | 10 |  | 2 |  | 0,1,2,3,6 |  | 2 |  |
| 11 | 3 |  | 0 |  | 1 |  | 11 |  | 2 |  | 0,1,2,3,6,8 |  | 2 |  |
| 12 | 3 |  | 1 |  | 1 |  | 12 |  | 2 |  | 0,1,2,3,6,7,8 |  | 2 |  |
| 13 | 3 |  | 2 |  | 1 |  | 13 |  | 2 |  | 0,1,2,3,6,7,8,9 |  | 2 |  |
| 14 | 3 |  | 3 |  | 1 |  | 14 |  | 1 |  | 0,1,6,7,12 |  | 2 |  |
| 15 | 3 |  | 4 |  | 1 |  | 15 |  | 1 |  | 0,1,6,7,12,18 |  | 2 |  |
| 16 | 3 |  | 5 |  | 1 |  | 16 |  | 1 |  | 0,1,6,7,12,13,18 |  | 2 |  |
| 17 | 3 |  | 0,1 |  | 1 |  | 17 |  | 1 |  | 0,1,6,7,12,13,18,19 |  | 2 |  |
| 18 | 3 |  | 2,3 |  | 1 |  | 18 |  | 2 |  | 0,1,6,7,12 |  | 2 |  |
| 19 | 3 |  | 4,5 |  | 1 |  | 19 |  | 2 |  | 0,1,6,7,12,18 |  | 2 |  |
| 20 | 3 |  | 0-2 |  | 1 |  | 20 |  | 2 |  | 0,1,6,7,12,13,18 |  | 2 |  |
| 21 | 3 |  | 3-5 |  | 1 |  | 21 |  | 2 |  | 0,1,6,7,12,13,18,19 |  | 2 |  |
| 22 | 3 |  | 0-3 |  | 1 |  | 22 |  | 3 |  | 0,1,6,7,12 |  | 2 |  |
| 23 | 2 |  | 0,2 |  | 1 |  | 23 |  | 3 |  | 0,1,6,7,12,18 |  | 2 |  |
| 24 | 1 |  | 12 |  | 1 |  | 24 |  | 3 |  | 0,1,6,7,12,13,18 |  | 2 |  |
| 25 | 1 |  | 13 |  | 1 |  | 25 |  | 3 |  | 0,1,6,7,12,13,18,19 |  | 2 |  |
| 26 | 1 |  | 12,13 |  | 1 |  | 26~255 |  | Reserved |  | Reserved |  | Reserved |  |
| 27 | 2 |  | 12 |  | 1 |  |  |  |  |  |  |  |  |  |
| 28 | 2 |  | 13 |  | 1 |  |  |  |  |  |  |  |  |  |
| 29 | 2 |  | 14 |  | 1 |  |  |  |  |  |  |  |  |  |
| 30 | 2 |  | 15 |  | 1 |  |  |  |  |  |  |  |  |  |
| 31 | 2 |  | 12,13 |  | 1 |  |  |  |  |  |  |  |  |  |
| 32 | 2 |  | 14,15 |  | 1 |  |  |  |  |  |  |  |  |  |
| 33 | 3 |  | 12 |  | 1 |  |  |  |  |  |  |  |  |  |
| 34 | 3 |  | 13 |  | 1 |  |  |  |  |  |  |  |  |  |
| 35 | 3 |  | 14 |  | 1 |  |  |  |  |  |  |  |  |  |
| 36 | 3 |  | 15 |  | 1 |  |  |  |  |  |  |  |  |  |
| 37 | 3 |  | 16 |  | 1 |  |  |  |  |  |  |  |  |  |
| 38 | 3 |  | 17 |  | 1 |  |  |  |  |  |  |  |  |  |
| 39 | 3 |  | 12,13 |  | 1 |  |  |  |  |  |  |  |  |  |
| 40 | 3 |  | 14,15 |  | 1 |  |  |  |  |  |  |  |  |  |
| 41 | 3 |  | 16,17 |  | 1 |  |  |  |  |  |  |  |  |  |
| 42 | 1 |  | 0,1,12 |  | 1 |  |  |  |  |  |  |  |  |  |
| 43 | 1 |  | 0,1,12,13 |  | 1 |  |  |  |  |  |  |  |  |  |
| 44 | 2 |  | 0,1,12 |  | 1 |  |  |  |  |  |  |  |  |  |
| 45 | 2 |  | 0,1,12,13 |  | 1 |  |  |  |  |  |  |  |  |  |
| 46 | 2 |  | 2,3,14 |  | 1 |  |  |  |  |  |  |  |  |  |
| 47 | 2 |  | 2,3,14,15 |  | 1 |  |  |  |  |  |  |  |  |  |
| 48 | 3 |  | 0,1,12 |  | 1 |  |  |  |  |  |  |  |  |  |
| 49 | 3 |  | 0,1,12,13 |  | 1 |  |  |  |  |  |  |  |  |  |
| 50 | 3 |  | 2,3,14 |  | 1 |  |  |  |  |  |  |  |  |  |
| 51 | 3 |  | 2,3,14,15 |  | 1 |  |  |  |  |  |  |  |  |  |
| 52 | 3 |  | 4,5,16 |  | 1 |  |  |  |  |  |  |  |  |  |
| 53 | 3 |  | 4,5,16,17 |  | 1 |  |  |  |  |  |  |  |  |  |
| 54 | 3 |  | 0 |  | 2 |  |  |  |  |  |  |  |  |  |
| 55 | 3 |  | 1 |  | 2 |  |  |  |  |  |  |  |  |  |
| 56 | 3 |  | 2 |  | 2 |  |  |  |  |  |  |  |  |  |
| 57 | 3 |  | 3 |  | 2 |  |  |  |  |  |  |  |  |  |
| 58 | 3 |  | 4 |  | 2 |  |  |  |  |  |  |  |  |  |
| 59 | 3 |  | 5 |  | 2 |  |  |  |  |  |  |  |  |  |
| 60 | 3 |  | 6 |  | 2 |  |  |  |  |  |  |  |  |  |
| 61 | 3 |  | 7 |  | 2 |  |  |  |  |  |  |  |  |  |
| 62 | 3 |  | 8 |  | 2 |  |  |  |  |  |  |  |  |  |
| 63 | 3 |  | 9 |  | 2 |  |  |  |  |  |  |  |  |  |
| 64 | 3 |  | 10 |  | 2 |  |  |  |  |  |  |  |  |  |
| 65 | 3 |  | 11 |  | 2 |  |  |  |  |  |  |  |  |  |
| 66 | 3 |  | 0,1 |  | 2 |  |  |  |  |  |  |  |  |  |
| 67 | 3 |  | 2,3 |  | 2 |  |  |  |  |  |  |  |  |  |
| 68 | 3 |  | 4,5 |  | 2 |  |  |  |  |  |  |  |  |  |
| 69 | 3 |  | 6,7 |  | 2 |  |  |  |  |  |  |  |  |  |
| 70 | 3 |  | 8,9 |  | 2 |  |  |  |  |  |  |  |  |  |
| 71 | 3 |  | 10,11 |  | 2 |  |  |  |  |  |  |  |  |  |
| 72 | 3 |  | 0,1,6 |  | 2 |  |  |  |  |  |  |  |  |  |
| 73 | 3 |  | 2,3,8 |  | 2 |  |  |  |  |  |  |  |  |  |
| 74 | 3 |  | 4,5,10 |  | 2 |  |  |  |  |  |  |  |  |  |
| 75 | 3 |  | 0,1,6,7 |  | 2 |  |  |  |  |  |  |  |  |  |
| 76 | 3 |  | 2,3,8,9 |  | 2 |  |  |  |  |  |  |  |  |  |
| 77 | 3 |  | 4,5,10,11 |  | 2 |  |  |  |  |  |  |  |  |  |
| 78 | 1 |  | 0 |  | 2 |  |  |  |  |  |  |  |  |  |
| 79 | 1 |  | 1 |  | 2 |  |  |  |  |  |  |  |  |  |
| 80 | 1 |  | 6 |  | 2 |  |  |  |  |  |  |  |  |  |
| 81 | 1 |  | 7 |  | 2 |  |  |  |  |  |  |  |  |  |
| 82 | 1 |  | 0,1 |  | 2 |  |  |  |  |  |  |  |  |  |
| 83 | 1 |  | 6,7 |  | 2 |  |  |  |  |  |  |  |  |  |
| 84 | 2 |  | 0,1 |  | 2 |  |  |  |  |  |  |  |  |  |
| 85 | 2 |  | 2,3 |  | 2 |  |  |  |  |  |  |  |  |  |
| 86 | 2 |  | 6,7 |  | 2 |  |  |  |  |  |  |  |  |  |
| 87 | 2 |  | 8,9 |  | 2 |  |  |  |  |  |  |  |  |  |
| 88 | 3 |  | 12 |  | 2 |  |  |  |  |  |  |  |  |  |
| 89 | 3 |  | 13 |  | 2 |  |  |  |  |  |  |  |  |  |
| 90 | 3 |  | 14 |  | 2 |  |  |  |  |  |  |  |  |  |
| 91 | 3 |  | 15 |  | 2 |  |  |  |  |  |  |  |  |  |
| 92 | 3 |  | 16 |  | 2 |  |  |  |  |  |  |  |  |  |
| 93 | 3 |  | 17 |  | 2 |  |  |  |  |  |  |  |  |  |
| 94 | 3 |  | 18 |  | 2 |  |  |  |  |  |  |  |  |  |
| 95 | 3 |  | 19 |  | 2 |  |  |  |  |  |  |  |  |  |
| 96 | 3 |  | 20 |  | 2 |  |  |  |  |  |  |  |  |  |
| 97 | 3 |  | 21 |  | 2 |  |  |  |  |  |  |  |  |  |
| 98 | 3 |  | 22 |  | 2 |  |  |  |  |  |  |  |  |  |
| 99 | 3 |  | 23 |  | 2 |  |  |  |  |  |  |  |  |  |
| 100 | 3 |  | 12,13 |  | 2 |  |  |  |  |  |  |  |  |  |
| 101 | 3 |  | 14,15 |  | 2 |  |  |  |  |  |  |  |  |  |
| 102 | 3 |  | 16,17 |  | 2 |  |  |  |  |  |  |  |  |  |
| 103 | 3 |  | 18,19 |  | 2 |  |  |  |  |  |  |  |  |  |
| 104 | 3 |  | 20,21 |  | 2 |  |  |  |  |  |  |  |  |  |
| 105 | 3 |  | 22,23 |  | 2 |  |  |  |  |  |  |  |  |  |
| 106 | 1 |  | 12 |  | 2 |  |  |  |  |  |  |  |  |  |
| 107 | 1 |  | 13 |  | 2 |  |  |  |  |  |  |  |  |  |
| 108 | 1 |  | 18 |  | 2 |  |  |  |  |  |  |  |  |  |
| 109 | 1 |  | 19 |  | 2 |  |  |  |  |  |  |  |  |  |
| 110 | 1 |  | 12,13 |  | 2 |  |  |  |  |  |  |  |  |  |
| 111 | 1 |  | 18,19 |  | 2 |  |  |  |  |  |  |  |  |  |
| 112 | 2 |  | 12,13 |  | 2 |  |  |  |  |  |  |  |  |  |
| 113 | 2 |  | 14,15 |  | 2 |  |  |  |  |  |  |  |  |  |
| 114 | 2 |  | 18,19 |  | 2 |  |  |  |  |  |  |  |  |  |
| 115 | 2 |  | 20,21 |  | 2 |  |  |  |  |  |  |  |  |  |
| 116 | 2 |  | 0,1,12 |  | 2 |  |  |  |  |  |  |  |  |  |
| 117 | 2 |  | 0,1,12,13 |  | 2 |  |  |  |  |  |  |  |  |  |
| 118 | 2 |  | 6,7,18 |  | 2 |  |  |  |  |  |  |  |  |  |
| 119 | 2 |  | 6,7,18,19 |  | 2 |  |  |  |  |  |  |  |  |  |
| 120 | 2 |  | 2,3,14 |  | 2 |  |  |  |  |  |  |  |  |  |
| 121 | 2 |  | 2,3,14,15 |  | 2 |  |  |  |  |  |  |  |  |  |
| 122 | 2 |  | 8,9,20 |  | 2 |  |  |  |  |  |  |  |  |  |
| 123 | 2 |  | 8,9,20,21 |  | 2 |  |  |  |  |  |  |  |  |  |
| 124 | 3 |  | 0,1,12 |  | 2 |  |  |  |  |  |  |  |  |  |
| 125 | 3 |  | 0,1,12,13 |  | 2 |  |  |  |  |  |  |  |  |  |
| 126 | 3 |  | 6,7,18 |  | 2 |  |  |  |  |  |  |  |  |  |
| 127 | 3 |  | 6,7,18,19 |  | 2 |  |  |  |  |  |  |  |  |  |
| 128 | 3 |  | 2,3,14 |  | 2 |  |  |  |  |  |  |  |  |  |
| 129 | 3 |  | 2,3,14,15 |  | 2 |  |  |  |  |  |  |  |  |  |
| 130 | 3 |  | 8,9,20 |  | 2 |  |  |  |  |  |  |  |  |  |
| 131 | 3 |  | 8,9,20,21 |  | 2 |  |  |  |  |  |  |  |  |  |
| 132 | 3 |  | 4,5,16 |  | 2 |  |  |  |  |  |  |  |  |  |
| 133 | 3 |  | 4,5,16,17 |  | 2 |  |  |  |  |  |  |  |  |  |
| 134 | 3 |  | 10,11,22 |  | 2 |  |  |  |  |  |  |  |  |  |
| 135 | 3 |  | 10,11,22,23 |  | 2 |  |  |  |  |  |  |  |  |  |
| 136 | 2 |  | 0,2,3 |  | 1 |  |  |  |  |  |  |  |  |  |
| 137-255 | Reserved |  | Reserved |  | Reserved |  |  |  |  |  |  |  |  |  |

Table 7.3.1.2.2-11: TCI selection

| Bit field mapped to index | TCI selection |
| --- | --- |
| 0 | The first indicated joint/DL TCI state is applied to the scheduled PDSCH |
| 1 | The second indicated joint/DL TCI state is applied to the scheduled PDSCH |
| 2 | Both indicated joint/DL TCI states are applied to the scheduled PDSCH |
| 3 | Reserved |

##### 7.3.1.2.3	Format 1_2

DCI format 1_2 is used for the scheduling of PDSCH in one cell.

The following information is transmitted by means of the DCI format 1_2 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

< Unchanged parts are omitted >

-	HARQ-ACK retransmission indicator - 0 or 1 bit.

-	1 bit if higher layer parameter pdsch-HARQ-ACK-retxDCI-1-2 is configured.

-	0 bit otherwise.

-	Antenna port(s) - 0, 4, 5, 6, 7 or 8 bits

-	0 bit if higher layer parameter antennaPortsFieldPresenceDCI-1-2 is not configured;

-	Otherwise 4, 5, 6, 7 or 8 bits as defined by Tables 7.3.1.2.2-1/2/3/4/7/8/9/10 and Tables 7.3.1.2.2-1A/2A/3A/4A/7A/8A/9A/10A, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. The antenna ports p0,…,pv-1 shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4/7/8/9/10 or Tables 7.3.1.2.2-1A/2A/3A/4A/7A/8A/9A/10A. When a UE not configured with dl-OrJointTCI-StateList receives an activation command that maps at least one codepoint of DCI field 'Transmission Configuration Indication' to two TCI states, or when a UE configured with dl-OrJointTCI-StateList is having two indicated TCI states, the UE shall use Table 7.3.1.2.2-1A/2A/3A/4A/7A/8A/9A/10A; otherwise, it shall use Tables 7.3.1.2.2-1/2/3/4/7/8/9/10.

-	If a UE is configured with both dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 and is configured with higher layer parameter antennaPortsFieldPresenceDCI-1-2, the bitwidth of this field equals maxxA,xB, where xA is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and xB is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2. A number of xA-xB zeros are padded in the MSB of this field, if the mapping type of the PDSCH corresponds to the smaller value of xA and xB.

If a UE is not configured with higher layer parameter antennaPortsFieldPresenceDCI-1-2, antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.2.2-1/2/3/4/7/8/9/10.

-	Transmission configuration indication - 0 bit if higher layer parameter tci-PresentDCI-1-2 is not configured; otherwise 1 or 2 or 3 bits determined by higher layer parameter tci-PresentDCI-1-2 as defined in Clause 5.1.5 of [6, TS38.214].

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part,

-	if the higher layer parameter tci-PresentDCI-1-2 is not configured for the CORESET used for the PDCCH carrying the DCI format 1_2,

-	the UE assumes tci-PresentDCI-1-2 is not configured for all CORESETs in the indicated bandwidth part;

-	otherwise,

-	the UE assumes tci-PresentDCI-1-2 is configured for all CORESETs in the indicated bandwidth part with the same value configured for the CORESET used for the PDCCH carrying the DCI format 1_2.

< Unchanged parts are omitted >