3GPP TSG-RAN WG1 Meeting #115	R1-2312753

# Chicago, USA, November 13th – November 17th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0483 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of specification support for MIMO enhancements on CSI |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core |  |  |  | Date: |  | 2023-12-01 |
| Category: | F |  |  |  | Release: |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 5.2.1.4.1 - It is not clear that, for CJT and Doppler CSI, interference measurement on NZP-CSI-RS can only be configured on a single resource - agreement that, for TDCP, interference measurement is not supported In clause 5.2.2.2.8 - the numbering of parameter combinations can be made for efficient for RRC In clause signalling 5.2.2.2.9 - the numbering of parameter combinations can be made for efficient for RRC signalling In clause 5.2.2.5 - the conditions for dropping a CSI report if at least a measurement occasion is not received for each configured CMR and NZP-IMR and/or CSI-IM are missing, for CJT CSI and Doppler CSI - the condition for dropping a TDCP report if at least a measurement occasion is not received for each cofigred CSI-RS resource in the KTRS configured resource sets is missing In clause 5.2.2.5.1 - the UE assumptions for CQI/PMI/RI calculation are missing for the slot(s) associated with a CQI in a predicted CSI In clause 5.2.2.5.1b - The clause “transmitted on P antenna ports of CSI-RS resource σj” may suggest that PDSCH EPRE is assumed per TRP (rather than across all TRPs – which is the case for CJT) for CQI calculation In clause 5.2.1.6 - the number of CPUs occupied per aperiodic CSI-RS resource in Doppler CSI calculation is too small for the capability value Y1=2/3 In clause 5.2.3, in the equation of Pril,ij,f,j for Further Enhanced Type II Port Selection codebook for CJT reports, the number of selected ports should be K1,σk instead of K1,k. The K1,σk is the number of selected ports for the k-th selected CSI-RS resource, while K1,kis the k-th gNB-configured CSI-RS resource. Using K1,k will result in a miscalculated priority value, when the {αn} combination is unequal. In clause 5.2.2.2.9, capture the following agreement: [114bis] Agreement: For the Rel-18 Type-II codebook refinement for CJT mTRP, with respect to L or , the supported Parameter Combinations is enumerated for each NTRP value (up to 5 for Rel-16-based and 8 for Rel-17-based), rather than enumerating across all NTRP values of 1, 2, 3, and 4 (up to 17 for Rel-16-based and 20 for Rel-17-based). - Note: in TS38.214, this affects Tables 5.2.2.2.8-1, 5.2.2.2.8-3, 5.2.2.2.9-1, and 5.2.2.2.9-3 In clause 5.2.1.6, [114bis] Agreement For the Type-II codebook refinement for high/medium velocities, regarding CPU allocation, remove Y=2/3 (previously agreed) and add the support for OCPU=8 for K=12 for AP-CSIRS Furthermore, for Rel-18 CJT CSI, the number of CPU is OCPU=X⋅NTRP, where X∈{1, 1.5, 2} is reported by UE capability indication. When NTRP=3, the value can be 4.5 based on the reported value. A ceil operation is expected here. In clause 5.2.1.4.1, [112bis-e] Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding CSI calculation and measurement, - The number of CSI-RS ports is the same for all the K configured CSI-RS resources comprising the CMR and the antenna ports for the same antenna port index across the K CSI-RS resources are the same. - All the K configured CSI-RS resources comprising the CMR share the same BW and RE locations - For interference measurement, legacy specification is fully reused, including the configuration for NZP CSI-RS for interference measurement or CSI-IM in relation to the configured CMR, i.e. only one NZP CSI-RS resource for interference measurement or only one CSI-IM resource can be configured irrespective of the value of K …… Conclusion (RAN1#112bis-e) [Rel18-CJT] For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding interference measurement, beyond that supported in legacy specification, there is no consensus on supporting any additional enhancement on IMR (including the configuration for NZP CSI-RS for interference measurement or CSI-IM in relation to the configured CMR(s)). - Note: This implies that only one NZP CSI-RS resource for interference measurement or only one CSI-IM resource can be configured irrespective of the value of NTRP In clauses 5.2.1.2 and 5.2.1.4.1, there is no agreement to support KTRS=1 with aperiodic TRS regardless of the interpretation of the previous conclusions (below) [113] Conclusion For the Rel-18 TRS-based TDCP reporting, for TDCP measurement and calculation, there is no consensus on supporting the following: joint use of P and AP-TRS resource sets for TDCP measurement and calculation is supported at least for Y=1 as a UE-optional feature [114] Conclusion: For the Rel-18 TRS-based TDCP reporting, there is no consensus on supporting the following proposals: - additional D value(s) - TRS resource configuration where all the configured KTRS resource sets are aperiodic In clause 5.2.2.2.9 capture the following agreement: [114bis] Agreement: For the Rel-18 Type-II codebook refinement for CJT mTRP, with respect to L or , the supported Parameter Combinations is enumerated for each NTRP value (up to 5 for Rel-16-based and 8 for Rel-17-based), rather than enumerating across all NTRP values of 1, 2, 3, and 4 (up to 17 for Rel-16-based and 20 for Rel-17-based). - Note: in TS38.214, this affects Tables 5.2.2.2.8-1, 5.2.2.2.8-3, 5.2.2.2.9-1, and 5.2.2.2.9-3 In clause 5.2.2.5 To address FFS in the previous agreement [114bis] Agreement For the Rel-18 TRS-based TDCP reporting, the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion for each CSI-RS resource for KTRS CSI-RS resource sets configured for TDCP reporting no later than CSI reference resource, otherwise drops the report. - This includes the cases of CSI report (re)configuration, serving cell activation, BWP change - FFS (RAN1#115): Whether DRX configuration needs to be included as a case In clause 5.2.3, for enhanced type II for predicted PMI, it includes two cases, i.e., predicted PMI configured with higher layer parameter N4=1 and N4>1. The content of PMI for the two cases is different as described in section 5.2.3 of TS 38.214. However, for Enhance Type II for predicted PMI with N4>1, it describes the part 2 contains the PMI of the Enhanced Type II for predicted PMI in the current specification, which is not clear. |  |  |  |  |  |
| Summary of change: |  | In clause 5.2.1.4.1 - interference measurement on NZP-CSI-RS on a single resource for CJT and Doppler CSI - interference measurement on CSI-IM and/or NZP-IMR is excluded for TDCP In clause 5.2.2.2.8 - renumbering of parameter combinations in tables 1 and 3 In clause 5.2.2.2.9 - renumbering of parameter combinations in tables 1 and 3 In clause 5.2.2.5 - the conditions are added for dropping a CSI report if at least a measurement occasion is not received for each configured CMR and NZP-IMR and/or CSI-IM, for CJT CSI and Doppler CSI - the conditions are added for dropping a TDCP report if at least a measurement occasion is not received for each cofigred CSI-RS resource in the KTRS configured resource sets In clause 5.2.2.5.1 - the UE assumptions for CQI/PMI/RI calculation in the CSI reference resource are extended to the slot(s) associated with a CQI in a predicted CSI. In clause 5.2.2.5.1b - rewording to clarify that, for the powerControlOffset in CJT, the PDSCH EPRE is calculated across TRPs, whereas the CSI-RS EPRE is calculated per per TRP In clause 5.2.1.6 - the capability value Y1=2/3 is removed for Doppler CSI CPU calculation and the CPU value OCPU=8 is introduced for K=12 aperiodic CSI-RS In clause 5.2.3, regarding the priority value for Further Enhanced Type II Port Selection codebook for CJT reports, K1,k and K1,j are changed to K1,σk and K1,σj respectively. In clause 5.2.2.2.9, similar corrections on the text of the indices of paramCombination-CJT-PS-alpha-r18 which UE is not expected to be configured with, should be applied to further enhanced Type-II port selection codebook for CJT. In clause 5.2.1.6, deleted Y=2/3 for P/SP CSI-RS based doppler CSI reporting, and add ceil operation for CPU calculation of CJT CSI. In clause 5.2.1.4.1, added a list of relevant reportQuantity types. In clauses 5.2.1.2 and 5.2.1.4.1, deleted the description on supporting AP TRS set only for TDCP measurement. In clause 5.2.2.2.9 similar corrections on the text of the indices of paramCombination-CJT-PS-alpha-r18 which UE is not expected to be configured with, should be applied to further enhanced Type-II port selection codebook for CJT. In clause 5.2.2.5 added DRX as a case In clause 5.2.3, clarify that Part 2 contains the second CQI (if the higher layer parameter TDCQI is set to '2') and the PMI of the Enhanced Type II for predicted PMI with N_4>1 for Enhanced Type II for predicted PMI with N_4>1. |  |  |  |  |  |

| Consequences if not approved: | Specification of CJT CSI, Doppler CSI and TDCP incomplete, incorrect or unclear. |  |  |  |
| --- | --- | --- | --- | --- |
| Clauses affected: | 5.2.1.2, 5.2.1.4.1, 5.2.2.2.8, 5.2.2.2.9, 5.2.2.5, 5.2.2.5.1, 5.2.2.5.1b, 5.2.1.6, 5.2.3 |  |  |  |
|  | Y | N |  |  |
| Other specs | X |  | Other core specifications | TS 38.211, TS 38.212, TS 38.213 |
| affected: |  | X | Test specifications | TS/TR ... CR ... |
| (show related CRs) |  | X | O&M Specifications | TS/TR ... CR ... |
| Other comments: |  |  |  |  |
| This CR's revision history: |  |  |  |  |

<omitted text>

#### 5.2.1.2	Resource settings

Each CSI Resource Setting CSI-ResourceConfig contains a configuration of a list of S≥1 CSI Resource Sets (given by higher layer parameter csi-RS-ResourceSetList), where the list is comprised of references to either or both of NZP CSI-RS resource set(s) and SS/PBCH block set(s) or the list is comprised of references to CSI-IM resource set(s). Each CSI Resource Setting is located in the DL BWP identified by the higher layer parameter BWP-id, and all CSI Resource Settings linked to a CSI Report Setting have the same DL BWP.

The time domain behavior of the CSI-RS resources within a CSI Resource Setting are indicated by the higher layer parameter resourceType and can be set to aperiodic, periodic, or semi-persistent. For periodic and semi-persistent CSI Resource Settings, when the UE is configured with groupBasedBeamReporting-r17 or groupBasedBeamReporting-v18, the number of CSI Resource Sets configured is S=2, otherwise the number of CSI-RS Resource Sets configured is limited to S=1, except for periodic CSI Resource Settings, when the UE is configured with TDCP reporting, for which the number of CSI-RS Resource Sets in the CSI Resource Setting for channel measurement is KTRS∈{1,2,3} and all the CSI-RS Resource Sets are configured with the higher layer parameter trs-Info. For periodic and semi-persistent CSI Resource Settings, the configured periodicity and slot offset is given in the numerology of its associated DL BWP, as given by BWP-id. When a UE is configured with multiple CSI-ResourceConfigs consisting the same NZP CSI-RS resource ID, the same time domain behavior shall be configured for the CSI-ResourceConfigs. When a UE is configured with multiple CSI-ResourceConfigs consisting the same CSI-IM resource ID, the same time-domain behavior shall be configured for the CSI-ResourceConfigs. All CSI Resource Settings linked to a CSI Report Setting shall have the same time domain behavior.

The following are configured via higher layer signaling for one or more CSI Resource Settings for channel and interference measurement:

-	CSI-IM resource for interference measurement as described in Clause 5.2.2.4.

-	NZP CSI-RS resource for interference measurement as described in Clause 5.2.2.3.1.

-	NZP CSI-RS resource for channel measurement as described in Clause 5.2.2.3.1.

The UE may assume that the NZP CSI-RS resource(s) for channel measurement and the CSI-IM resource(s) for interference measurement configured for one CSI reporting are resource-wise QCLed with respect to 'typeD'. When NZP CSI-RS resource(s) is used for interference measurement, the UE may assume that the NZP CSI-RS resource for channel measurement and the CSI- IM resource or NZP CSI-RS resource(s) for interference measurement configured for one CSI reporting are QCLed with respect to 'typeD'.

For TDCP measurement, one periodic CSI Resource Setting is configured, and the Resource Setting is for channel measurement on CSI-RS for tracking.

For L1-SINR measurement:

-	When one Resource Setting is configured, the Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is for channel and interference measurement on NZP CSI-RS for L1-SINR computation. UE may assume that same 1 port NZP CSI-RS resource(s) with density 3 REs/RB is used for both channel and interference measurements.

-	When two Resource Settings are configured, the first one Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is for channel measurement on SSB or NZP CSI-RS and the second one (given by either higher layer parameter csi-IM-ResourcesForInterference or higher layer parameter nzp-CSI-RS-ResourcesForInterference) is for interference measurement performed on CSI-IM or on 1 port NZP CSI-RS with density 3 REs/RB, where each SSB or NZP CSI-RS resource for channel measurement is associated with one CSI-IM resource or one NZP CSI-RS resource for interference measurement by the ordering of the SSB or NZP CSI-RS resource for channel measurement and CSI-IM resource or NZP CSI-RS resource for interference measurement in the corresponding resource sets. The number of SSB(s) or CSI-RS resources for channel measurement equals to the number of CSI-IM resources or the number of NZP CSI-RS resource for interference measurement.

-	UE may apply the SSB, or 'typeD' RS configured with qcl-Type set to 'typeD' to the NZP CSI-RS resource for channel measurement, as the reference RS for determining 'typeD' assumption for the corresponding CSI-IM resource or the corresponding NZP CSI-RS resource for interference measurement configured for one CSI reporting.

-	UE may expect that the NZP CSI-RS resource set for channel measurement and the NZP-CSI-RS resource set for interference measurement, if any, are configured with the higher layer parameter repetition.

Each LTM CSI Resource Setting [LTM-CSI-ResourceConfig] contains configuration of a [LTM-CSI-SSB-ResourceSet] which comprises of a list of Z ≥ 1 SS/PBCH blocks indices (given by [LTM-csi-SSB-ResourceList]) and a list of Z [PCI indices] (given by [LTM-CandidateId-list]) referring to cells associated with the SS/PBCH block indices. The UE determines the time domain behavior of a SS/PBCH block from ssb-Periodicity and ssb-PositionsInBurst and the frequency domain behavior of a SS/PBCH block is determined by the higher layer parameters subcarrierspacing, ssbFrequency.

<omitted text>

##### 5.2.1.4.1	Resource Setting configuration

For aperiodic CSI, each trigger state configured using the higher layer parameter CSI-AperiodicTriggerState is associated with one or multiple CSI-ReportConfig where the CSI-ReportConfig not configured with groupBasedBeamReporting-r17 is linked to periodic, or semi-persistent, or aperiodic resource setting(s):

-	When one Resource Setting is configured, the Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is for channel measurement for L1-RSRP or for channel and interference measurement for L1-SINR computation.

-	When two Resource Settings are configured, the first one Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is for channel measurement and the second one (given by either higher layer parameter csi-IM-ResourcesForInterference or higher layer parameter nzp-CSI-RS-ResourcesForInterference) is for interference measurement performed on CSI-IM or on NZP CSI-RS.

-	When three Resource Settings are configured, the first Resource Setting (higher layer parameter resourcesForChannelMeasurement) is for channel measurement, the second one (given by higher layer parameter csi-IM-ResourcesForInterference) is for CSI-IM based interference measurement and the third one (given by higher layer parameter nzp-CSI-RS-ResourcesForInterference) is for NZP CSI-RS based interference measurement.

For aperiodic CSI, and for periodic and semi-persistent CSI resource settings, each trigger state configured using the higher layer parameter CSI-AperiodicTriggerState is associated with one or multiple CSI-ReportConfig where the CSI-ReportConfig configured with groupBasedBeamReporting-r17 is linked to periodic or semi-persistent, setting(s):

-	When one Resource Setting is configured, the Resource setting is given by resourcesForChannelMeasurement for L1-RSRP measurement. In such a case, the number of configured CSI Resource Sets in the Resource Setting is S=2

For aperiodic CSI, and for aperiodic CSI resource settings, each trigger state configured using the higher layer parameter CSI-AperiodicTriggerState is associated with one or multiple CSI-ReportConfig where the CSI-ReportConfig configured with groupBasedBeamReporting-r17 is associated with resourcesForChannel and resourcesForChannel2, which correspond to first and second resource sets, respectively, for L1-RSRP measurement.

For semi-persistent or periodic CSI, each CSI-ReportConfig is linked to periodic or semi-persistent Resource Setting(s):

-	When one Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is configured, the Resource Setting is for channel measurement for L1-RSRP or for channel and interference measurement for L1-SINR computation.

-	When two Resource Settings are configured, the first Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is for channel measurement and the second Resource Setting (given by higher layer parameter csi-IM-ResourcesForInterference) is used for interference measurement performed on CSI-IM. For L1-SINR computation, the second Resource Setting (given by higher layer parameter csi-IM-ResourcesForInterference or higher layer parameter nzp-CSI-RS-ResourceForInterference) is used for interference measurement performed on CSI-IM or on NZP CSI-RS.

For aperiodic CSI, a UE configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'tdcp' is expected to be configured with one CSI Resource Setting (given by higher layer parameter resourcesForChannelMeasurement). The CSI Resource Setting may be periodic, with KTRS∈{1, 2, 3} CSI-RS Resource Sets configured with higher layer parameter trs-Info. The support of KTRSP=2 or 3 is subject to UE capability indication. For a periodic CSI-ResourceConfig, the UE can assume that all the CSI-RS resources in the KTRS CSI-RS Resource Sets share the same QCL-TypeA/C and, if applicable, TypeD. The UE expects that all the CSI-RS resources in the CSI-RS Resource Set(s) are configured with the same bandwidth and subcarrier locations. A UE configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'tdcp' is not expected to be configured with interference measurement on CSI-IM and/or NZP-CSI-RS.

A UE is not expected to be configured with more than one CSI-RS resource in resource set for channel measurement for a CSI-ReportConfig with the higher layer parameter codebookType set to 'typeII', 'typeII-PortSelection', 'typeII-r16', 'typeII-PortSelection-r16', or 'typeII-PortSelection-r17'. A UE is not expected to be configured with more than 64 NZP CSI-RS resources and/or SS/PBCH block resources in resource setting for channel measurement for a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'none', 'cri-RI-CQI', 'cri-RSRP', 'ssb-Index-RSRP', 'cri-SINR' or 'ssb-Index-SINR', 'cri-RSRP- Index', 'ssb-Index-RSRP- Index', 'cri-SINR- Index' or 'ssb-Index-SINR- Index'. If interference measurement is performed on CSI-IM, each CSI-RS resource for channel measurement is resource-wise associated with a CSI-IM resource by the ordering of the CSI-RS resource and CSI-IM resource in the corresponding resource sets. The number of CSI-RS resources for channel measurement equals to the number of CSI-IM resources.

A UE configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI' and codebookType set to 'typeII-CJT-r18' or 'typeII-CJT-PortSelection-r18' is expected to be configured with 1≤K≤4 CSI-RS resources in a resource set for channel measurement. If interference measurement is performed on CSI-IM, only one resource is configured in the corresponding csi-IM-ResourceSet. If interference measurement is performed on NZP CSI-RS, only one resource is configured in the corresponding NZP-CSI-RS-ResourceSet for interference measurement.

A UE configured with a CSI-ReportConfig with the higher layer parameter N4 and reportQuantity set to 'cri-RI-PMI-CQI', is expected to be configured with K∈{4,8,12} aperiodic CSI-RS resources or with a single periodic or semi-persistent CSI-RS resource in the resource set for channel measurement. For an aperiodic CSI-RS resource set for channel measurement, the K CSI-RS resources are triggered by the same triggering instance and the separation between two consecutive CSI-RS resources is m∈{1,2} slots, which is configured by higher layer parameter in the NZP-CSI-RS-ResourceSet. The K aperiodic CSI-RS resources are transmitted following the order of the CSI-RS resource IDs configured in the CSI-RS resource set. The UE shall assume that the antenna port with the same port index of the K aperiodic CSI-RS resources is the same. If interference measurement is performed on CSI-IM, only one resource is configured in the corresponding csi-IM-ResourceSet. If interference measurement is performed on NZP CSI-RS, only one resource is configured in the corresponding NZP-CSI-RS-ResourceSet for interference measurement.

An NZP CSI-RS Resource Set for channel measurement with 2≤Ks≤8 resources can be configured with two Resource Groups, with K1≥1 resources in Group 1 and K2≥1 resources in Group 2, such that K1+K2=Ks, and with N∈{1,2} Resource Pairs. Each Resource Pair consists of one resource from Group 1 and one resource from Group 2. The same resource can be associated with two Resource Pairs in frequency range 1 but not in frequency range 2.

Except for L1-SINR, codebookType set to 'typeII-CJT-r18', 'typeII-CJT-PortSelection-r18', 'typeII-Doppler-r18', or 'typeII-Doppler-PortSelection-r18', if interference measurement is performed on NZP CSI-RS, a UE does not expect to be configured with more than one NZP CSI-RS resource in the associated resource set within the resource setting for channel measurement. Except for L1-SINR, the UE configured with the higher layer parameter nzp-CSI-RS-ResourcesForInterference may expect no more than 18 NZP CSI-RS ports configured in a NZP CSI-RS resource set.

For CSI measurement(s) other than L1-SINR, a UE assumes:

-	each NZP CSI-RS port configured for interference measurement corresponds to an interference transmission layer.

-	all interference transmission layers on NZP CSI-RS ports for interference measurement take into account the associated EPRE ratios configured in 5.2.2.3.1;

-	other interference signal on REs of NZP CSI-RS resource for channel measurement, NZP CSI-RS resource for interference measurement, or CSI-IM resource for interference measurement.

For L1-SINR measurement with dedicated interference measurement resources, a UE assumes:

-	the total received power on dedicated NZP CSI-RS resource for interference measurement or dedicated CSI-IM resource for interference measurement corresponds to interference and noise.

<omitted text>

##### 5.2.1.4.5	TDCP Reporting

For a CSI-ReportConfig with higher layer parameter reportQuantity set to ‘tdcp’ and higher layer parameters Y≥1 and {D1,…,DY}, the reported TDCP amplitude(s) corresponding to the Y configured delays are indicated by

kTDCP=[k1…kY]

ki∈{0,1,…,15}

and the corresponding amplitude values are obtained from: 1-ai, for i=1,…,Y, where the mapping from ki to ai is given in Table 5.2.1.4.5-1.

Table 5.2.1.4.5-1: Mapping of elements of kTDCP: ki to 1-ai

| ki; ai. 0; 1256. 1; 11282. 2; 1128. 3; 1642. | ki; ai. 4; 164. 5; 1322. 6; 132. 7; 1162. | ki; ai. 8; 116. 9; 182. 10; 18. 11; 142. | ki; ai. 12; 14. 13; 122. 14; 12. 15; 12. |
| --- | --- | --- | --- |

For Y>1, if the higher layer parameter phase is configured, the reported TDCP phases are indicated by

cTDCP=[c1…cY]

ci∈{0,1,…,15}

and the corresponding phase values are given by: ej2πci16.

<omitted text>

#### 5.2.1.6	CSI processing criteria

The UE indicates the number of supported simultaneous CSI calculations NCPU with parameter simultaneousCSI-ReportsPerCC in a component carrier, and simultaneousCSI-ReportsAllCC across all component carriers. If a UE supports NCPU simultaneous CSI calculations it is said to have NCPU CSI processing units for processing CSI reports. If L CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU-L unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU-L CPUs are unoccupied, where each CSI report n=0, …, N-1 corresponds to OCPU(n), the UE is not required to update the N-M requested CSI reports with lowest priority (according to Clause 5.2.5), where 0≤M≤N is the largest value such that n=0M-1OCPU(n)≤NCPU-L  holds.

A UE is not expected to be configured with an aperiodic CSI trigger state containing more than NCPU Reporting Settings. Processing of a CSI report occupies a number of CPUs for a number of symbols as follows:

-	OCPU=0  for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none' and CSI-RS-ResourceSet with higher layer parameter trs-Info configured

-	OCPU=1  for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RSRP', 'ssb-Index-RSRP', 'cri-SINR', 'ssb-Index-SINR', 'cri-RSRP- Index', 'ssb-Index-RSRP- Index', 'cri-SINR- Index', 'ssb-Index-SINR- Index ' or 'none' (and CSI-RS-ResourceSet with higher layer parameter trs-Info not configured)

-	OCPU=Y+1⋅X, for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'tdcp' and with number of delays Y configured by higher layer parameter Y, where the value of X∈{1, 2} is reported by UE capability.

-	for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', or 'cri-RI-LI-PMI-CQI',

-	if max{ µPDCCH, µCSI-RS, µUL} ≤ 3, and if a CSI report is aperiodically triggered without transmitting a PUSCH with either transport block or HARQ-ACK or both when L = 0 CPUs are occupied, where the CSI corresponds to a single CSI with wideband frequency-granularity and to at most 4 CSI-RS ports in a single resource without CRI report and where codebookType is set to 'typeI-SinglePanel' or where reportQuantity is set to 'cri-RI-CQI', OCPU=NCPUOCPU=NCPU,

-	if a CSI-ReportConfig is configured with codebookType set to 'typeI-SinglePanel' and the corresponding CSI-RS Resource Set for channel measurement is configured with two Resource Groups and N Resource Pairs, OCPU=X⋅N+M, where X is the number of CPUs occupied by a pair of CMRs subject to mTRP-CSI-numCPU-r17 and M is defined in clause 5.2.1.4.2,

-	if a CSI-ReportConfig is configured with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', codebookType set to 'typeII-CJT-r18' or 'typeII-CJT-PortSelection-r18' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is configured with 1<NTRP≤4 resources, OCPU=ceil(X⋅NTRP)OCPU=X⋅NTRP, where X∈{1, 1.5, 2} is reported by UE capability indication,

-	if a CSI-ReportConfig is configured with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI' and with codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18',

-	if the corresponding CSI-RS Resource Set for channel measurement is aperiodic and configured with K CSI-RS resources, OCPU=8 for K=12 and OCPU=Y1⋅K, for K<12, where Y1∈{2/3, 1, 2, 3} is reported by UE capability indication,

-	if the corresponding CSI-RS Resource Set for channel measurement is periodic or semi-persistent and configured with a single CSI-RS resource, OCPU=4 for N4=1 and OCPU=Y2⋅N4≥4, for N4>1, where the value of N4 is configured by the higher layer parameter N4, and Y2∈{2/3, 1, 2, 3} is reported by UE capability indication,

-	otherwise, OCPU=KsOCPU=KS, where Ks Ks is the number of CSI-RS resources in the CSI-RS resource set for channel measurement.

For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to 'none', the CPU(s) are occupied for a number of OFDM symbols as follows:

-	A periodic or semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report and a semi-persistent CSI report on PUSCH configured with the higher layer parameter codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18') occupies CPU(s) from the first symbol of the earliest one of each CSI-RS/CSI-IM/SSB resource for channel or interference measurement, respective latest CSI-RS/CSI-IM/SSB occasion no later than the corresponding CSI reference resource, until the last symbol of the configured PUSCH/PUCCH carrying the report.

-	An aperiodic CSI report occupies CPU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the CPU occupation duration, the PDCCH candidate that ends later in time is used.

-	An initial semi-persistent CSI report on PUSCH after the PDCCH trigger occupies CPU(s) from the first symbol after the PDCCH until the last symbol of the scheduled PUSCH carrying the report. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the CPU occupation duration, the PDCCH candidate that ends later in time is used.

-   A semi-persistent CSI report on PUSCH configured with the higher layer parameter codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18' occupies CPU(s) from the first symbol of KP-th latest consecutive periodic/semi-persistent CSI-RS occasions no later than CSI reference resource, until the last symbol of the PUSCH carrying the report, where the value of KP∈{1,2,4} is indicated by UE capability.

For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none' and CSI-RS-ResourceSet with higher layer parameter trs-Info not configured, the CPU(s) are occupied for a number of OFDM symbols as follows:

-	A semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report) occupies CPU(s) from the first symbol of the earliest one of each transmission occasion of periodic or semi-persistent CSI-RS/SSB resource for channel measurement for L1-RSRP computation, until Z3' symbols after the last symbol of the latest one of the CSI-RS/SSB resource for channel measurement for L1-RSRP computation in each transmission occasion.

-	An aperiodic CSI report occupies CPU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol between Z3 symbols after the first symbol after the PDCCH triggering the CSI report and Z3' symbols after the last symbol of the latest one of each CSI-RS/SSB resource for channel measurement for L1-RSRP computation.

where (Z3,Z3') are defined in the table 5.4-2.

In any slot, the UE is not expected to have more active CSI-RS ports or active CSI-RS resources in active BWPs than reported as capability. NZP CSI-RS resource is active in a duration of time defined as follows. For aperiodic CSI-RS, starting from the end of the PDCCH containing the request and ending at the end of the scheduled PUSCH containing the report associated with this aperiodic CSI-RS. When the PDCCH candidates are associated with a search space set configured with searchSpaceLinking, for the purpose of determining the NZP CSI-RS resource active duration, the PDCCH candidate that ends later in time among the two linked PDCCH candidates is used. For semi-persistent CSI-RS, starting from the end of when the activation command is applied, and ending at the end of when the deactivation command is applied. For periodic CSI-RS, starting when the periodic CSI-RS is configured by higher layer signalling, and ending when the periodic CSI-RS configuration is released.

If a CSI-RS resource is referred N times by one or more CSI Reporting Settings, the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted N times.

For a CSI-RS Resource Set for channel measurement configured with two Resource Groups and N Resource Pairs, if a CSI-RS resource is referred X times by one of the M CSI-RS resources, where M is defined in clause 5.2.1.4.2, and/or one or two Resource Pairs, the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted X times.

For a periodic or semi-persistent CSI-RS resource in a CSI-RS resource set for channel measurement linked to a CSI-ReportConfig configured with the higher layer parameter codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18', the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted KP times, where the value of KP∈{1,2,4} is indicated by UE capability.

<omitted text>

##### 5.2.2.2.8	Enhanced Type II codebook for CJT

For 4 antenna ports {3000, 3001, …, 3003}, 8 antenna ports {3000, 3001, …, 3007}, 12 antenna ports {3000, 3001, …, 3011}, 16 antenna ports {3000, 3001, …, 3015}, 24 antenna ports {3000, 3001, …, 3023}, and 32 antenna ports {3000, 3001, …, 3031} per CSI-RS resource, and the UE configured with NTRP∈{1,2,3,4} CSI-RS resources in a resource set for channel measurement and with higher layer parameter codebookType set to 'typeII-CJT-r18'

-	the values of N1, N2  and O1, O2 are the same for all NTRP CSI-RS resources and configured with the higher layer parameter. The supported configurations of N1,N2 for a given number of CSI-RS ports and the corresponding values of O1,O2  are given in Table 5.2.2.2.1-2. The number of CSI-RS ports, PCSI-RS, is 2N1N2 for each of the NTRP CSI-RS resources.

-	A set of NL∈{1,2,4} combinations of values of {L1,…,LNTRP} is configured by the higher layer parameter paramCombination-CJT-L-r18, where the value of NL is configured by the higher layer parameter numberOfSDCombinations and the mapping is given in Table 5.2.2.2.8-1, with the value of Ln corresponding to CSI-RS resource n, for n=1,…,NTRP. A single value of pν and β is configured by the higher layer parameter paramCombination-CJT-r18, where the mapping is given in Table 5.2.2.2.8-2. The configurable combinations of {L1,…,LNTRP} and {pυ,β} are marked with 'x' in Table 5.2.2.2.8-3.

-	The UE is not expected to be configured with paramCombination-CJT-L-r18 equal to

-	2, 3, for NTRP=1, 2, 3, 4, for NTRP=2, 2, 3, 4, 5, for NTRP=3, 2, 3 or 4, for NTRP=4, when PCSI-RS=4,

-	3, for NTRP=1, when PCSI-RS<32

-	3, for NTRP=1, when higher layer parameter typeII-CJT-RI-Restriction-r18 is configured with ri=1 for any i>1.

-	3, for NTRP=1, when R=2.

-	If NL>1, the UE is expected to select one of the NL configured combinations of {L1,…,LNTRP} and report the index of the selected combination, where the index value 0 corresponds to the first configured combination and the index value NL-1 corresponds to the NL-th configured combination. If NL=1, a single combination of {L1,…,LNTRP} is configured and the selection is not reported.

Table 5.2.2.2.8-1: Codebook parameter configurations for {L1,…, LNTRP}

| NTRP | paramCombination-CJT-L-r18 | {L1,…, LNTRP} |
| --- | --- | --- |
| 1 | 1 | {2} |
|  | 2 | {4} |
|  | 3 | {6} |
| 2 | 1 | {2,2} |
|  | 2 | {2,4} |
|  | 3 | {4,2} |
|  | 4 | {4,4} |
| 3 | 1 | {2,2,2} |
|  | 2 | {2,2,4} |
|  | 3 | {2,4,2} |
|  | 4 | {4,2,2} |
|  | 5 | {4,4,4} |
| 4 | 1 | {2,2,2,2} |
|  | 2 | {2,2,2,4} |
|  | 3 | {2,2,4,4} |
|  | 4 | {4,4,4,4} |

Table 5.2.2.2.8-2: Codebook parameter configurations for {pυ,β}

| paramCombination-CJT-r18 | pυ |  | β |
| --- | --- | --- | --- |
|  | υ ∈1,2 | υ ∈3,4 |  |
| 1 | 1/8 | 1/16 | ¼ |
| 2 | 1/8 | 1/16 | ½ |
| 3 | ¼ | 1/8 | ¼ |
| 4 | ¼ | 1/8 | ½ |
| 5 | ¼ | ¼ | ¾ |
| 6 | ½ | ¼ | ½ |
| 7 | ½ | ½ | ½ |

Table 5.2.2.2.8-3: Configurable combinations of {L1,…, LNTRP} and {pυ,β}

| NTRP | paramCombination-CJT-L-r18 | paramCombination-CJT-r18 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | 1 |  |  | x | x |  |  |  |
|  | 2 |  |  | x | x | x | x |  |
|  | 3 |  |  |  | x | x |  |  |
| 2 | 1 | x |  |  |  |  |  |  |
|  | 2 | x |  |  |  |  |  |  |
|  | 3 | x |  |  |  |  |  |  |
|  | 4 |  | x |  | x |  |  | x |
| 3 | 1 | x | x |  |  |  |  |  |
|  | 2 | x | x |  |  |  |  |  |
|  | 3 | x | x |  |  |  |  |  |
|  | 4 | x | x |  |  |  |  |  |
|  | 5 | x | x | x | x | x |  | x |
| 4 | 1 | x |  |  |  |  |  |  |
|  | 2 | x |  |  |  |  |  |  |
|  | 3 |  |  |  | x | x |  |  |
|  | 4 |  | x |  | x | x |  |  |

-	The value of R∈{1,2} is configured with the higher-layer parameter numberOfPMI-SubbandsPerCQI-Subband-CJT-r18, where R and the corresponding value of N3 are defined as in clause 5.2.2.2.5.

-	The UE shall report the RI value υ according to the configured higher layer parameter typeII-CJT-RI-Restriction-r18. The UE shall not report υ>4. The bitmap parameter typeII-CJT-RI-Restriction-r18 forms the bit sequence r3,r2,r1,r0 where r0 is the LSB and r3 is the MSB. When ri is zero, i∈0,1,…,3, PMI and RI reporting are not allowed to correspond to any precoder associated with υ=i+1 layers.

-	The UE may be configured with higher layer parameter restrictedCMR-Selection. If restrictedCMR-Selection is configured, the number of selected CSI-RS resources is N0=NTRP. Otherwise, the UE is expected to select N0 CSI-RS resources, with 1≤N0≤NTRP, and the selection is reported with an NTRP-bit bitmap, bNTRP, …,b1, where the CSI-RS resources are mapped from bit b1 to bit bNTRP by their ordering in the resource set and the first of the N0 selected CSI-RS resources corresponds to the nonzero bit with lowest index.

The PMI value for the N0 selected CSI-RS resources corresponds to the codebook indices of i1 and i2 where

i1=i1,1i1,2i1,5i1,6,1i1,7,1i1,8,1 i1,9                                                                                 υ=1i1,1i1,2     i1,5i1,6,1i1,7,1    i1,8,1i1,6,2     i1,7,2     i1,8,2     i1,9υ=2i1,1i1,2     i1,5i1,6,1i1,7,1    i1,8,1i1,6,2     i1,7,2     i1,8,2     i1,6,3     i1,7,3     i1,8,3     i1,9   υ=3i1,1i1,2     i1,5i1,6,1i1,7,1    i1,8,1i1,6,2i1,7,2     i1,8,2     i1,6,3i1,7,3i1,8,3     i1,6,4i1,7,4     i1,8,4i1,9υ=4i2=i2,3,1i2,4,1i2,5,1                                                                                                                           υ=1i2,3,1i2,4,1i2,5,1    i2,3,2i2,4,2i2,5,2υ=2i2,3,1i2,4,1i2,5,1    i2,3,2i2,4,2i2,5,2    i2,3,3i2,4,3i2,5,3                                             υ=3i2,3,1i2,4,1i2,5,1    i2,3,2i2,4,2i2,5,2    i2,3,3i2,4,3i2,5,3     i2,3,4i2,4,4i2,5,4υ=4

The precoding matrices indicated by the PMI are determined from j=1N0Lσj+Mυ vectors, where {σ1,…,σN0} are the indices of the N0 selected CSI-RS resources in increasing order, such that 1≤σ1<…<σN0≤NTRP, and {Lσ1, …, LσN0} are the corresponding values from the selected combination of {L1, …, LNTRP}.

The Lσj vectors, vm1,ji,m2,j(i), i=0,1,…,Lσj-1, corresponding to the j-th selected CSI-RS resource, for j=1,…,N0, are indicated by i1,1, i1,2, where

i1,1=[i1,1,1 … i1,1,N0]

i1,1,j=[q1,j q2,j]

q1,j∈{0,1,…,O1-1}

q2,j∈{0,1,…,O2-1}

and

i1,2=[i1,2,1 …i1,2,N0]

i1,2,j∈0,1,…, N1N2Lσj-1

Let

n1=[n1,1… n1,N0]

n1,j=n1,j0…n1,jLσj-1

n1,j(i)∈{0,1,…,N1-1}

and

n2=[n2,1… n2,N0]

n2,j=n2,j0…n2,jLσj-1

n2,j(i)∈{0,1,…,N2-1}

the index i1,2,j, for the j-th selected CSI-RS resource, j=1,…,N0, is obtained from the Lσj indices nj(i)=N1n2,j(i)+ n1,j(i), i=0,…,Lσj-1, as described in Clause 5.2.2.2.3 for the indicator i1,2, obtained from the L indices n(i)=N1n2(i)+ n1(i), i=0,…,L-1. Vector vm1,ji,m2,j(i) is then derived from the indices m1,j(i)=O1n1,j(i)+q1,j and m2,j(i)=O2n2,j(i)+q2,j, as described in Clause 5.2.2.2.3 for vector vm1i,m2(i), derived from indices m1(i), m2(i).

The Mυ=pυN3R vectors, y0,lf,y1,lf,…,yN3-1,lfT, f=0,1,…,Mυ-1, for layer l=1,…,υ are common for all the N0 selected CSI-RS resources and are indicated by i1,5 (for N3>19) and i1,6,l for Mυ>1 and l=1,…, υ, which are obtained as described in Clause 5.2.2.2.5, where i1,6,l is derived from the selected vectors identified by the indices n3,l(f), f=0,1,…,Mυ-1. The Mυ vectors’ elements are given by

yt,l(f)=ej2πtn3,l(f)N3

for t=0,1,…N3-1, and f=0,1,…,Mυ-1.

If the higher layer parameter codebookMode is set to 'mode1', an offset dj is reported for the j-th selected CSI-RS resource, with j=2,…,N0, relative to the first of the N0 selected CSI-RS resources. The N0-1 reported offsets are common for all ν layers and are indicated by i1,9, given by

i1,9=d2… dN,  dj∈{0, 1,…,N3O3-1}

where the value of O3∈{1,4} is configured by higher layer parameter numberOfO3. The offsets are represented by

ψ=[ψ2… ψN0]

ψj=djO3

If codebookMode is set to 'mode2’, the offset indicator, i1,9, is not reported and ψj=0 for j=2,…,N0.

The reference amplitude coefficient indicator i2,3,l, for layer l=1,…υ, is given by

i2,3,l=kl,01kl,11

kl,p(1)∈1,…,15

The reference amplitude coefficients for layer l=1,…,υ are represented by

pl(1)=[pl,01 pl,1(1)]

and the mapping from kl,p(1), to pl,p(1) is given in Table 5.2.2.2.5-2.

The amplitude coefficient indicator i2,4,l, for layer l=1,…,υ, is given by

i2,4,l=[i2,4,l,1…i2,4,l,N0]

i2,4,l,j=kl,0,j(2)…kl,Mυ-1,j(2)

kl,f,j(2)=kl,0,f,j(2)…kl,2Lσj-1,f,j(2)

kl,i,f,j(2)∈0,…,7

The amplitude coefficients for layer l=1,…,υ are represented by

pl(2)=[pl,12 … pl,N0(2)]

pl,j(2)=[pl,0,j2…pl,Mν-1,j(2)]

pl,f,j(2)=[pl,0,f,j2…pl,2Lσj-1,f,j(2)]

and the mapping from kl,i,f,j(2) to pl,i,f,j(2) is given in Table 5.2.2.2.5-3.

The phase coefficient indicator i2,5,l, for l=1,…,υ, is given by

i2,5,l=[i2,5,l,1…i2,5,l,N0]

i2,5,l,j=cl,0,j…cl,Mυ-1,j

cl,f,j=cl,0,f,j…cl,2Lσj-1,f,j

cl,i,f,j∈0,…,15

The phase coefficients for layer l=1,…,υ are represented by

φl=[φl,1…φl,N0]

φl,j=φl,0,j…φl,Mυ-1,j

φl,f,j=φl,0,f,j…φl,2Lσj-1,f,j

and the mapping from cl,i,f,j to φl,i,f,j is given by

φl,i,f,j=ej2πcl,i,f,j16

Let K0=2βM1j=1N0Lσj. The bitmap whose nonzero bits identify which coefficients in i2,4,l and i2,5,l are reported for layer l=1,…,υ, is indicated by i1,7,l

i1,7,l=[i1,7,l,1…i1,7,l,N0]

i1,7,l,j=kl,0,j3…kl,Mυ-1,j3

kl,f,j3=kl,0,f,j3…kl,2Lσj-1,f,j3

kl,i,f,j(3)∈0,1

Let KlNZ=j=1N0i=02Lσj-1f=0Mυ-1kl,i,f,j(3)≤K0 be the number of nonzero coefficients for layer l=1,…,υ, the total number of nonzero coefficients is reported and given by KNZ=l=1υKlNZ≤2K0.

The indices of i2,4,l, i2,5,l and i1,7,l are associated to the N0 selected CSI-RS resources.

Let fl*∈0,1,…,Mυ-1 and il*∈0,1,…,2j=1N0Lσj-1 be the indices which identify the strongest coefficient of layer l, for  l=1,…,υ, i.e., the element kl,yl*,fl*,jl*(2) of i2,4,l, where the indices jl*∈{1,…,N0}, yl*∈0,…,2Lσjl*-1 are such that il*=2k=1jl*-1Lσk+yl*. The codebook indices of n3,lkmi are remapped with respect to n3,l(fl*) as n3,l(f)=n3,l(f)-n3,l(fl*)mod N3kmi*=0, such that n3,l(fl*)=0, after remapping. The index f is remapped with respect to fl* as f=f-fl*mod Mυ, such that the index of the strongest coefficient is fl*=0, for each layer l=1,…,υ, after remapping. The indices of i2,4,l, i2,5,l and i1,7,l indicate amplitude coefficients, phase coefficients and bitmap after remapping.

The strongest coefficient of layer l=1,…, υ is identified by

i1,8,l∈0,1,…,2j=1N0Lσj-1and is obtained as follows, where, for υ=1, κ1,I,0(3)=k1,i,0,j(3) and index I is such that I=2k=1j-1Lσk+i

i1,8,l=I=0i1*κ1,I,0(3)-1υ=1il*1<υ≤4

The amplitude and phase coefficient indicators are reported as follows:

-	kl,yl*Lσjl*(1)=15, kl,yl*,0,jl*(2)=7, kl,yl*,0,jl*(3)=1 and cl,yl*,0,jl*=0, for l=1,…,υ. The reference amplitude, kl,yl*Lσjl*(1), amplitude coefficient, kl,yl*,0,jl*(2) and phase coefficient, cl,yl*,0,jl*, are not reported for l=1,…,υ.

-	The reference amplitude, kl,yl*Lσjl*+1 mod 2(1), is reported for l=1,…,υ.

-	The KNZ-υ amplitude coefficients, kl,i,f,j(2), for which kl,i,f,j(3)=1, i≠yl*, f≠0, j≠jl* are reported.

-	The KNZ-υ phase coefficients, cl,i,f,j, for which kl,i,f,j(3)=1, i≠yl*, f≠0, j≠jl* are reported.

-	The remaining 2j=1N0Lσj⋅Mυ⋅ν-KNZ amplitude coefficients, kl,i,f,j(2), are not reported.

-	The remaining 2j=1N0Lσj⋅Mυ⋅ν-KNZ phase coefficients, cl,i,f,j, are not reported.

The codebooks for 1-4 layers are given in Table 5.2.2.2.8-4, where t=0,1,…,N3-1, is the index associated with the precoding matrix, l=1,…, υ is the layer index, and where, for coefficients with kl,i,f,j(3)=0, amplitude and phase are set to zero, i.e., pl,i,f,j(2)=0 and φl,i,f,j=0.

Table 5.2.2.2.8-4: Codebook for 1-layer. 2-layer, 3-layer and 4-layer CSI reporting using antenna ports 3000 to 2999+PCSI-RS of N selected CSI-RS resources

| Layers |  |
| --- | --- |
| υ=1 | Wq1,q2, n1, n2,n3,1,p1(1),p12,φ1,ψ,t(1)=Wq1,q2, n1, n2,n3,1,p1(1),p12,φ1,ψ,t1 |
| υ=2 | Wq1,q2, n1, n2,n3,1,p1(1),p12,φ1, n3,2,p2(1),p22,φ2,ψ,t(2)=12Wq1,q2, n1, n2,n3,1,p1(1),p12,φ1,ψ,t1 Wq1,q2, n1, n2,n3,2,p2(1),p22,φ2,ψ,t2 |
| υ=3 | Wq1,q2, n1, n2,n3,1,p1(1),p12,φ1,n3,2,p2(1),p22,φ2,n3,3,p3(1),p32,φ3,ψ,t(3)=13Wq1,q2, n1, n2,n3,1,p1(1),p12,φ1,ψ,t1 Wq1,q2, n1, n2,n3,2,p2(1),p22,φ2,ψ,t2 Wq1,q2, n1, n2,n3,3,p3(1),p32,φ3,ψ,t3 |
| υ=4 | Wq1,q2, n1, n2,n3,1,p1(1),p12,φ1,n3,2,p2(1),p22,φ2,n3,3,p3(1),p32,φ3,n3,4,p4(1),p42,φ4,ψ,t(4)=12Wq1,q2, n1, n2,n3,1,p1(1),p12,φ1,ψ,t1 Wq1,q2, n1, n2,n3,2,p2(1),p22,φ2,ψ,t2 Wq1,q2, n1, n2,n3,3,p3(1),p32,φ3,ψ,t3 Wq1,q2, n1, n2,n3,4,p4(1),p42,φ4,ψ,t4 |
| Where Wq1,q2, n1, n2,n3,l,pl(1),pl2,φl,ψ,tl=1N1N2γt,li=0Lσ1-1vm1,1(i),m2,1(i)pl,0(1)f=0Mυ-1yt,l(f)pl,i,f,1(2)φl,i,f,1i=0Lσ1-1vm1,1(i),m2,1(i)pl,1(1)f=0Mυ-1yt,l(f)pl,i+Lσ1,f,1(2)φl,i+Lσ1,f,1ej2πtψ2N3i=0Lσ2-1vm1,2(i),m2,2(i)pl,0(1)f=0Mυ-1yt,l(f)pl,i,f,2(2)φl,i,f,2ej2πtψ2N3i=0Lσ2-1vm1,2(i),m2,2(i)pl,1(1)f=0Mυ-1yt,l(f)pl,i+Lσ2,f,2(2)φl,i+Lσ2,f,2⋮ej2πtψN0N3i=0LσN0-1vm1,N0(i),m2,N0(i)pl,0(1)f=0Mυ-1yt,l(f)pl,i,f,N0(2)φl,i,f,N0ej2πtψN0N3i=0LσN0-1vm1,N0(i),m2,N0(i)pl,1(1)f=0Mυ-1yt,l(f)pl,i+LσN0,f,N0(2)φl,i+LσN0,f,N0 γt,l=j=1N0i=02Lσj-1pl,iLσj12f=0Mυ-1yt,lfpl,i,f,j2φl,i,f,j2, l=1,2,3,4, and the mappings from i1 to q1, q2, n1, n2, n3,1, n3,2, n3,3, n3,4, ψ, and from i2 to p1(1), p2(1), p3(1), p4(1), p1(2), p2(2), p3(2), p4(2), φ1, φ2, φ3, φ4 are as described above, including the ranges of the constituent indices of i1 and i2. |  |

The bitmap parameter n1-n2-codebookSubsetRestriction-CJT-r18 is configured per CSI-RS resource and for at least one of the NTRP CSI-RS resources, and it is configured as described in Clause 5.2.2.2.5, where only the bit values ‘00’ or ‘11’ of Table 5.2.2.2.5-6 are configurable. If parameter n1-n2-codebookSubsetRestriction-CJT-r18 is not configured for a CSI-RS resource, no restriction is applied to the selection of vectors vm1,ji,m2,j(i) corresponding to that resource.

##### 5.2.2.2.9	Further enhanced Type II port selection codebook for CJT

For 4 antenna ports {3000, 3001, …, 3003}, 8 antenna ports {3000, 3001, …, 3007}, 12 antenna ports {3000, 3001, …, 3011}, 16 antenna ports {3000, 3001, …, 3015}, 24 antenna ports {3000, 3001, …, 3023}, and 32 antenna ports {3000, 3001, …, 3031} per CSI-RS resource, the UE configured with NTRP∈{1,2,3,4} CSI-RS resources in a resource set for channel measurement and with higher layer parameter codebookType set to 'typeII-CJT-PortSelection-r18'

-	the number of CSI-RS ports for each CSI-RS resource, PCSI-RS, is configured as in clause 5.2.2.2.4.

-	A set of NL∈{1,2,4} combinations of values of {α1,…,αNTRP} is configured by the higher layer parameter paramCombination-CJT-PS-alpha-r18, where the value of NL is configured by the higher layer parameter numberOfSDCombinations-PS and the mapping is given in Table 5.2.2.2.9-1, with the value of αn corresponding to CSI-RS resource n, for n=1,…,NTRP. A single value of M and β is configured by the higher layer parameter paramCombination-CJT-PS-r18, where the mapping is given in Table 5.2.2.2.9-2. The configurable combinations of {α1,…,αNTRP} and {M,β} are marked with 'x' in Table 5.2.2.2.9-3.

-	The UE is not expected to be configured with paramCombination-CJT-PS-alpha-r18 equal to

-	2 for NTRP=1; 4 for NTRP=2; 2, 3 or 4 for NTRP=3; when PCSI-RS∈{4,12},

-	3 for NTRP=1; 5 for NTRP=2; 8 for NTRP=3; 4 for NTRP=4; when paramCombination-CJT-PS-r18 is configured to 4 or 5 and PCSI-RS=32,

-	1 for NTRP=1; 1, 2, 3 for NTRP=2; 1, 2, 3, 4, 5, 6,7 for NTRP=3; 1, 2, 3 for NTRP=4, when PCSI-RS=4 and higher layer parameter typeII-CJT-PS-RI-Restriction-r18 is configured with ri=1 for any i>1.

-	If NL>1, the UE is expected to select one of the NL configured combinations of {α1,…,αNTRP} and report the index of the selected combination, where the index value 0 corresponds to the first configured combination and the index value NL-1 corresponds to the NL-th configured combination. If NL=1, a single combination of {α1,…,αNTRP} is configured and the selection is not reported.

Table 5.2.2.2.9-1: Codebook parameter configurations for {α1,…, αNTRP}

| NTRP | paramCombination-CJT-PS-alpha-r18 | {α1,…, αNTRP} |
| --- | --- | --- |
| 1 | 1 | {1/2} |
|  | 2 | {3/4} |
|  | 3 | {1} |
| 2 | 1 | {1/2,1/2} |
|  | 2 | {1/2,1} |
|  | 3 | {1,1/2} |
|  | 4 | {3/4,3/4} |
|  | 5 | {1,1} |
| 3 | 1 | {1/2,1/2,1/2} |
|  | 2 | {1/2,1/2,3/4} |
|  | 3 | {1/2,3/4,1/2} |
|  | 4 | {3/4,1/2,1/2} |
|  | 5 | {1/2,1/2,1} |
|  | 6 | {1/2,1,1/2} |
|  | 7 | {1,1/2,1/2} |
|  | 8 | {1,1,1} |
| 4 | 1 | {1/2,1/2,1/2,1/2} |
|  | 2 | {1/2,1/2,1/2,1} |
|  | 3 | {1/2,1/2,1,1} |
|  | 4 | {1,1,1,1} |

Table 5.2.2.2.9-2: Codebook parameter configurations for {M,β}

| paramCombination-CJT-PS-r18 | M | β |
| --- | --- | --- |
| 1 | 1 | ½ |
| 2 | 1 | ¾ |
| 3 | 1 | 1 |
| 4 | 2 | ½ |
| 5 | 2 | ¾ |

Table 5.2.2.2.9-3: Configurable combinations of {α1,…, αNTRP} and {M,β}

| NTRP | paramCombination-CJT-PS-alpha-r18 | paramCombination-CJT-PS-r18 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | 5 |
| 1 | 1 |  |  |  | x |  |
|  | 2 | x |  |  | x |  |
|  | 3 | x | x | x | x | x |
| 2 | 1 | x |  |  | x |  |
|  | 2 | x |  |  |  |  |
|  | 3 | x |  |  |  |  |
|  | 4 |  | x |  |  |  |
|  | 5 |  | x |  | x |  |
| 3 | 1 | x |  |  | x |  |
|  | 2 | x |  |  |  |  |
|  | 3 | x |  |  |  |  |
|  | 4 | x |  |  |  |  |
|  | 5 |  | x |  | x |  |
|  | 6 |  | x |  | x |  |
|  | 7 |  | x |  | x |  |
|  | 8 |  | x |  |  | x |
| 4 | 1 | x |  |  |  |  |
|  | 2 | x |  |  |  |  |
|  | 3 |  | x | x | x |  |
|  | 4 |  |  | x |  |  |

-	The value of NMN∈{2,4} is configured with the higher-layer parameter valueOfN-CJT-r18, when M=2.

-	The value of R∈{1,2} is configured with the higher-layer parameter numberOfPMI-SubbandsPerCQI-Subband-CJT-PS-r18, when M=2, and R=1 when M=1, where R and the corresponding value of N3 are defined as in clause 5.2.2.2.5.

-	The UE shall report the RI value υ according to the configured higher layer parameter typeII-CJT-PS-RI-Restriction-r18. The UE shall not report υ>4. The bitmap parameter typeII-CJT-PS-RI-Restriction-r18 forms the bit sequence r3,r2,r1,r0, where r0 is the LSB and r3 is the MSB. When ri is zero, i∈0,1,…, 3, PMI and RI reporting are not allowed to correspond to any precoder associated with υ=i+1 layers.

-	The UE may be configured with higher layer parameter restrictedCMR-Selection. If restrictedCMR-Selection is configured, the number of selected CSI-RS resources is N0=NTRP. Otherwise, the UE is expected to select N0 CSI-RS resources, with 1≤N0≤NTRP, and the selection is reported with an NTRP-bit bitmap, bNTRP, …,b1, where the CSI-RS resources are mapped from bit b1 to bit bNTRP by their ordering in the resource set and the first of the N0 selected CSI-RS resources corresponds to the nonzero bit with lowest index.

The PMI value for the N0 selected CSI-RS resources corresponds to the codebook indices of i1 and i2 where

i1=i1,2i1,6i1,7,1i1,8,1i1,9                                                                                                    υ=1i1,2i1,6i1,7,1i1,8,1i1,7,2i1,8,2i1,9υ=2i1,2i1,6i1,7,1i1,8,1i1,7,2i1,8,2i1,7,3i1,8,3i1,9υ=3i1,2i1,6i1,7,1i1,8,1i1,7,2i1,8,2i1,7,3i1,8,3i1,7,4i1,8,4i1,9υ=4i2=i2,3,1i2,4,1i2,5,1                                                                                                    υ=1i2,3,1i2,4,1i2,5,1    i2,3,2i2,4,2i2,5,2υ=2i2,3,1i2,4,1i2,5,1    i2,3,2i2,4,2i2,5,2    i2,3,3i2,4,3i2,5,3                                               υ=3i2,3,1i2,4,1i2,5,1    i2,3,2i2,4,2i2,5,2    i2,3,3i2,4,3i2,5,3     i2,3,4i2,4,4i2,5,4υ=4

The precoding matrices indicated by the PMI are determined from j=1N0Lσj+M vectors, where {σ1,…,σN0} are the indices of the N0 selected CSI-RS resources in increasing order, such that 1≤σ1<…<σN0≤NTRP, and Lσj=K1,σj/2, K1,σj=ασjPCSI-RS, where {ασ1, …, ασN0} are the corresponding values from the selected combination of {α1, …, αNTRP}.

K1,σj ports are selected from the PCSI-RS ports of the j-th selected CSI-RS resource, for j=1,…,N0, based on Lσj vectors, vmji, i=0,1,…,Lσj-1, which are indicated by i1,2, where

i1,2=[i1,2,1 …i1,2,N0]

i1,2,j∈0,1,…, PCSI-RS/2Lσj-1

Let

m=[m1… mN0]

mj=[mj0…mj(Lσj-1)]

mj(i)∈0,1,…,PCSI-RS2-1

the index i1,2,j, for the j-th selected CSI-RS resource, j=1,…,N0, is obtained from the Lσj elements of mj, as described in Clause 5.2.2.2.7 for the indicator i1,2, obtained from the L elements of m. Vector vmj(i) is a PCSI-RS/2 -element column vector containing a value of 1 in the element of index mj(i) and zeros elsewhere, and where the first element is the element of index 0.

-	If ασj=1 for the j-th selected CSI-RS resource, mj(i)=i, for i=0,1,…,PCSI-RS2-1, and i1,2,j is not reported.

The M vectors, y0f,y1f,…,yN3-1fT, f=0,1,…,M-1, are common for all the N0 selected CSI-RS resources and are identified by n3, where

n3=n30… n3M-1

n3(f)∈{0}M=10,1,…,min⁡(NMN,N3)-1M=2.

with the indices f∈{0,…,M-1} assigned such that n3(f) increases with f. n3 is indicated by the index i1,6, when M=2 and NMN=4, where

i1,6∈0,1,2.

-	If M=1, or M=2 and NMN=2, i1,6 is not reported.

-	If M=2 and NMN=4, the nonzero offset between n3(0) and n3(1) is reported with i1,6 assuming that n3(0) (reference for the offset) is 0. The nonzero offset values are mapped to the index values of i1,6 in increasing order with offset value 1 mapped to index value '0'.

The M vectors’ elements are given by

yt(f)=ej2πtn3(f)N3

for t=0,1,…N3-1, and f=0,…,M-1.

If the higher layer parameter codebookMode is set to 'mode1', an offset dj is reported for the j-th selected CSI-RS resource, with j=2,…,N0, relative to the first of the N0N selected CSI-RS resources. The N0-1 reported offsets are common for all ν layers and are indicated by i1,9, given by

i1,9=[d2… dN0]

dj∈{0, 1,…,N3O3-1}

where the value of O3∈{1,4} is configured by higher layer parameter numberOfO3. The offsets are represented by

ψ=[ψ2… ψN0]

ψj=djO3

If codebookMode is set to 'mode2’, the offset indicator, i1,9, is not reported and ψj=0 for j=1,…,N0.

The reference amplitude coefficient indicator i2,3,l, for layer l=1,…υ, is given by

i2,3,l=kl,01kl,11

kl,p(1)∈1,…,15

The reference amplitude coefficients for layer l=1,…,υ are represented by

pl(1)=[pl,01 pl,1(1)]

and the mapping from kl,p(1) to pl,p(1) is given in Table 5.2.2.2.5-2.

The amplitude coefficient indicator i2,4,l, for layer l=1,…,υ, is given by

i2,4,l=[i2,4,l,1…i2,4,l,N0]

i2,4,l,j=kl,0,j(2)…kl,M-1,j(2)

kl,f,j(2)=kl,0,f,j(2)…kl,K1,σj-1,f,j(2)

kl,i,f,j(2)∈0,…,7

The amplitude coefficients for layer l=1,…,υ are represented by

pl(2)=[pl,12 … pl,N0(2)]

pl,j(2)=[pl,0,j2…pl,M-1,j(2)]

pl,f,j(2)=[pl,0,f,j2…pl,K1,σj-1,f,j(2)]

and the mapping from kl,i,f,j(2) to pl,i,f,j(2) is given in Table 5.2.2.2.5-3.

The phase coefficient indicator i2,5,l, for l=1,…,υ, is given by

i2,5,l=[i2,5,l,1…i2,5,l,N0]

i2,5,l,j=cl,0,j…cl,M-1,j

cl,f,j=cl,0,f,j…cl,K1,σj-1,f,j

cl,i,f,j∈0,…,15

The phase coefficients for layer l=1,…,υ are represented by

φl=[φl,1…φl,N0]

φl,j=φl,0,j…φl,M-1,j

φl,f,j=φl,0,f,j…φl,K1,σj-1,f,j

and the mapping from cl,i,f,j to φl,i,f,j is given by

φl,i,f,j=ej2πcl,i,f,j16

Let K0=βMj=1N0K1,σj. The bitmap whose nonzero bits identify which coefficients in i2,4,l and i2,5,l are reported for layer l=1,…,υ, is indicated by i1,7,l

i1,7,l=[i1,7,l,1…i1,7,l,N0]

i1,7,l,j=kl,0,j3…kl,M-1,j3

kl,f,j3=kl,0,f,j3…kl,K1,σj-1,f,j3

kl,i,f,j(3)∈0,1

Let KlNZ=j=1N0i=0K1,σj-1f=0M-1kl,i,f,j(3)≤K0 be the number of nonzero coefficients for layer l=1,…,υ, the total number of nonzero coefficients is reported and given by KNZ=l=1υKlNZ≤2K0.

-	If υ≤2 and KNZ=Mυj=1N0K1,σj, i1,7,l is not reported, for l=1,…,υ.

The indices of i2,4,l, i2,5,l and i1,7,l are associated to the N0 selected CSI-RS resources.

Let fl*∈0,…,M-1 and il*∈0,1,…,j=1N0K1,σj-1 be the indices which identify the strongest coefficient of layer l, for  l=1,…,υ, i.e., the element kl,yl*,fl*,jl*(2) of i2,4,l, where the indices jl*∈{1,…,N0}, yl*∈0,…,K1,σjl*-1 are such that il*=k=1jl*-1K1,σk+yl*. The strongest coefficient of layer l=1,…,υ is identified by the index

i1,8,l∈0,1,…,Mj=1N0K1,σj-1

which is found from

i1,8,l=fl*j=1N0K1,σj+il*.

The amplitude and phase coefficient indicators are reported as follows:

-	kl,yl*Lσjl*(1)=15, kl,yl*,fl*,jl*(2)=7, kl,yl*,fl*,jl*(3)=1 and cl,yl*,fl*,jl*=0, for l=1,…,υ. The reference amplitude, kl,yl*Lσjl*(1), amplitude coefficient, kl,yl*,fl*,jl*(2) and phase coefficient, cl,yl*,fl*,jl*, are not reported for l=1,…,υ.

-	The reference amplitude, kl,yl*Lσjl*+1 mod 2(1), is reported for l=1,…,υ.

-	The KNZ-υ amplitude coefficients, kl,i,f,j(2), for which kl,i,f,j(3)=1, i≠yl*, f≠fl*, j≠jl* are reported.

-	The KNZ-υ phase coefficients, cl,i,f,j, for which kl,i,f,j(3)=1, i≠yl*, f≠fl*, j≠jl* are reported.

-	The remaining j=1N0K1,σj⋅M⋅υ-KNZ amplitude coefficients, kl,i,f,j(2), are not reported.

-	The remaining j=1N0K1,σj⋅M⋅υ-KNZ phase coefficients, cl,i,f,j, are not reported.

The codebooks for 1-4 layers are given in Table 5.2.2.2.9-4, where t=0,1,…,N3-1, is the index associated with the precoding matrix, l=1,…, υ is the layer index, and where, for coefficients with kl,i,f,j(3)=0, amplitude and phase are set to zero, i.e., pl,i,f,j(2)=0 and φl,i,f,j=0.

Table 5.2.2.2.9-4: Codebook for 1-layer. 2-layer, 3-layer and 4-layer CSI reporting using antenna ports 3000 to 2999+PCSI-RS of N selected CSI-RS resources

| Layers |  |
| --- | --- |
| υ=1 | Wm,n3,p1(1),p12,φ1,ψ,t(1)=Wm,n3,p1(1),p12,φ1,ψ,t1 |
| υ=2 | Wm,n3,p1(1),p12,φ1,p2(1),p22,φ2,ψ,t(2)=12Wm,n3,p1(1),p12,φ1,ψ,t1 Wm,n3,p2(1),p22,φ2,ψ,t2 |
| υ=3 | Wm,n3,p1(1),p12,φ1,p2(1),p22,φ2,p3(1),p32,φ3,ψ,t(3)=13Wm,n3,p1(1),p12,φ1,ψ,t1 Wm,n3,p2(1),p22,φ2,ψ,t2 Wm,n3,p3(1),p32,φ3,ψ,t3 |
| υ=4 | Wm,n3,p1(1),p12,φ1,p2(1),p22,φ2,p3(1),p32,φ3,p4(1),p42,φ4,ψ,t(4)=12Wm,n3,p1(1),p12,φ1,ψ,t1 Wm,n3,p2(1),p22,φ2,ψ,t2 Wm,n3,p3(1),p32,φ3,ψ,t3 Wm,n3,p4(1),p42,φ4,ψ,t4 |
| Where Wm,n3,pl(1),pl2,φl,ψ,tl=1γt,li=0Lσ1-1vm1(i)pl,0(1)f=0M-1yt(f)pl,i,f,1(2)φl,i,f,1i=0Lσ1-1vm1(i)pl,1(1)f=0M-1yt,l(f)pl,i+Lσ1,f,1(2)φl,i+Lσ1,f,1ej2πtψ2N3i=0Lσ2-1vm2(i)pl,0(1)f=0M-1yt(f)pl,i,f,2(2)φl,i,f,2ej2πtψ2N3i=0Lσ2-1vm2(i)pl,1(1)f=0M-1yt(f)pl,i+Lσ2,f,2(2)φl,i+Lσ2,f,2⋮ej2πtψN0N3i=0LσN0-1vmN0(i)pl,0(1)f=0M-1yt(f)pl,i,f,N0(2)φl,i,f,N0ej2πtψN0N3i=0LσN0-1vmN0(i)pl,1(1)f=0M-1yt(f)pl,i+LσN0,f,N0(2)φl,i+LσN0,f,N0, γt,l=j=1N0i=02Lσj-1pl,iLσj12f=0Mυ-1yt,lfpl,i,f,j2φl,i,f,j2, l=1,2,3,4 and the mappings from i1 to m, n3, ψ and from i2 to p1(1), p2(1), p3(1), p4(1), p1(2), p2(2), p3(2), p4(2), φ1, φ2, φ3, φ4 are as described above, including the ranges of the constituent indices of i1 and i2. |  |

<omitted text>

#### 5.2.2.5	CSI reference resource definition

The CSI reference resource for a serving cell is defined as follows:

-	In the frequency domain, the CSI reference resource is defined by the group of downlink physical resource blocks corresponding to the band to which the derived CSI relates.

-	In the time domain, the CSI reference resource for a CSI reporting in uplink slot n' is defined by a single downlink slot n-nCSI_ref-Koffset⋅2μDL2μKoffset, where Koffset is a parameter configured by higher layer as specified in clause 4.2 of [6 TS 38.213], and where μKoffsetis the subcarrier spacing configuration for Koffset with a value of 0 for frequency range 1,

-	where n=n'⋅2μDL2μUL +Nslot,offset,ULCA2μoffset,UL-Nslot,offset,DLCA2μoffset,DL∙2μDL and μDLand μUL are the subcarrier spacing configurations for DL and UL, respectively, and Nslot, offsetCA and  are determined by higher-layer configured ca-SlotOffset for the cells transmitting the uplink and downlink, as defined in clause 4.5 of [4, TS 38.211]

-	where for periodic and semi-persistent CSI reporting

-	if a single CSI-RS/SSB resource is configured for channel measurement nCSI_ref is the smallest value greater than or equal to 4⋅2µDL, such that it corresponds to a valid downlink slot, or

-	if multiple CSI-RS/SSB resources are configured for channel measurement nCSI_ref is the smallest value greater than or equal to 5⋅2μDL, such that it corresponds to a valid downlink slot.

-	where for aperiodic CSI reporting, if the UE is indicated by the DCI to report CSI in the same slot as the CSI request, nCSI_ref is such that the reference resource is in the same valid downlink slot as the corresponding CSI request, otherwise nCSI_ref is the smallest value greater than or equal to Z'/Nsymbslot, such that slot n- nCSI_ref corresponds to a valid downlink slot, where Z' corresponds to the delay requirement as defined in Clause 5.4.

-	when periodic or semi-persistent CSI-RS/CSI-IM or SSB is used for channel/interference measurements, the UE is not expected to measure channel/interference on the CSI-RS/CSI-IM/SSB whose last OFDM symbol is received up to Z' symbols before transmission time of the first OFDM symbol of the aperiodic CSI reporting.

A slot in a serving cell shall be considered to be a valid downlink slot if:

-	it comprises at least one higher layer configured downlink or flexible symbol, and

-	it does not fall within a configured measurement gap for that UE

If there is no valid downlink slot for the CSI reference resource corresponding to a CSI Report Setting in a serving cell, CSI reporting is omitted for the serving cell in uplink slot n'.

After the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement no later than CSI reference resource and drops the report otherwise.

For a CSI-ReportConfig configured with codebookType set to 'typeII-CJT-r18' or 'typeII-CJT-PortSelection-r18', after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for each of the CSI-RS resources in the corresponding CSI-RS Resource Set for channel measurement and one CSI-RS and/or CSI-IM resource transmission occasion for the CSI-RS and/or CSI-IM resource in the corresponding Resource Set for interference measurement no later than the CSI reference resource and within the same DRX Active Time, when DRX is configured, and drops the report otherwise.

For a CSI-ReportConfig configured with codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18', after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only if receiving at least one aperiodic or Kp periodic or semipersistent consecutive CSI-RS transmission occasions for each CSI-RS resource in the corresponding CSI-RS Resource Set for channel measurement and one CSI-RS and/or CSI-IM resource transmission occasion for the CSI-RS and/or CSI-IM resource in the corresponding Resource Set for interference measurement no later than the CSI reference resource and within the same DRX Active Time, when DRX is configured, and drops the report otherwise. The value of Kp∈{1,2,4} is indicated by UE capability, as defined in clause 5.2.1.6.

For a CSI-ReportConfig configured with the higher layer parameter reportQuantity set to 'tdcp', after the CSI report (re)configuration, serving cell activation, BWP change, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for each CSI-RS resource in the KTRS CSI-RS Resource Sets of the CSI-RS Resource Setting for channel measurement no later than the CSI reference resource within the same DRX active time, when DRX is configured, and drop the report otherwise.

When DRX is configured, the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement in DRX Active Time no later than CSI reference resource and drops the report otherwise. When DRX is configured and the CSI-RS Resource Set for channel measurement corresponding to a CSI report is configured with two Resource Groups and N Resource Pairs, as described in clause 5.2.1.4.1, the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion for each CSI-RS resource in a Resource Pair within the same DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitOtherPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to quantities other than 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP- Index', and 'ssb-Index-RSRP- Index ' when drx-onDurationTimer is not started, the UE shall report CSI during the time duration indicated by drx-onDurationTimer in DRX-Config also outside active time according to the procedure described in Clause 5.2.1.4 if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer in DRX-Config outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitPeriodicL1-RSRP to report L1-RSRP with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP- Index', or 'ssb-Index-RSRP- Index' when drx-onDurationTimer is not started, the UE shall report L1-RSRP during the time duration indicated by drx-onDurationTimer in DRX-Config also outside active time according to the procedure described in clause 5.2.1.4 and when reportQuantity set to 'cri-RSRP' or 'cri-RSRP- Index' if receiving at least one CSI-RS transmission occasion for channel measurement during the time duration indicated by drx-onDurationTimer in DRX-Config outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise.

When deriving CSI feedback, the UE is not expected that a NZP CSI -RS resource for channel measurement overlaps with CSI-IM resource for interference measurement or NZP CSI -RS resource for interference measurement.

#### 5.2.2.5.1	UE assumptions for CQI/PMI/RI calculation

If configured to report CQI index, in the CSI reference resource, or in each of the slot(s) associated with a CQI in the predicted CSI, as defined in Clause 5.2.1.4.2, the UE shall assume the following for the purpose of deriving the CQI index, and if also configured, for deriving PMI and RI:

-	The first 2 OFDM symbols are occupied by control signaling.

-	The number of PDSCH and DM-RS symbols is equal to 12.

-	The same bandwidth part subcarrier spacing configured as for the PDSCH reception

-	The bandwidth as configured for the corresponding CQI report.

-	The IAB-MT shall only assume the frequency resources as indicated by the DL TX power adjustment MAC CE, if indicated for the slot of the CSI reference resource by DL Tx Power Adjustment MAC CE as described in [10, TS 38.321].

-	The reference resource uses the CP length and subcarrier spacing configured for PDSCH reception

-	No resource elements used by primary or secondary synchronization signals or PBCH.

-	Redundancy Version 0.

-	The ratio of PDSCH EPRE to CSI-RS EPRE is as given in Clause 5.2.2.3.1.

-	In addition, the IAB-MT shall apply the provided DL TX power adjustment, if indicated for the slot of the CSI reference resource by DL Tx Power Adjustment MAC CE as described in [10, TS 38.321].

-	Assume no REs allocated for NZP CSI-RS and ZP CSI-RS.

-	Assume the same number of front-loaded DM-RS symbols as the maximum front-loaded symbols configured by the higher layer parameter maxLength in DMRS-DownlinkConfig.

-	Assume the same number of additional DM-RS symbols as the additional symbols configured by the higher layer parameter dmrs-AdditionalPosition.

-	Assume the PDSCH symbols are not containing DM-RS.

-	Assume PRB bundling size of 2 PRBs.

-	The PDSCH transmission scheme where the UE may assume that PDSCH transmission would be performed with up to 8 transmission layers as defined in Clause 7.3.1.4 of [4, TS 38.211]. For CQI calculation, the UE should assume that PDSCH signals on antenna ports in the set [1000,…, 1000+ν-1] for ν layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…, 3000+P-1], as given by

y3000(i)⋯y3000+P-1(i)=W(i)x0(i)⋯xν-1(i)

where  is a vector of PDSCH symbols from the layer mapping defined in Clause 7.3.1.4 of [4, TS 38.211],  is the number of CSI-RS ports. If only one CSI-RS port is configured, W(i) is 1. If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to either 'cri-RI-PMI-CQI' or 'cri-RI-LI-PMI-CQI', W(i) is the precoding matrix corresponding to the reported PMI applicable to x(i). If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-CQI', W(i) is the precoding matrix corresponding to the procedure described in Clause 5.2.1.4.2. If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-i1-CQI', W(i) is the precoding matrix corresponding to the reported i1 according to the procedure described in Clause 5.2.1.4.2. The corresponding PDSCH signals transmitted on antenna ports [3000,…,3000 + P - 1] would have a ratio of EPRE to CSI-RS EPRE equal to the ratio given in Clause 5.2.2.3.1.

#### 5.2.2.5.1a	UE assumptions for CQI/PMI/RI calculation for NCJT

If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to either 'cri-RI-PMI-CQI' or 'cri-RI-LI-PMI-CQI', the corresponding CSI-RS Resource Set for channel measurement is configured with two Resource Groups and N Resource Pairs, as described in clause 5.2.1.4.1, the reported CRI corresponds to an entry of the N Resource Pairs, and the reported rank combination is {ν1,ν2}, as described in clause 5.2.1.4.2, for CQI calculation, the UE should assume that

-	PDSCH signals on antenna ports in the set [1000,…,1000+ν1-1] for ν1 layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…,3000+P-1] of the Group 1 CSI-RS resource in the Resource Pair indicated by the CRI, and PDSCH signals on antenna ports in the set [1000+ν1,…,1000+ν1+ν2-1] for ν2 layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…,3000+P-1] of the Group 2 CSI-RS resource in the Resource Pair indicated by the CRI, as given by

yj3000(i)⋮yj3000+P-1(i)=Wjix(j-1)⋅ν1i⋮xν1+(j-1)⋅ν2-1i

where Wj(i), j=1,2 are the two precoding matrices corresponding to the two reported PMIs applicable to x(i), as described in clause 5.2.1.4.2; and the indices j=1,2 are associated to the two Resource Groups configured in the corresponding CSI-RS Resource Set for channel measurement; that the signals yj, j=1,2, fully overlap in time and frequency, and that, for the calculation of RI, PMI and LI (if configured) of νj layers, j=1,2, the interference from the other νj mod 2+1 layers is derived from channel measurement and precoding matrix corresponding to the other νj mod 2+1 layers.

-	The UE shall assume that the corresponding PDSCH signals for νj layers transmitted on the P antenna ports of the CSI-RS resource in Group j would have a ratio of EPRE to CSI-RS EPRE equal to the powerControlOffset of the respective CSI-RS resource, for j=1,2.

#### 5.2.2.5.1b	UE assumptions for CQI/PMI/RI calculation for CJT

If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-PMI-CQI', the higher layer parameter codebookType is set to 'typeII-CJT-r18' or ' typeII-CJT-PortSelection-r18', and the corresponding CSI-RS Resource Set for channel measurement is configured with 1≤NTRP≤4 CSI-RS resources, for CQI calculation

-	a UE should assume PDSCH signals on antenna ports in the set [1000,…,1000+υ-1] for υ layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…,3000+P-1] of each of the N selected CSI-RS resources, as given by

yσ13000(i)⋮yσ13000+P-1(i)yσ23000(i)⋮yσ23000+P-1(i)⋮yσN03000(i)⋮yσN03000+P-1(i)=Wix0i⋮xυ-1i

where W(i) is the precoding matrix corresponding to the procedure described in Clause 5.2.2.2.8 and 5.2.2.2.9 for codebookType set to 'typeII-CJT-r18' and ' typeII-CJT-PortSelection-r18', respectively, and {σ1,…,σN0} are the indices of the N selected CSI-RS resources in increasing order, such that 1≤σ1<…<σN0≤NTRP. A UE should assume that the signals yσj, j=1,…,N0, fully overlap in time and frequency.

-	a UE can assume that the PDSCH signals for υ layers Pσjwould have the same ratio of EPRE to CSI-RS EPRE for all CSI-RS resources σj, with j=1,…,N0, equal to the powerControlOffset of the respective CSI-RS resource.

### 5.2.3	CSI reporting using PUSCH

A UE shall perform aperiodic CSI reporting using PUSCH on serving cell c upon successful decoding of a DCI format 0_1 or DCI format 0_2 which triggers an aperiodic CSI trigger state.

When a DCI format 0_1 schedules two PUSCH allocations, the aperiodic CSI report is carried on the second scheduled PUSCH. When a DCI format 0_1 schedules more than two PUSCH allocations, the aperiodic CSI report is carried on the penultimate scheduled PUSCH.

An aperiodic CSI report carried on the PUSCH supports wideband, and sub-band frequency granularities. An aperiodic CSI report carried on the PUSCH supports Type I, Type II, Enhanced Type II, Further Enhanced Type II Port Selection, Enhanced Type II for CJT, Further Enhanced Type II Port Selection for CJT, Enhanced Type II for predicted PMI, Further Enhanced Type II Port Selection for predicted PMI and TDCP reporting.

A UE shall perform semi-persistent CSI reporting on the PUSCH upon successful decoding of a DCI format 0_1 or DCI format 0_2 which activates a semi-persistent CSI trigger state. DCI format 0_1 and DCI format 0_2 contains a CSI request field which indicates the semi-persistent CSI trigger state to activate or deactivate. Semi-persistent CSI reporting on the PUSCH supports Type I, Type II with wideband, and sub-band frequency granularities, Enhanced Type II, Further Enhanced Type II Port Selection CSI, Enhanced Type II for CJT, Further Enhanced Type II Port Selection for CJT, Enhanced Type II for predicted PMI and Further Enhanced Type II Port Selection for predicted PMI. The PUSCH resources and MCS shall be allocated semi-persistently by an uplink DCI.

CSI reporting on PUSCH can be multiplexed with uplink data on PUSCH except that semi-persistent CSI reporting on PUSCH activated by a DCI format is not expected to be multiplexed with uplink data on the PUSCH. CSI reporting on PUSCH can also be performed without any multiplexing with uplink data from the UE.

Type I CSI feedback is supported for CSI Reporting on PUSCH. Type I wideband and sub-band CSI is supported for CSI Reporting on the PUSCH. Type II CSI is supported for CSI Reporting on the PUSCH.

For Type I, Type II, Enhanced Type II, Further Enhanced Type II Port Selection CSI, Enhanced Type II for CJT, Further Enhanced Type II Port Selection for CJT, Enhanced Type II for predicted PMI and Further Enhanced Type II Port Selection for predicted PMI feedback on PUSCH, a CSI report comprises of two parts. Part 1 has a fixed payload size and is used to identify the number of information bits in Part 2. Part 1 shall be transmitted in its entirety before Part 2.

-	For Type I CSI feedback, Part 1 contains RI (if reported), CRI (if reported), CQI for the first codeword (if reported). Part 2 contains PMI (if reported), LI (if reported) and contains the CQI for the second codeword (if reported) when RI is larger than 4. For a CSI-ReportConfig configured with codebookType set to 'typeI-SinglePanel' and the corresponding CSI-RS Resource Set for channel measurement configured with two Resource Groups and N Resource Pairs, Part 1 contains RI(s), CRI(s), CQI(s) for the first codeword and is zero padded to a fixed payload size (if needed). Part 2 contains the CQI(s) for the second codeword (if reported) when RI is larger than 4, LIs (if reported) and PMI(s).

-	For Type II CSI feedback, Part 1 contains RI (if reported), CQI, and an indication of the number of non-zero wideband amplitude coefficients per layer for the Type II CSI (see Clause 5.2.2.2.3). The fields of Part 1 – RI (if reported), CQI, and the indication of the number of non-zero wideband amplitude coefficients for each layer – are separately encoded. Part 2 contains the PMI and LI (if reported) of the Type II CSI. The elements of i1,4,l, i2,1,l (if reported) and i2,2,l (if reported) are reported in the increasing order of their indices, i=0,1,…, 2L-1, where the element of the lowest index is mapped to the most significant bits and the element of the highest index is mapped to the least significant bits. Part 1 and 2 are separately encoded.

-	For Enhanced Type II CSI feedback (see Clause 5.2.2.2.5), Further Enhanced Type II Port Selection CSI feedback (see Clause 5.2.2.2.7), Enhanced Type II for predicted PMI with N4=1 (see Clause 5.2.2.2.10) and Further Enhanced Type II Port Selection for predicted PMI (see Clause 5.2.2.2.11), Part 1 contains RI (if reported), CQI, and the total number of reported non-zero amplitude coefficients across layers. The fields of Part 1 – RI (if reported), CQI, and the total number of reported non-zero amplitude coefficients across layers – are separately encoded. Part 2 contains the PMI of the Enhanced Type II, Further Enhanced Type II Port Selection CSI, Enhanced Type II for predicted PMI with N4=1 or Further Enhanced Type II Port Selection for predicted PMI. Part 1 and 2 are separately encoded.

-	For Enhanced Type II for CJT (see Clause 5.2.2.2.8) and Further Enhanced Type II Port Selection for CJT (see Clause 5.2.2.2.9), Part 1 contains RI (if reported), CQI, the total number of reported non-zero amplitude coefficients across layers, the bitmap selecting N CSI-RS resources (if reported) and the selected combination of {L1,…,LNTRP} or {α1,…,αNTRP} (if reported). The fields of Part 1 – RI (if reported), CQI, the total number of reported non-zero amplitude coefficients across layers, the bitmap selecting N CSI-RS resources (if reported) and the selected combination of {L1,…,LNTRP} or {α1,…,αNTRP} (if reported) – are separately encoded. Part 2 contains the PMI of the Enhanced Type II for CJT or Further Enhanced Type II Port Selection for CJT. Part 1 and 2 are separately encoded.

-	For Enhanced Type II for predicted PMI with N4>1 (see Clause 5.2.2.2.10), Part 1 contains RI (if reported), the CQI (if the higher layer parameter TDCQI is set to '1-1' or '1-2') or the first CQI (if the higher layer parameter TDCQI is set to '2') and the total number of reported non-zero amplitude coefficients across layers. The fields of Part 1 – RI (if reported), CQI, and the total number of reported non-zero amplitude coefficients across layers – are separately encoded. Part 2 contains the second CQI (if the higher layer parameter TDCQI is set to '2') and the PMI of the Enhanced Type II for predicted PMI with N4>1. Part 1 and 2 are separately encoded.

A Type II CSI report that is carried on the PUSCH shall be computed independently from any Type II CSI report that is carried on the PUCCH formats 3 or 4 (see Clause 5.2.4 and 5.2.2).

When the higher layer parameter reportQuantity is configured with one of the values 'cri-RSRP', 'ssb-Index-RSRP', 'cri-SINR' or 'ssb-Index-SINR', or 'cri-RSRP-Index', 'ssb-Index-RSRP-Index ', 'cri-SINR-Index ', 'ssb-Index-SINR-Index', 'tdcp', the CSI feedback consists of a single part.

For both Type I and Type II reports configured for PUCCH but transmitted on PUSCH, the determination of the payload for CSI part 1 and CSI part 2 follows that of PUCCH as described in Clause 5.2.4.

When CSI reporting on PUSCH comprises two parts, the UE may omit a portion of the Part 2 CSI. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where  is the number of CSI reports configured to be carried on the PUSCH. Priority 0 is the highest priority and priority  is the lowest priority and the CSI report n corresponds to the CSI report with the nth smallest Prii,CSI(y,k,c,s) value among the  CSI reports as defined in Clause 5.2.5. The subbands for a given CSI report n indicated by the higher layer parameter csi-ReportingBand with value '1' are numbered continuously in increasing order with the lowest subband of csi-ReportingBand with value set to '1' as subband 0. When omitting Part 2 CSI information for a particular priority level, the UE shall omit all of the information at that priority level.

-	For Enhanced Type II and Enhanced Type II for predicted PMI configured with higher layer parameter N4=1, for a given CSI report n, each reported element of i2,4,l i2,5,l and i1,7,l, indexed by l,i and f, is associated with a priority value Pril,i,f=2⋅L⋅υ⋅πf+υ⋅i+l, with π(f)=min⁡(2⋅n3,lf,2⋅N3-n3,l(f)-1) with l=1,2,…,υ, i=0,1,…,2L-1, and f=0,1,…,Mυ-1, and where n3,l(f) is defined in Clause 5.2.2.2.5. The element with the highest priority has the lowest associated value Pril,i,f. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where

-	Group 0 includes indices i1,1 (if reported), i1,2 (if reported) and i1,8,l (l=1,…,υ).

-	Group 1 includes indices i1,5 (if reported), i1,6,l (if reported), the υ2LMυ-KNZ/2 highest priority elements of i1,7,l,  i2,3,l, the max0,KNZ2-υ highest priority elements of i2,4,l and the max0,KNZ2-υ highest priority elements of i2,5,l (l=1,…,υ).

-	Group 2 includes the KNZ/2 lowest priority elements of i1,7,l, the minKNZ-υ,KNZ2 lowest priority elements of i2,4,l and the minKNZ-υ,KNZ2 lowest priority elements of i2,5,l (l=1,…,υ).

-	For Further Enhanced Type II Port Selection and Further Enhanced Type II Port Selection for predicted PMI, for a given CSI report n, each reported element of i2,4,l i2,5,l and i1,7,l, indexed by l, i and f, is associated with a priority value Pril,i,f=K1⋅υ⋅f+υ⋅i+l, with l=1,2,…,υ, i=0,1,…,K1-1 and f=0,…,M-1. The element with the highest priority has the lowest associated value Pril,i,f. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where:

-	Group 0 includes i1,2 (if reported), i1,8,l (l=1,…,υ) and i1,6 (if reported).

-	Group 1 includes the υK1M-KNZ/2 highest priority elements of i1,7,l (if reported), i2,3,l, the max0,KNZ2-υ highest priority elements of i2,4,l and the  max0,KNZ2-υ highest priority elements of i2,5,l (l=1,…,υ).

-	Group 2 includes the KNZ/2 lowest priority elements of i1,7,l (if reported), the minKNZ-υ,KNZ2 lowest priority elements of i2,4,l and the minKNZ-υ,KNZ2 lowest priority elements of i2,5,l (l=1,…,υ).

-	For Enhanced Type II for CJT reports, for a given CSI report n, each reported element of i2,4,l i2,5,l and i1,7,l, indexed by l,ij, f and j, is associated with a priority value Pril,ij,f,j=2⋅k=1N0NLσk⋅υ⋅πf+υ⋅2k=1j-1Lσk+ij+l, with π(f)=min⁡(2⋅n3,lf,2⋅N3-n3,l(f)-1), for l=1,…,υ, ij=0,1,…,2Lσj-1, f=0,1,…,Mυ-1 and j=1,…,N0N,  and where n3,l(f) and Lσj are defined in Clause 5.2.2.2.8. The element with the highest priority has the lowest associated value Pril,ij,f,j. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where

-	Group 0 includes indices i1,1 (if reported), i1,2 (if reported) and i1,8,l (l=1,…,υ).

-	Group 1 includes indices i1,5 (if reported), i1,6,l (if reported), the υ2Mυk=1N0NLσk-KNZ/2 highest priority elements of i1,7,l,  i2,3,l, the max0,KNZ2-υ highest priority elements of i2,4,l, the max0,KNZ2-υ highest priority elements of i2,5,l (l=1,…,υ) and i1,9 (if reported).

-	Group 2 includes the KNZ/2 lowest priority elements of i1,7,l, the minKNZ-υ,KNZ2 lowest priority elements of i2,4,l and the minKNZ-υ,KNZ2 lowest priority elements of i2,5,l (l=1,…,υ).

-	For Further Enhanced Type II Port Selection for CJT reports, for a given CSI report n, each reported element of i2,4,l i2,5,l and i1,7,l, indexed by l, ij, f and j, is associated with a priority value Pril,ij,f,j=k=1N0NK1,σkk⋅υ⋅f+υ⋅k=1j-1K1,σkk+ij+l, for l=1,…,υ, ij=0,1,…,K1,σjj-1, f=0,…,M-1 and j=1,…,N0N,  and where K1,σjj is defined in Clause 5.2.2.2.8. The element with the highest priority has the lowest associated value Pril,ij,f,j. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where:

-	Group 0 includes i1,2 (if reported), i1,8,l (l=1,…,υ) and i1,6 (if reported).

-	Group 1 includes the υMk=1N0NK1,σkk-KNZ/2 highest priority elements of i1,7,l (if reported), i2,3,l, the max0,KNZ2-υ highest priority elements of i2,4,l, the  max0,KNZ2-υ highest priority elements of i2,5,l (l=1,…,υ) and i1,9 (if reported).

-	Group 2 includes the KNZ/2 lowest priority elements of i1,7,l (if reported), the minKNZ-ν,KNZ2 lowest priority elements of i2,4,l and the minKNZ-υ,KNZ2 lowest priority elements of i2,5,l (l=1,…,υ).

-	For Enhanced Type II for predicted PMI configured with N4>1, for a given CSI report n, each reported element of  i2,4,l i2,5,l and i1,7,l, indexed by l,i, f and j, is associated with a priority value Pril,i,f,j=2L⋅υ⋅Mν⋅j+2L⋅υ⋅f+υ⋅i+l, for l=1,…,υ, i=0,1,…,2L-1, f=0,1,…,Mυ-1 and j=0,1. The element with the highest priority has the lowest associated value Pril,i,f,j. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where

-	Group 0 includes indices i1,1 (if reported), i1,2 (if reported), i1,8,l (l=1,…,υ) and the second wideband CQI (if reported).

-	Group 1 includes indices i1,5 (if reported), i1,6,l (if reported), the υ2LMυQ-KNZ/2 highest priority elements of i1,7,l,  i2,3,l, the max0,KNZ2-υ highest priority elements of i2,4,l, the max0,KNZ2-υ highest priority elements of i2,5,l (l=1,…,υ), i1,10,l (if reported) and the second subband CQI of even subbands (if reported).

-	Group 2 includes the KNZ/2 lowest priority elements of i1,7,l, the minKNZ-υ,KNZ2 lowest priority elements of i2,4,l, the minKNZ-υ,KNZ2 lowest priority elements of i2,5,l (l=1,…,υ) and the second subband CQI of odd subbands (if reported).

Table 5.2.3-1: Priority reporting levels for Part 2 CSI

Priority 0: For CSI reports 1 to NRep, Group 0 CSI for CSI reports configured as 'typeII-r16', 'typeII-PortSelection-r16', 'typeII-PortSelection-r17', 'typeII-CJT-r18', 'typeII-CJT-PortSelection-r18', 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18'; Part 2 wideband CSI for CSI reports configured otherwise

Priority 1: Group 1 CSI for CSI report 1, if configured as 'typeII-r16', 'typeII-PortSelection-r16', 'typeII-PortSelection-r17', 'typeII-CJT-r18', 'typeII-CJT-PortSelection-r18', 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18'; Part 2 subband CSI of even subbands for CSI report 1, if configured otherwise

Priority 2: Group 2 CSI for CSI report 1, if configured as 'typeII-r16', 'typeII-PortSelection-r16', 'typeII-PortSelection-r17', 'typeII-CJT-r18', 'typeII-CJT-PortSelection-r18', 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18'; Part 2 subband CSI of odd subbands for CSI report 1, if configured otherwise

Priority 3: Group 1 CSI for CSI report 2, if configured as 'typeII-r16', 'typeII-PortSelection-r16', 'typeII-PortSelection-r17', 'typeII-CJT-r18', 'typeII-CJT-PortSelection-r18', 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18'; Part 2 subband CSI of even subbands for CSI report 2, if configured otherwise

Priority 4: Group 2 CSI for CSI report 2, if configured as 'typeII-r16', 'typeII-PortSelection-r16', 'typeII-PortSelection-r17', 'typeII-CJT-r18', 'typeII-CJT-PortSelection-r18', 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18'. Part 2 subband CSI of odd subbands for CSI report 2, if configured otherwise

⁞

Priority 2NRep-1: Group 1 CSI for CSI report NRep, if configured as 'typeII-r16', 'typeII-PortSelection-r16', 'typeII-PortSelection-r17', 'typeII-CJT-r18', 'typeII-CJT-PortSelection-r18', 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18'; Part 2 subband CSI of even subbands for CSI report NRep, if configured otherwise

Priority 2NRep: Group 2 CSI for CSI report NRep, if configured as 'typeII-r16', 'typeII-PortSelection-r16', 'typeII-PortSelection-r17', 'typeII-CJT-r18', 'typeII-CJT-PortSelection-r18', 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18'; Part 2 subband CSI of odd subbands for CSI report NRep, if configured otherwise

When the UE is scheduled to transmit a transport block on PUSCH not using repetition type B multiplexed with a CSI report(s), Part 2 CSI is omitted only when  is larger than α∙l=0Nsymb,allPUSCH-1MscUCIl-QACK/CG-UCI'-QCSI-1', where parameters , , , , , , , , QACK/CG-UCI' and are defined in Clause 6.3.2.4 of [5, TS 38.212].

Part 2 CSI is omitted level by level, beginning with the lowest priority level until the lowest priority level is reached which causes the  to be less than or equal to α∙l=0Nsymb,allPUSCH-1MscUCIl-QACK/CG-UCI'-QCSI-1'.

When the UE is scheduled to transmit a transport block on PUSCH using repetition type B multiplexed with a CSI report(s), Part 2 CSI is omitted only when

OCSI-2+LCSI-2∙βoffsetPUSCH∙l=0Nsymb,nominalPUSCH-1Msc,nominalUCIlr=0CUL-SCH-1Kr

is larger than

minα∙l=0Nsymb,nominalPUSCH-1Msc,nominalUCIl-QACK/CG-UCI'-QCSI-1' ,  l=0Nsymb,actualPUSCH-1Msc,actualUCIl-QACK/CG-UCI'-QCSI-1',

where parameters , , , Nsymb,nominalPUSCH, Nsymb,actualPUSCH, Msc,nominalUCIl, Msc,actualUCIl, , , QACK/CG-UCI', , and are defined in Clause 6.3.2.4 of [5, TS 38.212].

Part 2 CSI is omitted level by level, beginning with the lowest priority level until the lowest priority level is reached which causes

OCSI-2+LCSI-2∙βoffsetPUSCH∙l=0Nsymb,nominalPUSCH-1Msc,nominalUCIlr=0CUL-SCH-1Kr

to be less than or equal to

minα∙l=0Nsymb,nominalPUSCH-1Msc,nominalUCIl-QACK/CG-UCI'-QCSI-1' ,  l=0Nsymb,actualPUSCH-1Msc,actualUCIl-QACK/CG-UCI'-QCSI-1'.

When part 2 CSI is transmitted on PUSCH with no transport block, lower priority bits are omitted until Part 2 CSI code rate, which is given by OCSI-2+LCSI-2/(NL⋅Q'CSI,2⋅Qm) where OCSI-2, LCSI-2, NL,Q'CSI,2,Qm are given in clause 6.3.2.4 of [5, 38.212] before HARQ-ACK puncturing part 2 CSI if any, is below a threshold code rate lower than one, where

-	is the CSI offset value from Table 9.3-2 of [6, TS 38.213]

-	R is signaled code rate in DCI

If the UE is in an active semi-persistent CSI reporting configuration on PUSCH, the CSI reporting is deactivated when either the downlink BWP or the uplink BWP is changed. Another activation command is required to enable the semi-persistent CSI reporting.

<omitted text>