3GPP TSG-RAN WG1 Meeting #115 	R1-2312738

Chicago, USA, November 13-17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0164 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on Rel-18 NR sidelink evolution in 38.212 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-11-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Capture the agreements on Rel-18 sidelink evolution from RAN1#114bis and RAN1#115 meetings. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Section 7.3.1.4.1: Reflect agreements related to DCI format 3_0 on “Lowest index of the RB set allocation to the initial PSSCH transmission”. - Section 8.3.1.1: Reflect agreements related to SCI format 1-A on “COT sharing flag” and “2nd-stage SCI formats”. - Section 8.4.1.1: Reflect agreements related to SCI format 2-A on fields for COT sharing. - Scetion 8.4.1.3: Reflect agreements related to SCI format 2-C on fields to include RB set related information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification for NR sidelink evolution is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.4.1, 8.3.1.1, 8.4.1.1, 8.4.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.211, TS 38.214, TS 37.213 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 7.3.1.4	DCI formats for scheduling of sidelink

##### 7.3.1.4.1	Format 3_0

DCI format 3_0 is used for scheduling of NR PSCCH and NR PSSCH in one cell.

The following information is transmitted by means of the DCI format 3_0 with CRC scrambled by SL-RNTI or SL-CS-RNTI:

-	Resource pool index -log2I bits, where I is the total number of resource pools for transmission configured by the higher layer parameter sl-TxPoolScheduling, if configured, and sl-DiscTxPoolScheduling, if configured.

-	Time gap - 3 bits determined by higher layer parameter sl-DCI-ToSL-Trans, as defined in clause 8.1.2.1 of [6, TS 38.214]

-	HARQ process number - 4 bits.

-	New data indicator - 1 bit.

-	Lowest index of the subchannel allocation to the initial transmission -log2(N subChannel SL) bits as defined in Clause 8.1.2.2 of [6, TS 38.214].

-	Lowest index of the RB set allocation to the initial transmission - log2(NRBset) bits as defined in Clause 8.1.2.2 of [6, TS 38.214] if the higher layer parameter transmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured to 'interlaceRB'; 0 bit otherwise.

-	SCI format 1-A fields according to clause 8.3.1.1:

-	Frequency resource assignment.

-	Time resource assignment.

-	PSFCH-to-HARQ feedback timing indicator -log2Nfb_timing bits, where Nfb_timing is the number of entries in the higher layer parameter sl-PSFCH-ToPUCCH, as defined in clause 16.5 of [5, TS 38.213]

-	PUCCH resource indicator - 3 bits as defined in clause 16.5 of [5, TS 38.213].

-	Configuration index - 0 bit if the UE is not configured to monitor DCI format 3_0 with CRC scrambled by SL-CS-RNTI; otherwise 3 bits as defined in clause 8.1.2 of [6, TS 38.214]. If the UE is configured to monitor DCI format 3_0 with CRC scrambled by SL-CS-RNTI, this field is reserved for DCI format 3_0 with CRC scrambled by SL-RNTI.

-	Counter sidelink assignment index - 2 bits

-	2 bits as defined in clause 16.5.2 of [5, TS 38.213] if the UE is configured with pdsch-HARQ-ACK-Codebook = dynamic

-	2 bits as defined in clause 16.5.1 of [5, TS 38.213] if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static

-	Padding bits, if required

If the total number of transmit resource pools provided in sl-TxPoolScheduling, if configured, and sl-DiscTxPoolScheduling, if configured, is larger than one, zeros shall be appended to the DCI format 3_0 until the payload size is equal to the size of a DCI format 3_0 given by a configuration of the transmit resource pool resulting in the largest number of information bits for DCI format 3_0.

If the UE is configured to monitor DCI format 3_1 and the number of information bits in DCI format 3_0 is less than the payload of DCI format 3_1, zeros shall be appended to DCI format 3_0 until the payload size equals that of DCI format 3_1.

< Unchanged parts are omitted >

#### 8.3.1.1	SCI format 1-A

SCI format 1-A is used for the scheduling of PSSCH and 2nd-stage-SCI on PSSCH

The following information is transmitted by means of the SCI format 1-A:

-	Priority - 3 bits as specified in clause 5.4.3.3 of [12, TS 23.287] and clause 5.22.1.3.1 of [8, TS 38.321]. Value '000' of Priority field corresponds to priority value '1', value '001' of Priority field corresponds to priority value '2', and so on.

-	Frequency resource assignment - number of bits determined by the following:

-	If higher layer parameter transmissionStructureForPSCCHandPSSCH in SL-BWP-Config is not configured or configured to ‘contigousRB’

-	log2(N subChannel SLN subChannel SL + 12) bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 2; otherwise log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16) bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 3, as defined in clause 8.1.5 of [6, TS 38.214].

-	If the higher layer parameter transmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured to ‘interlaceRB’

-	X + Y bits provide the frequency domain resource allocation according to Clause x.x of [6, TS 38.214], where the X MSBs provide the RB set allocation and the Y LSBs provide the sub-channel allocation,

-	the value of X is determined by log2(NRBsetNRBset + 12)  when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 2, or determined by log2(NRBsetNRBset + 12NRBset + 16) when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 3, where NRBset is the number of RB sets in a resource pool

-	the value of Y is determined by log2(N subChannel SLN subChannel RBsetN subChannel SLN subChannel RBset + 12)  when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 2, or determined by log2(N subChannel SLN subChannel RBsetN subChannel SLN subChannel RBset + 12N subChannel SLN subChannel RBset + 16) when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 3, as defined in clause 8.1.5 of [6, TS 38.214].N subChannel RBset

-	Time resource assignment - 5 bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 2; otherwise 9 bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 3, as defined in clause 8.1.5 of [6, TS 38.214].

-	Resource reservation period -log2Nrsv_period bits as defined in clause 16.4 of [5, TS 38.213], where Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResource is configured; 0 bit otherwise.

-	DMRS pattern - log2Npattern bits as defined in clause 8.4.1.1.2 of [4, TS 38.211], where Npattern is the number of DMRS patterns configured by higher layer parameter sl-PSSCH-DMRS-TimePatternList.

-	2nd-stage SCI format - 2 bits as defined in Table 8.3.1.1-1.

-	Beta_offset indicator - 2 bits as provided by higher layer parameter sl-BetaOffsets2ndSCI and Table 8.3.1.1-2.

-	Number of DMRS port - 1 bit as defined in Table 8.3.1.1-3.

-	Modulation and coding scheme - 5 bits as defined in clause 8.1.3 of [6, TS 38.214].

-	Additional MCS table indicator - as defined in clause 8.1.3.1 of [6, TS 38.214]: 1 bit if one MCS table is configured by higher layer parameter sl-Additional-MCS-Table; 2 bits if two MCS tables are configured by higher layer parameter sl-Additional-MCS-Table; 0 bit otherwise.

-	PSFCH overhead indication - 1 bit as defined in clause 8.1.3.2 of [6, TS 38.214] if higher layer parameter sl-PSFCH-Period = 2 or 4; 0 bit otherwise.

-	Reserved - a number of bits as determined by the following:

-	Nreserved bits as configured by higher layer parameter sl-NumReservedBits, with value set to zero, if higher layer parameter sl-IndicationUE-B is not configured, or if higher layer parameter sl-IndicationUE-B is configured to 'disabled', and if higher layer parameter transmissionStructureForPSCCHandPSSCH in SL-BWP-Config is not configured;

-	(Nreserved-2) bits if higher layer parameter sl-IndicationUE-B is configured to 'enabled', and if higher layer parameter transmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured, with value set to zero.

-	(Nreserved-1) bits otherwise, with value set to zero.

-	COT sharing flag – 0 or 1 bit

-	1 bit as defined in [14, TS 37.213] if the higher layer parameter transmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured;

-	0 bit otherwise.

-	Conflict information receiver flag - 0 or 1 bit

-	1 bit if higher layer parameter sl-IndicationUE-B is configured to 'enabled', where the bit value of 0 indicates that the UE cannot be a UE to receive conflict information and the bit value of 1 indicates that the UE can be a UE to receive conflict information as defined in Clause 16.3.0 of [5, TS 38.213];

-	0 bit otherwise.

Table 8.3.1.1-1: 2nd-stage SCI formats

| Value of 2nd-stage SCI format field | 2nd-stage SCI format |
| --- | --- |
| 00 |  | SCI format 2-A |
| 01 |  | SCI format 2-B; or reserved if higher layer parameter transmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured |
| 10 |  | SCI format 2-C; or reserved if higher layer parameter transmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured and the COT sharing flag field is set to '1' |
| 11 |  | SCI format 2-D; or reserved if higher layer parameter transmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured |

Table 8.3.1.1-2: Mapping of Beta_offset indicator values to indexes in Table 9.3-2 of [5, TS38.213]

| Value of Beta_offset indicator | Beta_offset index in Table 9.3-2 of TS 38.213 [5] |
| --- | --- |
| 00 | 1st index provided by higher layer parameter sl-BetaOffsets2ndSCI |
| 01 | 2nd index provided by higher layer parameter sl-BetaOffsets2ndSCI |
| 10 | 3rd index provided by higher layer parameter sl-BetaOffsets2ndSCI |
| 11 | 4th index provided by higher layer parameter sl-BetaOffsets2ndSCI |

Table 8.3.1.1-3: Number of DMRS port(s)

| Value of the Number of DMRS port field | Antenna ports |
| --- | --- |
| 0 | 1000 |
| 1 | 1000 and 1001 |

< Unchanged parts are omitted >

#### 8.4.1.1	SCI format 2-A

SCI format 2-A is used for the decoding of PSSCH, with HARQ operation when HARQ-ACK information includes ACK or NACK, when HARQ-ACK information includes only NACK, or when there is no feedback of HARQ-ACK information.

The following information is transmitted by means of the SCI format 2-A:

-	HARQ process number - 4 bits.

-	New data indicator - 1 bit.

-	Redundancy version - 2 bits as defined in Table 7.3.1.1.1-2.

-	Source ID - 8 bits as defined in clause 8.1 of [6, TS 38.214].

-	Destination ID - 16 bits as defined in clause 8.1 of [6, TS 38.214].

-	HARQ feedback enabled/disabled indicator - 1 bit as defined in clause 16.3 of [5, TS 38.213].

-	Cast type indicator - 2 bits as defined in Table 8.4.1.1-1 and in clause 8.1 of [6, TS 38.214].

-	CSI request - 1 bit as defined in clause 8.2.1 of [6, TS 38.214] and in clause 8.1 of [6, TS 38.214].

If the 'COT sharing flag' field in SCI format 1-A is present and set to '1', all the remaining fields are set as follows:

-	CAPC – 2 bits. Values '00', '01', '10' and '11' correspond to CAPC values '1', '2', '3' and '4' as defined in Table 4.5-1 of [14, TS 37.213], respectively.

-	COT sharing cast type – 2 bits as defined in Table 8.4.1.1-1.

-	COT sharing additional ID – 24 bits. The 16 LSBs provide layer 1 destination ID and the 8 MSBs provide layer 1 source ID, as defined in [6, TS 38.214]. The 8 MSBs are reserved when the COT sharing cast type field is set to '00' or '01'.

-	Remaining COT duration – log2(10∙2μ) bits as defined in clause 4.5.3 of [14, TS 37.213], where μ is defined in Table 4.2-1 of Clause 4.2 of [4, TS 38.211].

Table 8.4.1.1-1: Cast type indicator or COT sharing cast type

| Value of Cast type indicator or COT sharing cast type | Cast type |
| --- | --- |
| 00 | Broadcast |
| 01 | Groupcast when HARQ-ACK information includes ACK or NACK |
| 10 | Unicast |
| 11 | Groupcast when HARQ-ACK information includes only NACK; or reserved, if higher layer parameter transmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured |

< Unchanged parts are omitted >

#### 8.4.1.3	SCI format 2-C

SCI format 2-C is used for the decoding of PSSCH, and providing inter-UE coordination information or requesting inter-UE coordination information. SCI format 2-C can be used only for unicast.

The following information is transmitted by means of the SCI format 2-C:

-	HARQ process number - 4 bits

-	New data indicator - 1 bit

-	Redundancy version - 2 bits as defined in Table 7.3.1.1.1-2

-	Source ID - 8 bits as defined in clause 8.1 of [6, TS 38.214]

-	Destination ID - 16 bits as defined in clause 8.1 of [6, TS 38.214]

-	HARQ feedback enabled/disabled indicator - 1 bit as defined in clause 16.3 of [5, TS 38.213]

-	CSI request - 1 bit as defined in clause 8.2.1 of [6, TS 38.214] and in clause 8.1 of [6, TS 38.214]

-	Providing/Requesting indicator - 1 bit, where value 0 indicates SCI format 2-C is used for providing inter-UE coordination information and value 1 indicates SCI format 2-C is used for requesting inter-UE coordination information

If the 'Providing/Requesting indicator' field is set to 0, all the remaining fields are set as follows:

-	Resource combinations - number of bits determined by the following:

-	If higher layer parameter transmissionStructureForPSCCHandPSSCH in SL-BWP-Config is not configured or configured to 'contigousRB'

-	2∙log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16)+9+Y bits as defined in Clause 8.1.5A of [6, TS 38.214];

-	If the higher layer parameter transmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured to 'interlaceRB'

-	2∙log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16)+log2(NRBsetNRBset + 12NRBset + 16)+9+Y bits as defined in Clause 8.1.5A of [6, TS 38.214];

where

-	Y=log2Nrsv_period and Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResource is configured; Y=0 otherwise

-	N subChannel SL is provided by the higher layer parameter sl-NumSubchannel as defined in Clause 8.1.5 of [6, TS 38.214]

-	NRBset is the number of RB sets in a resource pool

-	First resource location - 8 bits as defined in Clause 8.1.5A of [6, TS 38.214].

-	Reference slot location - (10+log2(10∙2μ)) bits as defined in Clause 8.1.5A of [6, TS 38.214], where μ is defined in Table 4.2-1 of Clause 4.2 of [4, TS 38.211].

-	Resource set type - 1 bit, where value 0 indicates preferred resource set and value 1 indicates non-preferred resource set.

-	Lowest subChannel indices - 2∙log2N subChannel SL bits as defined in Clause 8.1.5A of [6, TS 38.214].

-	Lowest RB set indices - 2∙log2NRBset bits as defined in Clause 8.1.5A of [6, TS 38.214] if the higher layer parameter transmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured to 'interlaceRB'; 0 bit otherwise.

If the 'Providing/Requesting indicator' field is set to 1, all the remaining fields are set as follows:

-	Priority - 3 bits as specified in clause 5.4.3.3 of [12, TS 23.287] and clause 5.22.1.3.1 of [8, TS 38.321]. Value '000' of Priority field corresponds to priority value '1', value '001' of Priority field corresponds to priority value '2', and so on.

-	Number of subchannels - log2N subChannel SL bits as defined in Clause 8.1.4A of [6, TS 38.214].

-	Number of RB sets - log2NRBset bits as defined in Clause 8.1.4A of [6, TS 38.214] if the higher layer parameter transmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured to 'interlaceRB'; 0 bit otherwise.

-	Resource reservation period - log2Nrsv_period bits as defined in Clause 8.1.4A of [6, TS 38.214], where Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResource is configured; 0 bit otherwise.

-	Resource selection window location - 2∙10+log2(10∙2μ) bits as defined in Clause 8.1.4A of [6, TS 38.214], where μ is defined in Table 4.2-1 of Clause 4.2 of [4, TS 38.211].

-	Resource set type - 1 bit, where value 0 indicates a request for inter-UE coordination information providing preferred resource set and value 1 indicates a request for inter-UE coordination information providing non-preferred resource set, if higher layer parameter sl-DetermineResourceType is configured to 'ueb'; otherwise, 0 bit.

-	Padding bits.

For operation in a same resource pool, zeros shall be appended to SCI format 2-C of which 'Providing/Requesting indicator' field is set to 1 until the payload size equals that of SCI format 2-C of which 'Providing/Requesting indicator' field is set to 0.

< Unchanged parts are omitted >