3GPP TSG-RAN WG1 Meeting #115	R1-2312747

Chicago, IL, USA, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR | 0115 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to NR Dynamic Spectrum Sharing (DSS) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_DSS_enh-Core |  |  |  |  |  | Date: |  |  | 2023-11-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The RRC parameters lte-CRS-PatternList3 and lte-CRS-PatternList4 are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the required RRC parameters. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete support for Rel-18 dyanmic spectrum sharing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 7.3.2.2	Control-resource set (CORESET)

A control-resource set consists of NRBCORESET resource blocks in the frequency domain and NsymbCORESET∈1,2,3 symbols in the time domain.

A control-channel element consists of 6 resource-element groups (REGs) where a resource-element group equals one resource block during one OFDM symbol. Resource-element groups within a control-resource set are numbered in increasing order in a time-first manner, starting with 0 for the first OFDM symbol and the lowest-numbered resource block in the control resource set.

A UE can be configured with multiple control-resource sets. Each control-resource set is associated with one CCE-to-REG mapping only.

The CCE-to-REG mapping for a control-resource set can be interleaved or non-interleaved and is described by REG bundles:

-	REG bundle i is defined as REGs iL, iL+1, …, iL+L-1 where L is the REG bundle size, i=0,1,…,NREGREGCORESETL-1, and NREGCORESET=NRBCORESETNsymbCORESET is the number of REGs in the CORESET

-	CCE  consists of REG bundles f6jL, f6jL+1, …, f(6jL+6L-1) where f(∙) is an interleaver

For non-interleaved CCE-to-REG mapping, L=6 and fx=x.

For interleaved CCE-to-REG mapping, L∈2,6 for NsymbCORESET=1 and L∈NsymbCORESET,6 for NsymbCORESET∈2,3. The interleaver is defined by

fx=rC+c+nshift mod NREGCORESETL
x=cR+r
r=0,1,…,R-1
c=0,1,…,C-1
C=NREGCORESET(LR)

where R∈2, 3, 6.

The UE is not expected to handle configurations resulting in the quantity C not being an integer.

For a CORESET configured by the ControlResourceSet IE:

-	NRBCORESET is given by the higher-layer parameter frequencyDomainResources;

-	NsymbCORESET is given by the higher-layer parameter duration, where NsymbCORESET=3 is supported only if the higher-layer parameter dmrs-TypeA-Position equals 3;

-	interleaved or non-interleaved mapping is given by the higher-layer parameter cce-REG-MappingType;

-	L equals 6 for non-interleaved mapping and is given by the higher-layer parameter reg-BundleSize for interleaved mapping;

-	R is given by the higher-layer parameter interleaverSize;

-	nshift∈0,1,…,274 is given by the higher-layer parameter shiftIndex if provided, otherwise nshift=NIDcell;

-	for both interleaved and non-interleaved mapping:

-	if the higher-layer parameter precoderGranularity equals sameAsREG-bundle the UE may assume the same precoding being used within a REG bundle

-	if the higher-layer parameter precoderGranularity equals allContiguousRBs,

-	the UE may assume the same precoding being used across the all resource-element groups within the set of contiguous resource blocks in the CORESET;

-	the UE may assume that no resource elements in the CORESET overlap with an SSB;

-	if the UE is not provided with the higher-layer parameter pdcchCandidateReception-WithCRSOverlap, the UE may assume that no resource elements in the CORESET overlap with LTE cell-specific reference signals as indicated by the higher-layer parameter lte-CRS-ToMatchAround, lte-CRS-PatternList1, lte-CRS-PatternList2, lte-CRS-PatternList3, or lte-CRS-PatternList4.

For CORESET 0 configured by the ControlResourceSetZero IE:

-	NRBCORESET and NsymbCORESET are defined by clause 13 of [5, TS 38.213];

-	the UE may assume interleaved mapping;

-	L=6;

-	R=2;

-	nshift=NIDcell;

-	the UE may assume normal cyclic prefix when CORESET 0 is configured by MIB or SIB1;

-	the UE may assume the same precoding being used within a REG bundle.

For CORESET 0 on a carrier where the SS/PBCH block is detected at sync raster points defined in Tables 5.4.3.1-2 or 5.4.3.1-3 of [14, TS 38.101-1] and configured by the ControlResourceSetZero IE:

-	NRBCORESET and NsymbCORESET are defined by Table 13-0 in clause 13 of [5, TS 38.213];

-	if NRBCORESET=12 on a carrier with a channel bandwidth of 3 MHz, the CORESET is obtained by applying the description above assuming interleaved mapping with R=2;

-	if NRBCORESET=24 on a carrier with a channel bandwidth of 3 MHz, the CORESET is obtained by applying the description above assuming interleaved mapping with R=2 or non-interleaved mapping as defined by clause 13 of [5, TS 38.213], followed by puncturing the 9 highest-numbered resource blocks to obtain the 15 resource blocks forming CORESET 0;

-	if NRBCORESET=24 on a carrier with a channel bandwidth of 5 MHz, the CORESET is obtained by applying the description above assuming interleaved mapping with R=2, followed by puncturing the 4 highest-numbered resource blocks to obtain the 20 resource blocks forming CORESET 0;

-	L=6;

-	the UE may assume normal cyclic prefix when CORESET 0 is configured by MIB or SIB1;

-	the UE may assume the same precoding being used within a REG bundle.