3GPP TSG-RAN WG1 Meeting #115	R1-2312742

# Chicago, USA, November 13-17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0168 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on Rel-18 Multi-carrier enhancements in 38.212 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MC_enh-Core |  |  |  |  |  | Date: |  |  | 2023-11-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Capture the agreed TPs and agreements from RAN1#114bis meeting and RAN1#115 meeting. - Reflect the agreement on FDRA when ScheduledCellCombo-ListDCI-0-3 is not configured from RAN1#114bis meeting. - Reflect the agreement on the minimum applicable scheduling offset indicator in DCI format 0_3/1_3 from RAN1#114bis meeting. - Reflect the agreements on the SCell dormancy indication in DCI format 0_3/1_3 from RAN1#114bis meeting and RAN1#115 meeting. - Reflect the agreement on TDRA from RAN1#114bis meeting. - Reflect the agreement on BWP indicator from RAN1#115 meeting. - Reflect the agreement on DCI size determination from RAN1#115 meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the clarification to FDRA field for DCI format 0_3 that all '0's for FDRA Type 2 with μ=1 or all ‘1’s for FDRA Type 2 with μ=0 indicates the corresponding cell is not scheduled. - Update the description for the minimum applicable scheduling offset indicator when the bit size is equal to 1. - Update the description for SCell dormancy indication in DCI format 0_3 and DCI format 1_3. - Update the description for TDRA in DCI format 0_3 and DCI format 1_3 to reflect that entries of the joint table for TDRA (i.e., TDRA-FieldIndexDCI-1-3) are configured for each BWP of each CC. - Update the description for BWP indicator field in DCI format 0_3/1_3. - Add description on DCI size determination if an SCell within the set of cells is deactivated or dormant. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification for Rel-18 Multi-carrier enhancements is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.1.4, 7.3.1.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.211, TS 38.213, TS 38. 214 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 7.3.1.1	DCI formats for scheduling of PUSCH

< Unchanged parts are omitted >

##### 7.3.1.1.4	Format 0_3

DCI format 0_3 is used for the scheduling of one PUSCH in one cell, or multiple PUSCHs in multiple cells with one PUSCH per cell.

The following information is transmitted by means of the DCI format 0_3 with CRC scrambled by C-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats - 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

-	Scheduled cell set indicator - log2Nset bits, where Nset is the number of cell sets which are configured by higher layer parameter MC-DCI-SetofCellsToAddModList to be respectively scheduled by DCI format 0_3/1_3 from the cell on which this format is carried by PDCCH. If present, this field is used to indicate the scheduled cell set according to Table 7.3.1.1.4-1; otherwise, the scheduled cell set is the cell set configured to be scheduled by DCI format 0_3/1_3 from the cell by higher layer parameter MC-DCI-SetofCellsToAddModList.

-	Scheduled cells indicator - number of bits determined by the following:

-	0 bit if the higher layer parameter ScheduledCellCombo-ListDCI-0-3 for the scheduled cell set is not configured;

-	otherwise log2IUL bits indicating the scheduled cells in the scheduled cell set according to Table 7.3.1.1.4-2, where IUL is the number of entries in the higher layer parameter ScheduledCellCombo-ListDCI-0-3. If only one entry is configured in the higher layer parameter ScheduledCellCombo-ListDCI-0-3, the scheduled cells are the cells configured by higher layer parameter ScheduledCellCombo-ListDCI-0-3.

-	Bandwidth part indicator - 0, 1 or 2 bits determined as log2nBWP, max, where

-	nBWP, max=nBWP,RRCmax+1 if nBWP,RRCmax≤3,  nBWP,RRCmax is the maximum number of UL BWPs configured by higher layers, excluding the initial UL bandwidth part, across all the cells configured by higher layer parameter ScheduledCell-ListDCI-0-3 in the scheduled cell set, in which case the bandwidth part indicator is equivalent to the ascending order of the higher layer parameter BWP-Id;

-	otherwise nBWP, max=nBWP,RRCmax, in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1;

The field is only applicable to a scheduled cell with the number of configured UL BWPs larger than 1, including the initial UL bandwidth part, and is applied to the applicable scheduled cells in the scheduled cell set independently. If a UE does not support active BWP change via DCI, the UE ignores this bit field. If this field indicates a code point that does not correspond to a configured BWP of a scheduled cell, the UE ignores this bit field for the scheduled cell, and operates on the active BWP of the scheduled cell.

-	Frequency domain resource assignment - number of bits determined by the following, where NRBUL, BWP is the size of the active UL bandwidth part:

-	block number 1, block number 2,…, block number NcellUL

If ScheduledCellCombo-ListDCI-0-3 for the scheduled cell set is configured with more than one entry, NcellUL is the number of scheduled cells indicated by Scheduled cells indicator field; if ScheduledCellCombo-ListDCI-0-3 for the scheduled cell set is configured with only one entry, NcellUL is the number of cells configured by higher layer parameter ScheduledCellCombo-ListDCI-0-3; otherwise, NcellUL is the number of cells configured by higher layer parameter ScheduledCell-ListDCI-0-3 in the scheduled cell set. Each block corresponds to the frequency domain resource assignment for a cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the frequency domain resource assignment for the cell with the smallest serving cell index. Each block is defined by the following fields:

-	If higher layer parameter useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is not configured

-	NRBG bits if only resource allocation type 0 is configured, where NRBG is defined in Clause 6.1.2.2.1 of [6, TS 38.214]

-	log2NRBG, K1NRBG, K1+1/2 bits if only resource allocation type 1 is configured, or max log2NRBG, K1NRBG, K1+1/2, NRBG+1 bits if resourceAllocationDCI-0-3 is configured as 'dynamicSwitch', where NRBG, K1=NRBUL, BWP+NUL, BWPstartmodK1/K1, NRBUL, BWP is the size of the active UL bandwidth part, NUL, BWPstart is defined as in clause 4.4.4.4 of [4, TS 38.211] and K1 is given by higher layer parameter resourceAllocationType1GranularityDCI-0-3. If the higher layer parameter resourceAllocationType1GranularityDCI-0-3 is not configured, K1 is equal to 1.

-	If resourceAllocationDCI-0-3 is configured as 'dynamicSwitch', the MSB bit is used to indicate resource allocation type 0 or resource allocation type 1, where the bit value of 0 indicates resource allocation type 0 and the bit value of 1 indicates resource allocation type 1.

-	For resource allocation type 0, the NRBG LSBs provide the resource allocation as defined in Clause 6.1.2.2.1 of [6, TS 38.214].

-	For resource allocation type 1, the log2NRBG, K1NRBG, K1+1/2 LSBs provide the resource allocation as follows:

-	For PUSCH hopping with resource allocation type 1:

-	NUL_hop MSB bits are used to indicate the frequency offset according to Clause 6.3 of [6, TS 38.214], where NUL_hop=1 if the higher layer parameter frequencyHoppingOffsetLists contains two offset values and NUL_hop=2 if the higher layer parameter frequencyHoppingOffsetLists contains four offset values

-	log2NRBG, K1NRBG, K1+1/2-NUL_hop bits provide the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214]

-	For non-PUSCH hopping with resource allocation type 1:

-	log2NRBG, K1NRBG, K1+1/2 bits provide the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214]

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and if resourceAllocationDCI-0-3 is configured as 'dynamicSwitch' for the indicated bandwidth part, the UE assumes resource allocation type 0 for the indicated bandwidth part if the bitwidth of the "Frequency domain resource assignment" field of the active bandwidth part is smaller than the bitwidth of the "Frequency domain resource assignment" field of the indicated bandwidth part.

-	If the higher layer parameter useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is configured

-	5 + Y bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 30 kHz. The 5 MSBs provide the interlace allocation and the Y LSBs provide the RB set allocation.

-	6 + Y bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 15 kHz. The 6 MSBs provide the interlace allocation and the Y LSBs provide the RB set allocation.

The value of Y is determined by log2NRB-set,ULBWPNRB-set,ULBWP+12  where NRB-set,ULBWP  is the number of RB sets contained in the active UL BWP as defined in clause 7 of [6, TS38.214].

If the higher layer parameter ScheduledCellCombo-ListDCI-0-3 for the scheduled cell set is not configured, each block is also used to indicate whether the corresponding cell is scheduled or not as follows:

-	if all bits of a block are set to 0 for resource allocation type 0, or set to 1 for resource allocation type 1, or set to 0 or 1 for dynamic switch resource allocation type, or set to 0 for resource allocation type 2 with μ=1, or set to 1 for resource allocation type 2 with μ=0, the cell corresponding to the block is not scheduled;

-	otherwise, the cell corresponding to the block is scheduled.

-	Time domain resource assignment - log2(ITDRA)log2(I) bits, where ITDRA I is the number of entries in the higher layer parameter TDRA-FieldIndexListDCI-0-3. This field is used to indicate an entry in the higher layer parameter TDRA-FieldIndexListDCI-0-3 according to Table 7.3.1.1.4-3. Each entry in the higher layer parameter TDRA-FieldIndexListDCI-0-3 contains the ‘Time domain resource assignment’ index for each BWP of each cell in the scheduled cell set, where the ‘Time domain resource assignment’ indexes for all the cells are placed according to an ascending order of a serving cell index, and the 'Time domain resource assignment' indexes for all the BWPs of a cell are placed according to an ascending order of the higher layer parameter BWP-Id.

-	Frequency hopping flag - 0 or 1 bit

-	0 bit if the higher layer parameter frequencyHopping is not configured for any cell configured by higher layer parameter ScheduledCell-ListDCI-0-3 in the scheduled cell set;

-	1 bit according to Table 7.3.1.1.1-3 otherwise, only applicable to resource allocation type 1, as defined in Clause 6.3 of [6, TS 38.214].

The field is only applicable to a scheduled cell configured with frequencyHopping, and is applied to the applicable scheduled cells independently.

-	Modulation and coding scheme - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellUL

Each block corresponds to the modulation and coding scheme for a scheduled cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the modulation and coding scheme for the cell with the smallest serving cell index. Each block is 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214].

-	New data indicator - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellUL

Each block corresponds to the new data indicator for a scheduled cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the new data indicator for the cell with the smallest serving cell index. Each block is 1 bit.

-	Redundancy version - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellUL

Each block corresponds to the redundancy version for a scheduled cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the redundancy version for the cell with the smallest serving cell index. Each block is 0, 1 or 2 bits determined by higher layer parameter numberOfBitsForRV-DCI-0-3 configured for the cell corresponding to the block,

-	If 0 bit is configured, rvid to be applied is 0;

-	1 bit according to Table 7.3.1.2.3-1;

-	2 bits according to Table 7.3.1.1.1-2.

-	HARQ process number - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellUL

Each block corresponds to the HARQ process number for a scheduled cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the HARQ process number for the cell with the smallest serving cell index. Each block is 0, 1, 2, 3, 4 or 5 bits determined by higher layer parameter harq-ProcessNumberSizeDCI-0-3 configured for the cell corresponding to the block.

-	1st downlink assignment index - 1 or 2 bits

-	1 bit for semi-static HARQ-ACK codebook;

-	2 bits for dynamic HARQ-ACK codebook.

When two HARQ-ACK codebooks are configured for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-3 is configured, if the bit width of the 1st downlink assignment index in DCI format 0_3 for one HARQ-ACK codebook is not equal to that of the 1st downlink assignment index in DCI format 0_3 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller 1st downlink assignment index until the bit width of the 1st downlink assignment index in DCI format 0_3 for the two HARQ-ACK codebooks are the same.

-	2nd downlink assignment index - 0 or 2 bits:

-	2 bits for dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks;

-	0 bit otherwise.

When two HARQ-ACK codebooks are configured for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-3 is configured, if the bit width of the 2nd downlink assignment index in DCI format 0_3 for one HARQ-ACK codebook is not equal to that of the 2nd downlink assignment index in DCI format 0_3 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller 2nd  downlink assignment index until the bit width of the 2nd downlink assignment index in DCI format 0_3 for the two HARQ-ACK codebooks are the same.

-	TPC command for scheduled PUSCH - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellUL

Each block corresponds to the TPC command for the scheduled PUSCH for a scheduled cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the TPC command for the scheduled PUSCH for the cell with the smallest serving cell index. Each block is 2 bits as defined in Clause 7.1.1 of [5, TS38.213].

-	SRS resource indicator - number of bits determined by the following:

-	If SRI-DCI0-3= type1a is configured by higher layer,

-	maxr∈{1,2,…,NcellUL,2}Msr bits applying to the scheduled cells with Msr>0 independently, where NcellUL, 2 is the number of cells configured by higher layer parameter ScheduledCell-ListDCI-0-3 in the scheduled cell set, r is mapped to the cells according to an ascending order of a serving cell index with r=1 corresponding to the cell with the smallest serving cell index, and Msr is defined below.

-	If SRI-DCI0-3= type2 is configured by higher layer,

-	block number 1, block number 2,…, block number NcellUL

Each block corresponds to the SRS resource indicator for a scheduled cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the SRS resource indicator for the cell with the smallest serving cell index. Each block is defined below.

Msr above for the case of SRI-DCI0-3= type1a or each block above for the case of SRI-DCI0-3= type2 is defined by the following:

-	log2k=1minLmax, NSRSNSRSk bits according to Tables 7.3.1.1.2-28/29/30/31 if the higher layer parameter txConfig = nonCodebook, where NSRS is the number of configured SRS resources in the first SRS resource set configured by higher layer parameter srs-ResourceSetToAddModList, and associated with the higher layer parameter usage of value 'nonCodeBook' and

-	if UE supports operation with maxMIMO-Layers and the higher layer parameter maxMIMO-Layers of PUSCH-ServingCellConfig of the serving cell is configured, Lmax is given by that parameter

-	otherwise, Lmax is given by the maximum number of layers for PUSCH supported by the UE for the serving cell for non-codebook based operation.

-	log2NSRS bits according to Tables 7.3.1.1.2-32, 7.3.1.1.2-32A and 7.3.1.1.2-32B if the higher layer parameter txConfig = codebook, where NSRS is the number of configured SRS resources in the first SRS resource set configured by higher layer parameter srs-ResourceSetToAddModList, and associated with the higher layer parameter usage of value 'codeBook'.

-	Precoding information and number of layers - number of bits determined by the following:

-	If TPMI-DCI0-3= type1a is configured by higher layer,

-	 maxr∈{1,2,…,NcellUL,2}Mpr bits applying to the scheduled cells with Mpr>0 independently, where r is mapped to the cells according to an ascending order of a serving cell index with r=1 corresponding to the cell with the smallest serving cell index, and Mpr is defined below.

-	If TPMI-DCI0-3= type2 is configured by higher layer,

-	block number 1, block number 2,…, block number NcellUL

Each block corresponds to the precoding information and number of layers for a scheduled cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the precoding information and number of layers for the cell with the smallest serving cell index. Each block is defined below.

Mpr above for the case of TPMI-DCI0-3= type1a or each block above for the case of TPMI-DCI0-3= type2 is defined by the following:

-	0 bits if the higher layer parameter txConfig = nonCodeBook;

-	0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook;

-	4, 5, or 6 bits according to Table 7.3.1.1.2-2 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank, and codebookSubset;

-	4 or 5 bits according to Table 7.3.1.1.2-2A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, maxRank=2, transform precoder is disabled, and according to the value of higher layer parameter codebookSubset;

-	4 or 6 bits according to Table 7.3.1.1.2-2B for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, maxRank=3 or 4, transform precoder is disabled, and according to the value of higher layer parameter codebookSubset;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank and codebookSubset;

-	3 or 4 bits according to Table 7.3.1.1.2-3A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, maxRank=1, and according to whether transform precoder is enabled or disabled, and the value of higher layer parameter codebookSubset;

-	2 or 4 bits according to Table7.3.1.1.2-4 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-4A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, transform precoder is disabled, maxRank=2, and codebookSubset=nonCoherent;

-	1 or 3 bits according to Table7.3.1.1.2-5 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-5A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, maxRank=1, and according to whether transform precoder is enabled or disabled, and the value of higher layer parameter codebookSubset.

For the higher layer parameter txConfig=codebook, if ul-FullPowerTransmission is configured to fullpowerMode2, maxRank is configured to be larger than 2, and at least one SRS resource with 4 antenna ports is configured in the SRS resource set indicated by SRS resource set indicator field if present, otherwise in an SRS resource set with usage set to 'codebook', and an SRS resource with 2 antenna ports is indicated via SRI in the same SRS resource set, then Table 7.3.1.1.2-4 is used.

For the higher layer parameter txConfig = codebook, if different SRS resources with different number of antenna ports are configured, the bitwidth is determined according to the maximum number of ports in an SRS resource among the configured SRS resources in all SRS resource set(s) with usage set to 'codebook'. If the number of ports for a configured SRS resource in the set is less than the maximum number of ports in an SRS resource among the configured SRS resources, a number of most significant bits with value set to '0' are inserted to the field.

-	Antenna ports - number of bits determined by the following:

-	If AntennaPortsDCI0-3= type1a is configured by higher layer,

-	maxr∈{1,2,…,NcellUL,2}MAr bits applying to the scheduled cells independently, where r is mapped to the cells according to an ascending order of a serving cell index with r=1 corresponding to the cell with the smallest serving cell index, and MAr is defined below.

-	If AntennaPortsDCI0-3= type2 is configured by higher layer,

-	block number 1, block number 2,…, block number NcellUL

Each block corresponds to the Antenna ports information for a scheduled cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the Antenna ports information for the cell with the smallest serving cell index. Each block is defined below.

MAr above for the case of AntennaPortsDCI0-3= type1a or each block above for the case of AntennaPortsDCI0-3= type2 is defined by the following:

-	2 bits as defined by Tables 7.3.1.1.2-6, if transform precoder is enabled, dmrs-Type=1, and maxLength=1, except that dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured and π/2 BPSK modulation is used;

-	2 bits as defined by Tables 7.3.1.1.2-6A, if transform precoder is enabled and dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured, π/2 BPSK modulation is used, dmrs-Type=1, and maxLength=1, where nSCID is the scrambling identity for antenna ports defined in clause 6.4.1.1.1.2, TS38.211];

-	4 bits as defined by Tables 7.3.1.1.2-7, if transform precoder is enabled, dmrs-Type=1, and maxLength=2, except that dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured and π/2 BPSK modulation is used;

-	4 bits as defined by Tables 7.3.1.1.2-7A, if transform precoder is enabled and dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured, π/2 BPSK modulation is used, dmrs-Type=1, and maxLength=2, where nSCID is the scrambling identity for antenna ports defined in clause 6.4.1.1.1.2, TS38.211];

-	3 bits as defined by Tables 7.3.1.1.2-8/9/10/11, if transform precoder is disabled, dmrs-Type=1, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	4 bits as defined by Tables 7.3.1.1.2-12/13/14/15, if transform precoder is disabled, dmrs-Type=1, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	4 bits as defined by Tables 7.3.1.1.2-16/17/18/19, if transform precoder is disabled, dmrs-Type=2, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	5 bits as defined by Tables 7.3.1.1.2-20/21/22/23, if transform precoder is disabled, dmrs-Type=2, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook.

where the number of CDM groups without data of values 1, 2, and 3 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively.

If a UE is configured with both dmrs-UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB, the bitwidth of this field equals maxxA,xB, where xA is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeA and  xB is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeB. A number of xA-xB zeros are padded in the MSB of this field, if the mapping type of the PUSCH corresponds to the smaller value of xA and xB.

-	SRS request - log2(ISRS) bits, where ISRS is the number of entries in the higher layer parameter srs-RequestListDCI-0-3. This field is used to indicate an entry in the higher layer parameter srs-RequestListDCI-0-3 according to Table 7.3.1.1.4-4. Each entry in the higher layer parameter srs-RequestListDCI-0-3 contains the ‘SRS request’ index for each cell in the scheduled cell set, where the ‘SRS request’ indexes for all the cells are placed according to an ascending order of a serving cell index. Each ‘SRS request’ index is defined by the following:

-	2 bits as defined by Table 7.3.1.1.2-24 for UEs not configured with supplementaryUplink in ServingCellConfig in the cell; 3 bits for UEs configured with supplementaryUplink in ServingCellConfig in the cell where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second and third bits are defined by Table 7.3.1.1.2-24. This bit field may also indicate the associated CSI-RS according to Clause 6.1.1.2 of [6, TS 38.214].

-	SRS offset indicator - log2(Ioffset) bits, whereIoffset is the number of entries in the higher layer parameter srs-OffsetListDCI-0-3. This field is used to indicate an entry in the higher layer parameter srs-OffsetListDCI-0-3 according to Table 7.3.1.1.4-5. Each entry in the higher layer parameter srs-OffsetListDCI-0-3 contains the ‘SRS offset indicator’ index for each cell in the scheduled cell set, where the ‘SRS offset indicator’ indexes for all the cells are placed according to an ascending order of a serving cell index. Each ‘SRS offset indicator’ index is defined by the following:

-	0 bit if higher layer parameter AvailableSlotOffset is not configured for any aperiodic SRS resource set in the scheduled cell, or if higher layer parameter AvailableSlotOffset is configured for at least one aperiodic SRS resource set in the scheduled cell and the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) is 1;

-	otherwise, log2(K) bits are used to indicate available slot offset according to Table 7.3.1.1.2-37 and Clause 6.2.1 of [6, TS 38.214],  where K is the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) in the scheduled cell;

-	CSI request - 0, 1, 2, 3, 4, 5, or 6 bits determined by higher layer parameter reportTriggerSize. This field is applied to the cell with the smallest serving cell index among the scheduled cells indicated by Scheduled cells indicator field or Frequency domain resource assignment field.

-	PTRS-DMRS association - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellUL

Each block corresponds to the PTRS-DMRS association information for a scheduled cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the PTRS-DMRS association information for the cell with the smallest serving cell index. Each block is defined by the following:

-	0 bit if PTRS-UplinkConfig is not configured in either dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB and transform precoder is disabled, or if transform precoder is enabled, or if maxRankDCI=1;

-	2 bits otherwise, where Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field.

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the "PTRS-DMRS association" field is present for the indicated bandwidth part but not present for the active bandwidth part, the UE assumes the "PTRS-DMRS association" field is not present for the indicated bandwidth part.

-	beta_offset indicator  -  0 or 2 bits

-	0 bit if betaOffsets = semiStatic is configured for all the cells configured by higher layer parameter ScheduledCell-ListDCI-0-3 in the scheduled cell set;

-	otherwise 2 bits as defined by Table 9.3-3 in [5, TS 38.213].

When two HARQ-ACK codebooks are configured for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-3 is configured, if the bit width of the beta_offset indicator in DCI format 0_3 for one HARQ-ACK codebook is not equal to that of the beta_offset indicator in DCI format 0_3 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format 0_3 for the two HARQ-ACK codebooks are the same.

The field is only applicable to a scheduled cell configured with betaOffsets = dynamic, and is applied to the applicable scheduled cells independently.

-	DMRS sequence initialization - 1 bit. This field is applied to all the scheduled cells indicated by Scheduled cells indicator field or Frequency domain resource assignment field independently.

-	UL-SCH indicator - 1 bit. A value of "1" indicates UL-SCH shall be transmitted on the PUSCH and a value of "0" indicates UL-SCH shall not be transmitted on the PUSCH. A UE is not expected to receive a DCI format 0_3 with UL-SCH indicator of "0" and CSI request of all zero(s). This field is applied to the cell with the smallest serving cell index among the scheduled cells indicated by Scheduled cells indicator field or Frequency domain resource assignment field.

-	ChannelAccess-CPext-CAPC - maxr∈{1,2,…,Ncell2}Mcr bits applying to the scheduled cells with Mcr>0 independently, where Ncell2 is the number of cells configured by higher layer parameter ScheduledCell-ListDCI-0-3 in the scheduled cell set, r is mapped to the cells according to an ascending order of a serving cell index with r=1 corresponding to the cell with the smallest serving cell index, and Mcr is defined by the following:

-	0, 1, 2, 3, 4, 5 or 6 bits. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter ul-AccessConfigListDCI-0-1 or in Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access in frequency range 1, or for operation in frequency range 2-2 if ChannelAccessMode2-r17 is provided; otherwise 0 bit. One or more entries from Table 7.3.1.1.2-35 or Table 7.3.1.1.2-35A are configured by the higher layer parameter ul-AccessConfigListDCI-0-1.

-	Open-loop power control parameter set indication - maxr∈{1,2,…,NcellUL,2}Mor bits applying to the scheduled cells with Mor>0 independently, where r is mapped to the cells according to an ascending order of a serving cell index with r=1 corresponding to the cell with the smallest serving cell index, and Mor is defined by the following:

-	0 bit if the higher layer parameter p0-PUSCH-SetList is not configured;

-	1 or 2 bits otherwise,

-	1 bit if SRS resource indicator is present in the DCI format 0_3;

-	1 or 2 bits as determined by higher layer parameter olpc-ParameterSetDCI-0-1 if SRS resource indicator is not present in the DCI format 0_3.

-	Priority indicator - 0 bit if higher layer parameter priorityIndicatorDCI-0-3 is not configured; otherwise 1 bit as defined in Clause 9 in [5, TS 38.213]. This field is applied to all the scheduled cells indicated by Scheduled cells indicator field or Frequency domain resource assignment field.

-	Minimum applicable scheduling offset indicator - 0 or 1 bit

-	0 bit if higher layer parameter minimumSchedulingOffsetK0DCI-0-3 is not configured;

-	1 bit otherwise. The 1 bit indication is used to determine the minimum applicable K2 for the active UL BWP and the minimum applicable K0 for the active DL BWP, if configured respectively, according to Table 7.3.1.1.2-33. If the minimum applicable K0 is indicated, the minimum applicable value of the aperiodic CSI-RS triggering offset for an active DL BWP for each scheduled cell shall be the same as the minimum applicable K0.

-	SCell dormancy indication - 0 bit if higher layer parameter dormancyDCI-0-3 or dormancyGroupWithinActiveTime is not configured; otherwise 1, 2, 3, 4, or 5 bits bitmap determined according to the number of different DormancyGroupID(s) provided by higher layer parameter dormancyGroupWithinActiveTime, where each bit corresponds to one of the SCell group(s) configured by higher layers parameter dormancyGroupWithinActiveTime, with MSB to LSB of the bitmap corresponding to the first to last configured SCell group in ascending order of DormancyGroupID. The field is only present when this format is carried by PDCCH on the primary cell within DRX Active Time and the UE is configured with at least two DL BWPs for an SCell.

-	PDCCH monitoring adaptation indication - 0, 1 or 2 bits

-	0 bit if higher layer parameter pdcchMonAdaptDCI-0-3 is not enabled;

-	otherwise,

-	1 or 2 bits, if searchSpaceGroupIdList-r17 is not configured and if pdcch-SkippingDurationList is configured

-	1 bit if the UE is configured with only one duration by pdcch-SkippingDurationList;

-	2 bits if the UE is configured with more than one duration by pdcch-SkippingDurationList.

-	1 or 2 bits, if pdcch-SkippingDurationList is not configured and if searchSpaceGroupIdList-r17 is configured

-	1 bit if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0 and search space set(s) with group index 1, and if the UE is not configured by searchSpaceGroupIdList-r17 with any search space set with group index 2;

-	2 bits if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0, search space set(s) with group index 1 and search space set(s) with group index 2;

-	2 bits, if pdcch-SkippingDurationList is configured and if searchSpaceGroupIdList-r17 is configured

If ScheduledCellCombo-ListDCI-0-3 for the cell set is configured, zeros shall be appended to DCI format 0_3 if needed until the payload size equals the size of DCI format 0_3 that is determined by the configuration of the corresponding active bandwidth part(s) of the scheduled cells in the entry which results in the largest size among the entries in the higher layer parameter ScheduledCellCombo-ListDCI-0-3.

If an SCell within the scheduled cell set is deactivated, the UE determines the bitwidth of the fields in DCI format 0_3 based on a UL BWP provided by firstActiveUplinkBWP-Id for the SCell.

Table 7.3.1.1.4-1: Scheduled cell set indicator in DCI format 0_3

| Bit field mapped to index | Scheduled cell set |
| --- | --- |
| 0 | The cell set configured by the 1st entry in MC-DCI-SetofCellsToAddModList |
| 1 | The cell set configured by the 2nd entry in MC-DCI-SetofCellsToAddModList |
| 2 | The cell set configured by the 3rd entry in MC-DCI-SetofCellsToAddModList, if any |
| 3 | The cell set configured by the 4th entry in MC-DCI-SetofCellsToAddModList, if any |

Table 7.3.1.1.4-2: Scheduled cells indicator in DCI format 0_3

| Bit field mapped to index | Scheduled cells |
| --- | --- |
| 0 | The cells configured by the 1st entry in ScheduledCellCombo-ListDCI-0-3 |
| 1 | The cells configured by the 2nd entry in ScheduledCellCombo-ListDCI-0-3 |
| 2 | The cells configured by the 3rd entry in ScheduledCellCombo-ListDCI-0-3, if any |
| 3 | The cells configured by the 4th entry in ScheduledCellCombo-ListDCI-0-3, if any |
| 4 | The cells configured by the 5th entry in ScheduledCellCombo-ListDCI-0-3, if any |
| 5 | The cells configured by the 6th entry in ScheduledCellCombo-ListDCI-0-3, if any |
| 6 | The cells configured by the 7th entry in ScheduledCellCombo-ListDCI-0-3, if any |
| 7 | The cells configured by the 8th entry in ScheduledCellCombo-ListDCI-0-3, if any |
| 8 | The cells configured by the 9th entry in ScheduledCellCombo-ListDCI-0-3, if any |
| 9 | The cells configured by the 10th entry in ScheduledCellCombo-ListDCI-0-3, if any |
| 10 | The cells configured by the 11th entry in ScheduledCellCombo-ListDCI-0-3, if any |
| 11 | The cells configured by the 12th entry in ScheduledCellCombo-ListDCI-0-3, if any |
| 12 | The cells configured by the 13th entry in ScheduledCellCombo-ListDCI-0-3, if any |
| 13 | The cells configured by the 14th entry in ScheduledCellCombo-ListDCI-0-3, if any |
| 14 | The cells configured by the 15th entry in ScheduledCellCombo-ListDCI-0-3, if any |
| 15 | The cells configured by the 16th entry in ScheduledCellCombo-ListDCI-0-3, if any |

Table 7.3.1.1.4-3: Time domain resource assignment in DCI format 0_3

| Bit field mapped to index | Indicated time domain resource allocation |
| --- | --- |
| 0 | The 1st entry in TDRA-FieldIndexListDCI-0-3 |
| 1 | The 2nd entry in TDRA-FieldIndexListDCI-0-3 |
| 2 | The 3rd entry in TDRA-FieldIndexListDCI-0-3, if any |
| … | … |
| i | The (i+1)th entry in TDRA-FieldIndexListDCI-0-3, if any |
| … | … |

Table 7.3.1.1.4-4: SRS request in DCI format 0_3

| Bit field mapped to index | Triggered aperiodic SRS resource set(s) |
| --- | --- |
| 0 | The 1st entry in srs-RequestListDCI-0-3 |
| 1 | The 2nd entry in srs-RequestListDCI-0-3 |
| 2 | The 3rd entry in srs-RequestListDCI-0-3, if any |
| … | … |
| 15 | The 16th entry in srs-RequestListDCI-0-3, if any |

Table 7.3.1.1.4-5: SRS offset indicator in DCI format 0_3

| Bit field mapped to index | Available slot offset |
| --- | --- |
| 0 | The 1st entry in srs-OffsetListDCI-0-3 |
| 1 | The 2nd entry in srs-OffsetListDCI-0-3 |
| 2 | The 3rd entry in srs-OffsetListDCI-0-3, if any |
| … | … |
| 7 | The 8th entry in srs-OffsetListDCI-0-3, if any |

< Unchanged parts are omitted >

#### 7.3.1.2	DCI formats for scheduling of PDSCH

< Unchanged parts are omitted >

##### 7.3.1.2.4	Format 1_3

DCI format 1_3 is used for the scheduling of one PDSCH in one cell, or multiple PDSCHs in multiple cells with one PDSCH per cell.

The following information is transmitted by means of the DCI format 1_3 with CRC scrambled by C-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats - 1 bits

-	The value of this bit field is always set to 1, indicating a DL DCI format

-	Scheduled cell set indicator -  log2Nset bits, where Nset is the number of cell sets which are configured by higher layer parameter MC-DCI-SetofCellsToAddModList to be respectively scheduled by DCI format 0_3/1_3 from the cell on which this format is carried by PDCCH. If present, this field is used to indicate the scheduled cell set according to Table 7.3.1.1.4-1; otherwise, the scheduled cell set is the cell set configured to be scheduled by DCI format 0_3/1_3 from the cell by higher layer parameter MC-DCI-SetofCellsToAddModList.

-	Scheduled cells indicator - number of bits determined by the following:

-	0 bit if the higher layer parameter ScheduledCellCombo-ListDCI-1-3 for the scheduled cell set is not configured;

-	otherwise log2IDL bits indicating the scheduled cells in the scheduled cell set according to Table 7.3.1.2.4-1, where IDL is the number of entries in the higher layer parameter ScheduledCellCombo-ListDCI-1-3. If only one entry is configured in the higher layer parameter ScheduledCellCombo-ListDCI-1-3, the scheduled cells are the cells configured by higher layer parameter ScheduledCellCombo-ListDCI-1-3.

-	Bandwidth part indicator - 0, 1 or 2 bits determined as log2nBWP, max, where

-	nBWP, max=nBWP,RRCmax+1 if nBWP,RRCmax≤3,  nBWP,RRCmax is the maximum number of DL BWPs configured by higher layers, excluding the initial DL bandwidth part, across all the cells configured by higher layer parameter ScheduledCell-ListDCI-1-3 in the scheduled cell set, in which case the bandwidth part indicator is equivalent to the ascending order of the higher layer parameter BWP-Id;

-	otherwise nBWP, max=nBWP,RRCmax, in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1;

The field is only applicable to a scheduled cell with the number of configured DL BWPs larger than 1, including the initial DL bandwidth part, and is applied to the applicable scheduled cells in the scheduled cell set independently. If a UE does not support active BWP change via DCI, the UE ignores this bit field. If this field indicates a code point that does not correspond to a configured BWP of a scheduled cell, the UE ignores this bit field for the scheduled cell, and operates on the active BWP of the scheduled cell.

-	Frequency domain resource assignment - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellDL

If ScheduledCellCombo-ListDCI-1-3 for the scheduled cell set is configured with more than one entry, NcellDL is the number of scheduled cells indicated by Scheduled cells indicator field; if ScheduledCellCombo-ListDCI-1-3 for the scheduled cell set is configured with only one entry, NcellDL is the number of cells configured by higher layer parameter ScheduledCellCombo-ListDCI-1-3; otherwise, NcellDL is the number of cells in the scheduled cell set configured by higher layer parameter ScheduledCell-ListDCI-1-3. Each block corresponds to the frequency domain resource assignment for a cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the frequency domain resource assignment for the cell with the smallest serving cell index. Each block is defined by the following fields:

-	NRBG bits if only resource allocation type 0 is configured, where NRBG is defined in Clause 5.1.2.2.1 of [6, TS 38.214]

-	log2NRBG, K2NRBG, K2+1/2 bits if only resource allocation type 1 is configured, or max log2NRBG, K2NRBG, K2+1/2, NRBG+1 bits if resourceAllocationDCI-1-3 is configured as 'dynamicSwitch', where NRBG, K2=NRBDL, BWP+NDL, BWPstartmodK2/K2, NRBDL, BWP is the size of the active DL bandwidth part, NDL, BWPstart is defined as in clause 4.4.4.4 of [4, TS 38.211] and K2 is given by higher layer parameter resourceAllocationType1GranularityDCI-1-3. If the higher layer parameter resourceAllocationType1GranularityDCI-1-3 is not configured, K2 is equal to 1.

-	If resourceAllocationDCI-1-3 is configured as 'dynamicSwitch', the MSB bit is used to indicate resource allocation type 0 or resource allocation type 1, where the bit value of 0 indicates resource allocation type 0 and the bit value of 1 indicates resource allocation type 1.

-	For resource allocation type 0, the NRBG LSBs provide the resource allocation as defined in Clause 5.1.2.2.1 of [6, TS 38.214].

-	For resource allocation type 1, the log2NRBG, K2NRBG, K2+1/2 LSBs provide the resource allocation as defined in Clause 5.1.2.2.2 of [6, TS 38.214].

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and if resourceAllocationDCI-1-3 is configured as 'dynamicSwitch' for the indicated bandwidth part, the UE assumes resource allocation type 0 for the indicated bandwidth part if the bitwidth of the "Frequency domain resource assignment" field of the active bandwidth part is smaller than the bitwidth of the "Frequency domain resource assignment" field of the indicated bandwidth part.

If the higher layer parameter ScheduledCellCombo-ListDCI-1-3 for the scheduled cell set is not configured, each block is also used to indicate whether the corresponding cell is scheduled or not as follows:

-	if all bits of a block are set to 0 for resource allocation type 0 or set to 1 for resource allocation type 1 or set to 0 or 1 for dynamic switch resource allocation type, the cell corresponding to the block is not scheduled;

-	otherwise, the cell corresponding to the block is scheduled.

-	Time domain resource assignment - log2(ITDRA) log2(I) bits, where ITDRAI is the number of entries in the higher layer parameter TDRA-FieldIndexListDCI-1-3. This field is used to indicate an entry in the higher layer parameter TDRA-FieldIndexListDCI-1-3 according to Table 7.3.1.2.4-2. Each entry in the higher layer parameter TDRA-FieldIndexListDCI-1-3 contains the ‘Time domain resource assignment’ index for each BWP of each cell in the scheduled cell set, where the ‘Time domain resource assignment’ indexes for all the cells are placed according to an ascending order of a serving cell index, and the 'Time domain resource assignment' indexes for all the BWPs of a cell are placed according to an ascending order of the higher layer parameter BWP-Id.

-	VRB-to-PRB mapping - 0 or 1 bit

-	0 bit if the higher layer parameter vrb-ToPRB-Interleaver is not configured for any cell configured by higher layer parameter ScheduledCell-ListDCI-1-3 in the scheduled cell set;

-	1 bit according to Table 7.3.1.2.2-5 otherwise, only applicable to resource allocation type 1, as defined in Clause 7.3.1.6 of [4, TS 38.211].

The field is only applicable to a scheduled cell configured with vrb-ToPRB-Interleaver, and is applied to the applicable scheduled cells independently.

-	PRB bundling size indicator - 0 or 1 bit

-	0 bit if the higher layer parameter prb-BundlingType is not configured or is set to 'staticBundling' for any cell configured by higher layer parameter ScheduledCell-ListDCI-1-3 in the scheduled cell set;

-	1 bit according to Clause 5.1.2.3 of [6, TS 38.214] otherwise.

The field is only applicable to a scheduled cell configured with prb-BundlingType set to 'dynamicBundling', and is applied to the applicable scheduled cells independently.

-	Rate matching indicator - log2(IRM) bits, where IRM is the number of entries in the higher layer parameter rateMatchListDCI-1-3. This field is used to indicate an entry in the higher layer parameter rateMatchListDCI-1-3 according to Table 7.3.1.2.4-3. Each entry in the higher layer parameter rateMatchListDCI-1-3 contains the ‘Rate matching indicator’ index for each cell configured with rateMatchPatternGroup1 or rateMatchPatternGroup2 on at least one DL BWP in the scheduled cell set, where the ‘Rate matching indicator’ indexes for all the cells are placed according to an ascending order of a serving cell index. Each ‘Rate matching indicator’ index is defined by the following:

-	0, 1, or 2 bits according to higher layer parameters rateMatchPatternGroup1 and rateMatchPatternGroup2, where the MSB is used to indicate rateMatchPatternGroup1 and the LSB is used to indicate rateMatchPatternGroup2 when there are two groups.

-	ZP CSI-RS trigger - log2(ICSIRS) bits, where ICSIRS is the number of entries in the higher layer parameter zp-CSI-RSListDCI-1-3. This field is used to indicate an entry in the higher layer parameter zp-CSI-RSListDCI-1-3 according to Table 7.3.1.2.4-4. Each entry in the higher layer parameter zp-CSI-RSListDCI-1-3 contains the ‘ZP CSI-RS trigger’ index for each cell configured with aperiodicZP-CSI-RS-ResourceSetsToAddModList on at least one DL BWP in the scheduled cell set, where the ‘ZP CSI-RS trigger’ indexes for all the cells are placed according to an ascending order of a serving cell index. Each ‘ZP CSI-RS trigger’ index is defined by the following:

-	0, 1, or 2 bits as defined in Clause 5.1.4.2 of [6, TS 38.214]. The bitwidth for this field is determined as log2(nZP+1) bits, where nZP is the number of aperiodic ZP CSI-RS resource sets configured by higher layer parameter aperiodicZP-CSI-RS-ResourceSetsToAddModList.

For transport block 1:

-	Modulation and coding scheme - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellDL

Each block corresponds to the modulation and coding scheme for a scheduled cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the modulation and coding scheme for the cell with the smallest serving cell index. Each block is 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214].

-	New data indicator - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellDL

Each block corresponds to the new data indicator for a scheduled cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the new data indicator for the cell with the smallest serving cell index. Each block is 1 bit.

-	Redundancy version - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellDL

Each block corresponds to the redundancy version for a scheduled cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the redundancy version for the cell with the smallest serving cell index. Each block is 0, 1 or 2 bits determined by higher layer parameter numberOfBitsForRV-DCI-1-3 configured for the cell corresponding to the block,

-	If 0 bit is configured, rvid to be applied is 0;

-	1 bit according to Table 7.3.1.2.3-1;

-	2 bits according to Table 7.3.1.1.1-2.

For transport block 2:

-	Modulation and coding scheme - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellDL,3

If ScheduledCellCombo-ListDCI-1-3 for the scheduled cell set is configured, NcellDL,3 is the number of scheduled cells indicated by Scheduled cells indicator field and configured with maxNrofCodeWordsScheduledByDCI = 2; otherwise, NcellDL,3 is the number of cells configured by higher layer parameter ScheduledCell-ListDCI-1-3 in the scheduled cell set and configured with maxNrofCodeWordsScheduledByDCI = 2. Each block corresponds to the modulation and coding scheme for a scheduled cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the modulation and coding scheme for the cell with the smallest serving cell index. Each block is 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214].

-	New data indicator - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellDL,3

Each block corresponds to the new data indicator for a scheduled cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the new data indicator for the cell with the smallest serving cell index. Each block is 1 bit.

-	Redundancy version - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellDL,3

Each block corresponds to the redundancy version for a scheduled cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the redundancy version for the cell with the smallest serving cell index. Each block is 0, 1 or 2 bits determined by higher layer parameter numberOfBitsForRV-DCI-1-3 configured for the cell corresponding to the block,

-	If 0 bit is configured, rvid to be applied is 0;

-	1 bit according to Table 7.3.1.2.3-1;

-	2 bits according to Table 7.3.1.1.1-2.

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the value of maxNrofCodeWordsScheduledByDCI for the indicated bandwidth part equals 2 and the value of maxNrofCodeWordsScheduledByDCI for the active bandwidth part equals 1, the UE assumes zeros are padded when interpreting the "Modulation and coding scheme", "New data indicator", and "Redundancy version" fields of transport block 2 according to Clause 12 of [5, TS38.213], and the UE ignores the "Modulation and coding scheme", "New data indicator", and "Redundancy version" fields of transport block 2 for the indicated bandwidth part.

-	HARQ process number - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellDL

Each block corresponds to the HARQ process number for a scheduled cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the HARQ process number for the cell with the smallest serving cell index. Each block is 0, 1, 2, 3, 4 or 5 bits determined by higher layer parameter harq-ProcessNumberSizeDCI-1-3 configured for the cell corresponding to the block.

-	Downlink assignment index - number of bits as defined in the following

-	4 bits if the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic, where the 2 MSB bits are the counter DAI and the 2 LSB bits are the total DAI;

-	0 bits otherwise.

If the UE is configured with a PUCCH-SCell, pdsch-HARQ-ACK-Codebook is replaced by pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup-r16 if present for the secondary PUCCH group.

If higher layer parameter priorityIndicatorDCI-1-3 is configured, if the bit width of the Downlink assignment index in DCI format 1_3 for one HARQ-ACK codebook is not equal to that of the Downlink assignment index in DCI format 1_3 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller Downlink assignment index until the bit width of the Downlink assignment index in DCI format 1_3 for the two HARQ-ACK codebooks are the same.

-	TPC command for scheduled PUCCH - 2 bits as defined in Clause 7.2.1 of [5, TS 38.213]

-	PUCCH resource indicator - 3 bits as defined in Clause 9.2.3 of [5, TS 38.213]

-	PDSCH-to-HARQ_feedback timing indicator - 0, 1, 2, or 3 bits as defined in Clause 9.2.3 of [5, TS 38.213]. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter dL-DataToUL-ACK.

If higher layer parameter priorityIndicatorDCI-1-3 is configured, if the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_3 for one HARQ-ACK codebook is not equal to that of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_3 for the other HARQ-ACK codebook on the same cell for PUCCH transmission, a number of most significant bits with value set to '0' are inserted to smaller PDSCH-to-HARQ_feedback timing indicator until the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_3 for the two HARQ-ACK codebooks are the same.

If higher layer parameter pucch-sSCellDynDCI-1-3 is configured, if the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_3 associated with one cell for PUCCH transmission is not equal to that of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_3 associated with the other cell for PUCCH transmission, a number of most significant bits with value set to '0' are inserted to smaller PDSCH-to-HARQ_feedback timing indicator until the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_3 associated with the two cells are the same.

-	One-shot HARQ-ACK request - 0 or 1 bit.

-	1 bit if higher layer parameter pdsch-HARQ-ACK-OneShotFeedbackDCI-1-3 or pdsch-HARQ-ACK-enhType3DCI-1-3 is configured;

-	0 bit otherwise.

-	Enhanced Type 3 codebook indicator - 0, 1, 2, or 3 bits.

-	0 bit if pdsch-HARQ-ACK-enhType3DCIfieldDCI-1-3 is not configured;

-	log2(nCB) bits otherwise, where nCB is the number of entries in the higher layer parameter pdsch-HARQ-ACK-EnhType3ToAddModList.

If the UE is configured with a PUCCH-SCell, pdsch-HARQ-ACK-EnhType3ToAddModList is replaced by pdsch-HARQ-ACK-EnhType3SecondaryList for the secondary PUCCH group.

-	HARQ-ACK retransmission indicator - 0 or 1 bit.

-	1 bit if higher layer parameter pdsch-HARQ-ACK-retxDCI-1-3 is configured.

-	0 bit otherwise.

-	Antenna ports - number of bits determined by the following:

-	If AntennaPortsDCI-1-3= type1a is configured by higher layer,

-	maxr∈{1,2,…,NcellDL,2}MAr bits applying to the scheduled cells independently, where NcellDL,2 is the number of cells configured by higher layer parameter ScheduledCell-ListDCI-1-3 in the scheduled cell set,  r is mapped to the cells according to an ascending order of a serving cell index with r=1 corresponding to the cell with the smallest serving cell index, and MAr is defined below.

-	If AntennaPortsDCI-1-3= type2 is configured by higher layer,

-	block number 1, block number 2,…, block number  NcellDL

Each block corresponds to the Antenna ports information for a scheduled cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the Antenna ports information for the cell with the smallest serving cell index. Each block is defined below.

MAr above for the case of AntennaPortsDCI-1-3= type1A or each block above for the case of AntennaPortsDCI-1-3= type2 is defined by the following:

-	4, 5, or 6 bits as defined by Tables 7.3.1.2.2-1/2/3/4 and Tables 7.3.1.2.2-1A/2A/3A/4A, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. The antenna ports p0,…,pv-1 shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4 or Tables 7.3.1.2.2-1A/2A/3A/4A.

If a UE is configured with both dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB, the bitwidth of this field equals maxxA,xB, where xA is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeA and xB is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeB. A number of xA-xB zeros are padded in the MSB of this field, if the mapping type of the PDSCH corresponds to the smaller value of xA and xB.

-	Transmission configuration indication - number of bits determined by the following:

-	0 bit if higher layer parameter tci-PresentInDCI is not enabled;

-	otherwise log2(ITCI) bits, where ITCI is the number of entries in the higher layer parameter tci-ListDCI-1-3. This field is used to indicate an entry in the higher layer parameter tci-ListDCI-1-3 according to Table 7.3.1.2.4-5. Each entry in the higher layer parameter tci-ListDCI-1-3 contains the ‘Transmission configuration indication’ index for each cell in the scheduled cell set, where the ‘Transmission configuration indication’ indexes for all the cells are placed according to an ascending order of a serving cell index. Each ‘Transmission configuration indication’ index is 3 bits as defined in Clause 5.1.5 of [6, TS38.214].

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part,

-	if the higher layer parameter tci-PresentInDCI is not enabled for the CORESET used for the PDCCH carrying the DCI format 1_3,

-	the UE assumes tci-PresentInDCI is not enabled for all CORESETs in the indicated bandwidth part;

-	otherwise,

-	the UE assumes tci-PresentInDCI is enabled for all CORESETs in the indicated bandwidth part.

-	SRS request - log2(ISRS) bits, where ISRS is the number of entries in the higher layer parameter srs-RequestListDCI-1-3. This field is used to indicate an entry in the higher layer parameter srs-RequestListDCI-1-3 according to Table 7.3.1.2.4-6. Each entry in the higher layer parameter srs-RequestListDCI-1-3 contains the ‘SRS request’ index for each cell in the scheduled cell set, where the ‘SRS request’ indexes for all the cells are placed according to an ascending order of a serving cell index. Each ‘SRS request’ index is defined by the following:

-	2 bits as defined by Table 7.3.1.1.2-24 for UEs not configured with supplementaryUplink in ServingCellConfig in the cell; 3 bits for UEs configured with supplementaryUplink in ServingCellConfig in the cell where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second and third bits are defined by Table 7.3.1.1.2-24. This bit field may also indicate the associated CSI-RS according to Clause 6.1.1.2 of [6, TS 38.214].

-	SRS offset indicator - log2(Ioffset) bits, whereIoffset is the number of entries in the higher layer parameter srs-OffsetListDCI-1-3. This field is used to indicate an entry in the higher layer parameter srs-OffsetListDCI-1-3 according to Table 7.3.1.2.4-7. Each entry in the higher layer parameter srs-OffsetListDCI-1-3 contains the ‘SRS offset indicator’ index for each cell in the scheduled cell set, where the ‘SRS offset indicator’ indexes for all the cells are placed according to an ascending order of a serving cell index. Each ‘SRS offset indicator’ index is defined by the following:

-	0 bit if higher layer parameter AvailableSlotOffset is not configured for any aperiodic SRS resource set in the scheduled cell, or if higher layer parameter AvailableSlotOffset is configured for at least one aperiodic SRS resource set in the scheduled cell and the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) is 1;

-	otherwise, log2(K) bits are used to indicate available slot offset according to Table 7.3.1.1.2-37 and Clause 6.2.1 of [6, TS 38.214],  where K is the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) in the scheduled cell;

-	DMRS sequence initialization - 1 bit. This field is applied to all the scheduled cells indicated by Scheduled cells indicator field or Frequency domain resource assignment field independently.

-	Priority indicator - 0 bit if higher layer parameter priorityIndicatorDCI-1-3 is not configured; otherwise 1 bit as defined in Clause 9 in [5, TS 38.213].

-	ChannelAccess-CPext - 0, 1, 2, 3 or 4 bits. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter ul-AccessConfigListDCI-1-1 or in Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access in frequency range 1, or for operation in frequency range 2-2 if ChannelAccessMode2-r17 is provided; otherwise 0 bit. One or more entries from Table 7.3.1.2.2-6 or Table 7.3.1.2.2-6A are configured by the higher layer parameter ul-AccessConfigListDCI-1-1.

-	Minimum applicable scheduling offset indicator - 0 or 1 bit

-	0 bit if higher layer parameter minimumSchedulingOffsetK0DCI-1-3 is not configured;

-	1 bit otherwise. The 1 bit indication is used to determine the minimum applicable K0 for the active DL BWP and the minimum applicable K2 for the active UL BWP, if configured respectively, according to Table 7.3.1.1.2-33. If the minimum applicable K0 is indicated, the minimum applicable value of the aperiodic CSI-RS triggering offset for an active DL BWP for each scheduled cell shall be the same as the minimum applicable K0.

-	SCell dormancy indication - 0 bit if higher layer parameter dormancyDCI-1-3 or dormancyGroupWithinActiveTime is not configured; otherwise 1, 2, 3, 4, or 5 bits bitmap determined according to the number of different DormancyGroupID(s) provided by higher layer parameter dormancyGroupWithinActiveTime, where each bit corresponds to one of the SCell group(s) configured by higher layers parameter dormancyGroupWithinActiveTime, with MSB to LSB of the bitmap corresponding to the first to the last configured SCell group in ascending order of DormancyGroupID. The field is only present when this format is carried by PDCCH on the primary cell within DRX Active Time and the UE is configured with at least two DL BWPs for an SCell.

If the “One-shot HARQ-ACK request” field is not present or set to '0', and if the “HARQ-ACK retransmission indicator” field is not present or set to ‘0’, and if all bits of the corresponding block(s) of the frequency domain resource assignment field are set to 0 for resource allocation type 0 or set to 1 for resource allocation type 1 or set to 0 or 1 for dynamic switch resource allocation type for one or more cells in the scheduled cell set, this field is reserved and the following fields, corresponding to the cell with smallest serving cell index among the one or more cell(s), among the fields above are used for SCell dormancy indication, where each bit corresponds to one of the configured SCell(s), with MSB to LSB of the following fields concatenated in the order below corresponding to the SCell with lowest to highest SCell index

-	Modulation and coding scheme of transport block 1

-	New data indicator of transport block 1

-	Redundancy version of transport block 1

-	HARQ process number

-	Antenna port(s) if AntennaPortsDCI1-3= type2 is configured by higher layer.

-	PDCCH monitoring adaptation indication - 0, 1 or 2 bits

-	0 bit if higher layer parameter pdcchMonAdaptDCI-1-3 is not enabled;

-	otherwise,

-	1 or 2 bits, if searchSpaceGroupIdList-r17 is not configured and if pdcch-SkippingDurationList is configured

-	1 bit if the UE is configured with only one duration by pdcch-SkippingDurationList;

-	2 bits if the UE is configured with more than one duration by pdcch-SkippingDurationList.

-	1 or 2 bits, if pdcch-SkippingDurationList is not configured and if searchSpaceGroupIdList-r17 is configured

-	1 bit if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0 and search space set(s) with group index 1, and if the UE is not configured by searchSpaceGroupIdList-r17 with any search space set with group index 2;

-	2 bits if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0, search space set(s) with group index 1 and search space set(s) with group index 2;

-	2 bits, if pdcch-SkippingDurationList is configured and if searchSpaceGroupIdList-r17 is configured

-	PUCCH Cell indicator - 0 or 1 bit.

-	1 bit if higher layer parameter pucch-sSCellDynDCI-1-3 is configured.

-	0 bit otherwise.

If ScheduledCellCombo-ListDCI-1-3 for the cell set is configured, zeros shall be appended to DCI format 1_3 if needed until the payload size equals the size of DCI format 1_3 that is determined by the configuration of the corresponding active bandwidth part(s) of the scheduled cells in the entry which results in the largest size among the entries in the higher layer parameter ScheduledCellCombo-ListDCI-1-3.

If an SCell within the scheduled cell set is deactivated and the firstActiveDownlinkBWP-Id corresponding to the SCell is not set to dormant BWP, the UE determines the bitwidth of the fields in DCI format 1_3 based on a DL BWP provided by firstActiveDownlinkBWP-Id for the SCell. If the active DL BWP of an SCell within the scheduled cell set is a dormant DL BWP, or if an SCell within the scheduled cell set is deactivated and the firstActiveDownlinkBWP-Id corresponding to the SCell is set to dormant BWP, the UE determines the bitwidth of the fields in DCI format 1_3 based on a DL BWP provided by firstWithinActiveTimeBWP-Id for the SCell if provided; otherwise, based on a DL BWP provided by firstOutsideActiveTimeBWP-Id for the SCell.

Table 7.3.1.2.4-1: Scheduled cells indicator in DCI format 1_3

| Bit field mapped to index | Scheduled cells |
| --- | --- |
| 0 | The cells configured by the 1st entry in ScheduledCellCombo-ListDCI-1-3 |
| 1 | The cells configured by the 2nd entry in ScheduledCellCombo-ListDCI-1-3 |
| 2 | The cells configured by the 3rd entry in ScheduledCellCombo-ListDCI-1-3, if any |
| 3 | The cells configured by the 4th entry in ScheduledCellCombo-ListDCI-1-3, if any |
| 4 | The cells configured by the 5th entry in ScheduledCellCombo-ListDCI-1-3, if any |
| 5 | The cells configured by the 6th entry in ScheduledCellCombo-ListDCI-1-3, if any |
| 6 | The cells configured by the 7th entry in ScheduledCellCombo-ListDCI-1-3, if any |
| 7 | The cells configured by the 8th entry in ScheduledCellCombo-ListDCI-1-3, if any |
| 8 | The cells configured by the 9th entry in ScheduledCellCombo-ListDCI-1-3, if any |
| 9 | The cells configured by the 10th entry in ScheduledCellCombo-ListDCI-1-3, if any |
| 10 | The cells configured by the 11th entry in ScheduledCellCombo-ListDCI-1-3, if any |
| 11 | The cells configured by the 12th entry in ScheduledCellCombo-ListDCI-1-3 if any |
| 12 | The cells configured by the 13th entry in ScheduledCellCombo-ListDCI-1-3, if any |
| 13 | The cells configured by the 14th entry in ScheduledCellCombo-ListDCI-1-3, if any |
| 14 | The cells configured by the 15th entry in ScheduledCellCombo-ListDCI-1-3, if any |
| 15 | The cells configured by the 16th entry in ScheduledCellCombo-ListDCI-1-3, if any |

Table 7.3.1.2.4-2: Time domain resource assignment in DCI format 1_3

| Bit field mapped to index | Indicated time domain resource allocation |
| --- | --- |
| 0 | The 1st entry in TDRA-FieldIndexListDCI-1-3 |
| 1 | The 2nd entry in TDRA-FieldIndexListDCI-1-3 |
| 2 | The 3rd entry in TDRA-FieldIndexListDCI-1-3, if any |
| … | … |
| i | The (i+1)th entry in TDRA-FieldIndexListDCI-1-3, if any |
| … | … |

Table 7.3.1.2.4-3: Rate matching indicator

| Bit field mapped to index | Indicated rate matching pattern |
| --- | --- |
| 0 | The 1st entry in rateMatchListDCI-1-3 |
| 1 | The 2nd entry in rateMatchListDCI-1-3 |
| 2 | The 3rd entry in rateMatchListDCI-1-3, if any |
| … | … |
| 15 | The 16th entry in rateMatchListDCI-1-3, if any |

Table 7.3.1.2.4-4: ZP CSI-RS trigger

| Bit field mapped to index | Triggered aperiodic ZP CSI-RS |
| --- | --- |
| 0 | The 1st entry in zp-CSI-RSListDCI-1-3 |
| 1 | The 2nd entry in zp-CSI-RSListDCI-1-3 |
| 2 | The 3rd entry in zp-CSI-RSListDCI-1-3, if any |
| … | … |
| 7 | The 8th entry in zp-CSI-RSListDCI-1-3, if any |

Table 7.3.1.2.4-5: Transmission configuration indication

| Bit field mapped to index | Indicated transmission configuration indication |
| --- | --- |
| 0 | The 1st entry in tci-ListDCI-1-3 |
| 1 | The 2nd entry in tci-ListDCI-1-3 |
| 2 | The 3rd entry in tci-ListDCI-1-3, if any |
| … | … |
| 15 | The 16th entry in tci-ListDCI-1-3, if any |

Table 7.3.1.1.4-6: SRS request in DCI format 1_3

| Bit field mapped to index | Triggered aperiodic SRS resource set(s) |
| --- | --- |
| 0 | The 1st entry in srs-RequestListDCI-1-3 |
| 1 | The 2nd entry in srs-RequestListDCI-1-3 |
| 2 | The 3rd entry in srs-RequestListDCI-1-3, if any |
| … | … |
| 15 | The 16th entry in srs-RequestListDCI-1-3, if any |

Table 7.3.1.1.4-7: SRS offset indicator in DCI format 1_3

| Bit field mapped to index | Available slot offset |
| --- | --- |
| 0 | The 1st entry in srs-OffsetListDCI-1-3 |
| 1 | The 2nd entry in srs-OffsetListDCI-1-3 |
| 2 | The 3rd entry in srs-OffsetListDCI-1-3, if any |
| … | … |
| 7 | The 8th entry in srs-OffsetListDCI-1-3, if any |