3GPP TSG RAN WG1 #115	        		   			R1-2312728

# Chicago, USA, November 13th – 17th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0577 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Maintenance of Network Controlled Repeaters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-Core |  |  |  |  |  | Date: |  |  | 2023-11-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Corrections on network controlled repeaters (NCR) in NR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the NCR abbreviation in Clause 3.3. - Capture that if an NCR does not support simultaneous transmissions on the control link and the backhaul link, the NCR-Fwd does not transmit over a time resource if the NCR-MT transmits over the time resource. - Capture that a periodic beam indication with priority flag has higher priority than a semi-persistent beam indication without priority flag. - Capture the SCSs for the slot of PDCCH reception, offset value in AperiodicBeamIndicationForAccessLink, and the reference slot for time resources, and define the reference slot based on the slot of PDCCH reception and the offset value. - Capture the timeline for application of a TCI state or SRI provided by a MAC CE command for transmissions/receptions on the backhaul link. - Capture that the CP for the NCR time resource is the CP of the active DL BWP. - Capture that the timing for transmissions/receptions by the NCR-Fwd on the backhaul link follows the frame timing for transmissions/receptions by the NCR-MT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete support for NCR in NR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 3.3	Abbreviations

## *** Unchanged parts are omitted ***

MCS	Modulation and coding scheme

NCR	Network-controlled repeater

NCR-Fwd	NCR forwarding

## *** Unchanged parts are omitted ***

# 20	Network controlled repeater

An NCR includes an NCR-MT entity and an NCR-Fwd entity [19, TS 38.300].

Throughout this specification, unless otherwise noted, statements using the term "UE" in Clauses 4 through 13 are equally applicable to the NCR-MT.

A procedure for the NCR-MT to perform cell search, system information acquisition, random access procedure, UCI reporting, or PDCCH monitoring is same as a corresponding one for a UE. A procedure for the NCR-MT to perform PDSCH reception, CSI-RS measurements and CSI determination, PUSCH transmission, or SRS transmission is same as a corresponding one for a UE as described in [6, TS 38.214].

The NCR-Fwd transmits or receives only after the NCR-MT receives on the control link an indication for one or more beams [20, TS 38.106] for the NCR-Fwd to use for transmissions or receptions over corresponding one or more time resources on the access link.

The timing for transmission and reception by the NCR-Fwd on the backhaul link follows the frame timing for transmission and reception, respectively, by the NCR-MT.

When the NCR-MT performs a link recovery procedure as described in Clause 6, the NCR-Fwd does not transmit or receive until the link recovery procedure is complete [11, TS 38.321].

The NCR can be provided, through the NCR-MT, tdd-UL-DL-ConfigurationCommon and can be additionally provided tdd-UL-DL-ConfigurationDedicated. The NCR-Fwd receives on the backhaul link or transmits on the access link only in symbols indicated as downlink by tdd-UL-DL-ConfigurationCommon and, if provided, tdd-UL-DL-ConfigurationDedicated. The NCR-Fwd receives on the access link or transmits on the backhaul link only in symbols indicated as uplink by tdd-UL-DL-ConfigurationCommon and, if provided, tdd-UL-DL-ConfigurationDedicated.

If the NCR does not support simultaneous transmissions on control link and the backhaul link, the NCR-Fwd does not transmit over a time resource if the NCR-MT transmits over the time resource.

When the NCR simultaneously receives via both the control link and the backhaul link in a set of symbols, a TCI state for receptions on the backhaul link is same as a TCI state for receptions on the control link in the set of symbols. When the NCR simultaneously transmits via both the control link and the backhaul link in a set of symbols, a spatial filter for transmissions on the backhaul link is same as a spatial filter for transmissions on the control link in the set of symbols.

When the NCR does not simultaneously receive on the control link and the backhaul link

-	if the NCR does not support determination of a TCI state for receptions on the backhaul link based on an indication of a TCI state by the serving cell, or if the NCR does not receive an indication of a TCI state, for receptions on the backhaul link [11, TS 38.321]

-	if the NCR does not receive an indication of a unified TCI state for receptions by the NCR-MT, receptions on the backhaul link use same QCL parameters as the ones for PDCCH receptions in a CORESET with the lowest controlResourceSetId in the active DL BWP

-	else, receptions on the backhaul link use the QCL parameters provided by an indicated unified TCI state for receptions by the NCR-MT

-	else receptions on the backhaul link use QCL parameters provided by a TCI state in NCR Downlink Backhaul Link Beam Indication MAC CE [11, TS 38.321].

When the NCR does not simultaneously transmit on the control link and the backhaul link

-	if the NCR does not support determination of a spatial filter for transmissions on the backhaul link based on an indication of a unified TCI state or of an SRI by the serving cell, or if the NCR-MT does not receive an indication of a unified TCI state or of an SRI for determining a spatial filter, for transmissions on the backhaul link

-	if the NCR does not receive an indication of a unified TCI state for transmissions by the NCR-MT, transmissions on the backhaul link use a same spatial filter as the one associated with the PUCCH resource with the smallest pucch-ResourceId in PUCCH-ResourceSet in the active UL BWP

-	else, transmissions on the backhaul link use a spatial filter corresponding to the indicated unified TCI state for transmissions by the NCR-MT.

-	else transmissions on the backhaul link use a spatial filter corresponding to a unified TCI state or an SRI provided in NCR Uplink Backhaul Link Beam Indication MAC CE [11, TS 38.321].

If the NCR receives an indication of a TCI state for receptions on the backhaul link in a MAC CE command, or an indication of a unified TCI state or of an SRI for determining a spatial filter for transmissions on the backhaul link in a MAC CE command, the NCR applies the MAC CE command from the first slot that is after slot k+3∙Nslotsubframe,μ where k is the slot where the NCR-MT would transmit a PUCCH with HARQ-ACK information associated with the PDSCH providing the MAC CE command, Nslotsubframe,μ is a number of slots per subframe for the SCS configuration μ of the PUCCH transmission.

The NCR-Fwd uses a same beam for transmissions and receptions on the access link during respective time resources associated with a same beam index.

The NCR can be provided by periodicFwdRsrcSetToAddModList a list of sets of resources for transmissions or receptions on the access link. A set of resources, from the list of sets of resources, is provided by NCR-PeriodicFwdResourceSet and occurs with a periodicity provided by periodicityAndOffset-r18. A resource from the set of resources is provided by NCR-PeriodicFwdResource and includes a pair of a time resource provided by periodicTimeRsrc and a beam [20, TS 38.106] with an index provided by beamIndex. The time resource starts at a slot that is offset by a number of slots provided by periodicityAndOffset-r18 from the start of the period for the set of resources and at a symbol that is offset by symbolOffset from the start of the slot, and has a duration provided by durationInSymbols for a SCS provided by referenceSCS and the cyclicPrefix of the active DL BWP.

The NCR can be provided by semiPersistentFwdRsrcSetToAddModList a list of sets of resources for transmissions or receptions on the access link and the NCR Access Link Beam Indication MAC CE command can indicate a set of resources for the NCR to use or to stop using based on a corresponding identity provided by semiPersistentFwdRsrcSetId [11, TS 38.321]. The NCR uses or stops using the set of resources starting from the first slot that is after slot k+3Nslotsubframe,μ where k is the slot where the NCR-MT would transmit a PUCCH with HARQ-ACK information associated with the PDSCH providing the MAC CE command and μ is the SCS configuration for the PUCCH transmission. The set of resources is provided by NCR-SemiPersistentFwdResourceSet and occurs with a periodicity provided by periodicityAndOffset-r18. A resource from the set of resources is provided by NCR-SemiPersistentFwdResource and includes a pair of a time resource provided by semiPersistentTimeRsrc and a beam with an index provided by beamIndex, where beamIndex can be updated by the NCR Access Link Beam Indication MAC CE command. The time resource starts at a slot that is offset by a number of slots provided by periodicityAndOffset-r18 from the start of the period for the set of resources and at a symbol that is offset by symbolOffset from the start of the slot, and has a duration provided by durationInSymbols for a SCS provided by referenceSCS and the cyclicPrefix of the active DL BWP.

The NCR-MT can be configured to monitor PDCCH according to USS sets for detection of a DCI format 2_8 with CRC scrambled by an NCR-RNTI. A time resource and a corresponding beam index for transmissions or receptions on the access link are indicated by corresponding fields in DCI format 2_8 [4, TS 38.212]. When the NCR detects more than one DCI formats 2_8 that indicate beam indexes for time resources overlapping in a set of symbols, the NCR uses for the set of symbols a beam index that is indicated by a DCI format 2_8 that the NCR-MT detects in a most recent PDCCH monitoring occasion. The time resource starts at a slot that is offset by slotOffsetAperiodic slots from a reference slot and at a symbol that is offset by symbolOffset from the start of the slot, and has a duration provided by durationInSymbols for a SCS provided by referenceSCS and the cyclicPrefix of the active DL BWP. The reference slot is the first slot with the SCS provided by referenceSCS that starts no earlier than the start of a slot that is after a slot of a PDCCH reception that provides the DCI format 2_8 by a number of slots indicated by AperiodicBeamIndicationForAccessLink [18, TS 38.306] with the SCS of PDCCH reception.

If

-	a first time resource provided by NCR-SemiPersistentFwdResourceSet is indicated by the NCR Access Link Beam Indication MAC CE command and is associated with a first beam index, and

-	a second time resource is provided by NCR-PeriodicFwdResourceSet and is associated with a second beam index, and

-	the first time resource overlaps with the second time resource in a set of symbols,

the NCR applies, for transmissions or receptions on the access link in the set of symbols, the second beam index if only NCR-PeriodicFwdResourceSet includes priorityFlag, and the first beam index otherwise.

If

-	a first time resource is provided by NCR-PeriodicFwdResourceSet, or provided by NCR-SemiPersistentFwdResourceSet and indicated by the NCR Access Link Beam Indication MAC CE command, and is associated with a first beam index, and

-	a second time resource is indicated by DCI format 2_8 and is associated with a second beam index provided by the DCI format 2_8, and

-	the first time resource overlaps with the second time resource in a set of symbols,

the NCR applies, for transmissions or receptions on the access link in the set of symbols,

-	the first beam index if NCR-PeriodicFwdResourceSet or NCR-SemiPersistentFwdResourceSet includes priorityFlag, and

-	the second beam index if NCR-PeriodicFwdResourceSet or NCR-SemiPersistentFwdResourceSet does not include priorityFlag.

The NCR does not expect overlapping time resources provided by either NCR-PeriodicFwdResourceSet or NCR-SemiPersistentFwdResourceSet to be associated with different beam indexes.