3GPP TSG RAN WG1 #114bis	R1-2310664

Xiamen, China, October 9th – 13th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0531 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on SL NACK report to gNB in mode 1 (Rel-18) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (NTT DOCOMO, INC.), vivo, Samsung, Qualcomm, Huawei/HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2023-10-13 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In current specification, for SL HARQ-ACK report to gNB in mode 1 resource allocation, NACK report is performed in the following two cases: - All PSFCH resources corresponding to PSSCH resources provided by gNB are not received due to prioritization - All PSSCH resources provided by gNB are not used for transmission due to prioritization However, mixture cases of above A and B are not considered in the description. For example, when gNB provides 3 resources, and 2 resources are used for PSSCH transmissions but corresponding PSFCH resources are not monitored, and if 1 resource is not used for transmission, the UE behavior is not specified. This case can be illustrated as below. One note is that Case A and Case B are described in the current spec as follows: …, due to prioritization, …, the UE does not receive PSFCH … …, due to prioritization, …, the UE does not receive a PSSCH … These texts imply that Case A does not include e.g., a case where a UE does not receive PSFCH due to failure of PSSCH transmission due to priortization. Another note is that “a PSSCH transmission” in e.g., “in any PSFCH reception occasion associated with a PSSCH transmission in a resource provided by a DCI format 3_0” would not mean to consider only PSSCH resource actually used for PSSCH transmission. This way of the terminology can be found in many parts of the current spec. That is, the 1st paragraph in this correction does not include the mixture case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For the mixture cases of above A and B as well as for only A or for only B, NACK is reported to gNB. The paragraph for only Case A includes additionally the mixture case by this correction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE behavior for the mixture cases of above A and B is not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 16.5	UE procedure for reporting HARQ-ACK on uplink

<Unchanged parts omitted>

The UE generates a NACK when, due to prioritization, as described in clause 16.2.4, the UE does not receive PSFCH in any PSFCH reception occasion associated with a PSSCH transmission in a resource provided by a DCI format 3_0 and the UE transmitted PSSCH in the resource or, for a configured grant, in a resource provided in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information and the UE transmitted PSSCH in the resource. The priority value of the NACK is same as the priority value of the PSSCH transmission.

The UE generates a NACK when, due to prioritization as described in clause 16.2.4, the UE does not transmit a PSSCH in any of the resources provided by a DCI format 3_0 or, for a configured grant, in any of the resources provided in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information. The priority value of the NACK is same as the priority value of the PSSCH that was not transmitted due to prioritization.

<Unchanged parts omitted>