3GPP TSG RAN WG1 Meeting #114-bis	R1- 2310660

# Xiamen, October 9th – October 13th, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.215 | CR | 0055 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Applicable RRC States for Sidelink Measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Qualcomm), Samsung, Huawei, HiSilicon, Nokia, NSB, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2023-10-13 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Sidelink measurements omit RRC_INACTIVE state from the list of applicable RRC states. Consequently, specifications could be interpreted in such a way that some sidelink operations cease when the UE enters RRC_INACTIVE. Sidelink PSBCH-RSRP, PSSCH-RSRP, and PSCCH-RSRP measurements omit RRC_CONNECTED intra-frequency state from the list of applicable RRC states. Consequently, specifications could be interpreted in such a way Mode 2 operation would not function correctly when sidelink and Uu are on the same carrier and the UE is in the RRC_CONNECTED state. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add RRC_INACTIVE to the list of applicable RRC states for slidelink measurements. Add RRC_CONNECTED intra-frequency to the list of applicable RRC states for PSBCH-RSRP, PSSCH-RSRP, and PSCCH-RSRP measurements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE would not perform necessary sidelink measurements when in RRC_INACTIVE state. UE would not perform some necessary sidelink measurements when in RRC_CONNECTED state if Uu and sidelink are on the same carrier |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.21, 5.1.22, 5.1.23, 5.1.24, 5.1.25, 5.1.26, 5.1.27 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------begin text proposal for 38.215----------------------------------------------------

5.1.22	PSBCH reference signal received power (PSBCH-RSRP)

| Definition | PSBCH Reference Signal Received Power (PSBCH-RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry demodulation reference signals associated with physical sidelink broadcast channel (PSBCH). For PSBCH-RSRP sidelink secondary synchronization signals in addition to demodulation reference signals for PSBCH may be used. PSBCH-RSRP using sidelink secondary synchronization signals shall be measured by linear averaging over the power contributions of the resource elements that carry corresponding reference signals. For frequency range 1, the reference point for the PSBCH RSRP shall be the antenna connector of the UE. For frequency range 2, PSBCH-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported PSBCH-RSRP value shall not be lower than the corresponding PSBCH-RSRP of any of the individual receiver branches. |
| --- | --- |
| Applicable for | RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_INACTIVE intra-frequency, RRC_INACTIVE Inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency |

NOTE 1:	The number of resource elements within the considered measurement frequency bandwidth and within the measurement period that are used by the UE to determine PSBCH-RSRP is left up to the UE implementation with the limitation that corresponding measurement accuracy requirements have to be fulfilled.

NOTE 2:	The power per resource element is determined from the energy received during the useful part of the symbol, excluding the CP.

NOTE 3:	It is up to UE implementation to use PSBCH DMRS only or both S-SSS and PSBCH DMRS for PSBCH-RSRP.

5.1.23	PSSCH reference signal received power (PSSCH-RSRP)

| Definition | PSSCH Reference Signal Received Power (PSSCH-RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements of the antenna port(s) that carry demodulation reference signals associated with physical sidelink shared channel (PSSCH), summed over the antenna ports. Demodulation reference signals transmitted on antenna ports 1000 and 1001 shall be used for PSSCH-RSRP determination if two antenna ports are indicated. For frequency range 1, the reference point for the PSSCH-RSRP shall be the antenna connector of the UE. For frequency range 2, PSSCH-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported PSSCH-RSRP value shall not be lower than the corresponding PSSCH-RSRP of any of the individual receiver branches. |
| --- | --- |
| Applicable for | RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_INACTIVE intra-frequency, RRC_INACTIVE Inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency |

NOTE 1:	The power per resource element is determined from the energy received during the useful part of the symbol, excluding the CP.

5.1.24	PSСCH reference signal received power (PSCCH-RSRP)

| Definition | PSCCH Reference Signal Received Power (PSCCH-RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry demodulation reference signals associated with physical sidelink control channel (PSCCH). For frequency range 1, the reference point for the PSCCH-RSRP shall be the antenna connector of the UE. For frequency range 2, PSCCH-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported PSCCH-RSRP value shall not be lower than the corresponding PSCCH-RSRP of any of the individual receiver branches. |
| --- | --- |
| Applicable for | RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_INACTIVE intra-frequency, RRC_INACTIVE Inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency |

NOTE 1:	The power per resource element is determined from the energy received during the useful part of the symbol, excluding the CP.

5.1.25	Sidelink received signal strength indicator (SL RSSI)

| Definition | Sidelink Received Signal Strength Indicator (SL RSSI) is defined as the linear average of the total received power (in [W]) observed in the configured sub-channel in OFDM symbols of a slot configured for PSCCH and PSSCH, starting from the 2nd OFDM symbol. For frequency range 1, the reference point for the SL RSSI shall be the antenna connector of the UE. For frequency range 2, SL RSSI shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported SL RSSI value shall not be lower than the corresponding SL RSSI of any of the individual receiver branches. |
| --- | --- |
| Applicable for | RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_INACTIVE intra-frequency, RRC_INACTIVE Inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency |

5.1.26	Sidelink channel occupancy ratio (SL CR)

| Definition | Sidelink Channel Occupancy Ratio (SL CR) evaluated at slot n is defined as the total number of sub-channels used for its transmissions in slots [n-a, n-1] and granted in slots [n, n+b] divided by the total number of configured sub-channels in the transmission pool over [n-a, n+b]. |
| --- | --- |
| Applicable for | RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_INACTIVE intra-frequency, RRC_INACTIVE Inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency |

NOTE 1:	a is a positive integer and b is 0 or a positive integer; a and b are determined by UE implementation with a+b+1 = 1000 or 1000·2µ slots, according to higher layer parameter sl-TimeWindowSizeCR, b < (a+b+1)/2, and n+b shall not exceed the last transmission opportunity of the grant for the current transmission.

NOTE 2:	SL CR is evaluated for each (re)transmission.

NOTE 3:	In evaluating SL CR, the UE shall assume the transmission parameter used at slot n is reused according to the existing grant(s) in slot [n+1, n+b] without packet dropping.

NOTE 4:	The slot index is based on physical slot index.

NOTE 5:	SL CR can be computed per priority level

NOTE 6:	A resource is considered granted if it is a member of a selected sidelink grant as defined in TS 38.321 [7].

5.1.27	Sidelink channel busy ratio (SL CBR)

| Definition | SL Channel Busy Ratio (SL CBR) measured in slot n is defined as the portion of sub-channels in the resource pool whose SL RSSI measured by the UE exceed a (pre-)configured threshold sensed over a CBR measurement window [n-a, n-1], wherein a is equal to 100 or 100·2µ slots, according to higher layer parameter sl-TimeWindowSizeCBR. When UE is configured to perform partial sensing by higher layers (including when SL DRX is configured), SL RSSI is measured in slots where the UE performs partial sensing and where the UE performs PSCCH/PSSCH reception within the CBR measurement window. The calculation of SL CBR is limited within the slots for which the SL RSSI is measured. If the number of SL RSSI measurement slots within the CBR measurement window is below a (pre-)configured threshold, a (pre-)configured SL CBR value is used. |
| --- | --- |
| Applicable for | RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_INACTIVE intra-frequency, RRC_INACTIVE Inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency |

NOTE 1:	The slot index is based on physical slot index.

-----------------------------------------------------end text proposal for 38.215-----------------------------------------------------