3GPP TSG-RAN WG1 Meeting #115 	R1-2312720

Chicago, USA, November 13-17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0162 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 editorial corrections for TS 38.212 (mirrored to Rel-18) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core, NR_ext_to_71GHz-Core |  |  |  |  |  | Date: |  |  | 2023-11-27 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The description on the value of nPRB,LBRM in clause 5.4.2.1 for PDSCH scheduled by DCI format 4_0/4_1/4_2 may result in misunderstanding that the case with more than one CFR configured to the UE is missing in the spec. - The name of the higher layer parameter pdsch-TimeDomainResourceAllocationListForMultiPDSCH is not aligned with the that defined in TS 38.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the description for the value of nPRB,LBRM in clause 5.4.2.1 to avoid the misunderstanding, per the outcome of the discussion under agenda 7.2 for MBS in RAN1#114bis meeting. - Replace pdsch-TimeDomainResourceAllocationListForMultiPDSCH with pdsch-TimeDomainAllocationListForMultiPDSCH to align the that defined in TS 38.331. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is unclear or incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.1, 7.3.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.4.2.1	Bit selection

The bit sequence after encoding  from Clause 5.3.2 is written into a circular buffer of length  for the -th coded block, where  is defined in Clause 5.3.2.

For the -th code block, let  if  and  otherwise, where, ,  is determined according to Clause 6.1.4.2 in [6, TS 38.214] for UL-SCH and Clause 5.1.3.2 in [6, TS 38.214] for DL-SCH/PCH, assuming the following:

For one TB for DL-SCH with PDSCH scheduled by DCI format 4_0/4_1/4_2,

-	if the PDSCH is scheduled by DCI format 4_1/4_2,

-	maximum number of layers is given by X, where

-	if the higher layer parameter maxMIMO-Layers of pdsch-ConfigMulticast is configured, X is given by that parameter;

-	otherwise, X equals to 1;

-	if the higher layer parameter mcs-Table given by a pdsch-ConfigMulticast for at least one common frequency resource (CFR) is set to 'qam256', maximum modulation order Qm=8 is assumed for DL-SCH; otherwise a maximum modulation order Qm=6 is assumed for DL-SCH;

-	if the PDSCH is scheduled by DCI format 4_0,

-	maximum number of layers is 1;

-	if the higher layer parameter mcs-Table given by a pdsch-ConfigMCCH is set to 'qam256', maximum modulation order Qm=8 is assumed for DL-SCH; otherwise a maximum modulation order Qm=6 is assumed for DL-SCH;

-	if the higher layer parameter mcs-Table given by a pdsch-ConfigMTCH is set to 'qam256', maximum modulation order Qm=8 is assumed for DL-SCH; otherwise a maximum modulation order Qm=6 is assumed for DL-SCH;

-	nPRB=nPRB,LBRM is given by Table 5.4.2.1-1, where the value of nPRB,LBRM for DL-SCH is determined according to the size of the associated CFR if configured to the UE;

-	maximum coding rate of 948/1024;

-	NRE=156∙nPRB;

-	C is the number of code blocks of the transport block determined according to Clause 5.2.2.

< Unchanged parts are omitted >

##### 7.3.1.2.2	Format 1_1

DCI format 1_1 is used for the scheduling of one or multiple PDSCH in one cell.

< Unchanged parts are omitted >

-	ZP CSI-RS trigger – 0, 1, or 2 bits as defined in Clause 5.1.4.2 of [6, TS 38.214]. The bitwidth for this field is determined as bits, where  is the number of aperiodic ZP CSI-RS resource sets configured by higher layer.

For transport block 1:

-	Modulation and coding scheme – 5 bits as defined in Clause 5.1.3.1 of [6, TS 38.214]

-	New data indicator – 1 bit if the number of scheduled PDSCH indicated by the Time domain resource assignment field is 1; otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined based on the maximum number of schedulable PDSCH among all entries in the higher layer parameter pdsch-TimeDomainAllocationListForMultiPDSCH, where each bit corresponds to one scheduled PDSCH as defined in clause 5.1.3 in [6, TS 38.214].

-	Redundancy version – number of bits determined by the following:

-	2 bits as defined in Table 7.3.1.1.1-2 if the number of scheduled PDSCH indicated by the Time domain resource assignment field is 1;

-	otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined by the maximum number of schedulable PDSCHs among all entries in the higher layer parameter pdsch-TimeDomainAllocationListForMultiPDSCH, where each bit corresponds to one scheduled PDSCH as defined in clause 5.1.3 in [6, TS 38.214] and redundancy version is determined according to Table 7.3.1.1.2-34.

For transport block 2 (only present if maxNrofCodeWordsScheduledByDCI equals 2):

-	Modulation and coding scheme – 5 bits as defined in Clause 5.1.3.1 of [6, TS 38.214]

-	New data indicator – 1 bit if the number of scheduled PDSCH indicated by the Time domain resource assignment field is 1; otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined based on the maximum number of schedulable PDSCH among all entries in the higher layer parameter pdsch-TimeDomainAllocationListForMultiPDSCH, where each bit corresponds to one scheduled PDSCH as defined in clause 5.1.3 in [6, TS 38.214].

-	Redundancy version – number of bits determined by the following:

-	2 bits as defined in Table 7.3.1.1.1-2 if the number of scheduled PDSCH indicated by the Time domain resource assignment field is 1;

-	otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined by the maximum number of schedulable PDSCHs among all entries in the higher layer parameter pdsch-TimeDomainAllocationListForMultiPDSCH, where each bit corresponds to one scheduled PDSCH as defined in clause 5.1.3 in [6, TS 38.214] and redundancy version is determined according to Table 7.3.1.1.2-34.

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the value of maxNrofCodeWordsScheduledByDCI for the indicated bandwidth part equals 2 and the value of maxNrofCodeWordsScheduledByDCI for the active bandwidth part equals 1, the UE assumes zeros are padded when interpreting the "Modulation and coding scheme", "New data indicator", and "Redundancy version" fields of transport block 2 according to Clause 12 of [5, TS38.213], and the UE ignores the "Modulation and coding scheme", "New data indicator", and "Redundancy version" fields of transport block 2 for the indicated bandwidth part.

< Unchanged parts are omitted >