3GPP TSG-RAN WG1 Meeting #115	R1-2312719

# Chicago, USA, November 13th – November 17th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0480 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 editorial corrections for TS 38.214 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Core, NR_SmallData_INACTIVE-Core NR_MBS-Core |  |  |  | Date: |  | 2023-12-01 |
| Category: | F |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NR_FeMIMO In clause 5.1.5: In TS 38.214, if the TCI-State or TCI-UL-State configurations are absent in a BWP of the CC, the UE can apply the TCI-State or TCI-UL-State configurations from a reference BWP of a reference CC, but how to determine the reference BWP of the reference CC is not clear. Indeed, an RRC parameter unifiedTCI-StateRef is used for this determination. This should be clarified in the spec explicitly, to avoid ambiguity. In clause 6.2.1.2: In RAN1#106-e, the extended SRS resource set configuration based on UE capability was adopted by the following agreement. Agreement For antenna switching SRS, support maximum one SRS resource set for periodic SRS and maximum 2 SRS resource sets for semi-persistent SRS. - Note: the two SP-SRS resource sets are not activated at the same time - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - Applies for all supported xTyR where y<=8 - For each xTyR antenna switching (except for 4T6R if supported), each periodic or semi-persistent resource set contains y/x resources. This feature is UE optional: For UEs that do not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS. Based on the above agreement, when a UE reports srs-AntennaSwitching2SP-1Periodic, the UE can be configured with up to 2 SRS resource set for semi-persistent and up to 1 SRS resource set for periodic, and this applies for all supported xTyR where y <=8. However, it is not clearly captured for the case of x=y (i.e., 1T1R, 2T2R, and 4T4R). Also, one additional minor correction is that, for a bullet of “otherwise”, which is correpsonding to another bullet for 2T4R if the UE is indicating srs-AntennaSwitching2SP-1Periodic and/or srs-ExtensionAperiodicSRS, it is not clear which antenna switching configuration is applied because an exact antenna switching configuration, e.g., “for 2T4R”, is missing. In clause 5.1.5, when the number of codepoints in the DCI field ‘Transmission Configuration Indication’ of DCI format 1_2 is less than the total number of TCI codepoints activated by a MAC CE activation command, a subset of activated TCI codepoints can be applied for DCI format 1_2. In the TS 38.214, however, such operation is not specified for Rel-17 unified TCI state activation command. In clause 6.2.1.2, first, for the case of 1T2R AS SRS, the UE is capable of SRS transmission on 1 Tx port over a total of 2 Rx ports through antenna switching. In such case, it is 1-to-1 mapping between SRS port and UE receive antenna port. However, it may mislead to 1-to-2 mapping between SRS port and UE receive antenna ports as per the current specification in the case of 1T2R AS SRS (i.e., SRS port of each SRS resource is associated with a different UE antenna port pair), which should be corrected. Second, for the case of 1T4R AS SRS, there is a syntax error when four aperiodic SRS resource sets are configured, i.e., “if UE is capable if supporting four sets”. Literally, it can be changed to “if UE is capable if of supporting four sets”. Nevertheless, note that the precondition of UE is indicating srs-ExtensionAperiodicSRS has already capture this point, and therefore it is more proper to remove this sentence and then keep alignment with other cases from the perspective of readability. Third, for the case of 1T6R/1T8R/2T6R/2T8R/4T8R AS SRS, some minor editorial updates with regards to singular and plural forms of “SRS resource set” were corrected for refinement. I.e., “zero or one SRS resource sets”, “zero or one or two SRS resource sets”. To avoid any ambiguity, the current specification in terms of the above parts should be modified accordingly. NR_SmallData_INACTIVE-Core In clause 6.2.2, a typo in a higher layer parameter. NR_MBS-Core In clause 5.1.2.1 The number of MBS SPS PDSCH repetitions is not clear according to the current specifications when repetitionNumber is configured in the pdsch-ConfigMulticast, in this case, pdsch-AggregationFactor is not configured in the associated SPS-Config according to the following agreement made in RAN1#106e-bis meeting. Agreement: For slot-level repetition for SPS GC-PDSCH for multicast RRC_CONNECTED UEs. - Config A or Config B can be configured to UE: - (Config A) UE can be optionally configured with pdsch-AggregationFactor per SPS-Config-Multicast. - (Config B) UE can be optionally configured with TDRA table with repetitionNumber as part of the TDRA table in PDSCH-Config-Multicast. If UE is configured with Config B, UE does not expect to be configured with Config A for the same SPS group-common PDSCH. - For Config A, if pdsch-AggregationFactor in SPS-Config-Multicast is not configured, default value is - Alt1: equal to 1. There can be two different interpretations according to the current specifications descriptions. - The repetition number is 1 according to the text “When receiving PDSCH scheduled by DCI format 4_1 or 4_2 for multicast reception in PDCCH with CRC scrambled by G-CS-RNTI, or PDSCH without corresponding PDCCH transmission using associated SPS-Config and activated by the DCI format 4_1 or 4_2 in PDCCH with CRC scrambled by G-CS-RNTI, the same symbol allocation is applied across the pdsch-AggregationFactor, in associated SPS-Config if configured, or 1 otherwise, consecutive slots.” The repetition number is repetitionNumber in the activating DCI format according to the text “ When receiving PDSCH scheduled by DCI in PDCCH with CRC scrambled by G-CS-RNTI for multicast reception or G-RNTI, if the DCI field 'Time domain resource assignment' indicates an entry which contains repetitionNumber in PDSCH-TimeDomainResourceAllocation in the pdsch-ConfigMulticast or PDSCH-Config-Broadcast, the same SLIV is applied for all PDSCH transmission occasions across the repetitionNumber consecutive slots. When receiving PDSCH scheduled without corresponding PDCCH transmission using associated SPS-Config and activated by DCI in PDCCH with CRC scrambled by G-CS-RNTI for multicast reception, if the DCI field 'Time domain resource assignment' of the activating DCI indicates an entry which contains repetitionNumber in PDSCH-TimeDomainResourceAllocation in the pdsch-ConfigMulticast, the same SLIV is applied for all PDSCH transmission occasions across the repetitionNumber consecutive slots.” In clause 5.1.6.5, if UE is configured to measure PRS, UE may be configured that PRS is lower priority than PDCCH and PDSCH with ‘priority indicator=1’ and higher priority than other PDSCH/CSI-RS, the specification text in TS 38.214 section 5.1.6.5 is copied as following - with value 'st2' where the DL PRS is lower priority than PDCCH and the PDSCH scheduled by DCI formats 1_1 or 1_2 with the priority indicator field in the corresponding DCI format set to 1, and is higher priority than other DL signals and channels. For supporting multicast transmission, priority indicator field can be included in the DCI format 4_2. When the priority indicator is set to 1, it means that the priority of multicast PSDCH is higher than other DL signals and shall not be dropped. But according to current specification, the multicast PDSCH with ‘priority indicator=1’ will be dropped due to PRS receiving. In clause 5.1.2.1, the number of PDSCH repetitions for MBS can be configured by RRC signaling or indicated by the DCI. For the second case, how to determine the redundancy version for PDSCH repetitions is still missing in the current specification. The same method can be applied as that for the first case. |
| Summary of change: |  | NR_FeMIMO In clause 5.1.5: The RRC parameter unifiedTCI-StateRef is explicitly used to determine the reference BWP of the reference CC if the TCI-State or TCI-UL-State configurations are absent in a BWP of the CC. In clause 6.2.1.2: - Clarify up to 2 semi-persistent and up to 1 periodic SRS resource set can be configured for 1T1R, 2T2R, and 4T4R, if the UE is indicating srs-AntennaSwitching2SP-1Periodic. - Add “for 2T4R” for a bullet of “otherwise” in order to clarify the bullet of “otherwise” is for the case of 2T4R. In clause 5.1.5, added the Rel-17 unified TCI state activation command to the case when the number of codepoints in the TCI field of DCI format 1_2 is less than the number of TCI codepoints activated by the activation command. In clause 6.2.1.2, a total of three changes were made: - For 1T2R AS SRS, correct that the mapping between SRS port and UE receive antenna port should be 1-to-1. - For 1T4R AS SRS, remove the sentence “if UE is capable if supporting four sets,” to avoid the syntax error and also keep alignment of description with other cases. - For 1T6R/1T8R/2T6R/2T8R/4T8R AS SRS, correct the singular and plural forms of “SRS resource set”. NR_SmallData_INACTIVE-Core In clause 6.2.2, corrected a typo in a higher layer parameter. NR_MBS-Core In clause 5.1.2.1, clarified that the number of MBS SPS PDSCH repetitions is determined based on activating DCI format if the DCI format indicates a TDRA row with repetitionNumber. In clause 5.1.6.5, added the case of DCI format 4_2 including the priority indicator field =1. In clause 5.1.2.1, added the RV determination for the PDSCH repetitions indicated by the TDRA table |  |  |  |  |  |

| Consequences if not approved: | NR_FeMIMO In clause 5.1.5: How to determine the reference BWP of the reference CC if the TCI-State or TCI-UL-State configurations are absent in a BWP of the CC is not clear. In clause 6.2.1.2: The possible cases of SRS resource / SRS resource set configuration for above cases of SRS for antenna switching is not clear. In clause 5.1.5, UE behavior of when the number of codepoints in the TCI field of DCI format 1_2 is less than the number of TCI codepoints activated by a Rel-17 unified TCI state activation command are undefined. In clause 6.2.1.2, for AS SRS, the mapping between SRS port and UE receive antenna port in case of 1T2R may be incorrectly deployed by UE, besides some ambiguities may negatively impact the understanding of the specification. NR_SmallData_INACTIVE-Core In clause 6.2.2, unclear specification. NR_MBS-Core In clause 5.1.2.1, the UE cannot determine the redundancy version for PDSCH repetitions indicated by the TDRA table |  |  |  |
| --- | --- | --- | --- | --- |
| Clauses affected: | 5.1.2.1, 5.1.5, 5.1.6.5, 6.2.1.2, 6.2.2 |  |  |  |
|  | Y | N |  |  |
| Other specs |  | X | Other core specifications | TS/TR ... CR ... |
| affected: |  | X | Test specifications | TS/TR ... CR ... |
| (show related CRs) |  | X | O&M Specifications | TS/TR ... CR ... |
| Other comments: |  |  |  |  |
| This CR's revision history: |  |  |  |  |

<omitted text>

#### 5.1.2.1	Resource allocation in time domain

<omitted text>

When receiving PDSCH scheduled by DCI format 4_1, or 4_2 in PDCCH with CRC scrambled by G-RNTI for multicast, if the UE is configured with pdsch-AggregationFactor in the MBS-RNTI-SpecificConfig associated with the corresponding G-RNTI for multicast, the same symbol allocation is applied across the pdsch-AggregationFactor consecutive slots. When receiving PDSCH scheduled by DCI format 4_1 or 4_2 for multicast reception in PDCCH with CRC scrambled by G-CS-RNTI, or PDSCH without corresponding PDCCH transmission using associated SPS-Config and activated by the DCI format 4_1 or 4_2 in PDCCH with CRC scrambled by G-CS-RNTI, the same symbol allocation is applied across the pdsch-AggregationFactor in associated SPS-Config if configured, or across repetitionNumber in PDSCH-TimeDomainResourceAllocation in pdsch-ConfigMulticast if provided by an entry indicated by the 'Time domain resource assignment' field of the activating DCI, or 1 otherwise, consecutive slots. The redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, …pdsch-AggregationFactor -1, is determined according to table 5.1.2.1-2 and "rvid indicated by the DCI scheduling the PDSCH" in table 5.1.2.1-2 is assumed to be 0 for PDSCH scheduled without corresponding PDCCH transmission using SPS-Config and activated by DCI format 4_1 or 4_2. When receiving PDSCH scheduled by DCI format 4_0 in PDCCH with CRC scrambled by G-RNTI for broadcast, if the UE is configured with pdsch-AggregationFactor in the PDSCH-ConfigPTM, the same symbol allocation is applied across the pdsch-AggregationFactor consecutive slots, and the redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, …pdsch-AggregationFactor -1, is determined according to table 5.1.2.1-2.

When receiving PDSCH scheduled by DCI in PDCCH with CRC scrambled by G-CS-RNTI for multicast reception or G-RNTI, if the DCI field 'Time domain resource assignment' indicates an entry which contains repetitionNumber in PDSCH-TimeDomainResourceAllocation in the pdsch-ConfigMulticast or PDSCH-Config-Broadcast, the same SLIV is applied for all PDSCH transmission occasions across the repetitionNumber consecutive slots. When receiving PDSCH scheduled without corresponding PDCCH transmission using associated SPS-Config and activated by DCI in PDCCH with CRC scrambled by G-CS-RNTI for multicast reception, if the DCI field 'Time domain resource assignment' of the activating DCI indicates an entry which contains repetitionNumber in PDSCH-TimeDomainResourceAllocation in the pdsch-ConfigMulticast, the same SLIV is applied for all PDSCH transmission occasions across the repetitionNumber consecutive slots. The redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, …repetitionNumber -1, is determined according to table 5.1.2.1-2 and "rvid indicated by the DCI scheduling the PDSCH" in table 5.1.2.1-2 is assumed to be 0 for PDSCH scheduled without corresponding PDCCH transmission using sps-Config and activated by DCI format with CRC scrambled by G-CS-RNTI.

If a UE is configured with higher layer parameter repetitionNumber or if the UE is configured by repetitionScheme set to one of ' fdmSchemeA', ' fdmSchemeB' and 'tdmSchemeA', the UE does not expect to be configured with pdsch-AggregationFactor for the same PDSCH.

If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH in which one or more rows contain multiple SLIVs for PDSCH, the UE does not expect to be configured with higher layer parameter repetitionNumber in pdsch-TimeDomainAllocationListForMultiPDSCH.

If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH in which one or more rows contain multiple SLIVs for PDSCH on a DL BWP of a serving cell, the UE does not apply pdsch-AggregationFactor in PDSCH-config, if configured, to DCI format 1_1 on the DL BWP of the serving cell.

If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH in which one or more rows contain multiple SLIVs for PDSCH on a DL BWP of a serving cell, when any two DL DCIs end in the same symbol and at least one of the DCIs schedules multiple PDSCHs, the UE does not expect that the scheduled PDSCH(s) by the two DCIs  have overlapping spans, where the span associated with a DCI is defined from the beginning of the first scheduled PDSCH or up to the end of the last scheduled PDSCH.

Table 5.1.2.1-2: Applied redundancy version when pdsch-AggregationFactor or repetitionNumber is present

| rvid indicated by the DCI scheduling the PDSCH | rvid to be applied to nth transmission occasion |  |  |  |
| --- | --- | --- | --- | --- |
|  | n mod 4 = 0 | n mod 4 = 1 | n mod 4 = 2 | n mod 4 = 3 |
| 0 | 0 | 2 | 3 | 1 |
| 2 | 2 | 3 | 1 | 0 |
| 3 | 3 | 1 | 0 | 2 |
| 1 | 1 | 0 | 2 | 3 |

<omitted text>

### 5.1.5	Antenna ports quasi co-location

The UE can be configured with a list of up to M TCI-State configurations within the higher layer parameter PDSCH-Config to decode PDSCH according to a detected PDCCH with DCI intended for the UE and the given serving cell, where M depends on the UE capability maxNumberConfiguredTCIstatesPerCC. Each TCI-State contains parameters for configuring a quasi co-location relationship between one or two downlink reference signals and the DM-RS ports of the PDSCH, the DM-RS port of PDCCH or the CSI-RS port(s) of a CSI-RS resource. The quasi co-location relationship is configured by the higher layer parameter qcl-Type1 for the first DL RS, and qcl-Type2 for the second DL RS (if configured). For the case of two DL RSs, the QCL types shall not be the same, regardless of whether the references are to the same DL RS or different DL RSs. The quasi co-location types corresponding to each DL RS are given by the higher layer parameter qcl-Type in QCL-Info and may take one of the following values:

-	'typeA': {Doppler shift, Doppler spread, average delay, delay spread}

-	'typeB': {Doppler shift, Doppler spread}

-	'typeC': {Doppler shift, average delay}

-	'typeD': {Spatial Rx parameter}

The UE can be configured with a list of up to 128 TCI-State configurations, within the higher layer parameter dl-OrJointTCI-StateList in PDSCH-Config for providing a reference signal for the quasi co-location for DM-RS of PDSCH and DM-RS of PDCCH in a BWP/CC, for CSI-RS, and to provide a reference, if applicable, for determining UL TX spatial filter for dynamic-grant and configured-grant based PUSCH and PUCCH resource in a BWP/CC, and SRS.

If the TCI-State or TCI-UL-State configurations are absent in a BWP of the CC, the UE can apply the TCI-State or TCI-UL-State configurations from a reference BWP of a reference CC configured by unifiedTCI-StateRef. The UE is not expected to be configured with tci-StatesToAddModList, SpatialRelationInfo or PUCCH-SpatialRelationInfo, except SpatialRelationInfoPos in a CC in a band, if the UE is configured with dl-OrJointTCI-StateList or ul-TCI-StateList in any CC in the same band. The UE can assume that when the UE is configured with tci-StatesToAddModList in any CC in the CC list configured by simultaneousTCI-UpdateList1-r16, simultaneousTCI-UpdateList2-r16, simultaneousSpatial-UpdatedList1-r16, or simultaneousSpatial-UpdatedList2-r16, the UE is not configured with dl-OrJointTCI-StateList or ul-TCI-StateList in any CC within the same band in the CC list.

The UE receives an activation command, as described in clause 6.1.3.14 of [10, TS 38.321] or 6.1.3.47 of [10, TS 38.321], used to map up to 8 TCI states and/or pairs of TCI states, with one TCI state for DL channels/signals and/or one TCI state for UL channels/signals to the codepoints of the DCI field 'Transmission Configuration Indication' for one or for a set of CCs/DL BWPs, and if applicable, for one or for a set of CCs/UL BWPs. When a set of TCI state IDs are activated for a set of CCs/DL BWPs and if applicable, for a set of CCs/UL BWPs, where the applicable list of CCs is determined by the indicated CC in the activation command, the same set of TCI state IDs are applied for all DL and/or UL BWPs in the indicated CCs. If the activation command maps TCI-State and/or TCI-UL-State to only one TCI codepoint, the UE shall apply the indicated TCI-State and/or TCI-UL-State to one or to a set of CCs /DL BWPs, and if applicable, to one or to a set of CCs /UL BWPs once the indicated mapping for the one single TCI codepoint is applied as described in [11, TS 38.133].

When the bwp-id or cell for QCL-TypeA/D source RS in a QCL-Info of the TCI state is not configured, the UE assumes that QCL-TypeA/D source RS is configured in the CC/DL BWP where TCI state applies.

When tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, a UE configured with dl-OrJointTCI-StateList with activated TCI-State or ul-TCI-StateList with activated TCI-UL-State receives DCI format 1_1/1_2 providing indicated TCI-State and/or TCI-UL-State for a CC or all CCs in the same CC list configured by simultaneousU-TCI-UpdateList1-r17, simultaneousU-TCI-UpdateList2-r17, simultaneousU-TCI-UpdateList3-r17, simultaneousU-TCI-UpdateList4-r17. The DCI format 1_1/1_2 can be with or without, if applicable, DL assignment. If the DCI format 1_1/1_2/ is without DL assignment, the UE can assume the following:

-	CS-RNTI is used to scramble the CRC for the DCI

-	The values of the following DCI fields are set as follows:

-	RV = all '1's

-	MCS = all '1's

-	NDI = 0

-	Set to all '0's for FDRA Type 0, or all '1's for FDRA Type 1, or all '0's for dynamicSwitch (same as in Table 10.2-4 of [6, TS 38.213]).

After a UE receives an initial higher layer configuration of dl-OrJointTCI-StateList with more than one TCI-State and before application of an indicated TCI state from the configured TCI states:

-	The UE assumes that DM-RS of PDSCH and DM-RS of PDCCH and the CSI-RS applying the indicated TCI state are quasi co-located with the SS/PBCH block the UE identified during the initial access procedure

After a UE receives an initial higher layer configuration of dl-OrJointTCI-StateList with more than one TCI-State or ul-TCI-StateList with more than one TCI-UL-State and before application of an indicated TCI state from the configured TCI states:

-	The UE assumes that the UL TX spatial filter, if applicable, for dynamic-grant and configured-grant based PUSCH and PUCCH, and for SRS applying the indicated TCI state, is the same as that for a PUSCH transmission scheduled by a RAR UL grant during the initial access procedure

After a UE receives a higher layer configuration of dl-OrJointTCI-StateList with more than one TCI-State as part of a Reconfiguration with sync procedure as described in [12, TS 38.331] and before applying an indicated TCI state from the configured TCI states:

-	The UE assumes that DM-RS of PDSCH and DM-RS of PDCCH, and the CSI-RS applying the indicated TCI state are quasi co-located with the SS/PBCH block or the CSI-RS resource the UE identified during the random access procedure initiated by the Reconfiguration with sync procedure as described in [12, TS 38.331].

After a UE receives a higher layer configuration of dl-OrJointTCI-StateList with more than one TCI-State or more than one TCI-UL-State as part of a Reconfiguration with sync procedure as described in [12, TS 38.331] and before applying an indicated TCI state from the configured TCI states:

-	The UE assumes that the UL TX spatial filter, if applicable, for dynamic-grant and configured-grant based PUSCH and PUCCH, and for SRS applying the indicated TCI state, is the same as that for a PUSCH transmission scheduled by a RAR UL grant during random access procedure initiated by the Reconfiguration with sync procedure as described in [12, TS 38.331].

If a UE receives a higher layer configuration of dl-OrJointTCI-StateList with a single TCI-State, that can be used as an indicated TCI state, the UE obtains the QCL assumptions from the configured TCI state for DM-RS of PDSCH and DM-RS of PDCCH, and the CSI -RS applying the indicated TCI state.

If a UE receives a higher layer configuration of dl-OrJointTCI-StateList with a single TCI-State or ul-TCI-StateList with a single TCI-UL-State, that can be used as an indicated TCI state, the UE determines an UL TX spatial filter, if applicable, from the configured TCI state for dynamic-grant and configured-grant based PUSCH and PUCCH, and SRS applying the indicated TCI state.

When a UE configured with dl-OrJointTCI-StateList would transmit a PUCCH with positive HARQ-ACK or a PUSCH with positive HARQ-ACK corresponding to the DCI carrying the TCI State indication and without DL assignment, or corresponding to the PDSCH scheduled by the DCI carrying the TCI State indication, and if the indicated TCI State is different from the previously indicated one, the indicated TCI-State and/or TCI-UL-State should be applied starting from the first slot that is at least  beamAppTime symbols after the last symbol of the PUCCH or the PUSCH. The first slot and the  beamAppTime symbols are both determined on the active BWP with the smallest SCS among the BWP(s) from the CCs applying the indicated TCI-State or TCI-UL-State that are active at the end of the PUCCH or the PUSCH carrying the positive HARQ-ACK.

If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH in which one or more rows contain multiple SLIVs for PDSCH on a DL BWP of a serving cell, and the UE is receiving a DCI carrying the TCI-State indication and without DL assignment, the UE does not expect that the number of indicated SLIVs in the row of the pdsch-TimeDomainAllocationListForMultiPDSCH by the DCI is more than one.

If the UE is configured with SSB-MTC-AddtionalPCI and with PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet, the UE receives an activation command for CORESET associated with each coresetPoolIndex, as described in clause 6.1.3.14 of [10, TS 38.321], used to map up to 8 TCI states to the codepoints of the DCI field 'Transmission Configuration Indication' in one CC/DL BWP. When a set of TCI state IDs are activated for a coresetPoolIndex, the activated TCI states corresponding to one coresetPoolIndex is associated with the serving cell physical cell ID and activated TCI states corresponding to another coresetPoolIndex can be associated with another physical cell ID.

When a UE supports two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' the UE may receive an activation command, as described in clause 6.1.3.24 of [10, TS 38.321], the activation command is used to map up to 8 combinations of one or two TCI states to the codepoints of the DCI field 'Transmission Configuration Indication'. The UE is not expected to receive more than 8 TCI states in the activation command.

When the DCI field 'Transmission Configuration Indication' is present in DCI format 1_2 and when the number of codepoints S in the DCI field 'Transmission Configuration Indication' of DCI format 1_2 is smaller than the number of TCI codepoints that are activated by the activation command, as described in clause 6.1.3.14, 6.1.3.24 and 6.1.3.47 of [10, TS38.321], only the first S activated codepoints are applied for DCI format 1_2.

When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, the indicated mapping between TCI states and codepoints of the DCI field 'Transmission Configuration Indication' should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+2μ2μKmac∙kmac where  is the SCS configuration for the PUCCH and μKmac is the subcarrier spacing configuration for kmac with a value of 0 for frequency range 1, and kmac is provided by K-Mac or kmac=0 if K-Mac is not provided. If tci-PresentInDCI is set to 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET scheduling the PDSCH, and the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than timeDurationForQCL if applicable, after a UE receives an initial higher layer configuration of TCI states and before reception of the activation command, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the SS/PBCH block determined in the initial access procedure with respect to qcl-Type set to 'typeA', and when applicable, also with respect to qcl-Type set to 'typeD'.

<omitted text>

#### 5.1.6.5	PRS reception procedure

The UE can be configured with one or more DL PRS resource set configuration(s) as indicated by the higher layer parameters NR-DL-PRS-ResourceSet and NR-DL-PRS-Resource as defined by Clause 6.4.3 [17, TS 37.355]. Each DL PRS resource set consists of K≥1 DL PRS resource(s) where each has an associated spatial transmission filter. The UE can be configured with one or more DL PRS positioning frequency layer configuration(s) as indicated by the higher layer parameter NR-DL-PRS-PositioningFrequencyLayer. A DL PRS positioning frequency layer is defined as a collection of DL PRS resource sets which have common parameters configured by NR-DL-PRS-PositioningFrequencyLayer.

<omitted text>

The UE is expected to measure the DL PRS outside the measurement gap, subject to UE capability, if the DL PRS is inside the active DL BWP and has the same numerology as the active DL BWP and is within the DL PRS processing window indicated by higher layer parameter DL-PPW-PreConfig. The UE is not expected to measure the DL PRS outside the measurement gap if the expected received timing difference between the DL PRS from the non-serving cell and that from the serving cell, determined by the higher layer parameters nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncertainty, is larger than maximum Rx timing difference provided by UE capability. For receiving the DL PRS outside the measurement gap and within the DL PRS processing window, the priority between DL PRS and SSB is defined in [11, TS 38.133] and the UE determines the DL PRS priority as indicated by higher layer parameter priority subject to UE capability or as implied by UE capability, except for SSB:

-	with value 'st1' where the DL PRS is higher priority than all the DL signals and channels, or

-	with value 'st2' where the DL PRS is lower priority than PDCCH and the PDSCH scheduled by DCI formats 1_1 or 1_2 or 4_2 with the priority indicator field in the corresponding DCI format set to 1, and is higher priority than other DL signals and channels, or

-	with value 'st3' where the DL PRS is lower priority than all the DL signals and channels.

Inside one DL-PPW-PreConfig the UE is only expected to measure a single DL PRS positioning frequency layer

<omitted text>

#### 6.2.1.2	UE sounding procedure for DL CSI acquisition

When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set as 'antennaSwitching', the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitch ('t1r2' for 1T2R, 't1r1-t1r2' for 1T=1R/1T2R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't1r1-t1r2-t1r4' for 1T=1R/1T2R/1T4R, 't1r4-t2r4' for 1T4R/2T4R, 't1r1-t1r2-t2r2-t2r4' for 1T=1R/1T2R/2T=2R/2T4R, 't1r1-t1r2-t2r2-t1r4-t2r4' for 1T=1R/1T2R/2T=2R/1T4R/2T4R, 't1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r1-t2r2' for 1T=1R/2T=2R, 't4r4' for 4T=4R, or 't1r1-t2r2-t4r4' for 1T=1R/2T=2R/4T=4R) or the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitchBeyond4Rx (‘t1r1’ for 1T=1R, ‘t2r2’ for 2T=2R, ‘t1r2’ for 1T2R, ‘t4r4’ for 4T=4R, ‘t2r4’ for 2T4R, ‘t1r4’ for 1T4R, ‘t2r6’ for 2T6R, ‘t1r6’ for 1T6R, ‘t4r8’ for 4T8R, ‘t2r8’ for 2T8R, ‘t1r8’ for 1T8R):

-	For 1T2R, if the UE is indicating srs-AntennaSwitching2SP-1Periodic and/or srs-ExtensionAperiodicSRS:

-	when the UE is indicating srs-AntennaSwitching2SP-1Periodic only, then up to two SRS resource sets with resourceType in SRS-ResourceSet set to 'semi-persistent' and up to one SRS resource set with resourceType in SRS-ResourceSet set to 'periodic' can be configured, or up to two SRS resource sets configured with a different value for the higher layer parameter resourceType in SRS-ResourceSet can be configured, where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time, each SRS resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of the second resource in the set is associated with a different UE antenna port than the SRS port of the first resource in the same set.

-	when the UE is indicating srs-ExtensionAperiodicSRS only, then up to two SRS resource sets with resourceType in SRS-ResourceSet set to 'aperiodic' and up to one SRS resource set with resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' can be configured, or up to two SRS resource sets configured with a different value for the higher layer parameter resourceType in SRS-ResourceSet can be configured. In the case of two resource sets with resourceType in SRS-ResourceSet set to 'aperiodic', a total of two SRS resources are transmitted in different symbols of two different slots, the SRS port of each SRS resource in the given two sets is associated with a different UE antenna port and the two sets are each configured with one SRS resource. In the case of the one resource set with resourceType in SRS-ResourceSet set to 'aperiodic' is configured, a total of two SRS resources transmitted in different symbols of one slot and where the SRS port of the second resource in the given set is associated with a different UE antenna port than the SRS port of the first resource in the same set. Each SRS resource set with 'resourceType' in SRS-ResourceSet set to 'periodic' or 'semi-persistent' has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of the second resource in the set is associated with a different UE antenna port than the SRS port of the first resource in the same set.

-	zero or one or two SRS resource sets configured with different value of resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' if the UE is not indicating srs-AntennaSwitching2SP-1Periodic, or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and up to one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' if the UE is indicating srs-AntennaSwitching2SP-1Periodic, where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time. An SRS resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of the second resource in the set is associated with a different UE antenna port than the SRS port of the first resource in the same set, and

-	zero or one SRS resource set with resourceType in SRS-ResourceSet set to 'aperiodic' if the UE is not indicating srs-ExtensionAperiodicSRS, or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic' if the UE is indicating srs-ExtensionAperiodicSRS, where in the case of one resource set, a total of two SRS resources transmitted in different symbols of one slot and where the SRS port of the second resource in the given set is associated with a different UE antenna port than the SRS port of the first resource in the same set. In the case of two resource sets, a total of two SRS resources are transmitted in different symbols of two different slots, the SRS port of each SRS resource in the given two sets is associated with a different UE antenna port. The two sets are each configured with one SRS resource, or

-	otherwise, for 1T2R, up to two SRS resource sets configured with a different value for the higher layer parameter resourceType in SRS-ResourceSet set, where each set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of the second resource in the set is associated with a different UE antenna port than the SRS port of the first resource in the same set, or

-	For 2T4R, if the UE is indicating srs-AntennaSwitching2SP-1Periodic and/or srs-ExtensionAperiodicSRS:

-	when the UE is indicating srs-AntennaSwitching2SP-1Periodic only, then up to two SRS resource sets with resourceType in SRS-ResourceSet set to 'semi-persistent' and up to one SRS resource set with resourceType in SRS-ResourceSet set to 'periodic' can be configured, or up to two SRS resource sets configured with a different value for the higher layer parameter resourceType in SRS-ResourceSet can be configured, where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time, each SRS resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of two SRS ports, and the SRS port pair of the second resource is associated with a different UE antenna port pair than the SRS port pair of the first resource.

-	when the UE is indicating srs-ExtensionAperiodicSRS only, then up to two SRS resource sets with resourceType in SRS-ResourceSet set to 'aperiodic' and up to one SRS resource set with resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' can be configured, or up to two SRS resource sets configured with a different value for the higher layer parameter resourceType in SRS-ResourceSet can be configured. In the case of two resource sets with resourceType in SRS-ResourceSet set to 'aperiodic', a total of two SRS resources are transmitted in different symbols of two different slots, the SRS port pair of each SRS resource in the given two sets is associated with a different UE antenna port pair and the two sets are each configured with one SRS resource. In the case of one resource set with resourceType in SRS-ResourceSet set to 'aperiodic' is configured, a total of two SRS resources transmitted in different symbols in the same slot, each SRS resource in a given set consisting of two SRS ports, and the SRS port pair of the second resource is associated with a different UE antenna port pair than the SRS port pair of the first resource. Each SRS resource set with 'resourceType' in SRS-ResourceSet set to 'periodic' or 'semi-persistent' has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of two SRS ports, and the SRS port pair of the second resource is associated with a different UE antenna port pair than the SRS port pair of the first resource.

-	zero or one or two SRS resource sets configured with a different value for the higher layer parameter resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' if the UE is not indicating srs-AntennaSwitching2SP-1Periodic, or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and up to one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' if the UE is indicating srs-AntennaSwitching2SP-1Periodic, where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time. Each SRS resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of two SRS ports, and the SRS port pair of the second resource is associated with a different UE antenna port pair than the SRS port pair of the first resource, and,

-	zero or one SRS resource set configured with resourceType in SRS-ResourceSet set to 'aperiodic' if the UE is not indicating srs-ExtensionAperiodicSRS, or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic' if the UE is indicating srs-ExtensionAperiodicSRS, where in the case of one resource set, a total of two SRS resources transmitted in different symbols in the same slot, each SRS resource in a given set consisting of two SRS ports, and the SRS port pair of the second resource is associated with a different UE antenna port pair than the SRS port pair of the first resource. In the case of two resource sets, a total of two SRS resources are transmitted in different symbols of two different slots, the SRS port pair of each SRS resource in the given two sets is associated with a different UE antenna port pair. The two sets are each configured with one SRS resource, or

-	otherwise, for 2T4R, up to two SRS resource sets configured with a different value for the higher layer parameter  resourceType in SRS-ResourceSet, where each SRS resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of two SRS ports, and the SRS port pair of the second resource is associated with a different UE antenna port pair than the SRS port pair of the first resource, or

-	For 1T4R, if the UE is indicating srs-AntennaSwitching2SP-1Periodic and/or srs-ExtensionAperiodicSRS and/or srs-OneAP-SRS,

-	zero or one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' if the UE is not indicating srs-AntennaSwitching2SP-1Periodic, or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and up to one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' if the UE is indicating srs-AntennaSwitching2SP-1Periodic, where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time. Each SRS resource set has four SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of each resource is associated with a different UE antenna port, and

-	zero or two SRS resource sets each configured with resourceType in SRS-ResourceSet set to 'aperiodic', if the UE is not indicating srs-ExtensionAperiodicSRS or srs-OneAP-SRS, or zero or one or two or four SRS resource sets each configured with resourceType in SRS-ResourceSet set to 'aperiodic' if the UE is indicating srs-ExtensionAperiodicSRS and srs-OneAP-SRS, or zero or two or four SRS resource sets each configured with resourceType in SRS-ResourceSet set to 'aperiodic' if the UE is indicating srs-ExtensionAperiodicSRS only, or zero or one or two SRS resource sets each configured with resourceType in SRS-ResourceSet set to 'aperiodic' if the UE is indicating srs-OneAP-SRS only. In the case of one resource set, a total of four SRS resources are transmitted in different symbols in the same slot, and the SRS port of each resource in the given set is associated with a different UE antenna port. In the case of two resource sets, a total of four SRS resources are transmitted in different symbols of two different slots, and the SRS port of each SRS resource in the given two sets is associated with a different UE antenna port. The two sets are each configured with two SRS resources, or one set is configured with one SRS resource and the other set is configured with three SRS resources. In the case of four resource sets, a total of four SRS resources are transmitted in different symbols of four different slots, and the SRS port of each SRS resource in the given four sets is associated with a different UE antenna port. The four sets are each configured with one SRS resource. The UE shall expect that the value of the higher layer parameter aperiodicSRS-ResourceTrigger or the value of an entry in AperiodicSRS-ResourceTriggerList in each SRS-ResourceSet is the same, and the value of the higher layer parameter slotOffset in each SRS-ResourceSet is different when none of the SRS resource sets is configured with parameter availableSlotOffsetList across all configured BWPs in a component carrier. Or,

-	otherwise, for 1T4R,

-	zero or one SRS resource set configured with higher layer parameter resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' with four SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of each resource is associated with a different UE antenna port, and

-	zero or two SRS resource sets each configured with higher layer parameter resourceType in SRS-ResourceSet set to 'aperiodic' and with a total of four SRS resources transmitted in different symbols of two different slots, and where the SRS port of each SRS resource in the given two sets is associated with a different UE antenna port. The two sets are each configured with two SRS resources, or one set is configured with one SRS resource and the other set is configured with three SRS resources. The UE shall expect that the value of the higher layer parameter aperiodicSRS-ResourceTrigger or the value of an entry in AperiodicSRS-ResourceTriggerList in each SRS-ResourceSet is the same, and the value of the higher layer parameter slotOffset in each SRS-ResourceSet is different when none of the SRS resource sets is configured with parameter availableSlotOffsetList across all configured BWPs in a component carrier. Or,

-	For 1T=1R, or 2T=2R, or 4T=4R, up to two SRS resource sets each with one SRS resource can be configured, where the number of SRS ports for each resource is equal to 1, 2, or 4 if the UE is not indicating srs-AntennaSwitching2SP-1Periodic. Up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and up to one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' can be configured and the two SRS resource sets configured with 'semi-persistent' are not activated at the same time, or up to two SRS resource sets can be configured, if the UE is indicating srs-AntennaSwitching2SP-1Periodic, where each SRS resource set has one SRS resource, the number of SRS ports for each resource is equal to 1, 2, or 4, or

-	For 1T6R, zero or one SRS resource sets configured with resourceType in SRS-ResourceSet set to 'periodic', where in the case of one resource set has six SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of the resource in the set is associated with a different UE antenna port, and

-	For 1T6R, zero or one SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' if the UE is not indicating srs-AntennaSwitching2SP-1Periodic, or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' if the UE is indicating srs-AntennaSwitching2SP-1Periodic, where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time. Each SRS resource set has six SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of the resource in the set is associated with a different UE antenna port, and

-	For 1T6R, zero or one or two or three SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic', where in the case of one resource set a total of six SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of each resource in the set is associated with a different UE antenna port. In the case of two resource sets a total of six SRS resources are transmitted in different symbols of two different slots, and the SRS port of each SRS resource in the given two sets is associated with a different UE antenna port. In the case of three resource sets, a total of six SRS resources are transmitted in different symbols of three different slots, and the SRS port of each SRS resource in the given three sets is associated with a different UE antenna port, or

-	For 1T8R, zero or one SRS resource sets configured with resourceType in SRS-ResourceSet set to 'periodic', where in the case of one resource set has eight SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of the resource in the set is associated with a different UE antenna port, and

-	For 1T8R, zero or one SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' if the UE is not indicating srs-AntennaSwitching2SP-1Periodic, or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' if the UE is indicating srs-AntennaSwitching2SP-1Periodic, where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time. Each SRS resource set has eight SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of the resource in the set is associated with a different UE antenna port, and

-	For 1T8R, zero or two or three or four SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic', where in the case of two resource sets a total of eight SRS resources transmitted in different symbols of two different slots, and where the SRS port of each SRS resource in the given two sets is associated with a different UE antenna port. In the case of three resource sets a total of eight SRS resources are transmitted in different symbols of three different slots, and the SRS port of each SRS resource in the given three sets is associated with a different UE antenna port. In the case of four resource sets a total of eight SRS resources are transmitted in different symbols of four different slots, and the SRS port of each SRS resource in the given four sets is associated with a different UE antenna port, or

-	For 2T6R, zero or one SRS resource sets configured with resourceType in SRS-ResourceSet set to 'periodic', where in the case of one resource set has three SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a two SRS ports, and the SRS port pair of the resource in the set is associated with a different UE antenna port pair, and

-	For 2T6R, zero or one SRS resource sets configured resourceType in SRS-ResourceSet set to 'semi-persistent' if the UE is not indicating srs-AntennaSwitching2SP-1Periodic, or up to two SRS resource sets configured with 'semi-persistent' and up to one SRS resource set configured with 'periodic' if the UE is indicating srs-AntennaSwitching2SP-1Periodic, where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time.  Each SRS resource set has three SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a two SRS ports, and the SRS port pair of the resource in the set is associated with a different UE antenna port pair, and

-	For 2T6R, zero or one or two or three SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic, where in the case of one resource set has three SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a two SRS ports, and the SRS port pair of the resource in the set is associated with a different UE antenna port pair. In the case of two resource sets a total of three SRS resources are transmitted in different symbols of two different slots, and the SRS port pair of each SRS resource in the given two sets is associated with a different UE antenna port pair. One set is configured with two SRS resources and another set with one resource. In the case of three resource sets a total of three SRS resources are transmitted in different symbols of three different slots, and the SRS port pair of each SRS resource in the given three sets is associated with a different UE antenna port pair, or

-	For 2T8R, zero or one SRS resource sets configured with resourceType in SRS-ResourceSet set to 'periodic', where in the case of one resource set has four SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a two SRS ports, and the SRS port pair of the resource in the set is associated with a different UE antenna port pair, and

-	For 2T8R, zero or one SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' if the UE is not indicating srs-AntennaSwitching2SP-1Periodic, or up to two SRS resource sets configured with 'semi-persistent' and up to one SRS resource set configured with 'periodic' if the UE is indicating srs-AntennaSwitching2SP-1Periodic, where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time. Each SRS resource set has four SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a two SRS ports, and the SRS port pair of the resource in the set is associated with a different UE antenna port pair, and

-	For 2T8R, zero or one or two or three or four SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic', where in the case of one resource set has four SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a two SRS ports, and the SRS port pair of the  resource in the set is associated with a different UE antenna port pair. In the case of two resource sets a total of four SRS resources transmitted in different symbols of two different slots, and where the SRS port pair of each SRS resource in the given two sets is associated with a different UE antenna port pair. In the case of three resource sets a total of four SRS resources transmitted in different symbols of three different slots, and where the SRS port pair of each SRS resource in the given three sets is associated with a different UE antenna port pair. Two sets are configured with one SRS resource in each set and one resource set is configured with two resources. In the case of four resource sets a total of four SRS resources transmitted in different symbols of four different slots, and where the SRS port pair of each SRS resource in the given four sets is associated with a different UE antenna port pair. Four sets are configured with one SRS resource in each set, or

-	For 4T8R, zero or one or two SRS resource sets configured with a different value of resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' if the UE is not indicating srs-AntennaSwitching2SP-1Periodic, or up to two SRS resource sets configured with 'semi-persistent' and up to one SRS resource set configured with 'periodic' if the UE is indicating srs-AntennaSwitching2SP-1Periodic, where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time. Each SRS resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a four SRS ports, and the SRS ports of the resource in the set are associated with a different UE antenna ports, or

-	For 4T8R, zero or one or two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic', where in the case of one resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a four SRS ports, and the SRS ports of the resource in the set are associated with a different UE antenna ports. In the case of two resource sets a total of two SRS resources are transmitted in different symbols of two different slots, and where the SRS ports of each SRS resource in the given two sets are associated with a different UE antenna ports.

The UE is configured with a guard period of Y symbols, in which the UE does not transmit any other signal, in the case the SRS resources of a set are transmitted in the same slot. The guard period is in-between the SRS resources of the set. For two SRS resource sets of an antenna switching located in two consecutive slots, if UE is capable of transmitting SRS in all symbols in one slot, a guard period of Y symbols exists between the last OFDM symbol occupied by the SRS resource set in the first slot and the first OFDM symbol occupied by the SRS resource set in the second slot.

For the inter-set guard period, the UE does not transmit any other signal on any symbols of the interval if the interval between SRS resource sets is Y symbols.

-	When both the SRS resource on all of the corresponding symbols prior to the gap and the SRS resource on all of the corresponding symbols after the gap are dropped due to collision handling, the gap period is also dropped with same priority and can be used for UL transmission.

The UE shall expect to be configured with the same number of SRS ports for all SRS resources in the SRS resource set(s) with higher layer parameter usage set as 'antennaSwitching'.

In the case that more than one SRS resource set configured with resourceType in SRS-ResourceSet set to ‘aperiodic’, the UE shall expect that the more than one set are configured with the same values of the higher layer parameters alpha, p0, pathlossReferenceRS, and srs-PowerControlAdjustmentStates in SRS-ResourceSet.

For 1T2R, 1T4R, 2T4R, 1T6R, 1T8R, 2T6R, 2T8R, or 4T8R, the UE shall not expect to be configured or triggered with more than one SRS resource set with higher layer parameter usage set as 'antennaSwitching' in the same slot. For 1T=1R, 2T=2R or 4T=4R, the UE shall not expect to be configured or triggered with more than one SRS resource set with higher layer parameter usage set as 'antennaSwitching' in the same symbol.

The value of Y is defined by Table 6.2.1.2-1.

Table 6.2.1.2-1: The minimum guard period between two SRS resources of an SRS resource set for antenna switching

|  |  | Y [symbol] |
| --- | --- | --- |
| 0 | 15 | 1 |
| 1 | 30 | 1 |
| 2 | 60 | 1 |
| 3 | 120 | 2 |
| 5 | 480 | 7 |
| 6 | 960 | 14 |

<omitted text>

### 6.2.2	UE DM-RS transmission procedure

<omitted text>

When transmitted PUSCH is scheduled by DCI format 0_1 with CRC scrambled by C-RNTI, CS-RNTI, SP-CSI-RNTI or MCS-C-RNTI, or corresponding to a configured grant, or being a PUSCH for Type-2 random access procedure,

-	for a configured-grant based PUSCH transmission in RRC_INACTIVE state, the UE is provided with a set of DM-RS port(s) by sdt-DMRS-Ports. The DM-RS port for the PUSCH is determined by the mapping between SS/PBCH block(s) and a PUSCH occasion and the associated DM-RS resource as described in Clause 19.1 of [6, TS 38.213].

-	the UE may be configured with higher layer parameter dmrs-Type in DMRS-UplinkConfig, and the configured DM-RS configuration type is used for transmitting PUSCH in as defined in Clause 6.4.1.1 of [4, TS 38.211].

-	the UE may be configured with the maximum number of front-loaded DM-RS symbols for PUSCH by higher layer parameter maxLength in DMRS-UplinkConfig, or by higher layer parameter msgA-MaxLength in msgA-DMRS-Config,

-	if maxLength is not configured, single-symbol DM-RS can be scheduled for the UE by DCI or configured by the configured grant configuration, and the UE can be configured with a number of additional DM-RS for PUSCH by higher layer parameter dmrs-AdditionalPosition, which can be 'pos0', 'pos1', 'pos2', 'pos3'.

-	if maxLength is configured, either single-symbol DM-RS or double symbol DM-RS can be scheduled for the UE by DCI or configured by the configured grant configuration, and the UE can be configured with a number of additional DM-RS for PUSCH by higher layer parameter dmrs-AdditionalPosition, which can be 'pos0' or 'pos1'.

-	for MsgA PUSCH for Type-2 random access procedure the UE can be configured with a number of additional DM-RS for PUSCH by higher layer parameter msgA-DMRS-AdditionalPosition, which can be 'pos0', 'pos1', 'pos2', 'pos3' for single-symbol DM-RS or 'pos0', 'pos1' for double-symbol DM-RS.

-	and, the UE shall transmit a number of additional DM-RS as specified in Table 6.4.1.1.3-3 and Table 6.4.1.1.3-4 in -Clause 6.4.1.1.3 of [4, TS 38.211].

<omitted text>