3GPP TSG-RAN WG1 Meeting #115	R1-2312776

# Chicago, USA, November 13-17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0173 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Rel-18 enhancements of NR Multicast and Broadcast Services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_enh-Core |  |  |  |  |  | Date: |  |  | 2023-11-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Capture the TPs achieved in RAN1#115 for Rel-18 enhancements of NR Multicast and Broadcast Services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support of Rel-18 enhancements of NR Multicast and Broadcast Services: - Update descriptions of DCI 4_0, 4_1 and 4_2 for multicast reception in RRC_INACTIVE state; - Update RRC parameters for Rel-18 enhancements of NR Multicast and Broadcast Services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification for Rel-18 enhancements of NR Multicast and Broadcast Services is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.1, 7.3.1, 7.3.1.5.1, 7.3.1.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS38.211, TS 38.213, TS 38.214 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.4.2.1	Bit selection

The bit sequence after encoding  from Clause 5.3.2 is written into a circular buffer of length  for the -th coded block, where  is defined in Clause 5.3.2.

For the -th code block, let  if  and  otherwise, where, ,  is determined according to Clause 6.1.4.2 in [6, TS 38.214] for UL-SCH and Clause 5.1.3.2 in [6, TS 38.214] for DL-SCH/PCH, assuming the following:

For one TB for DL-SCH with PDSCH scheduled by DCI format 4_0/4_1/4_2,

-	if the PDSCH is scheduled by DCI format 4_1/4_2,

-	maximum number of layers is given by X, where

-	if the higher layer parameter maxMIMO-Layers of pdsch-ConfigMulticast is configured, X is given by that parameter;

-	otherwise, X equals to 1;

-	if the higher layer parameter mcs-Table given by a pdsch-ConfigMulticast or by pdsch-ConfigMTCH for at least one common frequency resource (CFR) is set to 'qam256', maximum modulation order Qm=8 is assumed for DL-SCH; otherwise a maximum modulation order Qm=6 is assumed for DL-SCH;

-	if the PDSCH is scheduled by DCI format 4_0,

-	maximum number of layers is 1;

-	if the higher layer parameter mcs-Table given by a pdsch-ConfigMCCH is set to 'qam256', maximum modulation order Qm=8 is assumed for DL-SCH; otherwise a maximum modulation order Qm=6 is assumed for DL-SCH;

-	if the higher layer parameter mcs-Table given by a pdsch-ConfigMTCH is set to 'qam256', maximum modulation order Qm=8 is assumed for DL-SCH; otherwise a maximum modulation order Qm=6 is assumed for DL-SCH;

-	nPRB=nPRB,LBRM is given by Table 5.4.2.1-1, where the value of nPRB,LBRM for DL-SCH is determined according to the size of the CFR if only one CFR is configured to the UE;

< Unchanged parts are omitted >

### 7.3.1	DCI formats

The DCI formats defined in table 7.3.1-1 are supported.

Table 7.3.1-1: DCI formats

| DCI format | Usage |
| --- | --- |
| 0_0 | Scheduling of PUSCH in one cell |
| 0_1 | Scheduling of one or multiple PUSCH in one cell, or indicating downlink feedback information for configured grant PUSCH (CG-DFI) |
| 0_2 | Scheduling of PUSCH in one cell |
| 0_3 | Scheduling of one PUSCH in one cell, or multiple PUSCHs in multiple cells with one PUSCH per cell |
| 1_0 | Scheduling of PDSCH in one cell |
| 1_1 | Scheduling of one or multiple PDSCH in one cell, and/or triggering one shot HARQ-ACK codebook feedback |
| 1_2 | Scheduling of PDSCH in one cell |
| 1_3 | Scheduling of one PDSCH in one cell, or multiple PDSCHs in multiple cells with one PDSCH per cell |
| 2_0 | Notifying a group of UEs of the slot format, available RB sets, COT duration and search space set group switching |
| 2_1 | Notifying a group of UEs of the PRB(s) and OFDM symbol(s) where UE may assume no transmission is intended for the UE |
| 2_2 | Transmission of TPC commands for PUCCH and PUSCH |
| 2_3 | Transmission of a group of TPC commands for SRS transmissions by one or more UEs |
| 2_4 | Notifying a group of UEs of the PRB(s) and OFDM symbol(s) where UE cancels the corresponding UL transmission from the UE |
| 2_5 | Notifying the availability of soft resources as defined in Clause 9.3.1 of [10, TS 38.473] |
| 2_6 | Notifying the power saving information outside DRX Active Time for one or more UEs |
| 2_7 | Notifying paging early indication and TRS availability indication for one or more UEs. |
| 2_8 | Notifying the aperiodic beam indication and associated time resources |
| 2_9 | Activating or de-activating the cell DTX/DRX configuration of one or multiple serving cells for one or more UEs. |
| 3_0 | Scheduling of NR sidelink in one cell |
| 3_1 | Scheduling of LTE sidelink in one cell |
| 3_2 | Scheduling of NR SL PRS in one cell |
| 4_0 | Scheduling of PDSCH with CRC scrambled by MCCH-RNTI/G-RNTI for broadcast or by multicast-MCCH-RNTI for multicast in RRC_INACITIVE state |
| 4_1 | Scheduling of PDSCH with CRC scrambled by G-RNTI/G-CS-RNTI for multicast in RRC_CONNECTED state or by G-RNTI for multicast in RRC_INACITIVE state |
| 4_2 | Scheduling of PDSCH with CRC scrambled by G-RNTI/G-CS-RNTI for multicast in RRC_CONNECTED state |

< Unchanged parts are omitted >

##### 7.3.1.5.1	Format 4_0

DCI format 4_0 is used for the scheduling of PDSCH for broadcast or for multicast in RRC_INACTIVE state in DL cell.

The following information is transmitted by means of the DCI format 4_0 with CRC scrambled by MCCH-RNTI or G-RNTI for broadcast configured by MBS-SessionInfo, or by multicast-MCCH-RNTI for multicast configured by MBS-SessionInfoMulticast:

-	Frequency domain resource assignment – log2(NRBDL,CFR(NRBDL,CFR+1)2) bits where NRBDL,CFR equals to

-	the size of CORESET 0 if CORESET 0 is configured for the cell; and

-	the size of initial DL bandwidth part if CORESET 0 is not configured for the cell.

-	Time domain resource assignment – 4 bits as defined in Clause 5.1.2.1 of [6, TS38.214]

-	VRB-to-PRB mapping – 1 bit according to Table 7.3.1.2.2-5

-	Modulation and coding scheme – 5 bits as defined in Clause 5.1.3 of [6, TS38.214]

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

-	MCCH change notification – 2 bits as defined in Clause 5.9.1.3 and Clause 5.x.1.3 of [9, TS38.331] if the CRC of the DCI format 4_0 is scrambled by MCCH-RNTI and multicast-MCCH-RNTI respectively. Otherwise, this bit field is reserved.

-	Reserved bits – 14bits

##### 7.3.1.5.2	Format 4_1

DCI format 4_1 is used for the scheduling of PDSCH for multicast in DL cell.

The following information is transmitted by means of the DCI format 4_1 with CRC scrambled by G-RNTI for multicast or G-CS-RNTI configured by MBS-RNTI-SpecificConfig, or by G-RNTI for multicast configured by MBS-SessionInfoListMulticast:

-	Frequency domain resource assignment – log2(NRBDL,CFR(NRBDL,CFR+1)2) bits where NRBDL,CFR equals to

-	the size of CORESET 0 if CORESET 0 is configured for the cell; and

-	the size of initial DL bandwidth part if CORESET 0 is not configured for the cell.

-	Time domain resource assignment – 4 bits as defined in Clause 5.1.2.1 of [6, TS38.214]

-	VRB-to-PRB mapping – 1 bit according to Table 7.3.1.2.2-5

-	Modulation and coding scheme – 5 bits as defined in Clause 5.1.3 of [6, TS38.214]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

-	HARQ process number – 4 bits

-	Downlink assignment index – 2 bits as defined in Clause 9.1.3 of [5, TS 38.213], as counter DAI

-	PUCCH resource indicator – 3 bits as defined in Clause 9.2.3 of [5, TS38.213]

-	PDSCH-to-HARQ_feedback timing indicator – 3 bits as defined in Clause 9.2.3 of [5, TS38.213]

-	Reserved bits – 3 bits

< Unchanged parts are omitted >