## Characteristics template for 3GPP 5G NTN RIT (Release 17 and beyond)

The description templates provided by 3GPP are for the description of the characteristics of 5G satellite access developed by 3GPP. It includes one characteristics template for SRIT (encompassing component RITs for NR NTN (NR satellite access) and IoT NTN (consisting of NB-IoT satellite access and eMTC satellite access), and one characteristics template for NR NTN (NR satellite access) RIT.

This document provides the characteristics template for the description of the characteristics of the RIT based on 3GPP Rel-17 work.

For this characteristics template, 3GPP has addressed all of the characteristics, and it is expected that these descriptions are helpful to assist in evaluation activities for independent evaluation groups, as well as to facilitate the understanding of the state-of-art of 3GPP development on the RIT.

| Item | Item to be described |
| --- | --- |
| 8.2.5.1.1 | Test environment(s) |
| 8.2.5.1.1.1 | What test environments does this technology description template address? This proposal targets to address all the three test environments across the three usage scenarios (eMBB-s, mMTC-s, and HRC-s) as described in Report ITU-R M.2514-0. |
| 8.2.5.1.2 | Radio interface functional aspects |
| 8.2.5.1.2.1 | Multiple access schemes Which access scheme(s) does the proposal use: TDMA, FDMA, CDMA, OFDMA, IDMA, SDMA, hybrid, or another? Describe in detail the multiple access schemes employed with their main parameters. - Downlink and Uplink: The multiple access is a combination of - OFDMA: Synchronous/scheduling-based; the transmission to/from different UEs uses mutually orthogonal frequency assignments. Granularity in frequency assignment: One resource block consisting of 12 subcarriers. Multiple sub-carrier spacings are supported including 15kHz, 30kHz and 60kHz for data (see Item 8.2.5.1.7 and reference therein). - CP-OFDM is applied for both downlink and uplink. DFT-spread OFDM can also be configured for uplink. - Spectral confinement technique(s) (e.g. filtering, windowing, etc.) for a waveform at the transmitter is transparent to the receiver. When such confinement techniques are used, the spectral utilization ratio can be enhanced. - TDMA: Transmission to/from different UEs with separation in time. Granularity: One slot consisting of 14 OFDM symbols, or 2~13 OFDM symbols non-slot (for DL) or 1~13 OFDM symbols (for UL) within one slot. The physical length of one slot ranges from 0.25 to 1ms depending on the sub-carrier spacing (for more details on the frame structure, see Item 8.2.5.1.7 and the references therein). - CDMA: Inter-cell interference suppressed by processing gain of channel coding allowing for a frequency reuse of one (for more details on channel-coding, see Item 8.2.5.1.2.3 and the reference therein). - SDMA: Possibility to transmit to/from multiple users using the same time/frequency resource (SDMA a.k.a. “multi-user MIMO”) as part of the advanced-antenna capabilities (for more details on the advanced-antenna capabilities, see Item 8.2.5.1.9 and the reference therein) At least an UL transmission scheme without scheduling grant is supported. (Note: Synchronous means that timing offset between UEs is within cyclic prefix by e.g. timing alignment.) |
| 8.2.5.1.2.2 | Modulation scheme |
| 8.2.5.1.2.2.1 | What is the baseband modulation scheme? If both data modulation and spreading modulation are required, describe in detail. Describe the modulation scheme employed for data and control information. What is the symbol rate after modulation? - Downlink: - For both data and higher-layer control information: QPSK, 16QAM, and 64QAM (see [38.211] sub-clause 7.3.1.2). - L1/L2 control: QPSK (see [38.211] sub-clause 7.3.2.4). - Symbol rate: 1008 ksymbols/s per 360kHz resource block (equivalently 168ksymbols/s per 180kHz resource block) - Uplink: - For both data and higher-layer control information: π/2-BPSK (when precoding is enabled), QPSK, 16QAM, 64QAM and 256QAM (see [38.211] sub-clause 6.3.1.2). - L1/L2 control: BPSK, π/2-BPSK, QPSK (see [38.211] sub-clause 6.3.2). - Symbol rate: 672ksymbols/s per 1440kHz resource block (equivalently 168ksymbols/s per 180kHz resource block) The above is at least applied to eMBB-s. |
| 8.2.5.1.2.2.2 | PAPR What is the RF peak to average power ratio after baseband filtering (dB)? Describe the PAPR (peak-to-average power ratio) reduction algorithms if they are used in the proposed RIT. The PAPR depends on the waveform and the number of component carriers. The single component carrier transmission is assumed herein when providing the PAPR. For DFT-spread OFDM, PAPR would depend on modulation scheme as well. For uplink using DFT-spread OFDM, the cubic metric (CM) can also be used as one of the methods of predicting the power de-rating from signal modulation characteristics, if needed. - Downlink: The PAPR is 8.4dB (99.9%) - Uplink: - For CP-OFDM: The PAPR is 8.4dB (99.9%) - For DFT-spread OFDM: The PAPR is provided in the table below. Modulation; π/2 BPSK; QPSK; 16QAM; 64QAM; 256QAM. PAPR (99.9%); 4.5 dB; 5.8 dB; 6.5 dB; 6.6 dB; 6.7 dB. CM (99.9%); 0.3 dB; 1.2 dB; 2.1 dB; 2.3 dB; 2.4 dB. Note: The above values are derived without spectrum shaping. When spectrum shaping is considered for π/2 BPSK, lower PAPR and CM values can be derived, e.g., 1.75dB PAPR for π/2 BPSK, based on the trade-off between PAPR and demodulation performance. Any PAPR-reduction algorithm is transmitter-implementation specific for uplink and downlink. High PAPR means that power amplifiers (HPA) have to operate with increased back-off leading to reduced efficiency. This is not appropriate for the power-limited satellite downlink which typically requires operation in the nonlinear region of high power amplifiers. To alleviate PAPR problems in Satellite-based radio interface, PAPR-reduction algorithms that are transmitter implementation specific can be considered. Therefore, PAPR optimizations for downlink channels are not necessary to be specified for satellite radio interface access. |
| 8.2.5.1.2.3 | Error control coding scheme and interleaving |
| 8.2.5.1.2.3.1 | Provide details of error control coding scheme for both downlink and uplink? For example: – FEC or other schemes? – Unequal error protection? Explain the decoding mechanism employed. - Downlink and Uplink: - For data: BG#1 and BG#2 based Low density parity check (LDPC) coding, combined with rate matching based on shortening/puncturing/repetition to achieve a desired overall code rate (For more details, see [38.212] sub-clauses 5.3.2). LDPC channel coder facilitates low-latency and high-throughput decoder implementations. - For L1/L2 control: For DCI (Downlink Control Information)/UCI (Uplink Control Information) size larger than 11 bits, Polar coding, combined with rate matching based on shortening/puncturing/repetition to achieve a desired overall code rate (For more details, see [38.212] sub-clauses 5.3.1). Otherwise, repetition for 1-bit; simplex coding for 2-bit; Reed-Muller coding for 3~11-bit DCI/UCI size. The above scheme is at least applied to eMBB-s. Decoding mechanism is receiver-implementation specific. Example of information on the decoding mechanism will be provided together with self evaluation. |
| 8.2.5.1.2.3.2 | Describe the bit interleaving scheme for both uplink and downlink. - Downlink: - For data: bit interleaver is performed for LDPC coding after rate-matching (For more details, see [38.212] sub-clauses 5.4.2.2) - For L1/L2 control: Bit interleaving is performed as part of the encoding process for Polar coding (For more details, see [38.212] sub-clauses 5.4.1.1) - Uplink: - For data: bit interleaver is performed for LDPC coding after rate-matching (For more details, see [38.212] sub-clauses 5.4.2.2) - For L1/L2 control: Bit interleaving is performed for Polar coding after rate-matching (For more details, see [38.212] sub-clauses 5.4.1.3) The above scheme is at least applied to eMBB-s. |
| 8.2.5.1.3 | Describe channel tracking capabilities (e.g. channel tracking algorithm, pilot symbol configuration, etc.) to accommodate rapidly changing delay spread profile To support channel tracking, different types of reference signals can be transmitted on downlink and uplink respectively. - Downlink: - Primary and Secondary Synchronization signals (PSS and SSS) are transmitted periodically to the cell. The periodicity of these signals is network configurable. UEs can detect and maintain the cell timing based on these signals. If the gNB implements hybrid beamforming, then the PSS and SSS are transmitted separately to each analogue beam. Network can configure multiple PSS and SSS in frequency domain. - UE-specific Demodulation RS (DM-RS) for PDCCH can be used for downlink channel estimation for coherent demodulation of PDCCH (Physical Downlink Control Channel). DM-RS for PDCCH is transmitted together with the PDCCH. - UE-specific Demodulation RS (DM-RS) for PDSCH can be used for downlink channel estimation for coherent demodulation of PDSCH (Physical Downlink Shared Channel). DM-RS for PDSCH is transmitted together with the PDSCH. - UE-specific Phase Tracking RS (PT-RS) can be used in addition to the DM-RS for PDSCH for correcting common phase error between PDSCH symbols not containing DM-RS. It may also be used for Doppler and time varying channel tracking. PT-RS for PDSCH is transmitted together with the PDSCH upon need. - UE-specific Channel State Information RS (CSI-RS) can be used for estimation of channel-state information (CSI) to further prepare feedback reporting to gNB to assist in MCS selection, beamforming, MIMO rank selection and resource allocation. CSI-RS transmissions are transmitted periodically, aperiodically, and semi-persistently on a configurable rate by the gNB. CSI-RS also can be used for interference measurement and fine frequency/time tracking purposes. - Uplink: - UE-specific Demodulation RS (DM-RS) for PUCCH can be used for uplink channel estimation for coherent demodulation of PUCCH (Physical Uplink Control Channel). DM-RS for PUCCH is transmitted together with the PUCCH. - UE-specific Demodulation RS (DM-RS) for PUSCH can be used for uplink channel estimation for coherent demodulation of PUSCH (Physical Uplink Shared Channel). DM-RS for PUSCH is transmitted together with the PUSCH. - UE-specific Phase Tracking RS (PT-RS) can be used in addition to the DM-RS for PUSCH for correcting common phase error between PUSCH symbols not containing DM-RS. It may also be used for Doppler and time varying channel tracking. DM-RS for PUSCH is transmitted together with the PUSCH upon need. - UE-specific Sounding RS (SRS) can be used for estimation of uplink channel-state information to assist uplink scheduling, uplink power control, as well as assist the downlink transmission (e.g. the downlink beamforming in the scenario with UL/DL reciprocity). SRS transmissions are transmitted periodically, aperiodically, and semi-persistently by the UE on a gNB configurable rate. Terminology: To ease understanding of specific terms/abbreviations used in this item here after, few main acronyms and definitions are introduced: - Common TA: a network-controlled common TA, and may include any timing offset considered necessary by the network (e.g. feeder link delay). - Reference point for uplink time synchronization: The point at which DL and UL are frame aligned with an offset given by NTA,offset To accommodate the large round trip delay, timing drift and Doppler effect in Satellite radio access, the user terminal performs the uplink time pre-compensation and frequency pre-compensation. For the serving cell, the network broadcast valid ephemeris information and Common TA parameters. The UE shall have valid GNSS position as well as ephemeris and Common TA before connecting to an NTN cell. To achieve synchronisation, before and during connection to a cell, the UE shall compute the RTT between UE and the reference point (RP) based on the GNSS position, the ephemeris, and the Common TA parameters (see clause 4.2 in TS 38.213 [29]), and autonomously pre-compensate the TTA for the RTT between UE and the RP (see clause 4.3 of [TS 38.211]). The UE shall compute the frequency Doppler shift of the service link, and autonomously pre-compensate for it in the uplink transmissions, by considering UE position and the ephemeris. If the UE does not have a valid GNSS position and/or valid ephemeris and Common TA, it shall not transmit until both are regained. In connected mode, the UE shall be able to continuously update the Timing Advance and frequency pre-compensation. Details of channel-tracking/estimation algorithms are receiver-implementation specific, and not part of the specification. Details of channel-tracking/estimation algorithms are receiver-implementation specific, e.g. MMSE-based channel estimation with appropriate interpolation in time and frequency domain could be used. |
| 8.2.5.1.4 | Physical channel structure and multiplexing |
| 8.2.5.1.4.1 | What is the physical channel bit rate (Mbit/s) for supported bandwidths? i.e. the product of the modulation symbol rate (in symbols per second), bits per modulation symbol, and the number of streams supported by the antenna system. The physical channel bit rate depends on the modulation scheme, number of spatial-multiplexing layer, number of resource blocks in the channel bandwidth and the subcarrier spacing used. The physical channel bit rate per layer can be expressed as Rlayer = Nmod x NRB x 2µ x 168 kbps where - Nmod is the number of bits per modulation symbol for the applied modulation scheme (QPSK: 2, 16QAM: 4, 64QAM: 6, 256QAM: 8) - NRB is the number of resource blocks in the aggregated frequency domain which depends on the channel bandwidth. - µ depends on the subcarrier spacing, ∆f, given by ∆f=2μ∙15 kHz, μ=0,1 For example, a 30 MHz carrier with 78 resource blocks using 30 kHz subcarrier spacing, μ=1, and 64QAM modulation results in a physical channel bit rate of 157 Mbit/s per layer. |
| 8.2.5.1.4.2 | Layer 1 and Layer 2 overhead estimation Describe how the RIT accounts for all Layer 1 (PHY) and Layer 2 (MAC) overhead and provide an accurate estimate that includes static and dynamic overheads. - Downlink The downlink L1/L2 overhead includes: - Different types of reference signals - Demodulation reference signals for PDSCH (DMRS-PDSCH) - Phase-tracking reference signals for PDSCH (PTRS-PDSCH) - Demodulation reference signals for PDCCH - Reference signals specifically targeting estimation of channel-state information (CSI-RS) - Tracking reference signals (TRS) - L1/L2 control signalling transmitted on the up to three first OFDM symbols of each slot - Synchronization signals and physical broadcast control channel including demodulation reference signals included in the SS/PBCH block - PDU headers in L2 sub-layers (MAC/RLC/PDCP) The overhead due to different type of reference signals is given in the table below. Note that demodulation reference signals for PDCCH is included in the PDCCH overhead. Reference signal type; Example configurations; Overhead for example configurations. DMRS-PDSCH; As examples, DMRS can occupy 1/3, ½, or one full OFDM symbol. 1, 2, 3 or 4 symbols per slot can be configured to carry DMRS.; 2.4 % to 29 %. PTRS- PDSCH; 1 resource elements in frequency domain every second or fourth resource block. PTRS is mainly intended for FR2.; 0.2% or 0.5 % when configured.. CSI-RS; 1 resource element per resource block per antenna port per CSI-RS periodicity; 0.25 % for 8 antenna ports transmitted every 20 ms with 15 kHz subcarrier spacing. TRS; 2 slots with 1/2 symbol in each slot per transmission period; 0.36 % or 0.18% respectively for 20 ms and 40ms periodicity. The overhead due to the L1/L2 control signalling is depending on the size and periodicity of the configured CORESET in the cell and includes the overhead from the PDCCH demodulation reference signals. If the CORESET is transmitted in every slot, maximum control channel overhead is 21% assuming three symbols and whole carrier bandwidth used for CORESET, while a more typical overhead is 7% when 1/3 of the time and frequency resources in the first three symbols of a slot is allocated to PDCCH. The overhead due to the SS/PBCH block is given by the number of SS/PBCH blocks transmitted within the SS/PBCH block period, the SS/PBCH block periodicity and the subcarrier spacing. Assuming a 100 resource block wide carrier, the overhead for 20 ms periodicity is in the range of 0.6 % to 2.3 % if the maximum number of SS/PBCH blocks are transmitted. - Uplink L1/L2 overhead includes: - Different types of reference signals - Demodulation reference signal for PUSCH - Demodulation reference signal for PUCCH - Phase-tracking reference signals - Sounding reference signal (SRS) used for uplink channel-state estimation at the network side - L1/L2 control signalling transmitted on a configurable amount of resources (see also Item 8.2.5.1.4.5) - L2 control overhead due to e.g., random access, uplink time-alignment control, power headroom reports and buffer-status reports - PDU headers in L2 layers (MAC/RLC/PDCP) The overhead due to due to demodulation reference signal for PUSCH is the same as the overhead for demodulation reference signal for PDSCH, i.e. 4 % to 29 % depending on number of symbols configured. Also, the phase-tracking reference signal overhead is the same in UL as in DL. The overhead due to periodic SRS is depending on the number of symbols configured subcarrier spacing and periodicity. For 20 ms periodicity, the overhead is in the range of 0.4% to 1.4% assuming15 kHz subcarrier spacing. Amount of uplink resources reserved for random access depends on the configuration. The relative overhead due to uplink time-alignment control depends on the configuration and the number of active UEs within a cell. The amount of overhead for buffer status reports depends on the configuration. The amount of overhead caused by 4 highly depends on the data packet size. |
| 8.2.5.1.4.3 | Variable bit rate capabilities Describe how the proposal supports different applications and services with various bit rate requirements. For a given combination of modulation scheme, code rate, and number of spatial-multiplexing layers, the data rate available to a user can be controlled by the scheduler by assigning different number of resource blocks for the transmission. In case of multiple services, the available/assigned resource, and thus the available data rate, is shared between the services. |
| 8.2.5.1.4.4 | Variable payload capabilities Describe how the RIT supports IP-based application layer protocols/services (e.g. VoIP, video-streaming, interactive gaming, etc.) with variable-size payloads. See also 8.2.5.1.4.3. The transport-block size can vary between X bits and Y bits. The number of bits per transport block can be set with a fine granularity. See [38.214] sub-clause 5.1.3.2 for details |
| 8.2.5.1.4.5 | Signalling transmission scheme Describe how transmission schemes are different for signalling/control from that of user data. - Downlink L1/L2 control signalling is transmitted in assigned resources time and frequency multiplexed with data within the bandwidth part (BWP, see item 8.2.5.1.8.1). Control signalling is limited to QPSK modulation (QPSK, 16QAM, 64QAM and 256QAM for data). Control signalling error correcting codes are polar codes (LDPC codes for data). - Uplink L1/L2 control signalling transmitted in assigned resources and can be time and frequency multiplexed with data within the BWP. L1/L2 control signalling can also be multiplexed with data on the PUSCH. Modulation schemes for L1/L2 control signalling is π/2-BPSK, BPSK and QPSK Control signalling error correcting codes are block codes for small payload and polar codes for larger payloads (LDPC codes for data). For both downlink and uplink, higher-layer signalling (e.g. MAC, RLC, PDCP headers and RRC signalling) is carried within transport blocks and thus transmitted using the same physical-layer transmitter processing as user data. |
| 8.2.5.1.5 | Mobility management (Handover) |
| 8.2.5.1.5.1 | Describe the handover mechanisms and procedures which are associated with – Inter-System handover; – Intra-System handover; – Intra-frequency and Inter-frequency; – Within the RIT or between RITs within one SRIT (if applicable). Characterize the type of handover strategy or strategies. Inter-System handover: NTN – TN handover refers to mobility in both directions, i.e. from NTN to TN (hand-in) and from TN to NTN (hand-out). During mobility between NTN and Terrestrial Network (TN), a UE is not required to connect to both NTN and TN at the same time. Intra-System handover: Network controlled mobility applies to UEs in RRC_CONNECTED and is categorized into two types of mobility: - Cell level mobility requires explicit RRC signalling to be triggered, i.e. handover. For inter-gNB handover, handover request, handover acknowledgement, handover command, handover complete procedure are supported between source gNB and target gNB. The release of the resources at the source gNB during the handover completion phase is triggered by the target gNB. - Beam level mobility does not require explicit RRC signalling to be triggered - it is dealt with at lower layers - and RRC is not required to know which beam is being used at a given point in time. Data forwarding, in-sequence delivery and duplication avoidance at handover can be guaranteed between target gNB and source gNB. UE may support mobility between gNBs operating with satellite payloads in different orbits (e.g., GSO, NGSO at different altitudes). Service link and feeder link Switchover: A feeder link switchover is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for a specific NTN payload. The feeder link switchover is a Transport Network Layer procedure. Service link switch refers to a change of the serving NTN payload. A feeder link switch over may result in transferring the established connection for the affected UEs between two gNBs. For soft feeder link switch over, an NTN payload is able to connect to more than one NTN Gateway during a given period, i.e. a temporary overlap can be ensured during the transition between the feeder links. For hard feeder link switch over, an NTN payload connects to only one NTN Gateway at any given time, i.e. a radio link interruption may occur during the transition between the feeder links. For more details, refer to [38.300] sub-clauses 16.14.4. |
| 8.2.5.1.5.2 | What are the handover interruption times for: – within the RIT (intra- and inter-frequency); The mobility interruption time includes the time required to execute any radio access network procedure, radio resource control signalling protocol, or other message exchanges between the user terminal and base station, as applicable to the candidate RIT/SRIT. When moving within the same cell, the transmitting/receiving beam pair of the UE may need to be changed. For DL data transmission during UE mobility, gNB can configure different beams for this UE at different slots. It ensures appropriate transmit beam allocation to the UE for continuous DL transmission. Therefore, DL data packet transmission is kept during beam pair switching at different slots. For UL data transmission, PUSCH is sent using the beam configured by SRI (SRS resource indicator) by gNB. Accordingly, an appropriate gNB-side beam is selected for UL data reception. gNB may select different beams at different slots depending on the UE mobility. Therefore, UL data packet transmission is kept during beam pair switching at different slots. The UE can keep exchanging user plane packets with gNB during the mobility transitions without cell and satellite change. Therefore, 0ms mobility interruption time is achieved by NR NTN for this scenario. |
| 8.2.5.1.6 | Radio resource management |
| 8.2.5.1.6.1 | Describe the radio resource management, support of: – centralized and/or distributed RRM; – dynamic and flexible radio resource management; – efficient load balancing. General NR satellite access performs radio resource management to ensure the efficient use of the available radio resource. RRM functions include: - Radio bearer control (RBC): the establishment, maintenance and release of radio bearer involves the configuration of radio resource. This is located in gNB. - Radio Admission Control (RAC): RAC is to admit or reject the establishment of new radio bearer. It considers QoS requirement, the priority level, overall resource situation. This is located in gNB. - Connection Mobility Control (CMC): it controls the number of UEs in idle mode and connected mode. In idle mode, cell reselection algorithm is controlled by parameter setting and in the connected mode, gNB controls UE mobility via handover and RRC connection release with redirection. Dynamic/flexible radio resource management NR satellite access supports dynamic and flexible radio resource management by packet scheduling that allocates and de-allocates resources to user and control plane packets. Load balancing(LB) Load balancing has the task to handle uneven distribution of the traffic load over multiple cells. The purpose of LB is thus to influence the load distribution for the higher resource utilization and QoS. LB is achieved in NR satellite access with hand-over, redirection or cell reselection. |
| 8.2.5.1.6.2 | Inter-RIT interworking Describe the functional blocks and mechanisms for interworking (such as a network architecture model) between heterogeneous RITs within a SRIT, if supported. Mobility between NTN and TN is supported: NTN TN handover refers to mobility in both directions, i.e. from NTN to TN (hand-in) and from TN to NTN (hand-out). During mobility between NTN and Terrestrial Network (TN), a UE is not required to connect to both NTN and TN at the same time. |
| 8.2.5.1.6.3 | Connection/session management The mechanisms for connection/session management over the air-interface should be described. For example: – the support of multiple protocol states with fast and dynamic transitions; – the signalling schemes for allocating and releasing resources. NG-RAN support the following states: RRC_IDLE: - PLMN selection; - Broadcast of system information; - Cell re-selection mobility; - Paging for mobile terminated data is initiated by 5GC; - Paging for mobile terminated data area is managed by 5GC; - DRX for CN paging configured by NAS. - RRC_INACTIVE: - Broadcast of system information; - Cell re-selection mobility; - Paging is initiated by NG-RAN (RAN paging); - RAN-based notification area (RNA) is managed by NG- RAN; - DRX for RAN paging configured by NG-RAN; - 5GC - NG-RAN connection (both C/U-planes) is established for UE; - The UE AS context is stored in NG-RAN and the UE; - NG-RAN knows the RNA which the UE belongs to. - RRC_CONNECTED: - 5GC - NG-RAN connection (both C/U-planes) is established for UE; - The UE AS context is stored in NG-RAN and the UE; - NG-RAN knows the cell which the UE belongs to; - Transfer of unicast data to/from the UE; - Network controlled mobility including measurements. Transition between RRC states: - From RRC_IDLE to RRC_CONNECTED: RRC connection setup - From RRC_CONNECTED to RRC_IDLE: RRC connection release - From RRC_INACTIVE to RRC_CONNECTED: RRC connection resume - From RRC_CONNECTED to RRC_INACTIVE: RRC connection suspension - From RRC_INACTIVE to RRC_IDLE: RRC connection release - From RRC_IDLE to RRC_INACTIVE: not supported For more details, refer to [38.300]. |
| 8.2.5.1.7 | Frame structure |
| 8.2.5.1.7.1 | Describe the frame structure for downlink and uplink by providing sufficient information such as: – frame length; – the number of time slots per frame; – the number and position of switch points per frame for TDD; – guard time or the number of guard bits; – user payload information per time slot; – control channel structure and multiplexing; – power control bit rate. The frame structure related information is as follows: - Frame length, sub-carrier spacing, and time slots: One radio frame of length 10 ms consisting of 10 subframes, each of length 1 ms. Each subframe consists of an OFDM sub-carrier spacing dependent number of slots. Each slot consists of 14 OFDM symbols (twelve OFDM symbols in case of extended cyclic prefix) - 15 kHz SCS: 1 ms slot, 1 slot per sub-frame - 30 kHz SCS: 0.5 ms slot, 2 slots per sub-frame Data transmissions can be scheduled on a slot basis, as well as on a partial slot basis, where the partial slot transmissions that occur several times within one slot. The supported partial slot allocations and scheduling intervals are 2, 4 and 7 symbols for DL and 1-14 symbols for UL for normal cyclic prefix, and 2, 4 and 6 symbols for DL and 1-12 symbols for UL for extended cyclic prefix. - Downlink control channel structure: Downlink control signaling is time and frequency multiplexed with data on a scheduling interval basis. The control region can span over 1-3 OFDM symbols in the beginning of the allocation, flexibly allocating 1-14 symbols (at least 2 symbols for DL) for data transmission, including the time and frequency part of the control region that was not used for control signaling. - Uplink control channel structure: Uplink control signaling can be both time-multiplexed with the data of the same UE and time and frequency multiplexed with control and data of other UEs when the UE has no data to be transmitted. Uplink control signaling is piggy-backed with data i.e. transmitted with data on the PUSCH when the UE has data to be transmitted. - Power control bit rate: No specific power-control rate is defined, but a power control command can be sent at any slot, leading to a sub-carrier spacing specific maximum power control rate of 1/2/kHz for SCS of 15/30/kHz respectively. |
| 8.2.5.1.8 | Spectrum capabilities and duplex technologies NOTE 1 – Parameters for both downlink and uplink should be described separately, if necessary. |
| 8.2.5.1.8.1 | Spectrum sharing and flexible spectrum use Does the RIT/SRIT support flexible spectrum use and/or spectrum sharing for the bands for IMT? Provide details. - NR satellite access supports flexible spectrum use of spectrum defined for NR NTN in item 8.2.5.1.8.3 through mechanisms including the following: - within one component carrier, bandwidth part (BWP) is supported on downlink and uplink. The bandwidth of the component carrier can be divided into several bandwidth parts. From network perspective, different bandwidth parts can be associated with different numerologies (subcarrier spacing, cyclic prefix). UEs with smaller bandwidth support capability can work within a bandwidth part with an associated numerology. By this means UEs with different bandwidth support capability can work on large bandwidth component carrier. NR satellite access supports UE bandwidth part adaptation for UE power saving and numerology switching. The network can operate on a wide bandwidth carrier while it is not required for the UE to support the whole bandwidth carrier, but can work over activated bandwidth parts, thereby optimizing the use of radio resources to the traffic demand and minimizing interference to/from other systems. - NR satellite access addresses the uplink and downlink traffic asymmetry with flexible spectrum resource allocation by allowing FDD operation on a paired spectrum, different transmission directions in either part of a paired spectrum,. |
| 8.2.5.1.8.2 | Channel bandwidth scalability Describe how the proposal supports channel bandwidth scalability, including the supported bandwidths. Describe whether the proposed RIT supports extensions for scalable bandwidths wider than 30 MHz. Consider, for example: – the scalability of operating bandwidths; – the scalability using single and/or multiple RF carriers. Describe multiple contiguous (or non-contiguous) band aggregation capabilities, if any. Consider for example the aggregation of multiple channels to support higher user bit rates. One component carrier supports a scalable bandwidth, 5, 10, 15, 20, 30MHz for frequency range 410 MHz to 7125 MHz (see [38.108] for the actual support of bandwidth for each band), with guard band ratio from 20% to 2%;. |
| 8.2.5.1.8.3 | What are the frequency bands supported by the RIT? Please list. The following frequency bands are supported. Introduction of other ITU-R identified satellite bands are not precluded in the future. 3GPP technologies are also defined as appropriate to operate in other frequency arrangements and bands. 410 – 7125 MHz: Satellite operating band; Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high; Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high; Duplex mode. n256; 1980 MHz – 2010 MHz; 2170 MHz – 2200 MHz; FDD. n255; 1626.5 MHz – 1660.5 MHz; 1525 MHz – 1559 MHz; FDD. n254; 1610 MHz – 1626.5 MHz; 2483.5 MHz – 2500 MHz; FDD. Additional frequency bands can be introduced in the future in release independent manner. Support for frequency bands above 10 GHz is under study |
| 8.2.5.1.8.4 | What is the minimum amount of spectrum required to deploy a contiguous network, including guard bands (MHz)? The minimum amount of paired spectrum is 2 x 5 MHz. |
| 8.2.5.1.8.5 | What are the minimum and maximum transmission bandwidth (MHz) measured at the 3 dB down points? The 3 dB bandwidth is not part of the specifications, however: - The minimum 99% channel bandwidth (occupied bandwidth of single component carrier) is - 5 MHz for frequency range 410 – 7125 MHz; - The maximum 99% channel bandwidth (occupied bandwidth of single component carrier) is - 30 MHz for frequency range 410 – 7125 MHz; |
| 8.2.5.1.8.6 | What duplexing scheme(s) is (are) described in this template? (e.g. TDD, FDD or half-duplex FDD). Describe details such as: – What is the minimum (up/down) frequency separation in case of full- and half-duplex FDD? – What is the requirement of transmit/receive isolation in case of full- a half-duplex FDD? Does the RIT require a duplexer in either the mobile station (MS) or space segment? – What is the minimum (up/down) time separation in case of TDD? – Whether the DL/UL Ratio variable for TDD? What is the DL/UL ratio supported? If the DL/UL ratio for TDD is variable, what would be the coexistence criteria for adjacent cells? NR satellite access supports paired spectrum and allows FDD operation on a paired spectrum, different transmission directions in either part of a paired spectrum, - For FDD operation, it supports full-duplex FDD. - For both base station and terminal, a duplexer is needed for full-duplex FDD. - For full-duplex FDD, the required transmit/receive isolation is a UE function of; the Tx emission mask (emission level on the Rx frequency) , the TX-Rx frequency spacing , the Tx- Rx duplex filter isolation, the TX and RX configuration (RB location, RB power and RB allocation) and the required Rx desense criteria. For the supported operating bands, the minimum (up/down) Tx to Rx frequency separation is defined in the following Table: UE TX-RX frequency separation: NTN Satellite Operating Band; TX – RX carrier centre frequency separation. n256; 190 MHz. n255; -101.5 MHz. n254; 862 – 885 MHz. The spacing between carriers will depend on the deployment scenario, the size of the frequency block available and the Satellite access channel bandwidths. The nominal channel spacing between two adjacent Satellite access carriers is defined as following: - For operating bands with 100 kHz channel raster, Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2 where BWChannel(1) and BWChannel(2) are the Satellite access channel bandwidths of the two respective Satellite access carriers. The channel spacing can be adjusted depending on the channel raster to optimize performance in a particular deployment scenario. - For different transmission directions in either part of a paired spectrum, a duplexer is needed for both base station and the terminal. The required frequency separation between the paired spectrum is the same as full-duplex FDD. |
| 8.2.5.1.9 | Support of advanced antenna capabilities |
| 8.2.5.1.9.1 | Fully describe the multi-antenna systems supported in the User segment, Space segment, or both that can be used and/or must be used; characterize their impacts on systems performance; e.g. does the RIT have the capability for the use of: – spatial multiplexing techniques; – space-time coding (STC) techniques; – beam-forming techniques (e.g. adaptive or switched). The multi-antenna systems in NR satellite access supports the following MIMO transmission schemes at both the UE and the base station: Beamforming is supported. Beam management with periodic and aperiodic beam refinement is also supported at the UE and at the base station. |
| 8.2.5.1.9.2 | How many antennas are supported by the Space segment and MS for transmission and reception? Specify if correlated or uncorrelated antennas in co-polar or cross-polar configurations are used. What is the antenna spacing (in wavelengths)? NR satellite access supports {1, 2, 4, 8, 12, 16, 24, 32} antenna ports in the DL and {1, 2, 4} antenna ports in the UL. NTN Payload and UE support rectangular antenna arrays. The rectangular panel array antenna can be described by the following tuple (Mg,Ng, M,N, P), where Mg is the number of panels in a column, Ng is the number of panels in row, M,N are the number of vertical, horizontal antenna elements within a panel and P is number of polarizations per antenna element. The spacing in vertical and horizontal dimensions between the panels is specified by dg,V, dg,H and between antenna elements by dV, dH. NR satellite access specification is flexible to support various antenna spacing, number of antenna elements, antenna port layouts and antenna virtualization approaches. Satellite transmission and reception may also implement Circular polarization (RHCP and LHCP) |
| 8.2.5.1.9.3 | Provide details on the antenna configuration that is used in the self-evaluation. The information will be provided with self evaluation results. |
| 8.2.5.1.9.4 | If spatial multiplexing (MIMO) is supported, does the proposal support (provide details if supported): – Single codeword (SCW) and/or multi-codeword (MCW). – Open and/or closed loop MIMO. – Cooperative MIMO. – Single-user MIMO and/or multi-user MIMO. MIMO is not supported |
| 8.2.5.1.9.5 | Other antenna technologies Does the RIT/SRIT support other antenna technologies, for example Active Antenna System (AAS) If so, please describe. The use of Active Antenna System is supported by NR satellite access. RF requirements for satellite access node (SAN) types and those associated to Active Antenna System are found in [38.108] |
| 8.2.5.1.10 | Link adaptation and power control |
| 8.2.5.1.10.1 | Describe link adaptation techniques employed by RIT/SRIT, including: – the supported modulation and coding schemes; – the supporting channel quality measurements, the reporting of these measurements, their frequency and granularity; Provide details of any adaptive modulation and coding schemes, including: – hybrid ARQ or other retransmission mechanisms? – algorithms for adaptive modulation and coding, which are used in the self-evaluation; – other schemes? For data, the RIT supports dynamic indication of - combinations of modulation scheme and target code rate and, - the resource allocation in frequency and time (The resource allocation in frequency is within BWP) that the UE uses to determine the transport block size where the possible combinations cover a large range of possible data and channel coding rates. 28 different target coding rates can be indicated (29 if 256QAM is not enabled) and the target code rate range is 0.0293 to 0.896. In both downlink and uplink, link adaptation (selection of modulation scheme and code rate) is controlled by the base station. In the downlink, the network selection of modulation-scheme/code-rate combination can e.g. be based on channel state information (CSI) reported by the terminals. The RIT features a flexible CSI framework where the type of CSI, reporting quantity, frequency-granularity and time-domain behaviour can be configured. Both periodic and aperiodic(triggered) reporting modes are supported, controlled by the base station, where the aperiodic reporting allows the network to request which CSI-RS resources to report the CSI for. More details can be found in [38.214] section 5.2. In the uplink the base station may measure either the traffic channel or sounding reference signals and use this as input to the link adaptation. More details can be found in [38.214] section 6.2.1. On the MAC layer, hybrid ARQ with soft-combining between transmissions is supported. Different redundancy versions can be used for different transmissions. The modulation and coding scheme may be changed for retransmissions. In order to minimize delay and feedback, a set of parallel stop-and-wait protocols are used. For NR NTN operation the maximal supported HARQ process number is increased and it is up to 32 for both UL and DL. Further, in NTN, enabling and disabling HARQ feedback for downlink transmission can be semi-statically configurable per HARQ process via UE specific RRC signalling. To correct possible residual errors, the MAC ARQ is complemented by a robust selective-repeat ARQ protocol on the RLC layer. More details are found in [38.321] and [38.322]. |
| 8.2.5.1.10.2 | Provide details of any power control scheme included in the proposal, for example: – power control step size (dB); – power control cycles per second; – power control dynamic range (dB); – minimum transmit power level with power control; – associated signalling and control messages. Uplink power control is independent for uplink data (PUSCH), uplink control(PUCCH) and sounding reference signal (SRS). The uplink power control is based on both signal-strength measurements done by the terminal itself (open-loop power control), as well as measurements by the base station. The latter measurements are used to generate power-control commands that are subsequently fed back to the terminals as part of the downlink control signaling (closed-loop power control). Both absolute and relative power-control commands are supported. There are four available relative power adjustments (“step size”) in case of relative power control. For uplink data, multiple closed loop power control processes can be configured, including the possibility separate processes with transmission beam indication. The time between power-control commands for PUSCH and PUCCH is the same as the scheduling periodicity for the PUSCH and the PDSCH, respectively. More details about uplink power control are found in [38.213] section 7. Downlink power control is network-implementation specific and thus outside the scope of the specification. A simple and efficient power control strategy is to transmit with a constant output power. Variations in channel conditions and interference levels are adapted to by means of scheduling and link adaptation. |
| 8.2.5.1.11 | Power classes |
| 8.2.5.1.11.1 | Mobile station emitted power |
| 8.2.5.1.11.1.1 | What is the radiated antenna power measured at the antenna (dBm)? For frequency range 1, the maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The maximum output power is defined by UE power class as following table. <UE maximum output power for frequency range 1> NR satellite band; Class 3 (dBm); Tolerance (dB). n256; 23; ±2. n255; 23; ±2. n254; 23; ±2. |
| 8.2.5.1.11.1.2 | What is the maximum peak power transmitted while in active or busy state? See item 8.2.5.1.11.1.1. |
| 8.2.5.1.11.1.3 | What is the time averaged power transmitted while in active or busy state? Provide a detailed explanation used to calculate this time average power. Full power UE transmission has been predominantly assumed as representative of the power utilized by each UE to achieve the required performance, |
| 8.2.5.1.11.2 | Space segment emitted power |
| 8.2.5.1.11.2.1 | What is the base station transmit power per RF carrier? For the BS type 1-H, the BS conducted output power is measured at TAB connector for BS type 1-H. For the BS type 1-O, radiated transmit power is defined as the EIRP level for a declared beam at a specific beam peak direction - For a declared beam and beam direction pair, the rated beam EIRP level is the maximum power that the base station is declared to radiate at the associated beam peak direction during the transmitter ON period. Base Stations intended for general-purpose applications do not have limits on the maximum transmit power. However, there may exist regional regulatory requirements which limit the maximum transmit power. |
| 8.2.5.1.11.2.2 | What is the maximum peak transmitted power per RF carrier radiated from antenna? Base Stations intended for general-purpose applications do not have limits on the maximum transmit power. However, there may exist regional regulatory requirements which limit the maximum transmit power. |
| 8.2.5.1.11.2.3 | What is the average transmitted power per RF carrier radiated from antenna? The averaged transmitted carrier power is subject to the type of signal/channel to be transmitted, bandwidth, and deployment scenario, etc. |
| 8.2.5.1.12 | Scheduler, QoS support and management, data services |
| 8.2.5.1.12.1 | QoS support – What QoS classes are supported? – How QoS classes associated with each service flow can be negotiated. – QoS attributes, for example: – data rate (ranging from the lowest supported data rate to maximum data rate supported by the MAC/PHY); – control plane and user plane latency (delivery delay); – packet error rate (after all corrections provided by the MAC/PHY layers), and delay variation (jitter). – Is QoS supported when handing off between radio access networks? Please describe. – How users may utilize several applications with differing QoS requirements at the same time. In NR satellite access, QoS model is based on QoS Flows, and both GBR QoS Flows and non-GBR QoS Flows are supported. At NAS level, the QoS flow is the finest granularity of QoS differentiation in a PDU session. Each QoS Flow is associated with a QoS profile which contains QoS parameters including a 5G QoS Identifier (5QI), an Allocation/ Retention Priority (ARP), Reflective QoS Attribute (RQA) for non-GBR Flows, Guaranteed Flow Bit Rate (GFBR) and Maximum Flow Bit Rate (MFBR) for GBR QoS Flows, and optionally with Notification Control and Maximum Packet Loss Rate for GBR QoS Flows. The 5QI is an index representing the resource type, priority, packet delay budget, packet error rate, maximum data burst volume, and averaging window of a QoS Flow. 3GPP Release 17 has introduced a new 5QI (5QI = 10) with a packet delay budget (PDB) going up to 1100ms. This large PDB value is defined to support non-guaranteed bit rate (non-GBR) services provisioned via satellite that can accommodate the worst case latencies when GEO satellite are used. For each UE, one or multiple PDU sessions can be established, and within one PDU session, up to 64 QoS Flows can be allocated. At AS level, for each UE, one or multiple data bearers can be established, and QoS Flow to data bearer mapping is controlled by NG-RAN. Up to 29 data bearers can be established in parallel for a UE. One or more QoS flows can be mapped to a data bearer. Reflective mapping (UE applies the DL mapping rule to UL packets) is supported in both NAS level and AS level. QoS profile is provided by 5GC to NG-RAN and is used by NG-RAN to determine the treatment on the radio interface. The ARP as well as other QoS parameters could be used to determine which bearers to prioritise at handover. By using multiple QoS Flows / data bearers having different QoS profiles, multiple application flows with different QoS requirements could be accommodated. |
| 8.2.5.1.12.2 | Scheduling mechanisms – Exemplify scheduling algorithm(s) that may be used for full buffer in the technology proposal for evaluation purposes. Describe any measurements and/or reporting required for scheduling. In NR satellite access physical control and shared channels can be separately and dynamically scheduled for both uplink and downlink. A scheduling unit for downlink shared channel may span from 2-14 symbols and for uplink shared channel from 1-14 symbols (14 symbols comprise a “slot”). Sub-carrier spacing for different physical channels may be dynamically changed by switching bandwidth-parts (BWP). Typically, NR satellite access scheduling is based on the instantaneous radio-link quality as seen by the different users, and the traffic demand and quality-of-service requirements of individual users and in the cell as a whole. The former is based on CQI reports from the terminals (downlink) or measurements of sounding signals from the terminals (uplink). Based on this the base station may e.g. apply a proportional fair scheduling algorithm. The QoS assessment is supported by means of receiving QoS information from the “higher layers”. For non-full buffer traffic like VOIP (or any traffic having similar characteristics) semi-persistent scheduling in DL can be applied, by which a user can be allocated time-frequency resources in a semi-persistent manner, i.e., fixed resources are allocated at certain intervals without L1/L2 control signaling each time. This is especially useful to reduce the L1/L2 control signaling overhead and to increase VoIP capacity. In addition, with UL Configured Grants, the scheduler can allocate uplink resources to users. When a configured uplink grant is active, if the user cannot find an uplink grant assigned via downlink control channel an uplink transmission according to the configured uplink grant can be made. Otherwise, if the user finds an uplink grant assigned via downlink control channel, this assignment overrides the configured uplink grant. NR satellite access supports slot aggregation in downlink and uplink, by which time-frequency resources can be allocated consecutively to a user for a longer period than a slot by a single L1/L2 control signaling. A larger transport block size or a lower coding rate can be supported by this technique. This is especially useful when the coverage needs to be extended. As another option to extend coverage or improve reliability in addition to slot aggregation, a set of MCS tables supporting very low code rate for both DL and UL can be used. For the downlink and the uplink, intercell-interference coordination can be realized by the scheduler that is transparent to the physical layer. |
| 8.2.5.1.13 | Radio interface architecture and protocol stack |
| 8.2.5.1.13.1 | Describe details of the radio interface architecture and protocol stack such as: Logical channels – Control channels. – Traffic channels. Transport channels and/or physical channels. RAN/Radio Architectures: This RIT contains NR satellite access standalone architecture. The following paragraphs provide a high level summary of radio interface protocols and channels. Radio Protocols: The protocol stack for the user plane includes the following: SDAP, PDCP, RLC, MAC, and PHY sublayers (terminated in UE and gNB). On the Control plane, the following protocols are defined: - RRC, PDCP, RLC, MAC and PHY sublayers (terminated in UE and gNB); - NAS protocol (terminated in UE and AMF) For details on protocol services and functions, please refer to 3GPP specifications (e.g. [38.300]). Radio Channels (Physical, Transport and Logical Channels) The physical layer offers service to the MAC sublayer transport channels. The MAC sublayer offers service to the RLC sublayer logical channels. The RLC sublayer offers service to the PDCP sublayer RLC channels. The PDCP sublayer offers service to the SDAP and RRC sublayer radio bearers: data radio bearers (DRB) for user plane data and signalling radio bearers (SRB) for control plane data. The SDAP sublayer offers 5GC QoS flows and DRBs mapping function. The physical channels defined in the downlink are: - the Physical Downlink Shared Channel (PDSCH), - the Physical Downlink Control Channel (PDCCH), - the Physical Broadcast Channel (PBCH), The physical channels defined in the uplink are: - the Physical Random Access Channel (PRACH), - the Physical Uplink Shared Channel (PUSCH), - and the Physical Uplink Control Channel (PUCCH). In addition to the physical channels above, PHY layer signals are defined, which can be reference signals, primary and secondary synchronization signals. The following transport channels, and their mapping to PHY channels, are defined: Uplink: - Uplink Shared Channel (UL-SCH), mapped to PUSCH - Random Access Channel (RACH), mapped to PRACH Downlink: - Downlink Shared Channel (DL-SCH), mapped to PDSCH - Broadcast channel (BCH), mapped to PBCH - Paging channel (PCH), mapped to PDSCH Logical channels are classified into two groups: Control Channels and Traffic Channels. Control channels: - Broadcast Control Channel (BCCH): a downlink channel for broadcasting system control information. - Paging Control Channel (PCCH): a downlink channel that transfers paging information and system information change notifications. - Common Control Channel (CCCH): channel for transmitting control information between UEs and network. - Dedicated Control Channel (DCCH): a point-to-point bi-directional channel that transmits dedicated control information between a UE and the network. Traffic channels: Dedicated Traffic Channel (DTCH), which can exist in both UL and DL. In Downlink, the following connections between logical channels and transport channels exist: - BCCH can be mapped to BCH, or DL-SCH; - PCCH can be mapped to PCH; - CCCH, DCCH, DTCH can be mapped to DL-SCH; In Uplink, the following connections between logical channels and transport channels exist: - CCCH,DCCH, DTCH can be mapped to UL-SCH. Other aspects - NR satellite access QoS architecture The QoS architecture in NG-RAN (connected to 5GC), can be summarized as follows: For each UE, 5GC establishes one or more PDU Sessions. For each UE, the NG-RAN establishes one or more Data Radio Bearers (DRB) per PDU Session. The NG-RAN maps packets belonging to different PDU sessions to different DRBs. Hence, the NG-RAN establishes at least one default DRB for each PDU Session. NAS level packet filters in the UE and in the 5GC associate UL and DL packets with QoS Flows. AS-level mapping rules in the UE and in the NG-RAN associate UL and DL QoS Flows with DRBs For more details on NR satellite access Radio Protocol architecture and channels, refer to: [38.300], [38.401], [38.201], |
| 8.2.5.1.13.2 | What is the bit rate required for transmitting feedback information? As described in other sections (e.g. 8.2.5.1.3, 8.2.5.1.10, 8.2.5.1.13.1), from a Layer1 point of view (PHY/MAC), few control (feedback/HARQ) channels are defined (in UL and DL), with specific characteristics and transmission schemes/rates. At Layer2 level (i.e. RLC ARQ), assuming an RLC AM Status report is sent every 50 ms (configurable), with a size of few octets, e.g. 32 bits (including RLC/MAC header overhead), this results in a rate of 32/0.05= 640 bit/s. |
| 8.2.5.1.13.3 | Channel access Describe in detail how RIT/SRIT accomplishes initial channel access, (e.g. contention or non-contention based). Initial channel access is typically accomplished via the “random access procedure” (assuming no dedicated/scheduled resources are allocated). The random access procedure can be contention based (e.g. at initial connection from idle mode) or non-contention based (e.g. during Handover to a new cell). Random access resources and parameters are configured by the network and signalled to the UE (via broadcast or dedicated signaling). Contention based random access procedure encompasses the transmission of a random access preamble by the UE (subject to possible contention with other UEs), followed by a random access response (RAR) in DL (including allocating specific radio resources for the uplink transmission). Afterwards, the UE transmits the initial UL message (e.g. RRC connection Request) using the allocated resources, and wait for a contention resolution message in DL (to confirming access to that UE). The UE could perform multiple attempts until it is successful in accessing the channel or until a timer (supervising the procedure) elapses. Non-contention based random access procedure foresees the assignment of a dedicated random access resource/preamble to a UE (e.g. part of an HO command). This avoids the contention resolution phase, i.e. only the random access preamble and random access response messages are needed to get channel access. From a L1 perspective, a random access preamble is transmitted (UL) in a PRACH, random access response (DL) in a PDSCH, UL transmission in a PUSCH, and contention resolution message (DL) in a PDSCH. At the physical layer, 3GPP specified enhancements to meet the challenges related to large round trip delay, timing drift and Doppler shift/variation inherent to the satellite radio links: - For uplink time and frequency synchronization, NTN UE should support uplink time and frequency pre-compensation: o The UE shall calculate UE specific TA (corresponding to the round trip time or RTT on the service link) based on its GNSS-acquired position and the serving satellite ephemeris. o It shall calculate common TA (corresponding to any common delay e.g. RTT on the feeder link) according to the parameters provided by the network (UE considers common TA as 0 if the parameters are not provided) o The UE performs the pre-compensation of the calculated TA in its uplink transmissions in initial access and when connected. o For TA update in RRC_CONNECTED state, support of combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops o Support of frequency pre-compensation to counter shift the Doppler experienced on the service link. To calculate the frequency pre-compensation, the UE calculates the Doppler based on its GNSS-acquired position and the serving satellite ephemeris. For more details, refer to:[38.300], [38.321] and [38.213] |
| 8.2.5.1.14 | Beam selection |
| 8.2.5.1.14.1 | Describe in detail how the RIT/SRIT accomplishes cell selection to determine the serving cell for the users. Cell selection is based on the following principles: - The UE NAS layer identifies a selected PLMN (and equivalent PLMNs, if any); - For NR satellite access, a UE with location capability should use its awareness of its location to select a PLMN that is allowed to operate in the UE location as specified in TS 23.122. In order to ensure that the regulatory requirements are met, the network may be configured to enforce this UE choice by verifying the UE location Further, the following enhancements have been specified: - New “NG-RAN satellite” RAT type in USIM - Extension of the NAS supervision timers over satellite access for GEO and MEO RAT types (LEO uses legacy timers) - Modification of the higher priority PLMN selection procedure to include shared MCC 9xx - New minimum periodic search timer for higher priority PLMN search over satellite access when PLMN uses shared MCC - New trigger for PLMN selection upon transition in/out international areas (based on UE implementation) - New Forbidden List of PLMNs not allowed to operate at UE location and its handling - New 5GMM cause value#78 and its handling (related to the list in the previous bullet) - Support for multiple TACs for the same PLMN broadcast in the radio cell, including corresponding logic for determining the “Current TAI” and impact on ME-USIM procedures - The UE searches the supported frequency bands (RIT specific) and for each carrier frequency it searches and identifies the strongest cell. It reads cell broadcast information to identify its PLMN(s) and other relevant parameters (e.g. related to cell restrictions); - The UE seeks to identify a suitable cell; if it is not able to identify a “suitable” cell it seeks to identify an “acceptable” cell. - A cell is “suitable” if: the measured cell attributes satisfy the cell selection criteria (based on DL radio signal strength/quality); the cell belongs to the selected/equivalent PLMN; cell is not restricted (e.g. cell is not barred/reserved or part of "forbidden" roaming areas); - An “acceptable” cell is one for which the measured cell attributes satisfy the cell selection criteria and the cell is not barred. Among the identified suitable (or acceptable) cells, the UE selects the strongest cell, (technically it “camps” on that cell). As signalled/configured by the radio network, certain frequencies or RITs could be prioritized for camping. For NR satellite access, the following mobility in RRC_IDLE and RRC_INACTIVE enhancements have been specfied: o Cell (re)selection enhancements in the quasi-earth fixed cell scenario: UE can perform time-based and location-based measurement. The timing and location information associated to a cell are provided via system information. They refer respectively to the time when the serving cell is going to stop serving a geographical area and to the reference location and a distance threshold of serving cell. o Location assisted cell reselection, with the distance between UE and the reference location of the cell (serving cell and/or neighbor cell) taken into account, is supported for quasi-earth fixed cell o Measurement rules for cell re-selection with timing information and location information are specified in TS 38.304 For more details, refer to:[38.300] sub-clause 9.2.1.1 and [38.304] sub-clause 5.2 |
| 8.2.5.1.15 | Location determination mechanisms |
| 8.2.5.1.15.1 | Describe any location determination mechanisms that may be used, e.g. to support location-based services. Upon network request, after AS security is established in connected mode, a UE should report its coarse UE location information (most significant bits of the GNSS coordinates, ensuring an accuracy in the order of 2 km) to the NG-RAN if available. |
| 8.2.5.1.16 | Priority access mechanisms |
| 8.2.5.1.16.1 | Describe techniques employed to support prioritization of access to radio or network resources for specific services or specific users (e.g. to allow access by emergency services). NR Satellite access supports overload and access control functionality such as RACH back off, RRC Connection Reject, RRC Connection Release and UE based access barring mechanisms. One unified access control framework as specified in 3GPP TS 22.261 section 6.22 is applied for NR satellite access. For each access attempt one Access Category and one or more Access Identities are selected. NR satellite access broadcasts barring control information associated with Access Categories and Access Identities and the UE determines whether an identified access attempt is authorized or not, based on the broadcasted barring information and the selected Access Category and Access Identities. In the case of multiple core networks sharing the same RAN, the RAN provides broadcasted barring control information for each PLMN individually. The unified access control framework is applicable to all UE states (RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state). For NAS triggered requests, the UE NAS determines one access category and access identity(ies) for the given access attempt and provides them to RRC for access control check. The RRC performs access barring check based on the access control information and the determined access category and access identities. The RRC indicates whether the access attempt is allowed or not to NAS layer. The NAS also performs the mapping of the access category and access identity(ies) associated with the access attempt to establishment cause and provides the establishment cause to RRC for inclusion in connection request to enable the gNB to decide whether to reject the request. For AS triggered request (i.e. RNA update), the RRC determines the resume cause value and the corresponding access category. |
| 8.2.5.1.17 | Unicast, multicast and broadcast |
| 8.2.5.1.17.1 | Describe how the RIT enables: – broadcast capabilities; – multicast capabilities; – unicast capabilities; using both dedicated carriers and/or shared carriers. Please describe how all three capabilities can exist simultaneously. The RIT supports mostly unicast transmission of data to/from users. Broadcast capabilities pertain to support and transmission of cell-wide system information/parameters, as well as broadcast/based emergency services (e.g. public warning messages). |
| 8.2.5.1.17.2 | Describe whether the proposal is capable of providing multiple user services simultaneously to any user with appropriate channel capacity assignments? Multiple services per user can be supported by setting up multiple data radio bearers (DRBs) per user/device. Each radio bearer is characterized by an individual QoS profile/flow. Multiple services per user/device can also be supported by mapping multiple services to a single bearer, if the QoS is the same for these services. The new SDAP sublayer (in the Access Stratum) provides mapping function between (5GC) QoS flows and DRBs. See more details on QoS in 8.2.5.1.12.1 |
| 8.2.5.1.17.3 | Does the RIT support multiple voice and/or video codecs? Provide details. The RIT could support various voice and video codecs, as desired. In fact, the radio interface technology (fully IP-based) is mostly agnostic to such codecs, and capable of accommodating diverse range of codec types, rates and operation (fixed/dynamic/adaptive). This enables support for all main codecs used/defined today (e.g. AMR-NB/WB, EVS), as well as the capability to support more enhanced codecs that may be defined in future. |
| 8.2.5.1.18 | Privacy, authorization, encryption, authentication and legal intercept schemes |
| 8.2.5.1.18.1 | Any privacy, authorization, encryption, authentication and legal intercept schemes that are enabled in the radio interface technology should be described. Describe whether any synchronization is needed for privacy and encryptions mechanisms used in the RIT. Describe how the RIT may be protected against attacks, for example: – man in the middle; – replay; – denial of service. NR Satellite access supports subscriber’s privacy compared to earlier generations, see 3GPP TS 33.501. The most important enhancement is the concealment of subscription permanent identifier over-the-air. This feature is mainly aimed against the active attacker. Another enhancement is the guaranteed regular refreshment of subscription temporary identifier. This feature is mainly aimed against the passive attacker. Another enhancement is the decoupling of the permanent identifier from the Paging mechanism, i.e., there is no longer a Paging message with permanent identifier, the Paging timings are no longer based on permanent identifier. This feature mitigates privacy attacks that use side-channel information in the Paging protocol. Another enhancement comes from the best effort protection of information in the initial message, i.e., if security is setup, privacy sensitive information is concealed, otherwise skipped until the security is setup. Yet another effort is description of a device-assisted network-based framework for false base station detection. This feature can be used to thwart denial-of-service kind of attackers. One enhancement is to increase the authentication between the service network and the core network, or the function of the home network control which can be used to prevent false fraud of the service network. One enhancement is mitigation of bidding down attacks, this feature is to avoid the lower security features of the UE or network selection. |
| 8.2.5.1.19 | Frequency planning |
| 8.2.5.1.19.1 | How does the RIT support adding new cells or new RF carriers? Provide details. 1008 physical cell identities are supported. Thus, theoretically 1008-cell reuse is realized.. Actual cell deployment is operation specific. Self configuration can be also supported. |
| 8.2.5.1.20 | Interference mitigation within radio interface |
| 8.2.5.1.20.1 | Does the proposal support Interference mitigation? If so, describe the corresponding mechanism. The RIT has been designed with the aim to minimize the always-on signals to reduce the interference in the system. This is achieved by: - Support longer periodicities for synchronization signals, broadcast channels and periodic reference signals - Use UE-specific demodulation reference signals for control and data that are only transmitted when control and/or data is being transmitted - Control channel resource allocation in the frequency domain is configurable to reduce the interference to control channels in neighbouring cells |
| 8.2.5.1.20.2 | What is the signalling, if any, which can be used for inter-beam interference mitigation? To mitigate inter-beam interference, spatial frequency reuse schemes (Frequency reuse factor, FRF > 1) may be used. The traditional Frequency Reuse-3 (FRF-3) scheme, for example, offers a sufficient protection against inter-cell interference. Therefore, techniques such as Remote Interference Management (RIM) coordination or Cross-Link Interference (CLI) Management which are traditionally used in TDD systems and involving inter-base station signalling via the Xn interface are not needed for inter-beam interference mitigation in NR Satellite access. |
| 8.2.5.1.20.3 | Link level interference mitigation Describe the feature or features used to mitigate inter-symbol interference. Time and frequency synchronization to the DL and UL frame structures in combination with the use of a cyclic prefix OFDM transmission in both UL(with or without transform precoding) and DL, provides robustness against intersymbol interference. To achieve synchronisation, before and during connection to an NR Satellite access cell, the UE shall compute the RTT between UE and the reference point based on the GNSS position, the ephemeris, and the Common TA parameters (see clause 4.2 in TS 38.213), and autonomously pre-compensate the timing for the RTT between UE and the reference point (see clause 4.3 of TS 38.211). The UE shall compute the frequency Doppler shift of the service link, and autonomously pre-compensate for it in the uplink transmissions, by considering UE position and the ephemeris. If the UE does not have a valid GNSS position and/or valid ephemeris and Common TA, it shall not transmit until both are regained. In connected mode, the UE shall be able to continuously update the Timing Advance and frequency pre-compensation. |
| 8.2.5.1.20.4 | Describe the approach taken to cope with multipath propagation effects (e.g. via equalizer, rake receiver, cyclic prefix, etc.). The use of OFDM transmission in both UL and DL, in combination with a cyclic prefix, provides inherent robustness to time-dispersion/frequency-selectivity on the radio channel. In case of transform precoding in the UL, time-dispersion/frequency-selectivity on the radio channel can be handled by receiver-side equalization. |
| 8.2.5.1.20.5 | Diversity techniques Describe the diversity techniques supported in the MS and at the BS, including micro diversity and macro diversity, characterizing the type of diversity used, for example: – Time diversity: repetition, Rake-receiver, etc. – Space diversity: multiple sectors, etc. – Frequency diversity: frequency hopping (FH), wideband transmission, etc. – Code diversity: multiple PN codes, multiple FH code, etc. – Multi-user diversity: proportional fairness (PF), etc. – Other schemes. Characterize the diversity combining algorithm, for example, switched diversity, maximal ratio combining, equal gain combining. Provide information on the receiver/transmitter RF configurations, for example: – number of RF receivers; – number of RF transmitters. The RIT provides the following means for diversity: - Space diversity by means of multiple transmit and receiver antennas and beamforming - Number of TX-antenna ports: This is a deployment choice, but for the purpose of multi-layer transmissions up to 12 downlink and up to 4 uplink antenna ports have been defined where the mapping of ports to physical antennas is an implementation issue - Number of RX antenna ports: Implementation specific - Frequency diversity by means of wide overall transmission bandwidth and possibility for uplink frequency hopping and uplink and downlink frequency-distributed transmissions - Time diversity by means of fast retransmissions with hybrid ARQ protocol allowing combining of the retransmissions with the original transmission Multi-user diversity by means of channel-aware scheduling. |
| 8.2.5.1.21 | Synchronization requirements |
| 8.2.5.1.21.1 | Describe RIT’s timing requirements, e.g.: – is synchronization between inter-beam signals or inter-satellite signals required? Provide precise information, the type of synchronization, i.e. synchronization of carrier frequency, bit clock, spreading code or frame, and their accuracy; – is space segment-to-network synchronization required? State frequency and timing accuracy of space segment and user equipment transmit signal. Common general aspects Tight Satellite-to-Satellite synchronization is not required. Likewise, tight Satellite-to-network synchronization is not required. A certain RAN-CN Hyper SFN synchronization is required in case of extended Idle mode DRX. |
| 8.2.5.1.21.2 | Describe the synchronization mechanisms used in the proposal, including synchronization between a user terminal and a satellite access node (base station). Cell search is the procedure by which a UE acquires time and frequency synchronization with a cell and detects the physical layer Cell ID of that cell. A UE receives the following synchronization signals (SS) in order to perform cell search: the primary synchronization signal (PSS) and secondary synchronization signal (SSS). PSS is used (at least) for initial symbol boundary, cyclic prefix, sub frame boundary, initial frequency synchronization to the cell. SSS is used for radio frame boundary identification. PSS and SSS together used for cell ID detection. For uplink time synchronization in NR Satellite access, the UE should perform uplink time pre-compensation as described in 8.2.5.1.13.3 . To achieve synchronisation, before and during connection to an Satellite access cell, the UE shall compute the RTT between UE and the reference point based on the GNSS position, the ephemeris, and the Common TA parameters (see clause 4.2 in TS 38.213), and autonomously pre-compensate the timing for the RTT between UE and the reference point (see clause 4.3 of TS 38.211 [52]). The UE shall compute the frequency Doppler shift of the service link, and autonomously pre-compensate for it in the uplink transmissions, by considering UE position and the ephemeris. If the UE does not have a valid GNSS position and/or valid ephemeris and Common TA, it shall not transmit until both are regained. In connected mode, the UE shall be able to continuously update the Timing Advance and frequency pre-compensation See more information in [38.213] sub-clause 4 and [38.211] sub-clause 7.4.2. |
| 8.2.5.1.22 | Link budget template Proponents should provide the link budget results to this description template for the environments supported in the RIT. The information is provided with link budget template. |
| 8.2.5.1.23 | Other items |
| 8.2.5.1.23.1 | Coverage extension schemes Describe the capability to support/coverage extension schemes, such as relays or repeaters. NR Satellite access supports the use of the following mechanisms to improve the coverage - NR satellite access can use DFT-spreading and Pi/2 BPSK modulation to reduce PAPR and increase average Tx power for better coverage - NR satellite access can use very low coding rate for better coverage. - Slot aggregation for both control and data can be used for better coverage - High-aggregation level (up to 16) downlink control is possible for better coverage - Beam management is used to increase the coverage |
| 8.2.5.1.23.2 | Self-organization Describe any self-organizing aspects that are enabled by the RIT/SRIT. Support for Self Organizing Networks is an integrated part of NR satellite access. Two use cases that could benefit from SON have been introduced in the Release 15 and the work is continuing. NR satellite access currently supports the following Self-Organizing Network (SON) functions: (Details are provided in [38.300], [38.413], [38.423], [38.331]) – Automatic neighbor discovery: the mechanism allows an gNB to learn information on its neighbors. The discovery mechanism can utilize the assistance of the UE (aka ANR funtion [38.300, Sec. 15.3.3]) as well as the exchange of information over the network interfaces ([38.423; Sec 8.4.1, 8.4.2, 9.1.3.1, 9.1.3.2, 9.1.3.4, 9.1.3.5] as well as the radio resource control information [38.331; Sec 5.5.2, 6.3.2]). – Xn-C TNL address discovery: the mechanism allows a gNB to determine the TNL address on its neighbors candidate gNB. The discovery mechanism can utilize of the ANR function (aka ANR funtion [38.300, Sec. 15.3.4]) as well as the exchange of information over the network interfaces ([38.413; Sec8.8.1, 8.8.2, 9.2.7.1, 9.2.7.2 ) |
| 8.2.5.1.23.3 | Describe the frequency reuse schemes (including reuse factor and pattern) for the assessment of cell spectrum efficiency, cell edge user spectral efficiency. Uncoordinated frequency reuse one and three is used in the performance evaluations. |
| 8.2.5.1.23.4 | Is the RIT an evolution of an existing IMT-Advanced technology? Provide details. NO |
| 8.2.5.1.23.5 | Does the proposal satisfy a specific spectrum mask? Provide details. (This information is not intended to be used for sharing studies.) Yes. UE: For Frequency Range 1 (FR1) UE: For single-component-carrier transmission the spectrum mask is specified in terms of a normative (general) spectrum emission mask [38.101-5, section 6.5.2.2] and an additional spectrum mask [38.101-5, section 6.5.2.3]. This additional spectrum emission mask which is signaled by the network to the UE as a normative requirement can be used to address a specific regional regulatory requirement, a frequency band specific requirement, a roaming requirement and a specific deployment scenario. This additional spectrum emission mask can be used to support the many different sharing requirements in terms of co-existence for a global roaming terminal. BS: For single-component-carrier transmission and transmission of aggregated component-carriers the radiated spectrum mask requirements are defined in [38.108],. in form of OTA out-of-band emissions limits. The unwanted emission limits in the part of the downlink operating band that falls in the spurious domain are consistent with ITU-R Recommendation SM.329. For single-component-carrier transmission and transmission of aggregated component-carriers the conducted spectrum mask requirements are defined in [38.108], for for BS type 1-O and section. in form of OTA out-of-band emissions. |
| 8.2.5.1.23.6 | Describe any user terminal power saving mechanisms used in the RIT. Multiple features facilitating device power saving as already have been specified, including Discontinuous reception (DRX) in RRC_CONNECTED, RRC_INACTIVE and RRC_IDLE, Bandwidth part (BWP) adaptation, RRC_INACTIVE state, and Pipelining frame structure enabling micro-sleep within slots in which the UE is not scheduled. Details can be found in item in 8.2.5.1.24. |
| 8.2.5.1.23.7 | Simulation process issues Describe the methodology used in the analytical approach. Proponent should provide information on the width of confidence intervals of user and satellite system performance metrics of corresponding mean values, and evaluation groups are encouraged to provide this information. System simulations are iterated over M independent ‘drops’ of user locations. Statistics, mean and percentiles, are calculated over all drops, and confidence intervals are estimated by comparing the results of the different drops. The number of drops is up to each evaluator. |
| 8.2.5.1.24 | Other information Please provide any additional information that the proponent believes may be useful to the evaluation process. NR satellite access supports the following set of features for providing long battery life: - A configurable transmission and reception bandwidth for limiting the device modem power consumption. - DFT-spread OFDM modulation for limiting the peak to average ratio of the uplink waveform and increasing the device power amplifier efficiency. - Uplink power control which allows the device to adapt its transmit power to the actual radio environment. - Connected mode DRX cycles for reducing the device power consumption while in RRC Active state. - Measurement rules for reducing the RRC idle mode RRM activities. - Resumption of a previous connection for minimizing the control signalling when initiating a mobile originated or terminated data transmission. NR satellite access has the capability to support different satellite deployment scenarios at different orbits including Non-GEO and GEO based non-terrestrial access network (e.g., see [TR 38.821]). |

Reference

[22.261]	3GPP TS 22.261v17.11.0,”Service Requirements for the 5G System”

[38.101-5]    	3GPP TS 38.101-5v17.5.0,”User Equipment (UE) radio transmission and reception Part 5: Satellite access Radio Frequency (RF) and performance requirements”

[38.104]      	3GPP TS 38.104v17.11.0,”Base Station (BS) radio transmission and reception”

[38.108]      	3GPP TS 38.108v17.5.0,”Satellite Access Node radio transmission and reception”

[38.133]      	3GPP TS 38.133v17.11.0,” Requirements for support of radio resource management”

[38.201]	3GPP TS38.201 v17.0.0” Physical layer; General description”

[38.211]      	3GPP TS 38.211v17.6.0,”Physical channels and modulation”

[38.212]     	3GPP TS38.212v17.6.0,” Multiplexing and channel coding”

[38.213]	3GPP TS 38.213v17.7.0,” Physical layer procedures for control”

[38.214]	3GPP TS 38.214v17.7.0,” Physical layer procedures for data”

[38.215]      	3GPP TS 38.215v17.3.0,”Physical layer measurements”

[38.300]     	3GPP TS 38.300v 17.6.0,”Overall description; Stage-2”

[38.304]      	3GPP TS 38.304v17.6.0,”User Equipment (UE) procedures in idle mode and in RRC Inactive state”

[38.306]      	3GPP TS 38.306v17.6.0,”User Equipment (UE) radio access capabilities”

[38.321]      	3GPP TS 38.321v17.6.0,”Medium Access Control (MAC) protocol specification”

[38.322]	3GPP TS 38.322v17.3.0,”Radio Link Control (RLC) protocol specification”

[38.401]       	3GPP TS 38.401v17.6.0,”NG-RAN; Architecture description”

[38.413]         TS38.413v17.6.0“,NG-RAN; NG Application Protocol (NGAP)”

[38.423]         TS38.423v17.6.0, “NG-RAN; Xn Application Protocol (XnAP)”

[38.811]     	3GPP TR 38.811v15.4.0,“Study on New Radio (NR) to support non-terrestrial networks”

[38.821]      	3GPP TR 38.821v16.2.0,”Solutions for NR to support Non-Terrestrial Networks (NTN)”

[38.863]	        3GPP TR 38.863v17.2.0,”Non-terrestrial networks (NTN) related RF and co-existence aspects”

____________________

Developed by 3GPP as 5G NTN, Release 17 and beyond.