## Characteristics template for 3GPP 5G NTN SRIT (Release 17 and beyond)

The description templates provided by 3GPP are for the description of the characteristics of 5G satellite access developed by 3GPP. It includes one characteristics template for SRIT (encompassing component RITs for NR NTN (NR satellite access) and IoT NTN (consisting of NB-IoT satellite access and eMTC satellite access), and one characteristics template for NR NTN (NR satellite access) RIT. The description provided is common for all component RITs, unless otherwise stated. For IoT NTN, the description provided is common for both NB-IoT satellite access and eMTC satellite access, unless otherwise stated.

This document provides the characteristics template for the description of the characteristics of the SRIT based on 3GPP Rel-17 work.

For this characteristics template, 3GPP has addressed all of the characteristics, and it is expected that these descriptions are helpful to assist in evaluation activities for independent evaluation groups, as well as to facilitate the understanding of the state-of-art of 3GPP development on the RIT.

| Item | Item to be described |
| --- | --- |
| 8.2.5.1.1 | Test environment(s) |
| 8.2.5.1.1.1 | What test environments does this technology description template address? This proposal addresses all the three test environments across the three usage scenarios (eMBB-s, mMTC-s, and HRC-s) as described in Report ITU-R M.2514-0. |
| 8.2.5.1.2 | Radio interface functional aspects |
| 8.2.5.1.2.1 | Multiple access schemes Which access scheme(s) does the proposal use: TDMA, FDMA, CDMA, OFDMA, IDMA, SDMA, hybrid, or another? Describe in detail the multiple access schemes employed with their main parameters. For NR satellite access component RIT: - Downlink and Uplink: The multiple access is a combination of - OFDMA: Synchronous/scheduling-based; the transmission to/from different UEs uses mutually orthogonal frequency assignments. Granularity in frequency assignment: One resource block consisting of 12 subcarriers. Multiple sub-carrier spacings are supported including 15kHz, 30kHz and 60kHz for data (see Item 8.2.5.1.7 and reference therein). - CP-OFDM is applied for both downlink and uplink. DFT-spread OFDM can also be configured for uplink. - Spectral confinement technique(s) (e.g. filtering, windowing, etc.) for a waveform at the transmitter is transparent to the receiver. When such confinement techniques are used, the spectral utilization ratio can be enhanced. - TDMA: Transmission to/from different UEs with separation in time. Granularity: One slot consisting of 14 OFDM symbols, or 2~13 OFDM symbols non-slot (for DL) or 1~13 OFDM symbols (for UL) within one slot. The physical length of one slot ranges from 0.25 to 1ms depending on the sub-carrier spacing (for more details on the frame structure, see Item 8.2.5.1.7 and the references therein). - CDMA: Inter-cell interference suppressed by processing gain of channel coding allowing for a frequency reuse of one (for more details on channel-coding, see Item 8.2.5.1.2.3 and the reference therein). - SDMA: Possibility to transmit to/from multiple users using the same time/frequency resource (SDMA a.k.a. “multi-user MIMO”) as part of the advanced-antenna capabilities (for more details on the advanced-antenna capabilities, see Item 8.2.5.1.9 and the reference therein) At least an UL transmission scheme without scheduling grant is supported. (Note: Synchronous means that timing offset between UEs is within cyclic prefix by e.g. timing alignment.) For IoT NTN component RIT: For eMTC satellite access downlink and Uplink: The multiple access is a combination of - OFDMA: Synchronous/scheduling-based is supported for both DL and UL; the transmission to/from different UEs uses mutually orthogonal frequency assignments. Granularity in frequency assignment: For the UL: 3, 6 or 12 sub-carriers with a sub-carrier spacing of 15 kHz. For the DL: One resource block consisting of 12 subcarriers. Sub-carrier spacings of 15kHz is supported (see Item 5.2.3.2.7 and reference therein). - CP-OFDM is applied for downlink. DFT-spread OFDM is applied for uplink. - TDMA: Transmission to/from different UEs with separation in time. Granularity: One subframe of length 1 ms, (for more details on the frame structure, see Item 8.2.5.1.7 and the references therein). Repetition of a transmission is supported. - CDMA: Inter-cell interference suppressed by processing gain of channel coding allowing for a frequency reuse of one (for more details on channel-coding, see Item 8.2.5.1.2.3 and the reference therein). For NB-IoT satellite access, the multiple access is a combination of OFDMA, TDMA and CDMA, where OFDMA and TDMA are as follows: - OFDMA: - UL: DFT-spread OFDM. Granularity in frequency domain: A single sub-carrier with either 3.75 kHz or 15 kHz sub-carrier spacing, or 3, 6, or 12 sub-carriers with a sub-carrier spacing of 15 kHz. A resource block consists of 12 sub-carriers with 15 kHz sub-carrier spacing, or 48 sub-carriers with 3.75 kHz sub-carrier spacing → 180 kHz. - DL: Granularity in frequency domain: one resource block consisting of 6 or 12 subcarriers with 15 kHz sub-carrier spacing→90 or 180 kHz - TDMA: Transmission to/from different UEs with separation in time - UL: Granularity: One resource unit of 1 ms, 2 ms, 4 ms, 8 ms, with 15 kHz sub-carrier spacing, depending on allocated number of sub-carrier(s); or 32 ms with 3.75 kHz sub-carrier spacing (for more details on the frame structure, see Item 8.2.5.1.7 and the references therein) - DL: Granularity: One resource unit (subframe) of length 1 ms. Repetition of a transmission is supported. |
| 8.2.5.1.2.2 | Modulation scheme |
| 8.2.5.1.2.2.1 | What is the baseband modulation scheme? If both data modulation and spreading modulation are required, describe in detail. Describe the modulation scheme employed for data and control information. What is the symbol rate after modulation? For NR satellite access component RIT: - Downlink: - For both data and higher-layer control information: QPSK, 16QAM, and 64QAM (see [38.211] sub-clause 7.3.1.2). - L1/L2 control: QPSK (see [38.211] sub-clause 7.3.2.4). - Symbol rate: 1008 ksymbols/s per 360kHz resource block (equivalently 168ksymbols/s per 180kHz resource block) - Uplink: - For both data and higher-layer control information: π/2-BPSK (when precoding is enabled), QPSK, 16QAM, 64QAM and 256QAM (see [38.211] sub-clause 6.3.1.2). - L1/L2 control: BPSK, π/2-BPSK, QPSK (see [38.211] sub-clause 6.3.2). - Symbol rate: 672ksymbols/s per 1440kHz resource block (equivalently 168ksymbols/s per 180kHz resource block) The above is at least applied to eMBB-s. For IoT NTN component RIT: For eMTC satellite access downlink: - For both data and higher-layer control information: QPSK, 16QAM, 64QAM (see [36.211] sub-clause 6.3.2). - L1/L2 control: QPSK (see [36.211] sub-clauses 6.7.2, 6.8.3, and 6.8A.3) - Symbol rate: 168ksymbols/s per 180kHz resource block For eMTC satellite access uplink: - For both data and higher-layer control information: π/2-BPSK, QPSK and 16QAM are supported (see [36.211] sub-clause 5.3.2). - L1/L2 control: BPSK, QPSK (see [36.211] sub-clause 5.4) - Symbol rate: 168ksymbols/s per 180kHz resource block. For UL, less than one resource block may be allocated. For NB-IoT satellite access, the modulation scheme is as follows. - Data and higher-layer control: π/2-BPSK (uplink only), π/4-QPSK (uplink only), QPSK - L1/L2 control: π/2-BPSK (uplink), QPSK (downlink) - Symbol rate: 168 ksymbols/s per 180 kHz resource block. For UL, less than one resource block may be allocated. |
| 8.2.5.1.2.2.2 | PAPR What is the RF peak to average power ratio after baseband filtering (dB)? Describe the PAPR (peak-to-average power ratio) reduction algorithms if they are used in the proposed RIT. The PAPR depends on the waveform and the number of component carriers. The single component carrier transmission is assumed herein when providing the PAPR. For DFT-spread OFDM, PAPR would depend on modulation scheme as well. For uplink using DFT-spread OFDM, the cubic metric (CM) can also be used as one of the methods of predicting the power de-rating from signal modulation characteristics, if needed. For NR satellite access component RIT: - Downlink: The PAPR is 8.4dB (99.9%) - Uplink: - For CP-OFDM: The PAPR is 8.4dB (99.9%) - For DFT-spread OFDM: The PAPR is provided in the table below. Modulation; π/2 BPSK; QPSK; 16QAM; 64QAM; 256QAM. PAPR (99.9%); 4.5 dB; 5.8 dB; 6.5 dB; 6.6 dB; 6.7 dB. CM (99.9%); 0.3 dB; 1.2 dB; 2.1 dB; 2.3 dB; 2.4 dB. Note: The above values are derived without spectrum shaping. When spectrum shaping is considered for π/2 BPSK, lower PAPR and CM values can be derived, e.g., 1.75dB PAPR for π/2 BPSK, based on the trade-off between PAPR and demodulation performance. For IoT NTN component RIT: For eMTC satellite access downlink: The PAPR is 8.4dB (99.9%). For eMTC satellite access uplink: - For DFT-spread OFDM: The PAPR is provided in the table below. Modulation; π/2 BPSK; QPSK; 16QAM. PAPR (99.9%); 0.3 dB; 5.8 dB; 6.5 dB. CM (99.9%); 1.2 dB; 2.1 dB. For NB-IoT satellite access, - Downlink: The PAPR is 8.0dB (99.9%) on 180kHz resource. - Uplink: The PAPR is 0.23 – 5.6 dB (99.9 %) depending on sub-carriers allocated for available NB-IoT UL modulation. Additional general comments Any PAPR-reduction algorithm is transmitter-implementation specific for uplink and downlink. High PAPR means that power amplifiers (HPA) have to operate with increased back-off leading to reduced efficiency. This is not appropriate for the power-limited satellite downlink which typically requires operation in the nonlinear region of high power amplifiers. To alleviate PAPR problems in Satellite-based radio interface, PAPR-reduction algorithms that are transmitter implementation specific can be considered. Therefore, PAPR optimizations for downlink channels are not necessary to be specified for satellite radio interface access. |
| 8.2.5.1.2.3 | Error control coding scheme and interleaving |
| 8.2.5.1.2.3.1 | Provide details of error control coding scheme for both downlink and uplink? For example: – FEC or other schemes? – Unequal error protection? Explain the decoding mechanism employed. For NR satellite access component RIT: - Downlink and Uplink: - For data: BG#1 and BG#2 based Low density parity check (LDPC) coding, combined with rate matching based on shortening/puncturing/repetition to achieve a desired overall code rate (For more details, see [38.212] sub-clauses 5.3.2). LDPC channel coder facilitates low-latency and high-throughput decoder implementations. - For L1/L2 control: For DCI (Downlink Control Information)/UCI (Uplink Control Information) size larger than 11 bits, Polar coding, combined with rate matching based on shortening/puncturing/repetition to achieve a desired overall code rate (For more details, see [38.212] sub-clauses 5.3.1). Otherwise, repetition for 1-bit; simplex coding for 2-bit; Reed-Muller coding for 3~11-bit DCI/UCI size. The above scheme is at least applied to eMBB-s. Decoding mechanism is receiver-implementation specific. Example of information on the decoding mechanism will be provided together with self-evaluation. For IoT NTN component RIT: For eMTC satellite access downlink and uplink: - For data: Rate 1/3 Turbo coding, combined with rate matching based on puncturing/repetition to achieve a desired overall code rate. One transport block can be mapped to one or multiple resource units. (For more details, see [36.212] sub-clauses 5.1.3.2) - For L1/L2 control: Rate-1/3 tail-biting convolutional coding. Special block codes for some L1/L2 control signalling (For more details, see [36.212] sub-clauses 5.1.3.1) For NB-IoT satellite access, the coding scheme is as follows: - For data: Rate 1/3 Turbo coding in UL, and rate-1/3 tail-biting convolutional coding in DL, each combined with rate matching based on puncturing/repetition to achieve a desired overall code rate; one transport block can be mapped to one or multiple resource units (for more details, see [36.212] sub-clause 6.2) - For L1/L2 control: The same as above. For IoT NTN, the decoding mechanism is receiver-implementation specific. Example of information on the decoding mechanism will be provided together with self-evaluation. |
| 8.2.5.1.2.3.2 | Describe the bit interleaving scheme for both uplink and downlink. For NR satellite access component RIT: - Downlink: - For data: bit interleaver is performed for LDPC coding after rate-matching (For more details, see [38.212] sub-clauses 5.4.2.2) - For L1/L2 control: Bit interleaving is performed as part of the encoding process for Polar coding (For more details, see [38.212] sub-clauses 5.4.1.1) - Uplink: - For data: bit interleaver is performed for LDPC coding after rate-matching (For more details, see [38.212] sub-clauses 5.4.2.2) - For L1/L2 control: Bit interleaving is performed for Polar coding after rate-matching (For more details, see [38.212] sub-clauses 5.4.1.3) The above scheme is at least applied to eMBB-s. For IoT NTN component RIT: - Downlink and Uplink: Bit interleaving is performed as part of the encoding/rate-matching process, see [36.212] sub-clauses 5.1.3 and 5.1.4 for more details. Additional interleaving is performed in uplink, see [36.212] sub-clause 5.2.2.8 for more details. |
| 8.2.5.1.3 | Describe channel tracking capabilities (e.g. channel tracking algorithm, pilot symbol configuration, etc.) to accommodate rapidly changing delay spread profile For NR satellite access component RIT: To support channel tracking, different types of reference signals can be transmitted on downlink and uplink respectively. - Downlink: - Primary and Secondary Synchronization signals (PSS and SSS) are transmitted periodically to the cell. The periodicity of these signals is network configurable. UEs can detect and maintain the cell timing based on these signals. If the gNB implements hybrid beamforming, then the PSS and SSS are transmitted separately to each analogue beam. Network can configure multiple PSS and SSS in frequency domain. - UE-specific Demodulation RS (DM-RS) for PDCCH can be used for downlink channel estimation for coherent demodulation of PDCCH (Physical Downlink Control Channel). DM-RS for PDCCH is transmitted together with the PDCCH. - UE-specific Demodulation RS (DM-RS) for PDSCH can be used for downlink channel estimation for coherent demodulation of PDSCH (Physical Downlink Shared Channel). DM-RS for PDSCH is transmitted together with the PDSCH. - UE-specific Phase Tracking RS (PT-RS) can be used in addition to the DM-RS for PDSCH for correcting common phase error between PDSCH symbols not containing DM-RS. It may also be used for Doppler and time varying channel tracking. PT-RS for PDSCH is transmitted together with the PDSCH upon need. - UE-specific Channel State Information RS (CSI-RS) can be used for estimation of channel-state information (CSI) to further prepare feedback reporting to gNB to assist in MCS selection, beamforming, MIMO rank selection and resource allocation. CSI-RS transmissions are transmitted periodically, aperiodically, and semi-persistently on a configurable rate by the gNB. CSI-RS also can be used for interference measurement and fine frequency/time tracking purposes. - Uplink: - UE-specific Demodulation RS (DM-RS) for PUCCH can be used for uplink channel estimation for coherent demodulation of PUCCH (Physical Uplink Control Channel). DM-RS for PUCCH is transmitted together with the PUCCH. - UE-specific Demodulation RS (DM-RS) for PUSCH can be used for uplink channel estimation for coherent demodulation of PUSCH (Physical Uplink Shared Channel). DM-RS for PUSCH is transmitted together with the PUSCH. - UE-specific Phase Tracking RS (PT-RS) can be used in addition to the DM-RS for PUSCH for correcting common phase error between PUSCH symbols not containing DM-RS. It may also be used for Doppler and time varying channel tracking. DM-RS for PUSCH is transmitted together with the PUSCH upon need. - UE-specific Sounding RS (SRS) can be used for estimation of uplink channel-state information to assist uplink scheduling, uplink power control, as well as assist the downlink transmission (e.g. the downlink beamforming in the scenario with UL/DL reciprocity). SRS transmissions are transmitted periodically, aperiodically, and semi-persistently by the UE on a gNB configurable rate. Terminology: To ease understanding of specific terms/abbreviations used in this item here after, few main acronyms and definitions are introduced: - Common TA: a network-controlled common TA, and may include any timing offset considered necessary by the network (e.g. feeder link delay). - Reference point for uplink time synchronization: The point at which DL and UL are frame aligned with an offset given by NTA,offset To accommodate the large round trip delay, timing drift and Doppler effect in Satellite radio access, the user terminal performs the uplink time pre-compensation and frequency pre-compensation. For the serving cell, the network broadcast valid ephemeris information and Common TA parameters. The UE shall have valid GNSS position as well as ephemeris and Common TA before connecting to an NTN cell. To achieve synchronisation, before and during connection to a cell, the UE shall compute the RTT between UE and the reference point (RP) based on the GNSS position, the ephemeris, and the Common TA parameters (see clause 4.2 in TS 38.213 [29]), and autonomously pre-compensate the TTA for the RTT between UE and the RP (see clause 4.3 of TS 38.211 [30]). The UE shall compute the frequency Doppler shift of the service link, and autonomously pre-compensate for it in the uplink transmissions, by considering UE position and the ephemeris. If the UE does not have a valid GNSS position and/or valid ephemeris and Common TA, it shall not transmit until both are regained. In connected mode, the UE shall be able to continuously update the Timing Advance and frequency pre-compensation. Details of channel-tracking/estimation algorithms are receiver-implementation specific, and not part of the specification. Details of channel-tracking/estimation algorithms are receiver-implementation specific, e.g. MMSE-based channel estimation with appropriate interpolation in time and frequency domain could be used. For IoT NTN component RIT: - For eMTC satellite access downlink: - Cell-specific RS (CRS) are transmitted in every non-MBSFN subframe and over the entire frequency band. Up to four different CRS can be transmitted within a cell, with each CRS corresponding to one of up to four cell-specific antenna ports, referred to antenna port 0 to 3 respectively. The CRS can be used for downlink channel estimation for coherent demodulation of physical channels transmitted from antenna ports 0 to 3. The CRS can also be used to derive channel-state information (CSI) for the corresponding antenna ports. The CSI can e.g. be used to assist scheduling (including link adaptation, precoder-matrix/vector selection, etc.). For the detailed structure of CRS, see [36.211] sub-clause 6.10.1. - UE-specific RS and Demodulation Reference Signals can be used for downlink channel estimation for coherent demodulation of PDSCH (Physical Downlink Shared Channel) and MPDCCH (MTC Physical Downlink Shared Channel). In a given subframe, the UE-specific reference signals are only transmitted within the resource blocks that are used for PDSCH transmission to the specific UE within this subframe. For the detailed structure of UE-specific RS, see [36.211] sub-clause 6.10.3. The structure for the case of transmission from multiple antenna ports is an extension of the structure for the case of a single antenna port. - CSI-RS can be used for estimation of channel-state information (CSI) to further prepare feedback reporting to eNB (CQI for link adaptation, precoder- vector selection, etc.) to assist beamforming and scheduling. CSI-RS are transmitted periodically and aperiodically by the eNB. Periodic CSI-RS are transmitted in every Nth subframe, where N is configurable. - For eMTC satellite access uplink: - Demodulation RS (DMRS) can be used for channel estimation for coherent demodulation of the Physical Uplink Shared Channel (PUSCH), and the Physical Uplink Control Channel (PUCCH). Uplink DMRS for demodulation of PUSCH are transmitted once every slot (twice every subframe) in the subframes in which PUSCH is being transmitted. Up to four uplink DMRS can be transmitted from a UE. The instantaneous bandwidth of the uplink DMRS equals the instantaneous bandwidth of the corresponding PUSCH transmission. - For the detailed structure of uplink DMRS for PUSCH transmission for the case of single antenna transmission, see [36.211] sub-clause 5.5.2.1. - Sounding RS (SRS) can be used for estimation of uplink channel-state information to assist uplink scheduling, uplink power control, and also assist downlink transmissions (e.g. downlink beamforming in scenarios with UL/DL reciprocity). Uplink SRS are transmitted either periodically every Nth subframe, where N is configurable, or aperiodically when triggered by the network. For the detailed structure of uplink SRS see [36.211] sub-clause 5.5.3. - For NB-IoT satellite access downlink: - Narrowband Reference signals (NRS) are used. NRS are transmitted in a certain minimum set of subframes, and additionally in a configured set of subframes. NRS associated with paging, random access response, and multicast transmissions on non-anchor NB-IoT carriers do not have to be transmitted on subframes far away from the associated transmissions, even if they are in the configured set of subframes. Up to two different NRS can be transmitted within a cell, with each NRS corresponding to one of up to two cell-specific antenna ports, referred to as antenna port 2000 to 2001 respectively. The NRS can be used for downlink channel estimation for coherent demodulation of physical channels transmitted from antenna ports 2000 to 2001. For the detailed structure of NRS, see [36.211] sub-clause 10.2.6. - For NB-IoT satellite access uplink: - Demodulation RS (DMRS) for demodulation of NPUSCH are transmitted once every slot (twice every subframe) in the subframes in which NPUSCH is being transmitted. The instantaneous bandwidth of the uplink DMRS equals the instantaneous bandwidth of the corresponding NPUSCH transmission. One DMRS for NPUSCH transmission can be transmitted from a UE. For the detailed structure of uplink DMRS for NPUSCH transmission, see [36.211] sub-clause 10.1.4. Uplink frequency and time domain pre-compensation for IoT NTN requires the UE to adapt its uplink transmission to counter-act the effects of doppler and delay inherent in the uplink radio channel (which varies depending on the relative location and velocity between satellite and UE), and additional radio access delays inherent to the feeder link between satellite and ground station component of the Satellite Access node, to minimise interference that may otherwise be observed between different UEs’ transmissions at the network receiver. The UE is provided with satellite Ephemeris Information and common Timing Advance parameters. The UE uses this along with its own GNSS-acquired location to determine the level of uplink precompensation needed in terms of doppler and delay. This procedure is used both for initial access and while UE is in connected mode. The detailed description of the required UE behaviour for uplink precompensation is defined in [36.300] sub-clause 23.21.2.2. The support of uplink precompensation as well as other aspects inherent to the radio access design allows the system to cope with large ranges of doppler and delay channel characteristics between satellite and UE. Details of channel-tracking/estimation algorithms are receiver-implementation specific, e.g. MMSE-based channel estimation with appropriate interpolation in time and frequency domain could be used. |
| 8.2.5.1.4 | Physical channel structure and multiplexing |
| 8.2.5.1.4.1 | What is the physical channel bit rate (Mbit/s) for supported bandwidths? i.e. the product of the modulation symbol rate (in symbols per second), bits per modulation symbol, and the number of streams supported by the antenna system. For NR satellite access component RIT: The physical channel bit rate depends on the modulation scheme, number of spatial-multiplexing layer, number of resource blocks in the channel bandwidth and the subcarrier spacing used. The physical channel bit rate per layer can be expressed as Rlayer = Nmod x NRB x 2µ x 168 kbps where - Nmod is the number of bits per modulation symbol for the applied modulation scheme (QPSK: 2, 16QAM: 4, 64QAM: 6, 256QAM: 8) - NRB is the number of resource blocks in the aggregated frequency domain which depends on the channel bandwidth. - µ depends on the subcarrier spacing, ∆f, given by ∆f=2μ∙15 kHz, μ=0,1 For example, a 30 MHz carrier with 78 resource blocks using 30 kHz subcarrier spacing, μ=1, and 64QAM modulation results in a physical channel bit rate of 157 Mbit/s per layer. For IoT NTN component RIT: The physical channel bit rate depends on the modulation scheme and number of resource blocks in the channel bandwidth and the subcarrier spacing used. When the subcarrier spacing is 15 kHz, the physical channel bit rate per layer can be expressed as Rlayer = Nmod x NRB x 168 kbps where - Nmod is the number of bits per modulation symbol for the applied modulation scheme (QPSK: 2, 16QAM: 4, 64QAM: 6) - NRB is the number of resource blocks in the aggregated frequency domain. NRB = 6 for the eMTC channel bandwidth. NB-IoT only supports transmission of a single layer and the physical channel bit rate is as above, but with Nmod limited to 1(BPSK) or 2 (QPSK) and NRB= 1. |
| 8.2.5.1.4.2 | Layer 1 and Layer 2 overhead estimation Describe how the RIT accounts for all Layer 1 (PHY) and Layer 2 (MAC) overhead and provide an accurate estimate that includes static and dynamic overheads. For NR satellite access component RIT: - Downlink The downlink L1/L2 overhead includes: - Different types of reference signals - Demodulation reference signals for PDSCH (DMRS-PDSCH) - Phase-tracking reference signals for PDSCH (PTRS-PDSCH) - Demodulation reference signals for PDCCH - Reference signals specifically targeting estimation of channel-state information (CSI-RS) - Tracking reference signals (TRS) - L1/L2 control signalling transmitted on the up to three first OFDM symbols of each slot - Synchronization signals and physical broadcast control channel including demodulation reference signals included in the SS/PBCH block - PDU headers in L2 sub-layers (MAC/RLC/PDCP) The overhead due to different type of reference signals is given in the table below. Note that demodulation reference signals for PDCCH is included in the PDCCH overhead. Reference signal type; Example configurations; Overhead for example configurations. DMRS-PDSCH; As examples, DMRS can occupy 1/3, ½, or one full OFDM symbol. 1, 2, 3 or 4 symbols per slot can be configured to carry DMRS.; 2.4 % to 29 %. PTRS- PDSCH; 1 resource elements in frequency domain every second or fourth resource block. PTRS is mainly intended for FR2.; 0.2% or 0.5 % when configured.. CSI-RS; 1 resource element per resource block per antenna port per CSI-RS periodicity; 0.25 % for 8 antenna ports transmitted every 20 ms with 15 kHz subcarrier spacing. TRS; 2 slots with 1/2 symbol in each slot per transmission period; 0.36 % or 0.18% respectively for 20 ms and 40ms periodicity. The overhead due to the L1/L2 control signalling is depending on the size and periodicity of the configured CORESET in the cell and includes the overhead from the PDCCH demodulation reference signals. If the CORESET is transmitted in every slot, maximum control channel overhead is 21% assuming three symbols and whole carrier bandwidth used for CORESET, while a more typical overhead is 7% when 1/3 of the time and frequency resources in the first three symbols of a slot is allocated to PDCCH. The overhead due to the SS/PBCH block is given by the number of SS/PBCH blocks transmitted within the SS/PBCH block period, the SS/PBCH block periodicity and the subcarrier spacing. Assuming a 100 resource block wide carrier, the overhead for 20 ms periodicity is in the range of 0.6 % to 2.3 % if the maximum number of SS/PBCH blocks are transmitted. - Uplink L1/L2 overhead includes: - Different types of reference signals - Demodulation reference signal for PUSCH - Demodulation reference signal for PUCCH - Phase-tracking reference signals - Sounding reference signal (SRS) used for uplink channel-state estimation at the network side - L1/L2 control signalling transmitted on a configurable amount of resources (see also Item 8.2.5.1.4.5) - L2 control overhead due to e.g., random access, uplink time-alignment control, power headroom reports and buffer-status reports - PDU headers in L2 layers (MAC/RLC/PDCP) The overhead due to due to demodulation reference signal for PUSCH is the same as the overhead for demodulation reference signal for PDSCH, i.e. 4 % to 29 % depending on number of symbols configured. Also, the phase-tracking reference signal overhead is the same in UL as in DL. The overhead due to periodic SRS is depending on the number of symbols configured subcarrier spacing and periodicity. For 20 ms periodicity, the overhead is in the range of 0.4% to 1.4% assuming15 kHz subcarrier spacing. Amount of uplink resources reserved for random access depends on the configuration. The relative overhead due to uplink time-alignment control depends on the configuration and the number of active UEs within a cell. The amount of overhead for buffer status reports depends on the configuration. The amount of overhead caused by 4 highly depends on the data packet size. For IoT NTN component RIT: - For eMTC satellite access downlink The downlink L1/L2 overhead includes: - Different types of reference signals - Cell-specific RS (CRS) transmitted within each resource block - UE-specific demodulation RS - Reference signals specifically targeting estimation of channel-state information (CSI-RS) - L1/L2 control signalling transmitted on the up to four first OFDM symbols of each subframe. - Synchronization signal and physical broadcast control channel - PDU headers in L2 sub-layers (MAC/RLC/PDCP) The combined overhead due to CRS (1a) and L1/L2 control signaling (2) depends on the number of cell-specific antenna ports and the number of OFDM symbols used for the L1/L2 control signaling. Some examples, for the case of no CSI-RS and no UE-specific RS, see 4.2.3.2.3 above, are shown below: - 1 antenna port for CRS and 1 symbol for L1/L2 control: 10.7% - 1 antenna port for CRS and 3 symbols for L1/L2 control: 25% - 4 antenna ports for CRS and 1 symbol for L1/L2 control: 19% - 4 antenna ports for CRS and 3 symbols for L1/L2 control: 31% The amount of overhead caused by synchronization signals and broadcast channel is approximately 6%. The amount of overhead caused by L2 highly depends on the data packet size, and is approximately 2.7% for L1 data rates of 1Mbit/s, respectively. In a typical case, the relative overhead for CSI-RS (if present) is estimated to 0.06% per antenna port. The relative overhead due to UE-specific RS (if present) is estimated to be approximately 7%. - For eMTC satellite access uplink L1/L2 overhead includes: - Demodulation reference symbols used e.g. for uplink channel estimation for uplink coherent demodulation, transmitted once every 0.5 ms slot. - Sounding reference signal (SRS) used for uplink channel-state estimation at the network side - L1/L2 control signalling transmitted on configurable amount of resource blocks (see also Item 5.2.3.2.4.5) - L2 control overhead due to e.g., random access, uplink time-alignment control, power headroom reports and buffer-status reports - PDU headers in L2 layers (MAC/RLC/PDCP) The amount of overhead caused by 1 is approximately 14%, corresponding to one DFTS-OFDM symbol in each slot. The relative overhead is estimated to be independent of the rank of the transmission. The amount of SRS overhead depends on the SRS transmission interval, SRS bandwidth and the usage of UpPTS for SRS. With a 10 msec SRS transmission interval and full band SRS, the relative overhead is approximately 0.7% Amount of uplink resources reserved for random access depends on the configuration. A typical case with PRACH format 0 is six resource blocks per radio frame, implying a relative overhead of 0.6%, 1.2%, and 2.4% for a channel bandwidth of 20 MHz, 10 MHz, and 5 MHz respectively. The relative overhead due to uplink time-alignment control depends on the configuration and the number of active UEs within a cell. The absolute overhead is typically less than 32 bps per UE. The amount of overhead for buffer status reports depends on the configuration. With continuous data and a 10 - 20 ms reporting interval the absolute overhead is 0.8-3.2 kbps. The amount of overhead caused by 5 highly depends on the data packet size, and is approximately 2.7%, for L1 data rates of 1Mbit/s, respectively. Above overhead calculations are based on normal CP length. - For NB-IoT satellite access downlink: The overhead from Narrowband RS (NRS) is dependent on the number of cell-specific antenna ports N (1 or 2) and equals 8 x N / 168 %. The overhead from NB-IoT downlink control signaling is dependent on the amount of data to be transmitted. For small infrequent data transmissions, the downlink transmissions are dominated by the L2 signaling during the connection setup. The overhead from L1 signaling is dependent on the configured scheduling cycle. The overhead due to Narrowband synchronization signal and Narrowband system information broadcast messages is only applicable to the NB-IoT anchor carrier. The actual overhead depends on the broadcasted system information messages and their periodicity. The overhead can be estimated to be around 26.25%. - For NB-IoT satellite access uplink: Data and control share the same resources and the overhead from L1/L2 control signaling depends on the scheduled traffic in the DL. The UL control signaling is dominated by RLC and HARQ positive or negative acknowledgments. A typical NB-IoT NPRACH overhead is in the order of 5 %. |
| 8.2.5.1.4.3 | Variable bit rate capabilities Describe how the proposal supports different applications and services with various bit rate requirements. For both NR satellite access and IoT NTN component RITs: For a given combination of modulation scheme and code rate, the data rate available to a user can be controlled by the scheduler by assigning different number of resource blocks for the transmission. In case of multiple services, the available/assigned resource, and thus the available data rate, is shared between the services. |
| 8.2.5.1.4.4 | Variable payload capabilities Describe how the RIT supports IP-based application layer protocols/services (e.g. VoIP, video-streaming, interactive gaming, etc.) with variable-size payloads. For NR satellite access component RIT: The transport-block size can vary between X bits and Y bits. The number of bits per transport block can be set with a fine granularity. See [38.214] sub-clause 5.1.3.2 for details. For IoT NTN component RIT: The transport-block size can vary between 16 bits and ”maximum transport block size” bits. The number of bits per transport block can be assigned with a fine granularity. For eMTC satellite access, the maximum transport block size is 1000 bits in both UL and DL (optionally 2984 bits in uplink and 1736 bits in downlink) for the lowest UE category dedicated to eMTC and 4008 bits and 6968 bits for DL and UL respectively for the highest UE category dedicated to eMTC satellite access. See [36.213] sub-clause 7.1.7.2.1 for details. For NB-IoT satellite access, the maximum transport block size is 680 bits in the DL and 1000 bits in UL for the lowest UE category and 2536 bits (optionally 4968 bits) for DL and 2536 for UL for the highest UE category. See [36.213] sub-clause 16.4.1.5.1 for details. |

TABLE 2 (continued)

| Item | Item to be described |
| --- | --- |
| 8.2.5.1.4.5 | Signalling transmission scheme Describe how transmission schemes are different for signalling/control from that of user data. For NR satellite access component RIT: - Downlink L1/L2 control signalling is transmitted in assigned resources time and frequency multiplexed with data within the bandwidth part (BWP, see item 8.2.5.1.8.1). Control signalling is limited to QPSK modulation (QPSK, 16QAM, 64QAM and 256QAM for data). Control signalling error correcting codes are polar codes (LDPC codes for data). - Uplink L1/L2 control signalling transmitted in assigned resources and can be time and frequency multiplexed with data within the BWP. L1/L2 control signalling can also be multiplexed with data on the PUSCH. Modulation schemes for L1/L2 control signalling is π/2-BPSK, BPSK and QPSK Control signalling error correcting codes are block codes for small payload and polar codes for larger payloads (LDPC codes for data). For both downlink and uplink, higher-layer signalling (e.g. MAC, RLC, PDCP headers and RRC signalling) is carried within transport blocks and thus transmitted using the same physical-layer transmitter processing as user data. For IoT NTN component RIT: - Downlink L1 control signalling is time-multiplexed with data from a single UE perspective. Control signalling is limited to QPSK modulation. Control signalling relies on tail-biting convolutional coding. Turbo-codes are used for data with the exception for NB-IoT satellite access where also data transmission uses tail-biting convolutional coding. In case of eMTC and NB-IoT satellite access the L1 control signaling is confined to a configured set of resource blocks and can be time multiplexed with data and are transmitted in scheduled subframes. - Uplink L1 control signalling transmitted in one or multiple resource blocks typically at the edge of the system bandwidth and frequency multiplexed with data. For NB-IoT satellite access the L1 control signaling is time and frequency multiplexed with data. For both downlink and uplink, higher-layer signalling (e.g. MAC, RLC, PDCP headers and RRC signalling) is carried within transport blocks and thus transmitted using the same physical-layer transmitter processing as user data. |
| 8.2.5.1.5 | Mobility management (Handover) |
| 8.2.5.1.5.1 | Describe the handover mechanisms and procedures which are associated with – Inter-System handover; – Intra-System handover; – Intra-frequency and Inter-frequency; – Within the RIT or between RITs within one SRIT (if applicable). Characterize the type of handover strategy or strategies. For NR satellite access component RIT: - Inter-System handover: NTN – TN handover refers to mobility in both directions, i.e. from NTN to TN (hand-in) and from TN to NTN (hand-out). During mobility between NTN and Terrestrial Network (TN), a UE is not required to connect to both NTN and TN at the same time. - Intra-System handover: Network controlled mobility applies to UEs in RRC_CONNECTED and is categorized into two types of mobility: - Cell level mobility requires explicit RRC signalling to be triggered, i.e. handover. For inter-gNB handover, handover request, handover acknowledgement, handover command, handover complete procedure are supported between source gNB and target gNB. The release of the resources at the source gNB during the handover completion phase is triggered by the target gNB. - Beam level mobility does not require explicit RRC signalling to be triggered - it is dealt with at lower layers - and RRC is not required to know which beam is being used at a given point in time. Data forwarding, in-sequence delivery and duplication avoidance at handover can be guaranteed between target gNB and source gNB. UE may support mobility between gNBs operating with satellite payloads in different orbits (e.g., GSO, NGSO at different altitudes). Service link and feeder link Switchover: A feeder link switchover is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for a specific NTN payload. The feeder link switchover is a Transport Network Layer procedure. Service link switch refers to a change of the serving NTN payload. A feeder link switch over may result in transferring the established connection for the affected UEs between two gNBs. For soft feeder link switch over, an NTN payload is able to connect to more than one NTN Gateway during a given period, i.e. a temporary overlap can be ensured during the transition between the feeder links. For hard feeder link switch over, an NTN payload connects to only one NTN Gateway at any given time, i.e. a radio link interruption may occur during the transition between the feeder links. For more details, refer to [38.300] sub-clauses 16.14.4. For IoT NTN component RIT: - Terminology: To ease understanding of specific terms/abbreviations used in this item here after, few main acronyms and definitions are introduced: - eNB: E-UTRAN node, connecting to EPC - RAN: Radio Access Network - NTN: Non Terrestrial Network i.e. Satellite radio interface based network - Intra-system handover/mobility For eMTC satellite access RRC_CONNECTED state, network-controlled UE-assisted handovers between eMTC satellite access cells are performed and various DRX cycles are supported. Handover procedures, like processes that precede the final HO decision on the source network side (control and evaluation of UE and eNB measurements taking into account certain UE specific roaming and access restrictions), preparation of resources on the target network side, commanding the UE to the new radio resources and finally releasing resources on the (old) source network side. It contains mechanisms to transfer context data between evolved nodes, and to update node relations on C-plane and U-plane. Measurements to be performed by a UE for intra/inter-frequency mobility can be controlled by RAN, using broadcast or dedicated control. In RRC_IDLE state, a UE shall follow the measurement parameters defined for cell reselection specified by the RAN broadcast. The use of dedicated measurement control for RRC_IDLE state is possible through the provision of UE specific priorities. In RRC_CONNECTED state, a UE shall follow the measurement configurations specified by RRC directed from the RAN (e.g. as in UTRAN MEASUREMENT_CONTROL). In RRC_IDLE and RRC_CONNECTED the UE may be configured to monitor some eMTC carriers according to reduced performance requirements as specified in [36.133]. Intra-frequency neighbour (cell) measurements and inter-frequency neighbour (cell) measurements are defined as follows: - Intra-frequency neighbour (cell) measurements: Neighbour cell measurements performed by the UE are intra-frequency measurements when the current and target cell operates on the same carrier frequency. - Inter-frequency neighbour (cell) measurements: Neighbour cell measurements performed by the UE are inter-frequency measurements when the neighbour cell operates on a different carrier frequency, compared to the current cell. For more details, refer to [36.300] sub-clauses 10.1 & 10.2. For NB-IoT satellite access, network-controlled handover between NB-IoT satellite access cells is not supported. Instead, intra-system (intra-frequency, inter-frequency) mobility is supported via cell selection and cell re-selection mechanisms. The cell re-selection shall be controlled by the network indirectly based on measurements and associated value thresholds signalled to the UE in system information broadcasted in the cell, for which cell re-selection shall be controlled. Intra-frequency neighbour (cell) measurements and inter-frequency neighbour (cell) measurements take the same definition as for eMTC satellite access. - Inter-system handover/mobility Inter-system handover is not supported. Instead, inter-system (intra-frequency, inter-frequency) mobility is supported in the form of NTN-TN mobility between IoT NTN cells and (terrestrial) TN cells via cell selection and cell re-selection mechanisms for a terminal supporting both IoT NTN and TN RITs. For a terminal supporting multiple component RITs, inter-system mobility between IoT NTN and other RITs is also possible (depending on terminal implementation) via cell selection between those RITs. |
| 8.2.5.1.5.2 | What are the handover interruption times for: – within the RIT (intra- and inter-frequency); – between various RITs within a SRIT; – between the RIT and another IMT system. For NR satellite access component RIT: The mobility interruption time includes the time required to execute any radio access network procedure, radio resource control signalling protocol, or other message exchanges between the user terminal and base station, as applicable to the candidate RIT/SRIT. For NR NTN, the mobility interruption time is evaluated without cell and satellite change for the beam mobility scenario. When moving within the same cell, the transmitting/receiving beam pair of the UE may need to be changed. For DL data transmission during UE mobility, gNB can configure different beams for this UE at different slots. It ensures appropriate transmit beam allocation to the UE for continuous DL transmission. Therefore, DL data packet transmission is kept during beam pair switching at different slots. For UL data transmission, PUSCH is sent using the beam configured by SRI (SRS resource indicator) by gNB. Accordingly, an appropriate gNB-side beam is selected for UL data reception. gNB may select different beams at different slots depending on the UE mobility. Therefore, UL data packet transmission is kept during beam pair switching at different slots. Based on the above analysis, the UE can keep exchanging user plane packets with gNB during the mobility transitions without cell and satellite change. Therefore, 0ms mobility interruption time is achieved by NR NTN for this scenario. For IoT NTN component RIT: For eMTC satellite access, allowed times for handover interruption in supported handover scenarios are defined in [36.133 section 5.5A.2.3.4 and section 5.5A.3.1.2]. The minimum time is bound by 20ms at the UE side, but this does not consider RAN-UE and UE-RAN signalling delays, and time for the UE to acquire necessary system information on the target cell. In addition, it does not consider intra-network delays such as those caused by the feeder link. These additional components will increase the interruption time. For NB-IoT satellite access, network-controlled handover does not apply, so interruption time is not provided here. |
| 8.2.5.1.6 | Radio resource management |
| 8.2.5.1.6.1 | Describe the radio resource management, support of: – centralized and/or distributed RRM; – dynamic and flexible radio resource management; – efficient load balancing. For NR satellite access component RIT: General NR satellite access performs radio resource management to ensure the efficient use of the available radio resource. RRM functions include: - Radio bearer control (RBC): the establishment, maintenance and release of radio bearer involves the configuration of radio resource. This is located in gNB. - Radio Admission Control (RAC): RAC is to admit or reject the establishment of new radio bearer. It considers QoS requirement, the priority level, overall resource situation. This is located in gNB. - Connection Mobility Control (CMC): it controls the number of UEs in idle mode and connected mode. In idle mode, cell reselection algorithm is controlled by parameter setting and in the connected mode, gNB controls UE mobility via handover and RRC connection release with redirection. Dynamic/flexible radio resource management NR satellite access supports dynamic and flexible radio resource management by packet scheduling that allocates and de-allocates resources to user and control plane packets. Load balancing (LB) Load balancing has the task to handle uneven distribution of the traffic load over multiple cells. The purpose of LB is thus to influence the load distribution for the higher resource utilization and QoS. LB is achieved in NR satellite access with hand-over, redirection or cell reselection. For IoT NTN component RIT: RRM mechanism in the following is supported in both IoT NTN component RIT commonly. General IoT NTN RIT performs radio resource management to ensure the efficient use of the available radio resource. RRM functions include: - Radio bearer control (RBC): the establishment, maintenance and release of radio bearer involves the configuration of radio resource. This is located in eNB. - Radio Admission Control (RAC): RAC is to admit or reject the establishment of new radio bearer. It considers QoS requirement, the priority level, overall resource situation. This is located in eNB. - Connection Mobility Control (CMC): it controls the number of UEs in idle mode and connected mode. In idle mode for eMTC, and in idle mode and connected mode for NB-IoT, cell reselection algorithm is controlled by parameter setting and in the connected mode for eMTC, eNB controls UE mobility via handover and RRC connection release with redirection. Dynamic/flexible radio resource management IoT NTN supports dynamic and flexible radio resource management by packet scheduling that allocates and de-allocates resources to user and control plane packets. Load balancing (LB) Load balancing has the task to handle uneven distribution of the traffic load over multiple cells. The purpose of LB is thus to influence the load distribution for the higher resource utilization and QoS. LB is achieved with hand-over, redirection or cell reselection. |
| 8.2.5.1.6.2 | Inter-RIT interworking Describe the functional blocks and mechanisms for interworking (such as a network architecture model) between heterogeneous RITs within a SRIT, if supported. Mobility between NTN and TN is supported: For both component RITs, NTN TN handover refers to mobility in both directions, i.e. from NTN to TN (hand-in) and from TN to NTN (hand-out). During mobility between NTN and Terrestrial Network (TN), a UE is not required to connect to both NTN and TN at the same time. |
| 8.2.5.1.6.3 | Connection/session management The mechanisms for connection/session management over the air-interface should be described. For example: – the support of multiple protocol states with fast and dynamic transitions; – the signalling schemes for allocating and releasing resources. For NR satellite access component RIT: NG-RAN support the following states: RRC_IDLE: - PLMN selection; - Broadcast of system information; - Cell re-selection mobility; - Paging for mobile terminated data is initiated by 5GC; - Paging for mobile terminated data area is managed by 5GC; - DRX for CN paging configured by NAS. - RRC_INACTIVE: - Broadcast of system information; - Cell re-selection mobility; - Paging is initiated by NG-RAN (RAN paging); - RAN-based notification area (RNA) is managed by NG- RAN; - DRX for RAN paging configured by NG-RAN; - 5GC - NG-RAN connection (both C/U-planes) is established for UE; - The UE AS context is stored in NG-RAN and the UE; - NG-RAN knows the RNA which the UE belongs to. - RRC_CONNECTED: - 5GC - NG-RAN connection (both C/U-planes) is established for UE; - The UE AS context is stored in NG-RAN and the UE; - NG-RAN knows the cell which the UE belongs to; - Transfer of unicast data to/from the UE; - Network controlled mobility including measurements. Transition between RRC states: - From RRC_IDLE to RRC_CONNECTED: RRC connection setup - From RRC_CONNECTED to RRC_IDLE: RRC connection release - From RRC_INACTIVE to RRC_CONNECTED: RRC connection resume - From RRC_CONNECTED to RRC_INACTIVE: RRC connection suspension - From RRC_INACTIVE to RRC_IDLE: RRC connection release - From RRC_IDLE to RRC_INACTIVE: not supported For more details, refer to [38.300]. For IoT NTN component RIT: RRC_IDLE and RRC_CONNECTED are supported. - RRC_IDLE: - PLMN selection; - Broadcast of system information; - Cell re-selection mobility; - Paging for mobile terminated data is initiated by EPC; - Paging for mobile terminated data area is managed by EPC; - DRX for CN paging configured by NAS. - RRC_CONNECTED: - The UE AS context is stored in E-UTRAN and the UE; - E-UTRAN knows the cell which the UE belongs to; - Transfer of unicast data to/from the UE; - For eMTC, network controlled mobility including measurements; - For NB-IoT, UE controlled mobility including measurements; Transition between RRC states: - From RRC_IDLE to RRC_CONNECTED: RRC connection setup - From RRC_CONNECTED to RRC_IDLE: RRC connection release For more details, refer to [36.300] |
| 8.2.5.1.7 | Frame structure |
| 8.2.5.1.7.1 | Describe the frame structure for downlink and uplink by providing sufficient information such as: – frame length; – the number of time slots per frame; – the number and position of switch points per frame for TDD; – guard time or the number of guard bits; – user payload information per time slot; – control channel structure and multiplexing; – power control bit rate. For NR satellite access component RIT: The frame structure related information is as follows: - Frame length, sub-carrier spacing, and time slots: One radio frame of length 10 ms consisting of 10 subframes, each of length 1 ms. Each subframe consists of an OFDM sub-carrier spacing dependent number of slots. Each slot consists of 14 OFDM symbols (twelve OFDM symbols in case of extended cyclic prefix) - 15 kHz SCS: 1 ms slot, 1 slot per sub-frame - 30 kHz SCS: 0.5 ms slot, 2 slots per sub-frame Data transmissions can be scheduled on a slot basis, as well as on a partial slot basis, where the partial slot transmissions that occur several times within one slot. The supported partial slot allocations and scheduling intervals are 2, 4 and 7 symbols for DL and 1-14 symbols for UL for normal cyclic prefix, and 2, 4 and 6 symbols for DL and 1-12 symbols for UL for extended cyclic prefix. - Downlink control channel structure: Downlink control signaling is time and frequency multiplexed with data on a scheduling interval basis. The control region can span over 1-3 OFDM symbols in the beginning of the allocation, flexibly allocating 1-14 symbols (at least 2 symbols for DL) for data transmission, including the time and frequency part of the control region that was not used for control signaling. - Uplink control channel structure: Uplink control signaling can be both time-multiplexed with the data of the same UE and time and frequency multiplexed with control and data of other UEs when the UE has no data to be transmitted. Uplink control signaling is piggy-backed with data i.e. transmitted with data on the PUSCH when the UE has data to be transmitted. - Power control bit rate: No specific power-control rate is defined, but a power control command can be sent at any slot, leading to a sub-carrier spacing specific maximum power control rate of 1/2/kHz for SCS of 15/30/kHz respectively. For IoT NTN component RIT: - Frame length, sub-carrier spacing, and time slots: FDD frame structure is supported. One radio frame of length 10 ms consisting of 10 subframes, each of length 1 ms. Each subframe consists of two slots, each of length 0.5 ms. Each slot consists of seven OFDM symbols (six OFDM symbols in case of extended cyclic prefix). For eMTC satellite access, a minimum time unit for transmission is a TTI, which has a duration of one subframe. Sub-carrier spacings of 15kHz is supported. For NB-IoT satellite access, the minimum time unit for transmission is a subframe in the downlink and a resource unit in the uplink. The length of a resource unit is dependent on the subcarrier spacing and number of subcarriers. Up to ten subframes or resource units can be assigned to the UE for one transmission. Sub-carrier spacings of 15kHz is supported for DL, and sub-carrier spacings of 3.75kHz and 15kHz is supported for UL (see item 8.2.5.1.2.1for more details). For eMTC satellite access half-duplex FDD operation and NB-IoT satellite access, a guard period of 1ms is used for the switching time. - Downlink control channel structure: Downlink control signalling and data transmission to the same UE are time multiplexed on different subframes. - Uplink control channel structure: Uplink control signalling is frequency-multiplexed with data for other UEs (no time separation) when the UE has no data to be transmitted. Uplink control signalling is typically transmitted at the edges of the overall system bandwidth. For eMTC satellite access, uplink control signalling is piggy-backed with data i.e. transmitted with data on the PUSCH when the UE has data to be transmitted. - Power control bit rate: There is no specific power-control rate. At most one power-control command per subframe, implying 1 kHz maximum power-control rate. For more details on the (uplink) power control, see [36.213] sub-clause 5.1. For NB-IoT satellite access, only open-loop power control is supported. |
| 8.2.5.1.8 | Spectrum capabilities and duplex technologies NOTE 1 – Parameters for both downlink and uplink should be described separately, if necessary. |
| 8.2.5.1.8.1 | Spectrum sharing and flexible spectrum use Does the RIT/SRIT support flexible spectrum use and/or spectrum sharing for the bands for IMT? Provide details. For NR satellite access component RIT: - NR satellite access supports flexible spectrum use of spectrum defined for NR NTN in item 8.2.5.1.8.3 through mechanisms including the following: - within one component carrier, bandwidth part (BWP) is supported on downlink and uplink. The bandwidth of the component carrier can be divided into several bandwidth parts. From network perspective, different bandwidth parts can be associated with different numerologies (subcarrier spacing, cyclic prefix). UEs with smaller bandwidth support capability can work within a bandwidth part with an associated numerology. By this means UEs with different bandwidth support capability can work on large bandwidth component carrier. NR satellite access supports UE bandwidth part adaptation for UE power saving and numerology switching. The network can operate on a wide bandwidth carrier while it is not required for the UE to support the whole bandwidth carrier, but can work over activated bandwidth parts, thereby optimizing the use of radio resources to the traffic demand and minimizing interference to/from other systems. - NR satellite access addresses the uplink and downlink traffic asymmetry with flexible spectrum resource allocation by allowing FDD operation on a paired spectrum, different transmission directions in either part of a paired spectrum. For IoT NTN component RIT: For eMTC satellite access, - flexible use of spectrum defined for IoT NTN in item 8.2.5.1.8.3 is supported by using one or multiple carriers from the network point of view, which can be either contiguous or non-contiguous in the frequency domain. For NB-IoT satellite access, - Flexible use of spectrum defined for IoT NTN in item 8.2.5.1.8.3 is supported by using one or multiple NB-IoT carriers. - A single, anchor, NB-IoT carrier of 180 kHz each for UL and DL in FDD is the minimum required spectrum. - Additional non-anchor NB-IoT carrier(s), each of 180 kHz can be associated to the same NB-IoT cell, and a UE uses either the anchor or one non-anchor NB-IoT carrier. - The anchor carrier and non-anchor carrier(s) can be either contiguous or non-contiguous in the frequency domain. Functionality to allow NR satellite access to be configured to co-exist with IoT NTN using frequency division multiplexing (FDM) manner has been defined. However, RF requirements for such coexistence within the SAN have not been defined. - For NB-IoT satellite access, - The downlink co-existence can be made by NR by configuring reserved resource blocks (RBs) which are declared as not available for PDSCH for NR users. These reserved resource blocks can be used by NB-IoT anchor and non-anchor carriers. For NR users that are scheduled on the resource block group (RBG) which includes the reserved RB, NR will configure the rate match pattern for those users using dynamic or semi-static indication. - For uplink, NR can use appropriate uplink resource allocation to “reserve” RBs for NB-IoT users. For example, if some of the RBs are reserved for NB-IoT, NR will allocate other RBs to its users, by either frequency domain resource allocation type 0 or type 1. By the above means, NR and NB-IoT can co-exist without any impact to each other. - For eMTC satellite access, - The downlink co-existence can be made by NR by configuring reserved resources for eMTC. It can be achieved by resource element (RE) level and RB level resource reservation indication. - Similar mechanism to NB-IoT satellite access can be employed for uplink to enable NR satellite access co-existence with eMTC satellite access for uplink. |

TABLE 2 (continued)

| Item | Item to be described |
| --- | --- |
| 8.2.5.1.8.2 | Channel bandwidth scalability Describe how the proposal supports channel bandwidth scalability, including the supported bandwidths. Describe whether the proposed RIT supports extensions for scalable bandwidths wider than 30 MHz. Consider, for example: – the scalability of operating bandwidths; – the scalability using single and/or multiple RF carriers. Describe multiple contiguous (or non-contiguous) band aggregation capabilities, if any. Consider for example the aggregation of multiple channels to support higher user bit rates. For NR satellite access component RIT: One component carrier supports a scalable bandwidth, 5, 10, 15, 20, 30MHz for frequency range 410 MHz to 7125 MHz (see [38.108] for the actual support of bandwidth for each band), with guard band ratio from 20% to 2%. For IoT NTN component RIT For NB-IoT satellite access, the channel bandwidth is 200kHz. The channel bandwidth is not scalable at the UE because it is tailored for mMTC-s where supporting high peak user data rates are less important than maintaining low device complexity. There is no aggregation support by the UE of multiple NB-IoT carriers – see item 8.2.5.1.8.1 for more details. However, the system capacity can be scaled up by network deployment of multiple NB-IoT satellite access RF carriers (200 kHz each), which enables to also scale up the user connection density and capacity. The NB-IoT non-anchor carrier concept, where a user can be re-directed from anchor to non-anchor carrier for data transfer in order to increase capacity, can also be used to improve capacity (not peak user data rate). In this case multiple contiguous RF carriers can be operated with nominal 180kHz spacing between them, so requiring less overall bandwidth compared to independent carriers of 200kHz spacing. For eMTC satellite access, the channel bandwidth is 1.4MHz. The channel bandwidth is not scalable at the UE because it is tailored for mMTC-s where supporting high peak user data rates are less important than maintaining low device complexity. There is no aggregation support by the UE of multiple carriers – see item 8.2.5.1.8.1 for more details. The system capacity can be scaled up by network deployment of multiple eMTC satellite access RF carriers (1.4 MHz each), which enables to also scale up the user connection density and capacity. The SRIT supports high peak user data rates via NR satellite access component RIT. |
| 8.2.5.1.8.3 | What are the frequency bands supported by the RIT? Please list. For NR satellite access component RIT: The following frequency bands are supported. Introduction of other ITU-R identified satellite bands are not precluded in the future. 3GPP technologies are also defined as appropriate to operate in other frequency arrangements and bands. 410 – 7125 MHz: Satellite operating band; Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high; Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high; Duplex mode. n256; 1980 MHz – 2010 MHz; 2170 MHz – 2200 MHz; FDD. n255; 1626.5 MHz – 1660.5 MHz; 1525 MHz – 1559 MHz; FDD. n254; 1610 MHz – 1626.5 MHz; 2483.5 MHz – 2500 MHz; FDD. Additional frequency bands can be introduced in the future in release independent manner. Support for frequency bands above 10 GHz is under study For IoT NTN component RIT: The following frequency bands are currently specified for IoT NTN, in accordance with spectrum requirements defined by Report ITU-R M.2514-0. Introduction of other ITU-R identified bands are not precluded in the future. 3GPP technologies are also defined as appropriate to operate in other frequency arrangements and bands. Detailed information on the following bands can be found in [36.102] sub-clause 5.2A for eMTC satellite access and sub-clause 5.2B for NB-IoT satellite access. These frequency bands and general RF requirements are defined in [36.108] and [36.102] Release 18 versions. However, they are implementable by Release 17 equipment as defined in [36.307]. 450 – 6000 MHz: E-UTRA Operating Band; Uplink (UL) operating band BS receive UE transmit; Downlink (DL) operating band BS transmit UE receive; Duplex Mode. FUL_low – FUL_high; FDL_low – FDL_high. 256; 1980 MHz – 2010 MHz; 2170 MHz – 2200 MHz; FDD. 255; 1626.5 MHz – 1660.5 MHz; 1525 MHz – 1559 MHz; FDD. 254; 1610 MHz – 1626.5 MHz; 2483.5 MHz – 2500 MHz; FDD. NOTE: Satellite bands are numbered in descending order from 256. Additional frequency bands can be introduced in the future in release independent manner. |
| 8.2.5.1.8.4 | What is the minimum amount of spectrum required to deploy a contiguous network, including guard bands (MHz)? For NR satellite access component RIT: The minimum amount of paired spectrum is 2 x 5 MHz. For IoT NTN component RIT: For eMTC satellite access, the minimum amount of paired spectrum is 2 x 1.4 MHz. For NB-IoT satellite access, the minimum amount of paired spectrum is 2 x 0.2 MHz. Regarding guard bands, when the UE is operating in an NGSO (non-Geo-Stationary Orbit) deployment, to support coexistence, it is assumed that the following has been accounted for as part of system deployment configuration by the operator: - a guardband at least equivalent to the maximum doppler shift expected for the NGSO constellation between the channel edge of the channel bandwidth operated by the UE and the edge of the spectrum block assigned to the operator. |
| 8.2.5.1.8.5 | What are the minimum and maximum transmission bandwidth (MHz) measured at the 3 dB down points? For NR satellite access component RIT: The 3 dB bandwidth is not part of the specifications, however: - The minimum 99% channel bandwidth (occupied bandwidth of single component carrier) is - 5 MHz for frequency range 410 – 7125 MHz; - The maximum 99% channel bandwidth (occupied bandwidth of single component carrier) is - 30 MHz for frequency range 410 – 7125 MHz; For IoT NTN component RIT: The 3 dB bandwidth is not part of the specifications, however: - For eMTC, the 99% channel bandwidth (occupied bandwidth of single carrier) is 1.4 MHz. - For NB-IoT satellite access, the 99% channel bandwidth is 0.2 MHz. |
| 8.2.5.1.8.6 | What duplexing scheme(s) is (are) described in this template? (e.g. TDD, FDD or half-duplex FDD). Describe details such as: – What is the minimum (up/down) frequency separation in case of full- and half-duplex FDD? – What is the requirement of transmit/receive isolation in case of full- a half-duplex FDD? Does the RIT require a duplexer in either the mobile station (MS) or space segment? – What is the minimum (up/down) time separation in case of TDD? – Whether the DL/UL Ratio variable for TDD? What is the DL/UL ratio supported? If the DL/UL ratio for TDD is variable, what would be the coexistence criteria for adjacent cells? For NR satellite access component RIT: NR satellite access supports paired spectrum and allows FDD operation on a paired spectrum, different transmission directions in either part of a paired spectrum, - For FDD operation, it supports full-duplex FDD. - For both base station and terminal, a duplexer is needed for full-duplex FDD. - For full-duplex FDD, the required transmit/receive isolation is a UE function of; the Tx emission mask (emission level on the Rx frequency) , the TX-Rx frequency spacing , the Tx- Rx duplex filter isolation, the TX and RX configuration (RB location, RB power and RB allocation) and the required Rx desense criteria. For the supported operating bands, the the minimum (up/down) Tx to Rx frequency separation is defined in the following Table: UE TX-RX frequency separation: NTN Satellite Operating Band; TX – RX carrier centre frequency separation. n256; 190 MHz. n255; -101.5 MHz. n254; 862 – 885 MHz. The spacing between carriers will depend on the deployment scenario, the size of the frequency block available and the Satellite access channel bandwidths. The nominal channel spacing between two adjacent Satellite access carriers is defined as following: - For operating bands with 100 kHz channel raster, Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2 - where BWChannel(1) and BWChannel(2) are the Satellite access channel bandwidths of the two respective Satellite access carriers. The channel spacing can be adjusted depending on the channel raster to optimize performance in a particular deployment scenario. - For different transmission directions in either part of a paired spectrum, a duplexer is needed for both base station and the terminal. The required frequency separation between the paired spectrum is the same as full-duplex FDD. For IoT NTN component RIT: IoT NTN supports paired spectrum and allows FDD operation on a paired spectrum. For eMTC satellite access FDD operation, both half-duplex and full-duplex FDD are supported. - For the base station (from here onwards referred to as the “Satellite Access Node” (SAN)), a duplexer is needed for half-duplex/full-duplex FDD. For the terminal, a duplexer is needed for full-duplex FDD only. - For full-duplex FDD, the required transmit/receive isolation is a UE function of; the Tx emission mask (emission level on the Rx frequency), the TX-Rx frequency spacing , the Tx- Rx duplex filter isolation, the TX and RX configuration (RB location, RB power and RB allocation) and the required Rx desense criteria. For the supported RIT operating bands, the following parameters are defined in [36.102]: - the minimum (up/down) Tx to Rx frequency separation - the minimum Tx-Rx band gap - For half-duplex FDD, for the UE there is no specified transmit / receive isolation due half duplex mode. For NB-IoT, Full Duplex FDD is supported by the SAN, and Half-duplex FDD is supported by the UE. The terminal does not need a duplexer, and there is no specified transmit / receive isolation due to half-duplex mode. |
| 8.2.5.1.9 | Support of advanced antenna capabilities |
| 8.2.5.1.9.1 | Fully describe the multi-antenna systems supported in the User segment, Space segment, or both that can be used and/or must be used; characterize their impacts on systems performance; e.g. does the RIT have the capability for the use of: – spatial multiplexing techniques; – space-time coding (STC) techniques; – beam-forming techniques (e.g. adaptive or switched). For NR satellite access component RIT: The multi-antenna systems in NR satellite access supports the following MIMO transmission schemes at both the UE and the base station: Beamforming is supported. Beam management with periodic and aperiodic beam refinement is also supported at the UE and at the base station. For IoT NTN component RIT The multi-antenna systems in eMTC satellite access and NB-IoT satellite access supports the following MIMO transmission scheme at both the UE and the SAN: - Spatial transmit diversity is supported based on space frequency block coding for eMTC satellite access and NB-IoT satellite access, and frequency switched transmit diversity for eMTC satellite access. Specification transparent diversity schemes are also supported by eNB implementations. |
| 8.2.5.1.9.2 | How many antennas are supported by the Space segment and MS for transmission and reception? Specify if correlated or uncorrelated antennas in co-polar or cross-polar configurations are used. What is the antenna spacing (in wavelengths)? For NR satellite access component RIT: NR satellite access supports {1, 2, 4, 8, 12, 16, 24, 32} antenna ports in the DL and {1, 2, 4} antenna ports in the UL. NTN Payload and UE support rectangular antenna arrays. The rectangular panel array antenna can be described by the following tuple (Mg,Ng, M,N, P), where Mg is the number of panels in a column, Ng is the number of panels in row, M,N are the number of vertical, horizontal antenna elements within a panel and P is number of polarizations per antenna element. The spacing in vertical and horizontal dimensions between the panels is specified by dg,V, dg,H and between antenna elements by dV, dH. NR satellite access specification is flexible to support various antenna spacing, number of antenna elements, antenna port layouts and antenna virtualization approaches. Satellite transmission and reception may also implement Circular polarization (RHCP and LHCP). For IoT NTN component RIT IoT NTN supports {1, 2, 4} antenna ports in the DL and {1} antenna port in the UL. SAN supports e.g. rectangular antenna arrays described by the tuple (M,N,P). In such configuration scenario, M, N are the number of antenna elements in the vertical and horizontal dimensions. P is the number of polarizations per antenna element. Antenna elements are uniformly spaced in the horizontal direction with a spacing of dH and in the vertical direction with a spacing of dV. Satellite transmission and reception may also implement Circular polarization (RHCP and LHCP). IoT NTN supports various antenna spacing, antenna port layouts and antenna virtualization approaches. |
| 8.2.5.1.9.3 | Provide details on the antenna configuration that is used in the self-evaluation. The information will be provided with self-evaluation results. |
| 8.2.5.1.9.4 | If spatial multiplexing (MIMO) is supported, does the proposal support (provide details if supported): – Single codeword (SCW) and/or multi-codeword (MCW). – Open and/or closed loop MIMO. – Cooperative MIMO. – Single-user MIMO and/or multi-user MIMO. For NR satellite access component RIT: MIMO is not supported. For IoT NTN component RIT: MIMO is not supported. |

TABLE 2 (continued)

| Item | Item to be described |
| --- | --- |
| 8.2.5.1.9.5 | Other antenna technologies Does the RIT/SRIT support other antenna technologies, for example Active Antenna System (AAS) If so, please describe. The use of Active Antenna System is supported by NR satellite access and IoT NTN. RF requirements for satellite access node (SAN) types and those associated to Active Antenna System are found in [38.108] for NR satellite access and [36.108] for IoT NTN. |
| 8.2.5.1.10 | Link adaptation and power control |
| 8.2.5.1.10.1 | Describe link adaptation techniques employed by RIT/SRIT, including: – the supported modulation and coding schemes; – the supporting channel quality measurements, the reporting of these measurements, their frequency and granularity; Provide details of any adaptive modulation and coding schemes, including: – hybrid ARQ or other retransmission mechanisms? – algorithms for adaptive modulation and coding, which are used in the self-evaluation; – other schemes? For NR satellite access component RIT: For data, the RIT supports dynamic indication of - combinations of modulation scheme and target code rate and, - the resource allocation in frequency and time (The resource allocation in frequency is within BWP) that the UE uses to determine the transport block size where the possible combinations cover a large range of possible data and channel coding rates. 28 different target coding rates can be indicated (29 if 256QAM is not enabled) and the target code rate range is 0.0293 to 0.896. In both downlink and uplink, link adaptation (selection of modulation scheme and code rate) is controlled by the base station. In the downlink, the network selection of modulation-scheme/code-rate combination can e.g. be based on channel state information (CSI) reported by the terminals. The RIT features a flexible CSI framework where the type of CSI, reporting quantity, frequency-granularity and time-domain behaviour can be configured. Both periodic and aperiodic(triggered) reporting modes are supported, controlled by the base station, where the aperiodic reporting allows the network to request which CSI-RS resources to report the CSI for. More details can be found in [38.214] section 5.2. In the uplink the base station may measure either the traffic channel or sounding reference signals and use this as input to the link adaptation. More details can be found in [38.214] section 6.2.1. On the MAC layer, hybrid ARQ with soft-combining between transmissions is supported. Different redundancy versions can be used for different transmissions. The modulation and coding scheme may be changed for retransmissions. In order to minimize delay and feedback, a set of parallel stop-and-wait protocols are used. For NR NTN operation the maximal supported HARQ process number is increased and it is up to 32 for both UL and DL. Further, in NTN, enabling and disabling HARQ feedback for downlink transmission can be semi-statically configurable per HARQ process via UE specific RRC signalling. To correct possible residual errors, the MAC ARQ is complemented by a robust selective-repeat ARQ protocol on the RLC layer. More details are found in [38.321] and [38.322]. For IoT NTN component RIT: For data, eMTC satellite access supports the modulation schemes as described in section 8.2.5.1.2.2.1, and code rates between approximately 0.1 and 0.9. Different modulation-scheme/code-rate combinations exist. This is valid for both downlink and uplink. In both downlink and uplink, link adaptation (selection of modulation scheme, code rate and repetition number) is controlled by the SAN. In the downlink the network selection of modulation-scheme/code-rate combination can e.g. be based on Channel Quality Indicators (CQIs) reported by the terminals. Several different CQI modes exist, including frequency-selective and wideband modes, periodic and aperiodic modes. The CQI mode is controlled by the SAN. In the uplink the SAN may measure either the traffic channel or sounding reference signals and use this as input to link adaptation. More details are found in [36.213]. UE reports its headroom to its maximum power to assist the SAN with uplink link adaptation. On the MAC layer, hybrid ARQ with soft-combining between transmissions is supported. Different redundancy versions can be used for different transmissions. The modulation scheme may be changed for retransmissions. In order to minimize delay and feedback, a set of parallel stop-and-wait protocols are used. To correct possible residual errors, the MAC HARQ is complemented by a robust selective-repeat ARQ protocol on the RLC layer. More details are found in [36.321] and [36.322]. For NB-IoT satellite access, π/2BPSK and QPSK modulation schemes are supported. Transmissions of a transport block can be mapped to between 1 and 10 subframes to adapt the code rate of the transmission. In its most basic form the link adaptation supports 116 alternative modulation-scheme/code-rate combinations for the UL and 104 alternatives for the DL. To further enhance the link robustness NB-IoT supports repetition-based transmission scheme using up to 2048 repetitions of each modulation-scheme/code-rate combination. During the connection setup procedure NB-IoT satellite access supports a basic UE feedback mechanism which allows the SAN to access the coupling loss experienced by a UE. In connected mode HARQ and ARQ RLC/MAC feedback is supported in similarity to eMTC satellite access. |
| 8.2.5.1.10.2 | Provide details of any power control scheme included in the proposal, for example: – power control step size (dB); – power control cycles per second; – power control dynamic range (dB); – minimum transmit power level with power control; – associated signalling and control messages. For NR satellite access component RIT: Uplink power control is independent for uplink data (PUSCH), uplink control (PUCCH) and sounding reference signal (SRS). The uplink power control is based on both signal-strength measurements done by the terminal itself (open-loop power control), as well as measurements by the base station. The latter measurements are used to generate power-control commands that are subsequently fed back to the terminals as part of the downlink control signalling (closed-loop power control). Both absolute and relative power-control commands are supported. There are four available relative power adjustments (“step size”) in case of relative power control. For uplink data, multiple closed loop power control processes can be configured, including the possibility separate processes with transmission beam indication. The time between power-control commands for PUSCH and PUCCH is the same as the scheduling periodicity for the PUSCH and the PDSCH, respectively. More details about uplink power control are found in [38.213] section 7. Downlink power control is network-implementation specific and thus outside the scope of the specification. A simple and efficient power control strategy is to transmit with a constant output power. Variations in channel conditions and interference levels are adapted to by means of scheduling and link adaptation. For IoT NTN component RIT: The RIT uplink power control is based on both signal-strength measurements done by the terminal itself (open-loop power control), as well as measurements by the SAN. The later measurements are used to generate power-control commands that are subsequently fed back to the terminals as part of the downlink control signalling (closed-loop power control). Both absolute and relative power-control commands are supported. The available relative power adjustments (“step size”) in case of relative power control are [-1 dB, 0 dB, +1 dB, +3 dB]. The time between power-control commands can be down to 1ms. The minimum transmit power requirement, – 40dBm, yields a dynamic range of -40 to 23=63dB for a terminal with maximum power 23dBm. For both eMTC satellite access and NB-IoT satellite access, maximum UE power supported are 20 dBm and 23 dBm. NB-IoT satellite access only supports the open-loop power control, with a constraint that full power is used when the UE is commanded to use 2 or more repetitions of a physical channel. More details about uplink power control are found in [36.213]. Downlink power control is, with the exception for NB-IoT satellite access, network-implementation specific and thus outside the scope of the specification. A simple and efficient power control strategy is to transmit with a constant output power. Variations in channel conditions and interference levels are adapted to by means of scheduling and link adaptation rather than with power control. |
| 8.2.5.1.11 | Power classes |
| 8.2.5.1.11.1 | Mobile station emitted power |
| 8.2.5.1.11.1.1 | What is the radiated antenna power measured at the antenna (dBm)? For NR satellite access component RIT: For frequency range 1, the maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The maximum output power is defined by UE power class as following table. <UE maximum output power for frequency range 1> NR satellite band; Class 3 (dBm); Tolerance (dB). n256; 23; ±2. n255; 23; ±2. n254; 23; ±2. For IoT NTN component RIT: The maximum output power less than or equal to Power Class is measured at antenna connector assuming a 0 dBi antenna gain with omni-direction for horizontal plain. - For eMTC satellite access and NB-IoT satellite access, UE power classes with the maximum output power of 23dBm and 20dBm are defined. |
| 8.2.5.1.11.1.2 | What is the maximum peak power transmitted while in active or busy state? See item 8.2.5.1.11.1.1. |
| 8.2.5.1.11.1.3 | What is the time averaged power transmitted while in active or busy state? Provide a detailed explanation used to calculate this time average power. For NR satellite access component RIT: Full power UE transmission has been predominantly assumed as representative of the power utilized by each UE to achieve the required performance. For IoT NTN component RIT: For the evaluation for IoT NTN, full power UE transmission has been predominantly assumed as representative of the power utilized by each UE to achieve the required performance, for both NB-IoT satellite access and eMTC satellite access. For eMTC satellite access HD-FDD transmission and NB-IoT satellite access transmission, the average UE transmission power can be scaled down by P + (10*log10(N/10)), where P is equal to the maximum UE Tx power, and N is the number of subframes in a radio frame allocated for uplink transmission for the given UE. |
| 8.2.5.1.11.2 | Space segment emitted power |
| 8.2.5.1.11.2.1 | What is the base station transmit power per RF carrier? For NR satellite access component RIT: For the BS type 1-H, the BS conducted output power is measured at TAB connector for BS type 1-H. For the BS type 1-O, radiated transmit power is defined as the EIRP level for a declared beam at a specific beam peak direction - For a declared beam and beam direction pair, the rated beam EIRP level is the maximum power that the base station is declared to radiate at the associated beam peak direction during the transmitter ON period. Base Stations intended for general-purpose applications do not have limits on the maximum transmit power. However, there may exist regional regulatory requirements which limit the maximum transmit power. For IoT NTN component RIT: The SAN transmit power of one carrier is the mean power delivered to a load with resistance equal to the nominal load impedance of the transmitter. The SAN maximum transmit power of one carrier is the mean power level measured at the SAN antenna connector in a specified reference condition. SANs intended for general-purpose applications do not have limits on the maximum transmit power. However, there may exist regional regulatory requirements which limit the maximum transmit power. |
| 8.2.5.1.11.2.2 | What is the maximum peak transmitted power per RF carrier radiated from antenna? SANs intended for general-purpose applications do not have limits on the maximum transmit power. However, there may exist regional regulatory requirements which limit the maximum transmit power. |
| 8.2.5.1.11.2.3 | What is the average transmitted power per RF carrier radiated from antenna? The averaged transmitted carrier power is subject to the type of signal/channel to be transmitted, bandwidth, and deployment scenario, etc. |
| 8.2.5.1.12 | Scheduler, QoS support and management, data services |
| 8.2.5.1.12.1 | QoS support – What QoS classes are supported? – How QoS classes associated with each service flow can be negotiated. – QoS attributes, for example: – data rate (ranging from the lowest supported data rate to maximum data rate supported by the MAC/PHY); – control plane and user plane latency (delivery delay); – packet error rate (after all corrections provided by the MAC/PHY layers), and delay variation (jitter). – Is QoS supported when handing off between radio access networks? Please describe. – How users may utilize several applications with differing QoS requirements at the same time. For NR satellite access component RIT: In NR satellite access, QoS model is based on QoS Flows, and both GBR QoS Flows and non-GBR QoS Flows are supported. At NAS level, the QoS flow is the finest granularity of QoS differentiation in a PDU session. Each QoS Flow is associated with a QoS profile which contains QoS parameters including a 5G QoS Identifier (5QI), an Allocation/ Retention Priority (ARP), Reflective QoS Attribute (RQA) for non-GBR Flows, Guaranteed Flow Bit Rate (GFBR) and Maximum Flow Bit Rate (MFBR) for GBR QoS Flows, and optionally with Notification Control and Maximum Packet Loss Rate for GBR QoS Flows. The 5QI is an index representing the resource type, priority, packet delay budget, packet error rate, maximum data burst volume, and averaging window of a QoS Flow. 3GPP Release 17 has introduced a new 5QI (5QI = 10) with a packet delay budget (PDB) going up to 1100ms. This large PDB value is defined to support non-guaranteed bit rate (non-GBR) services provisioned via satellite that can accommodate the worst case latencies when GEO satellite are used. For each UE, one or multiple PDU sessions can be established, and within one PDU session, up to 64 QoS Flows can be allocated. At AS level, for each UE, one or multiple data bearers can be established, and QoS Flow to data bearer mapping is controlled by NG-RAN. Up to 29 data bearers can be established in parallel for a UE. One or more QoS flows can be mapped to a data bearer. Reflective mapping (UE applies the DL mapping rule to UL packets) is supported in both NAS level and AS level. QoS profile is provided by 5GC to NG-RAN and is used by NG-RAN to determine the treatment on the radio interface. The ARP as well as other QoS parameters could be used to determine which bearers to prioritise at handover. By using multiple QoS Flows / data bearers having different QoS profiles, multiple application flows with different QoS requirements could be accommodated. For IoT NTN component RIT: In IoT NTN, a bearer is the level of granularity for QoS control. Up to 15 data bearers can be established in parallel for a UE. Each bearer is associated with a QoS class index (QCI), and an Allocation/ Retention Priority (ARP), and optionally with guaranteed bitrate (GBR) and maximum bit rate (MBR). The QCI is an index representing the priority, allowable delay, and packet error rate of a bearer, and up to 256 QCIs could be defined by the operator (21 of which is standardised). The QCI, MBR, GBR and ARP are signalled from the CN to the RAN when the bearer is established or modified, so that the scheduler in the RAN could ensure the QoS for each bearer. The ARP as well as other QoS parameters could be used to determine which bearers to prioritise at handover. By using multiple bearers having different QoS profiles, multiple application flows with different QoS requirements could be accommodated. |

TABLE 2 (continued)

| Item | Item to be described |
| --- | --- |
| 8.2.5.1.12.2 | Scheduling mechanisms – Exemplify scheduling algorithm(s) that may be used for full buffer in the technology proposal for evaluation purposes. Describe any measurements and/or reporting required for scheduling. For NR satellite access component RIT: In NR satellite access physical control and shared channels can be separately and dynamically scheduled for both uplink and downlink. A scheduling unit for downlink shared channel may span from 2-14 symbols and for uplink shared channel from 1-14 symbols (14 symbols comprise a “slot”). Sub-carrier spacing for different physical channels may be dynamically changed by switching bandwidth-parts (BWP). Typically, NR satellite access scheduling is based on the instantaneous radio-link quality as seen by the different users, and the traffic demand and quality-of-service requirements of individual users and in the cell as a whole. The former is based on CQI reports from the terminals (downlink) or measurements of sounding signals from the terminals (uplink). Based on this the base station may e.g. apply a proportional fair scheduling algorithm. The QoS assessment is supported by means of receiving QoS information from the “higher layers”. For non-full buffer traffic like VOIP (or any traffic having similar characteristics) semi-persistent scheduling in DL can be applied, by which a user can be allocated time-frequency resources in a semi-persistent manner, i.e., fixed resources are allocated at certain intervals without L1/L2 control signaling each time. This is especially useful to reduce the L1/L2 control signaling overhead and to increase VoIP capacity. In addition, with UL Configured Grants, the scheduler can allocate uplink resources to users. When a configured uplink grant is active, if the user cannot find an uplink grant assigned via downlink control channel an uplink transmission according to the configured uplink grant can be made. Otherwise, if the user finds an uplink grant assigned via downlink control channel, this assignment overrides the configured uplink grant. NR satellite access supports slot aggregation in downlink and uplink, by which time-frequency resources can be allocated consecutively to a user for a longer period than a slot by a single L1/L2 control signaling. A larger transport block size or a lower coding rate can be supported by this technique. This is especially useful when the coverage needs to be extended. As another option to extend coverage or improve reliability in addition to slot aggregation, a set of MCS tables supporting very low code rate for both DL and UL can be used. For the downlink and the uplink, intercell-interference coordination can be realized by the scheduler that is transparent to the physical layer. For IoT NTN component RIT: Dynamic scheduling on a 1 ms (subframe) basis is applied to both uplink and downlink. Typically, IoT NTN scheduling is based on the instantaneous radio-link quality as seen by the different users, and the traffic demand and quality-of-service requirements of individual users and in the cell as a whole. The former is based on CQI reports from the terminals (downlink) or measurements of sounding signals from the terminals (uplink). Based on this the SAN may e.g. apply a proportional fair scheduling algorithm. The QoS assessment is supported by means of receiving QoS information from the “higher layers”. Moreover, the RIT supports repetitions / TTI bundling, by which time-frequency resources can be allocated consecutively to a user for a longer period than 1 ms by a single L1/L2 control signaling. A larger transport block size or a lower coding rate can be supported by this technique. This is especially useful when the coverage needs to be extended. Intercell-interference coordination mechanisms may also be realized by the scheduler. To aid inter-cell coordination, IoT NTN defines two indicators exchanged between SANs: The High-interference Indicator (HI) provides information to neighboring cells about the part of the cell bandwidth upon which the cell intends to schedule its cell-edge users. The Overload Indicator (OI) provides information on the uplink interference level experienced in each part of the cell bandwidth. For the downlink, intercell-interference coordination can be realized using a Relative Narrowband TX Power (RNTP) indicator. For NB-IoT satellite access, the scheduler controls the transmission duration of control channels in number of subframes in a semi-static fashion while the transmission duration of shared channels can be varied dynamically. This is beneficial for extending coverage. See [36.423] for details. |
| 8.2.5.1.13 | Radio interface architecture and protocol stack |
| 8.2.5.1.13.1 | Describe details of the radio interface architecture and protocol stack such as: Logical channels – Control channels. – Traffic channels. Transport channels and/or physical channels. For NR satellite access component RIT: This RIT contains NR satellite access standalone architecture. The following paragraphs provide a high level summary of radio interface protocols and channels. Radio Protocols: The protocol stack for the user plane includes the following: SDAP, PDCP, RLC, MAC, and PHY sublayers (terminated in UE and gNB). On the Control plane, the following protocols are defined: - RRC, PDCP, RLC, MAC and PHY sublayers (terminated in UE and gNB); - NAS protocol (terminated in UE and AMF) For detils on protocol services and functions, please refer to 3GPP specifications (e.g. [38.300]). Radio Channels (Physical, Transport and Logical Channels) The physical layer offers service to the MAC sublayer transport channels. The MAC sublayer offers service to the RLC sublayer logical channels. The RLC sublayer offers service to the PDCP sublayer RLC channels. The PDCP sublayer offers service to the SDAP and RRC sublayer radio bearers: data radio bearers (DRB) for user plane data and signalling radio bearers (SRB) for control plane data. The SDAP sublayer offers 5GC QoS flows and DRBs mapping function. The physical channels defined in the downlink are: - the Physical Downlink Shared Channel (PDSCH), - the Physical Downlink Control Channel (PDCCH), - the Physical Broadcast Channel (PBCH), The physical channels defined in the uplink are: - the Physical Random Access Channel (PRACH), - the Physical Uplink Shared Channel (PUSCH), - and the Physical Uplink Control Channel (PUCCH). In addition to the physical channels above, PHY layer signals are defined, which can be reference signals, primary and secondary synchronization signals. The following transport channels, and their mapping to PHY channels, are defined: Uplink: - Uplink Shared Channel (UL-SCH), mapped to PUSCH - Random Access Channel (RACH), mapped to PRACH Downlink: - Downlink Shared Channel (DL-SCH), mapped to PDSCH - Broadcast channel (BCH), mapped to PBCH - Paging channel (PCH), mapped to PDSCH Logical channels are classified into two groups: Control Channels and Traffic Channels. Control channels: - Broadcast Control Channel (BCCH): a downlink channel for broadcasting system control information. - Paging Control Channel (PCCH): a downlink channel that transfers paging information and system information change notifications. - Common Control Channel (CCCH): channel for transmitting control information between UEs and network. - Dedicated Control Channel (DCCH): a point-to-point bi-directional channel that transmits dedicated control information between a UE and the network. Traffic channels: Dedicated Traffic Channel (DTCH), which can exist in both UL and DL. In Downlink, the following connections between logical channels and transport channels exist: - BCCH can be mapped to BCH, or DL-SCH; - PCCH can be mapped to PCH; - CCCH, DCCH, DTCH can be mapped to DL-SCH; In Uplink, the following connections between logical channels and transport channels exist: - CCCH,DCCH, DTCH can be mapped to UL-SCH. Other aspects - NR satellite access QoS architecture The QoS architecture in NG-RAN (connected to 5GC), can be summarized as follows: For each UE, 5GC establishes one or more PDU Sessions. For each UE, the NG-RAN establishes one or more Data Radio Bearers (DRB) per PDU Session. The NG-RAN maps packets belonging to different PDU sessions to different DRBs. Hence, the NG-RAN establishes at least one default DRB for each PDU Session. NAS level packet filters in the UE and in the 5GC associate UL and DL packets with QoS Flows. AS-level mapping rules in the UE and in the NG-RAN associate UL and DL QoS Flows with DRBs For more details on NR satellite access Radio Protocol architecture and channels, refer to: [38.300], [38.401], [38.201], IoT NTN component RIT: eMTC/NB-IoT satellite access uses optimized physical layer and radio procedures (e.g. for very low power consumption), thus a number of E-UTRA protocol mechanisms and functions are either not used, or are specific to eMTC/NB-IOT satellite access only. Some examples of functionalities not specified for NB-IoT satellite access are: inter-RAT mobility, handover and measurement reports. Only few exceptions/variations are highlighted below; for details please refer to 3GPP specifications. Radio Protocol stack The protocol stack for the user plane includes PDCP, RLC, MAC, and PHY sublayers (terminated in UE and eNB). For NB-IoT, the user plane is not used when transferring user data over NAS. On the Control plane, the following protocols are defined: - RRC, PDCP, RLC, MAC and PHY sublayers (terminated in UE and eNB); - NAS protocol (terminated in UE and Core Network) For NB-IoT, if certain optimizations are supported, PDCP can be bypassed (at all, or until AS security is activated) For details on protocol services and functions, please refer to 3GPP specifications (e.g. TS 36.300). Radio Channels (Physical, Transport and Logical Channels) eMTC specific physical channels are: - MTC physical downlink control channel (MPDCCH) - Resynchronization Signal (RSS) - Physical broadcast channel (PBCH) - Carries the BCH transport channel - Physical downlink shared channel (PDSCH) - Carries the DL-SCH and PCH transport channels - Physical uplink control channel (PUCCH) - Physical uplink shared channel (PUSCH) - Carries the UL-SCH transport channel - Physical random access channel (PRACH) - Carries the RACH transport channel NB-IoT specific physical channels are: - Narrowband Physical broadcast channel (NPBCH) - Narrowband Physical downlink shared channel (NPDSCH) - Narrowband Physical downlink control channel (NPDCCH) - Narrowband Physical uplink shared channel Format 1 (NPUSCH F1) - Narrowband Physical uplink shared channel Format 2 (NPUSCH F2) - Narrowband Physical random access channel (NPRACH) In addition to the above channels, two types of physical signals are defined: reference or synchronization signals. E-UTRA Logical channels (at MAC/RLC sublayer) are: - Control Channels (for the transfer of control plane information), e.g.: - Broadcast Control Channel (BCCH) - For eMTC, Bandwidth Reduced Broadcast Control Channel (BR-BCCH) - Paging Control Channel (PCCH) - Common Control Channel (CCCH) - Dedicated Control Channel (DCCH) - Traffic Channels (for the transfer of user plane information), e.g.. - Dedicated Traffic Channel (DTCH) The following mapping between logical channels and transport channels is defined: In Uplink, CCCH, DCCH and DTCH can be mapped to UL-SCH; In Downlink, - BCCH can be mapped to BCH, or DL-SCH; - BR-BCCH can be mapped to DL-SCH; - PCCH can be mapped to PCH; - CCCH, DCCH and DTCH can be mapped to DL-SCH Other Aspects - NB-IOT satellite access Only a specific multi-carrier operation is defined (e.g. a RRC_CONNECTED UE can be configured to a non-anchor carrier, for all unicast transmissions). For more details on IoT NTN Radio Protocol architecture and channels, refer to: [36.300], [36.401], [36.201] |
| 8.2.5.1.13.2 | What is the bit rate required for transmitting feedback information? For NR satellite access component RIT: As described in other sections (e.g. 8.2.5.1.3, 8.2.5.1.10, 8.2.5.1.13.1), from a Layer1 point of view (PHY/MAC), few control (feedback/HARQ) channels are defined (in UL and DL), with specific characteristics and transmission schemes/rates. At Layer2 level (i.e. RLC ARQ), assuming an RLC AM Status report is sent every 50 ms (configurable), with a size of few octets, e.g. 32 bits (including RLC/MAC header overhead), this results in a rate of 32/0.05= 640 bit/s. For IoT NTN component RIT: As described in other sections (e.g. 8.2.5.1.3, 8.2.5.1.10, 8.2.5.1.13.1), from a Layer1 point of view (PHY/MAC), few control (feedback/HARQ) channels are defined (in UL and DL), with specific characteristics and transmission schemes/rates. At Layer2 level (i.e. RLC ARQ), assuming an RLC AM Status report is sent every 50 ms (configurable), with a size of few octets, e.g. 32 bits (including RLC/MAC header overhead), this results in a rate of 32/0.05= 640 bit/s. |
| 8.2.5.1.13.3 | Channel access Describe in detail how RIT/SRIT accomplishes initial channel access, (e.g. contention or non-contention based). For NR satellite access component RIT: Initial channel access is typically accomplished via the “random access procedure” (assuming no dedicated/scheduled resources are allocated). The random access procedure can be contention based (e.g. at initial connection from idle mode) or non-contention based (e.g. during Handover to a new cell). Random access resources and parameters are configured by the network and signalled to the UE (via broadcast or dedicated signaling). Contention based random access procedure encompasses the transmission of a random access preamble by the UE (subject to possible contention with other UEs), followed by a random access response (RAR) in DL (including allocating specific radio resources for the uplink transmission). Afterwards, the UE transmits the initial UL message (e.g. RRC connection Request) using the allocated resources, and wait for a contention resolution message in DL (to confirming access to that UE). The UE could perform multiple attempts until it is successful in accessing the channel or until a timer (supervising the procedure) elapses. Non-contention based random access procedure foresees the assignment of a dedicated random access resource/preamble to a UE (e.g. part of an HO command). This avoids the contention resolution phase, i.e. only the random access preamble and random access response messages are needed to get channel access. From a L1 perspective, a random access preamble is transmitted (UL) in a PRACH, random access response (DL) in a PDSCH, UL transmission in a PUSCH, and contention resolution message (DL) in a PDSCH. At the physical layer, 3GPP specified enhancements to meet the challenges related to large round trip delay, timing drift and Doppler shift/variation inherent to the satellite radio links: - For uplink time and frequency synchronization, NTN UE should support uplink time and frequency pre-compensation: o The UE shall calculate UE specific TA (corresponding to the round trip time or RTT on the service link) based on its GNSS-acquired position and the serving satellite ephemeris. o It shall calculate common TA (corresponding to any common delay e.g. RTT on the feeder link) according to the parameters provided by the network (UE considers common TA as 0 if the parameters are not provided) o The UE performs the pre-compensation of the calculated TA in its uplink transmissions in initial access and when connected. o For TA update in RRC_CONNECTED state, support of combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops o Support of frequency pre-compensation to counter shift the Doppler experienced on the service link. To calculate the frequency pre-compensation, the UE calculates the Doppler based on its GNSS-acquired position and the serving satellite ephemeris. For more details, refer to:[38.300], [38.321] and [38.213] For IoT NTN component RIT Initial channel access is typically accomplished via the “random access procedure” (assuming no dedicated/scheduled resources are allocated). The random access procedure can be contention based (e.g. at initial connection from idle mode) or non-contention based (e.g. for eMTC satellite access, during Handover to a new cell). Random access resources and parameters are configured by the network and signalled to the UE (via broadcast or dedicated signaling). Contention based random access procedure encompasses the transmission of a random access preamble by the UE (subject to possible contention with other UEs), followed by a random access response (RAR) in DL (including allocating specific radio resources for the uplink transmission). Afterwards, the UE transmits the initial UL message (e.g. RRC connection Request) using the allocated resources, and wait for a contention resolution message in DL (to confirming access to that UE). The UE could perform multiple attempts until it is successful in accessing the channel or until a timer (supervising the procedure) elapses. Non-contention based random access procedure foresees the assignment of a dedicated random access resource/preamble to a UE (e.g. for eMTC satellite access, part of a HO command). This avoids the contention resolution phase, i.e. only the random access preamble and random access response messages are needed to get channel access. From a L1 perspective, a random access preamble is transmitted (UL) in a PRACH, random access response (DL) in a PDSCH, UL transmission in a PUSCH, and contention resolution message (DL) in a PDSCH. Other/specific RIT aspects Detailed aspects and mechanisms of the random access procedure are specific/different for each RIT (NR and IoT NTN), e.g. physical resources/channels, timing, messages/parameters, etc. Pre-compensation as described in 8.2.5.1.3 of this document shall be performed by the UE prior to initial access. For eMTC/NB-IoT satellite access, there are also specific differences, e.g. - For eMTC: PRACH configuration/repetition, decoding/interpretation of Random Access Response (RAR), etc. - For NB-IoT: dedicated NPRACH channel, configuration, RAR decoding, etc. For more details, refer to [36.300], [36.321] and [36.213]. - |
| 8.2.5.1.14 | Beam selection |
| 8.2.5.1.14.1 | Describe in detail how the RIT/SRIT accomplishes cell selection to determine the serving cell for the users. For NR satellite access component RIT: Cell selection is based on the following principles: - The UE NAS layer identifies a selected PLMN (and equivalent PLMNs, if any); - For NR satellite access, a UE with location capability should use its awareness of its location to select a PLMN that is allowed to operate in the UE location as specified in TS 23.122. In order to ensure that the regulatory requirements are met, the network may be configured to enforce this UE choice by verifying the UE location Further, the following enhancements have been specified: - New “NG-RAN satellite” RAT type in USIM - Extension of the NAS supervision timers over satellite access for GEO and MEO RAT types (LEO uses legacy timers) - Modification of the higher priority PLMN selection procedure to include shared MCC 9xx - New minimum periodic search timer for higher priority PLMN search over satellite access when PLMN uses shared MCC - New trigger for PLMN selection upon transition in/out international areas (based on UE implementation) - New Forbidden List of PLMNs not allowed to operate at UE location and its handling - New 5GMM cause value#78 and its handling (related to the list in the previous bullet) - Support for multiple TACs for the same PLMN broadcast in the radio cell, including corresponding logic for determining the “Current TAI” and impact on ME-USIM procedures - The UE searches the supported frequency bands (RIT specific) and for each carrier frequency it searches and identifies the strongest cell. It reads cell broadcast information to identify its PLMN(s) and other relevant parameters (e.g. related to cell restrictions); - The UE seeks to identify a suitable cell; if it is not able to identify a “suitable” cell it seeks to identify an “acceptable” cell. - A cell is “suitable” if: the measured cell attributes satisfy the cell selection criteria (based on DL radio signal strength/quality); the cell belongs to the selected/equivalent PLMN; cell is not restricted (e.g. cell is not barred/reserved or part of "forbidden" roaming areas); - An “acceptable” cell is one for which the measured cell attributes satisfy the cell selection criteria and the cell is not barred. Among the identified suitable (or acceptable) cells, the UE selects the strongest cell, (technically it “camps” on that cell). As signalled/configured by the radio network, certain frequencies or RITs could be prioritized for camping. For NR satellite access, the following mobility in RRC_IDLE and RRC_INACTIVE enhancements have been specfied: o Cell (re)selection enhancements in the quasi-earth fixed cell scenario: UE can perform time-based and location-based measurement. The timing and location information associated to a cell are provided via system information. They refer respectively to the time when the serving cell is going to stop serving a geographical area and to the reference location and a distance threshold of serving cell. o Location assisted cell reselection, with the distance between UE and the reference location of the cell (serving cell and/or neighbor cell) taken into account, is supported for quasi-earth fixed cell o Measurement rules for cell re-selection with timing information and location information are specified in TS 38.304 For more details, refer to:[38.300] sub-clause 9.2.1.1 and [38.304] sub-clause 5.2 For IoT NTN component RIT: Cell selection is based on the following principles: - The UE NAS layer identifies a selected PLMN (and equivalent PLMNs, if any); - The UE searches the supported frequency bands (RIT specific) and for each carrier frequency it searches and identifies the strongest cell. It reads cell broadcast information to identify its PLMN(s) and other relevant parameters (e.g. related to cell restrictions); - The UE seeks to identify a suitable cell; if it is not able to identify a “suitable” cell it seeks to identify an “acceptable” cell. - A cell is “suitable” if: the measured cell attributes satisfy the cell selection criteria (based on DL radio signal strength/quality); the cell belongs to the selected/equivalent PLMN; cell is not restricted (e.g. cell is not barred/reserved or part of "forbidden" roaming areas); - An “acceptable” cell is one for which the measured cell attributes satisfy the cell selection criteria and the cell is not barred. Among the identified suitable (or acceptable) cells, the UE selects the strongest cell, (technically it “camps” on that cell). As signalled/configured by the radio network, certain frequencies or RITs could be prioritized for camping. Other/specific RIT aspects Detailed aspects and mechanisms of cell selection are specific/different for each RIT (NR and IoT NTN), e.g.: - frequency/cell search (using different DL sync signals and search procedures) - broadcast system information acquisition/signalling - RF measurement and metrics/thresholds for selection criteria Few specific mechanisms/restrictions apply to eMTC/NB-IoT, e.g.: - NB-IoT and eMTC uses specific DL signals and (optimized/limited) cell search and measurement procedures - Specific cell selection criteria may apply to both eMTC and NB-IoT satellite access such as cell “footprint information” to aid cell selection/re-selection. For more details, refer to [36.300] sub-clause 10.1.1.1 and [36.304] sub-clause 5.2. |
| 8.2.5.1.15 | Location determination mechanisms |
| 8.2.5.1.15.1 | Describe any location determination mechanisms that may be used, e.g. to support location-based services. For NR satellite access component RIT: Upon network request, after AS security is established in connected mode, a UE should report its coarse UE location information (most significant bits of the GNSS coordinates, ensuring an accuracy in the order of 2 km) to the NG-RAN if available. UE location verification based on multi-RTT with single satellite in view in supported: The RTT measurements (corresponding to the round trip time on the service link) can be repeated in different instants, and from different positions on the satellite orbit. For RTT determination, the following UE and gNB measurements specified in [38.215] are reported to the LMF: gNB receive-transmit time difference at the uplink time synchronization reference point, UE receive-transmit time difference, UE receive-transmit time difference subframe offset and DL timing drift. The assistance information reported to the CN may include ephemeris information including accurate satellite position and velocity at the time of multi-RTT measurement, and common TA parameters (ta-Common, ta-CommonDrift, ta-CommonDriftVariant, Epoch time). For IoT NTN component RIT: GNSS is supported by the UE for self-determination of location. |
| 8.2.5.1.16 | Priority access mechanisms |
| 8.2.5.1.16.1 | Describe techniques employed to support prioritization of access to radio or network resources for specific services or specific users (e.g. to allow access by emergency services). For NR satellite access component RIT: NR Satellite access supports overload and access control functionality such as RACH back off, RRC Connection Reject, RRC Connection Release and UE based access barring mechanisms. One unified access control framework as specified in [3GPP TS 22.261 section 6.22] is applied for NR satellite access. For each access attempt one Access Category and one or more Access Identities are selected. NR satellite access broadcasts barring control information associated with Access Categories and Access Identities and the UE determines whether an identified access attempt is authorized or not, based on the broadcasted barring information and the selected Access Category and Access Identities. In the case of multiple core networks sharing the same RAN, the RAN provides broadcasted barring control information for each PLMN individually. The unified access control framework is applicable to all UE states (RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state). For NAS triggered requests, the UE NAS determines one access category and access identity(ies) for the given access attempt and provides them to RRC for access control check. The RRC performs access barring check based on the access control information and the determined access category and access identities. The RRC indicates whether the access attempt is allowed or not to NAS layer. The NAS also performs the mapping of the access category and access identity(ies) associated with the access attempt to establishment cause and provides the establishment cause to RRC for inclusion in connection request to enable the gNB to decide whether to reject the request. For AS triggered request (i.e. RNA update), the RRC determines the resume cause value and the corresponding access category. For IoT NTN component RIT: IoT NTN RIT defines a range of access control mechanisms described in [3GPP TS 22.011 section 4] that can be used to restrict UEs’ access attempts to the system. These mechanisms are detailed in the bulleted list below. In addition, the IoT NTN specification defines mechanisms for the network to prioritize access attempts after the overall access control mechanism defined for the UEs has passed and the UE has initiated RRC connection establishment, at which point the network can allocate relative priority to the request relative to other requests based on the provided RRC Establishment cause. The specified mechanisms that can be used to restrict UEs’ access attempts to the system defined in different 3GPP releases: - Access Class Barring (ACB) If the UE is a member of at least one Access Class which corresponds to the permitted classes broadcast in the system information, and the Access Class is applicable in the serving network, access attempts are allowed. Additionally, in the case of the access network being UTRAN the serving network can indicate that UEs are allowed to respond to paging and perform location registration even if their access class is not permitted. Otherwise access attempts are not allowed. Any number of these classes may be barred at any one time, and in case of multiple core networks sharing the same access network, the access network is able to apply Access Class Barring for the different core networks individually. The network operator can take the network load into account when allowing UEs access to the network. Access Classes are applicable as follows: Classes 0 - 9 - Home and Visited PLMNs; Class 10 - This bit’s presence in the access class barring information broadcast to the cell indicates whether Emergency Calls are allowed for UEs with access classes 0 to 9 and UEs without an IMSI. For UEs with access classes 11 to 15, Emergency Calls are not allowed if both "Access class 10" and the relevant Access Class (11 to 15) are barred. Classes 11 and 15 - Home PLMN only if the EHPLMN list is not present or any EHPLMN; Classes 12, 13, 14 - Home PLMN and visited PLMNs of home country only. For this purpose the home country is defined as the country of the MCC part of the IMSI. - Coverage Enhancement level-based access class barring Supports barring of eMTC and NB-IoT devices in specific coverage enhancement levels The following mechanisms are only applicable to eMTC satellite access: - Service Specific Access Control (SSAC) SSAC provides a mechanism to minimize service availability degradation (i.e. radio resource shortage) due to the mass simultaneous mobile originating session requests and maximize the availability of the wireless access resources for non-barred services by applying independent access control for telephony services (MMTEL) for mobile originating session requests from idle-mode and connected-mode. EPS provides a capability to assign a service probability factor and mean duration of access control for each of MMTEL voice and MMTEL video by broadcasting system information parameters for: - assigning a barring rate (percentage) commonly applicable for Access Classes 0-9, - assigning a flag barring status (barred /unbarred) for each Access Class in the range 11-15, - SSAC does not apply to Access Class 10. - Extended Access Class Barring (EAB) EAB is a mechanism for controlling Mobile Originating access attempts from UEs that are configured for EAB in order to prevent overload of the access network and/or the core network. In congestion situations, the operator can restrict access from UEs configured for EAB while permitting access from other UEs. UEs configured for EAB are considered more tolerant to access restrictions than other UEs. When an operator determines that it is appropriate to apply EAB, the network broadcasts system information parameters for EAB UEs. - Overriding extended access barring Overriding Extended Access Barring is a mechanism for the operator to allow UEs that are configured for EAB to access the network under EAB conditions. The UE configured to override extended access class barring overrides EAB restrictions as long as it has an active PDN connection for which EAB has been configured to not apply. - Application Specific Congestion Control for Data Communication (ACDC) ACDC is an access control mechanism for the operator to allow/prevent new access attempts from particular, operator-identified applications in the UE in idle mode. ACDC does not apply to UEs in connected mode. For Access Control based on ACDC categories, at subscription at least four ACDC categories are allocated to the subscriber and stored in the ACDC MO or USIM. The network can prevent/mitigate overload of the access network and/or the core network. ACDC related parameterization is provided with system information broadcast. |
| 8.2.5.1.17 | Unicast, multicast and broadcast |
| 8.2.5.1.17.1 | Describe how the RIT enables: – broadcast capabilities; – multicast capabilities; – unicast capabilities; using both dedicated carriers and/or shared carriers. Please describe how all three capabilities can exist simultaneously. For NR satellite access component RIT: The RIT supports mostly unicast transmission of data to/from users. Broadcast capabilities pertain to support and transmission of cell-wide system information/parameters, as well as broadcast/based emergency services (e.g. public warning messages). For IoT NTN component RIT: The RIT is envisioned to support broadcast, multicast and unicast services. [Support for broadcast/multicast is via Multicast downlink transmission based on Single-Cell Point-to-Multipoint (SC-PTM). However, whether this functionality is sufficient for optimal support for IoT NTN has not been evaluated.] |
| 8.2.5.1.17.2 | Describe whether the proposal is capable of providing multiple user services simultaneously to any user with appropriate channel capacity assignments? For NR satellite access component RIT: Multiple services per user can be supported by setting up multiple data radio bearers (DRBs) per user/device. Each radio bearer is characterized by an individual QoS profile/flow. Multiple services per user/device can also be supported by mapping multiple services to a single bearer, if the QoS is the same for these services. The new SDAP sublayer (in the Access Stratum) provides mapping function between (5GC) QoS flows and DRBs. See more details on QoS in 8.2.5.1.12.1 For IoT NTN component RIT Multiple services per user can be supported by setting up multiple data radio bearers (DRBs) per user/device. Each radio bearer is characterized by an individual QoS profile/flow. Multiple services per user/device can also be supported by mapping multiple services to a single bearer, if the QoS is the same for these services. See more details on QoS in 8.2.5.1.12 and 8.2.5.1.13. |
| 8.2.5.1.17.3 | Does the RIT support multiple voice and/or video codecs? Provide details. For NR satellite access component RIT: The RIT could support various voice and video codecs, as desired. In fact, the radio interface technology (fully IP-based) is mostly agnostic to such codecs, and capable of accommodating diverse range of codec types, rates and operation (fixed/dynamic/adaptive). This enables support for all main codecs used/defined today (e.g. AMR-NB/WB, EVS), as well as the capability to support more enhanced codecs that may be defined in future. For IoT NTN component RIT [“Native” voice support has not been evaluated for IoT NTN operation. Support for voice with NB-IoT satellite access with current system would currently be proprietary and “non-native”]. |
| 8.2.5.1.18 | Privacy, authorization, encryption, authentication and legal intercept schemes |
| 8.2.5.1.18.1 | Any privacy, authorization, encryption, authentication and legal intercept schemes that are enabled in the radio interface technology should be described. Describe whether any synchronization is needed for privacy and encryptions mechanisms used in the RIT. Describe how the RIT may be protected against attacks, for example: – man in the middle; – replay; – denial of service. For NR satellite access component RIT: NR Satellite access supports subscriber’s privacy compared to earlier generations, see 3GPP TS 33.501. The most important enhancement is the concealment of subscription permanent identifier over-the-air. This feature is mainly aimed against the active attacker. Another enhancement is the guaranteed regular refreshment of subscription temporary identifier. This feature is mainly aimed against the passive attacker. Another enhancement is the decoupling of the permanent identifier from the Paging mechanism, i.e., there is no longer a Paging message with permanent identifier, the Paging timings are no longer based on permanent identifier. This feature mitigates privacy attacks that use side-channel information in the Paging protocol. Another enhancement comes from the best effort protection of information in the initial message, i.e., if security is setup, privacy sensitive information is concealed, otherwise skipped until the security is setup. Yet another effort is description of a device-assisted network-based framework for false base station detection. This feature can be used to thwart denial-of-service kind of attackers. One enhancement is to increase the authentication between the service network and the core network, or the function of the home network control which can be used to prevent false fraud of the service network. One enhancement is mitigation of bidding down attacks, this feature is to avoid the lower security features of the UE or network selection. For IoT NTN component RIT Security and privacy in the RIT will be provided at a level at least as high as that of “LTE-advanced”. This will be assured by reusing mechanisms of LTE-advanced (see [TS33.401]). A thorough analysis of the security requirements and different solution alternatives for the IoT NTN architecture has been performed, assuring that attacks based on interception, modification, replay, impersonation/man-in-the-middle, denial-of-service, etc, are properly countered. The security solution will thus provide (among other things): strong mutual subscriber/network authentication, user identity protection, strong encryption of user traffic, and strong encryption and integrity protection of signaling. Protection is applied over the air interface as well as to the internal backhaul/RAN-CN interfaces. Particular care has been taken to mitigate threats related to radio SAN security, e.g. protecting the network against hostile “capture” of SANs. Lawful intercept is provided via CN nodes. Furthermore, Control Plane optimisations for Internet of Things traffic have been defined where data is allowed to be transferred via the UE-CN control plane through Non Access Stratum (NAS) messages. Data transferred in this manner is protected in the same manner as control signalling i.e. with encryption and integrity protection applied. |
| 8.2.5.1.19 | Frequency planning |

TABLE 2 (continued)

| Item | Item to be described |
| --- | --- |
| 8.2.5.1.19.1 | How does the RIT support adding new cells or new RF carriers? Provide details. For NR satellite access component RIT: 1008 physical cell identities are supported. Thus, theoretically 1008-cell reuse is realized. Actual cell deployment is operation specific. Self-configuration can be also supported. For IoT NTN component RIT: 504 physical cell identities are supported. Thus, theoretically 504-cell reuse is realized. Actual cell deployment is operation specific. Self-configuration is also supported. |
| 8.2.5.1.20 | Interference mitigation within radio interface |
| 8.2.5.1.20.1 | Does the proposal support Interference mitigation? If so, describe the corresponding mechanism. For NR satellite access component RIT: The RIT has been designed with the aim to minimize the always-on signals to reduce the interference in the system. This is achieved by: - Support longer periodicities for synchronization signals, broadcast channels and periodic reference signals - Use UE-specific demodulation reference signals for control and data that are only transmitted when control and/or data is being transmitted - Control channel resource allocation in the frequency domain is configurable to reduce the interference to control channels in neighbouring cells For IoT NTN component RIT: Static inter-cell interference mitigation is supported by means of e.g. frequency reuse, soft frequency reuse, and reuse partitioning. Inter-cell interference mitigation is supported by means of exchanging interference measurements and scheduling decisions between SANs, see also 8.2.5.1.20.2 below. For eMTC and NB-IoT satellite access, a repetition-based transmission scheme is supported where coherent reception of repeated transmission supports suppression of interference. Cell and user-based scrambling is also implemented to support this mechanism. eMTC supports frequency hopping to reduce the impact from interference. |
| 8.2.5.1.20.2 | What is the signalling, if any, which can be used for inter-beam interference mitigation? For NR satellite access component RIT: To mitigate inter-beam interference, spatial frequency reuse schemes (Frequency reuse factor, FRF > 1) may be used. The traditional Frequency Reuse-3 (FRF-3) scheme, for example, offers a sufficient protection against inter-cell interference. Therefore, techniques such as Remote Interference Management (RIM) coordination or Cross-Link Interference (CLI) Management which are traditionally used in TDD systems and involving inter-base station signalling via the Xn interface are not needed for inter-beam interference mitigation in NR Satellite access. For IoT NTN component RIT: To aid inter-cell interference mitigation, the RIT defines three indicators exchanged between SANs: The High-interference Indicator (HI) which provides information to neighbouring cells about the part of the cell bandwidth upon which the cell intends to schedule its cell-edge user, the Overload Indicator (OI) which provides information on the uplink interference level experienced in each part of the cell bandwidth and Relative Narrowband TX Power (RNTP) indicator, which provides information on the downlink transmission power. |
| 8.2.5.1.20.3 | Link level interference mitigation Describe the feature or features used to mitigate inter-symbol interference. For NR satellite access component RIT: Time and frequency synchronization to the DL and UL frame structures in combination with the use of a cyclic prefix OFDM transmission in both UL(with or without transform precoding) and DL, provides robustness against intersymbol interference. To achieve synchronisation, before and during connection to an NR satellite access cell, the UE shall compute the RTT between UE and the reference point based on the GNSS position, the ephemeris, and the Common TA parameters (see clause 4.2 in TS 38.213), and autonomously pre-compensate the timing for the RTT between UE and the reference point (see clause 4.3 of TS 38.211). The UE shall compute the frequency Doppler shift of the service link, and autonomously pre-compensate for it in the uplink transmissions, by considering UE position and the ephemeris. If the UE does not have a valid GNSS position and/or valid ephemeris and Common TA, it shall not transmit until both are regained. In connected mode, the UE shall be able to continuously update the Timing Advance and frequency pre-compensation. For IoT NTN component RIT: Time and frequency synchronization to the DL and UL frame structures in combination with the use of a cyclic prefix OFDM transmission in both UL (with or without transform precoding) and DL, provides robustness against intersymbol interference. The uplink frequency and time precompensation functionality described in 8.2.5.1.3 shall also be used to avoid inter-symbol/subcarrier interference between users in uplink direction. Also see answer to 8.2.5.1.20.4 |
| 8.2.5.1.20.4 | Describe the approach taken to cope with multipath propagation effects (e.g. via equalizer, rake receiver, cyclic prefix, etc.). For NR satellite access component RIT: The use of OFDM transmission in both UL and DL, in combination with a cyclic prefix, provides inherent robustness to time-dispersion/frequency-selectivity on the radio channel. In case of transform precoding in the UL, time-dispersion/frequency-selectivity on the radio channel can be handled by receiver-side equalization. For IoT NTN component RIT: On the downlink, the use of OFDM transmission, in combination with a cyclic prefix, provides inherent robustness to time-dispersion/frequency-selectivity on the radio channel. On the uplink, time-dispersion/frequency-selectivity on the radio channel can be handled by receiver-side equalization. The detailed equalization approach is implementation dependent. Examples of equalization approaches include frequency-domain linear equalization and Turbo equalization. The use of cyclic prefix also for the uplink may simplify the equalizer implementation. |
| 8.2.5.1.20.5 | Diversity techniques Describe the diversity techniques supported in the MS and at the BS, including micro diversity and macro diversity, characterizing the type of diversity used, for example: – Time diversity: repetition, Rake-receiver, etc. – Space diversity: multiple sectors, etc. – Frequency diversity: frequency hopping (FH), wideband transmission, etc. – Code diversity: multiple PN codes, multiple FH code, etc. – Multi-user diversity: proportional fairness (PF), etc. – Other schemes. Characterize the diversity combining algorithm, for example, switched diversity, maximal ratio combining, equal gain combining. Provide information on the receiver/transmitter RF configurations, for example: – number of RF receivers; – number of RF transmitters. For NR satellite access component RIT: The RIT provides the following means for diversity: - Space diversity by means of multiple transmit and receiver antennas and beamforming - Number of TX-antenna ports: This is a deployment choice, but for the purpose of multi-layer transmissions up to 12 downlink and up to 4 uplink antenna ports have been defined where the mapping of ports to physical antennas is an implementation issue - Number of RX antenna ports: Implementation specific - Frequency diversity by means of wide overall transmission bandwidth and possibility for uplink frequency hopping and uplink and downlink frequency-distributed transmissions - Time diversity by means of fast retransmissions with hybrid ARQ protocol allowing combining of the retransmissions with the original transmission - Multi-user diversity by means of channel-aware scheduling For IoT NTN component RIT: The RIT provides the following means for diversity: - Space diversity by means of multiple transmit and receiver antennas - Number of TX antenna ports: Up to 4 (DL), up to 1 (UL) logical antenna ports are defined where the mapping of logical antenna ports to physical antennas is an implementation specific configuration. - Number of RX antenna ports: 1 - Frequency diversity by means of wide overall transmission bandwidth. Possibility for uplink frequency hopping on a slot basis and downlink frequency-distributed transmission - Time diversity by means of fast retransmissions |
| 8.2.5.1.21 | Synchronization requirements |
| 8.2.5.1.21.1 | Describe RIT’s timing requirements, e.g.: – is synchronization between inter-beam signals or inter-satellite signals required? Provide precise information, the type of synchronization, i.e. synchronization of carrier frequency, bit clock, spreading code or frame, and their accuracy; – is space segment-to-network synchronization required? State frequency and timing accuracy of space segment and user equipment transmit signal. For NR satellite access component RIT: Common general aspects Tight Satellite-to-Satellite synchronization is not required. Likewise, tight Satellite-to-network synchronization is not required. A certain RAN-CN Hyper SFN synchronization is required in case of extended Idle mode DRX. For IoT NTN component RIT: Common general aspects Tight SAN-to-SAN synchronization is not required. Likewise, tight SAN-to-network synchronization is not required. The SAN shall support a logical synchronization port for phase-, time- and/or frequency synchronization, e.g. to provide. - FDD time domain inter-cell interference coordination. A certain RAN-CN Hyper SFN synchronization is required in case of extended Idle mode DRX. Some accuracy requirements SAN transmit signals accuracy: - IoT NTN: - Frequency accuracy (wide area SAN): within ±0.05 ppm, observed over 1ms For more information please refer to - IoT NTN: [36.401], [36.133], [36.108]. |
| 8.2.5.1.21.2 | Describe the synchronization mechanisms used in the proposal, including synchronization between a user terminal and a satellite access node (base station). For NR satellite access and IoT NTN component RITs (common principles) Cell search is the procedure by which a UE acquires time and frequency synchronization with a cell and detects the physical layer Cell ID of that cell. A UE receives the following synchronization signals (SS) in order to perform cell search: the primary synchronization signal (PSS) and secondary synchronization signal (SSS). PSS is used (at least) for initial symbol boundary, cyclic prefix, sub frame boundary, initial frequency synchronization to the cell. SSS is used for radio frame boundary identification. PSS and SSS together used for cell ID detection. Other synchronization mechanisms are defined e.g. for Radio link monitoring, Transmission timing adjustments, Timing for cell activation / deactivation. For NR satellite access component RIT: For uplink time synchronization in NR Satellite access, the UE should perform uplink time pre-compensation as described in 8.2.5.1.13.3. To achieve synchronisation, before and during connection to a Satellite access cell, the UE shall compute the RTT between UE and the reference point based on the GNSS position, the ephemeris, and the Common TA parameters (see clause 4.2 in TS 38.213), and autonomously pre-compensate the timing for the RTT between UE and the reference point (see clause 4.3 of TS [38.211]). The UE shall compute the frequency Doppler shift of the service link, and autonomously pre-compensate for it in the uplink transmissions, by considering UE position and the ephemeris. If the UE does not have a valid GNSS position and/or valid ephemeris and Common TA, it shall not transmit until both are regained. In connected mode, the UE shall be able to continuously update the Timing Advance and frequency pre-compensation. See more information in [38.213] sub-clause 4 and [38.211] sub-clause 7.4.2. For IoT NTN component RIT: For uplink time synchronization, the UE should perform uplink time pre-compensation as described in 8.2.5.1.13.3, with the same mechanism as described for NR satellite access. - For NB-IoT satellite access NB-IoT cell search/synchronization is based on dedicated narrowband signals transmitted in the downlink: the primary and secondary narrowband synchronization signals. See more information in: - [36.213] and [36.211]. - For eMTC satellite access In addition to support for the PSS and SSS, eMTC supports the Resynchronization Signal (RSS) which facilitates an efficient resynchronization to the serving cell. |
| 8.2.5.1.22 | Link budget template Proponents should provide the link budget results to this description template for the environments supported in the RIT. The information is provided with link budget template. |
| 8.2.5.1.23 | Other items |
| 8.2.5.1.23.1 | Coverage extension schemes Describe the capability to support/coverage extension schemes, such as relays or repeaters. For NR satellite access component RIT: NR satellite access access supports the use of the following mechanisms to improve the coverage - NR satellite access can use DFT-spreading and Pi/2 BPSK modulation to reduce PAPR and increase average Tx power for better coverage - NR satellite access can use very low coding rate for better coverage. - Slot aggregation for both control and data can be used for better coverage - High-aggregation level (up to 16) downlink control is possible for better coverage - Beam management is used to increase the coverage NR Satellite access also supports the use of different types of repeater (amplify-and-forward) functionality. However, the details of such functionality is outside the scope of the specification as the use of repeaters is transparent to both the UE and the network. For IoT NTN component RIT: IoT NTN supports the use of the following mechanisms to improve the coverage - Semi-persistent scheduling can be used for repetition uplink transmission; and HARQ-less repetition can be used for downlink transmission, both are beneficial for coverage. - IoT NTN also supports the use of different types of repeater (amplify-and-forward) functionality. However, the details of such functionality is outside the scope of the specification as the use of repeaters is transparent to both the UE and the network. For NB-IoT / eMTC satellite access, - DFT-spreading and Pi/2 BPSK and Pi/4 QPSK modulation in NB-IoT for reduced PAPR for better coverage. - Support for single sub-carrier (NB-IoT) and sub-PRB (eMTC) uplink transmission to increase connection density in extended coverage - Repetition of a transmission for both control and data can be used for better coverage. - High-aggregation level downlink control is possible for better coverage. |
| 8.2.5.1.23.2 | Self-organization Describe any self-organizing aspects that are enabled by the RIT/SRIT. For NR satellite access component RIT: Support for Self-Organizing Networks is an integrated part of NR satellite access. Two use cases that could benefit from SON have been introduced in the Release 15 and the work is continuing. NR satellite access currently supports the following Self-Organizing Network (SON) functions: (Details are provided in [38.300], [38.413], [38.423], [38.331]) – Automatic neighbor discovery: the mechanism allows an gNB to learn information on its neighbors. The discovery mechanism can utilize the assistance of the UE (aka ANR funtion [38.300, Sec. 15.3.3]) as well as the exchange of information over the network interfaces ([38.423; Sec 8.4.1, 8.4.2, 9.1.3.1, 9.1.3.2, 9.1.3.4, 9.1.3.5] as well as the radio resource control information [38.331; Sec 5.5.2, 6.3.2]). – Xn-C TNL address discovery: the mechanism allows a gNB to determine the TNL address on its neighbors candidate gNB. The discovery mechanism can utilize of the ANR function (aka ANR function [38.300, Sec. 15.3.4]) as well as the exchange of information over the network interfaces ([38.413; Sec8.8.1, 8.8.2, 9.2.7.1, 9.2.7.2 ) For IoT NTN component RIT: Support for Self-Organizing Networks is an integrated part of IoT NTN. Several use cases that could benefit from SON have been introduced and the work is continuing. IoT NTN currently supports the following Self-Organizing Network (SON) functions: (Details are provided in [36.300], [36.413], [36.423], [36.314]) Self-configuration: - Dynamic configuration of S1 and X2; the mechanism allows an eNB to establish an S1 interface towards the MME and/or an X2 interface towards another eNB.([36.423, Sec8.3.3, 8.3.5] [36.413, Sec8.7.3, 8.7.4, 8.7.5]) – PCI selection; the mechanism allows an eNB to select its Physical Cell Identity (PCI). The selection can be based either on a centralized or distributed PCI assignment algorithm [36.300, Sec 22.3.5] – Automatic neighbor discovery: the mechanism allows an eNB to learn information on its neighbors. The discovery mechanism can utilize the assistance of the UE (aka ANR function [36.300, Sec. 22.3.3, 22.3.3]) as well as the exchange of information over the network interfaces ([36.423; Sec 8.3.3, 8.3.5] as well as the radio resource control information [36.300; Sec 5.5.2, 5.5.3, 5.5.4, 5.5.5, 6.3.5]). – TNL address discovery: the mechanism allows an eNB to determine the TNL address of the neighbor candidate eNB. The discovery mechanism can utilize of the ANR function (aka ANR function [36.300, Sec. 22.3.6]) as well as the exchange of information over the network interfaces ([36.413; Sec8.15, 8.16, 9.1.16, 9.1.17 ) Self-optimisation: – Mobility load balancing: the mechanism allows an eNB to distribute cell load evenly among cells or to transfer part of the traffic from congested cells. ([36.300, Sec 22.4.1] [36.423, 8.3.6, 8.3.7,8.3.8]) – Mobility Robustness optimization for eMTC: the mechanism is to detect radio link connection failure that occurs due to Too Early or Too Late Handovers, or Handover to Wrong cell. ([36.300, Sec 22.4.2] [36.423, Sec 8.3.9, 8.3.10] [36.331, Sec 5.3.11]) – RACH optimization: the mechanism is supported by UE reported information and by PRACH parameters exchange between eNBs to optimize some RACH configurations. ([36.300, Sec 22.4.3] [36.423, Sec 8.3.3, 8.3.5] [36.331, Sec 5.6.5.3]). – Energy Saving: the mechanism is to reduce operational expenses through energy savings ([36.300, Sec 22.4.4]). |

TABLE 2 (end)

| Item | Item to be described |
| --- | --- |
| 8.2.5.1.23.3 | Describe the frequency reuse schemes (including reuse factor and pattern) for the assessment of cell spectrum efficiency, cell edge user spectral efficiency. Uncoordinated frequency reuse factor of one and frequency reuse factor of three are both used in the performance evaluations. |
| 8.2.5.1.23.4 | Is the RIT an evolution of an existing IMT-Advanced technology? Provide details. The SRIT is an evolution of the 3GPP SRIT For NR satellite access component RIT: No For IoT NTN component RIT: No |
| 8.2.5.1.23.5 | Does the proposal satisfy a specific spectrum mask? Provide details. (This information is not intended to be used for sharing studies.) For NR satellite access component RIT: Yes. UE: For Frequency Range 1 (FR1) UE: For single-component-carrier transmission the spectrum mask is specified in terms of a normative (general) spectrum emission mask [38.101-5, section 6.5.2.2] and an additional spectrum mask [38.101-5, section 6.5.2.3]. This additional spectrum emission mask which is signaled by the network to the UE as a normative requirement can be used to address a specific regional regulatory requirement, a frequency band specific requirement, a roaming requirement and a specific deployment scenario. This additional spectrum emission mask can be used to support the many different sharing requirements in terms of co-existence for a global roaming terminal. BS: For single-component-carrier transmission and transmission of aggregated component-carriers the radiated spectrum mask requirements are defined in [38.108],. in form of OTA out-of-band emissions limits. The unwanted emission limits in the part of the downlink operating band that falls in the spurious domain are consistent with ITU-R Recommendation SM.329. For single-component-carrier transmission and transmission of aggregated component-carriers the conducted spectrum mask requirements are defined in [38.108], for for BS type 1-O and section. in form of OTA out-of-band emissions. For IoT NTN component RIT: Yes. UE: For carrier transmission the spectrum mask is specified in terms of a normative (general) spectrum emission mask [36.102, section 6.5A.3.2 and 6.5B.3.2 for eMTC satellite access and NB-IoT satellite access respectively] and an additional spectrum mask [36.102, section 6.5A.3.3 and 6.5B.3.3 for eMTC satellite access and NB-IoT satellite access respectively]. This additional spectrum emission mask which is signaled by the network to the UE as a normative requirement can be used to address a specific regional regulatory requirement, a frequency band specific requirement, a roaming requirement and a specific deployment scenario. This additional spectrum emission mask can be used to support the many different sharing requirements in terms of co-existence for a global roaming terminal. BS: For carrier transmission, spectrum mask requirements are defined in [36.108], section 6.6.3. in form of operating band unwanted emission limits. These Operating band unwanted emission limits are defined from 10 MHz below the lowest frequency of the downlink operating band up to 10 MHz above the highest frequency of the downlink operating band. The unwanted emission limits in the part of the downlink operating band that falls in the spurious domain are consistent with ITU-R Recommendation SM.329. |
| 8.2.5.1.23.6 | Describe any user terminal power saving mechanisms used in the RIT. For NR satellite access component RIT: Multiple features facilitating device power saving as already have been specified, including Discontinuous reception (DRX) in RRC_CONNECTED, RRC_INACTIVE and RRC_IDLE, Bandwidth part (BWP) adaptation, RRC_INACTIVE state, and Pipelining frame structure enabling micro-sleep within slots in which the UE is not scheduled. Details can be found in item in 8.2.5.1.24. For IoT NTN component RIT: Multiple features facilitating device energy efficiency have been specified for IoT NTN, including Discontinuous reception (DRX) in RRC connected mode and RRC idle mode, Extended Discontinuous reception (DRX) in RRC idle mode (up to 2.9 hours continuous DRX allowed), Paging with Wake-Up Signal in idle mode, and Power Saving Mode (PSM) operation in idle mode (which allows the device to sleep for multiple hours/days - for as long as it is not required to be contactable by the network). |
| 8.2.5.1.23.7 | Simulation process issues Describe the methodology used in the analytical approach. Proponent should provide information on the width of confidence intervals of user and satellite system performance metrics of corresponding mean values, and evaluation groups are encouraged to provide this information. System simulations are iterated over M independent ‘drops’ of user locations. Statistics, mean and percentiles, are calculated over all drops, and confidence intervals are estimated by comparing the results of the different drops. The number of drops is up to each evaluator. |
| 8.2.5.1.24 | Other information Please provide any additional information that the proponent believes may be useful to the evaluation process. For NR satellite access component RIT: NR satellite access supports the following set of features for providing long battery life: - A configurable transmission and reception bandwidth for limiting the device modem power consumption. - DFT-spread OFDM modulation for limiting the peak to average ratio of the uplink waveform and increasing the device power amplifier efficiency. - Uplink power control which allows the device to adapt its transmit power to the actual radio environment. - Connected mode DRX cycles for reducing the device power consumption while in RRC Active state. - Measurement rules for reducing the RRC idle mode RRM activities. - Resumption of a previous connection for minimizing the control signalling when initiating a mobile originated or terminated data transmission NR satellite access has the capability to support different satellite deployment scenarios at different orbits including Non-GEO and GEO based non-terrestrial access network (e.g., see [38.821]). For IoT NTN component RIT: - Range of deployment scenarios: IoT NTN has the capability to support different satellite deployment scenarios at different orbits including GEO and non-GEO based non-terrestrial access network, e.g. see [36.763]. - Network Energy consumption IoT NTN network also has the ability to support a high sleep ratio and long sleep duration. When no data transfer takes place, both NB-IoT satellite access and eMTC satellite access networks will keep periodical transmission of (N)PSS/(N)SSS/(N)PBCH/NRS or CRS and System information in order for UEs to detect and access the radio network. In terms of sleep ratio, values slightly >70% can be achievable, depending on network configuration. Due to the narrowband nature of IoT NTN, if multiple RF carriers are typically operated to increase capacity, then also deactivation of unused RF carriers by the network is another approach to reduce energy consumption when loading is sufficiently low. |

Reference

[22.011]	TS22.2011v17.6.0 “Service accessibility”

[23.122]	TS23.122, “Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode”

[22.261]	TS22.261v17.1.0 “Service requirements for next generation new services and markets”

[36.102]	TS36.102v18.3.0 “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception for satellite access”

[36.108]	TS36.108v18.3.0 “Evolved Universal Terrestrial Radio Access (E-UTRA); Satellite Access Node radio transmission and reception”

[36.133]	TS36.133v18.3.1 “Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management”

[36.201]	TS36.201v17.0.0 “Evolved Universal Terrestrial Radio Access (E-UTRA); LTE physical layer; General description”

[36.211]	TS36.211v17.4.0 “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation”

[36.212]	TS36.212v17.1.0 “Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding”

[36.213]	TS36.213v17.5.0 “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures”

[36.300]	TS36.300v17.5.0 “Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2”

[36.304]	TS36.304v17.4.0 “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode”

[36.307]	TS36.307v18.2.0 “Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements on User Equipments (UEs) supporting a release-independent frequency band”

[36.314]	TS36.314v17.0.0 “Evolved Universal Terrestrial Radio Access (E-UTRA); Layer 2 – Measurements”

[36.321]	TS36.321v17.6.0 “Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification”

[36.322]	TS36.322v17.0.0 “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Link Control (RLC) protocol specification”

[36.331]	TS36.322v17.6.0 “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Link Control (RLC) protocol specification”

[36.401]	TS36.401v17.1.0 “Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Architecture description”

[36.413]	TS36.413v17.5.0 “Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)”

[36.423]	TS36.423v17.6.0 “Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 Application Protocol (X2AP)”

[36.763]	TR36.763v17.0.0 “Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)”

[38.101-5] TS38.101-5v17.5.0, “NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements”

[38.108]	TS38.108v17.5.0, “NR; Satellite Access Node radio transmission and reception”

[38.300]	TS38.300v17.6.0, “NR; NR and NG-RAN Overall description; Stage-2”

[38.304]	TS38.304v17.6.0, “NR; User Equipment (UE) procedures in idle mode and in RRC Inactive state”

[38.321]	TS38.321v17.6.0, “NR; Medium Access Control (MAC) protocol specification”

[38.401] 	TS38.401v17.6.0, “NG-RAN; Architecture description”

[38.413]	TS38.413v17.6.0, “NG-RAN; NG Application Protocol (NGAP)”

[38.423]	TS38.423v17.6.0, “NG-RAN; Xn Application Protocol (XnAP)”

[38.811]	TR38.811v15.4.0 “Study on New Radio (NR) to support non-terrestrial networks”

[38.821]	TR38.821v16.2.0, “Solutions for NR to support Non-Terrestrial Networks (NTN)”

[38.863]	TR38.863v17.2.0 “Non-terrestrial networks (NTN) related RF and co-existence aspects”

Developed by 3GPP as 5G NTN, Release 17 and beyond.