3GPP TSG-RAN WG3 Meeting #122	R3-237128

Chicago, USA, 13th – 17th November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.480 | CR | 0005 | rev | 2 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Temp no data and DL data arrival for Activate Multicast Session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Huawei, ZTE, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2023-11-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TS 38.300 allows the gNB to move the UE to RRC_INACTIVE state in case there is temporarily no data for an active multicast session: When there is temporarily no data to be sent to the UEs for a multicast session that is active, the gNB may move the UE to RRC_INACTIVE state. But currently in E1AP, the gNB-CU-UP is not able to inform the gNB-CU-CP about “inactivity” and “DL data arrival”, therefore it is needed to introduce a new E1AP class 2 procedure to notify the gNB-CU-CP about these. And similar to PDU session handling, to help the gNB-CU-UP to trigger the above new procedure, there is also needed for the gNB-CU-CP to provide Multicast Session Inactivity Timer and MC Bearer Context Status Change in the MC BEARER CONTEXT SETUP/MODIFICATION REQUEST messages. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enable DL data arrival detection function and suspension/resumption of bearer context for MBS. Introduce new class 2 procedure to notify the gNB-CU-CP about “inactivity of MC Bearer context” and “DL data arrival”. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it only enable the gNB-CU-UP to inform the “temp no data” and “DL data arrival” to the gNB-CU-CP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The gNB is not able to move the UE to RRC_INACTIVE state in case there is temporarily no data for an active multicast session. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.7; 6.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 37.483 CR0079 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev-1: updated based on the discussion during the meeting. Rev-2: resubmit to RAN3#122 meeting |  |  |  |  |  |  |  |  |

-------------------------------------------------------the 1st change ---------------------------------------------------

### 5.1.7	E1 bearer context management function for NR MBS

The E1 bearer context management function for NR MBS consists of two sub-sets for functions, one for NR MBS broadcast, one for NR MBS multicast.

Both sets follow the principles of the E1 bearer context management functions, with the following differences.

-	E1 NR MBS procedure concerns a single MBS Session Resource only.

-	E1 NR MBS procedures concern the control of MRB resources in gNB-CU-UP.

-	E1 NR MBS procedures do not control security information, as for NR MBS, PDCP does not apply security as specified in TS 38.300 [6].

-	QoS flow to MRB mapping is determined by the gNB-CU-CP or, in case of shared NR-U terminations, the gNB-CU-UP may be notified about the QoS flow to MRB mapping already determined in the bearer context for the shared NR-U termination. The gNB-CU-CP may inform the gNB-CU-UP whether it is contended with the already determined mapping decision.

NOTE: Not all QoS flow parameters are applicable for NR MBS, as specified in TS 23.247 [10].

-	Data volume reporting is not applicable for NR MBS.

-	CA based packet duplication is not applicable for NR MBS.

-------------------------------------------------------the 2nd change ---------------------------------------------------

6.7	NR MBS procedures

The E1 MBS procedures are listed below:

-	Broadcast E1AP MBS procedures

-	BC Bearer Context Setup

-	BC Bearer Context Modification (gNB-CU-CP inititated)

-	BC Bearer Context Modification (gNB-CU-UP inititated)

-	BC Bearer Context Release (gNB-CU-CP inititated)

-	BC Bearer Context Release (gNB-CU-UP inititated)

-	Multicast E1AP MBS procedures

-	MC Bearer Context Setup

-	MC Bearer Context Modification (gNB-CU-CP inititated)

-	MC Bearer Context Modification (gNB-CU-UP inititated)

-	MC Bearer Context Release (gNB-CU-CP inititated)

-	MC Bearer Context Release (gNB-CU-UP inititated)

-	MC Bearer Notification

-------------------------------------------------------End of change ---------------------------------------------------