3GPP TSG RAN meeting #102§									RP-233868

Edinburgh, Scotland, December 11th -15th 2023

## Status Report to TSG

Agenda item:			9.2.2

| WI / SI Name | Study on Self-Evaluation towards the IMT-2020 submission of the 3GPP Satellite Radio Interface Technology |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_IMT_2020_Sat_eval |  |  |  |  |
| Unique ID |  |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-231945 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 12/2023 | Core part: | Performance part: | Testing part: |  |
| Overall Completion level | Study Item: 100 % | Core part: | Performance Part: | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN (ITU-R Ad-Hoc) |
| --- | --- | --- |
| Rapporteur | Name | Asbjörn Grövlen |
|  | Company | Ericsson |
|  | Email | asbjorn.grovlen@ericsson.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

## 2.0	RAN

#### 2.0.1	Agreements

#### RAN#102

The following contributions were approved:

- RP‑233952

  - TP for TR 37.911 (section 7 & 8) - IMT-2020 satellite RIT evaluation

- RP‑233979

  - pCR to TR 37.911 collecting inputs from RAN WGs for the Study on self-evaluation towards the IMT-2020 submission of the 3GPP Satellite Radio Interface Technology

- RP‑233980

  - TR 37.911 v1.0.0 on Study on self-evaluation towards the IMT-2020 submission of the 3GPP Satellite Radio Interface Technology

The study item was declared closed.

### 2.0.2	Remaining Open issues

- None

## 2.1	RAN1

#### 2.1.1	Agreements

#### RAN1#114bis

Agreement

Proposal 1.1 in section 6 of R1-2310471 is endorsed with TBS value 1736 for NR NTN Rural eMBB-s table.

Agreement

For the link budget template, the target elevation angle is 30 degrees, and the link budget template includes the link margin (in dB) at that elevation angle.

Agreement

Include the receiver interference in the link budget template for NTN, and reuse the following average INR results from TR 38.821:

- 4.4 dB for FRF3 DL,

- 7.6 dB for FRF3 UL

Agreement

The UE speed is updated as 3 km/h in the template for results collection for connection density.

Agreement

The maximum delay for reliability should be reported with at least consideration on the impact of assumed layout, number of repetitions, usage of HARQ, and feeder link delay.

Agreement

At least for eMBB-s spectral efficiency evaluation, a value of 0dB for scintillation loss can be optionally used (in addition to the already agreed 2.2dB) and results can be separately captured in TR 37.911.

Agreement

The results of CL, Geometry SIR and Geometry SINR simulated on DL and UL transmissions reported within the attached template will be captured in the TR 37.911.

Attachment: R1-2308869 1 Att Calibration_results v06

Agreement

Confirm the values of NR NTN peak spectral efficiency and Peak data rate reported in the pCR in RP-231946 for TR 37.911 with the following correction:

- Peak data rate (Mbit/s) in uplink should be 2.67 instead of 2.76

Agreement

Capture the following observation and table in the TR 37.911:

It is observed that NR NTN fulfils the mobility requirement under 250 km/h.

Table Evaluation results of NR NTN mobility under 250 km/h

| Frequency reuse factor | ITU Requirement |  |  | Number of samples |
| --- | --- | --- | --- | --- |
| FRF1 | Normalized traffic channel link data rate (bit/s/Hz) | 0.005 | [0.07] | [4] |
|  | Residual decoded packet error ratio | 1% | [0.16%] | [3] |
| FRF3 | Normalized traffic channel link data rate (bit/s/Hz) | 0.005 | [0.14] | [4] |
|  | Residual decoded packet error ratio | 1% | [0.34%] | [3] |

Agreement

Capture the following observation and table in the TR 37.911:

It is observed that NR NTN fulfils the reliability requirement for downlink.

Table 1 Evaluation results of DL reliability for NR NTN

| Frequency reuse factor | ITU Requirement | DL Reliability | Number of samples |
| --- | --- | --- | --- |
| FRF 1 | 99.9% | [99.98%] | [4] |
| FRF 3 | 99.9% | [99.96%] | [5] |

Agreement

Capture the following observation and table in the TR 37.911:

It is observed that NR NTN fulfils the reliability requirement for uplink.

Table Evaluation results of UL reliability for NR NTN

| Frequency reuse factor | ITU Requirement | UL Reliability | Number of samples |
| --- | --- | --- | --- |
| FRF 1 | 99.9% | [99.97%] | [4] |
| FRF 3 | 99.9% | [99.97%] | [5] |

Agreement

Capture the following observation and table in the TR 37.911:

It is observed that NR NTN fulfils connection density requirement.

Table Evaluation results of connection density for NR NTN

| Traffic model | Frequency reuse factor | ITU Requirement | Connection density(/km2) | Bandwidth (kHz) | Number of samples |
| --- | --- | --- | --- | --- | --- |
| 1 message/day/device | FRF1 | 500 | [7334] | 180 | [4] |
|  | FRF3 | 500 | [28115] | 540 | [4] |
| 1 message/2 hours/device | FRF1 | 500 | [611] | 180 | [4] |
|  | FRF3 | 500 | [2340] | 540 | [4] |

Agreement

The same energy efficiency aspects from the terrestrial self-evaluation in the report TR 37.910 apply for NTN as well on network side and device side. Thereby the text on the energy efficiency requirement from TR 37.910 can be reused for TR 37.911 with changes as necessary, considering only 15 kHz subcarrier spacing.

Agreement

For eMTC evaluations, companies to report the TBS size used.

#### RAN1#115

Agreement

The TP to TR 37.911 for Connection Density evaluation in R1-2312444 is endorsed.

Agreement

For the number of retransmissions in HRC-s:

- The SNR value (required SNR) will be obtained by averaging the submitted SNR values to R1#115.

- The transmission bit rate and spectral efficiency will be calculated assuming 4 transmissions (4kbps, 0.022 bps/Hz)

Agreement

The link budget template in R1-2312355 (v3) is endorsed, subject to further check during R1#115.

- The link budget template includes only the average SNR for each channel and scenario (and not the individual SNR values reported by companies)

Agreement

The updated results of CL, Geometry SIR and Geometry SINR simulated on DL and UL transmissions reported within the attached template will be captured in the TR 37.911 as in Attachment B.1_ Calibration.

Agreement

Confirm the values of the evaluation results of NR satellite access mobility under 250 km/h agreed in RAN1#114bis with the following modification.

Table  Evaluation results of NR satellite access mobility under 250 km/h

| Frequency reuse factor | ITU Requirement |  | Value | Number of samples |
| --- | --- | --- | --- | --- |
| FRF1 | Normalized traffic channel link data rate (bit/s/Hz) | 0.005 | 0.07 | 4 |
|  | Residual decoded packet error ratio | 1% | [0.16%] 0.18% | 4 |
| FRF3 | Normalized traffic channel link data rate (bit/s/Hz) | 0.005 | 0.14 | 4 |
|  | Residual decoded packet error ratio | 1% | [0.34%] 0.33% | 4 |

Agreement

Adopt the TP for clause 4.9 of TR 37.911 on Mobility evaluation in NR satellite access provided within the attached document (Attachment: Att  TP for TR37.911- Mobility) with the addition of “Value” at the top of the third column of the table.

Agreement

Confirm the following evaluation results of DL and UL reliability for NR satellite access:

Table 1: Evaluation results of DL reliability for NR satellite access

| Frequency Reuse Factor | ITU Requirement | DL Reliability | Number of samples |
| --- | --- | --- | --- |
| FRF 1 | 99.9% | 99.98% | 4 |
| FRF 3 | 99.9% | 99.96% | 5 |

Table 2: Evaluation results of UL reliability for NR satellite access

| Frequency Reuse Factor | ITU Requirement | UL Reliability | Number of samples |
| --- | --- | --- | --- |
| FRF 1 | 99.9% | 99.97% | 4 |
| FRF 3 | 99.9% | 99.97% | 5 |

Agreement

Adopt the TP for clause 6.1 of TR 37.911 on Reliability evaluation in NR satellite access provided within the attached document (Attachment: Att  TP for TR37.911- Reliability).

Agreement

The evaluation results of connection density for NR satellite access are updated as follow:

Table  Evaluation results of connection density for NR satellite access

| Traffic model | Frequency reuse factor | ITU Requirement | Connection density(/km2) | Bandwidth (kHz) | Number of samples |
| --- | --- | --- | --- | --- | --- |
| 1 message/day/device | FRF1 | 500 | 7205 [7334] | 180 | 4 |
|  | FRF3 | 500 | 27357 [28115] | 540 | 4 |
| 1 message/2 hours/device | FRF1 | 500 | 600 [611] | 180 | 4 |
|  | FRF3 | 500 | 2277 [2340] | 540 | 4 |

Agreement

Adopt the TP for clause 4.8 of TR 37.911 on Energy efficiency evaluation in NR satellite access provided within the attached document (Attachment: Att  TP for TR37.911- Energy efficiency).

Agreement

The proposals in section 12 of R1-2310942 are agreed.

Agreement

Unbounded BLER results: Do not capture the values not bounded by 10% BLER in the results spreadsheet. But add a note to the Results Spreadsheet that one company showed that with unbounded BLER for FRF=1, higher connection density values are feasible due to higher spectral efficiency vs SINR.

Agreement

99%ile packet latency values: Capture in the TR that for the given connection density values the 99th percentile packet latency fulfils the <10 seconds requirement by a large margin.

Agreement

Bandwidth capture: Capture the bandwidth needed by the average result. Also add a note to say that number of users will scale up further with higher bandwidth. Capture also for NR.

Agreement

Agree the following terminology: “IoT NTN, i.e. NB-IoT and eMTC satellite access”.

Agreement

The IoT NTN _ConnDensity_Simulation Results in attachment of R1-2312419 are endorsed.

#### 2.1.2	Remaining Open issues

None.

## 2.2	RAN2

#### 2.2.1	Agreements

#### RAN2#123bis

R2-2309714	Report of [Post123][102]NTN Self Ev] CPUP latency (Ericsson)	Ericsson	report	Rel-18

Adopt the following assumptions for the delay calculation of UP and CP latencies.

Agreed (assumption on processing time can be added)

Send an LS to RAN1 to inform about RAN2 progress and check the assumed delay values for UP and CP latency calculations.

Agreed

Adopt the attached TP to TS 37.911 about mobility interruption time.

Endorsed as a basis for further offline discussion during the week, also considering the input papers

R2-2311314	TP for TR 37.911	Ericsson	discussion Rel-18	FS_IMT2020_SAT_eval

TP is endorsed from RAN2 perspective

R2-2311315	LS to RAN1 on RAN2 progress of NTN Self Evaluation	Ericsson	LS out	Rel-18	FS_IMT2020_SAT_eval	To:RAN1

Approved

#### RAN2#124

R2-2313878	TP for IMT-2020 Satellite self-evaluation for Latency	THALES, Ericsson	discussion	Rel-18	FS_IMT-2020_Sat_eval

Endorsed from RAN2 perspective

R2-2313879	LS on IMT-2020 Satellite self-evaluation for Latency	LSout	To:RAN1	Rel-18	FS_IMT-2020_Sat_eval

Approved

#### 2.2.2	Remaining Open issues

None

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

#### RAN2#124

R4-2321029  Draft TP for TR 37.911 - Study on self-evaluation towards the IMT-2020 submission of the 3GPP Satellite Radio Interface Technology

Type: pCR                          For: Approval
                                                                          37.911 v0.1.0      CR-  rev  Cat:  (Rel-18)

                                                                          Source: THALES

Abstract:

TP for on-going TR 37.911 - Clauses 7.2 and 7.3 (Bandwidth and Spectrum)

Decision:                   Approved.

#### 2.4.2	Remaining Open issues

None

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

## 4.	References

#### RAN1#114bis

R1-2308868	Self-evaluation results for NR NTN	THALES

R1-2308869	Feature Lead Summary#1 on self-evaluation results for NR NTN	THALES

R1-2308870	Feature Lead Summary#2 on self-evaluation results for NR NTN	THALES

R1-2308871	Feature Lead Summary#3 on self-evaluation results for NR NTN	THALES

R1-2308902	Evaluation methodology of IMT-2020 satellite	Huawei, HiSilicon

R1-2308903	Self-evaluation results for NR NTN	Huawei, HiSilicon

R1-2308904	Self-evaluation results for IoT NTN	Huawei, HiSilicon

R1-2309095	Discussions on self evaluation results for NR NTN	vivo

R1-2309153	Discussion on the evaluation methodology of the self-evaluation	ZTE

R1-2309154	Discussion on the Self-evaluation results for NR NTN	ZTE

R1-2309155	Discussion on the Self-evaluation results for IoT NTN	ZTE

R1-2309292	Initial results on NR-NTN self evaluation towards IMT 2020 submission	CEWiT

R1-2309399	Discussion on remaining issues for evaluation methodology	Samsung

R1-2309400	Discussion on evaluation results for NR NTN	Samsung

R1-2309439	Self-evaluation results for NR NTN	xiaomi

R1-2309509	Further evaluation for NR NTN	CATT

R1-2309607	Discussion on self-evaluation methodology for IMT-2020 satellite radio interface	OPPO

R1-2309608	Self-evaluation results for NR NTN	OPPO

R1-2309609	Self-evaluation results for IoT NTN	OPPO

R1-2309713	Evaluation results on NR-NTN for self-evaluation of IMT-2020 satellite radio interface technology	Panasonic

R1-2309738	Evaluation results related to IMT-2020 Satellite for NR over NTN for Earth Fixed Cell	Nokia, Nokia Shanghai Bell

R1-2309944	Self-evaluation results for NR NTN	CCU, ITRI

R1-2309984	Evaluation assumptions for HRC-s for IoT NTN	MediaTek Inc.

R1-2309985	Evaluation results for IoT NTN	MediaTek Inc.

R1-2310054	Calibration results and peak data rate analysis for NR NTN	NTT DOCOMO, INC.

R1-2310165	Further discussion on self-evaluation assumptions	Qualcomm Incorporated

R1-2310166	Self-evaluation results for NR NTN	Qualcomm Incorporated

R1-2310167	Self-evaluation results for IOT NTN	Qualcomm Incorporated

R1-2310189	Study on Self-Evaluation of Satellite RIT – Energy Efficiency	Fraunhofer IIS, Fraunhofer HHI

R1-2310220	Satellite IMT-2020 self-evaluation results for NR NTN	Ericsson

R1-2310222	Self-Evaluation Results for NR NTN (Target SNR for target BLER)	Fraunhofer IIS

R1-2310321	Evaluation results for IoT NTN	MediaTek Inc.

R1-2310358	Evaluation methodology of IMT-2020 satellite	Huawei, HiSilicon

R1-2310391	Self-evaluation results for NR NTN	Qualcomm Incorporated

R1-2310392	Self-evaluation results for IOT NTN	Qualcomm Incorporated

R1-2310471	Feature lead summary on Evaluation Methodology for IMT-2020 satellite	Moderator (Qualcomm Incorporated)

R1-2310497	Further evaluation for NR NTN	CATT

R1-2310500	Feature Lead summary for 8.15.3: Self-Evaluation Results for IoT NTN	Moderator (MediaTek)

R1-2310547	Session notes for 8.15 (Study on Self-Evaluation towards the 3GPP submission of a IMT-2020 Satellite Radio Interface Technology)	Ad-Hoc Chair (Huawei)

R1-2310558	Self-evaluation results for NR NTN	xiaomi

R1-2310587	Feature lead summary#2 on Evaluation Methodology for IMT-2020 satellite	Moderator (Qualcomm Incorporated)

#### RAN1#115

R1-2310856	Self-evaluation results for NR NTN	Huawei, HiSilicon

R1-2310940	Self-evaluation results for NR NTN	THALES

R1-2310941	Feature Lead Summary#1 on self-evaluation results for NR NTN	Moderator (THALES)

R1-2310942	Feature Lead Summary#2 on self-evaluation results for NR NTN	Moderator (THALES)

R1-2311116	Discussions on self-evaluation results for NR NTN	vivo

R1-2311204	Discussion on the evaluation methodology of the self-evaluation	ZTE

R1-2311205	Discussion on the Self-evaluation results for NR NTN	ZTE

R1-2311206	Discussion on the Self-evaluation results for IoT NTN	ZTE

R1-2311257	Discussion on self-evaluation methodology for IMT-2020 satellite radio interface	OPPO

R1-2311258	Self-evaluation results for NR NTN	OPPO

R1-2311259	Self-evaluation results for IoT NTN	OPPO

R1-2311562	Evaluation results of NR-NTN for IMT-2020 satellite radio interface technology	Panasonic

R1-2311566	Self-evaluation results for NR NTN	Beijing Xiaomi Mobile Software

R1-2311641	Self-evaluation results for NR NTN	NTT DOCOMO, INC.

R1-2311750	Self-evaluation results for NR NTN	CCU, ITRI

R1-2312001	Text Proposal to TR 37.911 for Connection Density evaluation for IoT NTN and NR NTN	MediaTek, THALES, Qualcomm

R1-2312002	Evaluation results for IoT NTN Connection Density	MediaTek Inc.

R1-2312058	Further discussion on self-evaluation assumptions	Qualcomm Incorporated

R1-2312059	Self-evaluation results for NR NTN	Qualcomm Incorporated

R1-2312060	Self-evaluation results for IOT NTN	Qualcomm Incorporated

R1-2312139	On evaluation criteria for adopting the evaluation results for TR 37.911	Nokia, Nokia Shanghai Bell

R1-2312140	Comparison results related to IMT-2020 Satellite for NR over NTN for Earth Fixed Cell Vs. Earth Moving Cell	Nokia, Nokia Shanghai Bell

R1-2312149	Evaluation methodology of IMT-2020 satellite	Huawei, HiSilicon

R1-2312150	Self-evaluation results for IoT NTN	Huawei, HiSilicon

R1-2312162	Satellite IMT-2020 self-evaluation results for NR NTN	Ericsson

R1-2312175	[Draft] Reply LS on RAN2 progress of NTN Self Evaluation	Ericsson

R1-2312265	Self-evaluation results for NR NTN	Qualcomm Incorporated

R1-2312322	Self-evaluation results for NR NTN	NTT DOCOMO, INC.

R1-2312355	Feature lead summary on Evaluation Methodology for IMT-2020 satellite	Moderator (Qualcomm Incorporated)

R1-2312419	Feature Lead summary for 8.15.3: Self-Evaluation Results for IoT NTN	Moderator (MediaTek)

R1-2312444	Text Proposal to TR 37.911 for Connection Density evaluation for IoT NTN and NR NTN	MediaTek, THALES, Qualcomm

R1-2312447	Further evaluation results for NR NTN	CATT

R1-2312511	Session notes for 8.15 (Study on Self-Evaluation towards the 3GPP submission of a IMT-2020 Satellite Radio Interface Technology)	Ad-Hoc Chair (Huawei)

#### RAN2#123bis

R2-2309714	Report of [Post123][102]NTN Self Ev] CPUP latency (Ericsson)	Ericsson

R2-2310086	Discussion on IMT-2020 Satellite self-evaluation for Latency	THALES

R2-2311112	CP and UP latency evaluation TP	Qualcomm Technologies Ireland

R2-2311215	[Draft] LS to RAN1 on RAN2 progress of NTN Self Evaluation	Huawei, HiSilicon

R2-2311233	Satellite IMT-2020 self-evaluation: CP latency	Ericsson

R2-2311234	Satellite IMT-2020 self-evaluation: UP latency	Ericsson

R2-2311314	TP for TR 37.911	Ericsson

R2-2311315	LS to RAN1 on RAN2 progress of NTN Self Evaluation	RAN2

#### RAN2#124

R2-2312865	Discussion on IMT-2020 Satellite self-evaluation for Latency	THALES

R2-2313878	TP for IMT-2020 Satellite self-evaluation for Latency	THALES, Ericsson

R2-2313879	LS to RAN1 on RAN2 addition to NTN Self Evaluation	RAN2

#### RAN4#109

R4-2321029	Draft TP for TR 37.911 - Study on self-evaluation towards the IMT-2020 submission of the 3GPP Satellite Radio Interface Technology	THALES

#### RAN#102

RP-233262	Description and Characteristics Template for IMT-2020 satellite SRIT submission	MediaTek Inc. (Editor)

RP-233263	Draft TP for TR 37.911 (section 7) - IMT-2020 satellite RIT evaluation	MediaTek Inc.

RP-233738	TR 37.911 v0.1.1 on Study on self-evaluation towards the IMT-2020 submission of the 3GPP Satellite Radio Interface Technology	Ericsson, MediaTek, Qualcomm, Thales

RP-233868	Status report for SI: Study on self-evaluation towards the IMT-2020 submission of the 3GPP Satellite Radio Interface Technology; rapporteur: Ericsson	Ericsson

RP-233944	TP for TR 37.911 - IMT-2020 satellite RIT evaluation	MediaTek Inc., Ericsson, Qualcomm Incorporated, Thales

RP-233945	TR 37.911 v0.2.0 on Study on self-evaluation towards the IMT-2020 submission of the 3GPP Satellite Radio Interface Technology	Ericsson

RP-233949	pCR to TR 37.911 collecting inputs from RAN WGs for the Study on self-evaluation towards the IMT-2020 submission of the 3GPP Satellite Radio Interface Technology	Ericsson, MediaTek, Qualcomm, Thales

RP-233952	TP for TR 37.911 (section 7 & 8) - IMT-2020 satellite RIT evaluation	MediaTek Inc., Ericsson, Qualcomm Incorporated, Thales