3GPP TSG-RAN WG2 Meeting #124	R2-2313787

Chicago, USA, 13rd – 17th Nov, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.321 | CR | 1573 | rev | 2 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 36.321 on Koffset handling during MAC reset |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson, Samsung, OPPO, Nokia, Qualcomm, MediaTek, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN |  |  |  |  |  | Date: |  |  | 2023-11-3 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Koffset comprises of two parts: the first part is cell specific Koffset, which is delivered in SIB or in handover command; the second part is UE specific Koffset, which is provided by the Differential Koffset MAC CE. There may be some ambiguity on Koffset usage for the following cases: - During handover, only the cell specific Koffset of the target cell can be configured in handover command. Thus the Differential Koffset from source cell shouldn’t be used after handover. - During RRC Re-establishment, the UE may select a new serving cell. In that case, the Differential Koffset from the old serving cell shouldn’t be used. - Upon going to RRC_IDLE with suspend indication, the UE may select a new serving cell. In that case, the Differential Koffset from the old serving cell shouldn’t be used after RRC connection is resumed. The CR is updated based on the IPA CR in R2-2311597. (Please see the revision history.) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the UE’s configured differential Koffset is cleared during MAC reset. Impact analysis Impacted functionality: IoT NTN Inter-operability issues: If the UE is implemented according to the CR while the network is not, there may be temporarily different understanding between the UE and the network about the Koffset used by the UE, before the network sends another Differential Koffset MAC CE. If the network is implemented according to the CR while the UE is not, there may be temporarily different understanding between the UE and the network about the Koffset used by the UE, before the network sends another Differential Koffset MAC CE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE may mistakenly use an outdated Differential Koffset. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The CR is updated based on the IPA CR in R2-2311597. The reason is that besides the HO case addressed in the IPA CR, more cases (e.g., RRC re-establishment) have been identified with similar issue. Then based on some offline discussion among interested companies (and also the suggestion from session chair), an updated CR is proposed to cover all problematic cases to avoid further complexity in the future. |  |  |  |  |  |  |  |  |

<Start of Change>

5.9 	MAC Reset

If a reset of the MAC entity is requested by upper layers, the MAC entity shall:

-	initialize Bj for each logical channel to zero;

-	except for pur-TimeAlignmentTimer, if configured, stop (if running) all timers;

-	except for pur-TimeAlignmentTimer, if configured, consider all timeAlignmentTimers as expired and perform the corresponding actions in clause 5.2;

-	set the NDIs for all uplink HARQ processes to the value 0;

-	stop, if any, ongoing RACH procedure;

-	discard explicitly signalled ra-PreambleIndex and ra-PRACH-MaskIndex, if any;

-	flush Msg3 buffer;

-	cancel, if any, triggered Scheduling Request procedure;

-	cancel, if any, triggered Buffer Status Reporting procedure;

-	cancel, if any, triggered Power Headroom Reporting procedure;

-	cancel, if any, triggered Recommended bit rate query procedure;

-	cancel, if any, triggered Timing Advance Reporting procedure;

-	flush the soft buffers for all DL HARQ processes;

-	for each DL HARQ process, consider the next received transmission for a TB as the very first transmission;

-	release, if any, Temporary C-RNTI.

-	clear, if any, Differential Koffset.

If a partial reset of the MAC entity is requested by upper layers, for a serving cell, the MAC entity shall for the serving cell:

-	set the NDIs for all uplink HARQ processes to the value 0;

-	flush all UL HARQ buffers;

-	stop all running drx-ULRetransmissionTimers;

-	stop all running UL HARQ RTT timers;

-	stop, if any, ongoing RACH procedure;

-	discard explicitly signalled ra-PreambleIndex and ra-PRACH-MaskIndex, if any;

-	flush Msg3 buffer;

-	release, if any, Temporary C-RNTI.

<End of Change>