3GPP TSG-RAN WG2 Meeting #124	R2-2313555

# Chicago, IL, USA, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0475 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to UE TEG Capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2023-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A LMF can request the target device to measure the same DL-PRS Resource of a TRP with N different UE RxTx TEGs: However, the corresponding UE capability for the maximum number of different UE RxTx TEGs that a UE supports to measure the same DL-PRS Resource of a TRP is missing. There is currently only the capability on the maximum number of Rx TEGs: RAN1 discussed this issue at RAN1#114 and made the following conclusion: Conclusion For a UE supporting FG 27-1-3 (“Support of UE-RxTxTEGs for Multi-RTT”), and - also supporting FG 27-1-4 (“Support of UE Rx TEGs for measuring the same DL PRS resource”), the reported value in FG 27-1-4 also corresponds to the maximum number of different UE-RxTx TEGs for measuring the same DL PRS of a TRP with the same UE Tx TEG, - also supporting FG 27-1-4a (“Support of UE Rx TEGs for measuring the same DL PRS resource simultaneously”), the reported value in FG 27-1-4a also corresponds to the maximum number of different UE-RxTx TEGs for measuring the same DL PRS resource simultaneously with the same UE Tx TEG. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A Note is added to the field description that the reported values for the UE capabilities measureSameDL-PRS-ResourceWithDifferentRxTEGs and measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul also apply to RxTx TEGs. Impact Analysis Impacted functionality: UE TEG Capability Inter-operability: If the UE is implemented according to this CR but the network is not, the NW may have a different understanding of the maximum number of different UE-RxTx TEGs the UE supports for measuring the same DL PRS Resource of a TRP. If the network is implemented according to this CR but the UE is not, the NW may have a different understanding of the maximum number of different UE-RxTx TEGs the UE supports for measuring the same DL PRS Resource of a TRP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE capability to measure the same DL-PRS Resource of a TRP with different RxTx TEGs together with the maximum number of different UE RxTx TEGs that a UE supports to measure the same DL-PRS Resource of a TRP remains undefined. Therfore, a LMF may request a measurement not supported by a UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev0: R2-2310909 (agreed in principle at RAN2#123bis) |  |  |  |  |  |  |  |  |

### 6.4.3	Common NR Positioning Information Elements

[…]

#### –	NR-UE-TEG-Capability

The IE NR-UE-TEG-Capability defines the TEG capability of the target device.

-- ASN1START

NR-UE-TEG-Capability-r17 ::= SEQUENCE {

nr-UE-TEG-ID-CapabilityBandList-r17		SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

NR-UE-TEG-ID-CapabilityPerBand-r17	OPTIONAL,

...

}

NR-UE-TEG-ID-CapabilityPerBand-r17 ::= SEQUENCE {

freqBandIndicatorNR-r17				FreqBandIndicatorNR-r16,

nr-UE-RxTEG-ID-MaxSupport-r17		ENUMERATED {n1, n2, n3, n4, n6, n8}			OPTIONAL,

nr-UE-TxTEG-ID-MaxSupport-r17		ENUMERATED {n1, n2, n3, n4, n6, n8}			OPTIONAL,

nr-UE-RxTxTEG-ID-MaxSupport-r17		ENUMERATED {n1, n2, n4, n6, n8, n12, n16,

n24, n32, n36, n48, n64}		OPTIONAL,

measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17

ENUMERATED {n2, n3, n4, n6, n8}				OPTIONAL,

measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul-r17

ENUMERATED {n1, n2, n3, n4, n6, n8}			OPTIONAL,

...

}

-- ASN1STOP

NR-UE-TEG-Capability field descriptions

nr-UE-RxTEG-ID-MaxSupport Indicates the maximum number of UE-RxTEGs, which is supported and reported by the UE. This field is applicable for UE assisted DL-TDOA and Multi-RTT positioning. The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList and any of maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer, maxNrOfTRP-AcrossFreqs, maxNrOfPosLayer, maxNrOfDL-PRS-ResourcesPerResourceSet and maxNrOfDL-PRS-ResourcesPerPositioningFrequencylayer. Otherwise, the UE does not include this field. NOTE: A single value is reported when both Multi-RTT and DL-TDOA are supported.

nr-UE-TxTEG-ID-MaxSupport Indicates the maximum number of UE-TxTEGs, which is supported and reported by the UE. This field is applicable for Multi-RTT and UL-TDOA positioning. For UL-TDOA, the UE can include this field only if the UE supports supportedSRS-PosResources defined in TS 38.331 [35]. For Multi-RTT, the UE can include this field only if the UE supports maxNrOfDL-PRS-ResourcesPerResourceSet, maxNrOfDL-PRS-ResourcesPerPositioningFrequencylayer and supportedSRS-PosResources defined in TS 38.331 [35]. Otherwise, the UE does not include this field.

nr-UE-RxTxTEG-ID-MaxSupport Indicates the maximum number of UE-RxTxTEGs, which is supported and reported by the UE. This field is applicable for Multi-RTT positioning. The UE can include this field only if the UE supports maxNrOfDL-PRS-ResourcesPerResourceSet, maxNrOfDL-PRS-ResourcesPerPositioningFrequencylayer and supportedSRS-PosResources defined in TS 38.331 [35]. Otherwise, the UE does not include this field.

measureSameDL-PRS-ResourceWithDifferentRxTEGs Indicates the maximum number of different UE-RxTEGs that a UE can support to measure the same DL-PRS Resource of a TRP. This field is applicable for UE assisted DL-TDOA and Multi-RTT positioning. The UE can include this field only if the UE supports nr-UE-RxTEG-ID-MaxSupport. Otherwise, the UE does not include this field. NOTE: If the UE supports nr-UE-RxTxTEG-ID-MaxSupport and measureSameDL-PRS-ResourceWithDifferentRxTEGs, the enumerated value of this field also corresponds to the maximum number of different UE-RxTx TEGs for measuring the same DL PRS Resource of a TRP with the same UE Tx TEG.

measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul Indicates the maximum number of UE Rx TEGs for measuring the same DL-PRS Resource simultaneously. This field is applicable for UE assisted DL-TDOA and Multi-RTT positioning. The UE can include this field only if the UE supports measureSameDL-PRS-ResourceWithDifferentRxTEGs. Otherwise, the UE does not include this field. NOTE: If the UE supports nr-UE-RxTxTEG-ID-MaxSupport and measureSameDL-PRS-ResourceWithDifferentRxTEGs and measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul, the enumerated value of this field also corresponds to the maximum number of different UE-RxTx TEGs for measuring the same DL PRS Resource simultaneously with the same UE Tx TEG.