3GPP TSG RAN meeting #104									           RP-240925

Shanghai, China, June 17-20th , 2024

## Status Report to TSG

Title:			Status report for New WID: Inter RAT mobility support from E-UTRAN TN to NR-NTN; rapporteur: Thales, CATT

Agenda item:			10.9.2

| WI / SI Name | Rel-19 Inter RAT mobility support from E-UTRAN TN to NR-NTN |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | LTE_TN_NR_NTN_mob |  |  |  |  |
| Unique ID | 1031076 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-240846 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2025 | Performance part: - | Testing part: |  |
| Overall Completion level | Study Item: N/A | Core part: Overall: 70% RAN2: 70% | Performance Part: - | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Flavien Ronteix-Jacquet |
|  | Company | Thales |
|  | Email | flavien.ronteix-jacquet@thalesaleniaspace.com |
|  | Name | Xiao XIAO |
|  | Company | CATT |
|  | Email | xiaoxiao@catt.cn |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

-

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN2

#### 2.2.1	Agreements

#### 2.1.1.1 Decisions during RAN2#125bis

##### 2.1.1.1.1 LTE to NR NTN mobility

Agreement:

1.	For idle mode mobility from LTE to NR NTN, at least normal LTE UE are in scope. Can come back in the next meeting to check if also eMTC UE and NB-IoT UEs could also be considered in scope

Working Assumption:

2.	We don’t introduce multiple SMTCs in LTE

#### 2.1.1.2 Decisions during RAN2#126

##### 2.1.1.2.1 LTE to NR NTN mobility

Agreements:

1.	For idle mode mobility from EUTRA TN to NR NTN, NB-IoT UEs are considered not in the scope.

2.	For idle mode mobility from EUTRA TN to NR NTN, we don’t consider specific optimizations for BL UEs and UEs in CE.

3.	SIB24 is reused to provide the NR NTN cell reselection related information (e.g. frequency information, SMTC config, etc.), introducing a satellite ID list in per frequency. The EUTRA cell provides the satellite assistance information for NR neighbor cell per satellite, as identified by the satellite ID.

4.	To support the idle mode mobility from EUTRA TN to NR NTN, the satellite assistance information for NR NTN neighbor cells is needed and should include the following parameters:

-	Satellite ephemeris information

-	TA common information

-	k-Mac

-	epoch time

-	validity duration

-	ntn-PolarizationDL (FFS if mandatory or optional)

5.	The Ephemeris information/epoch time/k-mac/validity duration IEs defined in SIB33 specified in TS36.331 should be reused for NR satellite assistance information.

6.	The signalling format for ntn-PolarizationDL and TA common related configurations within NTN-Config specified in TS38.331 should be introduced in TS36.331 for NR satellite assistance information.

7.	RAN2 will decide in the next meeting which of the following options to adopt for the provision of the NR satellite assistance information (based on TPs provided by the WI RRC Rapporteur):

Option 1: Introduce a new SIB to include the NR satellite assistance information.

Option 2: Define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33.

Option 3: Extend the NeighSatelliteInfo defined for IoT NTN to include the parameters needed for NR satellite, and reuse the neighSatelliteInfoList defined in SIB33 to provide either NR or IoT NTN information.

8.	Introduce the clarification in the field description of measTimingConfig (configured via SIB24 in TS 36.331) that it is configured based on the assumption that the gNB-UE propagation delay equals to 0 ms, and UE can adjust the offset based on the actual propagation delay, when the corresponding frequency is associated with a satellite ID.

Working Assumption:

1.	NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell (can come back in the next meeting to see if the WA can be confirmed)

#### 2.2.2	Remaining Open issues

Definition of necessary access layer features enabling support of idle mode mobility based on cell reselection from E-UTRA TN to NR NTN, where E-UTRA TN provides satellite information for NR NTN neighbor cells in a System Information Block.

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SA2

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

## 4.1	RAN2

RAN2#125bis meeting, Changsha, China, April 15 - 19, 2024:

- R2-2402154	discussion	Support of LTE TN to NR NTN mobility	China Telecom

- R2-2402195	discussion	Discussion on LTE to NR NTN idle mode mobility	OPPO

- R2-2402221	discussion	Discussion on LTE TN to NR NTN Mobility	vivo

- R2-2402545	discussion	Discussion on idle mode mobility enhancements for E-UTRAN TN to NR-NTN	CMCC

- R2-2402809	discussion	Idle mode mobility from LTE to NR NTN	Qualcomm Incorporated

- R2-2402827	discussion	Discussion on LTE TN to NR NTN mobility	Huawei, HiSilicon, Turkcell

- R2-2402834	discussion	Discussion on the cell reselection from LTE to NR NTN	Xiaomi

- R2-2402885	discussion	Mobility from LTE TN to NR NTN	Apple

- R2-2403035	discussion	Support of Idle Mode Mobility from EUTRA TN to NR NTN	CATT

- R2-2403066	discussion	Support for LTE to NR-NTN idle mode mobility	Telit Communications S.p.A. ; Thales

- R2-2403073	discussion	Consideration on idle mode mobility between LTE TN and NR NTN	ZTE Corporation, Sanechips

- R2-2403123	discussion	Discussion on support of LTE to NR NTN cell reselection	LG Electronics France

- R2-2403205	discussion	Discussion on LTE to NR NTN mobility	Interdigital, Inc.

- R2-2403226	discussion	Discussion on cell reselection from E-UTRA TN to NR NTN	MediaTek Inc.

- R2-2403307	discussion	On E-UTRA TN to NR NTN Mobility in IDLE mode	Nokia

- R2-2403339	discussion	E-UTRAN TN to NR NTN mobility	Samsung

- R2-2403640	discussion	E-UTRAN TN to NR-NTN mobility	Ericsson

RAN2#126 meeting, Fukuoka city, Japan, May 20 - 24, 2024:

- R2-2405630	discussion	Discussion for LTE to NR NTN mobility	Sharp

- R2-2405314	discussion	Consideration of LTE TN to NR NTN mobility	China Telecom

- R2-2405210	discussion	Discussion on support of LTE to NR NTN cell reselection	LG Electronics France

- R2-2404759	discussion	Discussion on cell reselection from E-UTRA TN to NR NTN	MediaTek Inc.

- R2-2405155	discussion	E-UTRAN TN to NR NTN mobility basic scenario and signalling	Samsung

- R2-2405127	discussion	Discussion on LTE to NR NTN mobility	Huawei, HiSilicon, Turkcell

- R2-2405146	discussion	On How to Address E-UTRA TN to NR NTN Mobility in IDLE mode	Nokia

- R2-2404986	discussion	Further discussion on idle mode cell reselection form LTE to NR NTN	Transsion Holdings

- R2-2405084	discussion	Consideration on idle mode mobility between LTE TN and NR NTN	ZTE Corporation, Sanechips

- R2-2405101	discussion	Discussion on the cell reselection from LTE to NR NTN	Xiaomi

- R2-2405108	discussion	Discussion on LTE to NR NTN mobility	Interdigital, Inc.

- R2-2405022	discussion	Considerations on cell reselection enhancements from E-UTRAN TN to NR-NTN	CMCC

- R2-2404800	discussion	On LTE to NR-NTN IDLE mobility	Lenovo

- R2-2404840	discussion	E-UTRAN TN to NR-NTN mobility	Ericsson

- R2-2404681	discussion	Idle mode mobility from LTE to NR NTN	Qualcomm Incorporated

- R2-2404656	discussion	Mobility from LTE TN to NR NTN	Apple

- R2-2404591	discussion	Discussion on LTE to NR NTN idle mode mobility	OPPO

- R2-2404162	discussion	Discussion on LTE TN to NR NTN Mobility	vivo

- R2-2404211	discussion	Support of Idle Mode Mobility from EUTRA TN to NR NTN	CATT

- R2-2404198	discussion	Support for LTE to NR-NTN idle mode mobility	Telit Communications S.p.A. ; Thales

END