3GPP TSG RAN meeting #104	RP-240928

Shanghai, China, June 17 – 20, 2024

## Status Report to TSG

Agenda item:	9.4.1.5

| WI / SI Name | NR sidelink evolution |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: No | Performance part: Yes |  | Testing part: No |
| Acronym | NR_SL_enh2 |  |  |  |  |
| Unique ID | 940097 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-232789 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: mm/yyyy | Core part: 12/2023 | Performance part: 06/2024 | Testing part: mm/yyyy |  |
| Overall Completion level | Study Item: xx % | Core part: xx % | Performance Part: 100% | Testing part: xx % |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Kevin Lin |
|  | Company | OPPO |
|  | Email | Kevin.Lin@oppo.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

- RRM performance part

- Progress made in RAN4#110-bis

Sub-topic 1-1: RRM performance requirement for SL CA

Issue 1-1: Whether to specify test for ”Selection/Reselection of synchronization reference source under NR SL CA operation”?

Agreement:

- Do not to introduce the agreed test on “Selection/Reselection of synchronization reference source under NR SL CA operation” due to the testability issue.

Sub-topic 1-2: RRM performance requirement for SL-U

Issue 2-1: RSSI accuracy requirements

Agreement:

- Do not define RSSI measurement accuracy requirements for SL-U LBT

Issue 2-2: Spec structure for accuracy requirements

Agreement:

- Introduce new sub-clause to capture SL-U measurement accuracy requirements

- Check the exact sub-clause number in the draft CR.

CCA model

Issue 2-3: CCA models for SL-U test cases

Agreement

- Define new CCA models for SL-U.

- Consider following CCA model assumption during test

- If LSLSS is larger than or equal to LSLSS,max during the test, the SLSS is transmitted from TE at the S-SSB occasion

- Number of additional SSB occasions: [1]

- FFS: For SL UE CCA model (DUTTE), PSL_UE can be set by 1 (i.e., no CCA failure) to transmit SLSS during T2 in the test (figure 1).

Issue 2-4: RMC for PSCCH and PSSCH due to CCA

Agreement:

- RAN4 to define new RMCs for PSCCH and PSSCH due to CCA.

The following CRs for RRM performance part were endorsed in RAN4#110bis:

- R4-2404731	Draft CR for measurement accuracy for SL-U

- R4-2406323	Draft CR on new CCA model for SLU

- R4-2404923	Draft CR on new RMC for PSCCH and PSSCH for SLU

- R4-2404923	Draft CR on new RMC for PSCCH and PSSCH for SLU

- R4-2406517	Draft Big CR for RRM performance requirements for NR sidelink evolution

- Progress made in RAN4#111

The following CRs for RRM performance part were endorsed/agreed in RAN4#111:

- R4-2410197	Draft CR on RMC table and clean up TC for NR SL-U

- R4-2410198	Big CR for RRM performance requirements for NR sidelink evolution\ Agreed.

- UE demodulation performance:

- Progress made in RAN4#110-bis

PSSCH performance requirements

Issue 1-1-1: NR sidelink CA Bandwidth combination for PSSCH performance tests

- CA bandwidth combination can be defined as rule in “applicability of requirement” section.

  - Additional section: 11.1.1.3 Applicability of CA requirements

  - Detail contents of additional section can be discussed.

Issue 1-1-2: Number of sub-channels configuration for NR sidelink CA PSSCH test

- Each CC is configured with 2 sub-channels (20RBs), existing requirements. e,g. Test 2 in Table 11.1.2.1.1-1 can be applied without simulation work

Issue 1-1-3: Applicability rule for NR sidelink CA PSSCH test

- Follow the precedence of CA applicability rule in 5.1.1.7.2

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order | PCell CC configuration |
| --- | --- | --- | --- | --- |
| Test 1 in Clause 5.2A.2.1 and 5.2A.3.1 | CA_C, CA_N, CA_AX | Table 5.1.1.7.2-2 | Largest aggregated CA bandwidth combination | Any of CCs |

PSCCH decoding capability test

Issue 1-1-4: Bandwidth combination for PSCCH decoding capability test

- Follow the precedence of CA applicability rule in 5.1.1.7.2

PSCCH decoding capability test

Issue 1-1-6: Bandwidth combination for PSFCH decoding capability test

- Follow the precedence of CA applicability rule in 5.1.1.7.2

Sub-topic 1-2 NR sidelink unlicensed band scenario

Issue 1-2-1: DMRS pattern for PSSCH test in SL-U

- Support DMRS pattern {2,2}.

Issue 1-2-2: Starting point of LBT model

- If NR-U LBT model in 38.101-4 B.5 is assumed to be not suitable for SL-U, RAN4 could consider the LBT model of LAA.

Issue 1-2-3: Starting point for LBT model

- If the LBT model of LAA is used, the value for p_LBT shall be defined. For alignment with 36.133, the value of 0.75 can be considered.

Issue 1-2-4: TE emulation and DUT UE PSFCH transmission related

- No further discussion on this issue

Issue 1-2-5: Number of slots for transmission burst and muting method

- Use following LBT model.

1) Select the number of slots N randomly from a given set of the number of slots S1={[2,4,8]} with equal probability as the total length of burst transmission format. 2) uniform random variable from [0, 1] is generated. - If the random variable is less than p which is given per test case, - Start burst transmission with length selected in 1) at the end of last slot of previous burst. - Otherwise, the burst transmission is muted and the muting duration is the same as the number of slots for determined burst format. Note: Each slot is fully occupied. (Start from symbol 0, end at symbol 13)

Issue 1-2-6: CPE in Gap

- Do not consider COT sharing. So, For Gap1, Type1 LBT can be applied.

Issue 1-2-7: Resource pool configuration

- Table 2-3: Proposed additional resource pool configuration

| Resource pool configuration | Parameter | Value |
| --- | --- | --- |
|  | Number of interlaces per sub-channel within a resource pool (sl-NumInterlacePerSubchannel-r18) | 1 |
|  | Reference number of PRBs of one interlace within 1 RB set for TBS determination (sl-NumReferencePRBs-OfInterlace) | 11 |
|  | Reference number of symbols for TBS determination. (sl-NumRefSymbolLength) | 14 |
|  | CPE starting position within the GP symbol before PSFCH transmission (sl-CPE-StartingPositionPSFCH) | 16 us after starting of the gap symbol before PSFCH transmisson |
|  | PSFCH transmission structure (sl-TransmissionStructureForPSFCH-r18) | CommonInterlace |
|  | Number of dedicated PRBs for PSFCH (sl-NumDedicatedPRBs-ForPSFCH-r18) | 1 |
|  | The common interlace index for PSFCH (sl-PSFCH-CommonInterlaceIndex-r18) | 0 |
|  | Number of PSFCH occasiopns corresponding to one PSSCH/PSCCH transmission. (sl-NumPSFCH-Occasions-r18) | 1 |

- Follow the legacy SL test

- Regarding the CPE before PSFCH transmission, it is not necessary because of Type 1 LBT operation for PSFCH transmission.

Sub-topic 1-3 CR works

Issue 1-3-1: Spec structure for SL-CA demodulation performance requirements

- Additional section lists

11.1.1.1.3	Applicability of requirements for CA  LGE

11.1.1.1.4	Applicability of requirements for PSSCH with shared spectrum access  Huawei

11.1.2.1.2 Minimum requirements for PSSCH with shared spectrum access  Huawei

11.1.2A PSSCH demodulation requirements for CA  LGE

11.1.8A PSCCH decoding capability test for CA  Qualcomm

11.1.9A PSFCH decoding capability test for CA  Nokia

B.5.2 Sidelink Transmission Model for bands with shared spectrum access  Huawei

- Affected section lists

11.1.1.1.2 Applicability of requirements for mandatory UE V2X features with capability signalling  Huawei

A.6.2.2 Reference measurement channels for SCS 30 kHz FR1  Huawei

B.5 Transmission Model for requirements on bands with shared spectrum access  Huawei

- Progress made in RAN4#111

Issue 1-1-1: Test setup for PSCCH decoding capability test

- Where Nk is the number of subchannels in the bandwidth defined in the test; the number xk is derived by first distributing the number of PSFCH(s) resources that the tested UE can transmit in a slot (i.e. [IE psfch-TxNumber] specified in clause 4.2.16.1.6 of  TS 38.306 [14]) across all carriers proportionally to the number of subchannels in each carrier, and carrier k gets yk, yk is an integer, then xk = min(yk , Nk). Note that summation of yk is [IE psfch-TxNumber] and xk should be larger than or equal to 1

Issue 1-1-2: Test setup for PSFCH decoding capability test

- The minimum requirements are specified in Table 11.1.9A.1.1-2 with the test parameters specified in Table 11.1.9A.1.1-1 and the test procedure specified as follows for each carrier with index k:

- -	Based on the selected Option, the UEs transmit PSFCHs to the tested UE per slot on xk PSFCH resources in each carrier index k.

- Where the number xk is derived by evenly distributing the number of PSFCH(s) resources that the tested UE can receive in a slot (i.e. [IE psfch-RxNumber] specified in clause 4.2.16.1.6 of TS 38.306 [14]) across all carriers. Note that summation of xk is [IE psfch-RxNumber]

#### 2.4.2	Remaining Open issues

N/A

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4#110-bis

- R4-2404212	Sidelink enhancement RRM performance discussion	Qualcomm Incorporated

- R4-2404730	Discussion on RRM performance for NR sidelink evolution	LG Electronics Inc.

- R4-2404731	Draft CR for measurement accuracy for SL-U	LG Electronics Inc.

- R4-2404921	Discussion on performance requirements for Rel-18 SL	OPPO

- R4-2404922	Draft CR on new CCA model for SLU	OPPO

- R4-2404923	Draft CR on new RMC for PSCCH and PSSCH for SLU	OPPO

- R4-2405601	Discussion on RRM test cases for R18 SL enhancement	Huawei, HiSilicon

- R4-2405602	DraftCR on Initiation/Cease of SLSS transmission tests for SL-U operation	Huawei, HiSilicon

- R4-2405816	RRM Performance Requirements for NR Sidelink Evolution	Nokia

- R4-2406323	Draft CR on new CCA model for SLU	OPPO

- R4-2406431	DraftCR on Initiation/Cease of SLSS transmission tests for SL-U operation	Huawei, HiSilicon

- R4-2406517	Draft Big CR for RRM performance requirements for NR sidelink evolution	LG Electronics, OPPO

- R4-2404213	Sidelink enhancement demod discussion	Qualcomm Incorporated

- R4-2404214	Sidelink demod PDCCH decoding capability draft CR	Qualcomm Incorporated

- R4-2404481	Draft CR for PSSCH demodulation requirements for SL-CA based on single carrier performance	LG Electronics Inc.

- R4-2404482	Simulation results for SL-U PSSCH demodulation	LG Electronics Inc.

- R4-2404483	Simulation results collection for SL-U PSSCH demodulation	LG Electronics Inc.

- R4-2405158	draft CR Introduction of PSSCH demodulation requirements for SL-U	Huawei,HiSilicon

- R4-2405159	Sidelink demodulation discussion	Huawei,HiSilicon

- R4-2405789	Draft CR on PSFCH Decodng Capability for Sidelink CA	Nokia

- R4-2405790	NR Sidelink Evolution: UE Demodulation Performance Requirements	Nokia

- R4-2406046	Sidelink demod PDCCH decoding capability draft CR	Qualcomm Incorporated

- R4-2406047	Draft CR for PSSCH demodulation requirements for SL-CA based on single carrier performance	LG Electronics Inc.

- R4-2406048	draft CR Introduction of PSSCH demodulation requirements for SL-U	Huawei,HiSilicon

- R4-2406049	Draft CR on PSFCH Decodng Capability for Sidelink CA	Nokia

- R4-2404835	Topic summary for [110bis][224] NR_SL_enh2	Moderator (OPPO)

- R4-2405277	Topic summary for [110bis][124] NR_SL_enh2_UERF_R18	Moderator(LGE)

- R4-2405848	Topic summary for [110bis][329] NR_SL_enh2_demod	Moderator (LGE)

- R4-2406012	Offline meeting minutes for [110bis][329] NR_SL_enh2_demod	LGE

- R4-2406050	Way Forward for [110bis][329] NR_SL_enh2_demod	LGE

- R4-2406051	Draft BigCR Sidelink enhancment demod performance	LGE

- R4-2406598	WF on SL enhancement for remaining NS values	LGE

RAN4#111

- R4-2407962	Draft CR on RMC table and clean up TC for NR SL-U	LG Electronics Inc.

- R4-2407963	Big CR for RRM performance requirements for NR sidelink evolution	LG Electronics Inc., OPPO

- R4-2410197	Draft CR on RMC table and clean up TC for NR SL-U	LG Electronics Inc.

- R4-2407327	Discussion on UE demodulation performance for NR SL evolution	LG Electronics Inc.

- R4-2407328	Draft CR to TS38.101-4 on Sidelink enhancement demod performance	LG Electronics Inc.

- R4-2407329	Big CR to TS38.101-4 on Sidelink enhancement demod performance	LG Electronics

- R4-2407332	Sidelink enhancement demod discussion	Qualcomm Incorporated

- R4-2408058	Draft CR on PSFCH Decodng Capability for Sidelink CA	Nokia

- R4-2408059	NR Sidelink Evolution: UE Demodulation Performance Requirements	Nokia

- R4-2409011	Discussion on Sidelink demodulation requirements	Huawei,HiSilicon

- R4-2409012	Draft CR for 38.101-4  Introduction of PSSCH demodulation requirements for SL-U	Huawei,HiSilicon

- R4-2408019	Topic summary for [111][222] NR_SL_enh2	Moderator (OPPO)

- R4-2408933	Topic summary for [111][122] NR_SL_enh2_UERF_R18	Moderator(LGE)

- R4-2409822	Offline meeting minutes for [111][329] NR_SL_enh2_demod	LGE

- R4-2410122	Topic summary for [111][329] NR_SL_enh2_demod	Moderator (LGE)

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template