3GPP TSG RAN Meeting #104	RP- 240937

Shanghai, China, June 17-20, 2024

Agenda item:	9.3.1.2

Source: 	New H3C

Title: 	Duplex WID Revision

Document for:	Discussion and Decision

- Introduction

The NR duplex enhancement (SBFD) WID [1] captured the work item as follows:

“Study and specify, if justified, SBFD operation to UE in RRC_IDLE/INACTIVE mode for random access [RAN1, RAN2]

•	RAN#104 to check whether to proceed normative work”.

This paper discussed about current status of RAN1 progress and share our view with SBFD operation to UE in RRC_IDLE/INACTIVE mode.

- Discussions

The Subband full duplexing WID [1] aims to study feasibility of SBFD operation to UE in RRC_IDLE/INACTIVE mode for random access. In terms of RAN1 evaluation result and decision [2], companies offered evaluation results to show benefit and performance gain of this feature and supporting SBFD operation to UE in RRC_IDLE/INACTIVE mode was approved by RAN1 WG in RAN1#117 meeting as follows:

Agreement Support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode.

From our perspective, SBFD operation to UE in RRC_IDLE/INACTIVE mode should be supported in Rel-19. RAN TSG should discuss and decide to support SBFD operation to UE in RRC_IDLE/INACTIVE mode in RAN#104. In this sense, we suggest as follows:

Proposal 1: RAN WG should discuss and decide to support SBFD operation to UE in RRC_IDLE/INACTIVE mode in RAN#104.

Proposal 2: Update WID objective related to UL subband as follows:

  - Study and Sspecify, if justified, SBFD operation to UE in RRC_IDLE/INACTIVE mode for random access [RAN1, RAN2]

    - RAN#104 to check whether to proceed normative work”.

- Conclusion

We propose the followings:

Proposal 1: RAN WG should discuss and decide to support SBFD operation to UE in RRC_IDLE/INACTIVE mode in RAN#104.

Proposal 2: Update WID objective related to UL subband as follows:

  - Study and Sspecify, if justified, SBFD operation to UE in RRC_IDLE/INACTIVE mode for random access [RAN1, RAN2]

    - RAN#104 to check whether to proceed normative work”.

References

[1] RP-240789, New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

[2] Chairman’s Notes, RAN1#117, final, May 20th – 24th, 2024

Appendix – Objectives of SBFD

4.1	Objective of Core part WI

The objectives are as follows:

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier:

  - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2]

    - Indication of time location of SBFD subbands in SIB is not precluded

  - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2]

    - Indication of frequency domain location of SBFD subbands in SIB is not precluded

  - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC CONNECTED mode [RAN1, RAN2]

  - Study and specify, if justified, SBFD operation to UE in RRC_IDLE/INACTIVE mode for random access [RAN1, RAN2]

    - RAN#104 to check whether to proceed normative work

  - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2]

    - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon

      - UL transmissions within UL subband only

      - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands

Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols

    - Enhancement on resource allocation in frequency domain in SBFD symbols, including

      - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols

      - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG

    - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including

      - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots

      - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots

    - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation

    - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol

  - Followings are assumed based on TR 38.858

    - SBFD at the gNB side

    - Half duplex operation at the UE side

    - FR1 and FR2-1

    - SBFD operation Option 4, i.e., both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs

    - Coexistence between non-SBFD aware UEs (including legacy UEs) and SBFD aware UEs in the cell operating SBFD at gNB side

    - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies

    - One UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol/slot) within a TDD carrier

    - Mechanisms for SBFD operation shall also consider the adjacent channel coexistence between two operators

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3]:

  - Support gNB-to-gNB CLI handling scheme(s) (the detailed schemes are to be down-selected from those in TR38.858 by RAN1#117)

  - Support UE-to-UE CLI handling scheme(s) (the detailed schemes are to be down-selected from those in TR38.858 by RAN1#117)

  - Note: Without dedicated optimization for dynamic/flexible TDD.

- Specify BS RF requirements for SBFD operation at gNB [RAN4]

- Specify applicable RRM core requirements for CLI handling mechanisms [RAN4]

- Specify other RRM core requirements for SBFD operation, if identified [RAN4]

- Note: RAN3 will not specify enhancements to network signalling to support inter-operator coordination for CLI handling