3GPP TSG-RAN5 Meeting #103	R5-243134

# Fukuoka, Japan, 20 May – 24 May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2821 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR V2X MOP for inter-band concurrent operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The NR V2X MOP for inter-band concurrent operation is still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Aligning the minimum requirements - Adding the test case of inter-band concurrent. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2E.1.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 0906 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.2E.1	UE maximum output power for V2X

6.2E.1.0	Minimum conformance requirements

6.2E.1.0.1	General

When NR V2X UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 5.2E.1-1, the allowed NR V2X UE maximum output power is specified in Table 6.2.1.3-1 in clause 6.2.1.

When a UE is configured for NR V2X sidelink transmissions in NR Band n47, the V2X UE shall meet the following additional requirements for transmission within the frequency ranges 5855-5925 MHz:

-	The maximum mean power spectral density shall be restricted to 23 dBm/MHz EIRP when the network signalling value NS_33 is indicated.

where the network signalling values are specified in clause 6.2E.3.0.

NOTE:	The PSD limit in EIRP shall be converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I in [11].

For NR V2X UE supporting SL MIMO, the maximum output power requirements in Table 6.2E.1.0.1-1 shall be met with the SL MIMO configurations specified in Table 6.2D.1.3-2. The maximum output power is defined as the sum of the maximum output power from each UE antenna connector. The period of measurement shall be at least one sub frame (1 ms).

Table 6.2E.1.0.1-1: NR V2X UE Power Class for SL-MIMO

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n38 |  |  |  |  | 23 | +2/-3 |  |  |
| n47 |  |  |  |  | 23 | +2/-3 |  |  |

If the UE transmits on one antenna connector at a time, the requirements in Table 6.2.1.3-1 shall apply to the active antenna connector.

6.2E.1.0.2	UE maximum output power for V2X con-current operation

For the NR V2X inter-band con-current operation, the maximum output power is specified in Table 6.2E.1.0.2-1 for each operating band. The period of measurement shall be at least one sub frame (1ms).

Table 6.2E.1.0.2-1: Power Class for NR V2X inter-band con-current combination (two bands)

| NR V2X con-current operating band Configuration | NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X_n71A-n47A | n71 |  |  |  |  | 23 | +2/-34 |  |  |
|  | n47 |  |  |  |  | 23 | +2/-3 |  |  |
| NOTE 1: For the con-current band combinations, the simultaneous transmission and reception of sidelink and Uu interfaces can be supported while operation is agnostic of the service used on each interface. NOTE 2: PPowerClass is the maximum output power specified without taking into account the tolerance for each operating band. NOTE 3: For inter-band con-current operation, the aggregation power apply to the total transmitted power over all component carriers (per UE). NOTE 4: 4 refers to the transmission bandwidths (Figure 5.6-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2E.1

6.2E.1.1	UE maximum output power for V2X / non-concurrent operation

6.2E.1.1.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2E.1.1.2	Test applicability

The requirements in this test are not testable due to lack of appropriate test points since there’s no configuration satisfying MPR=0dB requirements in TS 38.101-1.

No test case details are specified.

6.2E.1.1D	UE maximum output power for V2X / non-concurrent operation / SL-MIMO

6.2E.1.1D.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2E.1.1D.2	Test applicability

The requirements in this test are not testable due to lack of appropriate test points since there’s no configuration satisfying MPR=0dB requirements in TS 38.101-1.

No test case details are specified.

6.2E.1.2	UE maximum output power for V2X / concurrent operation

6.2E.1.2.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2E.1.2.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR V2X sidelink communication.

6.2E.1.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2E.1.0.

6.2E.1.2.4	Test description

6.2E.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2E.2-1 and Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2E.1.2.4.1-1. The details of the V2X reference measurement channels (RMCs) are specified in Annexes A.7.5. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2E.1.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range for NR Uu and V2X carrier High range for NR Uu and V2X carrier |  |  |  |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Lowest for NR Uu and V2X carrier Highest for NR Uu and V2X carrier |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest for NR Uu and V2X carrier Highest for NR Uu and V2X carrier |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |
|  | NR Uu Configuration |  |  |  | NR V2X Configuration to Transmit |  |  |
|  | Freq | Downlink | Uplink Configuration |  | Freq | Modulation | PSCCH |
|  |  | Configuration | Modulation | RB allocation (NOTE 1) |  |  | and PSSCH RB allocation (Note 2) |
| 1 | Default | N/A | DFT-s-OFDM QPSK | Inner Full | Default | CP-OFDM QPSK | Inner_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1E-1. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.9.3 for TE diagram and clause A.3.2.7 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] clause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The V2X Reference Measurement Channel and NR UL Reference Measurement Channel are set according to Table 6.2E.1.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameter Connectivity NR, Connected without release on, Test Mode On and Sidelink On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2E.1.2.4.3.

6.2E.1.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2E.1.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	SS sends sidelink scheduling information for each SL-HARQ process via PDCCH DCI format 3_0 for C_RNTI to schedule the Sidelink RMC according to Table 6.2E.1.2.4.1-1. UE is configured to transmit at Pcmax on the sidelink carrier. Since the UE has no payload and no loopback data to send the UE sends MAC padding bits on the Sidelink RMC.

4.	Measure the sum of the mean power of the UE at NR Uu carrier and V2X sidelink carrier in the channel bandwidth according to the test configuration from Table 6.2E.1.2.4.1-1. The period of measurement shall be at least continuous duration of one active sub-frame (1ms) excluding guard symbols.

6.2E.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] clause 4.6.

Table 6.2E.1.2.4.3-1: SL-ResourcePool

| Derivation Path: TS 38.508-1 [5], Table 4.6.6-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-PSCCH-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sl-TimeResourcePSCCH-r16 | As defined in Table 6.1E-2 |  |  |
| sl-FreqResourcePSCCH-r16 | As defined in Table 6.1E-2 |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-SubchannelSize-r16 | As defined in Table 6.1E-2 |  |  |
| sl-StartRB-Subchannel-r16 | As defined in Table 6.1E-2 |  |  |
| sl-NumSubchannel-r16 | As defined in Table 6.1E-2 |  |  |
| sl-PowerControl-r16 SEQUENCE { |  |  |  |
| sl-MaxTransPower-r16 | 24 |  |  |
| sl-Alpha-PSSCH-PSCCH-r16 | Not present |  |  |
| dl-Alpha-PSSCH-PSCCH-r16 | Not present |  |  |
| sl-P0-PSSCH-PSCCH-r16 | Not present |  |  |
| dl-P0-PSSCH-PSCCH-r16 | Not present |  |  |
| dl-Alpha-PSFCH-r16 | Not present |  |  |
| dl-P0-PSFCH-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

6.2E.1.2.5	Test requirement

The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2E.1.2.5-1.

Table 6.2E.1.2.5-1: UE MPR test requirement for inter-band con-current NR V2X operation

|  | PPowerClass,NR (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,c, NR (dBm) | PCMAX_H,c, NR (dB) | PPowerClass,V2X (dBm) | PCMAX_H,f,c,V2X (dBm) | PCMAX_L (dBm) | PCMAX_H (dBm) | T(PCMAX_L) (dB) | T(PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0 | 23 | 23 | 23 | 23 | 23 | 26 | 3 | 2 | 28+TT | 20-TT |

Table 6.2E.1.2.5-2: Test Tolerance for inter-band con-current NR V2X operation

|  | f ≤ 3.0GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB |

6.2E.2	UE maximum output power reduction for V2X

6.2E.2.0	Minimum conformance requirements

6.2E.2.0.1	General

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 5.2E.1-1, this clause specifies the allowed Maximum Power Reduction (MPR) power for V2X physical channels and signals due to PSCCH/PSSCH, PSFCH and S-SSB transmission.

< End of Changes >