3GPP TSG-RAN5 Meeting #103	R5-242584

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4382 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for V2X TC 12.1.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The corresponding step for Table 12.1.2.2.3.3-6 in specific message contents is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. The step number for Table 12.1.2.2.3.3-6 has been corrected. 2. Some editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors will exist in the TC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

12.1.2.2	PC5-only operation / Sidelink synchronization related procedure / SL-SSB transmission Initiation and Cease

12.1.2.2.1	Test Purpose (TP)

(1)

with { UE configured by upper layer to perform sidelink transmission }

ensure that {

when { UE selects GNSS as synchonization reference source }

then { UE keeps transmitting SL-SSB }

}

(2)

with { UE configured by upper layer to perform sidelink transmission and configured with syncTxThreshOoC in pre-configuration }

ensure that {

when { UE selects SyncRef UE as synchronization reference source and PSBCH-RSRP of the SyncRef UE is lower than syncTxThreshOoC }

then { UE starts transmitting SL-SSB }

}

(3)

with { UE configured by upper layer to perform sidelink transmission and configured with syncTxThreshOoC in pre-configuration }

ensure that {

when { UE selects SyncRef UE as synchronization reference source and PSBCH-RSRP of the SyncRef UE is above syncTxThreshOoC }

then { UE stops transmitting SL-SSB }

}

(4)

with { UE configured by upper layer to perform sidelink transmission and configured with syncTxThreshOoC in pre-configuration }

ensure that {

when { UE selects SyncRef UE as synchronization reference source and PSBCH-RSRP of the SyncRef UE is below syncTxThreshOoC }

then { UE starts transmitting SL-SSB }

}

12.1.2.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 [22], subclause 5.8.2. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.8.2]

The UE shall perform NR sidelink communication operation only if the conditions defined in this clause are met:

…

1>	if the UE has no serving cell (RRC_IDLE);

[TS 38.331, clause 5.8.5.1]

Figure 5.8.5.1-1: Synchronisation information transmission for NR sidelink communication, in (partial) coverage

Figure 5.8.5.1-2: Synchronisation information transmission for NR sidelink communication, out of coverage

The purpose of this procedure is to provide synchronisation information to a UE.

[TS 38.331, clause 5.8.5.2]

A UE capable of NR sidelink communication and SLSS/PSBCH transmission shall, when transmitting NR sidelink communication, and if the conditions for NR sidelink communication operation are met and when the following conditions are met:

…

1>	else:

2>	for the frequency used for NR sidelink communication, if syncTxThreshOoC is included in SidelinkPreconfigNR; and the UE is not directly synchronized to GNSS, and the UE has no selected SyncRef UE or the PSBCH-RSRP measurement result of the selected SyncRef UE is below the value of syncTxThreshOoC; or

2>	for the frequency used for NR sidelink communication, if the UE selects GNSS as the synchronization reference source:

3>	transmit sidelink SSB on the frequency used for NR sidelink communication in accordance with TS 38.211 [16], including the transmission of SLSS as specified in 5.8.5.3 and transmission of MasterInformationBlockSidelink as specified in 5.8.9.4.3;

12.1.2.2.3	Test description

12.1.2.2.3.1	Pre-test conditions

System Simulator:

-	NR-SS-UE

-	NR-SS-UE 1 operating as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

-	NR-SS-UE 1 transmits SL-SSB with SLSSID = 0, inCoverage = true in slots determined by sl-SSB-TimeAllocation1 and GNSS timing.

-	GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

-	UE is authorised to perform NR sidelink communication.

-	The UE is equipped with below information in UE or in a USIM containing default values (as per TS 38.508-1 [4]) except for those listed in Table 12.1.2.2.3.1-1.

Table 12.1.2.2.3.1-1: UE/ USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFUST |  | As per TS 36.508 [18] clause 4.9.3.4 |  |
| EFVST |  | Service n°119 is "available" |  |
| EFV2XP_PC5 |  | As per TS 38.508-1[4] clause 4.8.3.3.3 SL-PreconfigurationNR included in V2X data policy over PC5 is defined in Table 12.1.2.2.3.3-1 Table 12.1.2.2.3.3-1A |  |

Preamble:

-	The UE is in state 4-A as defined in TS 38.508-1 [4], subclause 4.4A, using generic procedure parameter Sidelink (On), test mode (On) and GNSS Sync (On) as defined in TS 38.508-1 [4], subclause 4.5.1.

12.1.2.2.3.2	Test procedure sequence

Table 12.1.2.2.3.2-1 illustrates the sidelink power levels to be applied for NR-SS-UE 1 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while the configuration marked "T1" and "T2", are applied at the point indicated in the Main behaviour description in Table 12.1.2.2.3.2-2.

Table 12.1.2.2.3.2-1: Time instances of NR-SS-UE power level and parameter changes in conducted test environment

|  | Parameter | Unit | NR-SS-UE 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | NR-SS-UE power | dBm/ SCS | OFF | NR-SS-UE 1 is powered off. |
|  | EPRE ratio of S-SSS to NR-SS-UE power | dB | - |  |
| T1 | NR-SS-UE power | dBm/ SCS | -106 | The power level to ensure that PSBCH-RSRP of NR-SS-UE 1 is lower than syncTxThreshOoC |
|  | EPRE ratio of S-SSS to NR-SS-UE power | dB | 0 |  |
| T2 | NR-SS-UE power | dBm/ SCS | -94 | The power level to ensure that PSBCH-RSRP of NR-SS-UE 1 is higher than syncTxThreshOoC |
|  | EPRE ratio of S-SSS to NR-SS-UE power | dB | 0 |  |

Table 12.1.2.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS triggers UE to close UE test loop mode E (Transmit Mode). NOTE: Closing of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| 1A | The UE starts broadcasting continuously. | - | - | - | - |
| 2 | The SS waits 10 seconds | - | - | - | - |
| 3 | Check: Does the UE transmit SL-SSBs in slots determined by sl-SSB-TimeAllocation1 and GNSS timing? | - | - | 1 | P |
| 4 | The SS powers off GNSS simulator. | - | - | - | - |
| 5 | The SS re-adjusts the NR-SS-UE power level according to row "T1" in table 12.1.2.2.3.2-1. | - | - | - | - |
| 6 | The SS waits 10 seconds | - | - | - | - |
| 7 | Check: Does the UE transmit SL-SSBs in slots determined by sl-SSB-TimeAllocation2 and NR-SS-UE 1 timing? | - | - | 2 | P |
| 8 | The SS re-adjusts the NR-SS-UE power level according to row "T2" in table 12.1.2.2.3.2-1. | - | - | - | - |
| 9 | The SS waits 10 seconds | - | - | - | - |
| 10 | Check: Does the UE transmit SL-SSBs in the next 1s? | - | - | 3 | F |
| 11 | The SS re-adjusts the NR-SS-UE power level according to row "T0" in table 12.1.2.2.3.2-1. | - | - | - | - |
| 12 | The SS waits 10 seconds | - | - | - | - |
| 13 | Check: Does the UE transmit SL-SSBs in slots determined by sl-SSB-TimeAllocation2 and NR-SS-UE 1 timing? | - | - | 4 | P |
| 14 | The SS triggers UE to open UE test loop mode E. NOTE: Opening of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |

12.1.2.2.3.3	Specific message contents

Table 12.1.2.2.3.3-1: V2X service identifier to default mode of communication mapping rule (Pre-configuration, UE under test)

| Derivation Path: TS 38.508-1 [4] Table 4.7D.2.2-53 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMC | '10'B | Default mode of communication is set to broadcast |  |

Table 12.1.2.2.3.3-1A: SL-SDAP-Config (Pre-configuration, UE under test)

| Derivation Path: TS 38.508-1 [4], Table 4.6.6-30 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-SDAP-Config-r16 ::= SEQUENCE { |  |  |  |
| sl-CastType-r16 | broadcast |  |  |
| } |  |  |  |

Table 12.1.2.2.3.3-2: +CCUTLE (Step 1, Table 12.1.2.2.3.2-2)

Derivation Path: TS 38.508-1 [4] Table 4.7B-1 with condition Close and Transmit

Table 12.1.2.2.3.3-3: MasterInformationBlockSidelink (Table 12.1.2.2.3.2-2, NR-SS-UE 1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1A-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MasterInformationBlockSidelink ::= SEQUENCE { |  |  |  |
| inCoverage-r16 | true |  |  |
| directFrameNumber-r16 | DFN determined based on the formula given in TS 38.331 [22] clause 5.8.12. |  |  |
| slotIndex-r16 | slot index determined based on the formula given in TS 38.331 [22] clause 5.8.12. |  |  |
| } |  |  |  |

Table 12.1.2.2.3.3-5: MasterInformationBlockSidelink (Steps 3, 7 and 13, Table 12.1.2.2.3.2-2,UE under test)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1A-1 with condition TX |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MasterInformationBlockSidelink ::= SEQUENCE { |  |  |  |
| inCoverage-r16 | true |  | Step 3 |
|  | false |  | Steps 7, 13 |
| directFrameNumber-r16 | DFN determined based on the formula given in TS 38.331 [22] clause 5.8.12 |  | Steps 3, 7 |
|  | Not checked |  | Step 13 |
| slotIndex-r16 | slot index determined based on the formula given in TS 38.331 [22] clause 5.8.12 |  | Steps 3, 7 |
|  | Not checked |  | Step 13 |
| } |  |  |  |

Table 12.1.2.2.3.3-6: +CCUTLE (Step 14, Table 12.1.2.2.3.2-2)

Derivation Path: TS 38.508-1 [4] Table 4.7B-1 with condition Open