3GPP TSG-RAN5 Meeting #103	R5-243329

# Fukuoka, Japan, May 20th - May 24th 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3213 | rev | - | Current version: | 18.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of EN-DC FR1 NR-U Random Access and Handover test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Technologies Int |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NR-U Random Access and EN-DC HO test cases 10.1.1.1.1, 10.1.1.1.2, 10.1.1.1.3, 10.1.1.1.4 and 10.1.2 are missing in the spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added NR-U Random Access and EN-DC HO test cases 10.1.1.1.1, 10.1.1.1.2, 10.1.1.1.3, 10.1.1.1.4 and 10.1.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan would be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.1.1.1(new), 10.1.1.1.2(new), 10.1.1.1.3(new), 10.1.1.1.4(new),10.1.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

## 10.1	RRC_CONNECTED state mobility

### 10.1.1	Random Access

##### 10.1.1.0	Minimum conformance requirements

###### 10.1.1.0.1	Minimum conformance requirements for 4-step RA type

The random access procedure is initiated to establish uplink time synchronization for a UE which either has not acquired or has lost its uplink synchronization, or to convey UE’s request Other SI, or for beam failure recovery. The random access is specified in clause 8 of TS 38.213 [8] and the control of the RACH transmission is specified in clause 5.1 of TS 38.321 [12]. Two types of procedure are defined for the random access, the 4-step RA type, and the 2-step RA type [7]. The decision on which type of procedure to adopt is as described in clause 5.1.1 of TS 38.321 [12]. The requirements for the 4-step RA type procedure are described in clause 6.2.2A.2, whereas the requirements for the 2-step RA type procedure are described in the clause 6.2.2A.3 of this specification.

The UE shall select the type of random access at initiation of the random access procedure based on network configuration, as specified in clause 5.1.1 in TS 38.321 [12].

The UE shall have capability to calculate PRACH transmission power according to the PRACH power formula defined in clause 7.4 of TS 38.213 [8] and apply this power level at the first preamble or additional preambles. The absolute power applied to the first preamble shall have an accuracy as specified in Table 6.3.4.2-1 of TS 38.101-1 [2]. The relative power applied to additional preambles shall have an accuracy as specified in Table 6.3.4.3-1 of TS 38.101-1 [2].

The UE shall indicate a random access problem to upper layers if the maximum number of preamble transmission counter has been reached for the random access procedure on PCell or PSCell as specified in clause 5.1.4 in TS 38.321 [12].

The requirements in this clause apply for UE operating in a carrier frequency with CCA in SA operation mode or any MR-DC operation mode, in a carrier frequency with CCA.

###### 10.1.1.0.1.1	Contention based random access

###### 10.1.1.0.1.1.1	Correct behaviour when transmitting Random Access Preamble

With the UE selected SSB with SS-RSRP above rsrp-ThresholdSSB, UE shall have the capability to select a Random Access Preamble randomly with equal probability from the Random Access Preambles associated with the selected SSB if the association between Random Access Preambles and SSB is configured, as specified in clause 5.1.2 in TS 38.321 [12].

If the UL CCA is successful on the next available PRACH occasion, with the UE selected SSB with SS-RSRP above rsrp-ThresholdSSB, UE shall transmit Random Access Preamble on the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured, if the association between PRACH occasions and SSBs is configured, and PRACH occasion shall be randomly selected with equal probability amongst the selected SSB associated PRACH occasions occurring simultaneously but on different subcarriers, as specified in clause 5.1.2 in TS 38.321 [12].

If UE is configured lbt-FailureRecoveryConfig and is capable of ul-LBT-FailureDetectionRecovery [2] then upon detecting uplink CCA failure during the random access procedure for random access preamble transmission, as outlined in Clause 5.21.2 of TS 38.321 [12], the UE shall perform the Random Access Resource selection procedure again, as specified in clause 5.1.3 in TS 38.321 [12].

If UE is not configured lbt-FailureRecoveryConfig or is not capable of ul-LBT-FailureDetectionRecovery [2] then upon detecting uplink CCA failure during the random access procedure for random access preamble transmission, as outlined in Clause 5.21.2 of TS 38.321 [12], the UE shall increment PREAMBLE_TRANSMISSION_COUNTER by 1. The UE shall again perform the Random Access Resource selection procedure if PREAMBLE_TRANSMISSION_COUNTER < preambleTransMax + 1, as specified in clause 5.1.3 in TS 38.321 [12].

###### 10.1.1.0.1.1.2	Correct behaviour when receiving Random Access Response

The UE may stop monitoring for Random Access Response(s) and shall transmit the msg3 if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble.

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power when the backoff time expires if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

###### 10.1.1.0.1.1.3	Correct behaviour when not receiving Random Access Response

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [12], if the UL CCA is successful, and transmit with the calculated PRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window defined in clause 5.1.4 in TS 38.321 [12].

###### 10.1.1.0.1.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission

The UE shall re-transmit the msg3 upon the reception of an UL grant for msg3 retransmission, if the UL CCA is successful,

10.1.1.0.1.1.5	Correct behaviour when receiving a message over Temporary C-RNTI

If the UL CCA is successful, The UE shall send ACK if the Contention Resolution is successful.

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power when the backoff time expires and the UL CCA is successful, unless the received message includes a UE Contention Resolution Identity MAC control element and the UE Contention Resolution Identity included in the MAC control element matches the CCCH SDU transmitted in the uplink message.

###### 10.1.1.0.1.1.6	Correct behaviour when contention Resolution timer expires

The UE shall re-select a preamble and transmit with the calculated PRACH transmission power when the backoff time expires if the Contention Resolution Timer expires.

The normative reference for this requirement is TS 38.133 [6] clauses 6.2.2A.2.

###### 10.1.1.0.1.2	Non-Contention based random access

###### 10.1.1.0.1.2.1	Correct behaviour when transmitting Random Access Preamble

If the UL CCA is successful on the next available PRACH occasion and if the contention-free Random Access Resources and the contention-free PRACH occasions associated with SSBs is configured, with the UE selected SSB with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs, UE shall have the capability to select the Random Access Preamble corresponding to the selected SSB, and to transmit Random Access Preamble on the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured, and PRACH occasion shall be randomly selected with equal probability amongst the selected SSB associated PRACH occasions occurring simultaneously but on different subcarriers, as specified in clause 5.1.2 in TS 38.321 [12].

If the UL CCA is successful, and if the random access procedure is initialized for beam failure recovery and if the contention-free Random Access Resources and the contention-free PRACH occasions for beam failure recovery request associated with SSBs configured, UE shall have the capability to select the Random Access Preamble corresponding to the selected SSB with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs, and to transmit Random Access Preamble on the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured, and PRACH occasion shall be randomly selected with equal probability amongst the selected SSB associated PRACH occasions occurring simultaneously but on different subcarriers, as specified in clause 5.1.2 in TS 38.321 [12].

If UE is configured lbt-FailureRecoveryConfig and is capable of ul-LBT-FailureDetectionRecovery [2] then upon detecting uplink CCA failure during the random access procedure for random access preamble transmission, as outlined in Clause 5.21.2 of TS 38.321 [12], the UE shall perform the Random Access Resource selection procedure again, as specified in clause 5.1.3 in TS 38.321 [12].

If UE is not configured lbt-FailureRecoveryConfig or is not capable of ul-LBT-FailureDetectionRecovery [2] then upon detecting uplink CCA failure during the random access procedure for random access preamble transmission, as outlined in Clause 5.21.2 of TS 38.321 [12], the UE shall increment PREAMBLE_TRANSMISSION_COUNTER by 1. The UE shall again perform the Random Access Resource selection procedure if PREAMBLE_TRANSMISSION_COUNTER < preambleTransMax + 1, as specified in clause 5.1.3 in TS 38.321 [12].

###### 10.1.1.0.1.2.2	Correct behaviour when receiving Random Access Response

The UE may stop monitoring for Random Access Response(s), if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble, unless the random access procedure is initialized for Other SI request from UE.

The UE may stop monitoring for Random Access Response(s) and shall monitor the Other SI transmission if the Random Access Response only contains a Random Access Preamble identifier which is corresponding to the transmitted Random Access Preamble and the random access procedure is initialized for SI request from UE, as specified in clause 5.1.4 in TS 38.321 [12].

The UE may stop monitoring for Random Access Response(s), if the contention-free Random Access Preamble for beam failure recovery request was transmitted and if the PDCCH addressed to UE’s C-RNTI is received, as specified in clause 5.1.4 in TS 38.321 [12].

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [12] for the next available PRACH occasion if the UL CCA is successful, and transmit the preamble with the calculated PRACH transmission power if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

###### 10.1.1.0.1.2.3	Correct behaviour when not receiving Random Access Response

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [12] for the next available PRACH occasion if the UL CCA is successful, and transmit the preamble with the calculated PRACH transmission power, if no Random Access Response is received within the RA Response window configured in RACH-ConfigCommon or if no PDCCH addressed to UE’s C-RNTI is received within the RA Response window configured in BeamFailureRecoveryConfig, as defined in clause 5.1.4 in TS 38.321 [12].

The normative reference for this requirement is TS 38.133 [6] clauses 6.2.2A.2.

###### 10.1.1.0.2	Minimum conformance requirements for 2-step RA type

The UE shall select the type of random access at initiation of the random access procedure based on network configuration, as specified in clause 5.1.1 in TS 38.321 [12].

The UE shall have capability to calculate MsgA PRACH transmission power according to the PRACH power formula defined in clause 7.4 of TS 38.213 [8] and the MsgA PUSCH power formula of clause 7.1.1 of TS 38.213 [8] and apply this power level at the first MsgA or additional MsgA repetitions. The absolute power applied to the first preamble shall have an accuracy as specified in Table 6.3.4.2-1 of TS 38.101-1 [2]. The relative power applied to additional preambles shall have an accuracy as specified in Table 6.3.4.3-1 of TS 38.101-1 [2].

The UE shall switch to 4-step RA type procedure if the MsgA transmission counter has exceeded msgA-TransMax, if configured, as specified in clause 5.1.4a of TS 38.321 [12]. The UE shall indicate a Random Access problem to upper layers if the maximum number of preamble transmission counter has been reached for the random access procedure on PCell or PSCell as specified in clause 5.1.4a in TS 38.321 [12].

The requirements in this clause apply for UE operating in a carrier frequency with CCA in SA operation mode or any MR-DC operation mode, in a carrier frequency with CCA.

###### 10.1.1.0.2.1	Contention based random access

###### 10.1.1.0.2.1.1	Correct behaviour when transmitting MsgA

With the UE selected SSB with SS-RSRP above msgA-RSRP-ThresholdSSB, the UE shall have the capability to select a Random Access Preamble randomly with equal probability from the Random Access Preambles associated with the selected SSB if the association between Random Access Preambles and SS blocks is configured, as specified in clause 5.1.2a in TS 38.321 [12].

If the UL CCA is successful on the next available PRACH occasion, with the UE selected SSB with SS-RSRP above msgA-RSRP-ThresholdSSB, UE shall have the capability to transmit MsgA PRACH on the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given first by the msgA-SSB-SharedRO-MaskIndex if configured, or next by the ra-ssb-OccasionMaskIndex if configured, if the association between PRACH occasions and SSBs is configured.

The PRACH preamble and PRACH occasion shall be randomly selected with equal probability amongst the selected SSB associated PRACH occasions occurring simultaneously but on different subcarriers, as specified in clause 5.1.2a in TS 38.321 [12].

In association with the MsgA PRACH, if the UL CCA is successful, the UE shall be able to transmit MsgA PUSCH on the corresponding PUSCH occasion associated with a DMRS resource, which is mapped from the MsgA PRACH occasion, and preamble index as defined in clause 8.1A in TS 38.213 [8].

If UE is configured lbt-FailureRecoveryConfig and is capable of ul-LBT-FailureDetectionRecovery [2] then upon detecting uplink CCA failure during the random access procedure for MsgA transmission, as outlined in Clause 5.21.2 of TS 38.321 [12], the UE shall cancel the transmission of the MsgA payload on the associated PUSCH resource and perform the Random Access Resource selection procedure, as specified in clause 5.1.3a in TS 38.321 [12].

If UE is not configured lbt-FailureRecoveryConfig or is not capable of ul-LBT-FailureDetectionRecovery [2] then upon detecting uplink CCA failure during the random access procedure for MsgA transmission, as outlined in Clause 5.21.2 of TS 38.321 [12], the UE shall cancel the transmission of the MsgA payload on the associated PUSCH resource and increment PREAMBLE_TRANSMISSION_COUNTER by 1. The UE shall again perform the Random Access Resource selection procedure if PREAMBLE_TRANSMISSION_COUNTER < preambleTransMax + 1, as specified in clause 5.1.3a in TS 38.321 [12]. If the Random Access Procedure is not complete and the UE is configured with msgA-TransMax then, as specified in clause 5.1.3a in TS 38.321 [12], the UE shall perform the Random Access Resource selection procedure with 4-step RA type provided that PREAMBLE_TRANSMISSION_COUNTER = msgA-TransMax + 1.

###### 10.1.1.0.2.1.2	Correct behaviour when receiving MsgB

The UE shall stop monitoring for MsgB, when the UE has successfully received the PDCCH addressed to UE as specified in clause 8.2A in TS 38.213 [8] containing a successRAR MAC subPDU or a fallbackRAR MAC subPDU as described in clause 5.1.4a in TS 38.321 [12].

If the UL CCA is successful, the UE shall send ACK if Success RAR is received in MsgB and the Contention Resolution is successful, as defined in clause 5.1.4a in TS 38.321 [12].

If MsgB contains a fallbackRAR MAC subPDU the UE shall fallback to the 4-step RA type by transmitting the msg3 containing the payload of MsgA PUSCH if the UL CCA is successful, and monitor contention resolution as described in clause 8.2A in TS 38.213 [8].

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2a in TS 38.321 [12], and transmit with the calculated MsgA PRACH and MsgA PUSCH transmission power, if the UL CCA is successful on the next available PRACH occasion, when the backoff time expires unless the Random Access Response reception is considered as successful, as defined in clause 5.1.4a in TS 38.321 [12].

###### 10.1.1.0.2.1.3	Correct behaviour when not receiving MsgB

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2a in TS 38.321 [12], and if the UL CCA is successful, transmit with the calculated MsgA PRACH and MsgA PUSCH transmission power if the UL CCA is successful on the next available PRACH occasion when the backoff time expires unless the Random Access Response reception is considered as successful, as defined in clause 5.1.4a in TS 38.321 [12].

The normative reference for this requirement is TS 38.133 [6] clauses 6.2.2A.2.

###### 10.1.1.0.2.2	Non-Contention based random access

###### 10.1.1.0.2.2.1	Correct behaviour when transmitting MsgA

If the UL CCA is successful, if the contention-free Random Access Resources and the contention-free PRACH occasions associated with SSBs is configured, with the UE selected SSB with SS-RSRP above msgA-RSRP-ThresholdSSB amongst the associated SSBs, UE shall have the capability to select the Random Access Preamble corresponding to the selected SSB, and to transmit Random Access Preamble on the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given first by the msgA-SSB-SharedRO-MaskIndex if configured, or next by the ra-ssb-OccasionMaskIndex if configured, and PRACH occasion shall be randomly selected with equal probability amongst the selected SSB associated PRACH occasions occurring simultaneously but on different subcarriers, as specified in clause 5.1.2a in TS 38.321 [12].

In association with the MsgA PRACH, the UE shall be able to transmit MsgA PUSCH, if the UL CCA is successful, on the corresponding PUSCH occasion associated with a DMRS resource, which is mapped from the MsgA PRACH occasion, and preamble index as defined in clause 8.1A in TS 38.213 [8].

If UE is configured lbt-FailureRecoveryConfig and is capable of ul-LBT-FailureDetectionRecovery [2] then upon detecting uplink CCA failure during the random access procedure for MsgA transmission, as outlined in Clause 5.21.2 of TS 38.321 [12], the UE shall have cancel the transmission of the MsgA payload on the associated PUSCH resource and perform the Random Access Resource selection procedure, as specified in clause 5.1.3a in TS 38.321 [12].

If UE is not configured lbt-FailureRecoveryConfig or is not capable of ul-LBT-FailureDetectionRecovery [2] then upon detecting uplink CCA failure during the random access procedure for MsgA transmission, as outlined in Clause 5.21.2 of TS 38.321 [12], the UE shall cancel the transmission of the MsgA payload on the associated PUSCH resource and increment PREAMBLE_TRANSMISSION_COUNTER by 1. The UE shall again perform the Random Access Resource selection procedure if PREAMBLE_TRANSMISSION_COUNTER < preambleTransMax + 1, as specified in clause 5.1.3a in TS 38.321 [12]. If the Random Access Procedure is not complete and the UE is configured with msgA-TransMax then, as specified in clause 5.1.3a in TS 38.321 [12], the UE shall perform the Random Access Resource selection procedure with 4-step RA type provided that PREAMBLE_TRANSMISSION_COUNTER = msgA-TransMax + 1.

###### 10.1.1.0.2.2.2	Correct behaviour when receiving MsgB

The UE may stop monitoring for MsgB, when the UE has successfully received the PDCCH addressed to UE as specified in clause 8.2A in TS 38.213 [8] containing a successRAR MAC subPDU or a fallbackRAR MAC subPDU as described in clause 5.1.4a in TS 38.321 [12].

If MsgB contains a fallbackRAR MAC subPDU the UE shall fallback to the 4-step RA type by transmitting the msg3 containing the payload of MsgA PUSCH if the UL CCA is successful, as described in clause 8.2A in TS 38.213 [8].

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2a in TS 38.321 [12] for the next available PRACH occasion, and transmit the preamble with the calculated MsgA PRACH and MsgA PUSCH transmission power if the UL CCA is successful, if all received MsgBs contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

###### 10.1.1.0.2.2.3	Correct behaviour when not receiving MsgB

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2a in TS 38.321 [12] for the next available PRACH occasion, and transmit MsgA with the calculated MsgA PRACH and MsgA PUSCH transmission power if the UL CCA is successful on the next available PRACH occasion, if no MsgB is received within the MsgB Response window configured in RACH-ConfigGenericTwoStepRA and the Random Access Response Reception has not been considered as successful as defined in clause 5.1.4a in TS 38.321 [12].

The normative reference for this requirement is TS 38.133 [6] clauses 6.2.2A.2.

##### 10.1.1.1.1	EN-DC FR1 4-step RA type contention based random access test for NR PSCell with CCA

Editor's Note:

- MU and TT analysis is incomplete

- Statistical analysis to determine test case verdict is FFS

- Message contents is missing

- Test procedure is missing

- Test requirement need to be updated

10.1.1.1.1.1	Test purpose

To verify that the behaviour of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits when subject to CCA. This test will verify the requirements in 38.133 [6] clause 6.2.2A.2 and clause 7.1.2 in an AWGN model.

10.1.1.1.1.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC with a UL shared spectrum NR carrier.

10.1.1.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 10.1.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.10.1.1.1.1.

10.1.1.1.1.4	Test description

10.1.1.1.1.4.1	Initial conditions

This test can be run in one of the configurations defined in Table 10.1.1.1.1.4.1-1.

Table 10.1.1.1.1.4.1-1: Supported test configurations for contention based random access test in FR1 for PSCell with CCA

| Configuration | Description |
| --- | --- |
| 10.1.1.1.1-1 | LTE FDD, With CCA: NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 10.1.1.1.1-2 | LTE TDD, With CCA: NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| NOTE: The UE is only required to be tested in one of the supported test configurations in FR1 depending on UE capability. |  |

Configure the test equipment and the DUT according to the parameters in Table 10.1.1.1.1.4.1-2

Table 10.1.1.1.1.4.1-2: Initial conditions for contention based random access test in FR1 for PSCell with CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.8-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.1.1.1.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 10.1.1.1.1.4.3.

2.	There are one E-UTRAN carrier and one NR carrier cells specified in the test. Cell 1 is the PCell on E-UTRAN carrier, Cell 2 is the PSCell on NR carrier. Cell 2 is subject to CCA. Cell 1 is the cell used for connection setup with the power level set according to Table A.6B.1-1. Cell 2 shall be configured according to clauses C.1.1 and C.1.2.

10.1.1.1.1.4.2	Test procedure

FFS

10.1.1.1.1.4.3	Message contents

FFS

10.1.1.1.1.5	Test requirement

Table 10.1.1.1.1.5-1: General test parameters for contention based random access test in FR1 for PSCell with CCA

| Parameter |  |  |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SSB Configuration | Note 4, 6 |  |  | Config 1,2 |  | SSB.1 CCA | As defined in A.3A.1 |
|  | Note 5, 6 |  |  | Config 1,2 |  | SSB.2 CCA | As defined in A.3A.1 |
| DBT Window Configuration |  |  |  | Config 1,2 |  | DBT.1 | As specified in A.12.1 |
| DL CCA model |  |  |  | Config 1,2 |  | As specified in clause D.7.2.1 |  |
| UL CCA model |  |  |  | Config 1,2 |  | As specified in clause D.7.2.2 |  |
| Duplex Mode for Cell 2 |  |  |  | Config 1,2 |  | TDD |  |
| TDD Configuration |  |  |  | Config 1,2 |  | TDDConf.1.1 CCA |  |
| OCNG Pattern Note 1 |  |  |  |  |  | OCNG pattern 1 | As defined in A.6B.1. |
| PDSCH parameters Note 3 |  |  |  | Config 1,2 |  | SR.1.1 CCA | As defined in A.1A.1.1 |
| NR RF Channel Number |  |  |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  | dB | 0 |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  | dB |  |  |
| SSB with index 0 |  |  |  |  | dB | 3+TT | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  |  |  |  | Config 1,2 | dBm/15kHz | -101+TT |  |
|  |  |  |  |  | dB | 3+TT |  |
|  |  | SS-RSRP |  |  | dBm/ SCS | -95+TT |  |
| SSB with index 1 |  |  |  |  | dB | -17+TT | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  |  |  |  | Config 1,2 | dBm/15kHz | -101+TT |  |
|  |  |  |  |  | dB | -17+TT |  |
|  |  | SS-RSRP |  |  | dBm/ SCS | -115+TT |  |
| Io Note 2 |  |  |  | Config 1,2 | dBm | -62.2/38.16MHz+TT | For symbols without SSB index 1 |
| ss-PBCH-BlockPower |  |  |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [13]. |
| Configured UE transmitted power () |  |  |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1. |
| PRACH Configuration |  |  |  |  |  | FR1 PRACH configuration 1 under CCA | As defined in A.7.1A.1. |
| DL CCA probability |  |  | Note 4, 6 |  |  | 0.9375 |  |
| PCCA_DL |  |  | Note 5, 6 |  |  | 0.75/0.75 |  |
| LCCA_DL Note 7 |  |  |  |  |  | 4 |  |
| WCCA_DL Note 8 |  |  |  |  |  | Inf |  |
| UL CCA probability |  |  | Note 4, 6 |  |  | 0.87 |  |
| PCCA_UL |  |  | Note 5, 6 |  |  | 0.75 |  |
| LCCA_UL Note 7 |  |  |  |  |  | 5 |  |
| WCCA_UL Note 8 |  |  |  |  |  | Inf |  |
| Semi-static channel access config period Note 4, 6 |  |  |  |  | ms | 2 |  |
| Propagation Condition |  |  |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. Note 4: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 5: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic channel occupancy and semi-static channel occupancy configuration. Note 7: LCCA_DL and LCCA_UL are chosen such that preambleTransMax > 5 + LCCA_DL + LCCA_UL. Note 8: A window WCCA_DL=WCCA_UL=Inf is used to indicate that LCCA_DL and LCCA_UL are considered during the entire duration of a test run. |  |  |  |  |  |  |  |

##### 10.1.1.1.2	EN-DC FR1 4-step RA type non-contention based random access for NR PSCell with CCA

Editor's Note:

- MU and TT analysis is incomplete

- Statistical analysis to determine test case verdict is FFS

- Message contents is missing

- Test procedure is missing

- Test requirement need to be updated

10.1.1.1.2.1	Test purpose

To verify that the behaviour of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits when subject to CCA. This test will verify the requirements in 38.133 [6] clause 6.2.2A.2 and clause 7.1.2 in an AWGN model.

10.1.1.1.2.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC with a UL shared spectrum NR carrier.

10.1.1.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 10.1.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.10.1.1.1.2.

10.1.1.1.2.4	Test description

10.1.1.1.2.4.1	Initial conditions

This test can be run in one of the configurations defined in Table 10.1.1.1.2.4.1-1.

Table 10.1.1.1.2.4.1-1: Supported test configurations for non-contention based random access test in FR1 for PSCell with CCA

| Configuration | Description |
| --- | --- |
| 10.1.1.1.2-1 | LTE FDD, With CCA: NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 10.1.1.1.2-2 | LTE TDD, With CCA: NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| NOTE: The UE is only required to be tested in one of the supported test configurations in FR1 depending on UE capability. |  |

Configure the test equipment and the DUT according to the parameters in Table 10.1.1.1.2.4.1-2

Table 10.1.1.1.2.4.1-2: Initial conditions for non-contention based random access test in FR1 for PSCell with CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.8-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.1.1.1.2.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 10.1.1.1.2.4.3.

2.	There are one E-UTRAN carrier and one NR carrier cells specified in the test. Cell 1 is the PCell on E-UTRAN carrier, Cell 2 is the PSCell on NR carrier. Cell 2 is subject to CCA. Cell 1 is the cell used for connection setup with the power level set according to Table A.6B.1-1. Cell 2 shall be configured according to clauses C.1.1 and C.1.2.

10.1.1.1.2.4.2	Test procedure

FFS

10.1.1.1.2.4.3	Message contents

FFS

10.1.1.1.2.5	Test requirement

Table 10.1.1.1.2.5-1: General test parameters for non-contention based random access test in FR1 for PSCell with CCA

| Parameter |  |  |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SSB Configuration | Note 4, 6 |  |  | Config 1,2 |  | SSB.1 CCA | As defined in A.3A.1 |
|  | Note 5, 6 |  |  | Config 1,2 |  | SSB.2 CCA | As defined in A.3A.1 |
| DBT Window Configuration |  |  |  | Config 1,2 |  | DBT.1 | As specified in A.12.1 |
| DL CCA model |  |  |  | Config 1,2 |  | As specified in clause D.7.2.1 |  |
| UL CCA model |  |  |  | Config 1,2 |  | As specified in clause D.7.2.2 |  |
| Duplex Mode for Cell 2 |  |  |  | Config 1,2 |  | TDD |  |
| TDD Configuration |  |  |  | Config 1,2 |  | TDDConf.1.1 CCA |  |
| OCNG Pattern Note 1 |  |  |  |  |  | OCNG pattern 1 | As defined in A.6B.1. |
| PDSCH parameters Note 3 |  |  |  | Config 1,2 |  | SR.1.1 CCA | As defined in A.1A.1.1 |
| NR RF Channel Number |  |  |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  | dB | 0 |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  | dB |  |  |
| SSB with index 0 |  |  |  |  | dB | 3+TT | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  |  |  |  | Config 1,2 | dBm/15kHz | -101+TT |  |
|  |  |  |  |  | dB | 3+TT |  |
|  |  | SS-RSRP |  |  | dBm/ SCS | -95+TT |  |
| SSB with index 1 |  |  |  |  | dB | -17+TT | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  |  |  |  | Config 1,2 | dBm/15kHz | -101+TT |  |
|  |  |  |  |  | dB | -17+TT |  |
|  |  | SS-RSRP |  |  | dBm/ SCS | -115+TT |  |
| Io Note 2 |  |  |  | Config 1,2 | dBm | -62.2/38.16MHz+TT | For symbols without SSB index 1 |
| ss-PBCH-BlockPower |  |  |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [13]. |
| Configured UE transmitted power () |  |  |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1. |
| PRACH Configuration |  |  |  |  |  | FR1 PRACH configuration 2 under CCA | As defined in A.7.1A.2. |
| DL CCA probability |  |  | Note 4, 6 |  |  | 0.9375 |  |
| PCCA_DL |  |  | Note 5, 6 |  |  | 0.75/0.75 |  |
| LCCA_DL Note 7 |  |  |  |  |  | 4 |  |
| WCCA_DL Note 8 |  |  |  |  |  | Inf |  |
| UL CCA probability |  |  | Note 4, 6 |  |  | 0.87 |  |
| PCCA_UL |  |  | Note 5, 6 |  |  | 0.75 |  |
| LCCA_UL Note 7 |  |  |  |  |  | 5 |  |
| WCCA_UL Note 8 |  |  |  |  |  | Inf |  |
| Semi-static channel access config period Note 4, 6 |  |  |  |  | ms | 2 |  |
| Propagation Condition |  |  |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. Note 4: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 5: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic channel occupancy and semi-static channel occupancy configuration. Note 7: LCCA_DL and LCCA_UL are chosen such that preambleTransMax > 5 + LCCA_DL + LCCA_UL. Note 8: A window WCCA_DL=WCCA_UL=Inf is used to indicate that LCCA_DL and LCCA_UL are considered during the entire duration of a test run. |  |  |  |  |  |  |  |

##### 10.1.1.1.3	EN-DC FR1 2-step RA type contention-based random access for NR PSCell with CCA

Editor's Note:

- MU and TT analysis is incomplete

- Statistical analysis to determine test case verdict is FFS

- Message contents is missing

- Test procedure is missing

- Test requirement need to be updated

10.1.1.1.3.1	Test purpose

To verify that the behaviour of the random access procedure is according to the requirements and that the MsgA PRACH, MsgA PUSCH power settings and timing are within specified limits when subject to CCA. This test will verify the requirements in 38.133 [6] clause 6.2.2A.3 and clause 7.1.2 in an AWGN model.

10.1.1.1.3.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC with a UL shared spectrum NR carrier.

10.1.1.1.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 10.1.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.10.1.1.1.3.

10.1.1.1.3.4	Test description

10.1.1.1.3.4.1	Initial conditions

This test can be run in one of the configurations defined in Table 10.1.1.1.3.4.1-1.

Table 10.1.1.1.3.4.1-1: Supported test configurations for 2-step RA type contention based random access test in FR1 for PSCell with CCA

| Configuration | Description |
| --- | --- |
| 10.1.1.1.3-1 | LTE FDD, With CCA: NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 10.1.1.1.3-2 | LTE TDD, With CCA: NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| NOTE: The UE is only required to be tested in one of the supported test configurations in FR1 depending on UE capability. |  |

Configure the test equipment and the DUT according to the parameters in Table 10.1.1.1.3.4.1-2

Table 10.1.1.1.3.4.1-2: Initial conditions for 2-step RA type contention based random access test in FR1 for PSCell with CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.8-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.1.1.1.3.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 10.1.1.1.3.4.3.

2.	There are one E-UTRAN carrier and one NR carrier cells specified in the test. Cell 1 is the PCell on E-UTRAN carrier, Cell 2 is the PSCell on NR carrier. Cell 2 is subject to CCA. Cell 1 is the cell used for connection setup with the power level set according to Table A.6B.1-1. Cell 2 shall be configured according to clauses C.1.1 and C.1.2.

10.1.1.1.3.4.2	Test procedure

FFS

10.1.1.1.3.4.3	Message contents

FFS

10.1.1.1.3.5	Test requirement

Table 10.1.1.1.3.5-1: General test parameters for 2-step RA type contention based random access test in FR1 for PSCell with CCA

| Parameter |  |  |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SSB Configuration | Note 4, 6 |  |  | Config 1,2 |  | SSB.1 CCA | As defined in A.3A.1 |
|  | Note 5, 6 |  |  | Config 1,2 |  | SSB.2 CCA | As defined in A.3A.1 |
| DBT Window Configuration |  |  |  | Config 1,2 |  | DBT.1 | As specified in A.12.1 |
| DL CCA model |  |  |  | Config 1,2 |  | As specified in clause D.7.2.1 |  |
| UL CCA model |  |  |  | Config 1,2 |  | As specified in clause D.7.2.2 |  |
| Duplex Mode for Cell 2 |  |  |  | Config 1,2 |  | TDD |  |
| TDD Configuration |  |  |  | Config 3,4 |  | TDDConf.1.1 CCA |  |
| OCNG Pattern Note 1 |  |  |  |  |  | OCNG pattern 1 | As defined in A.6B.1. |
| PDSCH parameters Note 3 |  |  |  | Config 1,2 |  | SR.1.1 CCA | As defined in A.1A.1.1. |
| NR RF Channel Number |  |  |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  | dB | 0 |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  | dB |  |  |
| SSB with index 0 |  |  |  |  | dB | 3+TT | Power of SSB with index 0 is set to be above configured msgA-RSRP-ThresholdSSB |
|  |  |  |  | Config 1,2 | dBm/15kHz | -101+TT |  |
|  |  |  |  |  | dB | 3+TT |  |
|  |  | SS-RSRP Note 2 |  |  | dBm/ SCS | -95+TT |  |
| SSB with index 1 |  |  |  |  | dB | -17+TT | Power of SSB with index 1 is set to be below configured msgA-RSRP-ThresholdSSB |
|  |  |  |  | Config 1,2 | dBm/15kHz | -101+TT |  |
|  |  |  |  |  | dB | -17+TT |  |
|  |  | SS-RSRP Note 2 |  |  | dBm/ SCS | -115+TT |  |
| Io |  |  |  | Config 1,2 | dBm | -62.2/38.16MHz+TT | For symbols without SSB index 1 |
| ss-PBCH-BlockPower |  |  |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [13]. |
| Configured UE transmitted power () |  |  |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1. |
| MsgA Configuration |  |  |  |  |  | FR1 MsgA configuration 1 under CCA | As defined in A.7.1A.1. |
| msgA-RSRP-ThresholdSSB |  |  |  |  | dBm | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [13]. |
| DL CCA probability |  |  | Note 4, 6 |  |  | 0.9375 |  |
| PCCA_DL |  |  | Note 5, 6 |  |  | 0.75/0.75 |  |
| LCCA_DL Note 7 |  |  |  |  |  | 4 |  |
| WCCA_DL Note 8 |  |  |  |  |  | Inf |  |
| UL CCA probability |  |  | Note 4, 6 |  |  | 0.87 |  |
| PCCA_UL |  |  | Note 5, 6 |  |  | 0.75 |  |
| LCCA_UL Note 7 |  |  |  |  |  | 5 |  |
| WCCA_UL Note 8 |  |  |  |  |  | Inf |  |
| Semi-static channel access config period Note 4, 6 |  |  |  |  | ms | 2 |  |
| Propagation Condition |  |  |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. Note 4: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 5: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic channel occupancy and semi-static channel occupancy configuration. Note 7: LCCA_DL and LCCA_UL are chosen such that preambleTransMax > 5 + LCCA_DL + LCCA_UL. Note 8: A window WCCA_DL=WCCA_UL=Inf is used to indicate that LCCA_DL and LCCA_UL are considered during the entire duration of a test run. |  |  |  |  |  |  |  |

##### 10.1.1.1.4	EN-DC FR1 2-step RA type non-contention based random access for NR PSCell with CCA

Editor's Note:

- MU and TT analysis is incomplete

- Statistical analysis to determine test case verdict is FFS

- Message contents is missing

- Test procedure is missing

- Test requirement need to be updated

10.1.1.1.4.1	Test purpose

To verify that the behaviour of the random access procedure is according to the requirements and that the MsgA PRACH, MsgA PUSCH power settings and timing are within specified limits when subject to CCA. This test will verify the requirements in 38.133 [6] clause 6.2.2A.3 and clause 7.1.2 in an AWGN model.

10.1.1.1.4.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC with a UL shared spectrum NR carrier.

10.1.1.1.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 10.1.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.10.1.1.1.4.

10.1.1.1.4.4	Test description

10.1.1.1.4.4.1	Initial conditions

This test can be run in one of the configurations defined in Table 10.1.1.1.4.4.1-1.

Table 10.1.1.1.4.4.1-1: Supported test configurations for non-contention based random access test for 2-step RA type in FR1 for PSCell with CCA

| Configuration | Description |
| --- | --- |
| 10.1.1.1.4-1 | LTE FDD, With CCA: NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 10.1.1.1.4-2 | LTE TDD, With CCA: NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| NOTE: The UE is only required to be tested in one of the supported test configurations in FR1 depending on UE capability. |  |

Configure the test equipment and the DUT according to the parameters in Table 10.1.1.1.4.4.1-2

Table 10.1.1.1.4.4.1-2: Initial conditions for non-contention based random access test in FR1 for PSCell with CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.8-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.1.1.1.4.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 10.1.1.1.4.4.3.

2.	There are one E-UTRAN carrier and one NR carrier cells specified in the test. Cell 1 is the PCell on E-UTRAN carrier, Cell 2 is the PSCell on NR carrier. Cell 2 is subject to CCA. Cell 1 is the cell used for connection setup with the power level set according to Table A.6B.1-1. Cell 2 shall be configured according to clauses C.1.1 and C.1.2.

10.1.1.1.4.4.2	Test procedure

FFS

10.1.1.1.4.4.3	Message contents

FFS

10.1.1.1.4.5	Test requirement

Table 10.1.1.1.4.5-1: General test parameters for non-contention based random access test for 2-step RA type in FR1 for PSCell with CCA

| Parameter |  |  |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SSB Configuration | Note 4, 6 |  |  | Config 1,2 |  | SSB.1 CCA | As defined in A.3A.1 |
|  | Note 5, 6 |  |  | Config 1,2 |  | SSB.2 CCA | As defined in A.3A.1 |
| DBT Window Configuration |  |  |  | Config 1,2 |  | DBT.1 | As specified in A.12.1 |
| DL CCA model |  |  |  | Config 1,2 |  | As specified in clause D.7.2.1 |  |
| UL CCA model |  |  |  | Config 1,2 |  | As specified in clause D.7.2.2 |  |
| Duplex Mode for Cell 2 |  |  |  | Config 1,2 |  | TDD |  |
| TDD Configuration |  |  |  | Config 1,2 |  | TDDConf.1.1 CCA |  |
| OCNG Pattern Note 1 |  |  |  |  |  | OCNG pattern 1 | As defined in A.6B.1. |
| PDSCH parameters Note 3 |  |  |  | Config 1,2 |  | SR.1.1 CCA | As defined in A.1A.1.1 |
| NR RF Channel Number |  |  |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  | dB | 0 |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  | dB |  |  |
| SSB with index 0 |  |  |  |  | dB | 3+TT | Power of SSB with index 0 is set to be above configured msgA-RSRP-ThresholdSSB |
|  |  |  |  | Config 1,2 | dBm/15kHz | -101+TT |  |
|  |  |  |  |  | dB | 3+TT |  |
|  |  | SS-RSRP |  |  | dBm/ SCS | -95+TT |  |
| SSB with index 1 |  |  |  |  | dB | -17+TT | Power of SSB with index 1 is set to be below configured msgA-RSRP-ThresholdSSB |
|  |  |  |  | Config 1,2 | dBm/15kHz | -101+TT |  |
|  |  |  |  |  | dB | -17+TT |  |
|  |  | SS-RSRP |  |  | dBm/ SCS | -115+TT |  |
| Io Note 2 |  |  |  | Config 1,2 | dBm | -62.2/38.16MHz+TT | For symbols without SSB index 1 |
| ss-PBCH-BlockPower |  |  |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [13]. |
| Configured UE transmitted power (PCMAX,f,c) |  |  |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1. |
| MsgA Configuration |  |  |  |  |  | FR1 MsgA configuration 2 under CCA | As defined in A.7.1A.2. |
| msgA-RSRP-ThresholdSSB |  |  |  |  | dBm | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [13]. |
| DL CCA probability |  |  | Note 4, 6 |  |  | 0.9375 |  |
| PCCA_DL |  |  | Note 5, 6 |  |  | 0.75/0.75 |  |
| LCCA_DL Note 7 |  |  |  |  |  | 4 |  |
| WCCA_DL Note 8 |  |  |  |  |  | Inf |  |
| UL CCA probability |  |  | Note 4, 6 |  |  | 0.87 |  |
| PCCA_UL |  |  | Note 5, 6 |  |  | 0.75 |  |
| LCCA_UL Note 7 |  |  |  |  |  | 5 |  |
| WCCA_UL Note 8 |  |  |  |  |  | Inf |  |
| Semi-static channel access config period Note 4, 6 |  |  |  |  | ms | 2 |  |
| Propagation Condition |  |  |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. Note 4: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 5: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic channel occupancy and semi-static channel occupancy configuration. Note 7: LCCA_DL and LCCA_UL are chosen such that preambleTransMax > 5 + LCCA_DL + LCCA_UL. Note 8: A window WCCA_DL=WCCA_UL=Inf is used to indicate that LCCA_DL and LCCA_UL are considered during the entire duration of a test run. |  |  |  |  |  |  |  |

### 10.1.2	NR FR1 Handover with PSCell from EN-DC to EN-DC with known target PSCell using CCA

Editor's Note:

- MU and TT analysis is incomplete

- Applicability needs to be updated

- Statistical analysis to determine test case verdict is FFS

- Message contents is missing

- Test procedure is missing

- Test requirement need to be updated

- Some parameters are still in brackets

10.1.2.1	Test purpose

To verify that the requirement for E-UTRA handover with NR PSCell change, where NR PSCell is on carrier with CCA. This test will verify the requirements for EN-DC HO with PSCell change on CCA specified in clause 5.9 in 36.133 [23] for the case when the target PSCell is on carrier with CCA.

10.1.2.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC with a UL shared spectrum NR carrier.

10.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 10.1.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.10.1.2.

10.1.2.4	Test description

10.1.2.4.1	Initial conditions

This test can be run in one of the configurations defined in Table 10.1.2.4.1-1.

Table 10.1.2.4.1-1: Supported test configurations for Handover with PSCell from EN-DC to EN-DC with CCA on NR Cell

| Configuration | Description |
| --- | --- |
| 10.1.2-1 | LTE FDD, With CCA: NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 10.1.2-2 | LTE TDD, With CCA: NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| NOTE: The UE is only required to be tested in one of the supported test configurations in FR1 depending on UE capability. |  |

Configure the test equipment and the DUT according to the parameters in Table 10.1.2.4.1-2

Table 10.1.2.4.1-2: Initial conditions for Handover with PSCell from EN-DC to EN-DC with CCA on NR Cell

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.8-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.1.2.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 10.1.2.4.3.

2.	There are four cells in the test. Cell1 and Cell2 are PCell and target PCell on E-UTRA carrier, Cell3 and Cell4 are PSCell and target PSCell on NR CCA carrier.  Cell 1 is the cell used for connection setup with the power level set according to Table A.6B.1-1. Cell 2 shall be configured according to clauses C.1.1 and C.1.2.

10.1.2.4.2	Test procedure

FFS

10.1.2.4.3	Message contents

FFS

10.1.2.5	Test requirement

Table 10.1.2.5-1: General test parameters for Handover from E-UTRA to E-UTRA cell in EN-DC to EN-DC

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1, 2 | One is E-UTRA RF channel and one is NR RF channel |
| Initial conditions | Active PCell |  | Cell1 | On E-UTRA RF channel number 1. |
|  | E-UTRA Neighbouring cell |  | Cell2 | On E-UTRA RF channel number 1. |
| Final conditions | Active PCell |  | Cell2 |  |
| CP length |  |  | Normal | Applicable to Cell1, Cell2, Cell3 and Cell4. |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between same RAT cells |  | µs | 3 | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 | Tinterrupt is defined in clause 6.1B.1.2 |

Table 10.1.2.5-2: E-UTRAN cell specific test parameters for Handover with PSCell from EN-DC to EN-DC

| Parameter | Unit | Cell1 |  |  | Cell2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Duplex mode |  | FDD or TDD |  |  | FDD or TDD |  |  |
| TDD special subframe configurationNote1 |  | 6 |  |  | 6 |  |  |
| TDD uplink-downlink configurationNote1 |  | 1 |  |  | 1 |  |  |
| BWchannel |  | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |  | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |  | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |  |
| OCNG Patterns defined in A.3.2.1 (FDD) and in A.3.2.2(TDD) Note2 |  | 5 MHz: OP.20 FDD 10MHz: OP.1 FDD 20 MHz: OP.17 FDD 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  | OP.18 FDD OP.2 FDD OP.14 FDD OP.10 TDD OP.2 TDD OP.8 TDD | 5MHz: OP.18 FDD 10MHz: OP.2 FDD 20MHz: OP.14 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  | OP.20 FDD OP.1 FDD OP.17 FDD OP.9 TDD OP.1 TDD OP.7 TDD |
| PRACH configuration |  | - |  |  | 4, As specified in table 5.7.1-2 in TS 36.211[24] |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote3 | dB |  |  |  |  |  |  |
| OCNG_RBNote3 | dB |  |  |  |  |  |  |
| NocNote4 | dBm/15 kHz | -98+TT |  |  |  |  |  |
| Ês/Noc | dB | 8+TT | 8+TT | 8+TT | -infinite | 11+TT | 11+TT |
| Ês/Iot | dB | 8+TT | -3.3+TT | -3.3+TT | -infinite | 2.36+TT | 2.36+TT |
| RSRP Note5 | dBm/15 kHz | -90+TT | -90+TT | -90+TT | -infinite | -87+TT | -87+TT |
| SCH_RP Note5 | dBm/15 kHz | -90+TT | -90+TT | -90+TT | -infinite | -87+TT | -87+TT |
| Io Note5 | dBm/Ch BW | -61.58+TT | -57.23+10log(NRB,c /50)+TT |  | N/A | -57.23+10log(NRB,c /50)+TT |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

Table 10.1.2.5-3: General test parameters for PSCell change from FR1 carrier under CCA to FR1 carrier under CCA

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 3 | On the carrier under CCA |
|  | Neighbouring cell |  | Cell 4 | On the carrier under CCA |
| Final condition | Active cell |  | Cell 4 | On the carrier under CCA |
| DL CCA model | Dynamic channel accessNote 1, 3 |  | As specified in clause A.3.20.2.1 |  |
|  | Semi-static channel access Note 2, 3 |  |  |  |
| UL CCA model | Dynamic channel access Note 1, 3 |  | As specified in clause A.3.20.2.2 |  |
|  | Semi-static channel access Note 2,3 |  |  |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T304 |  | ms | 500 |  |
| LCCA_DL |  |  | 5 |  |
| WCCA_DL |  | ms | T304 |  |
| LCCA_UL |  |  | 5 |  |
| WCCA_UL |  | ms | T304 |  |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | ≥ Tinterrupt | Tinterrupt is defined in clause 6.1B.1.2 |
| NOTE 1: For a UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 2: For a UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |

Table 10.1.2.5-4: Cell specific test parameters for PSCell change from FR1 carrier under CCA to FR1 carrier under CCA

| Parameter |  |  | Unit | Cell 3 |  |  | Cell 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  |  |  | 1 |  |  | 1 |  |  |
| PCCA_DL for dynamic channel access Note 4,6 |  |  | - | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |
| PCCA_DL for semi-static channel access Note 5,6 |  |  | - | PCCA_DL=0.9375 |  |  | PCCA_DL=0.9375 |  |  |
| PCCA_UL for dynamic channel access Note 4,6 |  |  | - | 0.75 |  |  | 0.75 |  |  |
| PCCA_UL for semi-static channel access Note 5,6 |  |  | - | 0.87 |  |  | 0.87 |  |  |
| TDD configuration |  | Config 1, 2 |  | TDDConf.1.1 CCA |  |  |  |  |  |
| BWchannel |  | Config 1, 2 |  | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | Config 1, 2 |  | 40: NRB,c = 106 |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference |  | Config 1, 2 |  | SR.1.1 CCA |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1, 2 |  | CR.1.1 CCA |  |  |  |  |  |
| Dedicated CORESET RMC configuration |  | Config 1, 2 |  | CCR.1.1 CCA |  |  |  |  |  |
| TRS configuration |  | Config 1, 2 |  | TRS.1.1 TDD |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |  |
| DBT window configuration |  | Config 1, 2 |  | DBT.1 |  |  |  |  |  |
| SSB configuration for semi-static channel accessNote 4, 6 |  | Config 1, 2 |  | SSB.1 CCA |  |  |  |  |  |
| SSB configuration for dynamic channel accessNote 5, 6 |  | Config 1, 2 |  | SSB.2 CCA |  |  |  |  |  |
| ssb-PositionQCL |  | Config 1, 2 |  | [1] |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2 | kHz | 30 kHz |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1, 2 | kHz | 30 kHz |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 under CCA |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98+TT |  |  |  |  |  |
| Note2 | Config 1, 2 |  | dBm/SCS | -95+TT |  |  |  |  |  |
|  |  |  | dB | 8+TT | -3.3+TT | -3.3+TT | -Infinity | 2.36+TT | 2.36+TT |
|  |  |  | dB | 8+TT | 8+TT | 8+TT | -Infinity | 11+TT | 11+TT |
| SSB_RP | Config 1, 2 |  | dBm/SCS | -87+TT | -87+TT | -87+TT | -Infinity | -84+TT | -84+TT |
| IoNote3 | Config 1, 2 |  | dBm/ 38.16MHz | -55.31+TT | -50.96+TT | -50.96+TT | -55.31+TT | -50.96+TT | -50.96+TT |
| Propagation condition |  |  | - | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 5: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 6: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |  |  |  |  |

The UE shall start to transmit the PRACH to Cell 2 less than 60 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE: The handover requirements for handover with PSCell for EN-DC is defined in clause 5.9 in 36.133 [23] as:

DHOwithPSCel_PCell = TRRC_delay + Tsearch + TIU + Tprocessing

Where:

TRRC_delay = 20 ms for ‘RRC connection reconfiguration (NR SCG establishment/ /modification/release)’.

Tsearch = 0 ms for known cell.

TIU = 15 ms in the test configuration.

Tprocessing = 25ms for source Cell and target Cell are in the same FR.

This gives a total of 60ms for handover delay.

The UE shall transmit the PRACH preamble to Cell 4 less than DHOwithPSCell_PSCell from the beginning of time period T3.

NOTE: The PSCell change delay for handover with PSCell for EN-DC is defined in clause 5.8 in 36.133 [23] as:

DHOwithPSCell_PSCell = TRRC_delay + Tprocessing + Tsearch + T∆ + TIU_PSCell + 2 ms

Where:

TRRC_delay = 20 ms for ‘RRC connection reconfiguration (NR SCG establishment/ /modification/release)’.

Tprocessing = 25ms for source Cell and target Cell are in the same FR.

Tsearch = 0 ms for known cell.

T∆ = (1+ L2) *20 ms.

TIU = (1+ L3) *10 + 10 ms

L2 is the number of SMTC occasions not available at the UE during the time tracking period where L2  LCCA_DL, and L3 is the number of consecutive SSB to PRACH occasion association periods during which no PRACH occasion is available for PRACH transmission due to UL CCA failure, where L3  LCCA_UL. L3 = 0 for Type 2C UL channel access procedure as defined in TS 37.213 [37]. The interruption time considering the potential extensions caused by L1, L2, L3 and by the UL CCA failure detection/recovery mechanism is limited by the T304 timer. The UE behaviour at the T304 timer expiry is detailed in TS 38.331 [13]. Test equipment should make sure that LCCA_DL and LCCA_UL are not exceeded during a test by monitoring the number of CCA failures and preventing additional CCA failures from happening after LCCA_DL or LCCA_UL is reached.

The rate of correct PSCell addition observed during repeated tests shall be at least 90%.

<< End of Changes >>