3GPP TSG RAN WG5#103	R5-243647

# Fukuoka, Japan, May 20 – May 24 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2777 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Table 6.F-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Correction to Table 6.F-1, table numbering is not correct |  |  |  |  |  |  |  |  |
| Summary of change: |  | Table numbering corrected TB corrected to N/A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Table numbering would remain incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1F |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### << Start of changes >>

## 6.1F	General

For wideband operations, the minimum requirements for the transmitter characteristics are specified for transmissions on one scheduled RB set or ≥ 1 scheduled contiguous RB set(s) within the UE channel. The requirements apply with configured UL intra-cell guard bands of non-zero size according to Table 5.3.3-2, with the union of the scheduled RB sets and the intra-cell guard bands.

Unless stated otherwise, when a clause is not present for shared spectrum channel access, the general requirements and the additional clause requirements (suffices A, B, D) in clause 6 apply.

Table 6.1F-1: Common uplink configuration for shared spectrum access

| Channel Bandwidth | SCS (kHz) | OFDM | RB allocation |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Full | [Partial_Left]2 | [Partial_Right]3 |
| 20MHz | 15 | DFT-s | 100@0 | N/A1 | N/A1 |
|  |  | CP | 106@0 | 11 | 11 |
|  | 30 | DFT-s | 50@0 | N/A1 | N/A1 |
|  |  | CP | 51@0 | 11 | 11 |
| 40MHz | 15 | DFT-s | 216@0 | N/A1 | N/A1 |
|  |  | CP | 216@0 | 22 | 22 |
|  | 30 | DFT-s | 100@0 | N/A1 | N/A1 |
|  |  | CP | 106@0 | 22 | 22 |
| 60MHz | 15 | DFT-s | N/A | N/A1 | N/A1 |
|  |  | CP | N/A | N/A | N/A |
|  | 30 | DFT-s | 162@0 | N/A1 | N/A1 |
|  |  | CP | 162@0 | 33 | 33 |
| 80MHz | 15 | DFT-s | N/A | N/A1 | N/A1 |
|  |  | CP | N/A | N/A | N/A |
|  | 30 | DFT-s | 216@0 | N/A1 | N/A1 |
|  |  | CP | 217@0 | 44 | 44 |
| Note 1: Partial RB allocation consists of non-contiguous RB allocation in one or several interlaces and therefore DFT-s-OFDM is not applicable. Note 2: A set of distributed RBs over the channel bandwidth for one Interlace only. The lowest RB index in the channel bandwidth is part of the distributed RB set. Note 3: A set of distributed RBs over the channel bandwidth for one Interlace only. The highest RB index in the channel bandwidth is part of the distributed RB set. |  |  |  |  |  |

#### << End of changes >>