3GPP TSG-RAN5 Meeting #103	R5-243572

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-7 | CR | 0061 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new TC 5.8 Rules based affiliation status change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2024-05-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There was no existing test case for MCData Rules based affiliation status change |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 5.8 Configuration / Rules based affiliation status change |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Devices could fail in the commercial field without testing the capability |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Final version of R5-243269 |  |  |  |  |  |  |  |  |

<START OF CHANGES>

## 5.8	Configuration / Rules based affiliation status change

5.8.1	Test Purpose (TP)

(1)

with { UE (MCData Client) registered and authorised for MCData Service }

ensure that {

when { UE enters into or exits from a location area that triggers a group de-affiliation }

then { UE (MCData Client) sends a SIP PUBLISH message to de-affiliate from a group }

}

(2)

with { UE (MCData Client) registered and authorised for MCData Service }

ensure that {

when { UE enters into or exits from a location area that triggers a group affiliation }

then { UE (MCData Client) sends a SIP PUBLISH message to affiliate to a group }

}

5.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.282 clause 8.2.6, 8.2.2. Unless otherwise stated these are Rel-16 requirements.

[TS 24.282, clause 8.2.6]

Rules based affiliation is controlled by the elements <RulesForAffiliation> or <RulesForDeaffiliation> of the MCData user profile document identified by the MCData ID of the MCData user (see the MCData user profile document specified in 3GPP TS 24.484 [50]). The rules can be composed of location criteria (including heading and speed) or functional alias based criteria. A rule is fulfilled if  any of the location criteria and any of the functional alias based criteria are met. These rules are evaluated whenever a change of location occurs and whenever a functional alias is activated or deactivated. If any defined rule is fulfilled, the MCData client shall initiate the affiliation status change procedure as specified in subclause 8.2.2.

NOTE:	Hysteresis can be applied to location changes to avoid too frequent affiliation changes. In addition, the definition of area entry and exit criteria can be specified to provide a buffer space to minimize ping-ponging into and out of an area.

[TS 24.282, clause 8.2.2]

In order:

-	to indicate that an MCData user is interested in one or more MCData group(s) at an MCData client;

-	to indicate that the MCData user is no longer interested in one or more MCData group(s) at the MCData client;

-	to refresh indication of an MCData user interest in one or more MCData group(s) at an MCData client due to near expiration of the expiration time of an MCData group with the affiliation status set to the "affiliated" state received in a SIP NOTIFY request in subclause 8.2.3;

-	to send an affiliation status change request in mandatory mode to another MCData user;

-	to indicate that an MCData user is interested in one or more MCData group(s) at an MCData client triggered by a location or functional alias activation criteria;

-	to indicate that the MCData user is no longer interested in one or more MCData group(s) at the MCData client client triggered by location or functional alias deactivation criteria; or

-	any combination of the above;

the MCData client shall generate a SIP PUBLISH request according to 3GPP TS 24.229 [5], IETF RFC 3903 [34], and IETF RFC 3856 [39].

When the MCData user indicates that he is no longer interested in one or more MCData group(s) at the MCData client, the MCData client shall first check value of the <manual-deaffiliation-not-allowed-if-affiliation-rules-are-met> element if present within the MCData user profile document (see the MCData user profile document specified in 3GPP TS 24.484 [50]). If the affiliation to the group has been activated due to a rule being fulfilled and the <manual-deaffiliation-not-allowed-if-affiliation-rules-are-met> element is present and is set to a value of "true", the MCData client shall suppress the MCData user’s request.

NOTE 0:	If the request is suppressed, a notification message can be displayed to the user.

In the SIP PUBLISH request, the MCData client:

1)	shall set the Request-URI to the public service identity identifying the originating participating MCData function serving the MCData user;

2)	shall include an application/vnd.3gpp.mcdata-info+xml MIME body. In the application/vnd.3gpp.mcdata-info+xml MIME body, the MCData client shall include the <mcdata-request-uri> element set to the MCData ID of the MCData user;

3)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata" (coded as specified in 3GPP TS 24.229 [5]), in a P-Preferred-Service header field according to IETF RFC 6050 [7];

4)	if the targeted MCData user is interested in at least one MCData group at the targeted MCData client, shall set the Expires header field according to IETF RFC 3903 [34], to 4294967295;

NOTE 1:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [4].

5)	if the targeted MCData user is no longer interested in any MCData group at the targeted MCData client, shall set the Expires header field according to IETF RFC 3903 [34], to zero; and

6)	shall include an application/pidf+xml MIME body indicating per-user affiliation information according to subclause 8.4.1. In the MIME body, the MCData client:

a)	shall include all MCData groups where the targeted MCData user indicates its interest at the targeted MCData client;

b)	shall include the MCData client ID of the targeted MCData client;

c)	shall not include the "status" attribute and the "expires" attribute in the <affiliation> element; and

d)	shall set the <p-id> child element of the <presence> root element to a globally unique value.

The MCData client shall send the SIP PUBLISH request according to 3GPP TS 24.229 [5].

5.8.3	Test description

5.8.3.1	Pre-test conditions

Test configuration:

-	Single cell configuration according to TS 36.579-1 [2] clause 5.2.2.2.1.

-	GNSS simulator to simulate a location.

Preamble:

-	The UE has performed procedure 'Initial registration' as described in TS 36.579-1 [2] clause 5.4.2.

-	The GNSS simulator is configured to simulate a location in the centre of Geographical area #1 to provide a location, as defined in TS 36.508 [24] Table 4.11.2-2, step 1 of scenario #4.

-	UE States at the end of the preamble:

-	The UE is in RRC_IDLE state.

-	The client is registered for MCData.

5.8.3.2	Test procedure sequence

Table 5.8.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Trigger the UE to reset UTC time and location. NOTE: The UTC time and location reset may be performed by MMI or AT command (+CUTCR). | - | - | - | - |
| 2 | Trigger the GNSS simulator to start step 2 of Scenario #4 to simulate the UE moving to location #7 inside Geographical area #1 as defined in TS 36.508 [24] Table 4.11.2-2. | - | - | - | - |
| 3 | Wait 10 sec to allow the simulated location to move approximately 100 m from the original location to location #7. | - | - | - | - |
| 4-6 | Check: Check: Does the UE (MCData Client) correctly perform steps 1a1-3 of procedure 'MCX Group Affiliation status change' as described in TS 36.579-1 [2] Table 5.3.34.3-1 to de-affiliate with group A? NOTE: The UE (MCData Client) sends a SIP PUBLISH message and the SS responds with a SIP 200(OK) message. | - | - | 1 | P |
| 7 | The procedure 'MCX communication release' as described in TS 36.579-1 [2] clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |
| 8 | Trigger the GNSS simulator to start step 4 of Scenario #4 to simulate the UE moving to location #8 inside Geographical area #1 as defined in TS 36.508 [24] Table 4.11.2-2. | - | - | - | - |
| 9 | Wait 10 sec to allow the simulated location to move approximately 100 m from the original location to location #8. | - | - | - | - |
| 10-12 | Check: Check: Does the UE (MCData Client) correctly perform steps 1a1-3 of procedure 'MCX Group Affiliation status change' as described in TS 36.579-1 [2] Table 5.3.34.3-1 to affiliate with group A? NOTE: The UE (MCData Client) sends a SIP PUBLISH message and the SS responds with a SIP 200(OK) message. | - | - | 2 | P |
| 13 | The procedure 'MCX communication release' as described in TS 36.579-1 [2] clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |

5.8.3.3	Specific message contents

Table 5.8.3.3-1: SIP PUBLISH from the UE (step 5, Table 5.8.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.34.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.11-1, condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Expires |  |  |  |  |
| delta-seconds | "0" |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 5.8.3.3-2 |  |  |  |
| MIME body part |  | PIDF |  |  |
| MIME-part-body | PIDF as described in Table 5.8.3.3-3 |  |  |  |

Table 5.8.3.3-2: MCData-Info in SIP PUBLISH (Table 5.8.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-request-uri | px_MCData_ID_User_A |  |  |  |

Table 5.8.3.3-3: PIDF in SIP PUBLISH (Table 5.8.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.5.1-3, condition AFFILIATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  |  |  |
| tuple |  |  |  |  |
| status | not present |  |  |  |
| ..p-id | any allowed value |  | TS 24.282 [31] clause 8.2.2 |  |

Table 5.8.3.3-4: SIP PUBLISH from the UE (step 11, Table 5.3.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.34.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.11-1, condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 5.8.3.3-2 |  |  |  |
| MIME body part |  | PIDF |  |  |
| MIME-part-body | PIDF as described in Table 5.8.3.3-5 |  |  |  |

Table 5.8.3.3-5: PIDF in SIP PUBLISH (Table 5.8.3.3-4

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.5.1-3, condition AFFILIATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  |  |  |
| ..p-id | any allowed value |  | TS 24.282 [31] clause 8.2.2 |  |

<END OF CHANGES>