3GPP TSG-RAN5 Meeting #103	R5-243544

# Fukuoka, Japan, 20 May – 24 May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-2 | CR | 0477 | rev | 1 | Current version: | 17.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding test applicability for multi-TRP PUSCH repetition test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The feature of PUSCH repetition with multi-TRP for type A and type B are specified under Rel-17 FeMIMO WI. New test cases are specifieed to verify the UE’s behavior. The relevant test applicabilities need to be specified along. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding test applicability for the new test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases can’t be executed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.508-2 CR 0643 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | 1. This CR is related to R5-243543. Condition A.4.3.2-1/HW1 and A.4.3.2-1/HW2 mentioned in Table 4.2-1 is introduced by R5-243543 and need to be implemented with a hard number by MCC 2. The interim number Cxxx_hw1 and Cxxx_hw2 introduced in Table 4.2-1 needs to be implemented with a hard number by MCC |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-243544. |  |  |  |  |  |  |  |  |

<Start of Changes>

4	Recommended Test Case Applicability

4.0	Introduction

The applicability of each individual test is identified in subclause 4.1. This is just a recommendation based on the purpose for which the test case was written.

The applicability of every test is formally expressed by the use of Boolean expressions that are based on parameters (ICS). The parameters (ICS) included in TS 38.508-2 [5] are used in the test case applicability condition without reference. Parameters (ICS) specified in 3GPP TS 36.523-2 [10] and 3GPP TS 34.229-2 [9] shall be referred with proper reference. The parameters (ICS) shall be set according to the capabilities of the UE on the operating band / band combination under test.

Additional information related to the Test Case (TC), e.g. affecting its dynamic behaviour or its execution may be provided as well.

The columns in subclause 4.1 have the following meaning:

Clause

The clause column indicates the clause number in TS 38.523-1 [2] that contains the test body.

Title

The title column describes the name of the test and contains the clause title of the clause in TS 38.523-1 [2] that contains the test body.

Release

The release column indicates the earliest release from which the test case is applicable. In some specific cases it may indicate the release(s) for which the TC is only applicable.

Note:	Some exceptions to this interpretation may be indicated in Notes in column 'Number of TC Executions'.

Applicability - Condition

The following notations are used for the applicability column:

R	recommended - the test case is recommended

O	optional – the test case is optional

N/A	not applicable - in the given context, the test case is not recommended.

Ci	conditional - the test is recommended ("R") or not ("N/A") depending on the support of other items. "i" is an integer identifying a unique conditional status expression which is defined immediately following the table. For nested conditional expressions, the syntax "IF ... THEN (IF ... THEN ... ELSE...) ELSE ..." is used to avoid ambiguities.

NOTE:	The conditions are defined in subclause 4.2.

Applicability - Comments

This column contains a verbal description of the condition.

Additional Information - Specific ICS

This column contains the mnemonics of ICS(s) affecting the dynamic behaviour of the TC.

Additional Information - Specific IXIT

This column contains the mnemonics of IXIT(s) affecting the dynamic behaviour of the TC.

Additional Information - Number of TC Executions

This column contains, wherever applicable, the recommended for certification purposes number of TC executions. It may contain also other information e.g. exceptions to the release applicable to the test. Clarifying notes are listed at the end of the same Table.

Additional Information - Release other RAT

In regard to a particular test case, this column provides information on the release which is used by the simulated network in the other (i.e. non 5GS) RAT(s) where applicable. For each applicable RAT the release shall be indicated in the format 'Rel-X RAT'. When multiple RATs are applicable the entries per RAT shall be separated by a comma. When a value for a 3GPP RAT is not provided but the RAT is in the scope of the test case then for this RAT the release indicated in the Release column applies (per default).

Note:	Currently for RedCap UEs that set the PICS complying with TS 38.508-2 [5] clause A.4.3.12, Rel-15 test cases and Rel-17 RedCap specific test cases are applicable. Applicability of other Rel-16 and Rel-17 test cases are under further study.

4.1	Protocol conformance test cases applicability

<Unchanged Text Skipped>

Table 4.1-2a: Applicability of Protocol conformance Layer 2 test cases, ref. TS 38.523-1 [2]

| Clause | TC Title | Release | Applicability Condition | Applicability Comment |
| --- | --- | --- | --- | --- |
| 7 | Layer 2 |  |  |  |
| 7.1 | NR Layer 2 |  |  |  |
| 7.1.1 | MAC |  |  |  |
| 7.1.1.1 | Random Access Procedures |  |  |  |
| 7.1.1.1.1 | Correct selection of RACH parameters / Random access preamble and PRACH resource explicitly signalled to the UE by RRC / contention free random access procedure | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.1.1a | Correct selection of RACH parameters / Random access preamble and PRACH resource explicitly signalled to the UE by PDCCH Order / contention free random access procedure | Rel-15 | R | UEs supporting 5GS |
| <Unchanged Lines Skipped> |  |  |  |  |
| 7.1.1.3.14.3 | Correct Handling of UL HARQ process / PUSCH Repetition Type A enhancement / repetition based on available slots / dynamic grant | Rel-17 | C290 | UEs supporting PUSCH repetitions based on available slots and dynamic indication of the number of repetitions for PUSCH |
| 7.1.1.3.14.4 | Correct Handling of UL HARQ process / PUSCH Repetition Type A enhancement / repetition based on available slots / configured grant | Rel-17 | C291 | UEs supporting PUSCH repetitions based on available slots and PUSCH transmissions with configured grant |
| 7.1.1.3.15 | Correct Handling of UL HARQ process / TBoMS procedure |  |  |  |
| 7.1.1.3.15.1 | Correct Handling of UL HARQ process / TBoMS procedure / DG and CG based transmission | Rel-17 | C292 | UEs supporting TB processing over multi-slot PUSCH |
| 7.1.1.3.15.2 | Correct Handling of UL HARQ process / TBoMS procedure / Repetition of TBoMS | Rel-17 | C293 | UEs supporting repetition of TB processing over multi-slot PUSCH |
| 7.1.1.3.16 | Correct Handling of UL grant / DRB configured with survival time |  |  |  |
| 7.1.1.3.16.1 | Correct Handling of UL grant / DRB configured with survival time / Split DRB | Rel-17 | C256 | UEs supporting services with survival time and NR-DC and PDCP-duplication over split DRB |
| 7.1.1.3.16.2 | Correct Handling of UL grant / DRB configured with survival time / MCG or SCG DRB / Intra-band contiguous CA | Rel-17 | C257 | UEs supporting services with survival time and intra-band contiguous CA and CA-based PDCP duplication over MCG or SCG DRB |
| 7.1.1.3.16.3 | Correct Handling of UL grant / DRB configured with survival time / MCG or SCG DRB / Intra-band non-contiguous CA | Rel-17 | C258 | UEs supporting services with survival time and intra-band non-contiguous CA and CA-based PDCP duplication over MCG or SCG DRB |
| 7.1.1.3.16.4 | Correct Handling of UL grant / DRB configured with survival time / MCG or SCG DRB / Inter-band CA | Rel-17 | C259 | UEs supporting services with survival time and inter-band CA and CA-based PDCP duplication over MCG or SCG DRB |
| 7.1.1.3.17 | Correct Handling of UL HARQ process / PUSCH Repetition / Multi-TRP |  |  |  |
| 7.1.1.3.17.1 | Correct Handling of UL HARQ process / PUSCH Repetition / Multi-TRP / PUSCH Repetition Type A / dynamic grant | Rel-17 | Cxxx_hw1 | UEs supporting 5GS and multi-TRP PUSCH repetition type A |
| 7.1.1.3.17.2 | Correct Handling of UL HARQ process / PUSCH Repetition / Multi-TRP / PUSCH Repetition Type B / dynamic grant | Rel-17 | Cxxx_hw2 | UEs supporting 5GS and multi-TRP PUSCH repetition type B |
| 7.1.1.4 | Transport Size Selection |  |  |  |
| 7.1.1.4.1 | DL-SCH Transport Block Size Selection |  |  |  |
| 7.1.1.4.1.1 | DL-SCH Transport Block Size selection / DCI format 1_0 | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.4.1.2 | Void |  |  |  |
| 7.1.1.4.1.3 | DL-SCH transport block size selection / DCI format 1_1 / RA type 0/RA Type 1 / 2 Codewords enabled | Rel-15 | C64 | UEs supporting 5GS and The maximum number of spatial multiplexing layer(s) supported by the UE for DL reception is 8 Layers. For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. If absent, the UE doesn’t support MIMO on this carrier |
| 7.1.1.4.1.4 | DL-SCH transport block size selection / DCI format 1_1 / RA type 0/RA Type 1 / 2 Codewords enabled / 256QAM | Rel-15 | C65 | UEs supporting 5GS and The maximum number of spatial multiplexing layer(s) supported by the UE for DL reception is 8 Layers. For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. If absent, the UE doesn’t support MIMO on this carrier and 256QAM for PUSCH |
| 7.1.1.4.1.5 | DL-SCH transport block size selection / DCI format 1_2 | Rel-16 | C146 | Ues supporting monitoring DCI format 1_2 for DL scheduling and monitoring DCI format 0_2 for UL scheduling |
| 7.1.1.4.2 | UL-SCH Transport Block Size Selection |  |  |  |
| 7.1.1.4.2.1 | UL-SCH Transport Block Size selection / DCI format 0_0 / Transform precoding disabled | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.4.2.2 | Void |  |  |  |
| 7.1.1.4.2.3 | UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / Transform precoding disabled | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.4.2.4 | UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / 256QAM / Transform precoding disabled | Rel-15 | C11 | UEs supporting 5GS and 256QAM for PDSCH for FR1/FR2 |
| 7.1.1.4.2.5 | UL-SCH Transport Block Size selection / DCI format 0_0 / Transform precoding and 64QAM | Rel-15 | R | UEs supporting 5GS |

<Unchanged Text Skipped>

4.2	Protocol conformance test cases applicability conditions

Table 4.2-1: Applicability of Protocol conformance test cases Conditions

| Condition | Test case Selection Expression | Comment |
| --- | --- | --- |
| C01 | IF A.4.1-3/2 THEN R ELSE N/A | UEs supporting EN-DC |
| C02 | IF A.4.3.4-1/2 OR A.4.3.4-1/3 THEN R ELSE N/A | UEs supporting 5GS and RLC UM Mode |
| C03 | IF A.4.3.5-1/1 THEN R ELSE N/A | UEs supporting 5GS and Long DRX Cycle |
| C04 | IF A.4.3.5-1/2 THEN R ELSE N/A | UEs supporting 5GS and short DRX cycle |
| C05 | IF A.4.3.4-1/3 THEN R ELSE N/A | UEs supporting 5GS and RLC UM with 6-bit length of RLC sequence number |
| <Unchanged Text Skipped> |  |  |
| C316 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [10] A.4.4-1/33 AND A.4.3.7-1/12 AND A.4.3.7-1/58 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and EPS IMS Voice (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and EPS fallback and being configured for No E-UTRA Disabling In 5GS |
| C317 | IF A.4.1-5/1 AND A.4.4-1/16 AND A.4.3.7-1/6 THEN R ELSE N/A | UEs supporting 5G Core and SDT via Configured Grant Type 1 in RRC_INACTIVE state and SMS over NAS |
| C318 | IF A.4.1-5/1 AND A.4.3.2-2/1 THEN R ELSE N/A | UEs supporting 5G Core and NR CA with NR shared spectrum channel access |
| C319 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.1-5/2 AND [10] A.4.1-1/5 AND A.4.3.7-1/13 AND A.4.3.7-1/60 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and 5G core over non-3GPP Access Network and WLAN and R17 ATSSS of establishing a PDN connection as the user plane resource of an MA PDU session in 5GS |
| C320 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.1-5/2 AND [10] A.4.1-1/5 AND A.4.3.7-1/13 AND A.4.3.7-1/48 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and 5G core over non-3GPP Access Network and WLAN and R16 ATSSS |
| C321 | IF A.4.1-5/1 AND A.4.4-1/17 AND A.4.3.2-1/138 THEN R ELSE N/A | UEs supporting 5G Core and NR NTN access and UE reporting of information related to TA pre-compensation |
| C322 | IF A.4.1-5/1 AND A.4.4-1/17 AND A.4.3.4-1/3 THEN R ELSE N/A | UEs supporting 5G Core and NR NTN access and RLC UM Mode |
| C323 | IF A.4.1-5/1 AND A.4.4-1/17 AND A.4.3.6-1/76 THEN R ELSE N/A | UE supporting 5G Core and NR NTN access and UE supporting location-based triggered measurement reporting (i.e., event D1) |
| C324 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.3.7-1/58 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and No E-UTRA Disabling In 5GS |
| Cxxx_hw1 | IF A.4.3.2-1/HW1 THEN R ELSE N/A | UEs supporting 5GS and multi-TRP PUSCH repetition type A |
| Cxxx_hw2 | IF A.4.3.2-1/HW2 THEN R ELSE N/A | UEs supporting 5GS and multi-TRP PUSCH repetition type B |

< End of Changes >