3GPP TSG-RAN5 Meeting #103	R5-243146

# Fukuoka, Japan, 20 May – 24 May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2829 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to NR V2X MPR test cases to add PC2 and intra-band concurrent operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN4 introduced V2X PC2 requirement and intra-band concurrent operation requirement under NR_SL_enh WI. Corresponding test cases need to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Aligning minimum requirements for the PC2 and intra-band concurrent operantion in 6.2E.2.0 - Adding test detail for PC2 in 6.2E.2.1 and 6.2E.2.1D - Adding test detail for intra-band concurrent operation in 6.2E.2.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test is missing for PC2 and intra-band concurrent operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2E.2.0, 6.2E.2.1, 6.2E.2.1D, 6.2E.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 0909 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.2E.2	UE maximum output power reduction for V2X

6.2E.2.0	Minimum conformance requirements

6.2E.2.0.1	General

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 5.2E.1-1, this clause specifies the allowed Maximum Power Reduction (MPR) power for V2X physical channels and signals due to PSCCH/PSSCH, PSFCH and S-SSB transmission.

6.2E.2.0.2	MPR for Power class 2 and Power class 3 V2X UE

For contiguous allocation of PSCCH and PSSCH simultaneous transmission, the allowed MPR for the maximum output power for NR V2X physical channels PSCCH and PSSCH shall be as specified in Table 6.2E.2.0.2-1 for Power class 3 NR V2X UE and Table 6.2E.2.0.2-2 for power class 2 NR V2X UE.

Table 6.2E.2.0.2-1: Maximum Power Reduction (MPR) for power class 3 NR V2X

| Modulation |  | Channel bandwidth/MPR (dB) |  |
| --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |
| CP-OFDM | QPSK | ≤ 4.5 | ≤ 2.5 |
|  | 16QAM | ≤ 4.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 4.5 |  |
|  | 256 QAM | ≤ 7.0 |  |

Table 6.2E.2.0.2-2: Maximum Power Reduction (MPR) for power class 2 NR V2X

| Modulation |  | Channel bandwidth/MPR (dB) |  |
| --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |
|  | QPSK | ≤ 5.5 | ≤ 2.5 |
| CP-OFDM | 16QAM |  |  |
|  | 64 QAM | ≤ 6 | ≤ 4.5 |
|  | 256 QAM | ≤ 7.0 |  |

For NR V2X UE supporting SL MIMO or Tx diversity, the allowed MPR for the maximum output power for NR V2X physical channels PSCCH and PSSCH are specified in Table 6.2E.2.2-3 for power class 2 UE.

Table 6.2E.2.0.2-3: Maximum Power Reduction (MPR) for power class 2 NR V2X with dual Tx

| Modulation |  | Channel bandwidth/MPR (dB) |  |
| --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 3.0 |
|  | 16QAM |  |  |
|  | 64 QAM | ≤ 7.0 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 |  |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1.

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB  ≤  ceil(NRB/2)

where ceil(x) is the smallest integer greater than or equal to x.

The RB allocation is an Outer RB allocation for all other allocations which are not an Inner RB allocation.

For PSFCH with single RB transmission for PC3 NR V2X UE, the required MPR is defined as follow

MPR_PSFCH = 3.5 dB

For contiguous and non-contiguous allocation for simultaneous PSFCH transmission for PC3 NR V2X UE, the required MPR are specified as follow

MPR_PSFCH = CEIL {MA_PSFCH, 0.5}

Where MA_PSFCH for power class 3 is defined as follows

MA_PSFCH =	7.5		; 0.00< NGap/NRB ≤ 0.55

=    12.0	; 0.55< NGap/NRB ≤1.0

For PSFCH with single RB transmission for PC2 NR V2X UE, the required MPR is defined as follow

MPR_PSFCH =  4.5 dB

For contiguous and non-contiguous allocation for simultaneous PSFCH transmission for PC2 NR V2X UE, the required MPR are specified as follow

MPR_PSFCH = CEIL {MA_PSFCH, 0.5}

Where MA is defined as follows

Where MA_PSFCH for power class 2 is defined as follows

MA_PSFCH =	8.5		; 0.00 ≤ NGap/NRB < 0.4

=   10.0	; 0.4 ≤ NGap/NRB < 0.55

=   14.0	; 0.55 ≤ NGap/NRB ≤ 1.0

Where,

NGap is the gap RB amount between RBstart and RBend for contiguous and non-contiguous allocation simultaneous PSFCH transmission. (NGap = RBend - RBstart)

CEIL{MA, 0.5} means rounding upwards to closest 0.5dB.

The allowed MPR for the maximum output power for NR V2X physical channels on S-SSB transmission shall be specified in Table 6.2E.2.0.2-2 for power class 3 and power class 2.

Table 6.2E.2.0.2-2: Maximum Power Reduction (MPR) for S-SSB transmission for power class 3 and power class 2 NR V2X

| Channel | MPRS-SSB (dB) |  |
| --- | --- | --- |
|  | Outer RB allocations | Inner RB allocations |
| S-SSB | ≤ 6.0 | ≤ 2.5 |

For NR V2X UE with two transmit antenna connectors, the allowed Maximum Power Reduction (MPR) values specified in clause 6.2E.2.0 for PC3 and PC2 shall apply to the maximum output power specified in Table 6.2E.1.1.3-1. For UE supporting SL MIMO, the requirements shall be met with SL MIMO configurations defined in Table 6.2D.1.3-2. For UE supporting SL MIMO or Tx diversity, the maximum output power is defined as the sum of the maximum output power from each UE antenna connector.

For the UE maximum output power modified by MPR, the power limits specified in clause 6.2E.4.0 apply.

6.2E.2.0.3	MPR for Power class 2 and Power class 3 V2X con-current operation

For the inter-band con-current NR V2X operation, the allowed maximum power reduction (MPR) for the maximum output power shall be applied per each component carrier. The MPR requirements in clause 6.2.2.3 apply for NR Uu operation in licensed band, and the MPR requirements in in clause 6.2E.2.0 apply for NR sidelink operation in licensed band or Band n47.

For the intra-band con-current NR V2X operation with contiguous RB allocation, the allowed maximum power reduction (MPR) for NR V2X physical channels PSCCH and PSSCH shall be as specified in Table 6.2E.2.0.3-1 for Power class 3 V2X con-current UE.

Table 6.2E.2.0.3-1: MPR for contiguous RB allocation for power class 3 NR V2X con-current UE

| Higher Modulation order between Sidelink and Uplink |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocation | Outer RB allocation |
| CP-OFDM | QPSK | ≤ 2.5 | ≤ 4.5 |
|  | 16QAM | ≤ 2.5 | ≤ 4.5 |
|  | 64QAM | ≤ 4.5 | ≤ 5.0 |
|  | 256QAM | ≤ 6.0 | ≤ 6.0 |

For bandwidth class B with contiguous RB allocation, the following parameters are defined to specify valid RB allocation ranges for Inner and Outer RB allocations:

An RB allocation is contiguous if LCRB1 = 0 or LCRB2 = 0 or (LCRB1  0 and LCRB2  0 and RBStart1 + LCRB1 = NRB1 and RBStart2 = 0), where RBStart1, LCRB1, and NRB1 are for SL CC1, RBStart2, LCRB2, and NRB2 are for UL CC2. SL CC1 is the component carrier with lower frequency.

In contiguous NR V2X intra-band con-current operation, a contiguous allocation is an inner allocation if

RBStart,Low  ≤  RBStart_SL&UL  ≤  RBStart,High, and NRB_alloc  ≤  ceil(NRB,agg /2),

where

RBStart,Low = max(1, floor(NRB_alloc /2))

RBStart,High = NRB,agg – RBStart,Low – NRB,alloc,

with

NRB_alloc= LCRB1 ∙ 2µ1 + LCRB2 ∙ 2µ2

NRB_alloc= (NRB1 - RBStart1)∙ 2µ1 + (RBStart2 + LCRB2 ) ∙ 2µ2,

NRB,agg=NRB1∙2µ1+ NRB2∙2µ2.

If LCRB1 =0, RBStart_SL&UL = NRB1∙2µ1+ RBStart2∙2µ2,

if LCRB1 > 0, RBStart_SL&UL = RBStart1∙2µ1.

Where, µ1 and µ2 is 0, 1 and 2 for SCS of 15kHz, 30kHz and 60kHz respectively.

A contiguous allocation that is not an Inner contiguous allocation is an Outer contiguous allocation.

For the intra-band con-current NR V2X operation with non-contiguous RB allocation, the allowed maximum power reduction (MPR) for NR V2X physical channels PSCCH and PSSCH shall be as specified in Table 6.2E.2.0.3-2 for Power class 3 V2X con-current UE.

Table 6.2E.2.0.3-2: MPR for non-contiguous RB allocation for power class 3 NR V2X con-current UE

| Higher Modulation order between Sidelink and Uplink |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner RB allocation | Outer1 RB allocation | Outer2 RB allocation |
| CP-OFDM | QPSK | ≤ 2.5 | ≤ 4.0 | ≤ 4.5 |
|  | 16QAM | ≤ 2.5 | ≤ 4.0 | ≤ 4.5 |
|  | 64QAM | ≤ 4.5 | ≤ 4.5 | ≤ 5.0 |
|  | 256QAM | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |

For bandwidth classes B with non-contiguous RB allocation, the following parameters are defined to specify valid RB allocation ranges for Inner, Outer1 and Outer2 RB allocations:

Non-Contiguous RB allocation is defined as RBStart1 + LCRB1 < NRB1, or RBStart2 > 0, when both SL CC and UL CC are activated and allocated with RB(s), where RBStart1, LCRB1, and NRB1 are for SL CC1, RBStart2, LCRB2, and NRB2 are for UL CC2. SL CC1 is the component carrier with lower frequency.

In contiguous NR V2X intra-band con-current operation, a non-contiguous RB allocation is a non-contiguous Inner RB allocation if the following conditions are met:

RBStart,Low  ≤  RBStart_CA  ≤  RBStart,High and NRB_alloc ≤  ceil((BWChannel_SL&UL / 3 – BWgap ) / 0.18MHz),

where

NRB_alloc = (NRB1 - RBStart1)∙ 2µ1 + (RBStart2 + LCRB2 ) ∙ 2µ2, RBStart_SL&UL = RBStart1∙21

RBStart,Low = max(1, floor(NRB_alloc + (BWgap – BWGB,low)/0.18MHz))

RBStart,High = floor((BWChannel_SL&UL – 2 ∙ BWgap – BWGB,low)/0.18MHz – 2 ∙ NRB_alloc)

BWGB,low =Foffset,low – (NRB1∙12+1)∙SCS1/2

BWgap is the bandwidth of the gap between NRB1 and NRB2 possible allocations of SL CC1 and UL CC2 respectively.

In contiguous NR V2X intra-band con-current operation, a non-contiguous RB allocation is a non-contiguous outer 1 RB allocation if the following conditions are met:

RBStart,Low  ≤  RBStart_SL&UL  ≤  RBStart,High and NRB_alloc ≤  ceil((3 BWChannel_SL&UL / 5 – BWgap) / 0.18MHz)

where

RBStart,Low = max(1, 2 ∙ NRB_alloc – floor( (BWChannel_SL&UL – 2 ∙ BWgap + BWGB,low)/0.18MHz)),

RBStart,High = floor((2 ∙ BWChannel_SL&UL – 3 ∙ BWgap – BWGB,low) / 0.18MHz – 3 ∙ NRB_alloc)

NRB_alloc , RBStart_SL&UL , BWgap and BWGB,low are as defined for the Inner region.

In contiguous NR V2X intra-band con-current operation, a non-contiguous allocation is an Outer 2 allocation if it is neither a non-contiguous Inner allocation nor an Outer 1 allocation.

For PSFCH with single RB transmission for PC3 NR V2X intra-band con-current UE, the required MPR is specified in clause 6.2E.2.0.2 shall be applied.

For the allowed MPR for S-SSB transmission for PC3 NR V2X intra-band con-current UE, the required MPR is specified in clasue 6.2E.2.0.2 shall be applied.

For the intra-band con-current NR V2X operation with contiguous RB allocation in contiguous carrier, the allowed maximum power reduction (MPR) for NR V2X physical channels PSCCH and PSSCH shall be as specified in Table 6.2E.2.0.3-3 for Power class 2 V2X con-current UE.

Table 6.2E.2.0.3-3: MPR for contiguous RB allocation for power class 2 NR V2X con-current UE

| Higher Modulation order between Sidelink and Uplink |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocation | Outer RB allocation |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 5.5 |
|  | 16QAM | ≤ 4.0 | ≤ 5.5 |
|  | 64QAM | ≤ 5.5 | ≤ 6.0 |
|  | 256QAM | ≤ 7.5 | ≤ 7.5 |

For the intra-band con-current NR V2X operation with non-contiguous RB allocation in contiguous carrier, the allowed maximum power reduction (MPR) for NR V2X physical channels PSCCH and PSSCH shall be as specified in Table 6.2E.2.0.3-4 for Power class 2 V2X con-current UE.

Table 6.2E.2.0.3-4: MPR for non-contiguous RB allocation for power class 2 NR V2X con-current UE

| Higher Modulation order between Sidelink and Uplink |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner RB allocation | Outer1 RB allocation | Outer2 RB allocation |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 5.5 | ≤ 6.0 |
|  | 16QAM | ≤ 4.5 | ≤ 5.5 | ≤ 6.5 |
|  | 64QAM | ≤ 5.5 | ≤ 6.5 | ≤ 7.0 |
|  | 256QAM | ≤ 8.0 | ≤ 8.0 | ≤ 8.0 |

The parameters in clause 6.2E.2.0.3 are considered to determine MPR values according to RB allocation.

For PSFCH with single RB transmission for PC2 NR V2X intra-band con-current UE, the required MPR is specified in clause 6.2E.2.0.2 shall be applied.

For the allowed MPR for S-SSB transmission for PC2 NR V2X intra-band con-current UE, the required MPR is specified in clause 6.2E.2.0.2 shall be applied.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2E.2

6.2E.2.1	UE maximum output power reduction for V2X / non-concurrent operation

Editor’s Note: The test case is not completed for PSFCH and PSBCH measurement due to the following aspects are not yet determined:

- Measurement period of PSFCH and PSBCH is FFS.

6.2E.2.1.1	Test purpose

Same test purpose as in 6.2.2.1.

6.2E.2.1.2	Test applicability

This test case applies to all types of UE release 16 and forward that support NR V2X sidelink communication.

NOTE:	Test execution is not necessary if TS 38.521-1 6.5E.2.4.1 is executed.

6.2E.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2E.2.0.

6.2E.2.1.4	Test description

6.2E.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2E.1-1 and Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2E.2.1.4.1-1 to 6.2E.2.1.4.1-3. The details of the V2X reference measurement channels (RMCs) are specified in Annex A.7.5 and the GNSS configuration in TS 38.508-1 [5] subclause 4.11.

Table 6.2E.2.1.4.1-1: Test Configuration Table for contiguous PSCCH and PSSCH allocation for PC3

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1.8 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | V2X Configuration to Transmit |  |  |
|  |  | Modulation |  | PSCCH and PSSCH RB allocation (Note 1) |
| 1 | Default | QPSK |  | Outer_Full |
| 2 | Default | QPSK |  | Inner_Full |
| 3 | Default | 16QAM |  | Outer_Full |
| 4 | Default | 16QAM |  | Inner_Full |
| 5 | Default | 64QAM |  | Outer_Full |
| 6 | Default | 256QAM |  | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1E-1. |  |  |  |  |

Table 6.2E.2.1.4.1-1a: Test Configuration Table for contiguous PSCCH and PSSCH allocation for PC2

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1.8 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | V2X Configuration to Transmit |  |  |
|  |  | Modulation |  | PSCCH and PSSCH RB allocation (Note 1) |
| 1 | Default | 16QAM |  | Outer_Full |
| 2 | Default | 16QAM |  | Inner_Full |
| 3 | Default | 64QAM |  | Outer_Full |
| 4 | Default | 64QAM |  | Inner_Full |
| 5 | Default | 256QAM |  | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1E-1. |  |  |  |  |

Table 6.2E.2.1.4.1-2: Test Configuration Table for PSFCH for PC2 and PC3

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1.8 |  |  | Low range, High range |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Freq | V2X Configuration to Transmit |  |
|  |  | PSFCH RB allocation (Note 1) |  |
| 1 | Low range | PSFCH_1RB_Left |  |
| 2 | High range | PSFCH_1RB_Right |  |
| 3 | Low range | PSFCH_2RB_Left |  |
| 4 | High range | PSFCH_2RB_Right |  |
| 5 | Default | PSFCH_Max_Gap |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1E-2. |  |  |  |

Table 6.2E.2.1.4.1-3: Test Configuration Table for S-SSB for PC2 and PC3

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1.8 |  |  | Low range, High range |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Freq | V2X Configuration to Transmit |  |
|  |  | S-SSB RB allocation (Note 1) |  |
| 1 | Low range | S-SSB_Low |  |
| 2 | High range | S-SSB_High |  |
| 3 | Default | S-SSB_Mid |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1E-3. |  |  |  |

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.9.1 for TE diagram and section A.3.2.7 for UE diagram.

2.	The parameter settings for the NR sidelink transmission over PC5 are pre-configured according to TS 38.508-1 [5] subclause 4.10. Message content exceptions are defined in clause 6.2E.2.1.4.3.

3.	The V2X Reference Measurement Channel is set according to Table 6.2E.2.1.4.1-1 to Table 6.2E.2.1.4.1-3.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS 38.508-1 [5] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0.

6.2E.2.1.4.2	Test procedure

Subtest 1: PSCCH/PSSCH

1.	Ensure the UE is in state Out_of_Coverage with generic procedure parameters Sidelink On, Test Loop Function On with UE test loop mode E closed for Transmit Mode according to TS 38.508-1 [5] clause 4.5.

2.	The UE starts to perform the NR sidelink communication according to SL-PreconfigurationNR. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the NR sidelink RMC.

3.	Measure the mean power of the UE in the channel bandwidth according to the test configuration from Table 6.2E.2.1.4.1-1. The period of measurement shall be at least continuous duration of one active sub-frame (1ms) excluding guard symbols.

Subtest 2: PSFCH

1.	Ensure the UE is in state Out_of_Coverage with generic procedure parameters Sidelink On, Cast type Unicast, Test Loop Function On with UE test loop mode E closed for Receive Mode according to TS 38.508-1 [5] clause 4.5.

2.	The UE starts to perform the NR sidelink reception according to SL-PreconfigurationNR.

3.	The UE’s PSFCH transmission occasion is on slot n according to Table 6.2E.2.1.4.1-2. SS transmits PSSCH on combination of slot and subchannel as below:

a) Test ID 1: slot n-6, Lowest sub-channel

b) Test ID 2: slot n-3, Highest sub-channel

c) Test ID 3: slot n-6 and n-5, Lowest sub-channel

d) Test ID 4: slot n-4 and n-3, Highest sub-channel

e) Test ID 5: slot n-6, Highest sub-channel and slot n-3, Highest sub-channel

4.	Measure the mean power of the UE on slot n in the channel bandwidth according to the test configuration from Table 6.2E.2.1.4.1-2. The period of measurement is FFS.

Subtest 3: S-SSB

1.	Ensure the UE is in state Out_of_Coverage with generic procedure parameters Sidelink On according to TS 38.508-1 [5] clause 4.5. The UE is synchronized to GNSS,

2.	The UE transmits PSBCH according SL-PreconfigurationNR.

3.	Measure the mean power of the S-SSB in the channel bandwidth according to the test configuration from Table 6.2E.2.1.4.1-3. The period of measurement is FFS.

6.2E.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.10 with the following exceptions.

Table 6.2E.2.1.4.3-1: SL-ResourcePool for PSCCH/PSSCH Testing

| Derivation Path: TS 38.508-1 [5], Table 4.6.6-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-PSCCH-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sl-TimeResourcePSCCH-r16 | As defined in Table 6.1E-2 |  |  |
| sl-FreqResourcePSCCH-r16 | As defined in Table 6.1E-2 |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-SubchannelSize-r16 | As defined in Table 6.1E-2 |  |  |
| sl-StartRB-Subchannel-r16 | As defined in Table 6.1E-2 |  |  |
| sl-NumSubchannel-r16 | As defined in Table 6.1E-2 |  |  |
| } |  |  |  |

Table 6.2E.2.1.4.3-2: SL-ResourcePool for PSFCH Testing

| Derivation Path: TS 38.508-1 [5], Table 4.6.6-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-PSFCH-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sl-PSFCH-Period-r16 | sl4 |  |  |
| sl-PSFCH-RB-Set-r16 | As defined in Table 6.1E-2 |  |  |
| sl-NumMuxCS-Pair-r16 | n1 |  |  |
| sl-MinTimeGapPSFCH-r16 | sl3 |  |  |
| sl-PSFCH-HopID-r16 | Not present | Default frequency hopping ID 0 is used |  |
| sl-PSFCH-CandidateResourceType-r16 | startSubCH |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-SubchannelSize-r16 | As defined in Table 6.1E-2 |  |  |
| sl-StartRB-Subchannel-r16 | As defined in Table 6.1E-2 |  |  |
| sl-NumSubchannel-r16 | As defined in Table 6.1E-2 |  |  |
| } |  |  |  |

Table 6.2E.2.1.4.3-3: SL-FreqConfigCommon for S-SSB Testing

| Derivation Path: TS 38.508-1 [5], Table 4.6.6-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-FreqConfigCommon-r16 ::= SEQUENCE { |  |  |  |
| sl-AbsoluteFrequencySSB-r16 | According to section 4.3.1.8 of TS 38.508-1 [5] |  |  |
| } |  |  |  |

6.2E.2.1.5	Test requirement

The maximum output power, derived in step 3 of Subtest 1, step 4 of Subtest 2 and step 3 of Subtest 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2E.2.1.5-1 to Table 6.2E.2.1.5-6.

Table 6.2E.2.1.5-1: PC3 UE MPR test requirement for contiguous PSCCH/PSSCH (Bands n38, n47)

| Test ID | PPowerClass (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 4.5 | 0 | 18.5 | 4 | 25.0 + TT | 14.5-TT |
| 2 | 23 | 2.5 | 0 | 20.5 | 2.5 | 25.0 + TT | 18-TT |
| 3 | 23 | 4.5 | 0 | 18.5 | 4 | 25.0 + TT | 14.5-TT |
| 4 | 23 | 2.5 | 0 | 20.5 | 2.5 | 25.0 + TT | 18-TT |
| 5 | 23 | 4.5 | 0 | 18.5 | 4 | 25.0 + TT | 14.5-TT |
| 6 | 23 | 7.0 | 0 | 16 | 5 | 25.0 + TT | 11-TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2E.2.1.5-4. |  |  |  |  |  |  |  |

Table 6.2E.2.1.5-2: PC3 UE MPR test requirement for PSFCH (Bands n38, n47)

| Test ID | PPowerClass (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 3.5 | 0 | 19.5 | 3.5 | 25.0 + TT | 16-TT |
| 2 | 23 | 7.5 | 0 | 15.5 | 5 | 25.0 + TT | 10.5-TT |
| 3 | 23 | 7.5 | 0 | 15.5 | 5 | 25.0 + TT | 10.5-TT |
| 4 | 23 | 12 | 0 | 11 | 6 | 25.0 + TT | 5-TT |
| 5 | 23 | 12 | 0 | 11 | 6 | 25.0 + TT | 5-TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2E.2.1.5-4. |  |  |  |  |  |  |  |

Table 6.2E.2.1.5-3: PC2 and PC3 UE MPR test requirement for S-SSB (Bands n38, n47)

| Test ID | PPowerClass (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 6 | 0 | 17 | 5 | 25.0 + TT | 12-TT |
| 2 | 23 | 6 | 0 | 17 | 5 | 25.0 + TT | 12-TT |
| 3 | 23 | 2.5 | 0 | 20.5 | 2.5 | 25.0 + TT | 18-TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2E.2.1.5-4. |  |  |  |  |  |  |  |

Table 6.2E.2.1.5-4: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | FFS | FFS |

Table 6.2E.2.1.5-5: PC2 UE MPR test requirement for contiguous PSCCH/PSSCH (Bands n38, n47)

| Test ID | PPowerClass (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 5.5 | 0 | 20.5 | 2.5 | 28.0 + TT | 18-TT |
| 2 | 26 | 2.5 | 0 | 23.5 | 2.0 | 28.0 + TT | 21.5-TT |
| 3 | 26 | 6 | 0 | 20 | 2.5 | 28.0 + TT | 17.5-TT |
| 4 | 26 | 4.5 | 0 | 21.5 | 2.0 | 28.0 + TT | 19.5-TT |
| 5 | 26 | 7 | 0 | 19 | 3.5 | 28.0 + TT | 15.5-TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2E.2.1.5-4. |  |  |  |  |  |  |  |

Table 6.2E.2.1.5-6: PC2 UE MPR test requirement for PSFCH (Bands n38, n47)

| Test ID | PPowerClass (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 4.5 | 0 | 21.5 | 2.0 | 28.0 + TT | 19.5-TT |
| 2 | 26 | 8.5 | 0 | 17.5 | 5.0 | 28.0 + TT | 12.5-TT |
| 3 | 26 | 8.5 | 0 | 17.5 | 5.0 | 28.0 + TT | 12.5-TT |
| 4 | 26 | 14 | 0 | 12 | 6.0 | 28.0 + TT | 6.0-TT |
| 5 | 26 | 14 | 0 | 12 | 6.0 | 28.0 + TT | 6.0-TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2E.2.1.5-4. |  |  |  |  |  |  |  |

6.2E.2.1D	UE maximum output power reduction for V2X / non-concurrent operation / SL-MIMO

Editor’s Note: The test case is not completed due to the following aspects are not yet determined:

- Uplink RMC is TBD in TS 38.101-1

- Preconfiguration is not complete in 38.508-1

- Test state and generic procedure are TBD in 38.508-1

- Measurement period of PSFCH and PSBCH is FFS.

- Connection diagram for SL-MIMO is TBD

6.2E.2.1D.1	Test purpose

Same test purpose as in 6.2E.2.1.

6.2E.2.1D.2	Test applicability

This test case applies to all types of UE release 16 and forward that support NR V2X sidelink communication and SL-MIMO.

NOTE:	Test execution is not necessary if TS 38.521-1 6.5E.2.4.1D is executed.

6.2E.2.1D.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2E.2.0.

6.2E.2.1D.4	Test description

6.2E.2.1D.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.2E.1-1 and table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2E.2.1D.4.1-1. The details of the V2X reference measurement channels (RMCs) are specified in Annexes TBD and the GNSS configuration in TS 38.508-1 [5] subclause 4.11.

Table 6.2E.2.1D.4.1-1: Test Configuration Table for contiguous PSCCH and PSSCH allocation

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1.8 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | V2X Configuration to Transmit |  |  |
|  |  | Modulation |  | PSCCH and PSSCH RB allocation (Note 1) |
| 1 | Default | QPSK |  | Outer_Full |
| 2 | Default | QPSK |  | Inner_Full |
| 3 | Default | 16QAM |  | Outer_Full |
| 4 | Default | 16QAM |  | Inner_Full |
| 5 | Default | 64QAM |  | Outer_Full |
| 6 | Default | 256QAM |  | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1E-1. |  |  |  |  |

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure TBD for TE diagram and section TBD for UE diagram.

2.	The parameter settings for the NR sidelink transmission over PC5 are pre-configured according to TS 38.508-1 [5] subclause 4.10. Message content exceptions are defined in clause 6.2E.2.1D.4.3.

3.	The V2X Reference Measurement Channel is set according to Table 6.2E.2.1D.4.1-1.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS38.508-1 [5] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state 4-A as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On) and Transmit Mode with SL-MIMO.

6.2E.2.1D.4.2	Test procedure

1.	The UE starts to perform the NR sidelink communication according to SL-PreconfigurationNR with 2-layer MIMO codebook TPMI 0. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the NR sidelink RMC.

2.	Measure the sum of mean power of the UE at each transmit antenna connector in the channel bandwidth according to the test configuration from Table 6.2E.2.1D.4.1-1. The period of measurement shall be at least continuous duration of one active sub-frame (1ms) excluding guard symbols.

6.2E.2.1D.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.10 with the following exceptions.

Table 6.2E.2.1D.4.3-1: SL-ResourcePool for PSCCH/PSSCH Testing

| Derivation Path: TS 38.508-1 [5], Table 4.6.6-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-PSCCH-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sl-TimeResourcePSCCH-r16 | As defined in Table 6.1E-2 |  |  |
| sl-FreqResourcePSCCH-r16 | As defined in Table 6.1E-2 |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-SubchannelSize-r16 | As defined in Table 6.1E-2 |  |  |
| sl-StartRB-Subchannel-r16 | As defined in Table 6.1E-2 |  |  |
| sl-NumSubchannel-r16 | As defined in Table 6.1E-2 |  |  |
| } |  |  |  |

6.2E.2.1D.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2E.2.1D.5-1.

Table 6.2E.2.1D.5-1: PC3 UE MPR test requirement for contiguous PSCCH/PSSCH (Bands n38, n47)

| Test ID | PPowerClass (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 4.5 | 0 | 18.5 | 5 | 3 | 25.0 + TT | 13.5 + TT |
| 2 | 23 | 2.5 | 0 | 20.5 | 6 | 3 | 25.0 + TT | 14.5 + TT |
| 3 | 23 | 4.5 | 0 | 18.5 | 5 | 3 | 25.0 + TT | 13.5 + TT |
| 4 | 23 | 2.5 | 0 | 20.5 | 6 | 3 | 25.0 + TT | 14.5 + TT |
| 5 | 23 | 4.5 | 0 | 18.5 | 5 | 3 | 25.0 + TT | 13.5 + TT |
| 6 | 23 | 7.0 | 0 | 16 | 5 | 3 | 25.0 + TT | 11.0 + TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2E.2.1D.5-2. |  |  |  |  |  |  |  |  |

Table 6.2E.2.1D.5-2: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | FFS | FFS |

Table 6.2E.2.1D.5-3: PC2 UE MPR test requirement for contiguous PSCCH/PSSCH (Bands n38, n47)

| Test ID | PPowerClass (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 6.0 | 0 | 20.0 | 6 | 3 | 28.0 + TT | 14.0 + TT |
| 2 | 26 | 3.0 | 0 | 23.0 | 3 | 3 | 28.0 + TT | 20.0 + TT |
| 3 | 26 | 7.0 | 0 | 19.0 | 5 | 3 | 28.0 + TT | 14.0 + TT |
| 4 | 26 | 5.5 | 0 | 20.5 | 6 | 3 | 28.0 + TT | 14.5 + TT |
| 5 | 26 | 9.0 | 0 | 17.0 | 5 | 3 | 28.0 + TT | 12.0 + TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2E.2.1D.5-2. |  |  |  |  |  |  |  |  |

6.2E.2.2	UE maximum output power reduction for V2X / concurrent operation

Editor’s Note:

-  MU/TT for aggregated BW > 100MHz is TBD.

The test of intra-band concurrent operation is not completed due to the following aspects are not yet determined:

- RB allocation for intra-band concurrent operation is TBD

- Test frequency for intra-band concurrent operation is TBD

6.2E.2.2.1	Test purpose

To verify that the reduction of UE transmitted power due to higher order modulations and transmit bandwidth configurations is within the allowed range. An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2E.2.2.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR V2X sidelink communication.

6.2E.2.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2E.2.0.

6.2E.2.2.4	Test description

6.2E.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2E.2-1 and Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2E.2.2.4.1-1. The details of the V2X reference measurement channels (RMCs) are specified in Annexes A.7.5. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2E.2.2.4.1-1: Test Configuration Table for PC3 inter-band concurrent operation

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range for NR Uu and V2X carrier High range for NR Uu and V2X carrier |  |  |  |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Lowest for NR Uu and V2X carrier Highest for NR Uu and V2X carrier |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest for NR Uu and V2X carrier Highest for NR Uu and V2X carrier |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |
|  | NR Uu Configuration |  |  |  | NR V2X Configuration to Transmit |  |  |
|  | Freq | Downlink | Uplink Configuration |  | Freq | Modulation | PSCCH |
|  |  | Configuration | Modulation | RB allocation (NOTE 1) |  |  | and PSSCH RB allocation (Note 2) |
| 1 | Default |  | DFT-s-OFDM QPSK | Inner Full | Default | CP-OFDM QPSK | Inner_Full |
| 2 | Default | N/A | CP-OFDM 256 QAM | Outer Full | Default | CP-OFDM 256QAM | Outer_Full |
| 3 | Default |  | DFT-s-OFDM QPSK | Inner Full | Default | CP-OFDM 256QAM | Outer_Full |
| 4 | Default |  | CP-OFDM 256 QAM | Outer Full | Default | CP-OFDM QPSK | Inner_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1E-1. |  |  |  |  |  |  |  |

Table 6.2E.2.2.4.1-2: Test Configuration Table for PC3 intra-band concurrent operation (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg, Highest NRB_agg |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest for NR Uu and V2X carrier Highest for NR Uu and V2X carrier |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | NR DL Configuration | NR UL & V2X Configuration |  |
|  |  |  | Modulation | RB allocation (NOTE 1) |
| 1 | Default | N/A | CP-OFDM 16QAM | Inner Full |
| 2 | Default |  | CP-OFDM 16QAM | Outer Full |
| 3 | Default |  | CP-OFDM 64QAM | Inner Full |
| 4 | Default |  | CP-OFDM 64QAM | Outer Full |
| 5 | Default |  | CP-OFDM 256QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table FFS. |  |  |  |  |

Table 6.2E.2.2.4.1-3: Test Configuration Table for PC2 intra-band concurrent operation (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg, Highest NRB_agg |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest for NR Uu and V2X carrier Highest for NR Uu and V2X carrier |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | NR DL Configuration | NR UL & V2X Configuration |  |
|  |  |  | Modulation | RB allocation (NOTE 1) |
| 1 | Default | N/A | CP-OFDM QPSK | Inner Full |
| 1 | Default |  | CP-OFDM 16QAM | Inner Full |
| 2 | Default |  | CP-OFDM 16QAM | Outer Full |
| 3 | Default |  | CP-OFDM 64QAM | Inner Full |
| 4 | Default |  | CP-OFDM 64QAM | Outer Full |
| 5 | Default |  | CP-OFDM 256QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table FFS. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.9.3 for TE diagram and clause A.3.2.7 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] clause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The V2X Reference Measurement Channel and NR UL Reference Measurement Channel are set according to Table 6.2E.2.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameter Connectivity NR, Connected without release on, Test Mode On and Sidelink On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2E.2.2.4.3.

6.2E.2.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2E.2.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	SS sends sidelink scheduling information for each SL-HARQ process via PDCCH DCI format 3_0 for C_RNTI to schedule the Sidelink RMC according to Table 6.2E.2.2.4.1-1. UE is configured to transmit at Pcmax on the sidelink carrier. Since the UE has no payload and no loopback data to send the UE sends MAC padding bits on the Sidelink RMC.

4.	Measure the sum of the mean power of the UE at NR Uu carrier and V2X sidelink carrier in the channel bandwidth according to the test configuration from Table 6.2E.2.2.4.1-1. The period of measurement shall be at least continuous duration of one active sub-frame (1ms) excluding guard symbols.

6.2E.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] clause 4.6.

Table 6.2E.2.2.4.3-1: SL-ResourcePool

| Derivation Path: TS 38.508-1 [5], Table 4.6.6-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-PSCCH-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sl-TimeResourcePSCCH-r16 | As defined in Table 6.1E-2 |  |  |
| sl-FreqResourcePSCCH-r16 | As defined in Table 6.1E-2 |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-SubchannelSize-r16 | As defined in Table 6.1E-2 |  |  |
| sl-StartRB-Subchannel-r16 | As defined in Table 6.1E-2 |  |  |
| sl-NumSubchannel-r16 | As defined in Table 6.1E-2 |  |  |
| sl-PowerControl-r16 SEQUENCE { |  |  |  |
| sl-MaxTransPower-r16 | 24 |  |  |
| sl-Alpha-PSSCH-PSCCH-r16 | Not present |  |  |
| dl-Alpha-PSSCH-PSCCH-r16 | Not present |  |  |
| sl-P0-PSSCH-PSCCH-r16 | Not present |  |  |
| dl-P0-PSSCH-PSCCH-r16 | Not present |  |  |
| dl-Alpha-PSFCH-r16 | Not present |  |  |
| dl-P0-PSFCH-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

6.2E.2.2.5	Test requirement

The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2E.2.2.5-1.

Table 6.2E.2.2.5-1: UE MPR test requirement for inter-band con-current NR V2X operation

|  | PPowerClass,NR (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,c, NR (dBm) | PCMAX_H,c, NR (dB) | PPowerClass,V2X (dBm) | PCMAX_H,f,c,V2X (dBm) | PCMAX_L (dBm) | PCMAX_H (dBm) | T(PCMAX_L) (dB) | T(PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0 | 23 | 23 | 23 | 23 | 23 | 26 | 3 | 2 | 28+TT | 20-TT |
| 2 | 23 | 6.5 | 0 | 16.5 | 23 | 23 | 23 | 16.5 | 26 | 5 | 2 | 28+TT | 11.5-TT |
| 3 | 23 | 0 | 0 | 23 | 23 | 23 | 23 | 23 | 26 | 3 | 2 | 28+TT | 20-TT |
| 4 | 23 | 6.5 | 0 | 16.5 | 23 | 23 | 23 | 16.5 | 26 | 5 | 2 | 28+TT | 11.5-TT |

Table 6.2E.2.2.5-2: Test Tolerance (inter-band con-current NR V2X operation)

|  | f ≤ 3.0GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB |

Table 6.2E.2.2.5-3: Test Tolerance (intra-band con-current NR V2X operation)

|  | f ≤ 3.0GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| Aggregated BW ≤ 100MHz | 0.7 dB | 1.0 dB |

Table 6.2E.2.2.5-4: UE MPR test requirement for PC3 intra-band con-current NR V2X operation

|  | PPowerClass,con-current (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L (dBm) | PCMAX_H (dB) | T(PCMAX_L) (dB) | T(PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 2.5 | 0 | 20.5 | 23 | 2.5 | 2 | 25+TT | 20.5-TT |
| 2 | 23 | 4.5 | 0 | 18.5 | 23 | 4.0 | 2 | 25+TT | 19.0-TT |
| 3 | 23 | 4.5 | 0 | 18.5 | 23 | 4.0 | 2 | 25+TT | 19.0-TT |
| 4 | 23 | 5.0 | 0 | 18.0 | 23 | 4.0 | 2 | 25+TT | 19.0-TT |
| 5 | 23 | 6.0 | 0 | 17.0 | 23 | 5.0 | 2 | 25+TT | 18.0-TT |

Table 6.2E.2.2.5-5: UE MPR test requirement for PC2 intra-band con-current NR V2X operation

|  | PPowerClass,con-current (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L (dBm) | PCMAX_H (dB) | T(PCMAX_L) (dB) | T(PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 3.0 | 0 | 23.0 | 26 | 2.0 | 2 | 28+TT | 21.0-TT |
| 2 | 26 | 4.0 | 0 | 22.0 | 26 | 2.0 | 2 | 28+TT | 20.0-TT |
| 3 | 26 | 5.5 | 0 | 20.5 | 26 | 2.5 | 2 | 28+TT | 18.0-TT |
| 4 | 26 | 5.5 | 0 | 20.5 | 26 | 2.5 | 2 | 28+TT | 18.0-TT |
| 5 | 26 | 6.0 | 0 | 20.0 | 26 | 2.5 | 2 | 28+TT | 17.5-TT |
| 6 | 26 | 7.5 | 0 | 18.5 | 26 | 4.0 | 2 | 28+TT | 14.5-TT |

< End of Changes >