3GPP TSG-RAN5 Meeting #103	R5-242806

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3185 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of 5G ProSe direct discovery messages |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA,CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some 5G ProSe direct discovery message contents are still FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating 5G ProSe direct discovery message contents. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test spec will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7C.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | [x3] will be solved in TS 38.508-1 CR 3188 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

## 4.7C	Default 5G ProSe message and information elements contents

4.7C.2	Contents of 5G ProSe direct discovery messages

–	PROSE PC5 DISCOVERY

Table 4.7C.2-1: PROSE PC5 DISCOVERY message content for open 5G ProSe direct discovery announcement

| Derivation Path: TS 24.554 [58] Table 10.2.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ProSe direct discovery PC5 message type | '01000001'B |  |  |
| UTC-based counter LSB |  |  |  |
| UTC-based counter LSB contents | four least significant bits of the UTC-based counter associated with the discovery transmission opportunity used by the UE |  |  |
| Spare | ‘0000’B |  |  |
| MIC | Computed by UE |  |  |
| ProSe application code |  |  |  |
| PLMN ID |  |  |  |
| Scope | '10' | country-specific scope |  |
| Spare | '0' |  |  |
| E | '1' | MCC and MNC included |  |
| MCC | The MCC part of the PLMN ID of the ProSe Function that assigned the ProSe Application Code (the relevant PLMN will be set in the TC) |  |  |
| MNC | The MNC part of the PLMN ID of the ProSe Function that assigned the ProSe Application Code (the relevant PLMN will be set in the TC) |  |  |
| TemporaryID | 1111111100000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000011111111 |  |  |
| Metadata | Not checked |  |  |

Table 4.7C.2-2: PROSE PC5 DISCOVERY message content restricted 5G ProSe direct discovery announcement

| Derivation Path: TS 24.554 [58] Table 10.2.1.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ProSe direct discovery PC5 message type | '10000001'B |  |  |
| UTC-based counter LSB |  |  |  |
| UTC-based counter LSB contents | four least significant bits of the UTC-based counter associated with the discovery transmission opportunity used by the UE |  |  |
| Spare | ‘0000’B |  |  |
| MIC | Computed by UE |  |  |
| ProSe restricted code |  |  |  |
| PLMN ID | the relevant PLMN will be set in the TC |  |  |
| TemporaryID | 1111111100000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000011111111 |  |  |
| Metadata | Not checked |  |  |

Table 4.7C.2-3: PROSE PC5 DISCOVERY message content for restricted 5G ProSe direct discovery solicitation

| Derivation Path: TS 24.554 [58] Table 10.2.1.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ProSe direct discovery PC5 message type | '10000100'B |  |  |
| UTC-based counter LSB |  |  |  |
| UTC-based counter LSB contents | four least significant bits of the UTC-based counter associated with the discovery transmission opportunity used by the UE |  |  |
| Spare | ‘0000’B |  |  |
| MIC | Computed by UE |  |  |
| ProSe query code |  |  |  |
| PLMN ID | the relevant PLMN will be set in the TC |  |  |
| TemporaryID | 1111111100000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000011111111 |  |  |

Table 4.7C.2-4: PROSE PC5 DISCOVERY message for restricted 5G ProSe direct discovery response

| Derivation Path: TS 24.554 [58] Table 10.2.1.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ProSe direct discovery PC5 message type | '10000010'B |  |  |
| UTC-based counter LSB |  |  |  |
| UTC-based counter LSB contents | four least significant bits of the UTC-based counter associated with the discovery transmission opportunity used by the UE |  |  |
| Spare | ‘0000’B |  |  |
| MIC | Computed by UE |  |  |
| ProSe response code |  |  |  |
| PLMN ID | the relevant PLMN will be set in the TC |  |  |
| TemporaryID | 1111111100000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000011111111 |  |  |
| Metadata | Not present |  |  |

Table 4.7C.2-5: PROSE PC5 DISCOVERY message for group member discovery announcement

| Derivation Path: TS 24.554 [58] Table 10.2.1.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ProSe direct discovery PC5 message type | '10011001'B |  |  |
| UTC-based counter LSB |  |  |  |
| UTC-based counter LSB contents | four least significant bits of the UTC-based counter associated with the discovery transmission opportunity used by the UE |  |  |
| Spare | ‘0000’B |  |  |
| MIC | Computed by UE |  |  |
| Application layer group ID |  |  |  |
| Length of application layer group ID contents | Set to the actual length of 'application layer group ID contents' in bytes |  |  |
| Application layer group ID contents | '01 01'H |  |  |
| Announcer info | ‘000000000001’H |  |  |
| Metadata | Not checked |  |  |

Table 4.7C.2-6: PROSE PC5 DISCOVERY message for group member discovery solicitation

| Derivation Path: TS 24.554 [58] Table 10.2.1.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ProSe direct discovery PC5 message type | '10011110'B |  |  |
| UTC-based counter LSB |  |  |  |
| UTC-based counter LSB contents | four least significant bits of the UTC-based counter associated with the discovery transmission opportunity used by the UE |  |  |
| Spare | ‘0000’B |  |  |
| MIC | Computed by UE |  |  |
| Application layer group ID |  |  |  |
| Length of application layer group ID contents | Set to the actual length of 'application layer group ID contents' in bytes |  |  |
| Application layer group ID contents | '01 01'H |  |  |
| Discoverer info | ‘000000000001’H |  |  |
| Target user info | Not checked |  |  |

Table 4.7C.2-7: PROSE PC5 DISCOVERY message for group member discovery response

| Derivation Path: TS 24.554 [58] Table 10.2.1.7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ProSe direct discovery PC5 message type | '10011010'B |  |  |
| UTC-based counter LSB |  |  |  |
| UTC-based counter LSB contents | four least significant bits of the UTC-based counter associated with the discovery transmission opportunity used by the UE |  |  |
| Spare | ‘0000’B |  |  |
| MIC | Computed by UE |  |  |
| Application layer group ID |  |  |  |
| Length of application layer group ID contents | Set to the actual length of 'application layer group ID contents' in bytes |  |  |
| Application layer group ID contents | '01 01'H |  |  |
| Discoveree info | ‘000000000001’H |  |  |
| Metadata | Not present |  |  |

Table 4.7C.2-8: PROSE PC5 DISCOVERY message for UE-to-network relay discovery announcement

| Derivation Path: TS 24.554 [58] Table 10.2.1.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ProSe direct discovery PC5 message type | '10010001'B |  |  |
| UTC-based counter LSB |  |  |  |
| UTC-based counter LSB contents | four least significant bits of the UTC-based counter associated with the discovery transmission opportunity used by the UE |  |  |
| Spare | ‘0000’B |  |  |
| MIC | Computed by UE |  | L2RelayUE |
|  | Computed as described in TS 33.503 [x3]. |  | L2RemoteUE |
| Announcer info | ‘000000000001’H |  |  |
| Relay service code | ‘000001’H |  |  |
| Status indicator |  |  |  |
| RSI | '0'B | the UE does not have resources available to provide a connectivity service for additional ProSe-enabled UEs |  |
| Spare | '0000000'B |  |  |
| NCGI | Global ID of the serving cell |  |  |
| RRC container |  |  |  |
| Length of RRC container contents | Set to the actual length of 'application layer group ID contents' in bytes |  |  |
| RRC container contents | coded as SL-AccessInfo-L2U2N defined in Table 4.12.1-1 |  |  |

| Condition | Explanation |
| --- | --- |
| L2RelayUE | For L2 U2N Relay UE test cases. |
| L2RemoteUE | For L2 U2N Remote UE test cases. |

Table 4.7C.2-9: PROSE PC5 DISCOVERY message for UE-to-network relay discovery solicitation

| Derivation Path: TS 24.554 [58] Table 10.2.1.9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ProSe direct discovery PC5 message type | '10010110'B |  |  |
| UTC-based counter LSB |  |  |  |
| UTC-based counter LSB contents | four least significant bits of the UTC-based counter associated with the discovery transmission opportunity used by the UE |  |  |
| Spare | ‘0000’B |  |  |
| MIC | Computed by UE |  | L2RemoteUE |
|  | Computed as described in TS 33.503 [x3]. |  | L2RelayUE |
| Discoverer info | ‘000000000001’H |  |  |
| Relay service code | ‘000001’H |  |  |
| Target discoveree info | Not checked |  | L2RemoteUE |
|  | Not present |  | L2RelayUE |

| Condition | Explanation |
| --- | --- |
| L2RelayUE | For L2 U2N Relay UE test cases. |
| L2RemoteUE | For L2 U2N Remote UE test cases. |

Table 4.7C.2-10: PROSE PC5 DISCOVERY message for UE-to-network relay discovery response

| Derivation Path: TS 24.554 [58] Table 10.2.1.10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ProSe direct discovery PC5 message type | '10010010'B |  |  |
| UTC-based counter LSB |  |  |  |
| UTC-based counter LSB contents | four least significant bits of the UTC-based counter associated with the discovery transmission opportunity used by the UE |  |  |
| Spare | ‘0000’B |  |  |
| MIC | Computed by UE |  | L2RelayUE |
|  | Computed as described in TS 33.503 [x3]. |  | L2RemoteUE |
| Discoveree info | ‘000000000001’H |  |  |
| Relay service code | ‘000001’H |  |  |
| Status indicator |  |  |  |
| RSI | '0'B | the UE does not have resources available to provide a connectivity service for additional ProSe-enabled UEs |  |
| Spare | '0000000'B |  |  |
| NCGI | Global ID of the serving cell |  |  |
| RRC container |  |  |  |
| Length of RRC container contents | Set to the actual length of 'application layer group ID contents' in bytes |  |  |
| RRC container contents | coded as SL-AccessInfo-L2U2N defined in Table 4.12.1-1 |  |  |

| Condition | Explanation |
| --- | --- |
| L2RelayUE | For L2 U2N Relay UE test cases. |
| L2RemoteUE | For L2 U2N Remote UE test cases. |

Table 4.7C.2-11: PROSE PC5 DISCOVERY message for relay discovery additional information

| Derivation Path: TS 24.554 [58] Table 10.2.1.11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ProSe direct discovery PC5 message type | '10100001'B |  |  |
| UTC-based counter LSB |  |  |  |
| UTC-based counter LSB contents | four least significant bits of the UTC-based counter associated with the discovery transmission opportunity used by the UE |  |  |
| Spare | ‘0000’B |  |  |
| MIC | Computed by UE |  |  |
| Relay service code | ‘000001’H |  |  |
| Announcer info | ‘000000000001’H |  |  |
| NCGI | Not checked |  |  |
| Relay TAI | Not checked |  |  |

<End of Changed Section>