3GPP TSG-RAN5 Meeting #103	R5-243550

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3187 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Default TLS message and information element contents |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Default TLS message and information element contents haven’t been clarified according to TS 24.109 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the TLS message and information element contents |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases for NR sidelink U2N Relay will be not implementable |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7F(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.508-1 CR 3188 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | [x1] and [x4] will be solved in TS 38.508-1 CR 3188 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

4.7F	Default TLS message and information element contents for communication with 5G ProSe

-	ClientHello

This message is sent by the UE to the SS.

Table 4.7F-1: ClientHello

| Derivation Path: RFC 5246 [x1] |  |  |  |
| --- | --- | --- | --- |
| Field | Value/remark | Comment | Condition |
| Protocolversion | At least TLS 1.2 | TLS protocol versions 1.2 is defined in RFCs 5246 [x1]. |  |
| Random | Not checked |  |  |
| SessionID | Empty |  |  |
| CipherSuite | A list of supported cryptographic options including one or more PSK-based ciphersuites |  |  |
| CompressionMethod | A list including CompressionMethod.null |  |  |
| Extension | Not checked |  |  |

-	ServerHello

This message is sent by the SS to the UE.

Table 4.7F-2: ServerHello

| Derivation Path: RFC 5246 [x1] |  |  |  |
| --- | --- | --- | --- |
| Field | Value/remark | Comment | Condition |
| Protocolversion | TLS 1.2 |  |  |
| Random | A SS generated random structure different from ClientHello |  |  |
| SessionID | Identity of the TLS session generated by SS |  |  |
| CipherSuite | One of the PSK-based ciphersuites listed in ClientHello | Ciphersuite according to PIXIT parameter |  |
| CompressionMethod | CompressionMethod.null |  |  |
| Extension | Empty |  |  |

-	ServerKeyExchange

This message is sent by the SS to the UE.

Table 4.7F-3: ServerKeyExchange

| Derivation Path: RFC 5246 [x1] |  |  |  |
| --- | --- | --- | --- |
| Field | Value/remark | Comment | Condition |
| psk identity hint | “3GPP-bootstrapping” |  |  |

-	ServerHelloDone

This message is sent by the SS to the UE.

Table 4.7F-4: ServerHelloDone

| Derivation Path: RFC 5246 [x1] |  |  |  |
| --- | --- | --- | --- |
| Field | Value/remark | Comment | Condition |
| Note: No fields present. |  |  |  |

-	ClientKeyExchange

This message is sent by the UE to the SS.

Table 4.7F-5: ClientKeyExchange

| Derivation Path: RFC 5246 [x1] |  |  |  |
| --- | --- | --- | --- |
| Field | Value/remark | Comment | Condition |
| psk identity | Concatenation of “3GPP-bootstrapping”, “.” and B-TID |  |  |

-	ChangeCipherSpec

This message is sent by the UE or the SS.

Table 4.7F-6: ChangeCipherSpec

| Derivation Path: RFC 5246 [x1] |  |  |  |
| --- | --- | --- | --- |
| Field | Value/remark | Comment | Condition |
| Enum | 1 |  |  |

-	Finished

This message is sent by the UE or the SS.

Table 4.7F-7: Finished

| Derivation Path: RFC 5246 [x1] |  |  |  |
| --- | --- | --- | --- |
| Field | Value/remark | Comment | Condition |
| opaque | Not checked |  | UE |
| opaque | SS generated verify_data | The verify data contains the hash of the handshake messages. For details, see the RFC for TLS defined in annex E of 3GPP TS 33.310 [x4]. | SS |

| Condition | Explanation |
| --- | --- |
| UE | Sent by the UE. |
| SS | Sent by the SS. |

<End of Changed Section>