3GPP TSG-RAN5 Meeting #103	R5-243857

# Fukuoka City, Japan, 20th – 24th May 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-1 | CR | 0514 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Test Case 14.2.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test Case 14.2.5 is missing from the specification |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test Case 14.2.5 has been added to the specification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test Case 14.2.5 will remain missing from the specification, |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is the final outcome of one revision of R5-242959 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

### 14.2.5	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA

Editor’s note: This test case is incomplete. The following aspect is either missing or TBD:

- Test tolerance is missing

#### 14.2.5.1	Test purpose

The purpose of the test is to verify that the RSTD measurement meets the requirements specified in TS 38.133 [50] Clause 9.9.2 in an environment with AWGN propagation conditions in FR1 in standalone scenario when single positioning frequency layer is configured.

#### 14.2.5.2	Test applicability

This test applies to all types of NR UE release 17 onwards that supports DL-TDOA positioning and supportedDL-PRS-ProcessingSamples-RRC-CONNECTED.

#### 14.2.5.3	Minimum conformance requirements

When physical layer receives last of NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message from LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 9.9.2.3) DL RSTD measurements, defined in TS 38.215 [4], during the measurement period TRSTD,Total defined as:

TRSTD,Total=i=1LTRSTD,i+ L-1*maxTeffect,i

Where ,

i is the index of positioning frequency layer,

L is total number of positioning frequency layers, and

Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i

TRSTD,i is the measurement period for PRS RSTD measurement in positioning frequency layer i as specified below:

TRSTD,i=kmultiTEG,i*CSSFPRS,i*ceil( Kp,PRS,i)*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast,i ,

where:

NRxBeam,i is the UE Rx beam sweeping factor. In FR1, NRxBeam,i = 1;

and in FR2, NRxBeam,i is equal to the value reported by the UE in supportedLowerRxBeamSweepingFactor-FR2 if the UE supports the capability for the band containing positioning frequency layer i, and the LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-TDOA-RequestLocationInformation. NRxBeam,i is equal to 8, otherwise.

CSSFPRS,i is the carrier-specific scaling factor for NR PRS-based positioning measurements in positioning frequency layer i as defined in TS 38.133[50] clause 9.1.5.2.

kmultiTEG,i is the scaling factor for measurement of same PRS resource with multiple Rx TEGs.

kmultiTEG,i=1 if UE is not requested by LMF to measure a PRS resource with multiple Rx TEGs via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] in NR-DL-TDOA-RequestLocationInformation;

otherwise,

kmultiTEG,i =NTEG,i, if UE is not capable of receiving same DL PRS resource simultaneously from multiple Rx TEGs, and

kmultiTEG,i =NTEG,ikTEG,simul,i if UE is capable of receiving the same DL PRS resource simultaneously from multiple Rx TEGs.

where

NTEG,i is the number of Rx TEGs with which UE is requested to measure a PRS resource indicated via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] in NR-DL-TDOA-RequestLocationInformation, and in case ‘n0’ is indicated, NTEG,i is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource as reported in NR-UE-TEG-Capability, and

kTEG,simul,i is the number of Rx TEGs UE can measure simultaneously which is reported via measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul.

Kp,PRS,i is a scaling factor for a positioning frequency layer to be measured within the associated measurement gap pattern, which is defined as Kp,PRS,i = Ntotal / Navailable for UE configured with concurrent measurement gap, and Kp,PRS,i = 1 for UE not configured with concurrent measurement gaps.

-	For a window W of duration max(TPRS,i,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the positioningfrequency layer, and starting at the beginning of any associated gap occasions covering the PRS occasion:

-	Ntotal is the total number of associated gap occasions covering PRS occasions within the window, including both dropped and non-dropped instances of the associated measurement gap within the window, and

-	Navailable is the number of non-dropped associated gap occasions covering PRS occasions within the window W, after further accounting for MG collisions by applying the selected gap collision rule

-	Requirements do not apply if Navailable =0.

NPRS,islot is the maximum number of DL PRS resources in positioning frequency layer i configured in a slot.

Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only the PRS resources unmuted and fully or partially overlapped with MG are considered.

Nsample is the number of PRS RSTD measurement samples, where

-	Nsample= 1 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and meets the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 2 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and does not meet the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 4 otherwise.

Tlast,i is the measurement duration for the last PRS RSTD sample in positioning frequency layer i, including the

sampling time and processing time. If all of the PRS resources to be measured are available in the same MG occasion during Tavailabe, Tlast,i = Ti +MGL. Otherwise, Tlast,i = Ti + Tavailable_PRS,i ,

Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i defined as:

Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i

Where,

Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [49],

Tavailable_PRS,i= LCMTPRS,i,MGRPi, the least common multiple between TPRS,i and MGRPi.

MGRPi is the repetition periodicity of the measurement gap applicable for measurement in the PRS frequency layer i.	TPRS,i is the periodicity of DL PRS resource with muting on positioning frequency layer i.

If more than one PRS periodicities are configured in positioning frequency layer i, the least common multiple of PRS periodicities TperPRS with muting among all DL PRS resource sets in the positioning frequency layer is used to derive TPRS,i, where,

TperPRS with muting=Nmuting*TperPRS, is the PRS periodicity with muting per PRS resource,

TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where

TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

-	Note: For the purpose of calculating TPRS,i, only the PRS resources fully or partially covered by the MG are considered.

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSysmbols in TS 37.355 [49] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [49] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [49].

N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfDL-PRS-ResProcessedPerSlot specified in TS 37.355 [49].

#### 14.2.5.4	Test description

##### 14.2.5.4.1	Initial conditions

The supported test configuration is listed in Table 14.2.5.4-1.

Table 14.2.5.4-1: Test Configuration

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Test Environment: Normal, as defined in TS 38.508-1 [45] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [45] clause 4.3.1.

Channel bandwidth to be tested: are specified in Table 14.2.5.4-1.

1.	Connect the SS and AWGN noise sources to the UE antenna connector or antenna connectors as shown in Annex A, Figure A.12.

2.	The general test parameter settings are set up according to Table 14.2.5.5-1, Table 14.2.5.5-2 and Table 14.2.5.5-3.

3.	Propagation conditions are set according to clause 4.15.2.

4.	Message contents are defined in clause 14.2.5.4.3.

5.	In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All 3 cells are on the same RF channel in FR1.

##### 14.2.5.4.2	Test procedure

The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall not have any timing information of Cell 2 and Cell 3. All three cells transmit PRS during T2. The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources.

The NR-DL-TDOA-ProvideAssistanceData and nr-DL-TDOA-RequestLocationInformation as defined in TS 37.355 [49] clause 6.5.12.1, shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-TDOA assistance data and location information request.

The UE is configured with measurement gap pattern ID # 24 or #0 before T2.

- Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, according to TS 38.508-1 [45] clause 4.5.

2.	The SS shall send a RESET UE POSITIONING STORED INFORMATION message.

3.	Set the parameters according to Table 14.2.5.5-1, Table 14.2.5.5-2 and Table 14.2.5.5-3. Propagation conditions are set according to clause 4.15.2.

4.	T1 starts.

5.	The SS shall transmit an LPP REQUEST CAPABILITIES message.

6.	The UE shall transmit an LPP PROVIDE CAPABILITIES message indicating the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED in the NR-DL-PRS-ProcessingCapability IE and DL-TDOA capabilities supported by the UE in the NR-DL-TDOA-ProvideCapabilities IE.

7.	The SS shall send a LPP PROVIDE ASSISTANCE DATA message, including the NR-DL-TDOA-ProvideAssistanceData IE. The position of neighbour Cell 2 and the position of neighbour Cell 3 are described in 3GPP TS 37.571-5 [20]. If the UE message at step 6 includes the ackRequested IE set to TRUE, then the SS shall send an acknowledgment in the LPP PROVIDE ASSISTANCE DATA message.

8.	The SS shall send a LPP REQUEST LOCATION INFORMATION message, including the reducedDL-PRS-ProcessingSamples-r17 in the NR-DL-TDOA-RequestLocationInformation IE such that the UE receives the message T ms before the start of T2, where T = 50 ms.

9.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.2.5.5-3.

10.	The UE shall transmit a LPP PROVIDE LOCATION INFORMATION message including the NR-DL-TDOA-ProvideLocationInformation IE within the response time (see clause 4.15.3). The UE shall perform and report the RSTD measurements for both Cell 2 and Cell 3 with respect to the reference cell in the DL-TDOA assistance data, Cell 1. If the UE transmits an NR-DL-TDOA-ProvideLocationInformation IE including the nr-RSTD field for both Cell 2 and Cell 3 within the response time then the number of successful tests is increased by one. If the UE fails to report the NR-DL-TDOA-ProvideLocationInformation IE with both the nr-RSTD fields included within the response time then the number of failure tests is increased by one.

11.	If the UE message at step 10 includes the ackRequested IE set to TRUE, the SS shall send a LPP acknowledgement message.

12.	Repeat steps 2-11 in Table 14.2.5.4-1 until the confidence level according to Annex D is achieved.

##### 14.2.5.4.3	Message contents

Table 14.2.5.4.3-1: RESET UE POSITIONING STORED INFORMATION

| Derivation Path: 38.509 [44] clause 6.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE Positioning Technology | 0 0 0 0 0 1 1 1 | DL-TDOA |  |

Table 14.2.5.4.3-2: LPP Request Capabilities

| Information Element | Value/remark |
| --- | --- |
| nr-DL-TDOA-RequestCapabilities-r16 | TRUE |

Table 14.2.5.4.3-3: LPP RequestLocationInformation

| Derivation Path: TS 37.355 [49] clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| initiator | locationServer |  |  |
| transactionNumber | 1 |  |  |
| } |  |  |  |
| endTransaction | FALSE |  |  |
| sequenceNumber | Not present |  |  |
| acknowledgement | Not present |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestLocationInformation-r9 SEQUENCE { |  |  |  |
| commonIEsRequestLocationInformation SEQUENCE { |  |  |  |
| locationInformationType | locationMeasurementsPreferred |  |  |
| triggeredReporting | Not present |  |  |
| periodicalReporting | Not present |  |  |
| additionalInformation | onlyReturnInformationRequested |  |  |
| qos SEQUENCE { |  |  |  |
| horizontalAccuracy | Not present |  |  |
| verticalCoordinateRequest | FALSE |  |  |
| verticalAccuracy | Not present |  |  |
| responseTime SEQUENCE { |  |  |  |
| time | See 14.2.5.5 | Result of the response time calculation rounded up to the next second |  |
| responseTimeEarlyFix-r12 | Not present |  | Rel-12 onwards |
| unit-r15 | Not present |  |  |
| } |  |  |  |
| velocityRequest | FALSE |  |  |
| } |  |  |  |
| environment | Not present |  |  |
| locationCoordinateTypes | Not present |  |  |
| velocityTypes | Not present |  |  |
| } |  |  |  |
| a-gnss-RequestLocationInformation | Not present |  |  |
| otdoa-RequestLocationInformation | Not present |  |  |
| ecid-RequestLocationInformation | Not present |  |  |
| epdu-RequestLocationInformation | Not Present |  |  |
| sensor-RequestLocationInformation-r13 | Not present |  |  |
| tbs-RequestLocationInformation-r13 | Not present |  |  |
| wlan-RequestLocationInformation-r13 | Not present |  |  |
| bt-RequestLocationInformation-r13 | Not present |  |  |
| nr-ECID-RequestLocationInformation-r16 | Not present |  |  |
| nr-Multi-RTT-RequestLocationInformation-r16 | Not present |  |  |
| nr-DL-AoD-RequestLocationInformation-r16 | Not present |  |  |
| nr-DL-TDOA-RequestLocationInformation-r16 SEQUENCE { |  |  |  |
| nr-DL-PRS-RstdMeasurementInfoRequest-r16 | Not present |  |  |
| nr-RequestedMeasurements-r16 | bit 0 = 0 (prsrsrpReq) |  |  |
| nr-AssistanceAvailability-r16 | FALSE |  |  |
| nr-DL-TDOA-ReportConfig-r16 SEQUENCE { |  |  |  |
| maxDL-PRS-RSTD-MeasurementsPerTRPPair-r16 | Not present |  |  |
| timingReportingGranularityFactor-r16 | Not present |  |  |
| measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 | Not present |  |  |
| reducedDL-PRS-ProcessingSamples-r17 | requested |  |  |
| lowerRxBeamSweepingFactor-FR2-r17 | Not present |  |  |
| } |  |  |  |
| additionalPaths-r16 | Not present |  |  |
| nr-UE-RxTEG-Request-r17 | Not present |  |  |
| nr-los-nlos-IndicatorRequest-r17 | Not present |  |  |
| additionalPathsExt-r17 | Not present |  |  |
| additionalPathsDL-PRS-RSRP-Request-r17 | Not present |  |  |
| multiMeasInSameReport-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.5.4.3-4: LPP ProvideAssistanceData

| Derivation Path: TS 37.355 [49] clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| initiator | locationServer |  |  |
| transactionNumber | (0..255) |  |  |
| } |  |  |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | Not present |  |  |
| acknowledgement | Not present |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData-r9 SEQUENCE { |  |  |  |
| commonIEsProvideAssistanceData | Not present |  |  |
| a-gnss-ProvideAssistanceData | Not present |  |  |
| otdoa-ProvideAssistanceData | Not present |  |  |
| epdu-Provide-AssistanceData | Not present |  |  |
| sensor-ProvideAssistanceData-r14 | Not present |  |  |
| tbs-ProvideAssistanceData-r14 | Not present |  |  |
| wlan-ProvideAssistanceData-r14 | Not present |  |  |
| nr-Multi-RTT-ProvideAssistanceData-r16 | Not present |  |  |
| nr-DL-AoD-ProvideAssistanceData-r16 | Not present |  |  |
| nr-DL-TDOA-ProvideAssistanceData-r16 SEQUENCE { |  |  |  |
| nr-DL-PRS-AssistanceData-r16 | As defined in Table 14.2.5.4.3-5 |  |  |
| nr-SelectedDL-PRS-IndexList-r16 | Not present |  |  |
| nr-PositionCalculationAssistance-r16 SEQUENCE { |  |  | Depending on UE capabilities, i.e. support for UE-based DL-TDOA |
| nr-TRP-LocationInfo-r16 | As defined in TS 37.571-5 [20] |  |  |
| nr-DL-PRS-BeamInfo-r16 | Not present |  |  |
| nr-RTD-Info-r16 | Not present |  |  |
| nr-TRP-BeamAntennaInfo-r17 | Not present |  |  |
| nr-DL-PRS-Expected-LOS-NLOS-Assistance-r17 | Not present |  |  |
| nr-DL-PRS-TRP-TEG-Info-r17 | Not present |  |  |
| } |  |  |  |
| nr-DL-TDOA-Error-r16 | Not present |  |  |
| nr-On-Demand-DL-PRS-Configurations-r17 | Not present |  |  |
| nr-On-Demand-DL-PRS-Configurations-Selected-IndexList-r17 | Not present |  |  |
| assistanceDataValidityArea-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.5.4.3-5: NR-DL-PRS-AssistanceData

| Derivation Path: TS 37.355 [49] clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ReferenceInfo-r16 SEQUENCE { |  |  |  |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-DL-PRS-ResourceID-List-r16 | Not present |  |  |
| nr-DL-PRS-ResourceSetID-r16 | Not present |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataList-r16 SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-DL-PRS-AssistanceDataPerFreq-r16 { | 1 entry |  |  |
| NR-DL-PRS-AssistanceDataPerFreq-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-PositioningFrequencyLayer-r16 SEQUENCE { |  |  |  |
| dl-PRS-SubcarrierSpacing-r16 | kHz15 |  | Config 1 and Config 2 |
|  | kHz30 |  | Config 3 |
| dl-PRS-ResourceBandwidth-r16 | 21 | 104 PRBs | Config 1 and Config 2 |
|  | 28 | 132 PRBs | Config 3 |
| dl-PRS-StartPRB-r16 | 0 |  |  |
| dl-PRS-PointA-r16 | absoluteFrequencyPointA as defined for the DL frequency of the Cell 1 |  |  |
| dl-PRS-CombSizeN-r16 | n4 |  |  |
| dl-PRS-CyclicPrefix-r16 | normal |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF NR-DL-PRS-AssistanceDataPerTRP-r16{ | 3 entries |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[1] SEQUENCE { |  | entry 1 | Cell 1 |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-PhysCellID-r16 | Cell 1 |  |  |
| nr-CellGlobalID-r16 | Cell 1 |  |  |
| nr-ARFCN-r16 | Cell 1 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 0 |  |  |
| nr-DL-PRS-Info-r16 | As specified in Table 14.2.1.4.3-6 |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[2] SEQUENCE { |  | entry 2 | Cell 2 |
| dl-PRS-ID-r16 | 1 |  |  |
| nr-PhysCellID-r16 | Cell 2 |  |  |
| nr-CellGlobalID-r16 | Cell 2 |  |  |
| nr-ARFCN-r16 | Cell 2 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 23 | About 3 ms |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 39 | About 5 ms |  |
| nr-DL-PRS-Info-r16 | As specified in Table 14.2.1.4.3-6 |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[3] SEQUENCE { |  | entry 3 | Cell 3 |
| dl-PRS-ID-r16 | 2 |  |  |
| nr-PhysCellID-r16 | Cell 3 |  |  |
| nr-CellGlobalID-r16 SEQUENCE { |  |  |  |
| mcc-r15 | Cell 3 |  |  |
| mnc-r15 | Cell 3 |  |  |
| nr-cellidentity-r15 | Cell 3 |  |  |
| } |  |  |  |
| nr-ARFCN-r16 | Cell 3 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 23 | About 3 s |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 38 | About 5 s |  |
| nr-DL-PRS-Info-r16 | As specified in Table 14.2.1.4.3-6 |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[4-16] | As specificied in TS 37.571-5 [20] |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.5.4.3-6: NR-DL-PRS-Info

| Derivation Path: TS 37.355 [49] clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-Info-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ResourceSetList-r16 SEQUENCE (SIZE (1..nrMaxSetsPerTrp-r16)) OF NR-DL-PRS-ResourceSet-r16 { | 1 entry |  |  |
| NR-DL-PRS-ResourceSet-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceSetID-r16 | 0 |  |  |
| dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16 CHOICE { |  |  |  |
| scs15-r16 CHOICE { |  |  | Config 1 and Config 2 |
| n160-r16 | 10 |  |  |
| } |  |  |  |
| scs30-r16 CHOICE { |  |  | Config 3 |
| n160-r16 | 20 |  |  |
| } |  |  |  |
| dl-PRS-ResourceRepetitionFactor-r16 | Not present |  |  |
| dl-PRS-ResourceTimeGap-r16 | s1 |  |  |
| dl-PRS-NumSymbols-r16 | n4 |  |  |
| dl-PRS-MutingOption1-r16 SEQUENCE { |  |  |  |
| dl-prs-MutingBitRepetitionFactor-r16 | Not present |  |  |
| nr-option1-muting-r16 CHOICE { |  |  |  |
| po2-r16 | 10 |  | Cell 1 and Cell 3 |
|  | 01 |  | Cell 2 |
| } |  |  |  |
| } |  |  |  |
| dl-PRS-MutingOption2-r16 | Not present |  |  |
| dl-PRS-ResourcePower-r16 | 27 |  |  |
| dl-PRS-ResourceList-r16 SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF NR-DL-PRS-Resource-r16 { | 1 entry |  |  |
| NR-DL-PRS-Resource-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceID-r16 | 0 |  |  |
| dl-PRS-SequenceID-r16 | 0 |  |  |
| dl-PRS-CombSizeN-AndReOffset-r16 CHOICE { |  |  |  |
| n4-r16 | 0 |  | Cell 1 and Cell 2 |
|  | 1 |  | Cell 3 |
| } |  |  |  |
| dl-PRS-ResourceSlotOffset-r16 | 4 |  |  |
| dl-PRS-ResourceSymbolOffset-r16 | 0 |  |  |
| dl-PRS-QCL-Info-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.5.4.3-7: LPP ProvideLocation Information

| Derivation Path: TS 37.355 [49] clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| initiator | locationServer |  |  |
| transactionNumber | 1 |  |  |
| } |  |  |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | (0..255) |  |  |
| acknowledgement |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideLocationInformation-r9 SEQUENCE { |  |  |  |
| commonIEsProvideLocationInformation | Not present. |  |  |
| a-gnss-ProvideLocationInformation | Not present |  |  |
| otdoa-ProvideLocationInformation | Not present |  |  |
| ecid-ProvideLocationInformation | Not present |  |  |
| epdu-ProvideLocationInformation | Not present |  |  |
| sensor-ProvideLocationInformation-r13 | Not present |  |  |
| tbs-ProvideLocationInformation-r13 | Not present |  |  |
| wlan-ProvideLocationInformation-r13 | Not present |  |  |
| bt-ProvideLocationInformation-r13 | Not present |  |  |
| nr-ECID-ProvideLocationInformation-r16 | Not present |  |  |
| nr-Multi-RTT-ProvideLocationInformation-r16 | Not present |  |  |
| nr-DL-AoD-ProvideLocationInformation-r16 | Not present |  |  |
| nr-DL-TDOA-ProvideLocationInformation-r16 SEQUENCE { |  |  |  |
| nr-DL-TDOA-SignalMeasurementInformation-r16 SEQUENCE { |  |  |  |
| dl-PRS-ReferenceInfo-r16 |  |  |  |
| nr-DL-TDOA-MeasList-r16 SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-DL-TDOA-MeasElement-r16 { | 3 entries |  |  |
| NR-DL-TDOA-MeasElement-r16[1] SEQUENCE { |  | entry 1 |  |
| dl-PRS-ID-r16 |  |  |  |
| nr-PhysCellID-r16 | Cell 1 |  |  |
| nr-CellGlobalID-r16 |  |  |  |
| nr-ARFCN-r16 |  |  |  |
| nr-DL-PRS-ResourceID-r16 |  |  |  |
| nr-DL-PRS-ResourceSetID-r16 |  |  |  |
| nr-TimeStamp-r16 |  |  |  |
| nr-RSTD-r16 | Present |  |  |
| nr-AdditionalPathList-r16 |  |  |  |
| nr-TimingQuality-r16 |  |  |  |
| nr-DL-PRS-RSRP-Result-r16 |  |  |  |
| nr-DL-TDOA-AdditionalMeasurements-r16 |  |  |  |
| nr-UE-Rx-TEG-ID-r17 |  |  |  |
| nr-DL-PRS-FirstPathRSRP-Result-r17 |  |  |  |
| nr-los-nlos-Indicator-r17 |  |  |  |
| nr-AdditionalPathListExt-r17 |  |  |  |
| nr-DL-TDOA-AdditionalMeasurementsExt-r17 |  |  |  |
| } |  |  |  |
| NR-DL-TDOA-MeasElement-r16[2] SEQUENCE { |  | entry 2 |  |
| dl-PRS-ID-r16 |  |  |  |
| nr-PhysCellID-r16 | Cell 2 |  |  |
| nr-CellGlobalID-r16 |  |  |  |
| nr-ARFCN-r16 |  |  |  |
| nr-DL-PRS-ResourceID-r16 |  |  |  |
| nr-DL-PRS-ResourceSetID-r16 |  |  |  |
| nr-TimeStamp-r16 |  |  |  |
| nr-RSTD-r16 | Present |  |  |
| nr-AdditionalPathList-r16 |  |  |  |
| nr-TimingQuality-r16 |  |  |  |
| nr-DL-PRS-RSRP-Result-r16 |  |  |  |
| nr-DL-TDOA-AdditionalMeasurements-r16 |  |  |  |
| nr-UE-Rx-TEG-ID-r17 |  |  |  |
| nr-DL-PRS-FirstPathRSRP-Result-r17 |  |  |  |
| nr-los-nlos-Indicator-r17 |  |  |  |
| nr-AdditionalPathListExt-r17 |  |  |  |
| nr-DL-TDOA-AdditionalMeasurementsExt-r17 |  |  |  |
| } |  |  |  |
| NR-DL-TDOA-MeasElement-r16[3] SEQUENCE { |  | entry 3 |  |
| dl-PRS-ID-r16 |  |  |  |
| nr-PhysCellID-r16 | Cell 3 |  |  |
| nr-CellGlobalID-r16 |  |  |  |
| nr-ARFCN-r16 |  |  |  |
| nr-DL-PRS-ResourceID-r16 |  |  |  |
| nr-DL-PRS-ResourceSetID-r16 |  |  |  |
| nr-TimeStamp-r16 |  |  |  |
| nr-RSTD-r16 | Present |  |  |
| nr-AdditionalPathList-r16 |  |  |  |
| nr-TimingQuality-r16 |  |  |  |
| nr-DL-PRS-RSRP-Result-r16 |  |  |  |
| nr-DL-TDOA-AdditionalMeasurements-r16 |  |  |  |
| nr-UE-Rx-TEG-ID-r17 |  |  |  |
| nr-DL-PRS-FirstPathRSRP-Result-r17 |  |  |  |
| nr-los-nlos-Indicator-r17 |  |  |  |
| nr-AdditionalPathListExt-r17 |  |  |  |
| nr-DL-TDOA-AdditionalMeasurementsExt-r17 |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nr-UE-RxTEG-TimingErrorMargin-r17 |  |  |  |
| } |  |  |  |
| nr-dl-tdoa-LocationInformation-r16 |  |  |  |
| nr-DL-TDOA-Error-r16 |  |  |  |
| nr-DL-TDOA-SignalMeasurementInstances-r17 |  |  |  |
| nr-DL-TDOA-LocationInformationInstances-r17 |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 14.2.5.5	Test requirement

Table 14.2.5.5-1, Table 14.2.5.5-2 and Table 14.2.5.5-3 define the primary level settings including test tolerances for the test.

Table 14.2.5.5-1: General test parameters for RSTD measurement reporting delay

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  |  | Cell 1 | Reference cell is the cell in the DL-TDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 38.215 [4] and TS 37.355 [34]. The reference cell is the PCell in this test case. |
| Neighbor cells |  |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at the first and second places in the neighbour cell list in the DL-TDOA assistance data. |
| SSB configuration | Config 1 |  | SSB.1 FR1 |  |
|  | Config 2 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.2 FR1 |  |
| SMTC configuration | Config 1 |  | SMTC.2 |  |
|  | Config 2 |  | SMTC.1 |  |
|  | Config 3 |  | SMTC.1 |  |
| PDSCH RMC configuration | Config 1 |  | SR.1.1 FDD |  |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration | Config 1 |  | CR.1.1 FDD | As specified in clause A.3.1.2.1 |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration | Config 1 |  | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Initial BWP configuration | Config 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration | Config 1,2,3 |  | DLBWP.1.1 |  |
| Active UL BWP configuration | Config 1,2,3 |  | ULBWP.1.1 |  |
| PRS Configuration | Config 1 |  | PRS.1.2 FR1 | As specified in clause A.3.31 |
|  | Config 2 |  | PRS.1.2 FR1 |  |
|  | Config 3 |  | PRS.2.2 FR1 |  |
| Physical cell ID PCI |  |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| Measurement gap |  |  | GP#24 or GP#0 | GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured |
| Radio frame receive time offset between the cells at the UE antenna connector |  | s | Cell 2 to Cell 1: 0 Cell 3 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Expected RSTD |  | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355 [34] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells |  | s | 5 | The corresponding parameter in the DL-TDOA assistance ta specified in TS 37.355 [34] is the expectedRSTD-Uncertainty index |
| Number of cells provided in DL-TDOA assistance data |  |  | 4 | Including the reference cell |
| PRS muting info |  |  | Cell 1: ‘10’ Cell 2: ‘01’ Cell 3: ‘10’ | Correponds to prs-MutingInfo defined in TS 37.355 [34] |
| PRS resource RE offset |  |  | Cell 1: 0 Cell 2: 0 Cell 3: 1 | Cell 1 and Cell 3 are configured with different resource offsets |
| T1 |  | s | 3 | The length of the time interval from the beginning of each test |
| T2 |  | s | 1.28 | The length of the time interval that follows immediately after time interval T1 |

Table 14.2.5.5-2: Cell-specific test parameters for RSTD measurement reporting delay during T1

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Positiong frequency layer |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | N/A | N/A |
| Note 3 | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 | dBm/SCS | -98 |  |  |
|  | Config 3 | dBm/SCS | -95 |  |  |
| PRS |  | dB | -Infinity | -Infinity | -Infinity |
| SSB |  | dB | 10 | -Infinity | -Infinity |
| Io Note 4 | Config 1 | dBm/ 19.08MHz | -56.54+TT | -56.54+TT | -56.54+TT |
|  | Config 2 | dBm/ 19.08MHz | -56.54+TT | -56.54+TT | -56.54+TT |
|  | Config 3 | dBm/ 47.88MHz | -52.56+TT | -52.56+TT | -52.56+TT |
| SSB RP Note4 | Config 1 | dBm/SCS | -88+TT | -Infinity | -Infinity |
|  | Config 2 | dBm/SCS | -88+TT | -Infinity | -Infinity |
|  | Config 3 | dBm/SCS | -88+TT | -Infinity | -Infinity |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

Table 14.2.5.5-2: Cell-specific test parameters for RSTD measurement reporting delay during T2

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T2 | T2 | T2 |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | OP.1 | OP.1 |
| PRACH configuration |  |  | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 |
| Note 3 | Config 1 | dBm/SCS | -98 | -98 | -98 |
|  | Config 2 | dBm/SCS | -98 | -98 | -98 |
|  | Config 3 | dBm/SCS | -95 | -95 | -95 |
| PRS | Config 1 | dB | 1 | -1.2 | -1.2 |
|  | Config 2 | dB | 1 | -1.2 | -1.2 |
|  | Config 3 | dB | 1 | -1.2 | -1.2 |
| Io Note 4 | Config 1 | dBm/ 19.08MHz | -61.18+TT | -61.18+TT | -61.18+TT |
|  | Config 2 | dBm/ 19.08MHz | -61.18+TT | -61.18+TT | -61.18+TT |
|  | Config 3 | dBm/ 47.88MHz | -57.20+TT | -57.20+TT | -57.20+TT |
| PRS |  | dB | -3 | -6 | -6 |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |  |

The RSTD measurement time fulfils the requirements specified in clause 14.2.5.3.

The test tolerances are FFS

< End of changes >