3GPP TSG-RAN5 Meeting #103	R5-242701

Fukuoka, Japan, May 20 – 24, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5509 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Inter-band DL CA and CA_30A-48A band for A-MPR & Spurious emission test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sporton, Bureau Veritas ADT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Inter-band DL CA and UL CA_30A-48A is missing for A-MPR & Spurious emission test cases in TS36.521-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of Inter-band DL CA and CA_30A-48A for A-MPR & Spurious emission test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inter-band DL CA and CA_30A-48A will be still incompleted |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4A.2, 6.6.2.2A.2, 6.6.3.1A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 36.905 CR 0273 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

6.2.4A.2	Additional Maximum Power Reduction (A-MPR) for CA (inter-band DL CA and UL CA)

Editor's note: For a transition period until RAN5#73, this test case in version 13.1.0 of 36.521-1 shall be used. This is to ensure no test coverage is lost before the UL 64QAM test case becomes available. Based on analysis for CA_3A-19A in TR 36.905 [19] no requirements in additional spurious emissions are required so A-MPR requirements are not tested for this CA configuration in any test case.

6.2.4A.2.1	Test purpose

Additional ACLR, spectrum emission and spurious emission requirements for carrier aggregation can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the CA Power Class as specified in Table 6.2.2A.0-1.

6.2.4A.2.2	Test applicability

The requirements of this test apply in test case 6.6.2.2A.2 Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA) for network signalled value NS_01/NS_03, NS_03/NS_03, NS_03/NS_06, NS_03/NS_07, NS_03/NS_01, NS_01/NS_06, NS_01/NS_21, NS_03/NS_21, NS_06/NS_21, NS_03/NS_28, NS_03/NS_29, NS_03/NS_30, NS_03/NS_31, NS_21/NS_27 to all types of E-UTRA UE release 11 and forward but do not support UL 64QAM or UL 256QAM.

The requirements of this test apply in test case 6.6.3.3A.2 Additional spurious emissions for CA (inter-band DL CA and UL CA) for network signalled values NS_05/NS_01, NS_03/NS_07, NS_01/NS_12 and NS_01/NS15, NS_01/NS_21, NS_03/NS_21, NS_06/NS_21, NS_03/NS_28, NS_03/NS_29, NS_03/NS_30, NS_03/NS_31, NS_21/NS_27 to all types of E-UTRA UE release 11 and forward but do not support UL 64QAM or UL 256QAM.

NOTE:	As a result TC 6.2.4A.2 has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

6.2.4A.2.3	Minimum conformance requirements

If for inter-band carrier aggregation the UE is configured for transmissions on a single serving cell, then subclauses 6.2.3 and 6.2 4 apply with the Network Signalling value indicated by the field additionalSpectrumEmission.

For inter-band carrier aggregation with the UE configured for transmissions on two serving cells the maximum output power reduction specified in Table 6.2.4-1 is allowed for each serving cell of the applicable uplink CA configuration according to the Network Signalling value indicated by the field additionalSprectrumEmission for the PCC and the CA network signalling value indicated by the field additionalSpectrumEmissionSCell-r10 for the SCC. The value of additionalSpectrumEmissionSCell-r10 is equal to that of additionalSprectrumEmission configured on the SCC. MPR as specified in subclause 6.2.3A is allowed in addition.

For PUCCH and SRS transmissions, the allowed A-MPR is according to that specified for PUSCH QPSK modulation for the corresponding transmission bandwidth. The normative reference for this requirement is in TS 36.101 [2] clause 6.2.4A.

6.2.4A.2.4	Test description

6.2.4A.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. For the UE maximum output power modified by A-MPR specified in table 6.2.4A.2.4.1-1, the power limits specified in subclause 6.2.5A.2.3 apply.

Table 6.2.4A.2.4.1-1: Additional Maximum Power Reduction (A-MPR) for inter-band CA

| Uplink CA Configuration | NS-value per CC (in same order as UL CA Configuration column) |  | Applicable emissions requirements (in same order as UL CA Configuration column) |  |
| --- | --- | --- | --- | --- |
| CA_1A-3A CA_1A-5A CA_1A-7A CA_1A-8A CA_1A-18A CA_1A-26A CA_1A-42A | NS_05 | NS_01 | Table 6.6.3.3.5.2-1 (Note 5) | N/A |
| CA_1A-19A | NS_05 | NS_08 | Table 6.6.3.3.5.2-1 (Note 5) | Table 6.6.3.3.5.4-1 |
| CA_1A-21A | NS_05 | NS_09 | Table 6.6.3.3.5.2-1 (Note 5) | Table 6.6.3.3.5.5-1 (Note 5) |
| CA_2A-4A, CA_2A-66A | NS_03 | NS_03 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.1-1 |
| CA_2A-12A, CA_2A-13A, CA_4A-12A, CA_4A-13A, CA_4A-17A | NS_03 | NS_06 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.3-1 |
| CA_2A-13A, CA_4A-13A | NS_03 | NS_07 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.3-1 Table 6.6.3.3.3.3-1 |
| CA_2A-30A | NS_03 | NS_21 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.1-1 Table 6.6.3.3.5.16-1 (Note 8) |
| CA_2A-46A | NS_03 | NS_28 | Table 6.6.2.2.3.1-1 | Table 6.6.3.3.3.22-1 |
| CA_2A-46A | NS_03 | NS_29 | Table 6.6.2.2.3.1-1 | Table 6.6.3.3.3.23-1 |
| CA_2A-46A | NS_03 | NS_30 | Table 6.6.2.2.3.1-1 | Table 6.6.3.3.3.24-1 Table 6.6.3.3.3.24-2 Table 6.6.3.3.3.24-3 |
| CA_2A-46A | NS_03 | NS_31 | Table 6.6.2.2.3.1-1 | Table 6.6.3.3.3.25-1 Table 6.6.3.3.3.25-2 Table 6.6.3.3.3.25-3 Table 6.6.3.3.3.25-4 |
| CA_3A-5A CA_3A-7A CA_3A-8A CA_3A-18A CA_5A-7A CA_11A-18A CA_39A-41A | NS_01 | NS_01 | N/A | N/A |
| CA_3A-19A | NS_01 | NS_08 | N/A | Table 6.6.3.3.5.4-1 |
| CA_3A-20A CA_7A-20A | NS_01 | NS_10 | N/A | N/A (Note 6) |
| CA_3A-26A | NS_01 | NS_12 | N/A | Table 6.6.3.3.3.7-1 (Note 5) |
| CA_3A-26A | NS_01 | NS_13 | N/A | Table 6.6.3.3.3.8-1 (Note 5) |
| CA_3A-26A | NS_01 | NS_14 | N/A | Table 6.6.3.3.3.9-1 (Note 5) |
| CA_3A-26A | NS_01 | NS_15 | N/A | Table 6.6.3.3.3.10-1 (Note 5) |
| CA_3A-41A | NS_01 | NS_04 | N/A | Table 6.6.2.2.3.2-1 Table 6.6.3.3.3.1-1 |
| CA_2A-5A CA_2A-7A CA_4A-5A CA_4A-7A | NS_03 | NS_01 | Table 6.6.2.2.3.1-1 | N/A |
| CA_5A-66ANote 7 | NS_01 | NS_03 | N/A | Table 6.6.2.2.3.1-1 |
| CA_5A-12A CA_5A-17A | NS_01 | NS_06 | N/A | Table 6.6.2.2.3.3-1 |
| CA_5A-30A | NS_01 | NS_21 | N/A | Table 6.6.2.2.3.1-1 Table 6.6.3.3.5.16-1 (Note 8) |
| 3A-28A CA_7A-28A CA_18A-28A | NS_01 | NS_17 | N/A | Table 6.6.3.3.5.12-1 (Note 5) |
| CA_3A-28A CA_7A-28A | NS_01 | NS_18 | N/A | Table 6.6.3.3.5.13-1 (Note 5) |
| CA_12A-30A | NS_06 | NS_21 | Table 6.6.2.2.3.3-1 | Table 6.6.2.2.3.3-1 Table 6.6.3.3.5.16-1 (Note 8) |
| CA_12A-66ANote 7 | NS_06 | NS_03 | Table 6.6.2.2.3.3-1 | Table 6.6.2.2.3.1-1 |
| CA_19A-21A | NS_08 | NS_09 | Table 6.6.3.3.5.4-1 (Note 5) | Table 6.6.3.3.5.5-1 (Note 5) |
| CA_1A-28A | NS_05 | NS_17 | Table 6.6.3.3.5.2-1 | Table 6.6.3.3.5.12-1 (Note 5) |
| CA_30A-48A | NS_21 | NS_27 | Table 6.6.2.2.3.1-1 Table 6.6.3.3.5.16-1 | Table 6.6.2.2.3.4-1 Table 6.6.3.3.5.21-1 |
| Note 1: For CCs with NS_01, there are no additional requirements defined in 36.101. Only CCs with NS-value different than NS_01 needs to be tested. Note 2: A-MPR values for NS-values in this table are the same as in section 6.2.4.3. Note 3: Selection of CA configurations and NS-values to test is derived in TR 36.905 [19] Note 4: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column Note 5: No test required since only A-Spur requirements apply, and the frequency range to test is unaffected by intermodulation products as described in section 4.3 in TR 36.905 [19] and refer to Table 4.1.3-1 in TR 36.905. Note 6: No test required since no A_SEM and/or A-Spur requirements apply Note 7: As an exception to note 4, PCC is mapped to the second band of the combination as listed in Uplink CA Configuration column. Note 8: Only A_SEM requirements apply, as the frequency range to test for A-Spur requirements is unaffected by intermodulation products as described in section 4.3 in TR 36.905 [19] and refer to Table 4.1.3-1 in TR 36.905. |  |  |  |  |

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-2. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.2.4A.2.4.1-2 to 6.2.4A.1.4.1-17. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.4A.2.4.1-2: Test Configuration Table (network signalled value NS_01/NS_06)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_01) | SCC (NS_06) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 15 | 15 | N/A for this test | QPSK | QPSK | 6 | 6 |
| 2 | 15 | 15 |  | 16QAM | 16QAM | 25 | 9 |
| 3 | 15 | 15 |  | QPSK | 16QAM | 6 | 9 |
| 4 | 50 | 50 |  | QPSK | QPSK | 12 | 12 |
| 5 | 50 | 50 |  | 16QAM | 16QAM | 50 | 15 |
| 6 | 50 | 50 |  | QPSK | 16QAM | 12 | 15 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-3: Test Configuration Table (network signalled value NS_01/NS_12)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for PCC SCC: (Note 3) For 5 MHz Channel Bandwidth: UL 816.7 MHz (NUL = 26717) For 10 MHz Channel Bandwidth: UL 819.2 MHz (NUL = 26742) For 15 MHz Channel Bandwidth: UL 821.7 MHz (NUL = 26767) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_01) | SCC (NS_12) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations (LCRB @ RBstart)(Note 4) |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 8 | 8@7 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 10 | 8 |
| 3 | 25 | 25 |  | QPSK | 16QAM | 8 | 8 |
| 4 | 50 | 50 |  | QPSK | QPSK | 12 | 12@16 |
| 5 | 50 | 50 |  | 16QAM | 16QAM | 15 | 18 |
| 6 | 50 | 50 |  | QPSK | 16QAM | 12 | 18 |
| 7 | 100 | 75 |  | QPSK | QPSK | 15 | 16@31 |
| 8 | 100 | 75 |  | 16QAM | 16QAM | 20 | 30 |
| 9 | 100 | 75 |  | QPSK | 16QAM | 15 | 30 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. only the combination with the highest NRB_PCC is tested. Note 3: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.2.4A.2.4.1-1 Note 4: Test point with max A-MPR in B26 selected in region A with largest allocation giving max A-MPR at RBstart0; test point with min A-MPR in B26 selected in region A with same allocation but with shifted RBstart to give A-MPR=0; |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-4: Test Configuration Table (network signalled value NS_01/NS_15)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | High range for both PCC and SCC(Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_01) | SCC (NS_15) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations (LCRB @ RBstart)(Note 4) |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 8 | 6@14 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 10 | 10@15 |
| 3 | 25 | 25 |  | QPSK | 16QAM | 8 | 10@15 |
| 4 | 50 | 50 |  | QPSK | QPSK | 12 | 12@25 |
| 5 | 50 | 50 |  | 16QAM | 16QAM | 15 | 15@35 |
| 6 | 50 | 50 |  | QPSK | 16QAM | 12 | 15@35 |
| 7 | 100 | 75 |  | QPSK | QPSK | 18 | 18@19 |
| 8 | 100 | 75 |  | 16QAM | 16QAM | 20 | 20@55 |
| 9 | 100 | 75 |  | QPSK | 16QAM | 18 | 20@55 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. only the combination with the highest NRB_PCC is tested. Note 3: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.2.4A.2.4.1-1. Note 4: Test point with max A-MPR in B26 selected in region C with allocation giving max MPR+A-MPR+deltaTc at max RBend; Test point with min A-MPR in B26 selected in region B with max allocation and RBend giving A-MPR=0. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-5: Test Configuration Table (network signalled value NS_01/NS_17)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_01) | SCC (NS_17) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 6 | 6 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 3 | 25 | 25 |  | 16QAM | QPSK | 25 | 6 |
| 4 | 25 | 50 |  | QPSK | QPSK | 6 | 12 |
| 5 | 25 | 50 |  | 16QAM | 16QAM | 25 | 25 |
| 6 | 25 | 50 |  | 16QAM | QPSK | 25 | 12 |
| 7 | 50 | 50 |  | QPSK | QPSK | 12 | 15 |
| 8 | 50 | 50 |  | 16QAM | 16QAM | 50 | 25 |
| 9 | 50 | 50 |  | 16QAM | QPSK | 50 | 12 |
| 10 | 75 | 50 |  | QPSK | QPSK | 16 | 12 |
| 11 | 75 | 50 |  | 16QAM | 16QAM | 50 | 25 |
| 12 | 75 | 50 |  | 16QAM | QPSK | 50 | 12 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-6: Test Configuration Table (network signalled value NS_01/NS_21)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_01) | SCC (NS_21) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 8 | 8 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 3 | 25 | 25 |  | QPSK | 16QAM | 8 | 25 |
| 4 (Note 4) | 50 | 50 |  | QPSK | QPSK | 12 | 12@7 |
| 5 (Note 5) | 50 | 50 |  | QPSK | QPSK | 12 | 12@31 |
| 6 | 50 | 50 |  | 16QAM | 16QAM | 50 | 1 |
| 7 | 50 | 50 |  | QPSK | 16QAM | 12 | 1 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.2.4A.2.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBend-LCRB+1. Note 4: Applicable only to Low range frequency testing. Note 5: Applicable only to High range frequency testing. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-7: Test Configuration Table (network signalled value NS_03/NS_01 or network signalled value NS_01/NS_03)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and high range for PCC Mid range for SCC(Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | NS_03 CC | NS_01 CC | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 6 | 8 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 3 | 25 | 25 |  | 16QAM | QPSK | 25 | 8 |
| 4 | 50 | 50 |  | QPSK | QPSK | 6 | 12 |
| 5 | 50 | 50 |  | 16QAM | 16QAM | 50 | 50 |
| 6 | 50 | 50 |  | 16QAM | QPSK | 50 | 12 |
| 7 | 50 | 100 |  | QPSK | QPSK | 6 | 18 |
| 8 | 50 | 100 |  | 16QAM | 16QAM | 50 | 100 |
| 9 | 50 | 100 |  | 16QAM | QPSK | 50 | 18 |
| 10 | 100 | 50 |  | QPSK | QPSK | 10 | 12 |
| 11 | 100 | 50 |  | 16QAM | 16QAM | 100 | 50 |
| 12 | 100 | 50 |  | 16QAM | QPSK | 100 | 12 |
| 13 | 100 | 100 |  | QPSK | QPSK | 10 | 18 |
| 14 | 100 | 100 |  | 16QAM | 16QAM | 100 | 100 |
| 15 | 100 | 100 |  | 16QAM | QPSK | 100 | 18 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBend-LCRB+1. For Mid range place test point with RBstart=0 and RBstart=RBend-LCRB+1. Note 4: Mapping of NS-value per CC and PCC/SCC selection are specified in Table 6.2.4A.2.4.1-1. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-8: Test Configuration Table (network signalled value NS_03/NS_03)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg (Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_03) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 15 | 25 | N/A for this test | QPSK | QPSK | 4 | 6 |
| 2 | 15 | 25 |  | 16QAM | 16QAM | 15 | 25 |
| 3 | 15 | 25 |  | 16QAM | QPSK | 15 | 6 |
| 4 | 25 | 25 |  | QPSK | QPSK | 6 | 6 |
| 5 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 6 | 25 | 25 |  | 16QAM | QPSK | 25 | 6 |
| 7 | 50 | 50 |  | QPSK | QPSK | 6 | 6 |
| 8 | 50 | 50 |  | 16QAM | 16QAM | 50 | 50 |
| 9 | 50 | 50 |  | 16QAM | QPSK | 50 | 6 |
| 10 | 100 | 100 |  | QPSK | QPSK | 10 | 10 |
| 11 | 100 | 100 |  | 16QAM | 16QAM | 100 | 100 |
| 12 | 100 | 100 |  | 16QAM | QPSK | 100 | 10 |
| 13 | 6 | 25 |  | QPSK | QPSK | 5 | 6 |
| 14 | 6 | 25 |  | 16QAM | 16QAM | 6 | 25 |
| 15 | 6 | 25 |  | 16QAM | QPSK | 6 | 6 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBend-LCRB+1. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-9: Test Configuration Table (network signalled value NS_03/NS_06)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_06) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 15 | 25 | N/A for this test | QPSK | QPSK | 4 | 8 |
| 2 | 15 | 25 |  | 16QAM | 16QAM | 15 | 25 |
| 3 | 15 | 25 |  | 16QAM | QPSK | 15 | 8 |
| 4 | 25 | 15 |  | QPSK | QPSK | 6 | 4 |
| 5 | 25 | 15 |  | 16QAM | 16QAM | 25 | 15 |
| 6 | 25 | 15 |  | 16QAM | QPSK | 25 | 4 |
| 7 | 25 | 25 |  | QPSK | QPSK | 6 | 8 |
| 8 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 9 | 25 | 25 |  | 16QAM | QPSK | 25 | 8 |
| 10 | 25 | 50 |  | QPSK | QPSK | 6 | 12 |
| 11 | 25 | 50 |  | 16QAM | 16QAM | 25 | 50 |
| 12 | 25 | 50 |  | 16QAM | QPSK | 25 | 12 |
| 13 | 50 | 50 |  | QPSK | QPSK | 6 | 12 |
| 14 | 50 | 50 |  | 16QAM | 16QAM | 50 | 50 |
| 15 | 50 | 50 |  | 16QAM | QPSK | 50 | 12 |
| 16 | 100 | 50 |  | QPSK | QPSK | 10 | 12 |
| 17 | 100 | 50 |  | 16QAM | 16QAM | 100 | 50 |
| 18 | 100 | 50 |  | 16QAM | QPSK | 100 | 12 |
| 19 | 75 | 25 |  | QPSK | QPSK | 8 | 8 |
| 20 | 75 | 25 |  | 16QAM | 16QAM | 75 | 25 |
| 21 | 75 | 25 |  | 16QAM | QPSK | 75 | 8 |
| 22 | 6 | 25 |  | QPSK | QPSK | 5 | 8 |
| 23 | 6 | 25 |  | 16QAM | 16QAM | 6 | 25 |
| 24 | 6 | 25 |  | 16QAM | QPSK | 6 | 8 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBend-LCRB+1. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-10: Test Configuration Table (network signalled value NS_03/NS_07)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_07) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 50 | N/A for this test | QPSK | QPSK | 6 | 6@13 |
| 2 | 25 | 50 |  | 16QAM | 16QAM | 25 | 50 |
| 3 | 25 | 50 |  | QPSK | 16QAM | 6 | 50 |
| 4 | 50 | 50 |  | QPSK | QPSK | 6 | 6@13 |
| 5 | 50 | 50 |  | 16QAM | 16QAM | 50 | 50 |
| 6 | 50 | 50 |  | QPSK | 16QAM | 6 | 50 |
| 7 | 100 | 50 |  | QPSK | QPSK | 10 | 6@13 |
| 8 | 100 | 50 |  | 16QAM | 16QAM | 100 | 50 |
| 9 | 100 | 50 |  | QPSK | 16QAM | 10 | 50 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBend-LCRB+1. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-11: Test Configuration Table (network signalled value NS_03/NS_21)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_21) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 6 | 8 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 3 | 25 | 25 |  | 16QAM | QPSK | 25 | 8 |
| 4 (Note 4) | 100 | 50 |  | QPSK | QPSK | 10 | 12@7 |
| 5 (Note 5) | 100 | 50 |  | QPSK | QPSK | 10 | 12@31 |
| 6 | 100 | 50 |  | 16QAM | 16QAM | 100 | 1 |
| 7 | 100 | 50 |  | QPSK | 16QAM | 10 | 1 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.2.4A.2.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBend-LCRB+1. Note 4: Applicable only to Low range frequency testing. Note 5: Applicable only to High range frequency testing. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-12: Test Configuration Table (network signalled value NS_05/NS_01)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low range, Mid range for PCC In case of Low range: - For 5MHz Channel Bandwidth: 1927.2MHz (NUL = 18072) - For 10 MHz Channel Bandwidth: 1934.7 MHz (NUL = 18147) - For 20 MHz Channel Bandwidth: 1930 MHz (NUL = 18100) Mid range for SCC(Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg (Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_05) | SCC (NS_01) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 15 | N/A for this test | QPSK | QPSK | 6 | 4 |
| 2 | 25 | 15 |  | 16QAM | 16QAM | 25 | 6 |
| 3 | 25 | 15 |  | 16QAM | QPSK | 25 | 4 |
| 4 | 25 | 25 |  | QPSK | QPSK | 6 | 6 |
| 5 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 6 | 25 | 25 |  | 16QAM | QPSK | 25 | 6 |
| 7 | 25 | 50 |  | QPSK | QPSK | 6 | 12 |
| 8 | 25 | 50 |  | 16QAM | 16QAM | 25 | 25 |
| 9 | 25 | 50 |  | 16QAM | QPSK | 25 | 12 |
| 10 | 50 | 50 |  | QPSK | QPSK | 12 | 12 |
| 11 | 50 | 50 |  | 16QAM | 16QAM | 50 | 25 |
| 12 | 50 | 50 |  | 16QAM | QPSK | 50 | 12 |
| 13 | 100 | 50 |  | QPSK | QPSK | 18 | 12 |
| 14 | 100 | 50 |  | 16QAM | 16QAM | 50 | 25 |
| 15 | 100 | 50 |  | 16QAM | QPSK | 50 | 12 |
| 16 | 100 | 75 |  | QPSK | QPSK | 18 | 16 |
| 17 | 100 | 75 |  | 16QAM | 16QAM | 50 | 25 |
| 18 | 100 | 75 |  | 16QAM | QPSK | 50 | 16 |
| 19 | 100 | 100 |  | QPSK | QPSK | 18 | 18 |
| 20 | 100 | 100 |  | 16QAM | 16QAM | 50 | 25 |
| 21 | 100 | 100 |  | 16QAM | QPSK | 50 | 18 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. only the combination with the highest NRB PCC is tested. Note 3: For Low range place test point with RBstart=0. For Mid range place test point with RBstart=RBend-LCRB+1. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-13: Test Configuration Table (network signalled value NS_06/NS_21)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_06) | SCC (NS_21) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 8 | 8 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 3 | 25 | 25 |  | 16QAM | QPSK | 25 | 8 |
| 4 (Note 4) | 50 | 50 |  | QPSK | QPSK | 12 | 12@7 |
| 5 (Note 5) | 50 | 50 |  | QPSK | QPSK | 12 | 12@31 |
| 6 | 50 | 50 |  | 16QAM | 16QAM | 50 | 1 |
| 7 | 50 | 50 |  | QPSK | 16QAM | 12 | 1 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.2.4A.2.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBend-LCRB+1. Note 4: Applicable only to Low range frequency testing. Note 5: Applicable only to High range frequency testing. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-14: Test Configuration Table (network signalled value NS_03/NS_28)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Mid range for PCC. For SCC a. 5340 MHz for config ID 1-4 b. 5520 MHz for config ID 5-8 |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_06) | SCC (NS_21) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 100 | N/A for this test | QPSK | QPSK | 6 | L=1 |
| 2 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 3 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 4 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| 5 | 25 | 100 |  | QPSK | QPSK | 6 | L=1 |
| 6 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 7 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 8 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.2.4A.2.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBend-LCRB+1. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-15: Test Configuration Table (network signalled value NS_03/NS_29)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Mid range for PCC. For SCC, for configuration ID: a. 5180 MHz for config ID 1-4 b. 5240 MHz for config ID 5-8 c. 5260 MHz for config ID 9-12 d. 5320 MHz for config ID 13-16 e. 5500 MHz for config ID 17-20 f. 5700 MHz for config ID 21-24 |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_06) | SCC (NS_21) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 100 | N/A for this test | QPSK | QPSK | 6 | L=1 |
| 2 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 3 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 4 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| 5 | 25 | 100 |  | QPSK | QPSK | 6 | L=1 |
| 6 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 7 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 8 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| 9 | 25 | 100 |  | QPSK | QPSK | 6 | L=1 |
| 10 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 11 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 12 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| 13 | 25 | 100 |  | QPSK | QPSK | 6 | L=1 |
| 14 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 15 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 16 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| 17 | 25 | 100 |  | QPSK | QPSK | 6 | L=1 |
| 18 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 19 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 20 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| 21 | 25 | 100 |  | QPSK | QPSK | 6 | L=1 |
| 22 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 23 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 24 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.2.4A.2.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBend-LCRB+1. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-16: Test Configuration Table (network signalled value NS_03/NS_30)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for PCC. For SCC, for configuration ID: a. 5160 MHz for config ID 1-4 b. 5320 MHz for config ID 5-8 c. 5480 MHz for config ID 9-12 d. 5700 MHz for config ID 13-16 e. 5180 MHz for config ID 17-20 |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_06) | SCC (NS_21) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 100 | N/A for this test | QPSK | QPSK | 6 | L=1 |
| 2 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 3 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 4 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| 5 | 25 | 100 |  | QPSK | QPSK | 6 | L=1 |
| 6 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 7 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 8 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| 9 | 25 | 100 |  | QPSK | QPSK | 6 | L=1 |
| 10 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 11 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 12 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| 13 | 25 | 100 |  | QPSK | QPSK | 6 | L=1 |
| 14 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 15 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 16 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| 17 | 25 | 100 |  | QPSK | QPSK | 6 | L=1 |
| 18 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 19 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 20 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.2.4A.2.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBend-LCRB+1. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-17: Test Configuration Table (network signalled value NS_03/NS_31)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for PCC. For SCC, for configuration ID: a. 5160 MHz for config ID 1-4 b. 5260 MHz for config ID 5-8 c. 5700 MHz for config ID 9-12 d. 5745 MHz for config ID 13-16 e. 5600 MHz for config ID 17-20 |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_06) | SCC (NS_21) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 100 | N/A for this test | QPSK | QPSK | 6 | L=1 |
| 2 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 3 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 4 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| 5 | 25 | 100 |  | QPSK | QPSK | 6 | L=1 |
| 6 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 7 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 8 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| 9 | 25 | 100 |  | QPSK | QPSK | 6 | L=1 |
| 10 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 11 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 12 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| 13 | 25 | 100 |  | QPSK | QPSK | 6 | L=1 |
| 14 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 15 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 16 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| 17 | 25 | 100 |  | QPSK | QPSK | 6 | L=1 |
| 18 | 25 | 100 |  | 16QAM | 16QAM | 25 | L=10 |
| 19 | 100 | 100 |  | QPSK | QPSK | 10 | L=1 |
| 20 | 100 | 100 |  | 16QAM | 16QAM | 100 | L=10 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.2.4A.2.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBend-LCRB+1. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-18: Test Configuration Table (network signalled value NS_01/NS_04)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for PCC. Low range for SCC |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_01) | SCC (NS_04) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 6@0 | 6@9 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 25@0 | 25@0 |
| 3 | 25 | 25 |  | QPSK | 16QAM | 6@0 | 25@0 |
| 4 | 100 | 100 |  | QPSK | QPSK | 18@0 | 18@77 |
| 5 | 100 | 100 |  | 16QAM | 16QAM | 100@0 | 100@0 |
| 6 | 100 | 100 |  | QPSK | 16QAM | 18@0 | 100@0 |
| 7 | 15 | 25 |  | QPSK | QPSK | 4@0 | 6@9 |
| 8 | 15 | 25 |  | 16QAM | 16QAM | 15@0 | 25@0 |
| 9 | 15 | 25 |  | QPSK | 16QAM | 4@0 | 25@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.2.4A.2.4.1-1. |  |  |  |  |  |  |  |

Table 6.2.4A.2.4.1-19: Test Configuration Table (network signalled value NS_21/NS_27)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_21) | SCC (NS_27) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 8 | 8 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 3 | 25 | 25 |  | 16QAM | QPSK | 25 | 8 |
| 4 (Note 4) | 50 | 100 |  | QPSK | QPSK | 12@7 | 20 |
| 5 (Note 5) | 50 | 100 |  | QPSK | QPSK | 12@31 | 20 |
| 6 | 50 | 100 |  | 16QAM | 16QAM | 50 | 20 |
| 7 | 50 | 100 |  | QPSK | 16QAM | 12 | 20 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.2.4A.2.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBend-LCRB+1. Note 4: Applicable only to Low range frequency testing. Note 5: Applicable only to High range frequency testing. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to the applicable table from Table 6.2.4A.2.4.1-2 to 6.2.4A.2.4.1-13.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.2.4A.2.4.3.

6.2.4A.2.4.2	Test procedure

1.	 Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.2.4A.2.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to the applicable from Table 6.2.4A.2.4.1-2 to Table 6.2.4A.2.4.1-13 on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms for the UE to reach PUMAX level.

6.	Measure the mean transmitted power over all component carriers in the CA configuration of the radio access mode. The period of measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

6.2.4A.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. The following exceptions apply for each network signalled value.

6.2.4A.2.4.3.1	Message contents exceptions (network signalled value " NS_03" on PCC)

1.	Information element additionalSpectrumEmission is set to NS_03 for PCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.1-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_03" on PCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_03) |  |  |

6.2.4A.2.4.3.2	Message contents exceptions (network signalled value " NS_03" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_03 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.2-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_03" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_03) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_03 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.2.4A.2.4.3.2-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_03" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 3 (NS_03) |  |  |

6.2.4A.2.4.3.3	Message contents exceptions (network signalled value " NS_05" on PCC)

1.	Information element additionalSpectrumEmission is set to NS_05 for PCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.3-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_05" on PCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 5 (NS_05) |  |  |

6.2.4A.2.4.3.4	Message contents exceptions (network signalled value " NS_06" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_06 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.4-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_06" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 6 (NS_06) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_06 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.2.4A.2.4.3.4-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_06" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 6 (NS_06) |  |  |

6.2.4A.2.4.3.5	Message contents exceptions (network signalled value " NS_07" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_07 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.5-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_07" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 7 (NS_07) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_07 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.2.4A.2.4.3.5-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_07" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 7 (NS_07) |  |  |

6.2.4A.2.4.3.6	Message contents exceptions (network signalled value " NS_12" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_12 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.6-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_12" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 12 (NS_12) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_12 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.2.4A.2.4.3.6-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_12" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 12 (NS_12) |  |  |

6.2.4A.2.4.3.7	Message contents exceptions (network signalled value " NS_15" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_15 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.7-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_15" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 15 (NS_15) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_15 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.2.4A.2.4.3.7-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_15" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 15 (NS_15) |  |  |

6.2.4A.2.4.3.8	Message contents exceptions (network signalled value " NS_21" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_21 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.8-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_21" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 21 (NS_21) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_21 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.2.4A.2.4.3.8-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_21" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 21 (NS_21) |  |  |

6.2.4A.2.4.3.9	Message contents exceptions (network signalled value " NS_28" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_28 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.9-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_28" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 28 (NS_28) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_28 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.2.4A.2.4.3.9-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_28" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 28 (NS_28) |  |  |

6.2.4A.2.4.3.10	Message contents exceptions (network signalled value " NS_29" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_29 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.10-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_29" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 29 (NS_29) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_29 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.2.4A.2.4.3.10-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_29" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 29 (NS_29) |  |  |

6.2.4A.2.4.3.11	Message contents exceptions (network signalled value " NS_30" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_30 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.11-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_30" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 30 (NS_30) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_30 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.2.4A.2.4.3.11-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_30" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 30 (NS_30) |  |  |

6.2.4A.2.4.3.12	Message contents exceptions (network signalled value " NS_31" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_31 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.12-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_31" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 31 (NS_31) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_31 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.2.4A.2.4.3.12-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_31" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 31 (NS_31) |  |  |

6.2.4A.2.4.3.13	Message contents exceptions (network signalled value " NS_04" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_04 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.13-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_04" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 04 (NS_04) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_04 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.2.4A.2.4.3.13-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_04" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 04 (NS_04) |  |  |

6.2.4A.2.4.3.14	Message contents exceptions (network signalled value " NS_27" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_27 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.14-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_27" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 27 (NS_27) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_27 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.2.4A.2.4.3.14-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_27" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 27 (NS_27) |  |  |

6.2.4A.2.4.3.15	Message contents exceptions (network signalled value " NS_21" on PCC)

1.	Information element additionalSpectrumEmission is set to NS_21 for PCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.2.4A.2.4.3.15-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_21" on PCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 21 (NS_21) |  |  |

6.2.4A.2.5	Test requirements

The maximum output power, derived in step 6 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2.4A.2.5-1 to Table 6.2.4A.2.5-16. For the UE maximum output power modified by A-MPR specified in sub-clause 6.2.4A.2.3, the power limits specified in Table 6.2.5A.3.3-1 apply.

Table 6.2.4A.2.5-1: Test requirement (network signalled value NS_01/NS_03 or network signalled value NS_03/NS_01)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 2 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 3 | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 4 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 5 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 6 | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 7 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 8 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 9 | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 10 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 11 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 12 | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 13 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 14 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 15 | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. Note 2: ΔTC,c does not apply in all bands. For bands without this lower limit is 19.3 dBm in all test points |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-2: Test requirement (network signalled value NS_01/NS_06)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 0 | 1.5 | 23 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 2 | 2 | 2 | 0 | 0 | 0 | 1.5 | 21 | 19.5 | 23.0 | 3 | 25.7 | 19.3 |
| 3 | 0 | 2 | 0 | 0 | 0 | 1.5 | 23 | 19.5 | 23.0 | 3 | 25.7 | 19.3 |
| 4 | 0 | 0 | 0 | 0 | 0 | 1.5 | 23 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 5 | 2 | 2 | 0 | 0 | 0 | 1.5 | 21 | 19.5 | 23.0 | 3 | 25.7 | 19.3 |
| 6 | 0 | 2 | 0 | 0 | 0 | 1.5 | 23 | 19.5 | 23.0 | 3 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-3: Test requirement (network signalled value NS_01/NS_12)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 1.5 | 1.5 | 21.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 2 | 2 | 1 | 0 | 5 | 1.5 | 1.5 | 19.5 | 15.5 | 21.0 | 5 | 25.7 | 15.3 |
| 3 | 0 | 1 | 0 | 5 | 1.5 | 1.5 | 21.5 | 15.5 | 22.5 | 5 | 25.7 | 16.8 |
| 4 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 5 | 2 | 2 | 0 | 4 | 1.5 | 1.5 | 19.5 | 15.5 | 21.0 | 5 | 25.7 | 15.3 |
| 6 | 0 | 2 | 0 | 4 | 1.5 | 1.5 | 21.5 | 15.5 | 22.5 | 5 | 25.7 | 16.8 |
| 7 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 8 | 2 | 2 | 0 | 4 | 0 | 0 | 21 | 17 | 22.5 | 5 | 25.7 | 16.8 |
| 9 | 0 | 2 | 0 | 4 | 1.5 | 0 | 21.5 | 17 | 22.8 | 5 | 25.7 | 17.1 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-4: Test requirement (network signalled value NS_01/NS_15)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 1.5 | 1.5 | 21.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 2 | 2 | 2 | 0 | 9 | 1.5 | 1.5 | 19.5 | 10.5 | 20.0 | 6 | 25.7 | 13.3 |
| 3 | 0 | 2 | 0 | 9 | 1.5 | 1.5 | 21.5 | 10.5 | 21.8 | 5 | 25.7 | 16.1 |
| 4 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 5 | 2 | 2 | 0 | 9 | 1.5 | 1.5 | 19.5 | 10.5 | 20.0 | 6 | 25.7 | 13.3 |
| 6 | 0 | 2 | 0 | 9 | 1.5 | 1.5 | 21.5 | 10.5 | 21.8 | 5 | 25.7 | 16.1 |
| 7 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 8 | 2 | 2 | 0 | 9 | 0 | 0 | 21 | 12 | 21.5 | 5 | 25.7 | 15.8 |
| 9 | 0 | 2 | 0 | 9 | 0 | 0 | 21.5 | 12 | 23 | 5 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-5: Test requirement (network signalled value NS_01/NS_17)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 2 | 2 | 2 | 0 | 0 | 0 | 0 | 21 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 3 | 2 | 0 | 0 | 0 | 0 | 0 | 21 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 4 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 5 | 2 | 2 | 0 | 0 | 0 | 0 | 21 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 6 | 2 | 0 | 0 | 0 | 0 | 0 | 21 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 7 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 8 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 9 | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 10 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 11 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 12 | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-6: Test requirement (network signalled value NS_01/NS_21)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3.0 | 25.7 | 19.3 |
| 2 | 2 | 2 | 0 | 0 | 0 | 0 | 21 | 21 | 23.0 | 3.0 | 25.7 | 19.3 |
| 3 | 0 | 2 | 0 | 0 | 0 | 0 | 23 | 21 | 23.0 | 3.0 | 25.7 | 19.3 |
| 4 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3.0 | 25.7 | 19.3 |
| 5 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3.0 | 25.7 | 19.3 |
| 6 | 2 | 1 | 0 | 4 | 0 | 0 | 21 | 18 | 22.8 | 5.0 | 25.7 | 17.1 |
| 7 | 0 | 1 | 0 | 4 | 0 | 0 | 23 | 18 | 23.0 | 3.0 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-7: Test requirement (network signalled value NS_03/NS_03)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 2 | 2 | 2 | 1 | 1 | 1.5 | 0 | 18.5 | 20 | 22.3 | 5 | 25.7 | 16.6 |
| 3 | 2 | 0 | 1 | 0 | 1.5 | 0 | 18.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 4 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 5 | 2 | 2 | 1 | 1 | 0 | 0 | 20 | 20 | 23 | 3 | 25.7 | 19.3 |
| 6 | 2 | 0 | 1 | 0 | 1.5 | 0 | 18.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 7 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 8 | 2 | 2 | 1 | 1 | 0 | 0 | 20 | 20 | 23.0 | 3 | 25.7 | 19.3 |
| 9 | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 10 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 11 | 2 | 2 | 1 | 1 | 0 | 0 | 20 | 20 | 23.0 | 3 | 25.7 | 19.3 |
| 12 | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 13 | 0 | 0 | 0 | 0 | 1.5 | 1.5 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 14 | 2 | 2 | 0 | 0 | 1.5 | 1.5 | 19.5 | 19.5 | 22.5 | 5 | 25.7 | 16.8 |
| 15 | 2 | 0 | 0 | 0 | 1.5 | 1.5 | 19.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. Note 2: ΔTC,c does not apply in all bands. For bands without this lower limit is 19.3 dBm in all test points |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-8: Test requirement (network signalled value NS_03/NS_06)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 1.5 | 1.5 | 21.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 2 | 2 | 2 | 1 | 0 | 1.5 | 0 | 18.5 | 21 | 22.9 | 5 | 25.7 | 17.2 |
| 3 | 2 | 0 | 1 | 0 | 1.5 | 1.5 | 18.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 4 | 0 | 0 | 0 | 0 | 1.5 | 1.5 | 21.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 5 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 6 | 2 | 0 | 1 | 0 | 0 | 1.5 | 20 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 7 | 0 | 0 | 0 | 0 | 1.5 | 1.5 | 21.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 8 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 9 | 2 | 0 | 1 | 0 | 0 | 1.5 | 20 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 10 | 0 | 0 | 0 | 0 | 1.5 | 1.5 | 21.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 11 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 12 | 2 | 0 | 1 | 0 | 0 | 1.5 | 20 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 13 | 0 | 0 | 0 | 0 | 1.5 | 1.5 | 21.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 14 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 15 | 2 | 0 | 1 | 0 | 0 | 1.5 | 20 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 16 | 0 | 0 | 0 | 0 | 1.5 | 1.5 | 21.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 17 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 18 | 2 | 0 | 1 | 0 | 0 | 1.5 | 20 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 19 | 0 | 0 | 0 | 0 | 1.5 | 1.5 | 21.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 20 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 21 | 2 | 0 | 1 | 0 | 0 | 1.5 | 20 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 22 | 0 | 0 | 0 | 0 | 1.5 | 1.5 | 21.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 23 | 2 | 2 | 0 | 0 | 1.5 | 1.5 | 19.5 | 19.5 | 22.5 | 5 | 25.7 | 16.8 |
| 24 | 2 | 0 | 0 | 0 | 1.5 | 1.5 | 19.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. Note 2: ΔTC,c does not apply in all bands. For bands without this lower limit is 19.3 dBm in all test points |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-9: Test requirement (network signalled value NS_03/NS_07)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| CA_2A-13A |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 2 | 2 | 2 | 1 | 12 | 0 | 0 | 20 | 9 | 20.3 | 6 | 25.7 | 13.6 |
| 3 | 0 | 2 | 0 | 12 | 1.5 | 0 | 21.5 | 9 | 21.7 | 5 | 25.7 | 16.0 |
| 4 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 5 | 2 | 2 | 1 | 12 | 0 | 0 | 20 | 9 | 20.3 | 6 | 25.7 | 13.6 |
| 6 | 0 | 2 | 0 | 12 | 1.5 | 0 | 21.5 | 9 | 21.7 | 5 | 25.7 | 16.0 |
| 7 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 8 | 2 | 2 | 1 | 12 | 0 | 0 | 20 | 9 | 20.3 | 6 | 25.7 | 13.6 |
| 9 | 0 | 2 | 0 | 12 | 1.5 | 0 | 21.5 | 9 | 21.7 | 5 | 25.7 | 16.0 |
| CA_4A-13A |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 2 | 2 | 2 | 1 | 12 | 0 | 0 | 20 | 9 | 20.3 | 6 | 25.7 | 13.6 |
| 3 | 0 | 2 | 0 | 12 | 0 | 0 | 23 | 9 | 23.0 | 3 | 25.7 | 19.3 |
| 4 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 5 | 2 | 2 | 1 | 12 | 0 | 0 | 20 | 9 | 20.3 | 6 | 25.7 | 13.6 |
| 6 | 0 | 2 | 0 | 12 | 0 | 0 | 23 | 9 | 23.0 | 3 | 25.7 | 19.3 |
| 7 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 8 | 2 | 2 | 1 | 12 | 0 | 0 | 20 | 9 | 20.3 | 6 | 25.7 | 13.6 |
| 9 | 0 | 2 | 0 | 12 | 0 | 0 | 23 | 9 | 23.0 | 3 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. Note 2: ΔTC,c does not apply in all bands. For bands without this lower limit is 19.3 dBm in all test points |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-10: Test requirement (network signalled value NS_03/NS_21)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3.0 | 25.7 | 19.3 |
| 2 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3.0 | 25.7 | 19.3 |
| 3 | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.0 | 3.0 | 25.7 | 19.3 |
| 4 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3.0 | 25.7 | 19.3 |
| 5 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3.0 | 25.7 | 19.3 |
| 6 | 2 | 1 | 1 | 4 | 0 | 0 | 20 | 18 | 22.1 | 5.0 | 25.7 | 16.4 |
| 7 | 0 | 1 | 0 | 4 | 1.5 | 0 | 21.5 | 18 | 23.0 | 3.0 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-11: Test requirement (network signalled value NS_05/NS_01)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| CA_1A-3A / CA_1A-5A / CA_1A-7A / CA_1A-8A / CA_1A-18A / CA_1A-26A |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 2 | 2 | 2 | 0 | 0 | 0 | 0 | 21 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 3 | 2 | 0 | 0 | 0 | 0 | 0 | 21 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 4 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 5 | 2 | 2 | 0 | 0 | 0 | 0 | 21 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 6 | 2 | 0 | 0 | 0 | 0 | 0 | 21 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 7 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 8 | 2 | 2 | 0 | 0 | 0 | 0 | 21 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 9 | 2 | 0 | 0 | 0 | 0 | 0 | 21 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 10 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 11 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 12 | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 13 | 0 | 0 | 11 | 0 | 0 | 0 | 12 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 14 | 2 | 2 | 11 | 0 | 0 | 0 | 10 | 21 | 21.3 | 5 | 25.7 | 15.6 Note 3 |
| 15 | 2 | 0 | 11 | 0 | 0 | 0 | 10 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 16 | 0 | 0 | 11 | 0 | 0 | 0 | 12 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 17 | 2 | 2 | 11 | 0 | 0 | 0 | 10 | 21 | 21.3 | 5 | 25.7 | 15.6 Note 3 |
| 18 | 2 | 0 | 11 | 0 | 0 | 0 | 10 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 19 | 0 | 0 | 11 | 0 | 0 | 0 | 12 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 20 | 2 | 2 | 11 | 0 | 0 | 0 | 10 | 21 | 21.3 | 5 | 25.7 | 15.6 Note 3 |
| 21 | 2 | 0 | 11 | 0 | 0 | 0 | 10 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| CA_1A-42A |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 26.0 | 19.0 |
| 2 | 2 | 2 | 0 | 0 | 0 | 0 | 21 | 21 | 23.0 | 3 | 26.0 | 19.0 |
| 3 | 2 | 0 | 0 | 0 | 0 | 0 | 21 | 23 | 23.0 | 3 | 26.0 | 19.0 |
| 4 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 26.0 | 19.0 |
| 5 | 2 | 2 | 0 | 0 | 0 | 0 | 21 | 21 | 23.0 | 3 | 26.0 | 19.0 |
| 6 | 2 | 0 | 0 | 0 | 0 | 0 | 21 | 23 | 23.0 | 3 | 26.0 | 19.0 |
| 7 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 26.0 | 19.0 |
| 8 | 2 | 2 | 0 | 0 | 0 | 0 | 21 | 21 | 23.0 | 3 | 26.0 | 19.0 |
| 9 | 2 | 0 | 0 | 0 | 0 | 0 | 21 | 23 | 23.0 | 3 | 26.0 | 19.0 |
| 10 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 26.0 | 19.0 |
| 11 | 2 | 2 | 1 | 0 | 0 | 0 | 20 | 21 | 23.0 | 3 | 26.0 | 19.0 |
| 12 | 2 | 0 | 1 | 0 | 0 | 0 | 20 | 23 | 23.0 | 3 | 26.0 | 19.0 |
| 13 | 0 | 0 | 11 | 0 | 0 | 0 | 12 | 23 | 23.0 | 3 | 26.0 | 19.0 |
| 14 | 2 | 2 | 11 | 0 | 0 | 0 | 10 | 21 | 21.3 | 5 | 26.0 | 15.3 Note 4 |
| 15 | 2 | 0 | 11 | 0 | 0 | 0 | 10 | 23 | 23.0 | 3 | 26.0 | 19.3 |
| 16 | 0 | 0 | 11 | 0 | 0 | 0 | 12 | 23 | 23.0 | 3 | 26.0 | 19.3 |
| 17 | 2 | 2 | 11 | 0 | 0 | 0 | 10 | 21 | 21.3 | 5 | 26.0 | 15.3 Note 4 |
| 18 | 2 | 0 | 11 | 0 | 0 | 0 | 10 | 23 | 23.0 | 3 | 26.0 | 19.0 |
| 19 | 0 | 0 | 11 | 0 | 0 | 0 | 12 | 23 | 23.0 | 3 | 26.0 | 19.0 |
| 20 | 2 | 2 | 11 | 0 | 0 | 0 | 10 | 21 | 21.3 | 5 | 26.0 | 15.3 Note 4 |
| 21 | 2 | 0 | 11 | 0 | 0 | 0 | 10 | 23 | 23.0 | 3 | 26.0 | 19.0 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. Note 2: Void Note 3: For PCC in mid frequency range the lower limit is 19.3 dBm. Note 4: For PCC in mid frequency range the lower limit is 19.0 dBm. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-12: Test requirement (network signalled value NS_06/NS_21)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3.0 | 25.7 | 19.3 |
| 2 | 2 | 2 | 0 | 0 | 0 | 0 | 21 | 21 | 23.0 | 3.0 | 25.7 | 19.3 |
| 3 | 2 | 0 | 0 | 0 | 0 | 0 | 21 | 23 | 23.0 | 3.0 | 25.7 | 19.3 |
| 4 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3.0 | 25.7 | 19.3 |
| 5 | 0 | 0 | 0 | 0 | 1.5 | 0 | 21.5 | 23 | 23.0 | 3.0 | 25.7 | 19.3 |
| 6 | 2 | 1 | 0 | 4 | 0 | 0 | 21 | 18 | 22.8 | 5.0 | 25.7 | 17.1 |
| 7 | 0 | 1 | 0 | 4 | 1.5 | 0 | 21.5 | 18 | 23.0 | 3.0 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-13: Test requirement (network signalled value NS_03/NS_28)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 2.5 | 0 | 8 | 0 | 0 | 23 | 12.5 | 23.0 | 3 | 25.7 | 18.5 |
| 2 | 2 | 2 | 1 | 9 | 0 | 0 | 20 | 12 | 20.6 | 6 | 25.7 | 13.1 |
| 3 | 0 | 2.5 | 0 | 8 | 0 | 0 | 23 | 12.5 | 23.0 | 3 | 25.7 | 18.5 |
| 4 | 2 | 2 | 1 | 9 | 0 | 0 | 20 | 12 | 20.6 | 6 | 25.7 | 13.1 |
| 5 | 0 | 2.5 | 0 | 2 | 0 | 0 | 23 | 18.5 | 23.0 | 3 | 25.7 | 18.5 |
| 6 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| 7 | 0 | 2.5 | 0 | 2 | 0 | 0 | 23 | 18.5 | 23.0 | 3 | 25.7 | 18.5 |
| 8 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-14: Test requirement (network signalled value NS_03/NS_29)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 2.5 | 0 | 1 | 0 | 0 | 23 | 19.5 | 23.0 | 3 | 25.7 | 18.5 |
| 2 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| 3 | 0 | 2.5 | 0 | 1 | 0 | 0 | 23 | 19.5 | 23.0 | 3 | 25.7 | 18.5 |
| 4 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| 5 | 0 | 2.5 | 0 | 1 | 0 | 0 | 23 | 19.5 | 23.0 | 3 | 25.7 | 18.5 |
| 6 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| 7 | 0 | 2.5 | 0 | 1 | 0 | 0 | 23 | 19.5 | 23.0 | 3 | 25.7 | 18.5 |
| 8 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| 9 | 0 | 2.5 | 0 | 1 | 0 | 0 | 23 | 19.5 | 23.0 | 3 | 25.7 | 18.5 |
| 10 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| 11 | 0 | 2.5 | 0 | 1 | 0 | 0 | 23 | 19.5 | 23.0 | 3 | 25.7 | 18.5 |
| 12 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| 13 | 0 | 2.5 | 0 | 1 | 0 | 0 | 23 | 19.5 | 23.0 | 3 | 25.7 | 18.5 |
| 14 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| 15 | 0 | 2.5 | 0 | 1 | 0 | 0 | 23 | 19.5 | 23.0 | 3 | 25.7 | 18.5 |
| 16 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| 17 | 0 | 2.5 | 0 | 1 | 0 | 0 | 23 | 19.5 | 23.0 | 3 | 25.7 | 18.5 |
| 18 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| 19 | 0 | 2.5 | 0 | 1 | 0 | 0 | 23 | 19.5 | 23.0 | 3 | 25.7 | 18.5 |
| 20 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| 21 | 0 | 2.5 | 0 | 1 | 0 | 0 | 23 | 19.5 | 23.0 | 3 | 25.7 | 18.5 |
| 22 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| 23 | 0 | 2.5 | 0 | 1 | 0 | 0 | 23 | 19.5 | 23.0 | 3 | 25.7 | 18.5 |
| 24 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-15: Test requirement (network signalled value NS_03/NS_30)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 2.5 | 0 | 16 | 0 | 0 | 23 | 4.5 | 23.0 | 3 | 25.7 | 18.5 |
| 2 | 2 | 2 | 1 | 15 | 0 | 0 | 20 | 6 | 20.2 | 6 | 25.7 | 12.7 |
| 3 | 0 | 2.5 | 0 | 16 | 0 | 0 | 23 | 4.5 | 23.0 | 3 | 25.7 | 18.5 |
| 4 | 2 | 2 | 1 | 15 | 0 | 0 | 20 | 6 | 20.2 | 6 | 25.7 | 12.7 |
| 5 | 0 | 2.5 | 0 | 3 | 0 | 0 | 23 | 17.5 | 23.0 | 3 | 25.7 | 18.5 |
| 6 | 2 | 2 | 1 | 3 | 0 | 0 | 20 | 18 | 22.1 | 5 | 25.7 | 15.6 |
| 7 | 0 | 2.5 | 0 | 3 | 0 | 0 | 23 | 17.5 | 23.0 | 3 | 25.7 | 18.5 |
| 8 | 2 | 2 | 1 | 3 | 0 | 0 | 20 | 18 | 22.1 | 5 | 25.7 | 15.6 |
| 9 | 0 | 2.5 | 0 | 11 | 0 | 0 | 23 | 9.5 | 23.0 | 3 | 25.7 | 18.5 |
| 10 | 2 | 2 | 1 | 11 | 0 | 0 | 20 | 10 | 20.4 | 6 | 25.7 | 12.9 |
| 11 | 0 | 2.5 | 0 | 11 | 0 | 0 | 23 | 9.5 | 23.0 | 3 | 25.7 | 18.5 |
| 12 | 2 | 2 | 1 | 11 | 0 | 0 | 20 | 10 | 20.4 | 6 | 25.7 | 12.9 |
| 13 | 0 | 2.5 | 0 | 6 | 0 | 0 | 23 | 14.5 | 23.0 | 3 | 25.7 | 18.5 |
| 14 | 2 | 2 | 1 | 6 | 0 | 0 | 20 | 15 | 21.2 | 5 | 25.7 | 14.7 |
| 15 | 0 | 2.5 | 0 | 6 | 0 | 0 | 23 | 14.5 | 23.0 | 3 | 25.7 | 18.5 |
| 16 | 2 | 2 | 1 | 6 | 0 | 0 | 20 | 15 | 21.2 | 5 | 25.7 | 14.7 |
| 17 | 0 | 2.5 | 0 | NA | 0 | 0 | 23 | NA | NA | NA | NA | NA |
| 18 | 2 | 2 | 1 | 1 | 0 | 0 | 20 | 20 | 23.0 | 3 | 25.7 | 18.5 |
| 19 | 0 | 2.5 | 0 | NA | 0 | 0 | 23 | NA | NA | NA | NA | NA |
| 20 | 2 | 2 | 1 | 1 | 0 | 0 | 20 | 20 | 23.0 | 3 | 25.7 | 18.5 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-16: Test requirement (network signalled value NS_03/NS_31)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 2.5 | 0 | 7 | 0 | 0 | 23 | 13.5 | 23.0 | 3 | 25.7 | 18.5 |
| 2 | 2 | 2 | 1 | 7 | 0 | 0 | 20 | 14 | 21.0 | 5 | 25.7 | 14.5 |
| 3 | 0 | 2.5 | 0 | 7 | 0 | 0 | 23 | 13.5 | 23.0 | 3 | 25.7 | 18.5 |
| 4 | 2 | 2 | 1 | 7 | 0 | 0 | 20 | 14 | 21.0 | 5 | 25.7 | 14.5 |
| 5 | 0 | 2.5 | 0 | 6 | 0 | 0 | 23 | 14.5 | 23.0 | 3 | 25.7 | 18.5 |
| 6 | 2 | 2 | 1 | 6 | 0 | 0 | 20 | 15 | 21.2 | 5 | 25.7 | 14.7 |
| 7 | 0 | 2.5 | 0 | 6 | 0 | 0 | 23 | 14.5 | 23.0 | 3 | 25.7 | 18.5 |
| 8 | 2 | 2 | 1 | 6 | 0 | 0 | 20 | 15 | 21.2 | 5 | 25.7 | 14.7 |
| 9 | 0 | 2.5 | 0 | 6 | 0 | 0 | 23 | 14.5 | 23.0 | 3 | 25.7 | 18.5 |
| 10 | 2 | 2 | 1 | 6 | 0 | 0 | 20 | 15 | 21.2 | 5 | 25.7 | 14.7 |
| 11 | 0 | 2.5 | 0 | 6 | 0 | 0 | 23 | 14.5 | 23.0 | 3 | 25.7 | 18.5 |
| 12 | 2 | 2 | 1 | 6 | 0 | 0 | 20 | 15 | 21.2 | 5 | 25.7 | 14.7 |
| 13 | 0 | 2.5 | 0 | 6 | 0 | 0 | 23 | 14.5 | 23.0 | 3 | 25.7 | 18.5 |
| 14 | 2 | 2 | 1 | 6 | 0 | 0 | 20 | 15 | 21.2 | 5 | 25.7 | 14.7 |
| 15 | 0 | 2.5 | 0 | 6 | 0 | 0 | 23 | 14.5 | 23.0 | 3 | 25.7 | 18.5 |
| 16 | 2 | 2 | 1 | 6 | 0 | 0 | 20 | 15 | 21.2 | 5 | 25.7 | 14.7 |
| 17 | 0 | 2.5 | 0 | 1 | 0 | 0 | 23 | 19.5 | 23.0 | 3 | 25.7 | 18.5 |
| 18 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| 19 | 0 | 2.5 | 0 | 1 | 0 | 0 | 23 | 19.5 | 23.0 | 3 | 25.7 | 18.5 |
| 20 | 2 | 2 | 1 | 2 | 0 | 0 | 20 | 19 | 22.5 | 5 | 25.7 | 16.0 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2.5-17: Test requirement (network signalled value NS_01/NS_04)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 1.5 | 1.5 | 21.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 2 | 2 | 2 | 0 | 2 | 1.5 | 1.5 | 19.5 | 17.5 | 21.6 | 5 | 25.7 | 15.9 |
| 3 | 0 | 2 | 0 | 2 | 1.5 | 1.5 | 21.5 | 17.5 | 23.0 | 3 | 25.7 | 19.3 |
| 4 | 0 | 0 | 0 | 0 | 1.5 | 1.5 | 21.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 5 | 2 | 2 | 0 | 3 | 1.5 | 1.5 | 19.5 | 16.5 | 21.3 | 5 | 25.7 | 15.6 |
| 6 | 0 | 2 | 0 | 3 | 1.5 | 1.5 | 21.5 | 16.5 | 22.7 | 5 | 25.7 | 19.0 |
| 7 | 0 | 0 | 0 | 0 | 1.5 | 1.5 | 21.5 | 21.5 | 23.0 | 3 | 25.7 | 19.3 |
| 8 | 2 | 2 | 0 | 2 | 1.5 | 1.5 | 19.5 | 17.5 | 21.6 | 5 | 25.7 | 15.9 |
| 9 | 0 | 2 | 0 | 2 | 1.5 | 1.5 | 21.5 | 17.5 | 23.0 | 3 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.2_1.5-18: Test requirement (network signalled value NS_21/NS_27)

| Config ID | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | PCMAX,c (dBm) |  | PCMAX (dBm) | TLOW(PCMAX) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 2 | 2 | 2 | 0 | 0 | 0 | 0 | 21 | 21 | 23.0 | 3 | 25.7 | 19.3 |
| 3 | 2 | 0 | 0 | 0 | 0 | 0 | 21 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 4 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 5 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 23 | 23.0 | 3 | 25.7 | 19.3 |
| 6 | 2 | 1 | 4 | 4 | 0 | 0 | 17 | 18 | 20.5 | 6 | 25.7 | 13.8 |
| 7 | 0 | 1 | 0 | 4 | 0 | 0 | 23 | 18 | 23.0 | 3 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

6.6.2.2A.2	Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA)

6.6.2.2A.2.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth for CA under the deployment scenarios where additional requirements are specified.

6.6.2.2A.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support inter-band DL CA and UL CA but do not support UL 64QAM or UL 256QAM.

6.6.2.2A.2.3	Minimum conformance requirements

The minimum requirements are the same as in section 6.6.2.2.3 per component carrier.

6.6.2.2A.2.4	Test description

6.6.2.2A.2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. For Uplink CA Configurations listed in table 6.6.2.2A.2.4.1-1, Additional Spectrum emission requirements are tested.

Table 6.6.2.2A.2.4.1-1: Additional Spectrum Emission mask for inter-band CA

| Uplink CA Configuration | NS-value per CC (in same order as UL CA Configuration column) |  | Applicable spectrum emission requirements (in same order as UL CA Configuration column) |  |
| --- | --- | --- | --- | --- |
| CA_1A-3A CA_1A-5A CA_1A-7A CA_1A-8A CA_1A-18A CA_1A-26A CA_1A-42A | NS_05 | NS_01 | N/A | N/A |
| CA_1A-19A | NS_05 | NS_08 | N/A | N/A |
| CA_1A-21A | NS_05 | NS_09 | N/A | N/A |
| CA_1A-28A | NS_05 | NS_17 | N/A | N/A |
| CA_2A-4A, CA_2A-66A | NS_03 | NS_03 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.1-1 |
| CA_2A-12A, CA_2A-13A, CA_4A-12A, CA_4A-13A, CA_4A-17A | NS_03 | NS_06 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.3-1 |
| CA_2A-30A | NS_03 | NS_21 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.1-1 |
| CA_3A-5A CA_3A-7A CA_3A-8A CA_5A-7A CA_39A-41A CA_41A-42A | NS_01 | NS_01 | N/A | N/A |
| CA_3A-19A | NS_01 | NS_08 | N/A | N/A |
| CA_3A-20A CA_7A-20A | NS_01 | NS_10 | N/A | N/A |
| CA_2A-13A, CA_4A-13A | NS_03 | NS_07 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.3-1 |
| CA_2A-5A CA_2A-7A, CA_4A-5A, CA_4A-7A | NS_03 | NS_01 | Table 6.6.2.2.3.1-1 | N/A |
| CA_5A-66ANote 5 | NS_01 | NS_03 | N/A | Table 6.6.2.2.3.1-1 |
| CA_5A-12A CA_5A-17A | NS_01 | NS_06 | N/A | Table 6.6.2.2.3.3-1 |
| CA_5A-30A | NS_01 | NS_21 | N/A | Table 6.6.2.2.3.1-1 |
| CA_7A-28A CA_18A-28A | NS_01 | NS_17 | N/A | N/A |
| CA_7A-28A | NS_01 | NS_18 | N/A | N/A |
| CA_12A-30A | NS_06 | NS_21 | Table 6.6.2.2.3.3-1 | Table 6.6.2.2.3.3-1 |
| CA_12A-66ANote 5 | NS_06 | NS_03 | Table 6.6.2.2.3.3-1 | Table 6.6.2.2.3.1-1 |
| CA_19A-21A | NS_08 | NS_09 | N/A | N/A |
| CA_30A-48A | NS_21 | NS_27 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.4-1 |
| Note 1: For CCs with NS_01, there are no additional requirements defined in 36.101. Only CCs with NS-value different than NS_01 needs to be tested. Note 2: A-MPR values for NS-values in this table are the same as in section 6.2.4.3. Note 3: Selection of CA configurations and NS-values to test is derived in TR 36.905 [19]. Note 4: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column. Note 5: As an exception to note 4, PCC is mapped to the second band of the combination as listed in Uplink CA Configuration column. |  |  |  |  |

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in Table 5.4.2A.1-2 for CA configurations with two bands or Table 5.4.2A.1-2a for CA configurations with three bands. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in tables 6.6.2.2A.2.4.1-2 to 6.6.2.2A.2.4.1-9. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.2A.2.4.1-2: Test Configuration Table (network signalled value NS_01/NS_03 or network signalled value NS_03/NS_01)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and high range for PCC Mid range for SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | NS_03 CC | NS_01 CC | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 6 | 8 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 3 | 25 | 25 |  | 16QAM | QPSK | 25 | 8 |
| 4 | 50 | 50 |  | QPSK | QPSK | 6 | 12 |
| 5 | 50 | 50 |  | 16QAM | 16QAM | 50 | 50 |
| 6 | 50 | 50 |  | 16QAM | QPSK | 50 | 12 |
| 7 | 50 | 100 |  | QPSK | QPSK | 6 | 18 |
| 8 | 50 | 100 |  | 16QAM | 16QAM | 50 | 100 |
| 9 | 50 | 100 |  | 16QAM | QPSK | 50 | 18 |
| 10 | 100 | 50 |  | QPSK | QPSK | 10 | 12 |
| 11 | 100 | 50 |  | 16QAM | 16QAM | 100 | 50 |
| 12 | 100 | 50 |  | 16QAM | QPSK | 100 | 12 |
| 13 | 100 | 100 |  | QPSK | QPSK | 10 | 18 |
| 14 | 100 | 100 |  | 16QAM | 16QAM | 100 | 100 |
| 15 | 100 | 100 |  | 16QAM | QPSK | 100 | 18 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. Only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. For Mid range place test point with RBstart=0 and RBstart=RBmax-LCRB+1. Note 4: Mapping of NS-value per CC and PCC/SCC selection are specified in Table 6.6.2.2A.2.4.1-1. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2.4.1-3: Test Configuration Table (network signalled value NS_01/NS_06)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_07) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 6 | 6 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 25 | 9 |
| 3 | 25 | 25 |  | QPSK | 16QAM | 6 | 9 |
| 4 | 50 | 50 |  | QPSK | QPSK | 12 | 12 |
| 5 | 50 | 50 |  | 16QAM | 16QAM | 50 | 15 |
| 6 | 50 | 50 |  | QPSK | 16QAM | 12 | 15 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. Only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2.4.1-4: Test Configuration Table (network signalled value NS_01/NS_21)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_01) | SCC (NS_21) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 8 | 8 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 3 | 25 | 25 |  | QPSK | 16QAM | 8 | 25 |
| 4 (Note 4) | 50 | 50 |  | QPSK | QPSK | 12 | 12@7 |
| 5 (Note 5) | 50 | 50 |  | QPSK | QPSK | 12 | 12@31 |
| 6 | 50 | 50 |  | 16QAM | 16QAM | 50 | 1 |
| 7 | 50 | 50 |  | QPSK | 16QAM | 12 | 1 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.6.2.2A.2.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBmax-LCRB+1. Note 4: Applicable only to Low range frequency testing. Note 5: Applicable only to High range frequency testing. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2.4.1-5: Test Configuration Table (network signalled value NS_03/NS_03)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg (Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_03) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 15 | 25 | N/A for this test | QPSK | QPSK | 4 | 6 |
| 2 | 15 | 25 |  | 16QAM | 16QAM | 15 | 25 |
| 3 | 15 | 25 |  | 16QAM | QPSK | 15 | 6 |
| 4 | 25 | 25 |  | QPSK | QPSK | 6 | 6 |
| 5 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 6 | 25 | 25 |  | 16QAM | QPSK | 25 | 6 |
| 7 | 50 | 50 |  | QPSK | QPSK | 6 | 6 |
| 8 | 50 | 50 |  | 16QAM | 16QAM | 50 | 50 |
| 9 | 50 | 50 |  | 16QAM | QPSK | 50 | 6 |
| 10 | 100 | 100 |  | QPSK | QPSK | 10 | 10 |
| 11 | 100 | 100 |  | 16QAM | 16QAM | 100 | 100 |
| 12 | 100 | 100 |  | 16QAM | QPSK | 100 | 10 |
| 13 | 6 | 25 |  | QPSK | QPSK | 5 | 6 |
| 14 | 6 | 25 |  | 16QAM | 16QAM | 6 | 25 |
| 15 | 6 | 25 |  | 16QAM | QPSK | 6 | 6 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. Only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2.4.1-6: Test Configuration Table (network signalled value NS_03/NS_06)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_06) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 15 | 25 | N/A for this test | QPSK | QPSK | 4 | 8 |
| 2 | 15 | 25 |  | 16QAM | 16QAM | 15 | 25 |
| 3 | 15 | 25 |  | 16QAM | QPSK | 15 | 8 |
| 4 | 25 | 15 |  | QPSK | QPSK | 6 | 4 |
| 5 | 25 | 15 |  | 16QAM | 16QAM | 25 | 15 |
| 6 | 25 | 15 |  | 16QAM | QPSK | 25 | 4 |
| 7 | 25 | 25 |  | QPSK | QPSK | 6 | 8 |
| 8 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 9 | 25 | 25 |  | 16QAM | QPSK | 25 | 8 |
| 10 | 25 | 50 |  | QPSK | QPSK | 6 | 12 |
| 11 | 25 | 50 |  | 16QAM | 16QAM | 25 | 50 |
| 12 | 25 | 50 |  | 16QAM | QPSK | 25 | 12 |
| 13 | 50 | 50 |  | QPSK | QPSK | 6 | 12 |
| 14 | 50 | 50 |  | 16QAM | 16QAM | 50 | 50 |
| 15 | 50 | 50 |  | 16QAM | QPSK | 50 | 12 |
| 16 | 100 | 50 |  | QPSK | QPSK | 10 | 12 |
| 17 | 100 | 50 |  | 16QAM | 16QAM | 100 | 50 |
| 18 | 100 | 50 |  | 16QAM | QPSK | 100 | 12 |
| 19 | 75 | 25 |  | QPSK | QPSK | 8 | 8 |
| 20 | 75 | 25 |  | 16QAM | 16QAM | 75 | 25 |
| 21 | 75 | 25 |  | 16QAM | QPSK | 75 | 8 |
| 22 | 6 | 25 |  | QPSK | QPSK | 5 | 8 |
| 23 | 6 | 25 |  | 16QAM | 16QAM | 6 | 25 |
| 24 | 6 | 25 |  | 16QAM | QPSK | 6 | 8 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. Only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2.4.1-7: Test Configuration Table (network signalled value NS_03/NS_07)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_07) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 50 | N/A for this test | QPSK | QPSK | 6 | 6@13 |
| 2 | 25 | 50 |  | 16QAM | 16QAM | 25 | 50 |
| 3 | 25 | 50 |  | QPSK | 16QAM | 6 | 50 |
| 4 | 50 | 50 |  | QPSK | QPSK | 6 | 6@13 |
| 5 | 50 | 50 |  | 16QAM | 16QAM | 50 | 50 |
| 6 | 50 | 50 |  | QPSK | 16QAM | 6 | 50 |
| 7 | 100 | 50 |  | QPSK | QPSK | 10 | 6@13 |
| 8 | 100 | 50 |  | 16QAM | 16QAM | 100 | 50 |
| 9 | 100 | 50 |  | QPSK | 16QAM | 10 | 50 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. Only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2.4.1-8: Test Configuration Table (network signalled value NS_03/NS_21)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_21) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 6 | 8 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 3 | 25 | 25 |  | 16QAM | QPSK | 25 | 8 |
| 4 (Note 4) | 100 | 50 |  | QPSK | QPSK | 10 | 12@7 |
| 5 (Note 5) | 100 | 50 |  | QPSK | QPSK | 10 | 12@31 |
| 6 | 100 | 50 |  | 16QAM | 16QAM | 100 | 1 |
| 7 | 100 | 50 |  | QPSK | 16QAM | 10 | 1 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.6.2.2A.2.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBmax-LCRB+1. Note 4: Applicable only to Low range frequency testing. Note 5: Applicable only to High range frequency testing. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2.4.1-9: Test Configuration Table (network signalled value NS_06/NS_21)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_06) | SCC (NS_21) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 8 | 8 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 3 | 25 | 25 |  | 16QAM | QPSK | 25 | 8 |
| 4 (Note 4) | 50 | 50 |  | QPSK | QPSK | 12 | 12@7 |
| 5 (Note 5) | 50 | 50 |  | QPSK | QPSK | 12 | 12@31 |
| 6 | 50 | 50 |  | 16QAM | 16QAM | 50 | 1 |
| 7 | 50 | 50 |  | QPSK | 16QAM | 12 | 1 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.6.2.2A.2.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBmax-LCRB+1. Note 4: Applicable only to Low range frequency testing. Note 5: Applicable only to High range frequency testing. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2.4.1-10: Test Configuration Table (network signalled value NS_21/NS_27)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_21) | SCC (NS_27) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 8 | 8 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 3 | 25 | 25 |  | 16QAM | QPSK | 25 | 8 |
| 4 (Note 4) | 50 | 100 |  | QPSK | QPSK | 12@7 | 20 |
| 5 (Note 5) | 50 | 100 |  | QPSK | QPSK | 12@31 | 20 |
| 6 | 50 | 100 |  | 16QAM | 16QAM | 50 | 20 |
| 7 | 50 | 100 |  | QPSK | 16QAM | 12 | 20 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.2.4A.2.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBend-LCRB+1. Note 4: Applicable only to Low range frequency testing. Note 5: Applicable only to High range frequency testing. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.2A.2.4.1-2 to 6.6.2.2A.2.4.1-9.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.2A.2.4.3.

6.6.2.2A.2.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.2.2A.2.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.2A.2.4.1-2 to 6.6.2.2A.2.4.1-9 on both PCC and SCC. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the mean power over all component carriers in the CA configuration of the radio access mode, which shall meet the requirements described in Tables 6.2.4A.2.5-2 to 6.2.4A.2.5-12. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

7.	Measure the power of the transmitted signal for PCC if NS-value other than NS_01 with a measurement filter of bandwidths according to tables 6.6.2.2A.2.5.1-1 to 6.6.2.2A.2.5.1-4 as applicable and using a rms detector. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

8.	Measure the power of the transmitted signal for SCC if NS-value other than NS_01 with a measurement filter of bandwidths according to tables 6.6.2.2A.2.5.1-1 to 6.6.2.2A.2.5.1-4 as applicable and using a rms detector. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.2A.2.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6. The following exceptions apply for each network signalled value. These exceptions indicate that the UE shall meet the additional spectrum emission requirement for a specific deployment scenario.

6.6.2.2A.2.4.3.1	Message contents exceptions (network signalled value " NS_03" on PCC)

1.	Information element additionalSpectrumEmission is set to NS_03 for PCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.6.2.2A.2.4.3.1-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_03" on PCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_03) |  |  |

6.6.2.2A.2.4.3.2	Message contents exceptions (network signalled value " NS_03" on SCC)

1.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_03 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.6.2.2A.2.4.3.2-1: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_03" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 3 (NS_03) |  |  |

6.6.2.2A.2.4.3.3	Message contents exceptions (network signalled value " NS_06" on SCC)

1.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_06 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.6.2.2A.2.4.3.3-1: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_06" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 6 (NS_06) |  |  |

6.6.2.2A.2.4.3.4	Message contents exceptions (network signalled value " NS_07" on SCC)

1.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_07 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.6.2.2A.1.4.3.4-1: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_07" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 7 (NS_07) |  |  |

6.6.2.2A.2.4.3.5	Message contents exceptions (network signalled value " NS_21" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_21 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.6.2.2A.2.4.3.5-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_21" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 21 (NS_21) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_21 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.6.2.2A.2.4.3.5-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_21" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 21 (NS_21) |  |  |

6.6.2.2A.2.4.3.6	Message contents exceptions (network signalled value " NS_21" on PCC)

1.	Information element additionalSpectrumEmission is set to NS_21 for PCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.6.2.2A.2.4.3.6-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_21" on PCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 21 (NS_21) |  |  |

6.6.2.2A.2.4.3.7	Message contents exceptions (network signalled value " NS_27" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_27 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.6.2.2A.2.4.3.7-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_27" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 27 (NS_27) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_27 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.6.2.2A.2.4.3.7-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_27" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 27 (NS_27) |  |  |

6.6.2.2A.2.5	Test Requirements

The measured UE mean power with the aggregated channel bandwidth as specified in clause 5.4.2A, derived in step 6 , shall fulfil requirements in Tables 6.2.4A.2.5-1 to 6.2.4A.2.5-12 as appropriate,

and

The power of any UE emission per component carrier derived in step 7 and 8 shall not exceed the levels specified in Tables 6.6.2.2A.2.5.1-1 to 6.6.2.2A.2.5.1-4.

Table 6.6.2.2A.2.5-1: Additional requirements (network signalled value "NS_03", “NS_11”, “NS_20” and “NS_21”), E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 |  |  |  |  | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_03 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2.5-2: Additional requirements (network signalled value "NS_03", “NS_11”, “NS_20” and “NS_21”), 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.2 | -11.2 | -13.2 | -16.2 | -18.2 | -19.2 | 30 kHz |
| 1-2.5 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 |  |  |  |  | 1 MHz |
| 6-10 |  |  | -23.2 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.2 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.2 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_03 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2.5-3: Additional requirements (network signalled value "NS_06" or "NS_07"), E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | Measurement bandwidth |
| 0-0.1 | -11.5 | -11.5 | -13.5 | -16.5 | 30 kHz |
| 0.1-1 | -11.5 | -11.5 | -11.5 | -11.5 | 100 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 |  |  | 1 MHz |
| 6-10 |  |  | -23.5 |  | 1 MHz |
| 10-15 |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.085 MHz. The first and last measurement position with a 100 kHz filter is at ΔfOOB equals to 0.15 MHz and 0.95 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_06 and NS_07 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |

Table 6.6.2.2A.2.5-4: Additional requirements (network signalled value "NS_06" or "NS_07"), 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | Measurement bandwidth |
| 0-0.1 | -11.2 | -11.2 | -13.2 | -16.2 | 30 kHz |
| 0.1-1 | -11.2 | -11.2 | -11.2 | -11.2 | 100 kHz |
| 1-2.5 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 |  |  | 1 MHz |
| 6-10 |  |  | -23.2 |  | 1 MHz |
| 10-15 |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.085 MHz. The first and last measurement position with a 100 kHz filter is at ΔfOOB equals to 0.15 MHz and 0.95 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel. Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_06 and NS_07 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |

Table 6.6.2.2A.2.5-5: Additional requirements (network signalled value “NS_27” or “NS_43”), 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -11.2 | -11.2 | -11.2 | -11.2 | Note 6 |
| 1-10 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 10-15 |  | -23.2 | -23.2 | -23.2 | 1 MHz |
| 15-20 |  |  | -23.2 | -23.2 | 1 MHz |
| 20-25 |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.085 MHz. The first and last measurement position with a 100 kHz filter is at ΔfOOB equals to 0.15 MHz and 0.95 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_06 and NS_07 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. Note 6: The measurement bandwidth is 1% of the applicable E-UTRA channel bandwidth. |  |  |  |  |  |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

## << Unchanged sections omitted >>

6.6.3.1A.2	Transmitter Spurious emissions for CA (inter-band DL CA and UL CA)

6.6.3.1A.2.1	Test purpose

To verify that transmitter of UE that support CA does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

6.6.3.1A.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support inter-band DL CA and UL CA. The requirements also apply to all types of E-UTRA UE release 14 and forward that support uplink LAA.

6.6.3.1A.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.3.1A.0.

6.6.3.1A.2.4	Test description

6.6.3.1A.2.4.1	Initial condition

Same as in clause 6.6.3.1A.1.4.1 with the following exceptions:

-	Instead of Table 5.4.2A.1-1 use Table 5.4.2A.1-2 for CA configurations with two bands or Table 5.4.2A.1-2a for CA configurations with three bands.

-	Instead of Table 6.6.3.1A.1.4.1-1  use Table 6.6.3.1A.2.4.1-1 or Table 6.6.3.1A.2.4.1-2.

-	Instead of clause 6.6.3.1A.1.4.3 use clause 6.6.3.1A.2.4.3.

Table 6.6.3.1A.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 6 | 25 | N/A for this test | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 6 | 25 |  | QPSK | 2 | P_1@5 | S_1@24 | - | - |
| 6 | 25 |  | QPSK | 31 | P_6@0 | S_25@0 | - | - |
| 6 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 6 | 50 |  | QPSK | 2 | P_1@5 | S_1@49 | - | - |
| 6 | 50 |  | QPSK | 56 | P_6@0 | S_50@0 | - | - |
| 25 | 15 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 15 |  | QPSK | 2 | P_1@24 | S_1@14 | - | - |
| 25 | 15 |  | QPSK | 40 | P_25@0 | S_15@0 | - | - |
| 25 | 25 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 25 |  | QPSK | 2 | P_1@24 | S_1@24 | - | - |
| 25 | 25 |  | QPSK | 50 | P_25@0 | S_25@0 | - | - |
| 25 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 50 |  | QPSK | 2 | P_1@24 | S_1@49 | - | - |
| 25 | 50 |  | QPSK | 75 | P_25@0 | S_50@0 | - | - |
| 50 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 50 | 50 |  | QPSK | 2 | P_1@49 | S_1@49 | - | - |
| 50 | 50 |  | QPSK | 100 | P_50@0 | S_50@0 | - | - |
| 50 | 100 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 50 | 100 |  | QPSK | 2 | P_1@49 | S_1@99 | - | - |
| 50 | 100 |  | QPSK | 150 | P_50@0 | S_100@0 | - | - |
| 75 | 25 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 75 | 25 |  | QPSK | 2 | P_1@74 | S_1@24 | - | - |
| 75 | 25 |  | QPSK | 100 | P_75@0 | S_25@0 | - | - |
| 75 | 75 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 75 | 75 |  | QPSK | 2 | P_1@74 | S_1@74 | - | - |
| 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 | - | - |
| 100 | 75 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 75 |  | QPSK | 2 | P_1@99 | S_1@74 | - | - |
| 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 | - | - |
| 100 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 50 |  | QPSK | 2 | P_1@99 | S_1@49 | - | - |
| 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 | - | - |
| 100 | 100 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 100 |  | QPSK | 2 | P_1@99 | S_1@99 | - | - |
| 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |

Table 6.6.3.1A.2.4.1-2: Test Configuration Table for UL LAA

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC RB Allocation (LCRB @ RBstart) SCC RB Allocations(L@ RBstart) (Note 3) |  |  |  |
| 6 | 100 | N/Afor this test | QPSK | 11 | P_1@0 | S_1@0 | - | - |
| 6 | 100 |  | QPSK | 11 | P_1@5 | S_1@9 | - | - |
| 6 | 100 |  | QPSK | 106 | P_6@0 | S_10@0 | - | - |
| 15 | 100 |  | QPSK | 11 | P_1@0 | S_1@0 | - | - |
| 15 | 100 |  | QPSK | 11 | P_1@14 | S_1@9 | - | - |
| 15 | 100 |  | QPSK | 115 | P_15@0 | S_10@0 | - | - |
| 25 | 100 |  | QPSK | 11 | P_1@0 | S_1@0 | - | - |
| 25 | 100 |  | QPSK | 11 | P_1@24 | S_1@9 | - | - |
| 25 | 100 |  | QPSK | 125 | P_25@0 | S_10@0 | - | - |
| 50 | 100 |  | QPSK | 11 | P_1@0 | S_1@0 | - | - |
| 50 | 100 |  | QPSK | 11 | P_1@49 | S_1@9 | - | - |
| 50 | 100 |  | QPSK | 150 | P_50@0 | S_10@0 | - | - |
| 75 | 100 |  | QPSK | 11 | P_1@0 | S_1@0 | - | - |
| 75 | 100 |  | QPSK | 11 | P_1@74 | S_1@9 | - | - |
| 75 | 100 |  | QPSK | 175 | P_75@0 | S_10@0 | - | - |
| 100 | 100 |  | QPSK | 11 | P_1@0 | S_1@0 | - | - |
| 100 | 100 |  | QPSK | 11 | P_1@99 | S_1@9 | - | - |
| 100 | 100 |  | QPSK | 200 | P_100@0 | S_10@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. Note 3: For eLAA SCC, the RBstart and L are defined in TS 36.213[10] clause 8.1.4. |  |  |  |  |  |  |  |  |

6.6.3.1A.2.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2. The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.3.1A.2.4.3

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 (or DCI format 0A for SCC in the case of uplink LAA) for C_RNTI to schedule the UL RMC according to Table 6.6.3.1A.2.4.1-1 (or Table 6.6.3.1A.2.4.1-2 for SCC in the case of uplink LAA) on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control “up” commands in every uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.6.3.1A.2.5-1 for each component carrier. The centre frequency of the filter shall be stepped in contiguous steps according to Table 6.6.3.1A.2.5-1. The measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS.

6.6.3.1A.2.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6 with exceptions as in clause 6.6.3.1A.1.4.3.

For SCC in the case of uplink LAA, message contents are according to TS 36.508[7] subclause4.6 with the following exceptions specified in Tables 6.6.3.1A.1.2.4.3-1 and 6.6.3.1A.1.2.4.3-2.

Table 6.6.3.1A.1.2.4.3-1 LogicalChannelConfig-DRB (Initial Condition, step 14, Table 4.5.2.3-1)

| Derivation Path: 36.508 Table 4.8.2.1.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LogicalChannelConfig-DRB ::= SEQUENCE { |  |  |  |  |
| ul-SpecificParameters SEQUENCE { |  |  |  |  |
| laa-UL-Allowed-r14 |  | true |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.6.3.1A.1.2.4.3-2 PhysicalConfigDedicatedSCell (Step 2, test procedure)

| Derivation Path: 36.508 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| laa-SCellConfiguration-v1430 ::= SEQUENCE { |  |  |  |
|  | NULL |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.3.1A.2.5	Test requirement

This clause specifies the requirements for the specified E-UTRA band for Transmitter Spurious emissions requirement with frequency range as indicated in table 6.6.3.1A.2.5-1.

For frequencies ΔfOOB greater than FOOB as specified in Table 6.6.3.1A.0-1the measured average power of spurious emission, derived in step 6, shall not exceed the described value in Table 6.6.3.1A.2.5-1.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 6.6.3.1A.2.5-1: Spurious emissions limits

| Frequency Range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| Test requirements for CA_1A-3A Configuration |  |  |  |
| 135 MHz £ f £ 270 MHz | -36 dBm | 100 kHz |  |
| 1440 MHz £ f £ 1650 MHz 2055 MHz £ f £ 2250 MHz 3630 MHz £ f £ 3765 MHz 5340 MHz £ f £ 5745 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_1A-5A Configuration |  |  |  |
| 222 MHz £ f £ 332 MHz | -36 dBm | 100 kHz |  |
| 1071 MHz £ f £ 1156 MHz 2744 MHz £ f £ 2829 MHz 2991 MHz £ f £ 3136 MHz 3568 MHz £ f £ 3678 MHz 4664 MHz £ f £ 4809 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_1A-7A Configuration |  |  |  |
| 520 MHz £ f £ 650 MHz | -36 dBm | 100 kHz |  |
| 1270 MHz £ f £ 1460 MHz 3020 MHz £ f £ 3220 MHz 4420 MHz £ f £ 4550 MHz 6340 MHz £ f £ 6530 MHz 6920 MHz £ f £ 7120 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_1A-8A Configuration |  |  |  |
| 90 MHz £ f £ 220 MHz | -36 dBm | 100 kHz |  |
| 1005 MHz £ f £ 1100 MHz 2800 MHz £ f £ 2895 MHz 2925 MHz £ f £ 3080 MHz 3680 MHz £ f £ 3810 MHz 4720 MHz £ f £ 4875 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_1A-18A Configuration |  |  |  |
| 260 MHz £ f £ 350 MHz | -36 dBm | 100 kHz |  |
| 1090 MHz £ f £ 1165 MHz 2735 MHz £ f £ 2810 MHz 3010 MHz £ f £ 3145 MHz 3550 MHz £ f £ 3640 MHz 4655 MHz £ f £ 4790 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_1A-19A Configuration |  |  |  |
| 230 MHz £ f £ 320 MHz | -36 dBm | 100 kHz |  |
| 1075 MHz £ f £ 1150 MHz 2750 MHz £ f £ 2825 MHz 2995 MHz £ f £ 3130 MHz 3580 MHz £ f £ 3670 MHz 4670 MHz £ f £ 4805 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_1A-21A Configuration |  |  |  |
| 457.1 MHz £ f £ 532.1 MHz 915.8 MHz £ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz £ f £ 1005.8 MHz 2377.1 MHz £ f £ 2512.1 MHz 3367.9 MHz £ f £ 3442.9 MHz 4515.8 MHz £ f £ 4905.8 MHz 5287.9 MHz £ f £ 5422.9 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_1A-26A Configuration |  |  |  |
| 222 MHz £ f £ 352 MHz | -36 dBm | 100 kHz |  |
| 1071 MHz £ f £ 1166 MHz 2734 MHz £ f £ 2829 MHz 2991 MHz £ f £ 3146 MHz 3548 MHz £ f £ 3678 MHz 4654 MHz £ f £ 4809 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_1A-28A Configuration |  |  |  |
| 424 MHz £ f £ 574 MHz | -36 dBm | 100 kHz |  |
| 1172 MHz £ f £ 1277 MHz 2623 MHz £ f £ 2728 MHz 3092 MHz £ f £ 3257 MHz 3326 MHz £ f £ 3476 MHz 4543 MHz £ f £ 4708 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_1A-42A Configuration |  |  |  |
| 240 MHz £ f £ 560 MHz | -36 dBm | 100 kHz |  |
| 1420 MHz £ f £ 1680 MHz 4820 MHz £ f £ 5280 MHz 5320 MHz £ f £ 5580 MHz 7240 MHz £ f £ 7560 MHz 8720 MHz £ f £ 9180 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_2A-4A Configuration |  |  |  |
| 95 MHz £ f £ 200 MHz | -36 dBm | 100 kHz |  |
| 1510 MHz £ f £ 1660 MHz 1945 MHz £ f £ 2110 MHz 3560 MHz £ f £ 3665 MHz 5270 MHz £ f £ 5575 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_2A-5A Configuration |  |  |  |
| 152 MHz £ f £ 262 MHz | -36 dBm | 100 kHz |  |
| 1001 MHz £ f £ 1086 MHz 2674 MHz £ f £ 2759 MHz 2851 MHz £ f £ 2996 MHz 3498 MHz £ f £ 3608 MHz 4524 MHz £ f £ 4669 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_2A-7A Configuration |  |  |  |
| 590 MHz £ f £ 720 MHz | -36 dBm | 100 kHz |  |
| 1130 MHz £ f £ 1320 MHz 3090 MHz £ f £ 3290 MHz 4350 MHz £ f £ 4480 MHz 6200 MHz £ f £ 6390 MHz 6850 MHz £ f £ 7050 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_2A-12A Configuration |  |  |  |
| 418 MHz £ f £ 512 MHz | -36 dBm | 100 kHz |  |
| 1134 MHz £ f £ 1211 MHz 2549 MHz £ f £ 2626 MHz 2984 MHz £ f £ 3121 MHz 3248 MHz £ f £ 3342 MHz 4399 MHz £ f £ 4536 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_2A-13A Configuration |  |  |  |
| 276 MHz £ f £ 356 MHz | -36 dBm | 100 kHz |  |
| 1063 MHz £ f £ 1133 MHz 2627 MHz £ f £ 2697 MHz 2913 MHz £ f £ 3043 MHz 3404 MHz £ f £ 3484 MHz 4477 MHz £ f £ 4607 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_2A-30A Configuration |  |  |  |
| 395 MHz £ f £ 465 MHz | -36 dBm | 100 kHz |  |
| 1385 MHz £ f £ 1515 MHz 2700 MHz £ f £ 2780 MHz 4155 MHz £ f £ 4225 MHz 6005 MHz £ f £ 6135 MHz 6460 MHz £ f £ 6540 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_2A-46A Configuration |  |  |  |
| 1330 MHz ≤ f ≤ 2225 MHz 3240 MHz ≤ f ≤ 4075 MHz 7000 MHz ≤ f ≤ 7835 MHz 8390 MHz ≤ f ≤ 10000 MHz 12150 MHz ≤ f ≤ 13760 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_2A-66A Configuration |  |  |  |
| 70 MHz £ f £ 200 MHz | -36 dBm | 100 kHz |  |
| 1510 MHz £ f £ 1710 MHz 1920 MHz £ f £ 2110 MHz 3560 MHz £ f £ 3690 MHz 5270 MHz £ f £ 5600 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_3A-5A Configuration |  |  |  |
| 12 MHz £ f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz £ f £ 137 MHz 861 MHz £ f £ 961 MHz | -36 dBm | 100 kHz |  |
| 2534 MHz £ f £ 2746 MHz 3358 MHz £ f £ 3483 MHz 4244 MHz £ f £ 4419 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_3A-7A Configuration |  |  |  |
| 715 MHz £ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz £ f £ 1070 MHz 3215 MHz £ f £ 3430 MHz 4210 MHz £ f £ 4355 MHz 5920 MHz £ f £ 6140 MHz 6710 MHz £ f £ 6925 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_3A-8A Configuration |  |  |  |
| 25 MHz £ f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz £ f £ 120 MHz 795 MHz £ f £ 905 MHz | -36 dBm | 100 kHz |  |
| 2505 MHz £ f £ 2700 MHz 3470 MHz £ f £ 3615 MHz 4300 MHz £ f £ 4485 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_3A-18A Configuration |  |  |  |
| 50 MHz £ f £ 155 MHz 880 MHz £ f £ 970 MHz | -36 dBm | 100 kHz |  |
| 2525 MHz £ f £ 2755 MHz 3340 MHz £ f £ 3445 MHz 4235 MHz £ f £ 4400 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_3A-19A Configuration |  |  |  |
| 20 MHz £ f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz £ f £ 125 MHz 865 MHz £ f £ 955 MHz | -36 dBm | 100 kHz |  |
| 2540 MHz £ f £ 2740 MHz 3370 MHz £ f £ 3475 MHz 4250 MHz £ f £ 4415 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_3A-20A Configuration |  |  |  |
| 9 kHz £ f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz £ f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz £ f £ 121 MHz 848 MHz £ f £ 953 MHz | -36 dBm | 100 kHz |  |
| 2542 MHz £ f £ 2738 MHz 3374 MHz £ f £ 3509 MHz 4252 MHz £ f £ 4432 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_3A-26A Configuration |  |  |  |
| 12 MHz £ f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz £ f £ 157 MHz | -36 dBm | 100 kHz |  |
| 861 MHz £ f £ 971 MHz | -36 dBm | 100 kHz |  |
| 2524 MHz £ f £ 2756 MHz 3338 MHz £ f £ 3483 MHz 4234 MHz £ f £ 4419 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_3A-28A Configuration |  |  |  |
| 214 MHz £ f £ 379 MHz 962 MHz £ f £ 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz £ f £ 1082 MHz 2413 MHz £ f £ 2533 MHz 2672 MHz £ f £ 2867 MHz 3116 MHz £ f £ 3281 MHz 4123 MHz £ f £ 4318 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_3A-41A Configuration |  |  |  |
| 711 MHz £ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz £ f £ 1074 MHz 3207 MHz £ f £ 3670 MHz 4206 MHz £ f £ 4475 MHz 5916 MHz £ f £ 6260 MHz 6702 MHz £ f £ 7165 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_3A-42A Configuration |  |  |  |
| 170 MHz £ f £ 180 MHz | -36 dBm | 100 kHz |  |
| 1615 MHz £ f £ 1890 MHz 5015 MHz £ f £ 5490 MHz 6820 MHz £ f £ 7170 MHz 8510 MHz £ f £ 8985 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_4A-5A Configuration |  |  |  |
| 12 MHz £ f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz £ f £ 107 MHz 861 MHz £ f £ 931 MHz | -36 dBm | 100 kHz |  |
| 2534 MHz £ f £ 2686 MHz 3358 MHz £ f £ 3453 MHz 4244 MHz £ f £ 4359 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_4A-7A Configuration |  |  |  |
| 745 MHz £ f £ 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz £ f £ 1010 MHz 3245 MHz £ f £ 3430 MHz 4210 MHz £ f £ 4325 MHz 5920 MHz £ f £ 6080 MHz 6710 MHz £ f £ 6895 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_4A-12A Configuration |  |  |  |
| 278 MHz £ f £ 357 MHz 994 MHz £ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz £ f £ 1056 MHz 2409 MHz £ f £ 2471 MHz 2704 MHz £ f £ 2811 MHz 3108 MHz £ f £ 3187 MHz 4119 MHz £ f £ 4226 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_4A-13A Configuration |  |  |  |
| 136 MHz £ f £ 201 MHz 923 MHz £ f £ 978 MHz | -36 dBm | 100 kHz |  |
| 2487 MHz £ f £ 2542 MHz 2633 MHz £ f £ 2733 MHz 3264 MHz £ f £ 3329 MHz 4197 MHz £ f £ 4297 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_4A-17A Configuration |  |  |  |
| 278 MHz £ f £ 347 MHz 994 MHz £ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz £ f £ 1056 MHz 2414 MHz £ f £ 2471 MHz 2704 MHz £ f £ 2806 MHz 3118 MHz £ f £ 3187 MHz 4124 MHz £ f £ 4226 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_5A-7A Configuration |  |  |  |
| 802 MHz £ f £ 922 MHz | -36 dBm | 100 kHz |  |
| 1651 MHz £ f £ 1746 MHz 3324 MHz £ f £ 3419 MHz 4148 MHz £ f £ 4316 MHz 5824 MHz £ f £ 5989 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_5A-12A Configuration |  |  |  |
| 108 MHz ≤ f ≤ 150 MHz 549 MHz ≤ f ≤ 608 MHz 932 MHz ≤ f ≤ 999 MHz | -36 dBm | 100 kHz |  |
| 1523 MHz ≤ f ≤ 1565 MHz 2222 MHz ≤ f ≤ 2281 MHz 2347 MHz ≤ f ≤ 2414 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_5A-17A Configuration |  |  |  |
| 108 MHz £ f £ 145 MHz 559 MHz £ f £ 608 MHz 932 MHz £ f £ 99e4 MHz | -36 dBm | 100 kHz |  |
| 1528 MHz £ f £ 1565 MHz 2232 MHz £ f £ 2281 MHz 2352 MHz £ f £ 2414 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_5A-30A Configuration |  |  |  |
| 607 MHz £ f £ 667 MHz | -36 dBm | 100 kHz |  |
| 1456 MHz £ f £ 1491 MHz 3129 MHz £ f £ 3164 MHz 3761 MHz £ f £ 3806 MHz 3953 MHz £ f £ 4013 MHz 5434 MHz £ f £ 5479 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_5A-66A Configuration |  |  |  |
| 12 MHz £ f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz £ f £ 132 MHz 861 MHz £ f £ 956 MHz | -36 dBm | 100 kHz |  |
| 2534 MHz £ f £ 2736 MHz 3358 MHz £ f £ 3478 MHz 4244 MHz £ f £ 4409 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_7A-20A Configuration |  |  |  |
| 776 MHz £ f £ 906 MHz | -36 dBm | 100 kHz |  |
| 1638 MHz £ f £ 1738 MHz 3332 MHz £ f £ 3432 MHz 4138 MHz £ f £ 4308 MHz 5832 MHz £ f £ 6002 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_7A-28A Configuration |  |  |  |
| 1004 MHz £ f £ 1164 MHz 1752 MHz £ f £ 1867 MHz 3203 MHz £ f £ 3318 MHz 3906 MHz £ f £ 4066 MHz 4252 MHz £ f £ 4437 MHz 5703 MHz £ f £ 5888 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_11A-18A Configuration |  |  |  |
| 182.1 MHz £ f £ 232.1 MHz 597.9 MHz £ f £ 632.9 MHz | -36 dBm | 100 kHz |  |
| 2025.8 MHz £ f £ 2080.8 MHz 2242.9 MHz £ f £ 2277.9 MHz 3057.9 MHz £ f £ 3107.9 MHz 3670.8 MHz £ f £ 3725.8 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_12A-30A Configuration |  |  |  |
| 873 MHz £ f £ 917 MHz | -36 dBm | 100 kHz |  |
| 1589 MHz £ f £ 1616 MHz 3004 MHz £ f £ 3031 MHz 3703 MHz £ f £ 3747 MHz 3894 MHz £ f £ 3931 MHz 5009 MHz £ f £ 5046 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_18A-28A Configuration |  |  |  |
| 82 MHz £ f £ 127 MHz 576 MHz £ f £ 651 MHz 897 MHz £ f £ 957 MHz | -36 dBm | 100 kHz |  |
| 1518 MHz £ f £ 1563 MHz 2221 MHz £ f £ 2296 MHz 2333 MHz £ f £ 2393 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_19A-21A Configuration |  |  |  |
| 197.1 MHz £ f £ 242.1 MHz 602.9 MHz £ f £ 632.9 MHz | -36 dBm | 100 kHz |  |
| 2050.8 MHz £ f £ 2095.8 MHz 2277.9 MHz £ f £ 2307.9 MHz 3107.9 MHz £ f £ 3152.9 MHz 3725.8 MHz £ f £ 3770.8 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_26A-46A Configuration |  |  |  |
| 3542 MHz ≤ f ≤ 4297 MHz 4301 MHz ≤ f ≤ 5111 MHz 5964 MHz ≤ f ≤ 6774 MHz 6778 MHz ≤ f ≤ 7623 MHz 9541 MHz ≤ f ≤ 11036 MHz 11114 MHz ≤ f ≤ 12699 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_30A-48A Configuration |  |  |  |
| 910 MHz £ f £ 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz £ f £ 1080 MHz 1235 MHz £ f £ 1395 MHz 4785 MHz £ f £ 5095 MHz 5855 MHz £ f £ 6015 MHz 8160 MHz £ f £ 8333 MHz 9405 MHZ £ f £ 9715 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_39A-41A Configuration |  |  |  |
| 576 MHz £ f £ 810 MHz | -36 dBm | 100 kHz |  |
| 1070 MHz £ f £ 1344 MHz 3072 MHz £ f £ 3500 MHz 4376 MHz £ f £ 4610 MHz 6256 MHz £ f £ 6530 MHz 6872 MHz £ f £ 7300 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_41A-42A Configuration |  |  |  |
| 710 MHz £ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1000 MHz £ f £ 1104 MHz 1392 MHz £ f £ 1980 MHz 4110 MHz £ f £ 4704 MHz 5896 MHz £ f £ 6290 MHz 8392 MHz £ f £ 8980 MHz 9296 MHz £ f £ 9890 MHz | -30 dBm | 1 MHz |  |

## << End of changes >>