3GPP TSG-RAN5 Meeting #103	R5-242712

Fukuoka, Japan, May 20 – 24, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.905 | CR | 0273 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test point analysis for Inter-band DL CA and UL CA_30A-48A for A-MPR & Spurious emission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sporton |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Inter-band DL CA and UL CA_30A-48A is missing for A-MPR & Spurious emission test cases in TS36.905 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of the following analysis files as attachment in this CR: “TpAnalysisAMPR2UL_6.2.4A.2(NS_21+NS_27).zip” “TpAnalysisSpur(30A-48A).zip” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inter-band DL CA and CA_30A-48A will be still missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.3, 4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521-1 CR 5509 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

4.1.3	A-MPR test case for inter-band UL CA

This section contains information on test point selection for test case 6.2.4A.2 in [2], Additional Maximum Power Reduction (A-MPR) for CA (inter-band DL CA and UL CA).

TS 36.101 [3] specifies band dependent NS-values, which in the inter-band UL CA test cases become a combination of two NS-values. Testing all possible combinations would lead to too excessive testing and the combinations that are realistic should therefore be prioritized. This selection is documented in table 4.1.3-1.

Table 4.1.3-1: A-MPR test coverage per CA configuration for inter-band CA with 2 CC

| CA config with UL CA support (Note 1) | NS values in same order as Uplink CA Configuration column |  | Applicable test case | Comment/Justification |
| --- | --- | --- | --- | --- |
| CA_1A-3A | NS_05 | NS_01 | 6.6.3.3A.2 | Note 3 |
| CA_1A-5A | NS_05 | NS_01 | 6.6.3.3A.2 | Note 3 |
| CA_1A-7A | NS_05 | NS_01 | 6.6.3.3A.2 | Note 3 |
| CA_1A-8A | NS_05 | NS_01 | 6.6.3.3A.2 | Note 3 |
| CA_1A-18A | NS_05 | NS_01 | 6.6.3.3A.2 | Note 3 |
| CA_1A-19A | NS_05 | NS_08 | 6.6.3.3A.2 | Note 3 |
| CA_1A-21A | NS_05 | NS_09 | 6.6.3.3A.2 | Note 3 |
| CA_1A-26A | NS_05 | NS_01 | 6.6.3.3A.2 | Note 3 |
| CA_1A-28A | NS_05 | NS_17 | 6.6.3.3A.2 |  |
| CA_1A-42A | NS_05 | NS_01 | 6.6.3.3A.2 | Note 3 |
| CA_2A-4A | NS_03 | NS_03 | 6.6.2.2A.2 |  |
| CA_2A-5A | NS_03 | NS_01 | 6.6.2.2A.2 |  |
| CA_2A-7A | NS_03 | NS_01 | 6.6.2.2A.2 |  |
| CA_2A-12A | NS_03 | NS_06 | 6.6.2.2A.2 |  |
| CA_2A-13A | NS_03 | NS_06 | 6.6.2.2A.2 |  |
| CA_2A-13A | NS_03 | NS_07 | 6.6.2.2A.2, 6.6.3.3A.2 | Note 3 |
| CA_2A-30A | NS_03 | NS_21 | 6.6.2.2A.2, 6.6.3.3A.2 | Note 5 |
| CA_2A-66A | NS_03 | NS_03 | 6.6.2.2A.2 |  |
| CA_3A-5A | NS_01 | NS_01 | N/A | Note 2 |
| CA_3A-7A | NS_01 | NS_01 | N/A | Note 2 |
| CA_3A-8A | NS_01 | NS_01 | N/A | Note 2 |
| CA_3A-18A | NS_01 | NS_01 | N/A | Note 2 |
| CA_3A-19A | NS_01 | NS_08 | 6.6.3.3A.2 |  |
| CA_3A-20A | NS_01 | NS_10 | N/A | Note 4 |
| CA_3A-26A | NS_01 | NS_12, NS_13, NS_14, NS_15 | 6.6.3.3A.2 | Note 3 |
| CA_3A-28A | NS_01 | NS_17, NS_18 | N/A | Note 3 |
| CA_3A-41A | NS_01 | NS_04 |  |  |
| CA_4A-5A | NS_03 | NS_01 | 6.6.2.2A.2 |  |
| CA_4A-7A | NS_03 | NS_01 | 6.6.2.2A.2 |  |
| CA_4A-12A | NS_03 | NS_06 | 6.6.2.2A.2 |  |
| CA_4A-13A | NS_03 | NS_06 | 6.6.2.2A.2 |  |
| CA_4A-13A | NS_03 | NS_07 | 6.6.2.2A.2, 6.6.3.3A.2 | Note 3 |
| CA_4A-17A | NS_03 | NS_06 | 6.6.2.2A.2 |  |
| CA_5A-7A | NS_01 | NS_01 | N/A | Note 2 |
| CA_5A-12A | NS_01 | NS_06 | 6.6.2.2A.2 |  |
| CA_5A-17A | NS_01 | NS_06 | 6.6.2.2A.2 |  |
| CA_5A-30A | NS_01 | NS_21 | 6.6.2.2A.2, 6.6.3.3A.2 | Note 5 |
| CA_5A-66A | NS_01 | NS_03 | 6.6.2.2A.2 |  |
| CA_7A-20A | NS_01 | NS_10 | N/A |  |
| CA_7A-28A | NS_01 | NS_17 | 6.6.3.3A.2 | Note 3 |
| CA_7A-28A | NS_01 | NS_18 | 6.6.3.3A.2 | Note 3 |
| CA_11A-18A | NS_01 | NS_01 | N/A | Note 2 |
| CA_12A-30A | NS_06 | NS_21 | 6.6.2.2A.2, 6.6.3.3A.2 | Note 5 |
| CA_12A-66A | NS_06 | NS_03 | 6.6.2.2A.2 |  |
| CA_18A-28A | NS_01 | NS_17 | 6.6.3.3A.2 |  |
| CA_19A-21A | NS_08 | NS_09 | 6.6.3.3A.2 | Note 3 |
| CA_30A-48A | NS_21 | NS_27 | 6.6.2.2A.2, 6.6.3.3A.2 | Note 5 |
| CA_39A-41A | NS_01 | NS_01 | N/A |  |
| Note 1: As per TS 36.101 V12.10.1 Note 2: No test required since band combinations where only NS_01 is possible have no additional requirements, covered by section 6.2.3 test cases in TS 36.521-1 [2] Note 3: No test required since only A-Spur requirements apply, and the frequency range to test is unaffected by intermodulation products as described in section 4.3. Note 4: No test required since no A_SEM and/or A-Spur requirements apply. Note 5: Only A_SEM requirements apply, as the frequency range to test for A-Spur requirements is unaffected by intermodulation products as described in section 4.3. |  |  |  |  |

The analyses are performed per NS-value and are stored as zip-files as defined in annex A. The general principle for selection of test points is:

-	Test the minimum MPR + A-MPR value

-	Test the maximum MPR + A-MPR value

-	Test the maximum unbalanced total power backoff among CCs (max Pcmaxc difference).

The analyses are done for QPSK/16QAM test cases. For 64QAM test cases no analysis is made due to that it follows the same selection principle.

## << Unchanged sections omitted >>

4.3	Test points selection and Frequency ranges to cover in Tx spurious emissions test cases for UL CA

In this case, it is sufficient to verify the minimum requirements in frequency ranges affected by 2nd and 3rd order intermodulation products. The frequency ranges and UL RB allocations used in the test are calculated here.

The analyses are performed per CA configuration and are stored as zip-files as defined in annex A.

Table 4.3-1: Frequency range analysis availability per CA configuration

| CA config | File name | Comments |
| --- | --- | --- |
| CA_1A-3A | TpAnalysisSpur(1A-3A)_v4.zip | Added at RAN5#78 |
| CA_1A-5A | TpAnalysisSpur(1A-5A)_v3.zip | Added at RAN5#78 |
| CA_1A-7A | TpAnalysisSpur(1A-7A)_v3.zip | Added at RAN5#78 |
| CA_1A-8A | TpAnalysisSpur(1A-8A)_v3.zip | Added at RAN5#78 |
| CA_1A-18A | TpAnalysisSpur(1A-18A)_v2.zip | Added at RAN5#78 |
| CA_1A-19A | TpAnalysisSpur(1A-19A)_v3.zip | Added at RAN5#78 |
| CA_1A-21A | TpAnalysisSpur(1A-21A)_v3.zip | Added at RAN5#78 |
| CA_1A-26A | TpAnalysisSpur(1A-26A)_v3.zip | Added at RAN5#78 |
| CA_1A-28A | TpAnalysisSpur(1A-28A)_v3.zip | Added at RAN5#78 |
| CA_1A-42A | TpAnalysisSpur(1A-42A)_v3.zip | Added at RAN5#78 |
| CA_2A-4A | TpAnalysisSpur(2A-4A)_v4.zip | Added at RAN5#83 |
| CA_2A-5A | TpAnalysisSpur(2A-5A)_v4.zip | Added at RAN5#100 |
| CA_2A-7A | TpAnalysisSpur(2A-7A)_v2.zip | Added at RAN5#78 |
| CA_2A-12A | TpAnalysisSpur(2A-12A)_v4.zip | Added at RAN5#100 |
| CA_2A-13A | TpAnalysisSpur(2A-13A)_v5.zip | Added at RAN5#100 |
| CA_2A-30A | TpAnalysisSpur(2A-30A)_v1.zip | Added at RAN5#100 |
| CA_2A-46A | TpAnalysisSpur(2A-46A).zip | Added at RAN5#80 |
| CA_2A-48A | TpAnalysisSpur(2A-48A).zip | Added at RAN5#100 |
| CA_2A-66A | TpAnalysisSpur(2A-66A).zip | Added at RAN5#75 |
| CA_3A-5A | TpAnalysisSpur(3A-5A)_v3.zip | Added at RAN5#78 |
| CA_3A-7A | TpAnalysisSpur(3A-7A)_v3.zip | Added at RAN5#78 |
| CA_3A-8A | TpAnalysisSpur(3A-8A)_v3.zip | Added at RAN5#78 |
| CA_3A-18A | TpAnalysisSpur(3A-18A).zip | Added at RAN5#80 |
| CA_3A-19A | TpAnalysisSpur(3A-19A)_v2.zip | Added at RAN5#78 |
| CA_3A-20A | TpAnalysisSpur(3A-20A)_v3.zip | Added at RAN5#78 |
| CA_3A-26A | TpAnalysisSpur(3A-26A)_v3.zip | Added at RAN5#78 |
| CA_3A-28A | TpAnalysisSpur(3A-28A).zip | Added at RAN5#81 |
| CA_3A-41A | TpAnalysisSpur(3A-41A)_v2.zip | Added at RAN5#88-e |
| CA_3A-42A | TpAnalysisSpur(3A-42A)_v1.zip | Added at RAN5#80 |
| CA_4A-5A | TpAnalysisSpur(4A-5A)_v4.zip | Added at RAN5#83 |
| CA_4A-7A | TpAnalysisSpur(4A-7A)_v4.zip | Added at RAN5#78 |
| CA_4A-12A | TpAnalysisSpur(4A-12A)_v3.zip | Added at RAN5#78 |
| CA_4A-13A | TpAnalysisSpur(4A-13A)_v4.zip | Added at RAN5#83 |
| CA_4A-17A | TpAnalysisSpur(4A-17A)_v4.zip | Added at RAN5#83 |
| CA_5A-7A | TpAnalysisSpur(5A-7A)_v3.zip | Added at RAN5#78 |
| CA_5A-12A | TpAnalysisSpur(5A-12A)_v6.zip | Added at RAN5#100 |
| CA_5A-17A | TpAnalysisSpur(5A-17A)_v5.zip | Added at RAN5#100 |
| CA_5A-30A | TpAnalysisSpur(5A-30A)_v1.zip | Added at RAN5#100 |
| CA_5A-66A | TpAnalysisSpur(5A-66A).zip | Added at RAN5#75 |
| CA_7A-20A | TpAnalysisSpur(7A-20A)_v3.zip | Added at RAN5#78 |
| CA_7A-28A | TpAnalysisSpur(7A-28A)_v3.zip | Added at RAN5#78 |
| CA_8A-41A | TpAnalysisSpur(8A-41A).zip | Added at RAN5#100 |
| CA_11A-18A | TpAnalysisSpur(3A-28A).zip | Added at RAN5#81 |
| CA_12A-30A | TpAnalysisSpur(12A-30A)_v1.zip | Added at RAN5#100 |
| CA_18A-28A | TpAnalysisSpur(18A-28A)_v3.zip | Added at RAN5#78 |
| CA_19A-21A | TpAnalysisSpur(19A-21A)_v2.zip | Added at RAN5#72 |
| CA_26A-46A | TpAnalysisSpur(26A-46A).zip | Added at RAN5#80 |
| CA_30A-48A | TpAnalysisSpur(30A-48A).zip | Added at RAN5#103 |
| CA_39A-41A | TpAnalysisSpur(39A-41A)_v3.zip | Added at RAN5#78 |
| CA_41A-42A | TpAnalysisSpur(41A-42A)_v2.zip | Added at RAN5#78 |
| CA_41C-42C | TpAnalysisSpur(41C-42C).zip | Added at RAN5#93-e |

## << End of changes >>

The following zipfile is added to TS 36.905.

TpAnalysisAMPR2UL_6.2.4A.2(NS_21+NS_27).zip

TpAnalysisSpur(30A-48A).zip