3GPP TSG-RAN5 Meeting #103	R5-243205

# Fukuoka, Japan, May 20-24, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2833 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating AMPR testing for NS_43 for band n8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | HPUE_NR_FR1_FDD_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The PC2 AMPR testing for NS_43 is absent in the specification. The minimum requirement for NS_43 also need to be updated to align with the latest core specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating the minimum requirement for NS_43. Addding PC2 AMPR test configuration and test requirement for NS_43, and updating the PC3 test configuration table for NS_43. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NS_43 can’t be tested for PC2 UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 0912 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.2.3	UE additional maximum output power reduction

Editor’s note: The following aspects are either missing or not yet determined:

- Tests for network signalling values NS_40, NS_09 not complete.

- The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_07, NS_44, NS_46, NS_47, NS_48, and NS_49 to all types of NR Power Class 2 and 3 UE release 16 forward, and UE release 15 if the corresponding channel bandwidths are supported.

6.2.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1.3-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Outer and inner allocation notation used in clause 6.2.3 is defined in clause 6.2.2. Unless stated otherwise, Edge RB allocations get the same A-MPR as Outer RB allocations. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

6.2.3.2	Test applicability

The requirements of this test apply in test case 6.5.2.3 Additional Spectrum Emission mask for network signalling values NS_03, NS_03U, NS_04, NS_06, NS_07, NS_27 and NS_35 to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity and NR Power Class 1 UE release 15 and forward.

The requirements of this test apply in test case 6.5.2.4.2 Adjacent channel leakage ratio for network signalling values NS_03U, NS_05U, NS_43U and NS_100 to all types of NR Power Class 3 UE release 15 and forward that don’t support Tx diversity.

The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_04, NS_05, NS_05U, NS_07, NS_12, NS_13, NS_14, NS_15, NS_17, NS_18, NS_21, NS_24, NS_27, NS_37, NS_38, NS_39, NS_40, NS_41, NS_42, NS_43, NS_43U, NS_44, NS_45, NS_48, NS_49, NS_50 and NS_56 to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity.

NOTE:	Test execution is not necessary if 6.5.2.3, 6.5.2.4.2 and 6.5.3.3 are executed.

6.2.3.3	Minimum conformance requirements

<Unchanged Skipped>

6.2.3.3.6	A-MPR for NS_43 and NS_43U

Table 6.2.3.3.6-1: A-MPR regions for NS_43 (Power class 3 and 2)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 5 MHz | 902.5 ≤ FC < 912.5 |  | > 2.7 MHz/12/SCS | A1 |  |  |  |
| 10 MHz | FC = 910 |  | > 7.2 MHz/12/SCS | A2 |  | > 5.4 MHz/12/SCS | A4 |
|  |  |  |  |  |  | > 8.1 MHz/12/SCS |  | A3 |  |  |  | > 7.2 MHz/12/SCS |  | A5 |  |
| 15 MHz |  | FC = 907.5 |  | < 1.8 MHz /12/SCS |  | > 0 |  | A6 |  | > 1.8 MHz/12/SCS < 6.12 MHz/12/SCS |  | ≥ 7.2 MHz/12/SCS |  | A6 |  |
|  |  |  |  | > 12.24 MHz/12/SCS |  | > 0 |  | A6 |  | > 1.8 MHz/12/SCS < 6.12 MHz/12/SCS |  | < 7.2 MHz/12/SCS |  | A7 |  |
|  |  |  |  |  |  |  |  |  |  | ≥ 6.12 MHz/12/SCS ≤ 7.2 MHz/12/SCS |  | > 5.4 MHz/12/SCS |  | A7 |  |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.3.6-2. NOTE 2: 15 kHz SCS unless otherwise stated NOTE 3: Void |  |  |  |  |  |  |  |

Table 6.2.3.3.6-2: A-MPR for NS_43 (Power class 3)

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) |  | A3 (dB) |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer / Inner |
| DFT-s-OFDM | Pi/2 BPSK |  | N/A | ≤ 1.5 | N/A |  | N/A |  | N/A |  | N/A | ≤ 9 |
|  | QPSK | ≤ 2 |  |  |  |  |  | ≤ 2.5 |  |  |  | ≤ 9 |
|  | 16 QAM |  |  |  |  |  |  |  |  | ≤ 2.5 |  | ≤ 9 |
|  | 64 QAM |  |  |  |  | ≤ 2.5 |  |  |  |  |  | ≤ 9 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  | ≤ 9 |
| CP-OFDM | QPSK | ≤ 3.5 |  |  |  |  |  |  |  | ≤ 4 |  | ≤ 9 |
|  | 16 QAM | ≤ 3.5 |  |  |  |  |  |  |  | ≤ 4 |  | ≤ 9 |
|  | 64 QAM |  |  |  |  | ≤ 4 |  |  |  |  |  | ≤ 9 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  | ≤ 9 |

Table 6.2.3.3.6-2a: A-MPR for NS_43 (Power Class 2)

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) |  | A3 (dB) |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) | A7 (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer / Inner | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1 | N/A | ≤ 1.5 | N/A |  |  |  | N/A |  | N/A | ≤ 12 | ≤ 4 | 3 |
|  | QPSK | ≤ 3.5 |  |  |  |  |  | ≤ 3.5 |  | ≤ 5.5 |  | ≤ 12 | ≤ 4 | 3 |
|  | 16 QAM | ≤ 3.5 |  |  |  |  |  | ≤ 3.5 |  | ≤ 5.5 |  | ≤ 12 | ≤ 5 | 4 |
|  | 64 QAM | ≤ 3.5 |  | ≤ 4.5 |  | ≤ 5 |  |  |  |  |  | ≤ 12 | ≤ 5.5 | 5.5 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  | ≤ 12 | ≤ 6.5 | 5.5 |
| CP-OFDM | QPSK | ≤ 4 |  |  |  |  |  | ≤ 4 |  | ≤ 6 |  | ≤ 12 | ≤ 6 | 4.5 |
|  | 16 QAM | ≤ 4 |  |  |  |  |  | ≤ 4.5 |  | ≤ 6 |  | ≤ 12 | ≤ 6 | 5 |
|  | 64 QAM | ≤ 4 |  | ≤ 5 |  | ≤ 6.5 |  |  |  |  |  | ≤ 12 | ≤ 6.5 | 6 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  | ≤ 12 | ≤ 7 | 7 |

<Unchanged Skipped>

Table 6.2.3.4.1-6: Test Configuration table for NS_43 for PC3 UE

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | 5 MHz, 10 MHz, 15 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest |  |
| A-MPR test parameters for NS_43 |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | 910 | 10 | default | N/A for A-MPR | DFT-s--OFDM | PI/2 BPSK | Outer_Full (A2) |
| 2 | 907.5 | 15 |  |  |  | PI/2 BPSK | Edge_1RB_Left (A6) |
| 3 | 907.5 | 15 |  |  |  | PI/2 BPSK | Outer_Full (A6) |
| 43 | 902.5 | 5 |  |  |  | QPSK | Outer_Full (A1) |
| 5 | 910 | 10 |  |  |  | QPSK | Outer_Full (A4) |
| 6 | 907.5 | 15 |  |  |  | QPSK | Edge_1RB_Left (A6) |
| 7 | 907.5 | 15 |  |  |  | QPSK | Outer_Full (A6) |
| 8 | 910 | 10 |  |  |  | 16 QAM | Outer_Full (A5) |
| 9 | 907.5 | 15 |  |  |  | 16 QAM | Edge_1RB_Left (A6) |
| 10 | 907.5 | 15 |  |  |  | 16 QAM | Outer_Full (A6) |
| 114 | 910 | 10 |  |  |  | 64 QAM | Outer_Full (A3) |
| 12 | 907.5 | 15 |  |  |  | 64 QAM | Edge_1RB_Left (A6) |
| 13 | 907.5 | 15 |  |  |  | 64 QAM | Outer_Full (A6) |
| 14 | 907.5 | 15 |  |  |  | 256 QAM | Edge_1RB_Left (A6) |
| 15 | 907.5 | 15 |  |  |  | 256 QAM | Outer_Full (A6) |
| 16 | 902.5 | 5 |  |  | CP-OFDM | QPSK | Outer_Full (A1) |
| 17 | 910 | 10 |  |  |  | QPSK | Outer_Full (A5) |
| 18 | 907.5 | 15 |  |  |  | QPSK | Edge_1RB_Left (A6) |
| 19 | 907.5 | 15 |  |  |  | QPSK | Outer_Full (A6) |
| 20 | 902.5 | 5 |  |  |  | 16 QAM | Outer_Full (A1) |
| 21 | 910 | 10 |  |  |  | 16 QAM | Outer_Full (A5) |
| 22 | 907.5 | 15 |  |  |  | 16 QAM | Edge_1RB_Left (A6) |
| 23 | 907.5 | 15 |  |  |  | 16 QAM | Outer_Full (A6) |
| 243 | 910 | 10 |  |  |  | 64 QAM | Outer_Full (A3) |
| 25 | 907.5 | 15 |  |  |  | 64 QAM | Edge_1RB_Left (A6) |
| 26 | 907.5 | 15 |  |  |  | 64 QAM | Outer_Full (A6) |
| 27 | 907.5 | 15 |  |  |  | 256 QAM | Edge_1RB_Left (A6) |
| 28 | 907.5 | 15 |  |  |  | 256 QAM | Outer_Full (A6) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Not applicable when SCS=60kHz |  |  |  |  |  |  |  |

Table 6.2.3.4.1-6a: Test Configuration table for NS_43 for PC2 UE

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | 5 MHz, 10 MHz, 15 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest |  |
| A-MPR test parameters for NS_43 |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | 902.5 | 5 | Default | N/A for A-MPR | DFT-s--OFDM | Pi/2 BPSK | Outer_Full (A1) |
| 2 | 910 | 10 |  |  |  | Pi/2 BPSK | Outer_Full (A2) |
| 3 | 907.5 | 15 |  |  |  | Pi/2 BPSK | Outer_Full (A6) |
| 4 | 907.5 | 15 |  |  |  | Pi/2 BPSK | Edge_1RB_Left (A6) |
| 53 | 902.5 | 5 |  |  |  | QPSK | Outer_Full (A1) |
| 6 | 910 | 10 |  |  |  | QPSK | Outer_Full (A4) |
| 7 | 910 | 10 |  |  |  | QPSK | Outer_Full (A5) |
| 8 | 907.5 | 15 |  |  |  | QPSK | Outer_Full (A6) |
| 9 | 907.5 | 15 |  |  |  | QPSK | Edge_1RB_Left (A6) |
| 103 | 902.5 | 5 |  |  |  | 16QAM | Outer_Full (A1) |
| 11 | 910 | 10 |  |  |  | 16QAM | Outer_Full (A4) |
| 12 | 910 | 10 |  |  |  | 16QAM | Outer_Full (A5) |
| 13 | 907.5 | 15 |  |  |  | 16QAM | Outer_Full (A6) |
| 14 | 907.5 | 15 |  |  |  | 16QAM | Edge_1RB_Left (A6) |
| 153 | 902.5 | 5 |  |  |  | 64QAM | Outer_Full (A1) |
| 16 | 910 | 10 |  |  |  | 64QAM | Outer_Full (A2) |
| 173 | 910 | 10 |  |  |  | 64QAM | Outer_Full (A3) |
| 18 | 907.5 | 15 |  |  |  | 64QAM | Outer_Full (A6) |
| 19 | 907.5 | 15 |  |  |  | 64QAM | Edge_1RB_Left (A6) |
| 20 | 907.5 | 15 |  |  |  | 256QAM | Outer_Full (A6) |
| 21 | 907.5 | 15 |  |  |  | 256QAM | Edge_1RB_Left (A6) |
| 223 | 902.5 | 5 |  |  | CP-OFDM | QPSK | Outer_Full (A1) |
| 23 | 910 | 10 |  |  |  | QPSK | Outer_Full (A4) |
| 24 | 910 | 10 |  |  |  | QPSK | Outer_Full (A5) |
| 25 | 907.5 | 15 |  |  |  | QPSK | Outer_Full (A6) |
| 26 | 907.5 | 15 |  |  |  | QPSK | Edge_1RB_Left (A6) |
| 273 | 902.5 | 5 |  |  |  | 16QAM | Outer_Full (A1) |
| 28 | 910 | 10 |  |  |  | 16QAM | Outer_Full (A4) |
| 29 | 910 | 10 |  |  |  | 16QAM | Outer_Full (A5) |
| 30 | 907.5 | 15 |  |  |  | 16QAM | Outer_Full (A6) |
| 31 | 907.5 | 15 |  |  |  | 16QAM | Edge_1RB_Left (A6) |
| 323 | 902.5 | 5 |  |  |  | 64QAM | Outer_Full (A1) |
| 33 | 910 | 10 |  |  |  | 64QAM | Outer_Full (A2) |
| 343 | 910 | 10 |  |  |  | 64QAM | Outer_Full (A3) |
| 35 | 907.5 | 15 |  |  |  | 64QAM | Outer_Full (A6) |
| 36 | 907.5 | 15 |  |  |  | 64QAM | Edge_1RB_Left (A6) |
| 37 | 907.5 | 15 |  |  |  | 256QAM | Outer_Full (A6) |
| 38 | 907.5 | 15 |  |  |  | 256QAM | Edge_1RB_Left (A6) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Not applicable when SCS=60kHz |  |  |  |  |  |  |  |

<Unchanged Skipped>

Table 6.2.3.5-10: UE Power Class 3 test requirements (NS_43)

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 2 | 23 | 0.5 | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 3 | 23 | 0.5 | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 4 | 23 | 1.0 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 5 | 23 | 1.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 6 | 23 | 1.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 7 | 23 | 1.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 8 | 23 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 9 | 23 | 2.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 10 | 23 | 2.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 11 | 23 | 2.5 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 12 | 23 | 2.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 13 | 23 | 2.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 14 | 23 | 4.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 15 | 23 | 4.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 16 | 23 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 17 | 23 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 18 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 19 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 20 | 23 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 21 | 23 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 22 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 23 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 24 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 25 | 23 | 3.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 26 | 23 | 3.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 27 | 23 | 6.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 28 | 23 | 6.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-10a: UE Power Class 2 test requirements (NS_43)

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 26 |  | 0.5 |  | 1 |  | 0 |  | 25 |  | 2 |  | 3 |  | 28+TT |  | 22-TT |  |
| 2 |  | 26 |  | 0.5 |  | 1.5 |  | 0 |  | 24.5 |  | 2 |  | 3 |  | 28+TT |  | 21.5-TT |  |
| 3 |  | 26 |  | 0.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 4 |  | 26 |  | 3.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 5 |  | 26 |  | 1 |  | 3.5 |  | 0 |  | 22.5 |  | 2 |  | 3 |  | 28+TT |  | 19.5-TT |  |
| 6 |  | 26 |  | 1 |  | 3.5 |  | 0 |  | 22.5 |  | 2 |  | 3 |  | 28+TT |  | 19.5-TT |  |
| 7 |  | 26 |  | 1 |  | 5.5 |  | 0 |  | 20.5 |  | 2.5 |  | 3 |  | 28+TT |  | 17.5-TT |  |
| 8 |  | 26 |  | 1 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 9 |  | 26 |  | 3.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 10 |  | 26 |  | 2 |  | 3.5 |  | 0 |  | 22.5 |  | 2 |  | 3 |  | 28+TT |  | 19.5-TT |  |
| 11 |  | 26 |  | 2 |  | 3.5 |  | 0 |  | 22.5 |  | 2 |  | 3 |  | 28+TT |  | 19.5-TT |  |
| 12 |  | 26 |  | 2 |  | 5.5 |  | 0 |  | 20.5 |  | 2.5 |  | 3 |  | 28+TT |  | 17.5-TT |  |
| 13 |  | 26 |  | 2 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 14 |  | 26 |  | 3.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 15 |  | 26 |  | 2.5 |  | 3.5 |  | 0 |  | 22.5 |  | 2 |  | 3 |  | 28+TT |  | 19.5-TT |  |
| 16 |  | 26 |  | 2.5 |  | 4.5 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 17 |  | 26 |  | 2.5 |  | 5 |  | 0 |  | 21 |  | 2 |  | 3 |  | 28+TT |  | 18-TT |  |
| 18 |  | 26 |  | 2.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 19 |  | 26 |  | 3.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 20 |  | 26 |  | 4.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 21 |  | 26 |  | 4.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 22 |  | 26 |  | 3 |  | 4 |  | 0 |  | 22 |  | 2 |  | 3 |  | 28+TT |  | 19-TT |  |
| 23 |  | 26 |  | 3 |  | 4 |  | 0 |  | 22 |  | 2 |  | 3 |  | 28+TT |  | 19-TT |  |
| 24 |  | 26 |  | 3 |  | 6 |  | 0 |  | 20 |  | 2.5 |  | 3 |  | 28+TT |  | 17-TT |  |
| 25 |  | 26 |  | 3 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 26 |  | 26 |  | 3.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 27 |  | 26 |  | 3 |  | 4 |  | 0 |  | 22 |  | 2 |  | 3 |  | 28+TT |  | 19-TT |  |
| 28 |  | 26 |  | 3 |  | 4.5 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 29 |  | 26 |  | 3 |  | 6 |  | 0 |  | 20 |  | 2.5 |  | 3 |  | 28+TT |  | 17-TT |  |
| 30 |  | 26 |  | 3 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 31 |  | 26 |  | 3.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 32 |  | 26 |  | 3.5 |  | 4 |  | 0 |  | 22 |  | 2 |  | 3 |  | 28+TT |  | 19-TT |  |
| 33 |  | 26 |  | 3.5 |  | 5 |  | 0 |  | 21 |  | 2 |  | 3 |  | 28+TT |  | 18-TT |  |
| 34 |  | 26 |  | 3.5 |  | 6.5 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 35 |  | 26 |  | 3.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 36 |  | 26 |  | 3.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 37 |  | 26 |  | 6.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 38 |  | 26 |  | 6.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

<End of Changes>