3GPP TSG-RAN5 Meeting #103	R5-242387

# Fukuoka, Japan, 20 – 24 May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5505 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of band 106 transmitter requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_900MHz_US-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Missing band 106 transmitter requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added band 106 transmitter requirements into - clause 6.2.2 UE Maximum Output Power - clause 6.2.2EA UE MOP for category M1 - clause 6.2.2EC UE MOP for category M2 - clause 6.2.2F UE MOP for category NB1 and NB2 - clause 6.6.3.2 Spurious emission band UE co-existence |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Band 106 transmitter requirements will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2, 6.6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of changes --------

### 6.2.2	UE Maximum Output Power

Editor’s notes: The test applicability of Maximum Output Power for Band 47 is needed to be checked.

#### 6.2.2.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

#### 6.2.2.2	Test applicability

This test case applies to all types of E-UTRA Power Class 3 UE release 8 and forward.

#### 6.2.2.3	Minimum conformance requirements

The following UE Power Classes defines the maximum output power for any transmission bandwidth within the channel bandwidth. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.2.3-1: UE Power Class

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | ±2 |  |  |
| 2 |  |  |  |  | 23 | ±22 |  |  |
| 3 |  |  |  |  | 23 | ±22 |  |  |
| 4 |  |  |  |  | 23 | ±2 |  |  |
| 5 |  |  |  |  | 23 | ±2 |  |  |
| 6 |  |  |  |  | 23 | ±2 |  |  |
| 7 |  |  |  |  | 23 | ±22 |  |  |
| 8 |  |  |  |  | 23 | ±22 |  |  |
| 9 |  |  |  |  | 23 | ±2 |  |  |
| 10 |  |  |  |  | 23 | ±2 |  |  |
| 11 |  |  |  |  | 23 | ±2 |  |  |
| 12 |  |  |  |  | 23 | ±22 |  |  |
| 13 |  |  |  |  | 23 | ±2 |  |  |
| 14 |  |  |  |  | 23 | ±2 |  |  |
| … |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  | 23 | ±2 |  |  |
| 18 |  |  |  |  | 23 | ±26 |  |  |
| 19 |  |  |  |  | 23 | ±2 |  |  |
| 20 |  |  |  |  | 23 | ±22 |  |  |
| 21 |  |  |  |  | 23 | ±2 |  |  |
| 22 |  |  |  |  | 23 | +2/-3.52 |  |  |
| 23 |  |  |  |  | 235 | ±25 |  |  |
| 24 |  |  |  |  | 23 | +2/-32 |  |  |
| 25 |  |  |  |  | 23 | ±22 |  |  |
| 26 |  |  |  |  | 23 | ±22 |  |  |
| 27 |  |  |  |  | 23 | ±2 |  |  |
| 28 |  |  |  |  | 23 | +2/-2.5 |  |  |
| 30 |  |  |  |  | 23 | ±2 |  |  |
| 31 |  |  |  |  | 23 | ±2 |  |  |
| ... |  |  |  |  |  |  |  |  |
| 33 |  |  |  |  | 23 | ±2 |  |  |
| 34 |  |  |  |  | 23 | ±2 |  |  |
| 35 |  |  |  |  | 23 | ±2 |  |  |
| 36 |  |  |  |  | 23 | ±2 |  |  |
| 37 |  |  |  |  | 23 | ±2 |  |  |
| 38 |  |  |  |  | 23 | ±2 |  |  |
| 39 |  |  |  |  | 23 | ±2 |  |  |
| 40 |  |  |  |  | 23 | ±2 |  |  |
| 41 |  |  |  |  | 23 | ±22 |  |  |
| 42 |  |  |  |  | 23 | +2/-3 |  |  |
| 43 |  |  |  |  | 23 | +2/-3 |  |  |
| 44 |  |  |  |  | 23 | +2/[-3] |  |  |
| 45 |  |  |  |  | 23 | ±2 |  |  |
| 47 |  |  | 26 | ±2 | 23 | ±2 |  |  |
| 48 |  |  |  |  | 23 | +2/-3 |  |  |
| … |  |  |  |  |  |  |  |  |
| 53 |  |  |  |  | 23 | ±2 |  |  |
| 54 |  |  |  |  | 23 | ±2 |  |  |
| … |  |  |  |  |  |  |  |  |
| 65 |  |  |  |  | 23 | ±2 |  |  |
| 66 |  |  |  |  | 23 | ±2 |  |  |
| … |  |  |  |  |  |  |  |  |
| 68 |  |  |  |  | 23 | ±2 |  |  |
| … |  |  |  |  |  |  |  |  |
| 70 |  |  |  |  | 23 | ±2 |  |  |
| 71 |  |  |  |  | 23 | +2/-2.5 |  |  |
| 72 |  |  |  |  | 23 | ±2 |  |  |
| 73 |  |  |  |  | 23 | ±2 |  |  |
| 74 |  |  |  |  | 23 | ±2 |  |  |
| 85 |  |  |  |  | 23 | ±22 |  |  |
| 87 |  |  |  |  | 23 | ±2 |  |  |
| 88 |  |  |  |  | 23 | ±2 |  |  |
| 106 |  |  |  |  | 23 | ±2 |  |  |
| Note 1: Void Note 2: For transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB Note 3: PPowerClass is the maximum UE power specified without taking into account the tolerance Note 4: For the UE which supports both Band 11 and Band 21 operating frequencies, the tolerance is FFS. Note 5: When NS_20 is signalled, the total output power within 2000-2005 MHz shall be limited to 7 dBm. Note 6: For a UE that supports both Band 18 and Band 26, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5dB for transmission bandwidths confined within 815 MHz and 818 MHz |  |  |  |  |  |  |  |  |

The default power class PPowerClass_Default for an operating band is Power Class 3 unless otherwise stated.

For a power class 2 capable UE operating on Band 41, when an IE P-max as defined in TS 36.331 [5] of 23 dBm or lower is indicated in the cell or if the uplink/downlink configuration is 0 or 6, the requirements for power class 2 are not applicable, and the corresponding requirements for a power class 3 UE shall apply.

For each supported frequency band other than Band 14 and Band 41, the UE shall:

-	if the UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class:

-	if the band is a TDD band whose frame configuration is 0 or 6; or

-	if the IE P-Max as defined in TS 36.331 [5] is not provided; or

-	if the IE P-Max as defined in TS 36.331 [5] is provided and set to the maximum output power of the default power class or lower;

-	meet all requirements for the default power class of the operating band in which the UE is operating and set its configured transmitted power as specified in sub-clause 6.2.5;

-	else (i.e the IE P-Max as defined in TS 36.331 [5] is provided and set to the higher value than the maximum output power of the default power class):

-	meet all requirements for the supported power class and set its configured transmitted power class as specified in sub-clause 6.2.5;

The normative reference for this requirement is TS 36.101 clause 6.2.2.

#### 6.2.2.4	Test description

##### 6.2.2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  | Lowest, 5MHz, Highest |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |  |
| Ch BW | N/A for Max UE output power testing |  | Mod'n | RB allocation |  |
|  |  |  |  | FDD | TDD |
| 1.4MHz |  |  | QPSK | 1 | 1 |
| 1.4MHz |  |  | QPSK | 5 | 5 |
| 3MHz |  |  | QPSK | 1 | 1 |
| 3MHz |  |  | QPSK | 4 | 4 |
| 5MHz |  |  | QPSK | 1 | 1 |
| 5MHz |  |  | QPSK | 8 | 8 |
| 10MHz |  |  | QPSK | 1 | 1 |
| 10MHz |  |  | QPSK | 12 | 12 |
| 15MHz |  |  | QPSK | 1 | 1 |
| 15MHz |  |  | QPSK | 16 | 16 |
| 20MHz |  |  | QPSK | 1 | 1 |
| 20MHz |  |  | QPSK | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: For E-UTRA bands not applied with Note 2 in Table 6.2.2.3-1: - The 1 RB allocation shall be tested at RB#0 for low and mid range, RB #max for high range test frequency. - The RBstart of non-1RB allocation shall be RB #0 for low and mid range, RB# (max +1 - RB allocation) for high range test frequency. Note 3: For E-UTRA bands applied with Note 2 in Table 6.2.2.3-1: - If the test channel bandwidth is larger than 4MHz, then the 1 RB allocation shall be tested at both RB #0 and RB #max. - If the test channel bandwidth is smaller or equal to 4MHz, then the 1 RB allocation shall be tested at RB #0. - If the test channel bandwidth = (FUL_high - FUL_low) specified by the operating band, then only one frequency range shall be tested and the 1 RB allocation shall be tested at RB #0, RB # and RB #max. - For non-1RB allocation, test frequency is middle range, and the RBstart shall be RB #0. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.2.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RFaccording to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.2.2.4.3.

##### 6.2.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.2.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

##### 6.2.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6.

#### 6.2.2.5	Test requirements

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2.5-1.

Table 6.2.2.5-1: UE Power Class test requirements

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | ±2.7 |  |  |
| 2 |  |  |  |  | 23 | ±2.72 |  |  |
| 3 |  |  |  |  | 23 | ±2.72 |  |  |
| 4 |  |  |  |  | 23 | ±2.7 |  |  |
| 5 |  |  |  |  | 23 | ±2.7 |  |  |
| 6 |  |  |  |  | 23 | ±2.7 |  |  |
| 7 |  |  |  |  | 23 | ±2.72 |  |  |
| 8 |  |  |  |  | 23 | ±2.72 |  |  |
| 9 |  |  |  |  | 23 | ±2.7 |  |  |
| 10 |  |  |  |  | 23 | ±2.7 |  |  |
| 11 |  |  |  |  | 23 | ±2.7 |  |  |
| 12 |  |  |  |  | 23 | ±2.72 |  |  |
| 13 |  |  |  |  | 23 | ±2.7 |  |  |
| 14 |  |  |  |  | 23 | ±2.7 |  |  |
| … |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  | 23 | ±2.7 |  |  |
| 18 |  |  |  |  | 23 | ±2.76 |  |  |
| 19 |  |  |  |  | 23 | ±2.7 |  |  |
| 20 |  |  |  |  | 23 | ±2.72 |  |  |
| 21 |  |  |  |  | 23 | ±2.7 |  |  |
| 22 |  |  |  |  | 23 | +3.0/-4.52 |  |  |
| 23 |  |  |  |  | 235 | ±2.75 |  |  |
| 24 |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| 25 |  |  |  |  | 23 | ±2.72 |  |  |
| 26 |  |  |  |  | 23 | ±2.72 |  |  |
| 27 |  |  |  |  | 23 | ±2.7 |  |  |
| 28 |  |  |  |  | 23 | +2.7/-3.2 |  |  |
| 30 |  |  |  |  | 23 | ±2.7 |  |  |
| 31 |  |  |  |  | 23 | ±2.7 |  |  |
| ... |  |  |  |  |  |  |  |  |
| 33 |  |  |  |  | 23 | ±2.7 |  |  |
| 34 |  |  |  |  | 23 | ±2.7 |  |  |
| 35 |  |  |  |  | 23 | ±2.7 |  |  |
| 36 |  |  |  |  | 23 | ±2.7 |  |  |
| 37 |  |  |  |  | 23 | ±2.7 |  |  |
| 38 |  |  |  |  | 23 | ±2.7 |  |  |
| 39 |  |  |  |  | 23 | ±2.7 |  |  |
| 40 |  |  |  |  | 23 | ±2.7 |  |  |
| 41 |  |  |  |  | 23 | ±2.72 |  |  |
| 42 |  |  |  |  | 23 | +3.0/-4.0 |  |  |
| 43 |  |  |  |  | 23 | +3.0/-4.0 |  |  |
| 44 |  |  |  |  | 23 | +2.7/[-3.7] |  |  |
| 45 |  |  |  |  | 23 | ±2.7 |  |  |
| 47 |  |  | 26 | ±3.3 | 23 | ±3.3 |  |  |
| 48 |  |  |  |  | 23 | +3.0/-4.0 |  |  |
| … |  |  |  |  |  |  |  |  |
| 53 |  |  |  |  | 23 | +2.7/-2.7 |  |  |
| 54 |  |  |  |  | 23 | +2.7/-2.7 |  |  |
| … |  |  |  |  |  |  |  |  |
| 65 |  |  |  |  | 23 | +2.7/-2.7 |  |  |
| 66 |  |  |  |  | 23 | +2.7/-2.7 |  |  |
| … |  |  |  |  |  |  |  |  |
| 68 |  |  |  |  | 23 | +2.7/-2.7 |  |  |
| … |  |  |  |  |  |  |  |  |
| 70 |  |  |  |  | 23 | +2.7/-2.7 |  |  |
| 71 |  |  |  |  | 23 | +2.7/-3.2 |  |  |
| 72 |  |  |  |  | 23 | ±2.7 |  |  |
| 73 |  |  |  |  | 23 | ±2.7 |  |  |
| 74 |  |  |  |  | 23 | ±2.7 |  |  |
| 85 |  |  |  |  | 23 | ±2.72 |  |  |
| 87 |  |  |  |  | 23 | ±2.7 |  |  |
| 88 |  |  |  |  | 23 | ±2.7 |  |  |
| 106 |  |  |  |  | 23 | ±2.7 |  |  |
| Note 1: Void Note 2: For transmission bandwidths (Figure 5.4.2-1, Table 5.4.4-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB Note 3: PPowerClass is the maximum UE power specified without taking into account the tolerance. Note 4: For the UE which supports both Band 11 and Band 21 operating frequencies, the tolerance is FFS. Note 5: When NS_20 is signalled, the total output power within 2000-2005 MHz shall be limited to 7 dBm. Note 6: For a UE that supports both Band 18 and Band 26, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5dB for transmission bandwidths confined within 815 MHz and 818 MHz. |  |  |  |  |  |  |  |  |

For the UE which supports inter-band carrier aggregation configurations with uplink assigned to one E-UTRA band the ΔTIB,c in Tables 6.2.5.3-2, 6.2.5.3-3 and 6.2.5.3-4 shall be applied for applicable bands.

In case the UE supports more than one of the above inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one inter-band carrier aggregation configurations then:

-	When the E-UTRA operating band frequency range is ≤ 1GHz, the applicable additional tolerance shall be the average of the tolerances in Table 6.2.5.3-2, truncated to one decimal place for that operating band among the supported CA configurations. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported carrier aggregation configurations involving such band shall be applied

-	When the E-UTRA operating band frequency range is >1GHz, the applicable additional tolerance shall be the maximum tolerance in Table 6.2.5.3-2 that applies for that operating band among the supported CA configurations.

-------- Skipped unchanged clauses --------

### 6.2.2EA	UE Maximum Output Power for UE category M1

#### 6.2.2EA.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

#### 6.2.2EA.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward of UE category M1.

#### 6.2.2EA.3	Minimum conformance requirements

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth for non CA configuration and UL-MIMO unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.2EA-1: UE Power Class

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) | Class 6 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 2 |  |  | 23 | ±22 | 20 | ±22 | 14 | ±2.5 |
| 3 |  |  | 23 | ±22 | 20 | ±22 | 14 | ±2.5 |
| 4 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 5 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 7 |  |  | 23 | ±22 | 20 | ±22 | 14 | ±2.5 |
| 8 |  |  | 23 | ±22 | 20 | ±22 | 14 | ±2.5 |
| 11 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 12 |  |  | 23 | ±22 | 20 | ±22 | 14 | ±2.5 |
| 13 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 14 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 18 |  |  | 23 | ±25 | 20 | ±25 | 14 | ±2.5 |
| 19 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 20 |  |  | 23 | ±22 | 20 | ±22 | 14 | ±2.5 |
| 21 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 26 |  |  | 23 | ±22 | 20 | ±22 | 14 | ±2.5 |
| 27 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 28 |  |  | 23 | +2/-2.5 | 20 | +2/-2.5 | 14 | ±2.5 |
| 31 | 267 | ±2 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| … |  |  |  |  |  |  |  |  |
| 39 |  |  | 23 | ±2 | 20 | ±2 |  |  |
| 41 |  |  | 23 | ±22 | 20 | ±22 | 14 | ±2.5 |
| 42 |  |  | 23 | +2/-3 | 20 | +2/-3 | 14 | ±2.5 |
| 43 |  |  | 23 | +2/-3 | 20 | +2/-3 | 14 | ±2.5 |
| … |  |  |  |  |  |  | 14 | ±2.5 |
| 71 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 72 | 267 | ±2 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 73 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 74 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 85 |  |  | 23 | ±22 | 20 | ±22 | 14 | ±2.5 |
| 87 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 88 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 106 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| NOTE 1: Void NOTE 2: 2 refers to the transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 3: For the UE which supports both Band 11 and Band 21 operating frequencies, the tolerance is FFS. NOTE 4: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 5: For a UE that supports both Band 18 and Band 26, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB for transmission bandwidths confined within 815 MHz and 818 MHz. NOTE 6: Void NOTE 7: Applicable for category M1 and M2 HD-FDD UE. |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 6.2.2E.

#### 6.2.2EA.4	Test description

6.2.2EA.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in sub-clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.2EA.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.2.2EA.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  | Highest |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |  |
| Ch BW | N/A for Max UE output power testing |  | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | TDD |
| 5MHz |  |  | QPSK | 1 | 1 |
| 5MHz |  |  | QPSK | 3(Note 5) | 3(Note 5) |
| 10MHz |  |  | QPSK | 1 | 1 |
| 10MHz |  |  | QPSK | 4(Note 4), 5 (Note 5) | 4(Note 4), 5(Note 5) |
| 15MHz |  |  | QPSK | 1 | 1 |
| 15MHz |  |  | QPSK | 6 | 6 |
| 20MHz |  |  | QPSK | 1 | 1 |
| 20MHz |  |  | QPSK | 6 | 6 |
| Note 1: Test Channel Bandwidth is checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The 1 RB allocation shall be tested at RB#0 with narrowband index 0 for low and mid range, RB #5 with max narrowband index for high range test frequency. For E-UTRA bands applied with Note 2 in Table 6.2.2 EA -1 and when channel bandwidth is larger than 4 MHz, for 5 MHz channel bandwidth the 1 RB allocation shall additionally be tested at RB#2 with narrowband index [3] for low range, RB #3 with narrowband index 0 for high range test frequency, for 10 MHz channel bandwidth the 1 RB allocation shall additionally be tested at RB#1 with narrowband index 3 for low range, RB #4 with narrowband index 4 for high range test frequency, for 15 MHz channel bandwidth the 1 RB allocation shall additionally be tested at RB#0 with narrowband index 3 for low range, RB #5 with narrowband index 8 for high range test frequency, for 20 MHz channel bandwidth the 1 RB allocation shall additionally be tested at RB#3 with narrowband index 2 for low range, RB #2 with narrowband index 13 for high range test frequency. Note 3: The RBstart of non-1RB allocation shall be RB #0 with narrowband index 0 for low and mid range, RB# (6 - RB allocation) with max narrowband index for high range test frequency. Note 4: Only applicable for Power class 3 Note 5: Only applicable for Power class 5 and Power class 6 |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.2.2EA.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.2.2EA.4.3.

6.2.2EA.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to Table 6.2.2EA.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test. For HD-FDD slots with transient periods and Half-duplex guard subframe are not under test.

##### 6.2.2EA.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6.

#### 6.2.2EA.5	Test requirements

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2EA.5-1.

Table 6.2.2EA.5-1: UE Power Class test requirements

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) | Class 6 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| 2 |  |  | 23 | ±2.72 | 20 | ±2.72 | 14 | ±3.2 |
| 3 |  |  | 23 | ±2.72 | 20 | ±2.72 | 14 | ±3.2 |
| 4 |  |  | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| 5 |  |  | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| 7 |  |  | 23 | ±2.72 | 20 | ±2.72 | 14 | ±3.2 |
| 8 |  |  | 23 | ±2.72 | 20 | ±2.72 | 14 | ±3.2 |
| 11 |  |  | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| 12 |  |  | 23 | ±2.7 | 20 | ±2.72 | 14 | ±3.2 |
| 13 |  |  | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| 14 |  |  | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| 18 |  |  | 23 | ±2.75 | 20 | ±2.75 | 14 | ±3.2 |
| 19 |  |  | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| 20 |  |  | 23 | ±2.72 | 20 | ±2.72 | 14 | ±3.2 |
| 21 |  |  | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| 26 |  |  | 23 | ±2.72 | 20 | ±2.72 | 14 | ±3.2 |
| 27 |  |  | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| 28 |  |  | 23 | +2.7/-3.2 | 20 | +2.7/-3.2 | 14 | ±3.2 |
| 31 | 26 | ±2.77 | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| … |  |  |  |  |  |  |  |  |
| 39 |  |  | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| 41 |  |  | 23 | ±2.72 | 20 | ±2.72 | 14 | ±3.2 |
| 42 |  |  | 23 | +3.0/-4.0 | 20 | +3.0/-4.0 | 14 | ±3.5 |
| 43 |  |  | 23 | +3.0/-4.0 | 20 | +3.0/-4.0 | 14 | ±3.5 |
| … |  |  |  |  |  |  |  |  |
| 71 |  |  | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| 72 | 26 | ±2.77 | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| 73 |  |  | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| 74 |  |  | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| 85 |  |  | 23 | ±2.7 | 20 | ±2.72 | 14 | ±3.2 |
| 87 |  |  | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| 88 |  |  | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| 106 |  |  | 23 | ±2.7 | 20 | ±2.7 | 14 | ±3.2 |
| NOTE 1: Void NOTE 2: 2 refers to the transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 3: For the UE which supports both Band 11 and Band 21 operating frequencies, the tolerance is FFS. NOTE 4: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 5: For a UE that supports both Band 18 and Band 26, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB for transmission bandwidths confined within 815 MHz and 818 MHz. NOTE 6: Void NOTE 7: Applicable for category M1 and M2 HD-FDD UE |  |  |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

### 6.2.2EC	UE Maximum Output Power for UE category M2

#### 6.2.2EC.1	Test purpose

Same as in clause 6.2.2EA.1

#### 6.2.2EC.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward of UE category M2.

#### 6.2.2EC.3	Minimum conformance requirements

Same as in clause 6.2.2EA.3

#### 6.2.2EC.4	Test description

##### 6.2.2EC.4.1	Initial condition

Same as in clause 6.2.2EA.4.1 with the following exceptions:

-	Instead of Table 6.2.2EA.4.1-1  use Table 6.2.2EC.4.1-1

Table 6.2.2EC.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Highest |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |  |
| Configuration ID | Ch BW | N/A for Max UE output power testing | Mod'n | RB allocation |  |  |
|  |  |  |  | FDD and HD-FDD | TDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |  |
| 1 | 5MHz |  | QPSK | 1 | 1 | 0 |
| 2 | 5MHz |  | QPSK | 3(Note 6) | 3(Note 6) | 0 |
| 3 | 10MHz |  | QPSK | 1 | 1 | 0 |
| 4 | 10MHz |  | QPSK | 4(Note 5), 5 (Note 6) | 4(Note 5), 5 (Note 6) | 0 |
| 5 | 15MHz |  | QPSK | 1 | 1 | 0 |
| 6 | 15MHz |  | QPSK | 24 | 24 | 0 |
| 7 | 20MHz |  | QPSK | 1 | 1 | 0 |
| 8 | 20MHz |  | QPSK | 24 | 24 | 0 |
| High range |  |  |  |  |  |  |
| 1 | 5MHz |  | QPSK | 1 | 1 | 3 |
| 2 | 5MHz |  | QPSK | 3(Note 6) | 3(Note 6) | 3 |
| 3 | 10MHz |  | QPSK | 1 | 1 | 7 |
| 4 | 10MHz |  | QPSK | 4(Note 5), 5 (Note 6) | 4(Note 5), 5 (Note 6) | 7 |
| 5 | 15MHz |  | QPSK | 1 | 1 | 11 |
| 6 | 15MHz |  | QPSK | 24 | 24 | 8 |
| 7 | 20MHz |  | QPSK | 1 | 1 | 15 |
| 8 | 20MHz |  | QPSK | 24 | 24 | 12 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.4. Note 2: Test Channel Bandwidth is checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The 1 RB allocation shall be tested at RB#0 for low and mid range, RB #5 for high range test frequency. For E-UTRA bands applied with Note 2 in Table 6.2.2 EA -1 and when channel bandwidth is larger than 4 MHz, for 5 MHz channel bandwidth the 1 RB allocation shall additionally be tested at RB#2 with narrowband index 3 for low range, RB #3 withnarrowband index 0 for high range test frequency, for 10 MHz channel bandwidth the 1 RB allocation shall additionally be tested at RB#1 with narrowband index 3 for low range, RB #4 with narrowband index 4 for high range test frequency, for 15 MHz channel bandwidth the 1 RB allocation shall additionally be tested at RB#0 with narrowband index 3 for low range, RB #5 with narrowband index 8 for high range test frequency, for 20 MHz channel bandwidth the 1 RB allocation shall additionally be tested at RB#3 with narrowband index 2 for low range, RB #2 with narrowband index 13 for high range test frequency. Note 4: The RBstart of non-1RB allocation shall be RB #0 for low and mid range, RB# (6 – (RB allocation mod 6)) for high range test frequency. Note 5: Only applicable for Power class 3. Note 6: Only applicable for Power class 5 and Power class 6. |  |  |  |  |  |  |

##### 6.2.2EC.4.2	Test procedure

Same as in clause 6.2.2EA.4.1 with the following exceptions:

-	Instead of Table 6.2.2EA.4.1-1 use Table 6.2.2EC.4.1-1

##### 6.2.2EC.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6.

#### 6.2.2EC.5	Test requirements

Same as in clause 6.2.2EA.5.

### 6.2.2F	UE Maximum Output Power for category NB1 and NB2

6.2.2F.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2.2F.2	Test applicability

This test case applies to all types of NB-IoT FDD UE release 13 and forward of category NB1.

This test case applies to all types of NB-IoT FDD UE release 14 and forward of category NB2.

This test case applies to all types of NB-IoT TDD UE release 15 and forward of category NB1 and NB2.

6.2.2F.3	Minimum conformance requirements

Category NB1 and NB2 UE Power Classes are specified in Table 6.2.2F.3-1 and define the maximum output power for any transmission bandwidth within the category NB1 and NB2 channel bandwidth. For 3.75kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one slot (2ms) excluding the 2304Ts gap when UE is not transmitting. For 15kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one sub-frame (1ms).

Table 6.2.2F.3-1: UE Power Class

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 1 | 23 | ±2 | 20 | ±2 |
| 2 | 23 | ±2 | 20 | ±2 |
| 3 | 23 | ±2 | 20 | ±2 |
| 4 | 23 | ±2 | 20 | ±2 |
| 5 | 23 | ±2 | 20 | ±2 |
| 7 | 23 | ±2 | 20 | ±2 |
| 8 | 23 | ±2 | 20 | ±2 |
| 11 | 23 | ±2 | 20 | ±2 |
| 12 | 23 | ±2 | 20 | ±2 |
| 13 | 23 | ±2 | 20 | ±2 |
| 14 | 23 | ±2 | 20 | ±2 |
| 17 | 23 | ±2 | 20 | ±2 |
| 18 | 23 | ±2 | 20 | ±2 |
| 19 | 23 | ±2 | 20 | ±2 |
| 20 | 23 | ±2 | 20 | ±2 |
| 21 | 23 | ±2 | 20 | ±2 |
| 25 | 23 | ±2 | 20 | ±2 |
| 26 | 23 | ±2 | 20 | ±2 |
| 28 | 23 | ±2 | 20 | ±2 |
| 31 | 23 | ±2 | 20 | ±2 |
| 41 | 23 | ±2 | 20 | ±2 |
| 42 | 23 | ±2 | 20 | ±2 |
| 43 | 23 | ±2 | 20 | ±2 |
| 65 | 23 | ±2 | 20 | ±2 |
| 66 | 23 | ±2 | 20 | ±2 |
| 70 | 23 | ±2 | 20 | ±2 |
| 71 | 23 | ±2 | 20 | ±2 |
| 72 | 23 | ±2 | 20 | ±2 |
| 73 | 23 | ±2 | 20 | ±2 |
| 74 | 23 | ±2 | 20 | ±2 |
| 85 | 23 | ±2 | 20 | ±2 |
| 87 | 23 | ±2 | 20 | ±2 |
| 88 | 23 | ±2 | 20 | ±2 |
| 103 | 23 | ±2 | 20 | ±2 |
| 106 | 23 | ±2 | 20 | ±2 |

The normative reference for this requirement is TS 36.101 clause 6.2.2F.

#### 6.2.2F.4	Test description

##### 6.2.2F.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions and test frequencies based on the subset of E-UTRA operating bands defined for NB-IoT in clause 5.2F. All of these configurations shall be tested with applicable test parameters, and are shown in table 6.2.2F.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of NPDSCH and NPDCCH before measurement are specified in Annex C.2.

Table 6.2.2F.4.1-1: Test Configuration Initial Conditions for FDD & TDD

| Test Environment as specified in TS 36.508 [7] subclause 8.1.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| --- | --- | --- | --- | --- |
| Test Frequencies as specified in TS 36.508 [7] subclause 8.1.3.1 |  | Frequency ranges defined in Annex K.1.2 |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 (Note 2) |  | BPSK | 1@0 | 3.75 |
| 2 (Note 3) |  | BPSK | 1@47 | 3.75 |
| 3 (Note 2) |  | QPSK | 1@0 | 15 |
| 4 (Note 3) |  | QPSK | 1@11 | 15 |
| 5 (Note 1) |  | QPSK | 3@3 | 15 |
| Note 1: Applicable to UE supporting UL multi-tone transmissions Note 2: only applicable for low range Note 3: only applicable for high range |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A Figure A.3 using only the main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1.1 and H.4.0.

4.	The UL Reference Measurement channel is set according to Table 6.2.2F.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5. Message contents are defined in clause 6.2.2F.4.3.

##### 6.2.2F.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via NPDCCH with DCI format N0 for C_RNTI to schedule the UL RMC according to Table 6.2.2F.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC (UE should be already transmitting PUMAX after Initial Conditions setting).

2.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one sub-frame (1ms) for sub-carrier spacing of 15 kHz or one slot (2ms) excluding the 2304Ts gap when UE is not transmitting for sub-carrier spacing of 3.75 kHz. For TDD slots with transient periods are not under test. For Half-Duplex guard subframes are not under test.

NOTE 1:	For configuration IDs applicable to UE depending on UE capability in Test Configuration Table with different UL sub-carrier spacing, the SS shall release the connection through State 3A-NB and finally ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5 using the appropriate UL subcarrier spacing in Random Access Response message.

##### 6.2.2F.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 8.1.6.

#### 6.2.2F.5	Test requirements

The maximum output power, derived in step 2 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2F.5-1.

Table 6.2.2F.5-1: UE Power Class test requirement

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 1 | 23 | ±2.7 | 20 | ±2.7 |
| 2 | 23 | ±2.7 | 20 | ±2.7 |
| 3 | 23 | ±2.7 | 20 | ±2.7 |
| 4 | 23 | ±2.7 | 20 | ±2.7 |
| 5 | 23 | ±2.7 | 20 | ±2.7 |
| 7 | 23 | ±2.7 | 20 | ±2.7 |
| 8 | 23 | ±2.7 | 20 | ±2.7 |
| 11 | 23 | ±2.7 | 20 | ±2.7 |
| 12 | 23 | ±2.7 | 20 | ±2.7 |
| 13 | 23 | ±2.7 | 20 | ±2.7 |
| 14 | 23 | ±2.7 | 20 | ±2.7 |
| 17 | 23 | ±2.7 | 20 | ±2.7 |
| 18 | 23 | ±2.7 | 20 | ±2.7 |
| 19 | 23 | ±2.7 | 20 | ±2.7 |
| 20 | 23 | ±2.7 | 20 | ±2.7 |
| 21 | 23 | ±2.7 | 20 | ±2.7 |
| 25 | 23 | ±2.7 | 20 | ±2.7 |
| 26 | 23 | ±2.7 | 20 | ±2.7 |
| 28 | 23 | ±2.7 | 20 | ±2.7 |
| 31 | 23 | ±2.7 | 20 | ±2.7 |
| 41 | 23 | ±2.7 | 20 | ±2.7 |
| 42 | 23 | ±3.0 | 20 | ±3.0 |
| 43 | 23 | ±3.0 | 20 | ±3.0 |
| 65 | 23 | ±2.7 | 20 | ±2.7 |
| 66 | 23 | ±2.7 | 20 | ±2.7 |
| 70 | 23 | ±2.7 | 20 | ±2.7 |
| 71 | 23 | ±2.7 | 20 | ±2.7 |
| 72 | 23 | ±2.7 | 20 | ±2.7 |
| 73 | 23 | ±2.7 | 20 | ±2.7 |
| 74 | 23 | ±2.7 | 20 | ±2.7 |
| 85 | 23 | ±2.7 | 20 | ±2.7 |
| 87 | 23 | ±2.7 | 20 | ±2.7 |
| 88 | 23 | ±2.7 | 20 | ±2.7 |
| 103 | 23 | ±2.7 | 20 | ±2.7 |
| 106 | 23 | ±2.7 | 20 | ±2.7 |

-------- Skipped unchanged clauses --------

#### 6.6.3.2	Spurious emission band UE co-existence

Editor's note: This test case contains different requirements for different UE releases

##### 6.6.3.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to co-existing systems for the specified bands which has specific requirements in terms of transmitter spurious emissions.

##### 6.6.3.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 8 and forward.

##### 6.6.3.2.3	Minimum conformance requirements

This clause specifies the requirements for the specified E-UTRA band for coexistence with protected bands as indicated in Tables 6.6.3.2.3-1 to 6.6.3.2.3-1J.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

-------- Skipped unchanged clauses --------

Table 6.6.3.2.3-1J: Spurious emission band UE co-existence limits Rel-18

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| 1 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 15, 27 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 15, 26, 27, 44 |
| 2 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 53, 54, 66, 70, 71, 74, 85, 103, 106 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 3 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 41, 43, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73,74, 75, 76, 87, 88 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 22, 42, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 |  |
| 4 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 41, 43, 48, 50, 51, 53, 66, 70, 71, 74, 85, 103, 106 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 5 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74, 85, 103, 106 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | E-UTRA Band 41, 52, 53, 54 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 | 39 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 39 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 39 |
| 6 | E-UTRA Band 1, 9, 11, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 860 | - | 875 | -37 | 1 |  |
|  | Frequency range | 875 | - | 895 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1919.6 | -41 | 0.3 | 7 |
|  |  | 1884.5 | - | 1915.7 |  |  | 8 |
| 7 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 20, 22, 26, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85, 87, 88, 103 NR Band n77,n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 15, 21, 26 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 15, 21, 26 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 15, 21 |
| 8 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 45, 50, 51, 54, 65, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 23 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 15, 23 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 23 |
| 9 | E-UTRA Band 1, 3, 11, 18, 19, 21, 26, 28, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 10 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 41, 43, 54, 66, 70, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 42, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 11 | E-UTRA Band 1, 3, 11, 18, 19, 21, 28, 34, 40, 42, 65 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 12 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 53, 54, 70, 71, 74, 103, 106 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 48, 50, 51, 66 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 13 | E-UTRA Band 2, 4, 5, 12, 13, 17, 25, 26, 27, 29, 41, 48, 50, 51, 53, 54, 66, 70, 71, 74, 85, 103, 106 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 14 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 24, 30, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 15 |
| 14 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 23, 24, 25, 26, 27, 29, 30, 41, 48, 53. 54, 66, 70, 71, 85, 103, 106 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 12, 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 12, 15 |
| 17 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 54, 70, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 50, 51, 53, 54, 66 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 48, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 18 | E-UTRA Band 1, 3, 11, 21, 34, 40, 42, 65 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 15 |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 19 | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 42, 65 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 20 | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 38, 42, 52, 69 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| 21 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 42, 65 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 22 | E-UTRA Band 1, 3, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 43, 65, 67, 68, 69, 72, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 3510 | - | 3525 | -40 | 1 | 15 |
|  | Frequency range | 3525 | - | 3590 | -50 | 1 |  |
| 23 | E-UTRA Band 4, 5, 12, 13, 14, 17, 23, 24, 26, 27, 29, 30, 41, 54, 66, 103, 106 | FDL_low | - | FDL_high | -50 | 1 |  |
| 24 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70, 71, 85, 103, 106 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 25 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 53, 54, 66, 70, 71, 85, 103, 106 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 26 | E-UTRA Band 1, 2, 3, 4, 5, 11, 12, 13, 14, 17, 18,19, 21, 24, 25, 26, 29, 30, 31, 34, 39, 40, 42, 43, 48, 50, 51, 65, 66, 70, 71, 73,74, 85, 103, 106 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 53, 54 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 15 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 27 | E-UTRA Band 1, 2, 3, 4, 5, 7, 12, 13, 14, 17, 25, 26, 27, 29, 30, 31, 38, 40, 41, 42, 43, 65, 66, 73, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 28 | FDL_low | - | 790 | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 |  |
| 28 | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 65, 66, 73, 74, 75, 76 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 19, 25 |
|  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 40, 41, 52, 72, 87, 88 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 19, 24 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 15, 35 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 34 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 15 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 19 |
| 30 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 48, 53, 54, 66, 70, 71, 85, 103, 106 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 31 | E-UTRA Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31, 32, 33, 34, 38, 40, 42, 43, 50, 51, 52, 65, 67, 68, 69, 74, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| … |  |  |  |  |  |  |  |
| 33 | E-UTRA Band 1, 7, 8, 20, 22, 28, 31, 32, 34, 38, 40, 42, 43, 52, 65, 67, 69, 72, 73, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 34 | E-UTRA Band 1, 3, 7, 8, 11, 18, 19, 20, 21, 22, 26, 28, 31, 32, 33, 38,39, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2, 5 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 35 |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  |  |
| 37 |  |  | - |  |  |  |  |
| 38 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 20, 22, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85, 87, 88, 103 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 15, 22, 26 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 15, 22 |
| 39 | E-UTRA Band 1, 8, 22, 26, 28, 34, 40, 41, 42, 44, 45, 50, 51, 52, 73, 74 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 |  | 1855 | -40 | 1 | 33 |
|  | Frequency range | 1855 |  | 1880 | -15.5 | 5 | 15,26,33 |
| 40 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13 , 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44, 45, 48, 50, 51, 52, 54, 65, 66, 70, 71, 73, 74, 85, 103, 106 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 30 |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 8, 30 |
| 42 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 18, 19, 20, 21, 25, 26, 27, 28, 31, 32, 33, 34, 38, 40, 41, 44, 45, 50, 51, 65, 66, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 43 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 20, 25, 26, 27, 28, 31,32, 33, 34, 38, 40, 50, 51, 65, 66, 67, 68, 69, 72, 73, 74, 75, 76, 85, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
| 44 | E-UTRA Band 1, 40, 42, 45 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 5, 8, 34, 39, 41, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
| 45 | E-UTRA Band 1, 3, 5, 8, 34, 39, 40, 41, 42, 44, 52, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
| … |  |  |  |  |  |  |  |
| 47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 65, 68, 72, 73 NR band n77, n78 , n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 5925 | - | 5950 | -30 EIRP | 1 | 38, 40, 43 |
|  | Frequency range | 5815 | - | 5855 | -30 EIRP | 1 | 38, 43, 45 |
| 48 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 54, 66, 70, 71, 74, 85, 103, 106 | FDL_low | - | FDL_high | -50 | 1 |  |
| 50 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| 51 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| 52 | E-UTRA Band 1, 3, 5, 7, 8, 20, 28, 31, 33, 34, 38, 39, 40, 41, 45, 47, 50, 51, 68, 72, 73, 74, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
| 53 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 48, 54, 66, 70, 71, 85, 103, 106 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 |  |
| 54 | E-UTRA Band 2, 4, 5, 8, 12, 13, 14, 17, 23, 24, 25, 26, 29, 30, 48, 50, 51, 53, 66, 70, 71, 85, 103, 106 NR NTN Band n255, n256 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| … |  |  |  |  |  |  |  |
| 65 | E-UTRA Band 1, 3, 7, 8, 20, 22, 28, 31, 32, 38, 40, 42, 43, 50, 51, 65, 68, 69, 72, 74, 75, 76, 87, 88 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 5, 11, 18, 19, 21, 26, 27, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 36 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 37 |
|  | Frequency range | 1900 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 15, 26, 27 |
| 66 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 53, 66, 70, 71, 74, 85, 103, 106 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 68 | E-UTRA Band 3, 7, 8, 20, 28, 31, 38, 40, 47, 72, 74, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 22, 42, 43, 50, 51, 52, 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| … |  |  |  |  |  |  |  |
| 70 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 53, 66, 70, 71, 85, 103, 106 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 53, 54, 66, 85, 103, 106 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 15 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 72 | E-UTRA Band 1, 7, 20, 22, 28, 31, 32, 33, 34, 38, 42, 43, 47, 52, 65, 68, 72, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 8, 40 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| 73 | E-UTRA Band 1, 26, 28, 33, 34, 39, 41, 42, 43, 44, 45, 47, 52 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 5, 8, 27, 40 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 74 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 18, 19, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85, 103 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 1400 | - | 1427 | -32 | 27 | 15, 41 |
|  | Frequency range | 1475 | - | 1488 | -50 | 1 | 42 |
|  | Frequency range | 1488 | - | 1518 | -50 | 1 | 15 |
| 85 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 53, 54, 70, 71, 74, 103, 106 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 48, 51, 66 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 87 | E-UTRA Band 1, 3, 7, 8, 22, 28, 31, 32, 33, 34, 38, 40, 42, 43, 47, 52, 65, 68, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band, 20 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 87, 88 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| 88 | E-UTRA Band 1, 3, 7, 8, 20, 22, 28, 31, 32, 33, 34, 38, 40, 42, 43, 47, 52, 65, 68, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 87 | FDL_low | - | FDL_high | -20 | 1 | 15 |
|  | E-UTRA Band 88 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| 103 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 25, 26, 27, 29, 41, 48, 50, 51, 53, 54, 66, 70, 71, 74, 85, 103, 106 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 24, 30, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 |  |  |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 |  |  |
| 106 | E-UTRA Band 2, 4, 12, 13, 14, 23, 24, 25, 30, 53, 54, 66, 70, 71, 85, 103, 106 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 41, 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 5, 26 | FDL_low | - | FDL_high | -30 | 1 |  |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: N/A NOTE 4: N/A NOTE 5: For non-synchronised TDD operation to meet these requirements some restriction will be needed for either the operating band or protected band NOTE 6: N/A NOTE 7: Applicable when co-existence with PHS system operating in 1884.5 -1919.6MHz. NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 9: N/A NOTE 10: N/A NOTE 11: Whether the applicable frequency range should be 793-805MHz instead of 799-805MHz is TBD NOTE 12: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB NOTE 13: N/A NOTE 14: N/A NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 16: N/A NOTE 17: N/A NOTE 18: N/A NOTE 19: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 20: N/A NOTE 21: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. For power class 2 UE for any channel bandwidths within the range 2570 - 2615 MHz, NS_44 shall apply. For power class 2 or 3 UE for carriers with channel bandwidth overlapping the frequency range 2615 - 2620 MHz the requirement applies with the maximum output power configured to +19 dBm in the IE P-Max. NOTE 23: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 24: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 25: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 27: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 28: N/A NOTE 29: N/A NOTE 30: This requirement applies when the E-UTRA carrier is confined within 2545-2575MHz or 2595-2645MHz and the channel bandwidth is 10 or 20 MHz NOTE 31: N/A NOTE 32: Void NOTE 33: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 34: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. NOTE 35: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 36: This requirement is applicable for E-UTRA channel bandwidth allocated within 1920-1980 MHz. NOTE 37: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980MHz. NOTE 38: Applicable when NS_33 or NS_34 is configured by the pre-configured radio parameters. NOTE 39: Applicable only when the assigned E-UTRA carrier is confined within 824 MHz and 849 MHz for UE category M1, M2 and UE category NB1 and NB2. NOTE 40: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. NOTE 41: Applicable for all bandwidths, and when the lower edge of the assigned E-UTRA UL channel bandwidth frequency is greater than or equal to 1427 MHz + the channel BW assigned for 1.4, 3, 5 and 10 MHz bandwidth, and when the lower edge of the assigned E-UTRA UL channel bandwidth frequency is greater than or equal to 1440 MHz for 15 and 20 MHz bandwidth. This requirement shall be verified with UE transmission power of 15 dBm. NOTE 42: Applicable for 1.4 , 3 and 5 MHz bandwidth, and when the upper edge of the assigned E-UTRA UL channel bandwidth frequency is less than or equal to 1467 MHz assigned for10 MHz bandwidth, and when the upper edge of the assigned E-UTRA UL channel bandwidth frequency is less than or equal to 1463.8 MHz for 15 MHz bandwidth, and when the upper edge of the assigned E-UTRA UL channel bandwidth frequency is less than or equal to 1460.8 MHz for 20 MHz bandwidth. NOTE 43: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I. NOTE 44: For category NB1 and NB2 UE when carrier centre frequency is 1920.1 MHz, in case of single-tone uplink transmission the requirement is applicable only for sub-carrier index > 2. NOTE 45: Resolution BW is 10% of the measurement BW and the result should be integrated to achieve the measurement bandwidth. The sweep time shall be set at least as (sweep points)*(symbol length) to improve the measurement accuracy. |  |  |  |  |  |  |  |

##### 6.6.3.2.4	Test description

6.6.3.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 6.6.3.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.3.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) (Note 6, Note 7) |  |  |  | Low range, Mid range, High range (Note 6, 13, 16) |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Lowest, 5MHz, Highest (Note 13, 16) |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 1.4MHz | N/A for Spurious Emissions testing |  |  | QPSK | 6 | 6 |
| 1.4MHz |  |  |  | QPSK | 1 | 1 |
| 3MHz |  |  |  | QPSK | 15 | 15 |
| 3MHz |  |  |  | QPSK | 1 | 1 |
| 5MHz |  |  |  | QPSK | 25 | 2512 |
| 5MHz |  |  |  | QPSK | 1 | 112 |
| 5MHz |  |  |  | QPSK | 208 | - |
| 10MHz |  |  |  | QPSK | 50 | 50 |
| 10MHz |  |  |  | QPSK | 1 | 1 |
| 10MHz |  |  |  | QPSK | 3015 |  |
| 10MHz |  |  |  | QPSK | 329 | - |
| 10MHz |  |  |  | QPSK | 110, 14 | - |
| 15MHz |  |  |  | QPSK | 75 | 75 |
| 15MHz |  |  |  | QPSK | 543, 11 | 544, 18 |
| 15MHz |  |  |  | QPSK | 1 | 15 |
| 20MHz |  |  |  | QPSK | 100 | 100 |
| 20MHz |  |  |  | QPSK | 543, 11 | 544, 18 |
| 20MHz |  |  |  | QPSK | 1 | 15 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The 1 RB allocation shall be tested at both RB #0 and RB #max. Note 3: To be used for requirements under note 15 in table 6.6.3.2.3-1 and table 6.6.3.2.3-1A, note 16 in table 6.6.3.2.3-1B, note 21 in tables 6.6.3.2.3-1C to 6.6.3.2.3-1I in high range at RB#(full allocation – 54), instead of full allocation. Note 4: To be used for requirements under note 16 in table 6.6.3.2.3-1 and table 6.6.3.2.3-1A, note 17 in table 6.6.3.2.3-1B, note 22 in tables 6.6.3.2.3-1C to 6.6.3.2.3-1I in high range - 5MHz at RB#(full allocation – 54), instead of full allocation. Note 5: For requirements under note 16 in table 6.6.3.2-1 and table 6.6.3.2.3-1A, note 17 in table 6.6.3.2.3-1B, note 22 in tables 6.6.3.2.3-1C to 6.6.3.2.3-1I (high range – 5MHz) is tested instead of high channel. Note 6: Do not apply for requirements under Note 13 in Tables 6.6.3.2.3-1 and 6.6.3.2.3-1A. Test frequencies for these requirements are defined in Table 6.6.3.2.4.1-2. Note 7: Void Note 8: For requirements under note 17 in table 6.6.3.2.3-1A, note 18 in table 6.6.3.2.3-1B, note 23 in tables 6.6.3.2.3-1C to 6.6.3.2.3-1I (high range – 10MHz) is tested with RB #0 to RB #19 used. Note 9: For requirements under note 17 in table 6.6.3.2.3-1A, note 18 in table 6.6.3.2.3-1B, note 23 in tables 6.6.3.2.3-1C to 6.6.3.2.3-1I high range is tested with RB #4 to RB #35 used. Note 10: For requirements under note 17 in table 6.6.3.2.3-1A, note 18 in table 6.6.3.2.3-1B, note 23 in tables 6.6.3.2.3-1C to 6.6.3.2.3-1I high range is tested with RB #4 and (RB#max -4) used. Note 11: To be used for requirements under note 17 in table 6.6.3.2.3-1, note 18 in table 6.6.3.2.3-1A, note 19 in table 6.6.3.2.3-1B and note 27 in tables 6.6.3.2.3-1C 6.6.3.2.3-1I in low range at RB#0, instead of full allocation. Note 12: For requirements under note 16 in table 6.6.3.2.3-1 and table 6.6.3.2.3-1A, note 17 in table 6.6.3.2.3-1B, note 22 in tables 6.6.3.2.3-1C to 6.6.3.2.3-1I, the message exception in Table 6.6.3.2.4.3-1 is used to test with the high range. Note 13: For requirements under note 31 in table 6.6.3.2.3-1C and note 34 in tables 6.6.3.2.3-1D to 6.6.3.2.3-1I only 720.5 MHz is tested with 5 MHz and only 723 MHz is tested with 10 MHz. Note 14: For requirements under note 31 in table 6.6.3.2.3-1C and note 34 in tables 6.6.3.2.3-1D to 6.6.3.2.3-1I RB #2 and RB #47 used. Note 15: To be used for requirements under note 31 in table 6.6.3.2.3-1C and note 34 in tables 6.6.3.2.3-1D to 6.6.3.2.3-1I at RB#2, instead of full allocation. Note 16: For requirements under note 32 in table 6.6.3.2.3-1C and note 35 in tables 6.6.3.2.3-1D to 6.6.3.2.3-1I only low range and 728 MHz with 10 MHz is tested. Note 17: Void. Note 18: To be used for requirements under note 33 in tables 6.6.3.2.3-1D to 6.6.3.2.3-1I (high range-18 MHz for 15MHz channel bandwidth and high range-7 MHz for 20MHz channel bandwidth), instead of full allocation and high range. RB allocation shall start at RBstart=0. |  |  |  |  |  |  |

Table 6.6.3.2.4.1-2: Test frequencies for E-UTRA channel bandwidth for operating band 3 
with Note 13 (in Tables 6.6.3.2.3-1 and 6.6.3.2.3-1A)

| Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- |
| 1.4 | 19942 | 1784.2 | 1942 | 1879.2 |
| 3 | 19934 | 1783.4 | 1934 | 1878.4 |
| 5 | 19924 | 1782.4 | 1924 | 1877.4 |
| 10 | 19899 | 1779.9 | 1899 | 1874.9 |
| 15 | 19874 | 1777.4 | 1874 | 1872.4 |
| 20 | 19849 | 1774.9 | 1849 | 1869.9 |
| Note: 1.4 and 3 MHz only tested for Rel8 and Rel9. |  |  |  |  |

1.	Connect the SS to the UE to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure A.7.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.3.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.3.2.4.3.

6.6.3.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.3.2.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.6.3.2.3-1 to 6.6.3.2.3-1G. For band 14 measurements made in a bandwidth of 6.25kHz, measurement parameter settings defined in table 6.6.3.2.4.2-1 shall be used. The centre frequency of the filter shall be stepped in contiguous steps according to table 6.6.3.2.5-1. The measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS.

Table 6.6.3.2.4.2-1: Measurement setup for band 14

|  | Option 1: Measurement with No RMS VBW available | Option2 Measurement with VBW Filtering on Power scale |
| --- | --- | --- |
| VBW | >=62.5 kHz (10 times or more the RBW) | <=43Hz |
| RBW | <=6.25kHz | <=6.25kHz |
| Detector type | Averages signal envelope during each measurement point, such as "RMS detector" | Peak |
| Averaging mode (Trace averaging) | Power (RMS voltage) | Power (RMS voltage), as controlled by "Average Type" |
| Average Type (applies to detector) | Power (RMS voltage) (automatically occurs with "RMS detector") | Not applicable |
| Average Type (applies to VBW filter) | Not applicable | Power (RMS voltage) |
| Number of averages | 30, to reduce variance as required, or use an even longer sweep time | 1 or use an even narrower VBW filter, thus a longer sweep time |
| Sweep time | [Don't specify] | Sweep rate (span divided by sweep time) <= 0.8 * RBW*VBW |

6.6.3.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the following exceptions:

Table 6.6.3.2.4.3-1: SystemInformationBlockType1
for Note 16 in table 6.6.3.2.3-1 and table 6.6.3.2.3-1A,
Note 17 in table 6.6.3.2.3-1B, Note 22 in table 6.6.3.2.3-1C, table 6.6.3.2.3-1D, table 6.6.3.2.3-1E, table 6.6.3.2.3-1F and table 6.6.3.2.3-1G

| Derivation Path: TS 36.508 [7] clause 4.4.3.2, Table 4.4.3.2-3 SystemInformationBlockType1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 19 |  |  |

##### 6.6.3.2.5	Test requirement

Test requirements for Spurious Emissions UE Co-existence are the same as the minimum requirements and are not repeated in this section.

The measured average power of spurious emission, derived in step 3, shall not exceed the described value in tables 6.6.3.2.3-1 to 6.6.3.2.3-1G according to the following rule:

The requirements for the UE are release specific and can be found in Tables 6.6.3.2.3-1 to 6.6.3.2.3-1G. If the UE support a band, which is not defined in the table corresponding UE’s release, the requirements for this band are taken from the table of earliest release where requirements for this band are defined. This has been described in following Table 6.6.3.2.5-1.

Table 6.6.3.2.5-1: UE Requirements according to UE E-UTRA release and supported E-UTRA band

| Band | UE Requirements per release |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Rel-8 | Rel-9 | Rel-10 | Rel-11 | Rel-12 | Rel-13 |  | Rel-14 | Rel-15 | Rel-16 | Rel-17 | Rel-18 |
| 1 | Table 6.6.3.2.3-1 | Table 6.6.3.2.3-1A | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 2 | Table 6.6.3.2.3-1 | Table 6.6.3.2.3-1A | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 3 | Table 6.6.3.2.3-1 | Table 6.6.3.2.3-1A | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 4 | Table 6.6.3.2.3-1 | Table 6.6.3.2.3-1A | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 5 | Table 6.6.3.2.3-1 | Table 6.6.3.2.3-1A | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 6 | Table 6.6.3.2.3-1 | Table 6.6.3.2.3-1A | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 7 | Table 6.6.3.2.3-1 | Table 6.6.3.2.3-1A | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 8 | Table 6.6.3.2.3-1 | Table 6.6.3.2.3-1A | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 9 | Table 6.6.3.2.3-1 | Table 6.6.3.2.3-1A | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 10 | Table 6.6.3.2.3-1 | Table 6.6.3.2.3-1A | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 11 | Table 6.6.3.2.3-1 | Table 6.6.3.2.3-1A | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 12 | Table 6.6.3.2.3-1 | Table 6.6.3.2.3-1A | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 13 | Table 6.6.3.2.3-1 | Table 6.6.3.2.3-1A | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 14 | Table 6.6.3.2.3-1 | Table 6.6.3.2.3-1A | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| … |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 | Table 6.6.3.2.3-1 | Table 6.6.3.2.3-1A | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 18 | Table 6.6.3.2.3-1A | Table 6.6.3.2.3-1A | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 19 | Table 6.6.3.2.3-1A | Table 6.6.3.2.3-1A | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 20 | Table 6.6.3.2.3-1A | Table 6.6.3.2.3-1A | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 21 | Table 6.6.3.2.3-1A | Table 6.6.3.2.3-1A | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 22 | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 23 | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 24 | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 25 | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 26 | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 27 | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 28 | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 30 | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 31 | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| … |  |  |  |  |  |  |  |  |  |  |  |  |
| 33 | Table 6.6.3.2.3-1 | Table 6.6.3.2.3-1A | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 34 | Table 6.6.3.2.3-1 | Table 6.6.3.2.3-1A | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 35 |  |  |  |  |  |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  |  |  |  |  |  |  |
| 37 |  |  |  |  |  |  |  |  |  |  |  |  |
| 38 | Table 6.6.3.2.3-1 | Table 6.6.3.2.3-1A | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 39 | Table 6.6.3.2.3-1 | Table 6.6.3.2.3-1A | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 40 | Table 6.6.3.2.3-1 | Table 6.6.3.2.3-1A | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 41 | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 42 | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 43 | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1B | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 44 | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1C | Table 6.6.3.2.3-1D | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 45 | Table 6.6.3.2.3-1E | Table 6.6.3.2.3-1E | Table 6.6.3.2.3-1E | Table 6.6.3.2.3-1E | Table 6.6.3.2.3-1E | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| … |  |  |  |  |  |  |  |  |  |  |  | Table 6.6.3.2.3-1J |
| 48 | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1F |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| ….. |  |  |  |  |  |  |  |  |  |  |  |  |
| 53 | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1H |  | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 54 | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1H |  | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| … |  |  |  |  |  |  |  |  |  |  | Table 6.6.3.2.3-1I |  |
| 65 | N/A 1 | Table 6.6.3.2.3-1E | Table 6.6.3.2.3-1E | Table 6.6.3.2.3-1E | Table 6.6.3.2.3-1E | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 66 | N/A 2 | N/A 2 | Table 6.6.3.2.3-1E | Table 6.6.3.2.3-1E | Table 6.6.3.2.3-1E | Table 6.6.3.2.3-1E |  | Table 6.6.3.2.3-1F | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| … |  |  |  |  |  |  |  |  |  |  |  |  |
| 68 | N/A 1 | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G |  | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| … |  |  |  |  |  |  |  |  |  |  |  |  |
| 71 | N/A 1 | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G |  | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 72 | N/A 1 | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G |  | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| ….. |  |  |  |  |  |  |  |  |  |  |  |  |
| 74 | N/A 1 | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G |  | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 87 | N/A 1 | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G |  | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 88 | N/A 1 | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G |  | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1H | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 72 | N/A 1 | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G |  | Table 6.6.3.2.3-1G |  | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 74 | N/A 1 | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G |  | Table 6.6.3.2.3-1G |  | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 85 | N/A 1 | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G | Table 6.6.3.2.3-1G |  | Table 6.6.3.2.3-1G |  | Table 6.6.3.2.3-1I | Table 6.6.3.2.3-1J |
| 106 | N/A 1 | Table 6.6.3.2.3-1J | Table 6.6.3.2.3-1J | Table 6.6.3.2.3-1J | Table 6.6.3.2.3-1J | Table 6.6.3.2.3-1J | Table 6.6.3.2.3-1J |  | Table 6.6.3.2.3-1J | Table 6.6.3.2.3-1J | Table 6.6.3.2.3-1J | Table 6.6.3.2.3-1J |
| Note 1: Operating band with number > 64 requires Rel-9 signalling (refer to TS 36.307 Ch. 3A.1 [16]) and tests can only be done from Rel-9 and onwards. Note 2: Asymmetric operating band is release independent from release 10 (refer to TS 36.307 Ch. 3A.1 [16]) and tests can only be done from Rel-10 and onwards. Note 3: Protected bands with Note 38 in Table 6.6.3.2.3-1E are tested in 6.6.3EA.2 or 6.6.3F.2. |  |  |  |  |  |  |  |  |  |  |  |  |

NOTE 1:	The frequency range applicable with network signalled values of NS_05, NS_08, NS_09, NS_15,NS_16, NS_27, NS_36, NS_45 are covered in 6.6.3.3 Additional Spurious Emissions.

NOTE 2:	The following is applied to Note2 in Table 6.6.3.2.3-1, Table 6.6.3.2.3-1A, Table 6.6.3.2.3-1B, Table 6.6.3.2.3-1C and Table 6.6.3.2.3-1E. For frequency with 2nd, 3rd or 4th harmonic spurious emissions, the measurements are covered in 6.6.3.1.

NOTE 2:	The restriction on the maximum uplink transmission to 54 RB in Notes 14 and 15 of Table 6.6.3.2.5-1 is intended for conformance testing and may be applied to network operation to facilitate coexistence when the aggressor and victim bands are deployed in the same geographical area. The applicable spurious emission requirement of -15.5 dBm/5MHz is a least restrictive technical condition for FDD/TDD coexistence and may have to be revised in the future.

-------- End of changes --------