3GPP TSG-RAN5 Meeting #103	R5-242392

# Fukuoka, Japan, 20 – 24 May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5305 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding less than 5 MHz BW into Bandwidth dependent parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_900MHz_US-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Work Item introduce band 106 which is specified only for 1.4 MHz and 3 MHz channel bandwidths. Less than 5 MHz bandwidth dependent parameters are not specified |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added 3 MHz BW into: - Table 7.1.5.1.3.2-2: Bandwidth dependent parameters - Table 7.1.7.1.1.3.2-2a: Bandwidth Dependent Parameters and corresponding tables in sub clauses 7.1.7.1.1 - 7.1.7.1.12a, 7.1.7.2.1, 7.1.7.2.4 - Table A.8.21-3: Cell specific test parameters for E-UTRAN FDD-FDD inter frequency cell proximity detection test case 3 MHz is added as default and maximum for less than 5 MHz bandwidth |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Less than 5MHz Bandwidth dependent parameters will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.5.1.3.2, 7.1.7.1, 7.1.7.2, 8.3.4.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of changes --------

### 7.1.5	PUSCH Hopping

#### 7.1.5.1	Inter-TTI PUSCH hopping by uplink grant

7.1.5.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state and ‘Hopping-mode’ is set to ‘InterSubFrame’}

ensure that {

when { UE receives for a TTI an uplink grant with ‘Hopping flag’ set as 1, ‘NUL_hop’ bits indicating Type 1 PUSCH Hopping }

then { UE transmits UL data on resource blocks as per type 1 PUSCH hopping}

}

-------- Skipped unchanged clauses --------

7.1.5.1.3.2	Test procedure sequence

Table 7.1.5.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS ignores scheduling requests and does not allocate any uplink grant. | - | - | - | - |
|  | Exception: Steps 2 to 5 are executed as per table 7.1.5.1.3.2-2 |  |  |  |  |
| 2 | SS transmits a MAC PDU including 8 (FDD)/4(TDD) RLC SDU’s | <-- | MAC PDU | - | - |
| 3 | The SS waits for 60 ms | - | - | - | - |
|  | Exception: Steps 4 and 5 are repeated such that UE sends data in 8 ([FDD)/4(TDD) consecutive UL TTI’s. |  |  |  |  |
| 4 | The SS is configured for Uplink Grant Allocation Type 2. SS transmits an UL Grant, allowing the UE to return 1 RLC SDU as received in step 2. Hopping flag’ set as 1, all bits in ‘NUL_hop’ are set as per table 7.1.5.3.2-2 for various execution runs | <-- | (UL Grant (C-RNTI)) | - | - |
| 5 | Check: Does the UE transmit a MAC PDU corresponding to grant in step 4? | - | - | 1 | P |
| Note 1: Steps 4 and 5 are repeated for 8 times corresponding to 8 sub frames for FDD and 4 for default TDD configuration 1. Note 2: The grant allocated in step 4 is such that the loop back PDU's are received in 8 (FDD)/4(TDD) UL TTI's |  |  |  |  |  |

Table 7.1.5.1.3.2-2: Bandwidth dependent parameters

| System BW | Number of Hopping bits | Execution Counter K | Information in hopping bits |
| --- | --- | --- | --- |
| 3 MHz | 1 | 1 | 1 |
| 5 MHz | 1 | 1 | 0 |
| 10/20 MHz | 2 | 1 | 00 |
|  |  | 2 | 01 |
|  |  | 3 | 10 |

-------- Skipped unchanged clauses --------

#### 7.1.7.1	DL-SCH transport block size selection

##### 7.1.7.1.1	DL-SCH transport block size selection / DCI format 1 / RA type 0

7.1.7.1.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE on PDCCH receives DCI format 1 indicating Resource Allocation Type 0, a resource block assignment correspondent to  physical resource blocks and a modulation and coding scheme  }

then { UE decodes the received transport block of size correspondent to the read  and  and forwards it to higher layers }

}

-------- Skipped unchanged clauses --------

7.1.7.1.1.3.2	Test procedure sequence

Table 7.1.7.1.1.3.2-1: Maximum TBsize for different UE categories

| UE Category | Maximum number of bits of a DL-SCH transport block received within a TTI |
| --- | --- |
| Category 0 | 1000 |
| Category 1bis | 10296 |
| Category 1 | 10296 |
| Category 2 | 51024 |
| Category 3 | 75376 |
| Category 4 | 75376 |
| Category 5 | 149776 |
| Category 6 | 149776 |
| Category 7 | 149776 |
| Category 8 | 299856 |
| Category 9 | 149776 |
| Category 10 | 149776 |
| Category 11 | 149776 |
| Category 12 | 149776 |
| Category 13 | 195816 |
| Category 14 | 391656 |
| Category 15 | 149776 |
| Category 16 | 149776 |
| Category 17 | 391656 |
| Category 18 | 299856 |
| Category 19 | 299856 |
| Category 20 | 299856 |
| Category 21 | 149776 |

Table 7.1.7.1.1.3.2-2: Number of downlink PDCP SDUs and PDCP SDU size used as test data

| TBsize [bits] | Number of PDCP SDUs | PDCP SDU size [bits] See note 1 |
| --- | --- | --- |
| 104 ≤ TBsize ≤ 12096 note 2 | 1 | 8*FLOOR((TBsize – 96)/8) |
| 12097 ≤ TBsize ≤ 24128 | 2 | 8*FLOOR((TBsize – 128)/16) |
| 24129 ≤ TBsize ≤ 36152 | 3 | 8*FLOOR((TBsize – 152)/24) |
| 36153 ≤ TBsize ≤ 48184 | 4 | 8*FLOOR((TBsize – 184)/32) |
| 48185 ≤ TBsize ≤ 60208 | 5 | 8*FLOOR((TBsize – 208)/40) |
| 60209 ≤ TBsize ≤ 72240 | 6 | 8*FLOOR((TBsize – 240)/48) |
| TBsize> 72240 | 7 | 8*FLOOR((TBsize – 264)/56) |
| Note 1. Each PDCP SDU is limited to 1500 octets (to keep below maximum SDU size of ESM as specified in TS 24.301 clause 9.9.4.12). The PDCP SDU size of each PDCP SDU is PDCP SDU size = (TBsize – N*PDCP header size - AMD PDU header size - MAC header size – Size of Timing Advance – RLC Status PDU size) / N, where PDCP header size is 16 bits for the RLC AM and 12-bit SN case; AMD PDU header size is CEIL[(16+(N-1)*12)/8] bytes which includes 16 standard AM header and (N-1) Length indicators; and MAC header size = 40 bits as MAC header size can be 1) R/R/E/LCID MAC subheader (8 bits for Timing Advance) + R/R/E/LCID MAC subheader (16 bits for MAC SDU for RLC Status PDU)R/R/E/LCID MAC subheader (8 bits for MAC SDU) = for AMD PDU 8 + 8 + 16bits = 32 bits Or 2) R/R/E/LCID MAC subheader (8 bits for Timing Advance) + R/R/E/LCID MAC subheader (24 bits for MAC SDU for AMD PDU, Note: Length can be of 2 bytes depending upon the size of AMD PDU)+ R/R/E/LCID MAC subheader (8 bits for MAC SDU for RLC Status PDU) = 8+24 + 8 bits = 40 bits Therefore Maximum MAC header size can be 40 bits Size of Timing Advance MAC CE is 8 bits (if no Timing Advance and/or RLC status needs to be sent, padding will occur instead) RLC Status PDU size = 16 bits This gives: PDCP SDU size = 8*FLOOR((TBsize – N*16- 8*CEIL((16+(N-1)*12)/8) – 64)/(8*N)) bits Note 2: According to TS 36.213 Table 7.1.7.2.1-1 and the final PDCP SDU size formula in Note 1, the smallest TBsize that can be tested is 104 bits. |  |  |

Table 7.1.7.1.1.3.2-2a: Bandwidth Dependent Parameters

| Max Bandwidth | Max | Allowed Values |
| --- | --- | --- |
| 3 MHz | 15 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15 |
| 5 MHz | 25 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25 |
| 10 MHz | 50 | 2, 3, 5, 6, 8, 9, 11, 12, 14, 15, 17, 18, 20, 21, 23, 24, 26, 27, 29, 30, 32, 33, 35, 36, 38, 39, 41, 42, 44, 45, 47, 48, 50 |
| 15 MHz | 75 | 3, 4, 7, 8, 11, 12, 15, 16, 19, 20, 23, 24, 27, 28, 31, 32, 35, 36, 39, 40, 43, 44 ,47, 48, 51, 52, 55, 56, 59, 60, 63, 64, 67, 68, 71, 72, 75 |
| 20 MHz | 100 | 4, 8, 12, 16, 20, 24, 28, 32, 36, 40, 44, 48, 52, 56, 60, 64, 68, 72, 76, 80, 84, 88, 92, 96, 100 |
| Note : Maximum bandwidth for EUTRA bands is 3/5/10/15/20 MHz. |  |  |

-------- Skipped unchanged clauses --------

Table 7.1.7.1.2.3.2-2a: Bandwidth Dependent Parameters

| Max Bandwidth | Max |  |
| --- | --- | --- |
| 3 MHz | 15 | 6 |
| 5 MHz | 25 | 11 |
| 10 MHz | 50 | 14 |
| 15 MHz | 75 | 16 |
| 20 MHz | 100 | 22 |
| Note : Maximum bandwidth for EUTRA bands is 3/5/10/15/20 MHz. |  |  |

-------- Skipped unchanged clauses --------

Table 7.1.7.1.3.3.2-2a: Bandwidth Dependent Parameters

| Max Bandwidth | Max |
| --- | --- |
| 3 MHz | 15 |
| 5 MHz | 25 |
| 10 MHz | 50 |
| 15 MHz | 75 |
| 20 MHz | 100 |
| Note : Maximum bandwidth for EUTRA bands is 3/5/10/15/20 MHz. |  |

-------- Skipped unchanged clauses --------

Table 7.1.7.1.4.3.2-2a: Bandwidth Dependent Parameters

| Max Bandwidth | Max | MAX VRB |
| --- | --- | --- |
| 3 MHz | 15 | 14 |
| 5 MHz | 25 | 24 |
| 10 Mhz | 50 | 16 |
| 15 Mhz | 75 | 16 |
| 20 Mhz | 100 | 16 |
| Note : Maximum bandwidth for EUTRA bands is 3/5/10/15/20 MHz. |  |  |

-------- Skipped unchanged clauses --------

Table 7.1.7.1.5.3.2-2a: Bandwidth Dependent Parameters

| Max Bandwidth | Max | Allowed Values |
| --- | --- | --- |
| 3 MHz | 15 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15 |
| 5 MHz | 25 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25 |
| 10 MHz | 50 | 2, 3, 5, 6, 8, 9, 11, 12, 14, 15, 17, 18, 20, 21, 23, 24, 26, 27, 29, 30, 32, 33, 35, 36, 38, 39, 41, 42, 44, 45, 47, 48, 50 |
| 15 MHz | 75 | 3, 4, 7, 8, 11, 12, 15, 16, 19, 20, 23, 24, 27, 28, 31, 32, 35, 36, 39, 40, 43, 44 ,47, 48, 51, 52, 55, 56, 59, 60, 63, 64, 67, 68, 71, 72, 75 |
| 20 MHz | 100 | 4, 8, 12, 16, 20, 24, 28, 32, 36, 40, 44, 48, 52, 56, 60, 64, 68, 72, 76, 80, 84, 88, 92, 96, 100 |
| Note: Maximum bandwidth for EUTRA bands is 3/5/10/15/20 MHz. |  |  |

-------- Skipped unchanged clauses --------

Table 7.1.7.1.6.3.2-2a: Bandwidth Dependent Parameters

| Max Bandwidth | Max | Allowed Values |
| --- | --- | --- |
| 3 MHz | 15 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15 |
| 5 MHz | 25 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25 |
| 10 MHz | 50 | 2, 3, 5, 6, 8, 9, 11, 12, 14, 15, 17, 18, 20, 21, 23, 24, 26, 27, 29, 30, 32, 33, 35, 36, 38, 39, 41, 42, 44, 45, 47, 48, 50 |
| 15 MHz | 75 | 3, 4, 7, 8, 11, 12, 15, 16, 19, 20, 23, 24, 27, 28, 31, 32, 35, 36, 39, 40, 43, 44 ,47, 48, 51, 52, 55, 56, 59, 60, 63, 64, 67, 68, 71, 72, 75 |
| 20 MHz | 100 | 4, 8, 12, 16, 20, 24, 28, 32, 36, 40, 44, 48, 52, 56, 60, 64, 68, 72, 76, 80, 84, 88, 92, 96, 100 |
| Note: Maximum bandwidth for EUTRA bands is 3/5/10/15/20 MHz. |  |  |

-------- Skipped unchanged clauses --------

Table 7.1.7.1.6a.3.2-2a: Bandwidth Dependent Parameters (RA type 0)

| Max Bandwidth | Max | Allowed Values |
| --- | --- | --- |
| 3 MHz | 15 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15 |
| 5 MHz | 25 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25 |
| 10 MHz | 50 | 2, 3, 5, 6, 8, 9, 11, 12, 14, 15, 17, 18, 20, 21, 23, 24, 26, 27, 29, 30, 32, 33, 35, 36, 38, 39, 41, 42, 44, 45, 47, 48, 50 |
| 15 MHz | 75 | 3, 4, 7, 8, 11, 12, 15, 16, 19, 20, 23, 24, 27, 28, 31, 32, 35, 36, 39, 40, 43, 44 ,47, 48, 51, 52, 55, 56, 59, 60, 63, 64, 67, 68, 71, 72, 75 |
| 20 MHz | 100 | 4, 8, 12, 16, 20, 24, 28, 32, 36, 40, 44, 48, 52, 56, 60, 64, 68, 72, 76, 80, 84, 88, 92, 96, 100 |
| Note: Maximum bandwidth for EUTRA bands is 3/5/10/15/20 MHz. |  |  |

Table 7.1.7.1.6a.3.2-2b: Ambiguous Sizes of Information Bits

{12, 14, 16 ,20, 24, 26, 32, 40, 44, 56}

Table 7.1.7.1.6a.3.2-2c: Bandwidth Dependent Parameters (RA type 1)

| Max Bandwidth | Max |  |
| --- | --- | --- |
| 3 MHz | 15 | 6 |
| 5 MHz | 25 | 11 |
| 10 MHz | 50 | 14 |
| 15 MHz | 75 | 16 |
| 20 MHz | 100 | 22 |
| Note : Maximum bandwidth for EUTRA bands is 3/5/10/15/20 MHz. |  |  |

-------- Skipped unchanged clauses --------

Table 7.1.7.1.7.3.2-2a: Bandwidth Dependent Parameters

| Max Bandwidth | Max | Allowed Values |
| --- | --- | --- |
| 3 MHz | 15 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15 |
| 5 MHz | 25 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25 |
| 10 MHz | 50 | 2, 3, 5, 6, 8, 9, 11, 12, 14, 15, 17, 18, 20, 21, 23, 24, 26, 27, 29, 30, 32, 33, 35, 36, 38, 39, 41, 42, 44, 45, 47, 48, 50 |
| 15 MHz | 75 | 3, 4, 7, 8, 11, 12, 15, 16, 19, 20, 23, 24, 27, 28, 31, 32, 35, 36, 39, 40, 43, 44 ,47, 48, 51, 52, 55, 56, 59, 60, 63, 64, 67, 68, 71, 72, 75 |
| 20 MHz | 100 | 4, 8, 12, 16, 20, 24, 28, 32, 36, 40, 44, 48, 52, 56, 60, 64, 68, 72, 76, 80, 84, 88, 92, 96, 100 |
| Note: Maximum bandwidth for EUTRA bands is 3/5/10/15/20 MHz. |  |  |

-------- Skipped unchanged clauses --------

Table 7.1.7.1.8.3.2-2a: Bandwidth Dependent Parameters

| Max Bandwidth | Max | Allowed Values |
| --- | --- | --- |
| 3 MHz | 15 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15 |
| 5 MHz | 25 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25 |
| 10 MHz | 50 | 2, 3, 5, 6, 8, 9, 11, 12, 14, 15, 17, 18, 20, 21, 23, 24, 26, 27, 29, 30, 32, 33, 35, 36, 38, 39, 41, 42, 44, 45, 47, 48, 50 |
| 15 MHz | 75 | 3, 4, 7, 8, 11, 12, 15, 16, 19, 20, 23, 24, 27, 28, 31, 32, 35, 36, 39, 40, 43, 44 ,47, 48, 51, 52, 55, 56, 59, 60, 63, 64, 67, 68, 71, 72, 75 |
| 20 MHz | 100 | 4, 8, 12, 16, 20, 24, 28, 32, 36, 40, 44, 48, 52, 56, 60, 64, 68, 72, 76, 80, 84, 88, 92, 96, 100 |
| Note: Maximum bandwidth for EUTRA bands is 3/5/10/15/20 MHz. |  |  |

-------- Skipped unchanged clauses --------

Table 7.1.7.1.9.3.2-2a: Bandwidth Dependent Parameters

| Max Bandwidth | Max | Allowed Values |
| --- | --- | --- |
| 3 MHz | 15 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15 |
| 5 MHz | 25 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25 |
| 10 MHz | 50 | 2, 3, 5, 6, 8, 9, 11, 12, 14, 15, 17, 18, 20, 21, 23, 24, 26, 27, 29, 30, 32, 33, 35, 36, 38, 39, 41, 42, 44, 45, 47, 48, 50 |
| 15 MHz | 75 | 3, 4, 7, 8, 11, 12, 15, 16, 19, 20, 23, 24, 27, 28, 31, 32, 35, 36, 39, 40, 43, 44 ,47, 48, 51, 52, 55, 56, 59, 60, 63, 64, 67, 68, 71, 72, 75 |
| 20 MHz | 100 | 4, 8, 12, 16, 20, 24, 28, 32, 36, 40, 44, 48, 52, 56, 60, 64, 68, 72, 76, 80, 84, 88, 92, 96, 100 |
| Note : Maximum bandwidth for EUTRA bands is 3/5/10/15/20 MHz. |  |  |

-------- Skipped unchanged clauses --------

Table 7.1.7.1.10.3.2-2a: Bandwidth Dependent Parameters

| Max Bandwidth | Max | Allowed Values |
| --- | --- | --- |
| 3 MHz | 15 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15 |
| 5 MHz | 25 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25 |
| 10 MHz | 50 | 2, 3, 5, 6, 8, 9, 11, 12, 14, 15, 17, 18, 20, 21, 23, 24, 26, 27, 29, 30, 32, 33, 35, 36, 38, 39, 41, 42, 44, 45, 47, 48, 50 |
| 15 MHz | 75 | 3, 4, 7, 8, 11, 12, 15, 16, 19, 20, 23, 24, 27, 28, 31, 32, 35, 36, 39, 40, 43, 44 ,47, 48, 51, 52, 55, 56, 59, 60, 63, 64, 67, 68, 71, 72, 75 |
| 20 MHz | 100 | 4, 8, 12, 16, 20, 24, 28, 32, 36, 40, 44, 48, 52, 56, 60, 64, 68, 72, 76, 80, 84, 88, 92, 96, 100 |
| Note: Maximum bandwidth for EUTRA bands is 3/5/10/15/20 MHz. |  |  |

-------- Skipped unchanged clauses --------

Table 7.1.7.1.11.3.2-2a: Bandwidth Dependent Parameters

| Max Bandwidth | Max | Allowed Values |
| --- | --- | --- |
| 3 MHz | 15 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15 |
| 5 MHz | 25 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25 |
| 10 MHz | 50 | 2, 3, 5, 6, 8, 9, 11, 12, 14, 15, 17, 18, 20, 21, 23, 24, 26, 27, 29, 30, 32, 33, 35, 36, 38, 39, 41, 42, 44, 45, 47, 48, 50 |
| 15 MHz | 75 | 3, 4, 7, 8, 11, 12, 15, 16, 19, 20, 23, 24, 27, 28, 31, 32, 35, 36, 39, 40, 43, 44 ,47, 48, 51, 52, 55, 56, 59, 60, 63, 64, 67, 68, 71, 72, 75 |
| 20 MHz | 100 | 4, 8, 12, 16, 20, 24, 28, 32, 36, 40, 44, 48, 52, 56, 60, 64, 68, 72, 76, 80, 84, 88, 92, 96, 100 |
| Note : Maximum bandwidth for EUTRA bands is 3/5/10/15/20 MHz. |  |  |

-------- Skipped unchanged clauses --------

Table 7.1.7.1.12.3.2-2a: Bandwidth Dependent Parameters

| Max Bandwidth | Max | Allowed Values |
| --- | --- | --- |
| 3 MHz | 15 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15 |
| 5 MHz | 25 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25 |
| 10 MHz | 50 | 2, 3, 5, 6, 8, 9, 11, 12, 14, 15, 17, 18, 20, 21, 23, 24, 26, 27, 29, 30, 32, 33, 35, 36, 38, 39, 41, 42, 44, 45, 47, 48, 50 |
| 15 MHz | 75 | 3, 4, 7, 8, 11, 12, 15, 16, 19, 20, 23, 24, 27, 28, 31, 32, 35, 36, 39, 40, 43, 44 ,47, 48, 51, 52, 55, 56, 59, 60, 63, 64, 67, 68, 71, 72, 75 |
| 20 MHz | 100 | 4, 8, 12, 16, 20, 24, 28, 32, 36, 40, 44, 48, 52, 56, 60, 64, 68, 72, 76, 80, 84, 88, 92, 96, 100 |
| Note : Maximum bandwidth for EUTRA bands is 3/5/10/15/20 MHz. |  |  |

-------- Skipped unchanged clauses --------

Table 7.1.7.1.12a.3.2-2a: Bandwidth Dependent Parameters (RA type 0)

| Max Bandwidth | Max | Allowed Values |
| --- | --- | --- |
| 3 MHz | 15 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15 |
| 5 MHz | 25 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25 |
| 10 MHz | 50 | 2, 3, 5, 6, 8, 9, 11, 12, 14, 15, 17, 18, 20, 21, 23, 24, 26, 27, 29, 30, 32, 33, 35, 36, 38, 39, 41, 42, 44, 45, 47, 48, 50 |
| 15 MHz | 75 | 3, 4, 7, 8, 11, 12, 15, 16, 19, 20, 23, 24, 27, 28, 31, 32, 35, 36, 39, 40, 43, 44 ,47, 48, 51, 52, 55, 56, 59, 60, 63, 64, 67, 68, 71, 72, 75 |
| 20 MHz | 100 | 4, 8, 12, 16, 20, 24, 28, 32, 36, 40, 44, 48, 52, 56, 60, 64, 68, 72, 76, 80, 84, 88, 92, 96, 100 |
| Note: Maximum bandwidth for EUTRA bands is 3/5/10/15/20 MHz. |  |  |

Table 7.1.7.1.12a.3.2-2b: Ambiguous Sizes of Information Bits

{12, 14, 16 ,20, 24, 26, 32, 40, 44, 56}

Table 7.1.7.1.12a.3.2-2c: Bandwidth Dependent Parameters (RA type 1)

| Max Bandwidth | Max |  |
| --- | --- | --- |
| 3 MHz | 15 | 6 |
| 5 MHz | 25 | 11 |
| 10 MHz | 50 | 14 |
| 15 MHz | 75 | 16 |
| 20 MHz | 100 | 22 |
| Note : Maximum bandwidth for EUTRA bands is 3/5/10/15/20 MHz. |  |  |

-------- Skipped unchanged clauses --------

Table 7.1.7.2.1.3.2-2a: Bandwidth Dependent Parameters

| Max Bandwidth | Max |
| --- | --- |
| 3 MHz | 15 |
| 5 MHz | 25 |
| 10 MHz | 50 |
| 15 MHz | 75 |
| 20 MHz | 100 |
| Note: Maximum bandwidth for EUTRA bands is 3/5/10/15/20 MHz. |  |

-------- Skipped unchanged clauses --------

Table 7.1.7.2.4.3.2-2a: Bandwidth Dependent Parameters

| Max Bandwidth | Max |
| --- | --- |
| 3 MHz | 15 |
| 5 MHz | 25 |
| 10 MHz | 50 |
| 15 MHz | 75 |
| 20 MHz | 100 |
| Note: Maximum bandwidth for EUTRA bands is 3/5/10/15/20 MHz. |  |

-------- Skipped unchanged clauses --------

Table A.8.21-3: Cell specific test parameters for E-UTRAN FDD-FDD inter frequency cell proximity detection test case

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UARFCN |  | Channel 1 |  |  | Channel 2 |  |  |
| CSG indicator |  | False |  |  | True | N/A | True |
| Physical cell global identity |  | 1 | 1 | 1 | 2 | N/A | 2 |
| CSG identity |  | Not sent |  |  | Sent | N/A | Sent |
| BWchannel | MHz | 10 or 34 |  |  | 10 or 34 |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD | N/A | OP.2 FDD | OP.2 FDD | N/A | OP.2 FDD |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
|  | dB | 0 | -inf | 4 | 7 | -inf | 7 |
| Note 2 | dBm/15 kHz | -98 |  |  | -98 |  |  |
|  | dB | 0 | -inf | 4 | 7 | -inf | 7 |
| RSRP Note 3 | dBm/15 KHz | -98 | -inf | -94 | -91 | -inf | -91 |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: 3MHz channel bandwidth is used for the bands not supporting 5MHz or larger channel bandwidths. |  |  |  |  |  |  |  |

-------- End of changes --------