3GPP TSG-RAN5 Meeting #103	R5-242408

# Fukuoka, Japan, 20th May 2024 – 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3128 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add IE DL-PRS-MeasurementWithRxFH-RRC-Connected |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test |  |  |  |  |  | Date: |  |  | 2024-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add and align the IE DL-PRS-MeasurementWithRxFH-RRC-Connected with core specification updates. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The IE IE DL-PRS-MeasurementWithRxFH-RRC-Connected has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will not be fully applicable and aligned with the core specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of first modified section>

#### –	CodebookParameters

Table 4.6.4-6A: CodebookParameters

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CodebookParameters ::= SEQUENCE { |  |  |  |
| type1 SEQUENCE { |  |  |  |
| singlePanel SEQUENCE { |  |  |  |
| supportedCSI-RS-ResourceList SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource { | 1 entry |  |  |
| SupportedCSI-RS-Resource[1] SEQUENCE { |  | entry 1 |  |
| maxNumberTxPortsPerResource | Not checked |  |  |
| maxNumberResourcesPerBand | Not checked |  |  |
| totalNumberTxPortsPerBand | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| modes | Not checked |  |  |
| maxNumberCSI-RS-PerResourceSet | Not checked |  |  |
| } |  |  |  |
| multiPanel SEQUENCE { |  |  |  |
| supportedCSI-RS-ResourceList SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource { | 1 entry |  |  |
| SupportedCSI-RS-Resource[1] SEQUENCE { |  | entry 1 |  |
| maxNumberTxPortsPerResource | Not checked |  |  |
| maxNumberResourcesPerBand | Not checked |  |  |
| totalNumberTxPortsPerBand | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| modes | Not checked |  |  |
| nrofPanels | Not checked |  |  |
| maxNumberCSI-RS-PerResourceSet | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| type2 SEQUENCE { |  |  |  |
| supportedCSI-RS-ResourceList SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource { | 1 entry |  |  |
| SupportedCSI-RS-Resource[1] SEQUENCE { |  | entry 1 |  |
| maxNumberTxPortsPerResource | Not checked |  |  |
| maxNumberResourcesPerBand | Not checked |  |  |
| totalNumberTxPortsPerBand | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| parameterLx | Not checked |  |  |
| amplitudeScalingType | Not checked |  |  |
| amplitudeSubsetRestriction | Not checked |  |  |
| } |  |  |  |
| type2-PortSelection SEQUENCE { |  |  |  |
| supportedCSI-RS-ResourceList SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource { | 1 entry |  |  |
| SupportedCSI-RS-Resource[1] SEQUENCE { |  | entry 1 |  |
| maxNumberTxPortsPerResource | Not checked |  |  |
| maxNumberResourcesPerBand | Not checked |  |  |
| totalNumberTxPortsPerBand | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| parameterLx | Not checked |  |  |
| amplitudeScalingType | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	DL-PRS-MeasurementWithRxFH-RRC-Connected

Table 4.6.4-6B: DL-PRS-MeasurementWithRxFH-RRC-Connected

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DL-PRS-MeasurementWithRxFH-RRC-Connected-r18 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

<End of modified section>