3GPP TSG-RAN5 Meeting #103	R5-243808

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5516 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of narrow band blocking for CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI10_Test, LTE_CA_Rel12_3DL-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The PCC DL NRB for test ID2 in the default test setting for CA_XA-YA-ZA in Table 7.6.3A.5.4.1-1 is incorrect. Instead of Lowest NRB it shall be Highest NRB, i.e., the same for all three test IDs. Based on draft spec V16.3.0, CR R5-198105 introduced the change from Highest NRB to Lowest NRB. However, CR R5-198105 does not contain this change and updated only default test settings for CA_XB-YA and CA_XC-YA Configuration. Thus, due to an incorrect implementation of CR R5-198105, default test setting for CA_XA-YA-ZA has been changed. |  |  |  |  |  |  |  |  |
| . |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | The PCC NRB for CA_XA-YA-ZA has been corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UEs will be tested with the wrong configuration resulting in loss of test coverage. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.3A.5.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the final outcome of one revision of R5-243112. |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

#### 7.6.3A.5	Narrow band blocking for 3DL CA

##### 7.6.3A.5.1	Test Purpose

Same test purpose as 7.6.3A.1.

##### 7.6.3A.5.2	Test Applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support 3DL with Intra-band contiguous CA or 3DL with Inter-band CA or 3DL with Intra-band contiguous and  Inter-band CA.

This test case also applies to all types of E-UTRA UE release 11 and forward that support 3DL with Intra-band non-contiguous and  Inter-band CA or 3DL with Intra-band non-contiguous and Intra-band contiguous CA.

This test case also applies to all types of E-UTRA UE release 12 and forward that support 3DL with FDD-TDD CA.

##### 7.6.3A.5.3	Minimum Conformance Requirements

Same as in clause 7.6.3A.1.

##### 7.6.3A.5.4	Test Description

###### 7.6.3A.5.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1 through table 5.4.2A.1-2a. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 7.6.3A.5.4.1-1 through table 7.6.3A.5.4.1-2 as appropriate. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.6.3A.5.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  | Refer to “PCC NRB” and “SCCs NRB” columns |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  | NS_01 by default |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC |  |  |  | SCC1 |  |  |  | SCC2 |  |  |  |
|  | Band | Range | NRB |  | Band | Range | NRB |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | ULalloc (Note 2,3) | DLalloc | UL MOD | DL MOD | ULalloc | DLalloc | UL MOD | DL MOD | ULalloc | DLalloc |
| Default Test Settings for a CA_XD Configuration (Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid/CC1 | REFSENS | All RBs | X | Mid/CC2 | N/A | All RBs | X | Mid/CC3 | N/A | All RBs |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg |
| 2 | X | Mid/CC1 | REFSENS | All RBs | X | Mid/CC2 | N/A | All RBs | X | Mid/CC3 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| Default Test Settings for a CA_XA-YA-ZA Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid | REFSENS | All RBs | Y | Mid | N/A | All RBs | Z | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB |
| 2 | Y | Mid | REFSENS | All RBs | X | Mid | N/A | All RBs | Z | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB |
| 3 | Z | Mid | REFSENS | All RBs | X | Mid | N/A | All RBs | Y | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB |
| Test Settings for CA_1A-18A-28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Low | 50@0 | All RBs | 18 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 50 | N/A | QPSK |  | 50 |
| 2 | 1 | Low | 75@0 | All RBs | 18 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 50 | N/A | QPSK |  | 50 |
| 3 | 1 | Low | 100@0 | All RBs | 18 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 50 | N/A | QPSK |  | 50 |
| 4 | 1 | Low | 100@0 | All RBs | 18 | Mid | N/A | All RBs | 28 | High | N/A | All RBs |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 75 | N/A | QPSK |  | 50 |
| 5 | 18 | Low | 25@25 | All RBs | 1 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 50 | N/A | QPSK |  | 50 |
| 6 | 18 | Low | 25@25 | All RBs | 1 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 75 | N/A | QPSK |  | 50 |
| 7 | 18 | Low | 25@25 | All RBs | 1 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  | 50 |
| 8 | 18 | Mid | 25@50 | All RBs | 1 | Low | N/A | All RBs | 28 | High | N/A | All RBs |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 100 | N/A | QPSK |  | 25 |
| 9 | 18 | Mid | 25@50 | All RBs | 1 | Low | N/A | All RBs | 28 | High | N/A | All RBs |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 100 | N/A | QPSK |  | 50 |
| 10 | 28 | High | 25@0 | All RBs | 1 | Low | N/A | All RBs | 18 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 25 | N/A | QPSK |  | 100 | N/A | QPSK |  | 75 |
| 11 | 28 | Mid | 25@25 | All RBs | 1 | Low | N/A | All RBs | 18 | Low | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  | 50 |
| 12 | 28 | High | 25@25 | All RBs | 1 | High | N/A | All RBs | 18 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  | 75 |
| Default Test Settings for CA_XB-YA and CA_XC-YA Configuration (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid/CC1 | REFSENS | All RBs | X | Mid/CC2 | N/A | All RBs | Y | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB |
| 2 | X | Mid/CC1 | REFSENS | All RBs | X | Mid/CC2 | N/A | All RBs | Y | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Highest NRB |
| 3 | Y | Mid | REFSENS | All RBs | X | Mid/CC1 | N/A | All RBs | X | Mid/CC2 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 4 | Y | Mid | REFSENS | All RBs | X | Mid/CC1 | N/A | All RBs | X | Mid/CC2 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg |
| Default Test Settings for CA_XA-YB and CA_XA-YC Configuration (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Y | Mid/CC1 | REFSENS | All RBs | Y | Mid/CC2 | N/A | All RBs | X | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB |
| 2 | Y | Mid/CC1 | REFSENS | All RBs | Y | Mid/CC2 | N/A | All RBs | X | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Highest NRB |
| 3 | X | Mid | REFSENS | All RBs | Y | Mid/CC1 | N/A | All RBs | Y | Mid/CC2 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 4 | X | Mid | REFSENS | All RBs | Y | Mid/CC1 | N/A | All RBs | Y | Mid/CC2 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Use CA Configuration – specific test points if present in the table, otherwise use Default Test Settings test points. Note 3: Inter-band & Intra-band contiguous + Inter-band: If, according to the UE declared capability, UE does not support UL in an individual band within the CA Configuration, test points with that individual band as PCC are not applicable. Note 4: Intra-band contiguous: X corresponds to the band in the CA Configuration. E.g. for CA_41D, X=41 Note 5: Inter-band: X,Y,Z correspond to the 1st, 2nd and 3rd band in the CA Configuration. E.g. for CA_1A-3A-19A, X=1, Y=3, Z=19 Note 6: Intra-band contiguous + Inter-band: X,Y correspond to the different bands in the CA Configuration. E.g. for CA_1C-3A, X=1 and Y=3 Note 7: REFSENS refers to the PCC band’s and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 8: Intra-band contiguous: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose for testing the Combination with maximum NRB_PCC and then select maximum NRB_SCC1 for the chosen NRB_PCC. Note 9: Intra-band contiguous + Inter-band: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose the Combination with NRB_PCC = NRB_SCC1 for testing. If no such combination is supported, choose Combination with maximum NRB_PCC for testing. Note 10: The orders and numbering of SCCs in this table does not imply any order in test implementation of SCCs. Note 11: ID/test points that result in no refsens requirement shall be skipped. |  |  |  |  |  |  |  |  |  |  |  |  |

< End of changes >