3GPP TSG-RAN5 Meeting #103	R5-242337

# Fukuoka, Japan, 20th – 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0344 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of initial conditions and RRC/NAS signalling in clause 5.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Test configurations are introduced in clause 5.2 and it shall be clarified for which configuration a particular procedure is applicable. - Information about the RRC state has been missing in the initial conditions of the test procedures. - There is a new procedure 5.4.14 for MCX communication release. - Editorial issues. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Applicability for test configuraions added to initial conditions of the test prcedures. - RRC state added to the initial conditions of the test procedures. - Use of new procedure 5.4.14 for MCX communication release. - Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.579-1 CRs 0356, 0357 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 5.3	Generic test procedures for UE MCS operation

### 5.3.1	General

The purpose of the procedures specified in the following clauses is to facilitate test description by providing procedure sequences which can be referred from the relevant test cases specified e.g. in 3GPP TS 36.579-2 [2], 3GPP TS 36.579-3 [3], 3GPP TS 36.579-6 [84], 3GPP TS 36.579-7 [85].

The procedures specified are required to ensure that any MC service can take place or specific MC relevant pre-conditions are met before a test case can be executed.

Within the context of the procedures, MCX refers to MCPTT, MCVideo or MCData.

### 5.3.2	Initial MCX Authentication, Registration, Configuration and Subscription

##### 5.3.2.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

In addition:

-	The MCX Client has been provisioned either with the address information of the server from which the client can retrieve the MCX UE initial configuration document (steps 1a1-1a2 of procedure ‘MCX Initial Configuration and User Authentication', Table 5.3.2.3.1-1) or directly with the Initial UE Configuration Data as specified in Table 5.5.8.1-1.

-	According to TS 33.180 [94] all HTTP connections are secured by TLS.
The HTTP-1 interface authentication between the HTTP client in the MC UE and the HTTP server endpoint (HTTP proxy, IdM server or KMS) shall be performed by one-way authentication of the HTTP server endpoint based on server certificate as described in TS 33.180 [94] clause 6.1.1.

-	The UE User is provided with username/password for user authentication (px_MCX_User_A_username, px_MCX_User_A_password as provided in TS 36.579-5 [5], Table 9.2-1: MCX Client Common PIXIT).

-	The UE is provisioned with the names and values of the Transport Key (TrK) and the Integrity Key (InK), since the KMS shall encrypt the key material sent to the client with the TrK and sign the response with the TrK or the InK according to TS 33.180 [94].

##### 5.3.2.2	Main Procedure

5.3.2.2.1	Procedure

Table 5.3.2.2.1-1: Initial MCX Authentication, Registration, Configuration and Subscription

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: The procedures of steps 1 and 2 happen in parallel | - | - |
| 1 | The UE (MCX client) performs procedure ‘MCX Initial Configuration and User Authentication’ as described in Table 5.3.2.3.1-1 | - | - |
| 2 | The UE (MCX client) performs procedure ‘SIP registration’ as described in Table 5.3.2.4.1-1 (NOTE 1) | - | - |
| - | EXCEPTION: The procedures of steps 3, 4 and 5 happen in parallel | - | - |
| 3 | The UE (MCX client) performs procedure ‘Publication of MCX service settings‘ as described in Table 5.3.2.5.1-1 (NOTE 1) | - | - |
| 4 | The UE (MCX client) performs procedure ‘Configuration management subscription‘ as described in Table 5.3.2.6.1-1 | - | - |
| 5 | The UE (MCX client) performs procedure ‘Group management subscription with optional GMK retrieval‘ as described in Table 5.3.2.7.1-1 | - | - |
| 6 | The SS (MCX server) sends a SIP MESSAGE for configuration of Location Info reporting. | <-- | SIP MESSAGE |
| 7 | The UE (MCX client) responds with SIP 200 (OK) | --> | SIP 200 (OK) |
| NOTE 1: Based on UE implementation, the access token may be provided using either a SIP REGISTER at initial SIP registration (Table 5.3.2.4.1-1 step 3a1) or in the SIP PUBLISH for MCPTT server settings (Table 5.3.2.5.1-1 step 1b1); the SIP REGISTER can only be used when the access token is already available. |  |  |  |

5.3.2.2.2	Specific message contents

All message contents are as specified in clause 5.5 with the following clarifications:

Table 5.3.2.2.2-1: SIP MESSAGE (step 6, Table 5.3.2.2.1-1)

| Derivation Path: Table 5.5.2.7.2-1, condition LOCATION_CONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT/MCVideo/MCData Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 5.3.2.2.2-2 |  |  | MCPTT |
|  | MCVideo-Info as described in Table 5.3.2.2.2-3 |  |  | MCVIDEO |
|  | MCData-Info as described in Table 5.3.2.2.2-4 |  |  | MCDATA |

Table 5.3.2.2.2-2: MCPTT Info in SIP MESSAGE (Table 5.3.2.2.2-1)

| Derivation Path: Table 5.5.3.2.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-calling-user-id | not present |  |  |  |

Table 5.3.2.2.2-3: MCVideo Info in SIP MESSAGE (Table 5.3.2.2.2-1)

| Derivation Path: Table 5.5.3.2.2-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcvideoinfo |  |  |  |  |
| mcvideo-Params |  |  |  |  |
| mcvideo-calling-user-id | not present |  |  |  |

Table 5.3.2.2.2-4: MCData Info in SIP MESSAGE (Table 5.3.2.2.2-1)

| Derivation Path: Table 5.5.3.2.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdatainfo |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-calling-user-id | not present |  |  |  |

##### 5.3.2.3	MCX Initial Configuration and User Authentication

5.3.2.3.1	Procedure

Table 5.3.2.3.1-1: MCX Initial Configuration and User Authentication

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1a1-1a2 describe behaviour that depends on UE implementation. | - | - | - | - |
| 1a1 | IF the UE (MCX client) is capable of downloading the MCX UE initial configuration document THEN the UE (MCX client) sends an HTTP GET Request to retrieve the initial UE configuration from the server. NOTE: Otherwise the UE needs to be preconfigured with the Initial UE Configuration Data as initial condition. | --> | HTTP GET (initial UE configuration) | - | P |
| 1a2 | The SS sends an HTTP 200 (OK) including the initial UE configuration document | <-- | HTTP 200 (OK) | - | - |
| 2 | Void | - | - | - | - |
| - | EXCEPTION: The messages in steps 3a1-7 are transmitted over a secure TLS tunnel that has been established by the UE (MCX client) as specified by 3GPP TS 33.310 [70], to the authorisation endpoint of the IdM server as specified in 3GPP TS 33.180 [94] using the configured URL of the authorisation endpoint of the IdM server as specified in the "<x>/OnNetwork/AppServerInfo/IDMSAuthEndpoint" leaf node, Table 5.5.8.1-1. | - | - | - | - |
| - | EXCEPTION: Steps 3a1-3b1 describe behaviour that depends on UE implementation of the OpenID Connect protocol; the UE may either use an HTTP GET or an HTTP POST to send the OpenID Connect Authentication Request. | - | - | - | - |
| 3a1 | The UE (MCX client) sends an OpenID Connect Authentication Request using HTTP GET. | --> | HTTP GET (Authorization) | - | P |
| 3b1 | The UE (MCX client) sends an OpenID Connect Authentication Request using HTTP POST. | --> | HTTP POST (Authorization) | - | P |
| 4 | The SS sends an HTTP 200 (OK) including the HTML form requesting username and password. | <-- | HTTP 200 (OK) | - | - |
| 5 | Provide the UE (MCX client) with user credentials: username and password (px_MCX_User_A_username, px_MCX_User_A_password). (NOTE 1) | - | - | - | - |
| 6 | The UE (MCX client) sends an HTTP POST Request containing user name and password. | --> | HTTP POST | - | P |
| 7 | The SS sends a HTTP 302 (Found) as the OpenID Connect Authentication Response containing an authorization code. | <-- | HTTP 302 (Found) | - | - |
| 8 | Void | - | - | - | - |
| - | EXCEPTION: The messages in steps 9-10 are transmitted over a secure TLS tunnel that has been established by the UE (MCX client) as specified by 3GPP TS 33.310 [70] to the token endpoint of the IdM server as specified in 3GPP TS 33.180 [94] using the configured URL of the token endpoint of the IdM server as specified in the "/<x>/OnNetwork/AppServerInfo/IDMSTokenEndpoint" leaf node, Table 5.5.8.1-1. | - | - | - | - |
| 9 | The UE (MCX client) sends an HTTP POST Request (OIDC Token Request), passing the authorization code obtained in step 7. | --> | HTTP POST | - | P |
| 10 | The SS sends an HTTP 200 (OK) providing id_token, access_token and refresh token. | <-- | HTTP 200 (OK) | - | - |
| - | EXCEPTION: The messages in steps 11-14 are transmitted over a secure TLS tunnel that has been established by the UE (MCX client) as specified by 3GPP TS 33.310 [70] to the HTTP Proxy as specified in 3GPP TS 33.180 [94] using the configured URL of the HTTP Proxy as specified in the "/<x>/OnNetwork/AppServerInfo/HTTPproxy" leaf node, Table 5.5.8.1-1. | - | - | - | - |
| 11 | The UE (MCX client) sends an HTTP POST presenting the access token obtained in step 10. | --> | HTTP POST | - | P |
| 12 | The SS replies with identity specific key information. | <-- | HTTP 200 (OK) | - | - |
| 13 | The UE (MCX client) sends an HTTP POST presenting an access token for Key Material Request. | --> | HTTP POST | - | P |
| 14 | The SS replies to the UE with identity specific key information. | <-- | HTTP 200 (OK) | - | - |
| NOTE 1: The UE is expected to prompt the MCX user for username and password, or it may be stored on the UE. The provision of the username/password is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

5.3.2.3.2	Specific message contents

All message contents are as specified in clause 5.5 with the following clarifications:

Table 5.3.2.3.2-1: HTTP GET (Step 1, Table 5.3.2.3.1-1)

Derivation Path: Table 5.5.4.2-1, condition UEINITIALCONFIG

Table 5.3.2.3.2-2: HTTP 200 (OK) (Step 2, Table 5.3.2.3.1-1)

Derivation Path: Table 5.5.4.6-1, condition UEINITIALCONFIG

Table 5.3.2.3.2-3: HTTP GET (Step 3a1, Table 5.3.2.3.1-1)

Derivation Path: Table 5.5.4.2-1, condition AUTH

Table 5.3.2.3.2-4: HTTP POST (Step 3b1, Table 5.3.2.3.1-1)

Derivation Path: Table 5.5.4.3-1, condition AUTH

Table 5.3.2.3.2-5: HTTP 200 (OK) (Step 4, Table 5.3.2.3.1-1)

| Derivation Path: Table 5.5.4.6-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "text/html" |  | RFC 2854 [111] |  |
| Message-body |  |  |  |  |
| HTML form | <!DOCTYPE html> <html> <body> <form action="/idms/userauth" method="post"> Username: <input type="text" name="user"><br> Password: <input type="password" name="password"><button type="submit">Login</button> </form> </body> </html> | "/idms/userauth" given by tsc_MCX_IdMS_userauth_UriPath is the URI to be used by the UE as request URI in the HTTP POST request for user authentication | HTML 4.01 Specification [105] |  |

Table 5.3.2.3.2-6: HTTP POST (Step 6, Table 5.3.2.3.1-1)

Derivation Path: Table 5.5.4.3-1, condition USERAUTH

Table 5.3.2.3.2-7: HTTP 302 (Found) (Step 7, Table 5.3.2.3.1-1)

Derivation Path: Table 5.5.4.8-1, condition AUTH.

Table 5.3.2.3.2-8: HTTP POST (Step 9, Table 5.3.2.3.1-1)

Derivation Path: Table 5.5.4.3-1, condition TOKEN

Table 5.3.2.3.2-9: HTTP 200 (OK) (Step 10, Table 5.3.2.3.1-1)

Derivation Path: Table 5.5.4.6-1, condition TOKEN

Table 5.3.2.3.2-10: HTTP POST (Step 11, Table 5.3.2.3.1-1)

Derivation Path: Table 5.5.4.33-1, condition KMSINIT.

Table 5.3.2.3.2-11: HTTP 200 (OK) (Step 12, Table 5.3.2.3.1-1)

Derivation Path: Table 5.5.4.6-1, condition KMSINIT.

Table 5.3.2.3.2-12: HTTP POST (Step 13, Table 5.3.2.3.1-1)

Derivation Path: Table 5.5.4.3-1, condition KMSKEY.

Table 5.3.2.3.2-13: HTTP 200 (OK) (Step 14, Table 5.3.2.3.1-1)

Derivation Path: Table 5.5.4.6-1, condition KMSKEY.

##### 5.3.2.4	SIP Registration

5.3.2.4.1	Procedure

Table 5.3.2.4.1-1: SIP Registration

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The UE sends an initial registration for IMS services. | --> | SIP REGISTER |
| 2 | The SS responds with a valid AKAv1-MD5 authentication challenge and security mechanisms supported by the network. | <-- | SIP 401 Unauthorized |
| - | EXCEPTION: The UE completes the security negotiation procedures, sets up a temporary set of SAs and uses those for sending another SIP REGISTER with AKAv1-MD5 credentials at step 3a1 or 3a2 | - | - |
| - | EXCEPTION: Steps 3a1-3b1 describe behaviour that depends on UE implementation and on availability of an access-token (NOTE 1) | - | - |
| 3a1 | IF the client has retrieved the access token already at MCX User Authentication (Table 5.3.2.3.1-1 steps 9-10) THEN the UE may use the SIP REGISTER to provide access token and CSK for service authorisation (NOTE 2) | --> | SIP REGISTER (access token, CSK) |
| 3b1 | ELSE the UE sends SIP REGISTER without access token and CSK | --> | SIP REGISTER |
| 4 | The SS responds with 200 OK. | <-- | SIP 200 OK |
| NOTE 1: According to TS 33.180 [94], clause 5.1.3.2.1 sending of the SIP REGISTER shall not be delayed for lack of an access token If the client does not have the access token yet, the client shall sent the SIP REGISTER without service authorisation and shall provide the access token in the SIP PUBLISH (Table 5.3.2.5.1-1, step 1b1) NOTE 2: As the MCPTT/MCVideo/MCData Info containing the access token is security protected the client also needs to provide the CSK used for cyphering and integrity protection. |  |  |  |

5.3.2.4.2	Specific message contents

All message contents are as specified in clause 5.5 with the following clarifications:

Table 5.3.2.4.2-1: SIP REGISTER (Step 1, Table 5.3.2.4.1-1)

Derivation Path: Table 5.5.2.13-1, condition SIP_REGISTER_INITIAL

Table 5.3.2.4.2-2: SIP REGISTER (Step 3a1, Table 5.3.2.4.1-1)

| Derivation Path: Table 5.5.2.13-1, condition SERVICE_AUTH |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  | RFC 3261 [22] |  |
| MIME body part |  | MCPTT/MCVideo/MCData Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 5.3.2.4.2-3 |  | TS 24.379 [9] clause F.1 | MCPTT |
|  | MCVideo-Info as described in Table 5.3.2.4.2-4 |  | TS 24.281 [86] clause F.1 | MCVIDEO |
|  | MCData-Info as described in Table 5.3.2.4.2-5 |  | TS 24.282 [87] clause D.1 | MCDATA |

Table 5.3.2.4.2-3: MCPTT-Info in SIP REGISTER (Table 5.3.2.4.2-2)

Derivation Path: Table 5.5.3.2.1-1, condition CONFIG, REGISTER_PUBLISH

Table 5.3.2.4.2-4: MCVideo-Info in SIP REGISTER (Table 5.3.2.4.2-2)

Derivation Path: Table 5.5.3.2.1-2, condition CONFIG, REGISTER_PUBLISH

Table 5.3.2.4.2-5: MCData-Info in SIP REGISTER (Table 5.3.2.4.2-2)

Derivation Path: Table 5.5.3.2.1-3, condition CONFIG, REGISTER

##### 5.3.2.5	Publication of MCX service settings

5.3.2.5.1	Procedure

Table 5.3.2.5.1-1: Publication of MCX service settings

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
|  | EXCEPTION: Steps 1a1-1b1 describe behaviour that depends on whether or not the client has provided an access token for service authorisation already at SIP registration (Table 5.3.2.4.1-1) |  |  |  |  |
| 1a1 | IF the UE (MCX client) has provided the access token at SIP registration THEN the UE (MCX client) sends a SIP PUBLISH request for update of PoC-settings only. (NOTE 1). | --> | SIP PUBLISH | - | P |
| 1b1 | ELSE the UE (MCX client) sends a SIP PUBLISH request for service authorisation and update of PoC-settings. (NOTE 1). | --> | SIP PUBLISH | - | P |
| 2 | The SS (MCX server) sends SIP 200 (OK). | <-- | SIP 200 (OK) | - | - |
| NOTE 1: The PoC-settings document contains the user profile index of the selected user profile. In general the UE sends the SIP PUBLISH request not before it has retrieved the user profile at step 8 in Table 5.3.2.6.1-1. |  |  |  |  |  |

5.3.2.5.2	Specific message contents

All message contents are as specified in clause 5.5 with the following clarifications:

Table 5.3.2.5.2-1: SIP PUBLISH (Step 1a1, Table 5.3.2.5.1-1)

| Derivation Path: Table 5.5.2.11-1, condition POC-SETTINGS-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT/MCVideo/MCData Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 5.3.2.5.2-3 |  |  | MCPTT |
|  | MCVideo-Info as described in Table 5.3.2.5.2-4 |  |  | MCVIDEO |
|  | MCData-Info as described in Table 5.3.2.5.2-5 |  |  | MCDATA |

Table 5.3.2.5.2-2: SIP PUBLISH (Step 1a2, Table 5.3.2.5.1-1)

| Derivation Path: Table 5.5.2.11-1, condition POC-SETTINGS-EVENT, SERVICE_AUTH |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT/MCVideo/MCData Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 5.3.2.5.2-3 |  |  | MCPTT |
|  | MCVideo-Info as described in Table 5.3.2.5.2-4 |  |  | MCVIDEO |
|  | MCData-Info as described in Table 5.3.2.5.2-5 |  |  | MCDATA |

Table 5.3.2.5.2-3: MCPTT-Info in SIP PUBLISH (Table 5.3.2.5.2-1/2)

Derivation Path: Table 5.5.3.2.1-1, condition CONFIG, REGISTER_PUBLISH

Table 5.3.2.4.2-4: MCVideo-Info in SIP PUBLISH (Table 5.3.2.5.2-1/2)

Derivation Path: Table 5.5.3.2.1-2, condition CONFIG, REGISTER_PUBLISH

Table 5.3.2.4.2-5: MCData-Info in SIP PUBLISH (Table 5.3.2.5.2-1/2)

Derivation Path: Table 5.5.3.2.1-3, condition CONFIG, REGISTER

##### 5.3.2.6	Configuration management subscription

5.3.2.6.1	Procedure

Table 5.3.2.6.1-1: Configuration management subscription

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE (MCX client) sends a SIP SUBSCRIBE - subscription to multiple documents simultaneously - containing the access token and a resource list body containing a list adressing the following documents: MCX UE Configuration document, MCX User Profile Configuration Document, and the MCX Service configuration document. The base URI of each list entry is set to the CMS XCAP-ROOT-URI. | --> | SIP SUBSCRIBE | - | P |
| 2 | The SS sends a SIP 200 (OK). | <-- | SIP 200 (OK) | - | - |
| 3 | The SS sends a SIP NOTIFY containing the XCAP-URIs of the documents. | <-- | SIP NOTIFY | - | - |
| - | EXCEPTION: The order of steps 4, 5, 7 and 9 depends on UE and SS implementation and is not checked by the implementation | - | - | - | - |
| 4 | The UE (MCX client) sends a SIP 200 (OK). | --> | SIP 200 (OK) | - | P |
| 5 | The UE (MCX client) sends an HTTP GET Request containing the access token and the XCAP-URI of the MCX UE Configuration Document. NOTE: The MCX Client is requesting the MCX UE Configuration Document. | --> | HTTP GET | - | P |
| 6 | The SS sends an HTTP 200 (OK) including the MCX UE Configuration Document. | <-- | HTTP 200 (OK) | - | - |
| 7 | The UE (MCX client) sends an HTTP GET Request containing the access token and the XCAP-URI of the MCX User Profile Configuration Document. NOTE: The MCX Client is requesting the MCX User Profile Configuration Document. | --> | HTTP GET | - | P |
| 8 | The SS sends an HTTP 200 (OK) including the MCX User Profile Configuration Document. NOTE: The MCX User Profile Configuration Document includes information on MCX groups including for which groups the MCX Client is a member. The MCX User Profile Configuration Document includes Group A as a group for which the MCX Client is a member and is implicitly affiliated. Group A is used as the default group for all test cases in TS 36.579-2 and TS 36.579-3. | <-- | HTTP 200 (OK) | - | - |
| 9 | The UE (MCX client) sends an HTTP GET Request containing the access token and the XCAP-URI of the MCX Service Configuration Document. NOTE: The MCX Client is requesting the MCX Service Configuration Document. | --> | HTTP GET | - | P |
| 10 | The SS sends an HTTP 200 (OK) including the MCX Service Configuration Document. | <-- | HTTP 200 (OK) | - | -- |

5.3.2.6.2	Specific message contents

All message contents are as specified in clause 5.5 with the following clarifications:

Table 5.3.2.6.2-1: SIP SUBSCRIBE (Step 1, Table 5.3.2.6.1-1)

Derivation Path: Table 5.5.2.14-1, condition CONFIG

Table 5.3.2.6.2-2: SIP NOTIFY (Step 3, Table 5.3.2.6.1-1)

Derivation Path: Table 5.5.2.8-1, condition CONFIG

Table 5.3.2.6.2-3: HTTP GET (Step 5, Table 5.3.2.6.1-1)

Derivation Path: Table 5.5.4.2-1, condition UECONFIG.

Table 5.3.2.6.2-4: HTTP GET (Step 7, Table 5.3.2.6.1-1)

Derivation Path: Table 5.5.4.2-1, condition UEUSERPROF.

Table 5.3.2.6.2-5: HTTP GET (Step 9, Table 5.3.2.6.1-1)

Derivation Path: Table 5.5.4.2-1, condition UESERVCONFIG.

Table 5.3.2.6.2-6: HTTP 200 (OK) (Step 6, Table 5.3.2.6.1-1)

Derivation Path: Table 5.5.4.6-1, condition UECONFIG.

Table 5.3.2.6.2-7: HTTP 200 (OK) (Step 8, Table 5.3.2.6.1-1)

Derivation Path: Table 5.5.4.6-1, condition UEUSERPROF.

Table 5.3.2.6.2-8: HTTP 200 (OK) (Step 10, Table 5.3.2.6.1-1)

Derivation Path: Table 5.5.4.6-1, condition UESERVCONFIG.

##### 5.3.2.7	Group management subscription with optional GMK retrieval

5.3.2.7.1	Procedure

Table 5.3.2.7.1-1: Group management subscription with optional GMK retrieval

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1a1-1c4 describe behaviour that depends on UE implementation; the UE may either use a single SIP SUBSCRIBE for subscription to group A and the GKTP or it uses separate SIP SUBSCRIBE requests. |  |  |  |  |
| 1a1 | The UE (MCX client) sends a SIP SUBSCRIBE containing a resource-lists body with an entry for subscription to the group configuration document (group A) and an entry for subscription to the MCS GKTP document for Group communication key retrieval (GMK retrieval). | --> | SIP SUBSCRIBE (group A, GKTP) | - | P |
| 1a2 | The SS sends a SIP 200 (OK). | <-- | SIP 200 (OK) | - | - |
| 1b1 | The UE (MCX client) sends a SIP SUBSCRIBE containing a resource-lists body with a single entry for subscription to the group configuration document (group A). | --> | SIP SUBSCRIBE (group A) | - | - |
| 1b2 | The SS sends a SIP 200 (OK). | <-- | SIP 200 (OK) | - | - |
| 1c1 | The UE (MCX client) sends a SIP SUBSCRIBE containing a resource-lists body with a single entry for subscription to the MCS GKTP document for Group communication key retrieval (GMK retrieval). | --> | SIP SUBSCRIBE (GKTP) | - | - |
| 1c2 | The SS sends a SIP 200 (OK). | <-- | SIP 200 (OK) | - | - |
| 1c3 | The UE (MCX client) sends a SIP SUBSCRIBE containing a resource-lists body with a single entry for subscription to the group configuration document (group A). | --> | SIP SUBSCRIBE (group A) | - | - |
| 1c4 | The SS sends a SIP 200 (OK). | <-- | SIP 200 (OK) | - | - |
| 2 | Void | - | - | - | - |
| - | EXCEPTION: IF and only if the UE has performed steps 1b1-1b2 THEN in parallel to the events described in steps 3-6, the behaviour of Table 5.3.2.7.1-2 happens: The UE (MCX client) optionally subscribes to the MCS GKTP document for Group communication key retrieval (GMK retrieval). | - | - | - | - |
| 3 | The SS sends a SIP NOTIFY containing the XCAP-URI of the Group Configuration document for group A. | <-- | SIP NOTIFY (group A) | - | - |
| - | EXCEPTION: The order of steps 4 and 5 depends on UE and SS implementation and is not checked. | - | - | - | - |
| 4 | The UE (MCX client) sends a SIP 200 (OK). | --> | SIP 200 (OK) | - | P |
| 5 | The UE (MCX client) sends an HTTP GET Request containing the access token and the XCAP-URI of the Group Configuration document. | --> | HTTP GET (group A) | - | P |
| 6 | The SS sends an HTTP 200 (OK) containing the Group Configuration Document. | <-- | HTTP 200 (OK) | - | - |
| - | EXCEPTION: Steps 7a1-7a2 describe behaviour that depends on whether the UE has requested a GMK at step 1a1, step 1c1 or at step 2a1 of the parallel behaviour in Table 5.3.2.7.1-2 | - | - | - | - |
| 7a1 | IF the UE has requested a GMK THEN the SS sends a SIP NOTIFY containing the group key transport payloads (GKTP) document with the GMK. | <-- | SIP NOTIFY (GKTP) | - | - |
| 7a2 | The UE (MCX client) sends a SIP 200 (OK). | --> | SIP 200 (OK) | - | P |

Table 5.3.2.7.1-2: Stand-alone group communication key request

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS starts timer Timer_1 = 5 seconds. | - | - | - | - |
| - | EXCEPTION: Steps 2a1-2b1 describe behaviour that depends on UE implementation; in general the group communication key retrieval is optional at initial registration. (NOTE 1) | - | - | - | - |
| 2a1 | The UE (MCX client) sends a SIP SUBSCRIBE creating a new dialog and containing a resource list body containing a single entry for subscription to the MCS GKTP document for Group communication key retrieval (GMK retrieval). | --> | SIP SUBSCRIBE (GKTP) | - | P |
| 2a2 | The SS sends a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 2a3 | The SS stops Timer_1. | - | - | - | - |
| 2b1 | Timer_1 expires | - | - | - | - |
| NOTE 1: The key retrieval from the GMS is necessary for the MCX UE under test to enable ciphering exchanged media in group communications. |  |  |  |  |  |

5.3.2.7.2	Specific message contents

All message contents are as specified in clause 5.5 with the following clarifications:

Table 5.3.2.7.2-1: SIP SUBSCRIBE (Step 1a1, Table 5.3.2.7.1-1)

| Derivation Path: Table 5.5.2.14-1, condition GROUPCONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | Resource-lists |  |  |
| MIME-part-body | Resource-lists as described in Table 5.3.2.7.2-2 |  |  |  |

Table 5.3.2.7.2-2: Resource-Lists in SIP SUBSCRIBE (Table 5.3.2.7.2-1)

Derivation Path: Table 5.5.3.3.1A-1, condition GROUPCONFIG, GROUPKEY

Table 5.3.2.7.2-3: SIP SUBSCRIBE (Step 1b1, Table 5.3.2.7.1-1; step 1c3, Table 5.3.2.7.1-1)

| Derivation Path: Table 5.5.2.14-1, condition GROUPCONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | Resource-lists |  |  |
| MIME-part-body | Resource-lists as described in Table 5.3.2.7.2-4 |  |  |  |

Table 5.3.2.7.2-4: Resource-Lists in SIP SUBSCRIBE (Table 5.3.2.7.2-3)

Derivation Path: Table 5.5.3.3.1A-1, condition GROUPCONFIG

Table 5.3.2.7.2-5: SIP SUBSCRIBE (Step 1c1, Table 5.3.2.7.1-1; step 2a1, Table 5.3.2.7.1-2)

| Derivation Path: Table 5.5.2.14-1, condition GROUPCONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | Resource-lists |  |  |
| MIME-part-body | Resource-lists as described in Table 5.3.2.7.2-6 |  |  |  |

Table 5.3.2.7.2-6: Resource-Lists in SIP SUBSCRIBE (Table 5.3.2.7.2-5)

Derivation Path: Table 5.5.3.3.1A-1, condition GROUPKEY

Table 5.3.2.7.2-7: SIP NOTIFY (Step 3, Table 5.3.2.7.1-1)

Derivation Path: Table 5.5.2.8-1, condition GROUPCONFIG

Table 5.3.2.7.2-8: HTTP GET (Step 5, Table 5.3.2.7.1-1)

Derivation Path: Table 5.5.4.2-1, condition GROUPCONFIG

Table 5.3.2.7.2-9: HTTP 200 (OK) (Step 6, Table 5.3.2.7.1-1)

Derivation Path: Table 5.5.4.6-1, condition GROUPCONFIG.

Table 5.3.2.7.2-10: SIP NOTIFY (Step 7a1, Table 5.3.2.7.1-1)

| Derivation Path: Table 5.5.2.8-1, condition GROUPCONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| xcap-diff document | xcap-diff document as described in Table 5.3.2.7.2-11 |  |  |  |

Table 5.3.2.7.2-11: Xcap-Diff Document (Table 5.3.2.7.2-10)

Derivation Path: Table 5.5.3.12-2, condition GROUPKEY

### 5.3.3	MCX pre-established session establishment

5.3.3.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is in RRC_IDLE state.

5.3.3.2	Void

5.3.3.3	Procedure

Table 5.3.3.3-1: MCX pre-established session establishment CO

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The procedure 'MCX CO communication' as described in clause 5.4.3 is started to establish an RRC connection. | - | - | - | - |
| 2-7 | Void | - | - | - | - |
| 8 | Check: Does the UE (MCX Client) send a SIP INVITE message in order to create a pre-established session? | --> | SIP INVITE | - | P |
| - | EXCEPTION: In parallel to the steps below a dedicated bearer gets established as described in clause 5.4.3. | - | - | - | - |
| 8A | The SS sends a SIP 100 Trying | <-- | SIP 100 Trying | - | - |
| 9 | Void | - | - | - | - |
| 10 | The SS (MCX server) responds with a SIP 200 (OK) message. | <-- | SIP 200 (OK) | - | - |
| 10A | Check: Does the UE (MCX Client) respond with a SIP ACK message? | --> | SIP ACK | - | P |
| 11 | Void | - | - | - | - |
| 12 | The procedure 'MCX communication release' as described in clause 5.4.14 is performed to release the RRC connection keeping the dedicated bearer. | - | - | - | - |

5.3.3.4	Specific message contents

Table 5.3.3.4-1: SIP INVITE from the UE (step 8, Table 5.3.3.3-1)

| Derivation Path: Table 5.5.2.5.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Contact |  |  | RFC 3261 [22 RFC 3840 [33] |  |
| feature-param | "+g.3gpp.mcptt" | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Push To Talk (MCPTT) communication. |  | MCPTT |
|  | "+g.3gpp.mcvideo" | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Video (MCVideo) communication. |  | MCVIDEO |
|  | "+g.3gpp.mcdata.sds" | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports mission critical data (MCData) service.communication. |  | MCDATA_SDS |
| feature-param | "audio" | This feature tag indicates that the device supports audio as a streaming media type. |  | MCPTT OR MCVIDEO |
| feature-param | "video" | This feature tag indicates that the device supports video as a streaming media type. |  | MCVIDEO |
| feature-param | "text" | This feature tag indicates that the device supports text as a streaming media type. |  | MCDATA_SDS |
| Accept |  |  | RFC 3261 [22] |  |
| media-range[1] | "application/sdp” |  |  |  |
| Answer-Mode | not present |  |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/sdp" |  |  | MCPTT OR MCVIDEO |
| media-type | "multipart/mixed" |  |  | MCDATA_SDS |
| Message-body |  |  |  | MCPTT OR MCVIDEO |
| SDP Message | SDP message as described in Table 5.5.3.1.1-1 with conditions PRE_ESTABLISHED_SESSION, INITIAL_SDP_OFFER |  |  | MCPTT |
|  | SDP message as described in Table 5.5.3.1.1-2 with condition PRE_ESTABLISHED_SESSION, INITIAL_SDP_OFFER |  |  | MCVIDEO |
| Message-body |  |  |  | MCDATA_SDS |
| MIME body part |  | SDP message |  |  |
| MIME-part-body | SDP message as described in Table 5.5.3.1.1-3 with condition PRE_ESTABLISHED_SESSION, MCDATA_SDS, SDP_OFFER, SDS_SESSION |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info message as described in Table 5.5.3.2.1-3 with condition PRE_ESTABLISHED_SESSION |  |  |  |

Table 5.3.3.4-2: SIP 200 (OK) from the SS (step 10, Table 5.3.3.3-1)

| Derivation Path: Table 5.5.2.17.1.2-1, condition INVITE-RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Contact |  |  |  |  |
| addr-spec |  |  |  |  |
| user-info and host | tsc_MCX_SessionID_B | The URI that identifies the pre-established session |  |  |
| Message-body |  |  |  |  |
| SDP Message | SDP message as described in Table 5.5.3.1.2-1 with condition PRE_ESTABLISHED_SESSION, SDP_ANSWER |  |  | MCPTT |
|  | SDP message as described in Table 5.5.3.1.2-2 with condition PRE_ESTABLISHED_SESSION, SDP_ANSWER |  |  | MCVIDEO |
|  | SDP message as described in Table 5.5.3.1.2-3 with condition PRE_ESTABLISHED_SESSION, MCDATA_SDS, SDP_ANSWER, SDS_SESSION |  |  | MCDATA_SDS |

### 5.3.3A	Void

### 5.3.4	MCX CT session establishment/modification without provisional responses other than 100 Trying

5.3.4.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is either in RRC_IDLE state or in RRC_CONNECTED state.

5.3.4.2	Void

5.3.4.3	Procedure

Table 5.3.4.3-1: MCX CT session establishment/modification without provisional responses other than 100 Trying

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the UE's RRC state. | - | - | - | - |
| 1a1 | IF the UE is in RRC_IDLE state THEN the procedure 'MCX CT communication' as described in clause 5.4.4 is started to establish an RRC connection and a dedicated bearer. | - | - | - | - |
| 2 | The SS (MCX Server) sends a SIP INVITE requesting the establishment/modification of an MCX call. | <-- | SIP INVITE | - | - |
| - | EXCEPTION: Step 3a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE responds to a SIP INVITE with a SIP 100 (Trying). | - | - | - | - |
| 3a1 | The UE (MCX client) sends a SIP 100 (Trying) | --> | SIP 100 (Trying) | - | - |
| 4 | Check: Does the UE (MCX client) respond to the SIP INVITE with SIP 200 (OK)? | --> | SIP 200 (OK) | - | P |
| 5 | The SS (MCX server) sends a SIP ACK to acknowledge the session establishment/modification | <-- | SIP ACK | - | - |

5.3.4.4	Specific message contents

All message contents are as specified in clause 5.5 with the following clarifications:

None

Table 5.3.4.4-1: Void

### 5.3.5	MCX CT group call establishment with manual commencement

5.3.5.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is either in RRC_IDLE state or in RRC_CONNECTED state.

5.3.5.2	Void

5.3.5.3	Procedure

Table 5.3.5.3-1: MCX CT group call establishment with manual commencement

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the UE's RRC state. | - | - | - | - |
| 1a1 | IF the UE is in RRC_IDLE state THEN the procedure 'MCX CT communication' as described in clause 5.4.4 is started to establish an RRC connection and a dedicated bearer. | - | - | - | - |
| 2 | The SS (MCX Server) sends an initial SIP INVITE requesting the establishment of an MCX group call. | <-- | SIP INVITE | - | - |
| - | EXCEPTION: Step 3a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE responds to a SIP INVITE with a SIP 100 (Trying) | - | - | - | - |
| 3a1 | The UE (MCX client) sends SIP 100 (Trying). | --> | SIP 100 (Trying) | - | - |
| 4 | The SS starts timer Timer_1 = 5 seconds. | - | - | - | - |
| - | EXCEPTION: Steps 5a1 to 5c1 describe behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that may take place if the UE responds reliably or unreliably to a SIP INVITE with a SIP 183 (Session Progress) | - | - | - | - |
| 5a1 | Check: Does the UE (MCX client) send a SIP 183 (Session Progress) unreliably? | --> | SIP 183 (Session Progress) | - | P |
| 5a2 | The SS stops Timer_1. | - | - | - | - |
| 5b1 | Check: Does the UE (MCX client) send a SIP 183 (Session Progress) reliably? | --> | SIP 183 (Session Progress) | - | P |
| 5b2 | The SS stops Timer_1. | - | - | - | - |
| 5b3 | The SS (MCX Server) acknowledges the receipt of SIP 183 (Session Progress) | <-- | PRACK | - | - |
| 5b4 | The UE (MCX Client) responds PRACK with SIP 200 (OK) | --> | SIP 200 (OK) | - | - |
| 5c1 | Check: Does Timer_1 expire? | - | - | - | P |
| 5A | Check: Does the UE (MCX client) notify the User of the incoming call request? (NOTE 1) | - | - | - | P |
| 6 | Make UE (MCX User) accept the call. (NOTE 1) | - | - | - | - |
| 7 | Check: Does the UE (MCX client) respond to the SIP INVITE with SIP 200 (OK)? | --> | SIP 200 (OK) | - | P |
| 8 | The SS (MCX server) sends a SIP ACK to acknowledge the session establishment | <-- | SIP ACK | - | - |
| NOTE 1: This expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

5.3.5.4	Specific message contents

All message contents are as specified in clause 5.5 with condition GROUP-CALL where applicable and with the following clarifications:

None

Table 5.3.5.4-1..3: Void

### 5.3.6	MCX CT private call establishment with manual commencement

5.3.6.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is either in RRC_IDLE state or in RRC_CONNECTED state.

5.3.6.2	Void

5.3.6.3	Procedure

Table 5.3.6.3-1: MCX CT private call establishment with manual commencement

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the UE's RRC state. | - | - | - | - |
| 1a1 | IF the UE is in RRC_IDLE state THEN the procedure 'MCX CT communication' as described in clause 5.4.4 is started to establish an RRC connection and a dedicated bearer. | - | - | - | - |
| 2 | The SS (MCX Server) sends an initial SIP INVITE requesting the establishment of an MCX private call. | <-- | SIP INVITE | - | - |
| - | EXCEPTION: Step3a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE responds to a SIP INVITE with a SIP 100 (Trying) | - | - | - | - |
| 3a1 | The UE (MCX client) sends a SIP 100 (Trying). | --> | SIP 100 (Trying) | - | - |
| - | EXCEPTION: Steps 4a1 to 4b3 describe behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that takes place if the UE responds either unreliably or reliably to a SIP INVITE with a SIP 180 (Ringing) | - | - | - | - |
| 4a1 | Check: Does the UE (MCX client) send a SIP 180 (Ringing) unreliably? | --> | SIP 180 (Ringing) | - | P |
| 4b1 | Check: Does the UE (MCX client) send a SIP 180 (Ringing) reliably? | --> | SIP 180 (Ringing) | - | P |
| 4b2 | The SS (MCX Server) acknowledges the receipt of SIP 180 (Ringing) | <-- | PRACK | - | - |
| 4b3 | The UE (MCX Client) responds PRACK with SIP 200 (OK) | --> | SIP 200 (OK) | - | - |
| 4A | Check: Does the UE (MCX client) notify the user of the incoming call? (NOTE 1) | - | - | - | P |
| 5 | Make UE (MCX client) accept the call. (NOTE 1) | - | - | - | - |
| 6 | Check: Does the UE (MCX client) respond to the SIP INVITE with SIP 200 (OK)? | --> | SIP 200 (OK) | - | P |
| 7 | The SS (MCX server) sends a SIP ACK to acknowledge the session establishment | <-- | SIP ACK | - | - |
| NOTE 1: This expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

5.3.6.4	Specific message contents

All message contents are as specified in clause 5.5 with condition PRIVATE-CALL where applicable and in the test case calling the procedure, with the following clarifications:

Table 5.3.6.4-1..1A: Void

Table 5.3.6.4-2: SIP 180 (Ringing) (step 4b1, Table 5.3.6.3-1)

Derivation Path: Table 5.5.2.16.2.1-1, condition 100rel

Table 5.3.6.4-3: Void

### 5.3.7 - 5.3.9	Void

### 5.3.10	MCX CO call release

5.3.10.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is in RRC_CONNECTED state and a call is established.

5.3.10.2	Void

5.3.10.3	Procedure

Table 5.3.10.3-1: MCX CO call release

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCX Client) send a SIP BYE request to terminate the MCX session? | --> | SIP BYE | - | P |
| 2 | The SS (MCX Server) responds with a SIP 200 (OK) message? | <-- | SIP 200 (OK) | - | - |
| 3 | The procedure 'MCX communication release' as described in clause 5.4.14 is performed to deactivate the dedicated bearer and to release the RRC connection. | - | - | - | - |

5.3.10.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3.11	Void

### 5.3.12	MCX CT call release

5.3.12.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is in RRC_CONNECTED state and a call is established.

5.3.12.2	Void

5.3.12.3	Procedure

Table 5.3.12.3-1: MCX CT call release

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS (MCX Server) sends a SIP BYE request to terminate the MCX session. | <-- | SIP BYE | - | - |
| 2 | Check: Does the UE (MCX Client) respond with a SIP 200 (OK) message? | --> | SIP 200 (OK) | - | P |
| 3 | The procedure 'MCX communication release' as described in clause 5.4.14 is performed to deactivate the dedicated bearer and to release the RRC connection. | - | - | - | - |

5.3.12.4	Specific message contents

All message contents are as specified in clause 5.5. and in the test case calling the procedure, with the following clarifications:

None

### 5.3.13 - 21	Void

### 5.3.22	MCX NW initiated notifications regarding temporary group creation or tear down

5.3.22.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is in RRC_CONNECTED state.

5.3.22.2	Void

5.3.22.3	Procedure

Table 5.3.22.3-1: MCX NW initiated notifications regarding temporary group creation or tear down

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS (MCX server) sends a SIP NOTIFY informing about change of group A's configuration document. | <-- | SIP NOTIFY | - | - |
| 2 | The UE sends a SIP 200 (OK) message. | --> | SIP 200 (OK) | - | - |
| 2A-2F | Void | - | - | - | - |
| 3 | The UE (MCX client) sends an HTTP GET Request message containing the access token and the XCAP-URI of the Group Configuration document. | --> | HTTP GET | - | - |
| 4 | The SS (MCX server) sends the HTTP 200 (OK) message including the updated Group Document | <-- | HTTP 200 (OK) | - | - |
| 5 | The SS (MCX server) sends a SIP NOTIFY message containing the group key transport payloads (GKTP) document including the group keys. | <- | SIP NOTIFY | - | - |
| 5a1-5a2 | Void | - | - | - | - |
| 6 | The UE (MCX client) sends a SIP 200 (OK) message. | --> | SIP 200 (OK) | - | - |

5.3.22.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3.22.4-1: SIP NOTIFY (Step 1)

| Derivation Path: Table 5.5.2.8-1, condition GROUPCONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | xcap-diff |  |  |
| MIME-part-body | Xcap-diff as described in Table 5.3.22.4-1A |  |  |  |

Table 5.3.22.4-1A: Xcap-diff document in SIP NOTIFY (Table 5.3.22.4-1)

Derivation Path: Table 5.5.3.12-2, condition GROUPCONFIG

Table 5.3.22.4-2: SIP 200 (OK) (Steps 2, 6)

Derivation Path: Table 5.5.2.17.1.1-1

Table 5.3.22.4-2A..2G: Void

Table 5.3.22.4-3: HTTP GET (Step 3)

Derivation Path: Table 5.5.4.2-1, condition GROUPCONFIG

Table 5.3.22.4-4: HTTP 200 (OK) (Step 4)

| Derivation Path: Table 5.5.4.6-1, condition GROUPCONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| group-configuration | As described in Table 5.3.22.4-5 | Group Configuration document returned |  |  |

Table 5.3.22.4-5: Group Configuration document (Table 5.3.22.4-4)

| Derivation Path: Table 5.5.7.4-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| list-service[1] |  |  |  |  |
| mcpttgi:on-network-regrouped |  |  | TS 24.481 [31] clause 7.2.4.2 | TEMPGROUPCREATE |
| temporary-MCPTT-group-ID attribute | px_MCPTT_Group_T_ID | MCS temporary group identity | TS 24.481 [31] clause 7.2.4.2 | MCPTT |
|  | px_MCVideo_Group_T_ID |  |  | MCVIDEO |
|  | px_MCData_Group_T_ID |  |  | MCDATA |
| temporary-MCPTT-group-requestor attribute | px_MCPTT_ID_User_B | Identity of the responsible for formatting the MCS temporary group. | TS 24.481 [31] clause 7.2.4.2 | MCPTT |
|  | px_MCVideo_ID_User_B |  |  | MCVIDEO |
|  | px_MCData_ID_User_B |  |  | MCDATA |
| constituent-MCPTT-group-IDs |  |  | TS 24.481 [31] clause 7.2.4.2 |  |
| constituent-MCPTT-group-ID[1] | px_MCPTT_Group_A_ID | MCS group ID of a constituent MCS group of the temporary MCS group | TS 24.481 [31] clause 7.2.4.2 | MCPTT |
|  | px_MCVideo_Group_A_ID |  |  | MCVIDEO |
|  | px_MCData_Group_A_ID |  |  | MCDATA |
| constituent-MCPTT-group-ID[1] | px_MCPTT_Group_B_ID | MCS group ID of a constituent MCS group of the temporary MCS group | TS 24.481 [31] clause 7.2.4.2 | MCPTT |
|  | px_MCVideo_Group_B_ID |  |  | MCVIDEO |
|  | px_MCData_Group_B_ID |  |  | MCDATA |
| protect-media | "true" | Indicates whether confidentiality and integrity of media is required on the MCPTT temporary group | TS 24.481 [31] clause 7.2.4.2 |  |
| protect-floor-control-signalling | "true" | Indicates whether confidentiality and integrity of floor control signalling is required on the temporary MCPTT group | TS 24.481 [31] clause 7.2.4.2 |  |

| Condition | Explanation |
| --- | --- |
| TEMPGROUPCREATE | Procedure is used for creation of a temporary group (but not for tear down) |

Table 5.3.22.4-5A: Void

Table 5.3.22.4-6: SIP NOTIFY (Step 5)

| Derivation Path: Table 5.5.2.8-1, condition GROUPCONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| xcap-diff document | xcap-diff document as described in Table 5.3.22.4-7 |  |  |  |

Table 5.3.22.4-7: xcap-diff document for MCX group configuration (Table5.3.22.4-6)

| Derivation Path: Table 5.5.3.12-2, condition GROUPKEY |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| xcap-diff | encrypted according to NOTE 1 of Table 5.5.3.12-2 |  |  |  |
| element[1] |  |  |  |  |
| sel attribute | Doc-Sel & "~~" & Node-Sel | Document and node selector for Group T according to NOTEs 2a, 2b and 3 of Table 5.5.3.12-2 |  |  |
| GKTPs | group key transport payloads (GKTP) document as described in Table 5.3.22.4-8 |  |  |  |

Table 5.3.22.4-8: group key transport payloads (GKTP) document (Table 5.3.22.4-7)

| Derivation Path: TS 24.481 [11] clause 7.7 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| GKTPs |  |  |  |  |
| GMK-GKTPs |  |  |  |  |
| GKTP[1] | MIKEY message as used in group communication key retrieval procedure | MIKEY message containing the GMK for Group A | TS 33.180 [94] |  |
| id attribute | Same value as used in group communication key retrieval procedure |  |  |  |
| on-network-regrouped-GKTPs[1] |  |  |  | TEMPGROUPCREATE |
| temporary-MCPTT-group-ID attribute | px_MCPTT_Group_T_ID |  |  | MCPTT |
|  | px_MCVideo_Group_T_ID |  |  | MCVIDEO |
|  | px_MCData_Group_T_ID |  |  | MCDATA |
| GKTP[1] | MIKEY message as described in Table 5.3.22.4-9 | MIKEY message containing the GMK for Group T | TS 33.180 [94] |  |
| id attribute | arbitrary value | unique charstring assigned by the SS |  |  |

| Condition | Explanation |
| --- | --- |
| TEMPGROUPCREATE | Procedure is used for creation of a temporary group (but not for tear down) |

Table 5.3.22.4-9: MIKEY-SAKKE I_MESSAGE (GMK distribution by the SS) (Table 5.3.22.4-8)

| Derivation Path: Table 5.5.9.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| General Extension Payload { |  |  |  |
| Content { |  |  |  |
| Payload { |  |  |  |
| Data { |  | See TS 33.180 [94] clause E.6 |  |
| Group IDs { |  |  |  |
| Number of Group IDs | '1' |  |  |
| Group ID | px_MCPTT_Group_T_ID | The ID for the group associated with the key. | MCPTT |
|  | px_MCVideo_Group_T_ID |  | MCVIDEO |
|  | px_MCData_Group_T_ID |  | MCDATA |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ..} |  |  |  |
| } |  |  |  |

### 5.3.23 - 5.3.25	Void

### 5.3.26	MCX CO Group Creation

5.3.26.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is in RRC_CONNECTED state.

5.3.26.2	Void

5.3.26.3	Procedure

Table 5.3.26.3-1: MCX CO Group Creation procedure

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCX Client) send an HTTP PUT to request for creation of the new group? | --> | HTTP PUT | - | P |
| 2 | The SS (MCX Server) sends an HTTP 201 (Created). | <-- | HTTP 201 (Created) | - | - |

5.3.26.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3.27	MCX CO Temporary Group Creation

5.3.27.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is in RRC_CONNECTED state.

5.3.27.2	Void

5.3.27.3	Procedure

Table 5.3.27.3-1: MCX CO Temporary Group Creation procedure

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCX Client) send an HTTP POST to request for creation of a temporary group? | --> | HTTP POST | - | P |
| 2 | The SS (MCX Server) sends an HTTP 200 (OK) containing the GMOP group-regroup-creation-response. | <-- | HTTP 200 (OK) | - | - |

5.3.27.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

Table 5.3.27.4-1..2: Void

### 5.3.28	MCX CO Temporary Group Tear Down

5.3.28.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is in RRC_CONNECTED state.

5.3.28.2	Void

5.3.28.3	Procedure

Table 5.3.28.3-1: MCX CO Temporary Group Creation procedure

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCX Client) send an HTTP DELETE to request for tear down of a temporary group? | --> | HTTP DELETE | - | P |
| 2 | The SS (MCX Server) sends an HTTP 200 (OK). | <-- | HTTP 200 (OK) | - | - |

5.3.28.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3.29	MCX Subscription and Notification

5.3.29.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is either in RRC_IDLE state or in RRC_CONNECTED state.

5.3.29.2	Void

5.3.29.3	Procedure

Table 5.3.29.3-1: MCX Subscription and Notification

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the UE's RRC state. | - | - | - | - |
| 1a1 | IF the UE is in RRC_IDLE state THEN the procedure 'MCX CO communication' as described in clause 5.4.3 is started to establish an RRC connection. | - | - | - | - |
| 2 | Check: Does the UE (MCX Client) send a SIP SUBSCRIBE message request? | --> | SIP SUBSCRIBE | - | P |
| 3 | The SS (MCX Server) responds to the SIP SUBSCRIBE message with a SIP 200 (OK) message. | <-- | SIP 200 (OK) | - | - |
| 4 | The SS (MCX Server) sends a SIP NOTIFY message | <-- | SIP NOTIFY | - | - |
| 5 | The UE (MCX Client) responds with a SIP 200 (OK) message. | --> | SIP 200 (OK) | - | - |
| 6 |  | The procedure 'MCX communication release' as described in clause 5.4.14 is performed to release the RRC connection. |  | - |  | - |  | - |  | - |  |

5.3.29.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3.30	MCX SIP MESSAGE Request - Accept CO

5.3.30.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is either in RRC_IDLE state or in RRC_CONNECTED state.

5.3.30.2	Void

5.3.30.3	Procedure

Table 5.3.30.3-1: MCX SIP MESSAGE Request - Accept CO

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the UE's RRC state. | - | - | - | - |
| 1a1 | IF the UE is in RRC_IDLE state THEN the procedure 'MCX CO communication' as described in clause 5.4.3 is started to establish an RRC connection. | - | - | - | - |
| 2 | Check: Does the UE (MCX Client) send a SIP MESSAGE message? | --> | SIP MESSAGE | - | P |
| 3 | The SS (MCX Server) responds with a SIP 200 (OK) message? | <-- | SIP 200 (OK) | - | - |
| 4 | The SS (MCX server) sends SIP MESSAGE accepting the request. | <-- | SIP MESSAGE | - | - |
| 5 | Check: Does the UE (MCX Client) respond with a SIP 200 (OK) message? | --> | SIP 200 (OK) | - | P |
| 6 | The procedure 'MCX communication release' as described in clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |

5.3.30.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3.31	MCX SIP MESSAGE Request - Accept CT

5.3.31.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is either in RRC_IDLE state or in RRC_CONNECTED state.

5.3.31.2	Void

5.3.31.3	Procedure

Table 5.3.31.3-1: MCX SIP MESSAGE Request - Accept CT

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the UE's RRC state. | - | - | - | - |
| 1a1 | IF the UE is in RRC_IDLE state THEN the procedure 'MCX CT communication' as described in clause 5.4.4 is started to establish an RRC connection. | - | - | - | - |
| 2 | The SS (MCX server) sends SIP MESSAGE | <-- | SIP MESSAGE | - | - |
| 3 | Check: Does the UE (MCX Client) respond with a SIP 200 (OK) message? | --> | SIP 200 (OK) | - | P |
| 4 | Check: Does the UE (MCX Client) send a SIP MESSAGE message? | --> | SIP MESSAGE | - | P |
| 5 | The SS (MCX Server) responds with a SIP 200 (OK) message? | <-- | SIP 200 (OK) | - | - |
| 6 | The procedure 'MCX communication release' as described in clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |

5.3.31.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3.32	MCX SIP MESSAGE CO

5.3.32.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is either in RRC_IDLE state or in RRC_CONNECTED state.

5.3.32.2	Void

5.3.32.3	Procedure

Table 5.3.32.3-1: MCX SIP MESSAGE CO

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the UE's RRC state. | - | - | - | - |
| 1a1 | IF the UE is in RRC_IDLE state THEN the procedure 'MCX CO communication' as described in clause 5.4.3 is started to establish an RRC connection. | - | - | - | - |
| 2 | Check: Does the UE (MCX Client) send a SIP MESSAGE message? | --> | SIP MESSAGE | - | P |
| 3 | The SS (MCX Server) responds with a SIP 200 (OK) message? | <-- | SIP 200 (OK) | - | - |
| 4 | The procedure 'MCX communication release' as described in clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |

5.3.32.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3.33	MCX SIP MESSAGE CT

5.3.33.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is either in RRC_IDLE state or in RRC_CONNECTED state.

5.3.33.2	Void

5.3.33.3	Procedure

Table 5.3.33.3-1: MCX SIP MESSAGE CT

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the UE's RRC state. | - | - | - | - |
| 1a1 | IF the UE is in RRC_IDLE state THEN the procedure 'MCX CT communication' as described in clause 5.4.4 is started to establish an RRC connection. | - | - | - | - |
| 2 | The SS (MCX server) sends SIP MESSAGE | <-- | SIP MESSAGE | - | - |
| 3 | Check: Does the UE (MCX Client) respond with a SIP 200 (OK) message? | --> | SIP 200 (OK) | - | P |
| 4 | The procedure 'MCX communication release' as described in clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |

5.3.33.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3.34	MCX Group Affiliation Status Change

5.3.34.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is either in RRC_IDLE state or in RRC_CONNECTED state.

5.3.34.2	Void

5.3.34.3	Procedure

Table 5.3.34.3-1: MCX Group Affiliation Status Change

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the UE's RRC state. | - | - | - | - |
| 1a1 | IF the UE is in RRC_IDLE state THEN the procedure 'MCX CO communication' as described in clause 5.4.3 is started to establish an RRC connection. | - | - | - | - |
| 2 | Check: Does the UE (MCX Client) send a SIP PUBLISH message? | --> | SIP PUBLISH | - | P |
| 3 | The SS responds to the SIP PUBLISH message with a SIP 200 (OK) message. | <-- | SIP 200 (OK) | - | - |
| 4 | The SS sends a SIP NOTIFY message informing about the status change progress. | <-- | SIP NOTIFY | - | - |
| 5 | The UE responds with a SIP 200 (OK) | --> | SIP 200 (OK) | - | - |
| 6 | The SS sends a SIP NOTIFY informing about the affiliation status of the user. | <-- | SIP NOTIFY | - | - |
| 7 | The UE responds with a SIP 200 (OK) | --> | SIP 200 (OK) | - | - |
| 8 | The procedure 'MCX communication release' as described in clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |

5.3.34.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3.35	MCX CO private call establishment with manual commencement

5.3.35.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is in RRC_IDLE state.

5.3.35.2	Void

5.3.35.3	Procedure

Table 5.3.35.3-1: MCX CO private call establishment with manual commencement

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The procedure 'MCX CO communication' as described in clause 5.4.3 is started to establish an RRC connection. | - | - | - | - |
| 2 | Check: Does the UE (MCX client) send a SIP INVITE requesting the establishment of a private call? | --> | SIP INVITE | - | P |
| - | EXCEPTION: In parallel to the steps below a dedicated bearer gets established as described in clause 5.4.3. | - | - | - | - |
| 3 | The SS sends a SIP 100 Trying | <-- | SIP 100 (Trying) | - | - |
| 4 | The SS (MCX server) responds with a SIP 180 (Ringing) | <-- | SIP 180 (Ringing) | - | - |
| 5 | The SS (MCX server) responds with a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 6 | Check: Does the UE (MCX client) send a SIP ACK to acknowledge the session establishment/modification? | --> | SIP ACK | - | P |

5.3.35.4	Specific message contents

All message contents are as specified in clause 5.5 with condition PRIVATE-CALL where applicable and in the test case calling the procedure, with the following clarifications:

None

### 5.3.36	UE initiated MCX functional alias status determination and subscription

5.3.36.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is either in RRC_IDLE state or in RRC_CONNECTED state.

5.3.36.2	Void

5.3.36.3	Procedure

Table 5.3.36.3-1: MCX functional alias status determination and subscription

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCX client) request to determine the current status of a functional alias and later notification of status changes of a functional alias. (NOTE 1) | - | - | - | - |
| - | EXCEPTION: Step 2a1 describes behaviour that depends on the UE's RRC state. | - | - | - | - |
| 2a1 | IF the UE is in RRC_IDLE state THEN the procedure 'MCX CO communication' as described in clause 5.4.3 is started to establish an RRC connection. | - | - | - | - |
| 3 | Check: Does the UE (MCX client) send a SIP SUBSCRIBE requesting the status of any existing functional aliases? | --> | SIP SUBSCRIBE | - | P |
| 4 | The SS (MCX server) responds with a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 5 | The SS (MCX server) sends a SIP NOTIFY with functional alias information | <-- | SIP NOTIFY | - | - |
| 6 | Check: Does the UE (MCX client) send a SIP 200 (OK)? | --> | SIP 200 (OK) | - | P |
| 7 | The procedure 'MCX communication release' as described in clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI |  |  |  |  |  |

5.3.36.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure with the following clarifications:

Table 5.3.36.4-1: SIP SUBSCRIBE (step 3, Table 5.3.36.3-1)

| Derivation Path: Table 5.5.2.14-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Expires |  |  |  |  |
| value | "4294967295" | to receive the current status and later notification | TS 24.379 [9] clause 9A.2.1.3 TS 24.282 [87] clause 22.2.1.3 |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT Info |  | MCPTT |
| MIME-part-body | MCData-Info as described in Table 5.3.36.4-2 |  | TS 24.379 [9] clause 9A.2.1.3 |  |
| MIME body part |  | MCData Info |  | MCDATA |
| MIME-part-body | MCData-Info as described in Table 5.3.36.4-3 |  | TS 24.282 [87] clause 22.2.1.3 |  |

Table 5.3.36.4-2: MCPTT-Info in SIP SUBSCRIBE (Table 5.3.36.4-1)

| Derivation Path: Table 5.5.3.2.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-request-uri | px_MCPTT_ID_User_A |  | TS 24.379 [9] clause 9A.2.1.3 |  |
| anyExt |  |  |  |  |
| request-type | "functional-alias-status-determination" |  | TS 24.379 [9] clause 9A.2.1.3 |  |

Table 5.3.36.4-3: MCData-Info in SIP SUBSCRIBE (Table 5.3.36.4-1)

| Derivation Path: Table 5.5.3.2.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdatainfo |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "functional-alias-status-determination" |  | TS 24.282 [87] clause 22.2.1.3 |  |
| mcdata-request-uri | px_MCData_ID_User_A |  | TS 24.282 [87] clause 22.2.1.3 |  |

Table 5.3.36.4-4: SIP 200 (OK) (step 4, Table 5.3.36.3-1)

Derivation Path: Table 5.5.2.17.1.2-1, condition SUBSCRIBE-RSP

Table 5.3.36.4-5: SIP NOTIFY (step 5, Table 5.3.36.3-1)

| Derivation Path: Table 5.5.2.8-1, condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | PIDF |  |  |
| MIME-part-body | PIDF for MCPTT as described in Table 5.5.3.5.2-1 (NOTE 1) |  | TS 24.379 [9] clause 9A.2.2.2.5 | MCPTT |
| MIME-part-body | PIDF for MCData as described in Table 5.5.3.5.2-3 (NOTE 1) |  | TS 24.282 [87] clause 22.2.2.2.5 | MCDATA |
| NOTE 1: PIDF document contains tuple with empty <status> element (i.e. there are no <functionalAlias> entries at all) and not containing a <p-id-fa> element |  |  |  |  |

### 5.3.37	UE initiated MCX functional alias status change

5.3.37.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is either in RRC_IDLE state or in RRC_CONNECTED state.

5.3.37.2	Void

5.3.37.3	Procedure

Table 5.3.37.3-1: MCX functional alias status change

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCX client) request to change the status of a functional alias to 'activated'. (NOTE 1) | - | - | - | - |
| - | EXCEPTION: Step 2a1 describes behaviour that depends on the UE's RRC state. | - | - | - | - |
| 2a1 | IF the UE is in RRC_IDLE state THEN the procedure 'MCX CO communication' as described in clause 5.4.3 is started to establish an RRC connection. | - | - | - | - |
| 3 | Check: Does the UE (MCX client) send a SIP PUBLISH requesting the status change of a functional alias? | --> | SIP PUBLISH | - | P |
| 4 | The SS (MCX server) responds with a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 5 | The SS (MCX server) sends a SIP NOTIFY with functional alias information | <-- | SIP NOTIFY | - | - |
| 6 | Check: Does the UE (MCX client) send a SIP 200 (OK)? | --> | SIP 200 (OK) | - | P |
| 7 | The procedure 'MCX communication release' as described in clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI |  |  |  |  |  |

5.3.37.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure with the following clarifications:

Table 5.3.37.4-1: SIP PUBLISH (step 3, Table 5.3.37.3-1)

| Derivation Path: Table 5.5.2.11-1, condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT Info | TS 24.379 [9] clause 9A.2.1.2 | MCPTT |
| MIME-part-body | MCData-Info as described in Table 5.3.37.4-2 |  |  |  |
| MIME body part |  | MCData Info | TS 24.282 [87] clause 22.2.1.2 | MCDATA |
| MIME-part-body | MCData-Info as described in Table 5.3.37.4-3 |  |  |  |
| MIME body part |  | PIDF |  |  |
| MIME-part-body | PIDF for MCPTT as described in Table 5.3.37.4-4 |  | TS 24.379 [9] clause 9A.2.1.2 | MCPTT |
| MIME-part-body | PIDF for MCData as described in Table 5.3.37.4-5 |  | TS 24.282 [87] clause 22.2.1.2 | MCDATA |

Table 5.3.37.4-2: MCPTT-Info in SIP PUBLISH (Table 5.3.37.4-1)

| Derivation Path: Table 5.5.3.2.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-request-uri | px_MCPTT_ID_User_A |  | TS 24.379 [9] clause 9A.2.1.2 |  |

Table 5.3.37.4-3: MCData-Info in SIP PUBLISH (Table 5.3.37.4-1)

| Derivation Path: Table 5.5.3.2.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-request-uri | px_MCData_ID_User_A |  | TS 24.282 [87] clause 22.2.1.2 |  |

Table 5.3.37.4-4: PIDF for MCPTT in SIP PUBLISH (Table 5.3.37.4-1)

Derivation Path: Table 5.5.3.5.1-1, condition FUNCTIONAL_ALIAS_STATUS_CHANGE

Table 5.3.37.4-5: PIDF for MCData in SIP PUBLISH (Table 5.3.37.4-1)

Derivation Path: Table 5.5.3.5.1-3, condition FUNCTIONAL_ALIAS_STATUS_CHANGE

Table 5.3.37.4-6: SIP 200 (OK) (step 4, Table 5.3.37.3-1)

Derivation Path: Table 5.5.2.17.1.2-1, condition PUBLISH-RSP

Table 5.3.37.4-7: SIP NOTIFY (step 5, Table 5.3.37.3-1)

| Derivation Path: Table 5.5.2.8-1, condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | PIDF |  |  |
| MIME-part-body | PIDF for MCPTT as described in Table 5.3.37.4-8 |  | TS 24.379 [9] clause 9A.2.2.2.5 | MCPTT |
| MIME-part-body | PIDF for MCData as described in Table 5.3.37.4-9 |  | TS 24.282 [87] clause 22.2.2.2.5 | MCDATA |

Table 5.3.37.4-8: PIDF for MCPTT in SIP NOTIFY (Table 5.3.37.4-7)

Derivation Path: Table 5.5.3.5.2-1, condition FUNCTIONAL_ALIAS_ACTIVATED, NOTIFY_FOR_PUBLISH

Table 5.3.37.4-9: PIDF for MCData in SIP NOTIFY (Table 5.3.37.4-7)

Derivation Path: Table 5.5.3.5.2-3, condition FUNCTIONAL_ALIAS_ACTIVATED, NOTIFY_FOR_PUBLISH