3GPP TSG-RAN5 Meeting #103	R5-242340

# Fukuoka, Japan, 20th – 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0347 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of initial conditions and RRC/NAS signalling in clause 5.3C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Test configurations are introduced in clause 5.2 and it shall be clarified for which configuration a particular procedure is applicable. - Information about the RRC state has been missing in the initial conditions of the test procedures. - There is a new procedure 5.4.14 for MCX communication release. - Editorial issues. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Applicability for test configuraions added to initial conditions of the test prcedures. - RRC state added to the initial conditions of the test procedures. - Use of new procedure 5.4.14 for MCX communication release. - Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.579-1 CRs 0356, 0357 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 5.3C	Generic test procedures for UE MCData operation

### 5.3C.1	CO SDS or FD message transfer using signalling plane

5.3C.1.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is either in RRC_IDLE state or in RRC_CONNECTED state.

5.3C.1.2	Void

5.3C.1.3	Procedure

Table 5.3C.1.3-1: CO SDS or FD message transfer using signalling plane

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the UE's RRC state. | - | - | - | - |
| 1a1 | IF the UE is in RRC_IDLE state THEN the procedure 'MCX CO communication' as described in clause 5.4.3 is started to establish an RRC connection. | - | - | - | - |
| 2 | Check: Does the UE (MCData client) send a SIP MESSAGE request? | --> | SIP MESSAGE | - | P |
| 3 | The SS (MCData server) sends a SIP 202 (Accepted) response | <-- | SIP 202 (Accepted) | - | - |
| 4 | The procedure 'MCX communication release' as described in clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |

5.3C.1.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3C.2	CO MCData Call Establishment

5.3C.2.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is in RRC_IDLE state.

5.3C.2.2	Void

5.3C.2.3	Procedure

Table 5.3C.2.3-1: CO MCData Call Establishment

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The procedure 'MCX CO communication' as described in clause 5.4.3 is started to establish an RRC connection. | - | - | - | - |
| 2 | Check: Does the UE (MCData client) send a SIP INVITE requesting the establishment of an MCData call? | --> | SIP INVITE | - | P |
| - | EXCEPTION: In parallel to the steps below a dedicated bearer gets established as described in clause 5.4.3. | - | - | - | - |
| 3 | The SS sends a SIP 100 Trying | <-- | SIP 100 (Trying) | - | - |
| 4 | The SS (MCData server) responds with a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 5 | Check: Does the UE (MCData client) send a SIP ACK to acknowledge the session establishment/modification? | --> | SIP ACK | - | P |
| 6 | The UE (MCData client) connects to the TCP server at the SS side to establish an MSRP connection. (NOTE 1) | - | - | - | - |
| 7 | Check: Does the UE (MCData client) send an empty MSRP SEND request to bind the TCP connection to the MSRP session? | --> | MSRP SEND | - | P |
| 8 | The SS (MCData server) sends an MSRP 200 (OK) response. | <-- | MSRP 200 (OK) | - | - |
| NOTE 1: According to TS 24.282 [87] clauses 9.2.3.4.2, 9.2.4.4.2 and 10.2.5.4.2 the SS sets the a=setup attribute set to "passive" (see table 5.5.3.1.2-3) The UE's MCData client has the role of the active endpoint |  |  |  |  |  |

5.3C.2.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3C.2.4-1: MSRP SEND (Step 7, Table 5.3C.2.3-1)

Derivation Path: Table 5.5.12.1-1, condition EMPTY_SEND_REQ

### 5.3C.3	CT MCData Call Establishment

5.3C.3.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is in RRC_IDLE state.

5.3C.3.2	Void

5.3C.3.3	Procedure

Table 5.3C.3.3-1: CT MCData Call Establishment

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The procedure 'MCX CT communication' as described in clause 5.4.4 is started to establish an RRC connection and a dedicated bearer. | - | - | - | - |
| 2 | The SS (MCX Server) sends a SIP INVITE requesting the establishment of an MCData call. | <-- | SIP INVITE | - | - |
| - | EXCEPTION: Step 3a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE responds to a SIP INVITE with a SIP 100 (Trying) | - | - | - | - |
| 3a1 | The UE (MCX client) sends a SIP 100 (Trying) | --> | SIP 100 (Trying) | - | - |
| 4 | Check: Does the UE (MCX client) send a SIP 200 (OK)? | --> | SIP 200 (OK) | - | P |
| 5 | The SS (MCX server) sends a SIP ACK | <-- | SIP ACK | - | - |
| - | EXCEPTION: Steps 6a1 - 6b3 describe behaviour that depends on which role of an endpoint the UE (MCData client) has chosen in its SDP answer sent at step 4 | - | - | - | - |
| 6a1 | IF the UE (MCData client) acts as passive endpoint (NOTE 1) THEN the SS connects to the TCP server at the UE side to establish an MSRP connection | - | - | - | - |
| 6a2 | The SS sends an empty MSRP SEND request to bind the TCP connection to the MSRP session. | <-- | MSRP SEND | - | - |
| 6a3 | Check: Does the UE (MCData client) send an MSRP 200 (OK) response? | --> | MSRP 200 (OK) | - | P |
| 6b1 | ELSE (NOTE 2) the UE (MCData client) connects to the TCP server at the SS side to establish an MSRP connection | - | - | - | - |
| 6b2 | Check: Does the UE (MCData client) send an empty MSRP SEND request to bind the TCP connection to the MSRP session? | --> | MSRP SEND | - | P |
| 6b3 | The SS (MCData server) sends an MSRP 200 (OK) response. | <-- | MSRP 200 (OK) | - | - |
| NOTE 1: The MCData client indicates to act as passive endpoint by setting the a=setup attribute of the SDP answer at step 4 to "passive" (according to RFC 4145 [119]) NOTE 2: The MCData client indicates to act as active endpoint by setting the a=setup attribute of the SDP answer at step 4 to "active" (according to RFC 4145 [119]) |  |  |  |  |  |

5.3C.3.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3C.3.4-1: MSRP SEND (Step 6a2, Table 5.3C.3.3-1)

Derivation Path: Table 5.5.12.2-1, condition EMPTY_SEND_REQ

Table 5.3C.3.4-2: MSRP SEND (Step 6b2, Table 5.3C.3.3-1)

Derivation Path: Table 5.5.12.1-1, condition EMPTY_SEND_REQ

### 5.3C.4	CO MSRP message transfer

5.3C.4.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is in RRC_CONNECTED state with an MSRP connection established.

5.3C.4.2	Void

5.3C.4.3	Procedure

Table 5.3C.4.3-1: CO MSRP message transfer

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1-2 are repeated until the UE (MCData client) indicates the end of the message by setting the continuation-flag to "$" in the End-line of the MSRP SEND request at step 1 | - | - | - | - |
| 1 | Check: Does the UE (MCData client) send an MSRP SEND request? | --> | MSRP SEND | - | P |
| 2 | The SS (MCData server) sends an MSRP 200 (OK) response. | <-- | MSRP 200 (OK) | - | - |
| 3 | In case of chunking the SS reassembles the data contained in the bodies of the MSRP SEND requests. (NOTE 1) | - | - | - | - |
| NOTE 1: In case of no chunking there is only one MSRP SEND request which contains the entire data. In case of chunking there are more than one MSRP SEND requests containing the chunks of data and the content type shall be the same for all MSRP SEND requests. |  |  |  |  |  |

5.3C.4.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3C.5	CT MSRP message transfer

5.3C.5.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is in RRC_CONNECTED state with an MSRP connection established.

5.3C.5.2	Void

5.3C.5.3	Procedure

Table 5.3C.5.3-1: CT MSRP message transfer

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends an MSRP SEND request containing the entire data. (NOTE 1) | <-- | MSRP SEND | - | - |
| 2 | Check: Does the UE (MCData client) send an MSRP 200 (OK) response? | --> | MSRP 200 (OK) | - | P |
| NOTE 1: No chunking is applied in DL. |  |  |  |  |  |

5.3C.5.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3C.6	CO MCData call release

5.3C.6.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is in RRC_CONNECTED state with an MSRP connection established.

5.3C.6.2	Void

5.3C.6.3	Procedure

Table 5.3C.6.3-1: CO MCData call release

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCData client) send a SIP BYE request to terminate the MCData communication? | --> | SIP BYE | - | P |
| 2 | The SS (MCData server) sends a SIP 200 (OK) response. | <-- | SIP 200 (OK) | - | - |
| - | EXCEPTION: Steps 3a1 - 3b1 describe behaviour that depends on the endpoint role the UE (MCData client) has chosen at call establishment. (NOTE 1) | - | - | - | - |
| 3a1 | IF the client is the active endpoint THEN the SS waits 3s for the client to close the MSRP TCP connection. (NOTE 2) | - | - | - | - |
| 3b1 | ELSE the SS closes the MSRP TCP connection. (NOTE 3) | - | - | - | - |
| 4-5 | Steps 1-2 of procedure 'MCX communication release' as described in clause 5.4.14 are performed to deactivate the dedicated bearer. (NOTE 4) | - | - | - | - |
| NOTE 1: The endpoint role is negotiated in the SDP signalling at call establishment (table 5.3C.2.3-1 and 5.3C.3.3-1) NOTE 2: After the wait period the SS may stop the MSRP TCP server independent from whether or not the UE has closed the connection. NOTE 3: When the SS has the role of the active endpoint it means that the MCData client hosts the TCP server of the MSRP connection. NOTE 4: The RRC connection is kept to allow subsequent signalling using the control plane as e.g. an SDS NOTIFICATION in case of Standalone SDS. |  |  |  |  |  |

5.3C.6.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3C.7	CT MCData call release

5.3C.7.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is in RRC_CONNECTED state with an MSRP connection established.

5.3C.7.2	Void

5.3C.7.3	Procedure

Table 5.3C.7.3-1: CT MCData call release

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS (MCData server) sends a SIP BYE request to terminate the MCData communication. | <-- | SIP BYE | - | - |
| 2 | Check: Does the UE (MCData client) send a SIP 200 (OK) response? | --> | SIP 200 (OK) | - | P |
| - | EXCEPTION: Steps 3a1 - 3b1 describe behaviour that depends on the endpoint role the UE (MCData client) has chosen at call establishment. (NOTE 1) | - | - | - | - |
| 3a1 | IF the client is the active endpoint THEN the SS waits 3s for the client to close the MSRP TCP connection. (NOTE 2) | - | - | - | - |
| 3b1 | ELSE the SS closes the MSRP TCP connection. (NOTE 3) | - | - | - | - |
| 4-5 | Steps 1-2 of procedure 'MCX communication release' as described in clause 5.4.14 are performed to deactivate the dedicated bearer. (NOTE 4) | - | - | - | - |
| NOTE 1: The endpoint role is negotiated in the SDP signalling at call establishment (table 5.3C.2.3-1 and 5.3C.3.3-1) NOTE 2: After the wait period the SS may stop the MSRP TCP server independent from whether or not the UE has closed the connection.. NOTE 3: When the SS has the role of the active endpoint it means that the MCData client hosts the TCP server of the MSRP connection. NOTE 4: The RRC connection is kept to allow subsequent signalling using the control plane as e.g. an SDS NOTIFICATION in case of Standalone SDS. |  |  |  |  |  |

5.3C.7.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3C.8	Discovery of the absolute URI of the media storage function (one-to-one communication)

5.3C.8.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is either in RRC_IDLE state or in RRC_CONNECTED state.

5.3C.8.2	Void

5.3C.8.3	Procedure

Table 5.3C.8.3-1: Discovery of the absolute URI of the media storage function (one-to-one)

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the UE's RRC state and on the UE implementation. | - | - | - | - |
| 1a1 | IF the UE is in RRC_IDLE state and pc_MCData_MSFDiscoverySignalling THEN the procedure 'MCX CO communication' as described in clause 5.4.3 is started to establish an RRC connection. | - | - | - | - |
| - | EXCEPTION: Steps 2a1 – 2b1 describe behaviour that depends on the UE implementation | - | - | - | - |
| 2a1 | IF pc_MCData_MSFDiscoverySignalling THEN Check: Does the UE (MCData client) send a SIP MESSAGE request to discover the absolute URI of the media storage function? | --> | SIP MESSAGE | - | P |
| 2a2 | The SS (MCData server) sends a SIP 200 (OK) response. | <-- | SIP 200 (OK) | - | - |
| 2a3 | The SS (MCData server) sends a SIP MESSAGE request containing the absolute URI of the media storage function in the <mcdata-controller-psi> element of the mcdata-info. | <-- | SIP MESSAGE | - | - |
| 2a4 | Check: Does the UE (MCData client) send a SIP 200 (OK) response? | --> | SIP 200 (OK) | - | P |
| 2b1 | ELSE the UE determines the value of the absolute URI associated with the media storage function of the MCData content server from the <MCDataContentServerURI> element of the MCData user profile document | - | - | - | - |

5.3C.8.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3C.8.4-1: SIP MESSAGE from the UE (step 2a1, Table 5.3C.8.3-1)

| Derivation Path: Table 5.5.2.7.1-1, condition MCDATA_FD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 5.3C.8.4-2 |  |  |  |

Table 5.3C.8.4-2: MCDATA-Info from the UE (Table 5.3C.8.4-1)

| Derivation Path: Table 5.5.3.2.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "msf-disc-req" |  |  |  |

Table 5.3C.8.4-3: SIP MESSAGE from the SS (step 2a3, Table 5.3C.8.3-1)

| Derivation Path: Table 5.5.2.7.2-1, condition MCDATA_FD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Request-URI | tsc_MCData_PublicServiceId_A | According to TS 24.282 [87] clause 10.2.1.3.3 the participating function just forwards the SIP MESSAGE received from the controlling function to the client |  |  |
| Accept-Contact |  |  |  |  |
| ac-value[2] | not present |  |  |  |
| P-Asserted-Identity |  |  |  |  |
| name-addr | px_MCX_SIP_PublicUserId_A_1 | Public user ID of the calling MCData user (TS 24.282 [87] clause 10.2.1.3.4) |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 5.3C.8.4-4 |  |  |  |

Table 5.3C.8.4-4: MCDATA-Info from the SS (Table 5.3C.8.4-3)

| Derivation Path: Table 5.5.3.2.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "msf-disc-res" |  |  |  |
| mcdata-request-uri | not present |  |  |  |
| mcdata-calling-user-id | not present |  |  |  |
| mcdata-controller-psi | Encrypted <mcdata-controller-psi> with mcdataURI set to tsc_MCData_MSF_URI | Encrypted according to Table 5.5.3.2.2-3A |  |  |

### 5.3C.9	Discovery of the absolute URI of the media storage function (group communication)

5.3C.9.1	Initial conditions

Same as 5.3C.8.1.

5.3C.9.2	Void

5.3C.9.3	Procedure

Same as 5.3C.8.3.

5.3C.9.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3C.9.4-1: SIP MESSAGE from the UE (step 2a1, Table 5.3C.8.3-1)

| Derivation Path: Table 5.5.2.7.1-1, condition MCDATA_FD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 5.3C.9.4-2 |  |  |  |

Table 5.3C.9.4-2: MCDATA-Info from the UE (Table 5.3C.9.4-1)

| Derivation Path: Table 5.5.3.2.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "msf-disc-req" |  |  |  |
| mcdata-calling-group-id | Encrypted <mcdata-calling-group-id> with mcdataURI set to px_MCData_Group_A_ID | Encrypted according to Table 5.5.3.2.1-3A |  |  |

Table 5.3C.9.4-3: SIP MESSAGE from the SS (step 2a3, Table 5.3C.8.3-1)

Same as Table 5.3C.8.4-3

### 5.3C.10	FD file upload using HTTP

5.3C.10.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is either in RRC_IDLE state or in RRC_CONNECTED state.

5.3C.10.2	Void

5.3C.10.3	Procedure

Table 5.3C.10.3-1: FD file upload using HTTP

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the UE's RRC state. | - | - | - | - |
| 1a1 | IF the UE is in RRC_IDLE state THEN the procedure 'MCX CO communication' as described in clause 5.4.3 is started to establish an RRC connection. | - | - | - | - |
| 2 | Check: Does the UE (MCData client) send an HTTP POST request to upload a file to the media storage function? | --> | HTTP POST | - | P |
| 3 | The SS (MCData server) sends an HTTP 201 Created response containing a Location header field with a URL identifying the location of the resource where the file has been stored at the media storage function. | <-- | HTTP 201 Created | - | - |
| 4 | Check: Does the UE (MCData client) send a SIP MESSAGE request containing an FD SIGNALLING PAYLOAD with Payload content type "FILEURL" and with the Payload data containing the URL of the file? | --> | SIP MESSAGE | - | P |
| 5 | The SS (MCData server) sends a SIP 202 (Accepted) response | <-- | SIP 202 (Accepted) | - | - |
| 6 | The procedure 'MCX communication release' as described in clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |

5.3C.10.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3C.11	FD file accept and download using HTTP

5.3C.11.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is either in RRC_IDLE state or in RRC_CONNECTED state.

5.3C.11.2	Void

5.3C.11.3	Procedure

Table 5.3C.11.3-1: FD file accept and download using HTTP

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the UE's RRC state. | - | - | - | - |
| 1a1 | IF the UE is in RRC_IDLE state THEN the procedure 'MCX CO communication' as described in clause 5.4.3 is started to establish an RRC connection. | - | - | - | - |
| 2 | Check: Does the UE (MCData client) send a SIP MESSAGE request containing an FD NOTIFICATION with FD disposition notification type "FILE DOWNLOAD REQUEST ACCEPTED"? | --> | SIP MESSAGE | - | P |
| 3 | The SS (MCData server) sends a SIP 202 (Accepted) response | <-- | SIP 202 (Accepted) | - | - |
| 4 | Check: Does the UE (MCData client) send an HTTP GET request to download the file? | --> | HTTP GET | - | P |
| 5 | SS (MCData server) sends an HTTP 200 OK response containing the requested file. | <-- | HTTP 200 OK | - | - |
| - | EXCEPTION: Steps 6a1 describes behaviour that depends on the test case requirements; the "lower case letter" identifies a step sequence that takes place when the SS has included a FD disposition request of "FILE DOWNLOAD COMPLETED UPDATE" in the FD SIGNALLING PAYLOAD | - | - | - | - |
| 6a1 | Check: Does the UE (MCData client) send a SIP MESSAGE request containing an FD NOTIFICATION with disposition notification type "FILE DOWNLOAD COMPLETED"? | --> | SIP MESSAGE | - | P |
| 6a2 | The SS (MCData server) sends a SIP 202 (Accepted) response | <-- | SIP 202 (Accepted) | - | - |
| 7 | The procedure 'MCX communication release' as described in clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |

5.3C.11.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3C.12	CO MCData call establishment using a pre-established session

5.3C.12.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is either in RRC_IDLE state or in RRC_CONNECTED state.

5.3C.12.2	Void

5.3C.12.3	Procedure

Table 5.3C.12.3-1: CO MCData Call Establishment

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the UE's RRC state. | - | - | - | - |
| 1a1 | IF the UE is in RRC_IDLE state THEN the procedure 'MCX CO communication' as described in clause 5.4.3 is started to establish an RRC connection. | - | - | - | - |
| 2 | Check: Does the UE (MCData client) send a SIP REFER message to request the establishment of an MCPTT call using a pre-established session? | --> | SIP REFER | - | P |
| 3 | The SS (MCData server) responds with a SIP 200 (OK) message indicating that the MCPTT call has been established | <-- | SIP 200 (OK) | - | - |
| 4 | The SS (MCX Server) sends a SIP re-INVITE to verify that the MCData call has been established. | <-- | SIP INVITE | - | - |
| - | EXCEPTION: Step 5a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE responds to a SIP INVITE with a SIP 100 (Trying). | - | - | - | - |
| 5a1 | The UE (MCX client) sends a SIP 100 (Trying) | --> | SIP 100 (Trying) | - | - |
| 6 | Check: Does the UE (MCX client) respond to the SIP re-INVITE with SIP 200 (OK)? | --> | SIP 200 (OK) | - | P |
| 7 | The SS (MCX server) sends a SIP ACK in response to the SIP 200 (OK) message. | <-- | SIP ACK | - | - |
| 8 | The UE (MCData client) connects to the TCP server at the SS side to establish an MSRP connection. (NOTE 1) | - | - | - | - |
| 9 | Check: Does the UE (MCData client) send an empty MSRP SEND request to bind the TCP connection to the MSRP session? | --> | MSRP SEND | - | P |
| 10 | The SS (MCData server) sends an MSRP 200 (OK) response. | <-- | MSRP 200 (OK) | - | - |
| NOTE 1: According to TS 24.282 [87] clauses 9.2.3.4.2, 9.2.4.4.2 and 10.2.5.4.2 the SS sets the a=setup attribute set to "passive" (see table 5.5.3.1.2-3) The UE's MCData client has the role of the active endpoint |  |  |  |  |  |

5.3C.12.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3C.12.4-1: SIP re-INVITE from the SS (step 4, Table 5.3C.12.3-1)

| Derivation Path: Table 5.5.2.5.2-1, condition MCDATA_SDS, re_INVITE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Request-URI | tsc_MCX_SessionID_B | session identity of the pre-established session | TS 24.282 [87] clause 9.2.5.2.2.1 |  |
| Message-body |  |  |  |  |
| MIME body part | not present | SDP message |  |  |
| MIME body part |  | MCData Info |  |  |
| MIME-part-body | MCData-Info message as described in Table 5.3C.12.4-2 |  |  |  |
| Editor’s Note: TS 24.282 [87] clause 9.2.5.1.2 does not clearly specify the header fields of the INVITE and therefore the default header fields are used |  |  |  |  |

Table 5.3C.12.4-2: MCData-Info (Table 5.3C.12.4-1)

| Derivation Path: Table 5.5.3.2.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-request-uri | not present |  |  |  |
| mcdata-calling-user-id | not present |  |  |  |
| anyExt |  |  |  |  |
| mcdata-communication-state | "establish-success" |  | TS 24.282 [87] clause 9.2.5.1.2 |  |

Table 5.3C.12.4-3: SIP 200 (OK) from the UE (step 6, Table 5.3C.12.3-1)

| Derivation Path: Table 5.5.2.17.1.1-1, condition INVITE-RSP, MCDATA_SDS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type | not present |  |  |  |
| Message-body | not present |  |  |  |

Table 5.3C.12.4-4: MSRP SEND (Step 9, Table 5.3C.12.3-1)

Derivation Path: Table 5.5.12.1-1, condition EMPTY_SEND_REQ

### 5.3C.13	MCData CO call release keeping the pre-established session

5.3C.13.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is in RRC_CONNECTED state with an MSRP connection established using a pre-established session.

5.3C.13.2	Void

5.3C.13.3	Procedure

Table 5.3C.13.3-1: MCData CO call release keeping the pre-established session

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCData client) send a SIP REFER message with method “BYE” to release the MCData session and keep the pre-established session? | --> | SIP REFER | - | P |
| 2 | The SS (MCData server) responds with a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 3 | The SS (MCX Server) sends a SIP re-INVITE to verify the release of the MCData call. | <-- | SIP INVITE | - | - |
| - | EXCEPTION: Step 4a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE responds to a SIP INVITE with a SIP 100 (Trying). | - | - | - | - |
| 4a1 | The UE (MCX client) sends a SIP 100 (Trying) | --> | SIP 100 (Trying) | - | - |
| 5 | Check: Does the UE (MCX client) respond to the SIP re-INVITE with SIP 200 (OK)? | --> | SIP 200 (OK) | - | P |
| 6 | The SS (MCX server) sends a SIP ACK in response to the SIP 200 (OK) message. | <-- | SIP ACK | - | - |
| - | EXCEPTION: Steps 7a1 - 7b1 describe behaviour that depends on the endpoint role the UE (MCData client) has chosen at call establishment. (NOTE 1) | - | - | - | - |
| 7a1 | IF the client is the active endpoint THEN the SS waits 3s for the client to close the MSRP TCP connection. (NOTE 2) | - | - | - | - |
| 7b1 | ELSE the SS closes the MSRP TCP connection. (NOTE 3) | - | - | - | - |
| NOTE 1: The endpoint role is negotiated in the SDP signalling at call establishment. NOTE 2: After the wait period the SS may stop the MSRP TCP server independent from whether or not the UE has closed the connection. NOTE 3: When the SS has the role of the active endpoint it means that the MCData client hosts the TCP server of the MSRP connection. |  |  |  |  |  |

5.3C.13.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3C.13.4-1: SIP REFER (step 1, Table 5.3C.13.3-1)

Derivation Path: Table 5.5.2.12-1, condition METHOD-BYE

Table 5.3C.13.4-2: SIP 200 (OK) (step 2, Table 5.3C.13.3-1)

Derivation Path: Table 5.5.2.17.1.2-1, condition REFER-RSP

Table 5.3C.13.4-3: SIP re-INVITE from the SS (step 3, Table 5.3C.13.3-1)

| Derivation Path: Table 5.5.2.5.2-1, condition MCDATA_SDS, re_INVITE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Request-URI | tsc_MCX_SessionID_B | session identity of the pre-established session | TS 24.282 [87] clause 9.2.5.4.2.1 |  |
| Message-body |  |  |  |  |
| MIME body part | not present | SDP message |  |  |
| MIME body part |  | MCData Info |  |  |
| MIME-part-body | MCData-Info message as described in Table 5.3C.13.4-4 |  |  |  |
| Editor’s Note: TS 24.282 [87] clause 9.2.5.1.2 does not clearly specify the header fields of the INVITE and therefore the default header fields are used |  |  |  |  |

Table 5.3C.13.4-4: MCData-Info (Table 5.3C.13.4-3)

| Derivation Path: Table 5.5.3.2.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-request-uri | not present |  |  |  |
| mcdata-calling-user-id | not present |  |  |  |
| anyExt |  |  |  |  |
| mcdata-communication-state | "terminated" |  | TS 24.282 [87] clause 9.2.5.4.2.1 |  |

Table 5.3C.13.4-5: SIP 200 (OK) from the UE (step 5, Table 5.3C.13.3-1)

| Derivation Path: Table 5.5.2.17.1.1-1, condition INVITE-RSP, MCDATA_SDS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type | not present |  |  |  |
| Message-body | not present |  |  |  |