3GPP TSG-RAN5 Meeting #103	R5-242343

# Fukuoka, Japan, 20th – 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0350 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections of clause 5.5.2.15.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - For SIP UPDATE request sent by the SS there is no need to distinguish the Via header field for MO and MT call: In both cases it results in the same Via header field as specified for the SIP INVITE request sent by the SS but with updated via-branches. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Via header field clarified in Table 5.5.2.15.2-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.2.15.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 5.5.2.15.2	SIP UPDATE from the SS

Table 5.5.2.15.2-1: SIP UPDATE from the SS

| Derivation Path: TS 24.229 [16] A.2.1.4.14, A.2.2.4.14 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| Method | "UPDATE" |  |  |  |
| Request-URI | same URI as the UE has sent earlier in the Contact header of a response within the same dialog | Contact URI of the UE ("callee") |  |  |
| SIP-Version | 'SIP/2.0" |  |  |  |
| Via | same as specified for INVITE sent by the SS in Table 5.5.2.5.2-1 with updated via-branches |  | RFC 3261 [22] |  |
| From |  |  | RFC 3261 [22] |  |
| addr-spec | Same URI of the SS as used earlier in the dialog | Remote URI of the dialog (from the UE's point of view) |  |  |
| tag | Same tag of the SS as used earlier in the dialog | Remote tag of the dialog (from the UE's point of view) |  |  |
| To |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| addr-spec | Same URI of the UE as used earlier in the dialog | Local URI of the dialog (from the UE's point of view) |  |  |
| tag | Same tag of the UE as used earlier in the dialog | Local tag of the dialog (from the UE's point of view) |  |  |
| Call-ID |  |  | RFC 3261 [22] |  |
| callid | Same value as used in the INVITE initiating the dialog |  |  |  |
| Contact | same as in the response for the INVITE creating the dialog |  | RFC 3261 [22] | MO_CALL |
|  | same as in the INVITE creating the dialog |  |  | MT_CALL |
| CSeq |  |  | RFC 3261 [22] |  |
| value | value of CSeq sent by the endpoint within its previous request in the same dialog but increased by one |  |  |  |
| method | "UPDATE" |  |  |  |
| Max-Forwards |  |  | RFC 3261 [22] |  |
| value | "68" | The recommended initial value is 70 in RFC 3261 [22]. Assuming 2 hops as according to the Via header this results in a value of 68 in the message sent to the UE. |  |  |
| Content-Type |  |  | RFC 5621 [58] |  |
| media-type | "application/sdp" |  |  |  |
| Content-Length | length of message-body |  | RFC 3261 [22] |  |
| value | length of message-body |  |  |  |
| Message-body |  |  | RFC 3261 [22] |  |
| SDP Message | SDP Message as described in Table 5.5.3.1.1-2 |  |  |  |
|  | SDP Message as described in Table 5.5.3.1.2-2 |  |  | MCVIDEO |
|  | SDP Message as described in Table 5.5.3.1.2-3 |  |  | MCDATA |