3GPP TSG-RAN5 Meeting #103	R5-243182

# Fukuoka, Japan,  20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-2 | CR | 1454 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of applicability for test case 6.1.2.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, LTE_feMTC-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As part of the analysis of the applicability of legacy test cases for Rel-14 CAT-M1 UEs, several test cases became applicable. The corresponding applicability statements was updates in Tdoc R5-204529 at RAN5#88-e. During periodic GCF review it was discovered that the applicability for test case 6.1.2.9 was never implemented in the specification.This CR updates the applicability as intended by the original CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated test case applicability for test case 6.1.2.9 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case applicability for test case 6.1.2.9 will not be correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 4	Recommended Test Case Applicability

The applicability of each individual test is identified in Table 4-1. This is just a recommendation based on the purpose for which the test case was written.

The applicability of every test is formally expressed by the use of Boolean expression that are based on parameters (ICS) included in annex A of the present document.

Additional information related to the Test Case (TC), e.g. affecting its dynamic behaviour or its execution may be provided as well.

When a test case is to be executed against a category M1 UE and with IMS enabled, it is assumed that the UE is compliant to GSMA profile NG.108 [55].

The columns in Table 4-1 have the following meaning:

Clause

The clause column indicates the clause number in TS 36.523-1 [19] that contains the test body.

Title

The title column describes the name of the test and contains the clause title of the clause in TS 36.523-1 [19] that contains the test body.

Release

The release column indicates the earliest release from which the test case is applicable. In some specific cases it may indicate the release(s) for which the TC is only applicable.

Note:	Some exceptions to this interpretation may be indicated in Notes in column 'Release' e.g. see Note 3 Table 4-1.

Applicability - Condition

The following notations are used for the applicability column:

R	recommended - the test case is recommended

O	optional – the test case is optional

N/A	not applicable - in the given context, the test case is not recommended.

Ci	conditional - the test is recommended ("R") or not ("N/A") depending on the support of other items. "i" is an integer identifying an unique conditional status expression which is defined immediately following the table. For nested conditional expressions, the syntax "IF ... THEN (IF ... THEN ... ELSE...) ELSE ..." is used to avoid ambiguities.

NOTE 1:	The conditions are defined in Table 4-1a.

Applicability - Comments

This column contains a verbal description of the condition.

Additional Information - Specific ICS

This column contains the mnemonics of ICS(s) affecting the dynamic behaviour of the TC.

NOTE 1A:	ICS items specified in 3GPP TS 34.123-2 [8] and 3GPP TS 34.229-2 [45] can be referred, to avoid redundant definitions.

NOTE 1B:	The ICS items pc_eFDD and pc_eFDD, as well as pc_NB_FDD and pc_NB_TDD, specified in the present document (Table A.4.1-1) are used to identify that a test case can be run in FDD or/and TDD branch. When none of them is provided it is assumed that the test case requires both FDD and TDD.

Additional Information - Specific IXIT

This column contains the mnemonics of IXIT(s) affecting the dynamic behaviour of the TC.

Additional Information - Number of TC Executions

This column contains, wherever applicable, the recommended for certification purposes number of TC executions. It may contain also other information e.g. exceptions to the release applicable to the test. Clarifying notes are listed in Table 4-1b.

Additional Information - Release other RAT

In regard to a particular test case, this column provides information on the release which is used by the simulated network in the other (i.e. non E-UTRA) RAT(s) where applicable. For each applicable RAT the release shall be indicated in the format 'Rel-X RAT'. When multiple RATs are applicable the entries per RAT shall be separated by a comma. When a value for a 3GPP RAT is not provided but the RAT is in the scope of the test case then for this RAT the release indicated in the Release column applies (per default), a Note extending the release applicability to an earlier version for E-UTRA in the ‘Release’ column is not applicable to the other RATs.

EXAMPLES:

Rel-9 UTRA FDD, Rel-8 GERAN or simply as Rel-9 UTRA FDD
(meaning that the UTRA FDD will simulate Rel-9 and the GERAN Rel-8 behaviours)

Rel-9 UTRA TDD
(meaning that the UTRA LCR TDD network will simulate Rel-9 behaviours)

NOTE 1C:	Some exceptions to this interpretation may be indicated in Notes in column 'Release other RAT' e.g. see Note 7A Table 4-1.

NOTE 2:	To meet the validation requirements from certification bodies then there is a need to uniquely reference the FDD and TDD branch of common FDD and TDD test cases. The FDD and TDD branches of common FDD and TDD test cases can be referenced by amending a "FDD" or "TDD" suffix to the test case clause number. For example for AM RLC test case 7.2.3.13 the FDD and TDD branches can be identified by "7.2.3.13 FDD" and "7.2.3.13 TDD".

Table 4-1: Applicability of tests and additional information for testing

| Clause | TC Title | Release | Applicability Condition | Applicability Comment | Specific ICS | Specific IXIT | Number of TC Executions | Release other RAT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6 | Idle mode operations |  |  |  |  |  |  |  |
| 6.1.1.1 | PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  | Either TC 6.1.1.1 or TC 6.1.1.1b shall be executed. (Note 4) |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.1a | PLMN selection / Automatic mode / between FDD and TDD | Rel-8 | C142 | UEs supporting E-UTRA FDD and E-UTRA TDD |  |  |  |  |
| 6.1.1.1b | PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / Single Frequency operation | Rel-8 | R | UEs supporting E-UTRA. This test is 'cells on single frequency only' equivalent of TC 6.1.1.1 | pc_eFDD |  | Either TC 6.1.1.1 or TC 6.1.1.1b shall be executed. (Note 4) |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.2 | PLMN selection of "Other PLMN/access technology combinations" / Automatic mode | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  | Either TC 6.1.1.2 or TC 6.1.1.2a shall be executed. (Note 4) |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.2a | PLMN selection of "Other PLMN/access technology combinations" / Automatic mode / Single Frequency operation | Rel-8 | R | UEs supporting E-UTRA This test is 'cells on single frequency only' equivalent of 6.1.1.2 | pc_eFDD |  | Either TC 6.1.1.2 or TC 6.1.1.2a shall be executed. (Note 4) |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.3 | Cell reselection of ePLMN in manual mode | Rel-8 | C388 | UEs supporting E-UTRA and (( NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  | Either TC 6.1.1.3 or TC 6.1.1.3b shall be executed. (Note 4) |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.3a | Cell reselection of ePLMN in manual mode / between FDD and TDD | Rel-9 (Note 3) | C389 | UEs supporting E-UTRA FDD and E-UTRA TDD and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) |  |  |  |  |
| 6.1.1.3b | Cell reselection of ePLMN in manual mode / Single Frequency operation | Rel-8 | R | UEs supporting E-UTRA. This test is 'cells on single frequency only' equivalent of 6.1.1.3 | pc_eFDD |  | Either TC 6.1.1.3 or TC 6.1.1.3b shall be executed. (Note 4) |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.4 | PLMN selection in shared network environment / Automatic mode | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.4a | PLMN selection in shared network environment / Automatic mode / Between FDD and TDD | Rel-8 | C389 | UEs supporting E-UTRA FDD and E-UTRA TDD and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) |  |  |  |  |
| 6.1.1.5 | Void |  |  |  |  |  |  |  |
| 6.1.1.6 | PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / User reselection | Rel-8 | C157a | UEs supporting E-UTRA and user initiated PLMN reselection in automatic mode and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  | Either TC 6.1.1.6 or TC 6.1.1.6a shall be executed. (Note 4) |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.6a | PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / User reselection / Single Frequency operation | Rel-8 | C157 | UEs supporting E-UTRA and user initiated PLMN reselection in automatic mode. This test is 'cells on single frequency only' equivalent of 6.1.1.6 | pc_eFDD |  | Either TC 6.1.1.6 or TC 6.1.1.6a shall be executed. (Note 4) |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.6b | PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / User reselection / Two Frequencies operation | Rel-13 | C157b | UEs supporting E-UTRA and user initiated PLMN reselection in automatic mode. This test is 'cells on two frequencies only' and 'TDD cat.1bis UE only' equivalent of 6.1.1.6 | pc_eTDD |  | Either TC 6.1.1.6 or TC 6.1.1.6b shall be executed. (Note 21) |  |
| 6.1.1.7 | PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer | Rel-10 | C179a | UEs supporting E-UTRA and MinimumPeriodicSearchTimer and not supporting "Fast First Higher Priority PLMN search" and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  | Either TC 6.1.1.7 or TC 6.1.1.7a shall be executed. (Note 4) |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.7a | PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer / Single Frequency operation | Rel-10 | C179 | UEs supporting E-UTRA and MinimumPeriodicSearchTimer and not supporting "Fast First Higher Priority PLMN search". This test is 'cells on single frequency only' equivalent of 6.1.1.7 | pc_eFDD |  | Either TC 6.1.1.7 or TC 6.1.1.7a shall be executed. (Note 4) |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.8 | PLMN selection of RPLMN or (E)HPLMN; Automatic mode | Rel-8 | C212a | UEs supporting E-UTRA and EF_LRPLMSI_Exception and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.9 | PLMN selection of RPLMN or (E)HPLMN; Manual mode | Rel-8 | C213 | UEs supporting E-UTRA and ManualModeNetworkSelectionException | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.10 | eMTC / NTN | Rel-17 | C414 | UEs supporting E-UTRA and Category M1 and NTN access in CE Mode A | pc_eFDD |  | Note 22 |  |
| 6.1.1.11 | eMTC / NTN / Multi-TAC | Rel-17 | C414 | UEs supporting E-UTRA and Category M1 and NTN access in CE Mode A | pc_eFDD |  | Note 22 |  |
| 6.1.1.12 | eMTC / SENSE/ PLMN selection of RPLMN, HPLMN, UPLMN, OPLMN and Other PLMN / Automatic mode | Rel-18 | C425 | UEs supporting E-UTRA and Category M1 and operator controlled signal threshold per access technology. | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.13 | eMTC / SENSE/ PLMN selection of RPLMN or (E)HPLMN / Automatic mode | Rel-18 | C426 | UEs supporting E-UTRA and EF_LRPLMSI_Exception and Category M1 and operator controlled signal threshold per access technology. | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.14 | eMTC / SENSE/ Periodic attempts for signal level enhanced network selection; Automatic mode | Rel-18 | C425 | UEs supporting E-UTRA and Category M1 and operator controlled signal threshold per access technology. | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.2.1 | Void |  |  |  |  |  |  |  |
| 6.1.2.2 | Cell selection / Qrxlevmin | Rel-8 | C224c | UEs supporting E-UTRA and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.2.2a | Cell selection / Qqualmin | Rel-9 (Note 3) | C224c | UEs supporting E-UTRA and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.2.2b | Cell selection / UE Cat 0 not allowed | Rel-12 | C224 | UEs supporting E-UTRA and UE Category 0 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.2.2c | Cell selection / Qrxlevmin / Enhanced Coverage | Rel-13 | C254 | UEs supporting E-UTRA and (CE mode A or CE mode B) | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.2.2d | Cell selection / Qqualmin / Enhanced Coverage | Rel-13 | C254 | UEs supporting E-UTRA and (CE mode A or CE mode B) | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.2.3 | Cell selection / Intra E-UTRAN / Serving cell becomes non-suitable (S<0 or barred) | Rel-8 | C388 | UEs supporting E-UTRA and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.2.3a | Cell selection / Intra E-UTRAN / Serving cell becomes non-suitable (Srxlev > 0 and Squal < 0) | Rel-9 (Note 3) | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.2.4 | Cell reselection | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.2.5 | Cell reselection for interband operation | Rel-8 | C184a | UEs supporting E-UTRA and more than 1 FDD or TDD E-UTRA band and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.2.5a | Cell reselection for interband operation/ Power Class 2 UE operation/ Between FDD and TDD | Rel-14 (Note 17) | C281 | UEs supporting E-UTRA FDD and E-UTRA TDD and Bands38, 40, 41 or 42 Power class 2 operation and NOT Category M1 |  |  |  |  |
| 6.1.2.5b | Cell reselection for interband operation using Pcompensation / Between FDD and TDD | Rel-14 (Note 17) | C389 | UEs supporting E-UTRA FDD and E-UTRA TDD and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) |  |  |  |  |
| 6.1.2.5c | Inter-band Cell reselection / Extended frequency list | Rel-12 | C184a | UEs supporting E-UTRA and more than 1 FDD or TDD E-UTRA band and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.2.6 | Cell reselection using Qhyst, Qoffset and Treselection | Rel-8 | C224c | UEs supporting E-UTRA and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.2.6a | Cell reselection using Treselection / Enhanced Coverage | Rel-13 | C254 | UEs supporting E-UTRA and (CE mode A or CE mode B) | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.2.6b | Cell reselection from cell in enhanced coverage to inter-frequency cell in normal coverage | Rel-13 | C254b | UEs supporting E-UTRA and (CE mode A or CE mode B) and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.2.7 | Cell reselection / Equivalent PLMN | Rel-8 | C388 | UEs supporting E-UTRA and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  | Either TC 6.1.2.7 or TC 6.1.2.7a shall be executed. (Note 4) |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.2.7a | Cell reselection / Equivalent PLMN / Single Frequency operation | Rel-8 | R | UEs supporting E-UTRA. This test is 'cells on single frequency only ' equivalent of 6.1.2.7 | pc_eFDD |  | Either TC 6.1.2.7 or TC 6.1.2.7a shall be executed. (Note 4) |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.2.8 | Cell reselection using cell status and cell reservations / Access control class 0 to 9 | Rel-8 | C388 | UEs supporting E-UTRA and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  | Either TC 6.1.2.8 or TC 6.1.2.8a shall be executed. (Note 4) |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.2.8a | Cell reselection using cell status and cell reservations / Access control class 0 to 9 / Single Frequency operation | Rel-8 | R | UEs supporting E-UTRA. This test is 'cells on single frequency only ' equivalent of 6.1.2.8 | pc_eFDD |  | Either TC 6.1.2.8 or TC 6.1.2.8a shall be executed. (Note 4) |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.2.9 | Cell reselection using cell status and cell reservations / Access control class 11 to 15 | Rel-8 | C388 | UEs supporting E-UTRA and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  | Either TC 6.1.2.9 or TC 6.1.2.9a shall be executed. (Note 4) |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.2.9a | Cell reselection using cell status and cell reservations / Access control class 11 to 15 / Single Frequency operation | Rel-8 | R | UEs supporting E-UTRA. This test is 'cells on single frequency only ' equivalent of 6.1.2.9 | pc_eFDD |  | Either TC 6.1.2.9 or TC 6.1.2.9a shall be executed. (Note 4) |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.2.10 | Cell reselection in shared network environment | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| <<<< ROWS SKIPPED >>>> |  |  |  |  |  |  |  |  |