3GPP TSG-RAN5 Meeting #103	R5-242534

# Fukuoka City, Fukuoka, May 20 - 24 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3140 | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause structure of CA test frequencies |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Editorial updates of the clause structure in clause 4.3.1.1.5 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clause 4.3.1.1.5.48 moved to correct position. In addition, subchapter created for CA_n48(2A) configuration. - Clause 4.3.1.1.5.71: Subchapter created for CA_n71(2A) configuration. - Clause 4.3.1.1.5.77: Subchapter created for CA_n77(2A) configuration. - Clause 4.3.1.1.5.78: Subchapter created for CA_n78(2A) configuration. In addition, erroneous heading format updated. - Clause 6.2.3.4: References updated in Table 6.2.3.4-3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Clause structure does not support addition of CA configurations of type intra band non-contigous 3A. - Incorrect order of clauses. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.5.48, 4.3.1.1.5.71, 4.3.1.1.5.77, 4.3.1.1.5.78, 6.2.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | These clause changes should be implemented after any other CRs are implemented in the affected clauses. Hence the content of tables or text in clauses should not be copied from this CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### << Beginning of changes>>

###### 4.3.1.1.5.3 – 4.3.1.1.5.47	FFS

###### 4.3.1.1.5.48	CA_n48(xA)

4.3.1.1.5.48.1	CA_n48(2A)

Editor’s note:	Test frequencies for CA_n48(2A) with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.5.48.1-1: NR Intra-Band non-contiguous CA configuration CA_n48(2A) without UL CA, SCS=15 kHz, Max Wgap

| Sb CBW combination (BCS) | SB (Note 2) | Bandwidth [MHz] | Range (Note 1,3) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- |
| CA_n48(2A): n48A + n48A |  |  |  |  |  |
| CBW1+CBW2 (0) | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.48-1: Low range for CBW = 10, 15, 20, 40, 50 MHz |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.48-1: High range for CBW = 10, 15, 20, 40, 50 MHz |
| Note 1: Only one range has been specified as the maximum maxAggregatedFsBW for FR1 NR intra-band non-contiguous CA without UL CA equals the full bandwith of the NR band (Table C.2.4.3.1A-1). Note 2: PCC is configured on SB1 unless otherwise stated. Note 3: The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. Note 4: For the 50 MHz bandwidth or more, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. |  |  |  |  |  |

Table 4.3.1.1.5.48.1-2: NR Intra-Band non-contiguous CA configuration CA_n48(2A) without UL CA, SCS=30 kHz, Max Wgap

| SB CBW combination (BCS) | SB (Note 2) | Bandwidth [MHz] | Range (Note 1,3) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- |
| CA_n48(2A): n48A + n48A |  |  |  |  |  |
| CBW1+CBW2 (0) | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.48-2: Low range for CBW = 10, 15, 20, 40, 50, 60, 80, 90, 100 MHz |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.48-2: High range for CBW = 10, 15, 20, 40, 50, 60, 80, 90, 100 MHz |
| Note 1: Only one range has been specified as the maximum maxAggregatedFsBW for FR1 NR intra-band non-contiguous CA without UL CA equals the full bandwith of the NR band (Table C.2.4.3.1A-1). Note 2: PCC is configured on SB1 unless otherwise stated. Note 3: The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. Note 4: For the 50 MHz bandwidth or more, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. |  |  |  |  |  |

4.3.1.1.5.49 – 4.3.1.1.5.65	FFS

###### 4.3.1.1.5.66	CA_n66(xA)

4.3.1.1.5.66.1	CA_n66(2A)

Editor’s note:	Test frequencies for CA_n66(2A) with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.5.66.1-1: NR Intra-Band non-contiguous CA configuration CA_n66(2A) without UL CA, SCS=15 kHz, Max Wgap

| Sb CBW combination (BCS) | SB (Note 2) | Bandwidth [MHz] | Range (Note 1,3) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- |
| CA_n66(2A): n66A + n66A |  |  |  |  |  |
| CBW1+CBW2 (0) | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.66-1: Low range for CBW = 5, 10, 15, 20 MHz, and if SB1 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.66-1: High range for CBW = 5, 10, 15, 20, 40 MHz, and if SB2 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
| CBW1+CBW2 (1) | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.66-1: Low range for CBW = 5, 10, 15, 20, 25, 30, 40 MHz, and if SB1 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.66-1: High range for CBW = 5, 10, 15, 20, 25, 30, 40 MHz, and if SB2 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
| CBW1+CBW2 (2) | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.66-1: Low range for CBW = 5, 10, 15, 20, 40 MHz, and if SB1 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.66-1: High range for CBW = 5, 10, 15, 20, 40 MHz, and if SB2 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
| Note 1: Only one range has been specified as the maximum maxAggregatedFsBW for FR1 NR intra-band non-contiguous CA without UL CA equals the full bandwith of the NR band (Table C.2.4.3.1A-1). Note 2: PCC is configured on SB1 unless otherwise stated. Note 3: The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |

Table 4.3.1.1.5.66.1-2: NR Intra-Band non-contiguous CA configuration CA_n66(2A) without UL CA, SCS=30 kHz, Max Wgap

| SB CBW combination (BCS) | SB (Note 2) | Bandwidth [MHz] | Range (Note 1,3) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- |
| CA_n66(2A): n66A + n66A |  |  |  |  |  |
| CBW1+CBW2 (0) | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.66-2: Low range for CBW = 10, 15, 20 MHz, and if SB1 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.66-2: High range for CBW = 10, 15, 20, 40 MHz, and if SB2 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
| CBW1+CBW2 (1) | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.66-2: Low range for CBW = 10, 15, 20, 25, 30, 40 MHz, and if SB1 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.66-2: High range for CBW = 10, 15, 20, 25, 30, 40 MHz, and if SB2 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
| CBW1+CBW2 (2) | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.66-2: Low range for CBW = 10, 15, 20, 40 MHz, and if SB1 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.66-2: High range for CBW = 10, 15, 20, 40 MHz, and if SB2 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
| Note 1: Only one range has been specified as the maximum maxAggregatedFsBW for FR1 NR intra-band non-contiguous CA without UL CA equals the full bandwith of the NR band (Table C.2.4.3.1A-1). Note 2: PCC is configured on SB1 unless otherwise stated. Note 3: The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |

4.3.1.1.5.66.2	CA_n66(3A)

Editor’s note:	Test frequencies for CA_n66(3A) with mixed numerology is FFS.

Table 4.3.1.1.5.66.2-1: NR Intra-Band non-contiguous CA configuration CA_n66(3A) without UL CA, SCS 15 kHz, frequency raster 100 kHz, and Max Wgap

| SB CBW combination (BCS) | SB (Note 2) | Bandwidth [MHz] | Range (Note 1,3) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- |
| CA_n66(3A): n66A + n66A + n66A |  |  |  |  |  |
| CBW1+CBW2 +CBW3 (0) | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.66-1: Low range for CBW = 5, 10, 15, 20, 40 MHz, and if SB1 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.66-1: Mid range for CBW = 5, 10, 15, 20, 40 MHz, and if SB2 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
|  | SB3 | CBW3 | Downlink & Uplink |  | For CBW3, use Table 4.3.1.1.1.66-1: High range for CBW = 5, 10, 15, 20, 40 MHz, and if SB3 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
| Note 1: Only one range has been specified as the maximum maxAggregatedFsBW for FR1 NR intra-band non-contiguous CA without UL CA equals the full bandwith of the NR band (Table C.2.4.3.1A-1). Note 2: PCC is configured on SB1 unless otherwise stated. Note 3: The test frequencies for SB1, SB2 and SB3 have been specified for both downlink and uplink to enable that the CC of either SB1, SB2 or SB3 can be used as PCC. |  |  |  |  |  |

Table 4.3.1.1.5.66.2-2: NR Intra-Band non-contiguous CA configuration CA_n66(3A) without UL CA, SCS 30 kHz, frequency raster 100 kHz, and Max Wgap

| SB CBW combination (BCS) | SB (Note 2) | Bandwidth [MHz] | Range (Note 1,3) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- |
| CA_n66(3A): n66A + n66A + n66A |  |  |  |  |  |
| CBW1+CBW2 +CBW3 (0) | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.66-2: Low range for CBW = 10, 15, 20, 40 MHz, and if SB1 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.66-2: Mid range for CBW = 10, 15, 20, 40 MHz, and if SB2 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
|  | SB3 | CBW3 | Downlink & Uplink |  | For CBW3, use Table 4.3.1.1.1.66-2: High range for CBW = 10, 15, 20, 40 MHz, and if SB3 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
| Note 1: Only one range has been specified as the maximum maxAggregatedFsBW for FR1 NR intra-band non-contiguous CA without UL CA equals the full bandwith of the NR band (Table C.2.4.3.1A-1). Note 2: PCC is configured on SB1 unless otherwise stated. Note 3: The test frequencies for SB1, SB2 and SB3 have been specified for both downlink and uplink to enable that the CC of either SB1, SB2 or SB3 can be used as PCC. |  |  |  |  |  |

###### 4.3.1.1.5.67 – 4.3.1.1.5.70	FFS

###### 4.3.1.1.5.71	CA_n71(xA)

4.3.1.1.5.71.1	CA_n71(2A)

Editor’s note:	Test frequencies for CA_n71(2A) with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz; and SCS CC1=30kHz and SCS CC2=15 kHz is FFS.

Table 4.3.1.1.5.71.1-1: NR Intra-Band non-contiguous CA configuration CA_n71(2A) without UL CA, SCS=15 kHz, Max Wgap

| CBW combination | SB | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A); n71A (5MHz) + n71A (5-20MHz) |  |  |  |  |  |  |
| 5+5 | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=5 MHz |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=5 MHz |
| 5+10 | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=5 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=10 MHz |
| 5+15 | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=5 MHz |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=15 MHz |
| 5+20 | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=5 MHz |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=20 MHz |
| CA_n71(2A); n71A (10MHz) + n71A (5-20MHz) |  |  |  |  |  |  |
| 10+5 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=10 MHz |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=5 MHz |
| 10+10 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=10 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=10 MHz |
| 10+15 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=10 MHz |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=15 MHz |
| 10+20 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=10 MHz |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=20 MHz |
| CA_n71(2A); n71A (15MHz) + n71A (5-15MHz) |  |  |  |  |  |  |
| 15+5 | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=15 MHz |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=5 MHz |
| 15+10 | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=15 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=10 MHz |
| 15+15 | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=15 MHz |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=15 MHz |
| CA_n71(2A); n71A (20MHz) + n71A (5-10MHz) |  |  |  |  |  |  |
| 20+5 | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=20 MHz |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=5 MHz |
| 20+10 | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=20 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=10 MHz |
| Note 1: CA_n71(2A) is specified in [7] 38.101-1 without uplink CA. PCC is configured on SB1 unless otherwise stated. The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |  |

Table 4.3.1.1.5.71.1-2: NR Intra-Band non-contiguous CA configuration CA_n71(2A) without UL CA, SCS=30 kHz, Max Wgap

| SB CBW combination | SB | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A); n71A (10MHz) + n71A (10-20MHz) |  |  |  |  |  |  |
| 10+10 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-2: Low range for CBW=10 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.71-2: High range for CBW=10 MHz |
| 10+15 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-2: Low range for CBW=10 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.71-2: High range for CBW=15 MHz |
| 10+20 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-2: Low range for CBW=10 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.71-2: High range for CBW=20 MHz |
| CA_n71(2A); n71A (15MHz) + n71A (10-15MHz) |  |  |  |  |  |  |
| 15+10 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-2: Low range for CBW=15 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.71-2: High range for CBW=10 MHz |
| 15+15 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-2: Low range for CBW=15 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.71-2: High range for CBW=15 MHz |
| CA_n71(2A); n71A (20MHz) + n71A (10MHz) |  |  |  |  |  |  |
| 20+10 | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-2: Low range for CBW=20 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.71-2: High range for CBW=10 MHz |
| Note 1: CA_n71(2A) is specified in [7] 38.101-1 without uplink CA. PCC is configured on SB1 unless otherwise stated. The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |  |

###### 4.3.1.1.5.72 – 4.3.1.1.5.76	FFS

###### 4.3.1.1.5.77	CA_n77(xA)

4.3.1.1.5.77.1	CA_n77(2A)

Editor’s note:	Test frequencies for CA_n77(2A) with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.5.77.1-1: NR Intra-Band non-contiguous CA configuration CA_n77(2A) without UL CA, SCS=15 kHz, Max Wgap

| SB CBW combination (BCS) | SB (Note 2) | Bandwidth [MHz] | Range (Note 1,3) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- |
| CA_n77(2A): n77A + n77A |  |  |  |  |  |
| CBW1+CBW2 (0) | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.77-1: Low range for CBW = 20, 40 MHz |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.77-1: High range for CBW = 20, 40 MHz |
| CBW1+CBW2 (1) | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.77-1: Low range for CBW = 10, 15, 20, 25, 30, 40, 50 MHz |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.77-1: High range for CBW = 10, 15, 20, 25, 30, 40, 50 MHz |
| Note 1: Only one range has been specified as the maximum maxAggregatedFsBW for FR1 NR intra-band non-contiguous CA without UL CA equals the full bandwidth of the NR band (Table C.2.4.3.1A-1). Note 2: PCC is configured on SB1 unless otherwise stated. Note 3: The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |

Table 4.3.1.1.5.77.1-2: NR Intra-Band non-contiguous CA configuration CA_n77(2A) without UL CA, SCS=30 kHz, Max Wgap

| SB CBW combination (BCS) | SB (Note 2) | Bandwidth [MHz] | Range (Note 1,3) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- |
| CA_n77(2A): n77A + n77A |  |  |  |  |  |
| CBW1+CBW2 (0) | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.77-2: Low range for CBW = 20, 40, 80, 100 MHz |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.77-2: High range for CBW = 20, 40, 80, 100 MHz |
| CBW1+CBW2 (1) | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.77-2: Low range for CBW = 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 MHz |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.77-2: High range for CBW = 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 MHz |
| Note 1: Only one range has been specified as the maximum maxAggregatedFsBW for FR1 NR intra-band non-contiguous CA without UL CA equals the full bandwidth of the NR band (Table C.2.4.3.1A-1). Note 2: PCC is configured on SB1 unless otherwise stated. Note 3: The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |

Table 4.3.1.1.5.77.1-3: NR Intra-Band non-contiguous CA configuration CA_n77(2A) with UL CA (PCC=CC1 and SCC=CC2), SCS 15kHz

| Aggregated BW [MHz] (BCS) (CBW combination) | Range |  | CC Note 1 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Carrier centre [MHz] Note 1 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 3 | offsetToPointA (SIB1) [PRBs] Note 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | UL | LOW | CC1 | 10 | 15 | 52 | 3305.01 | 620334 | 3300.33 | 620022 | 0 | 30 | 7711 | 620352 | 6 | 1 | 1 (6) | 7 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 80 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (10+10) | DL |  | CC2 | 10 | 15 | 52 | 3394.98 Note 5 | 626332 | 3390.30 | 626020 | 0 | 30 | 7773 | 626304 | 8 | 1 | 0 (2) | 3 |
|  |  | HIGH | CC1 | 10 | 15 | 52 | 4105.02 Note 4 | 673668 | 4009.62 | 667308 | 504 | 30 | 8266 | 673632 | -0 | 1 | 0 (2) | 507 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 80 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 10 | 15 | 52 | 4194.99 | 679666 | 4099.59 | 673306 | 504 | 30 | 8329 | 679680 | 2 | 1 | 1 (6) | 511 |
| 30 | UL | LOW | CC1 | 20 | 15 | 106 | 3310.02 Note 4 | 620668 | 3300.48 | 620032 | 0 | 30 | 7711 | 620352 | 8 | 0 | 1 (6) | 6 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 70 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (20+10) | DL |  | CC2 | 10 | 15 | 52 | 3395.01 | 626334 | 3390.33 | 626022 | 0 | 30 | 7773 | 626304 | 6 | 1 | 0 (2) | 3 |
|  |  | HIGH | CC1 | 20 | 15 | 106 | 4110.00 | 674000 | 4009.74 | 667316 | 504 | 30 | 8266 | 673632 | 4 | 0 | 0 (2) | 506 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 70 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 10 | 15 | 52 | 4194.99 | 679666 | 4099.59 | 673306 | 504 | 30 | 8329 | 679680 | 2 | 1 | 1 (6) | 511 |
| 40 | UL | LOW | CC1 | 20 | 15 | 106 | 3310.02 Note 4 | 620668 | 3300.48 | 620032 | 0 | 30 | 7711 | 620352 | 8 | 0 | 1 (6) | 6 |
| (0) | & |  | MaxFsBW = 100 MHz, maxWgap = 60 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (20+20) | DL |  | CC2 | 20 | 15 | 106 | 3390.00 Note 5 | 626000 | 3380.46 | 625364 | 0 | 30 | 7766 | 625632 | 4 | 0 | 0 (2) | 2 |
|  |  | HIGH | CC1 | 20 | 15 | 106 | 4110.00 Note 4 | 674000 | 4009.74 | 667316 | 504 | 30 | 8266 | 673632 | 4 | 0 | 0 (2) | 506 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 60 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 20 | 15 | 106 | 4189.98 Note 5 | 679332 | 4089.72 | 672648 | 504 | 30 | 8322 | 679008 | 0 | 0 | 1 (6) | 510 |
| 40 | UL | LOW | CC1 | 30 | 15 | 160 | 3315.00 | 621000 | 3300.60 | 620040 | 0 | 30 | 7711 | 620352 | 0 | 0 | 1 (6) | 6 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 60 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (30+10) | DL |  | CC2 | 10 | 15 | 52 | 3394.98 Note 5 | 626332 | 3390.30 | 626020 | 0 | 30 | 7773 | 626304 | 8 | 1 | 0 (2) | 3 |
|  |  | HIGH | CC1 | 30 | 15 | 160 | 4115.01 Note 4 | 674334 | 4009.89 | 667326 | 504 | 30 | 8267 | 673728 | 6 | 3 | 1 (6) | 513 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 60 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 10 | 15 | 52 | 4194.99 | 679666 | 4099.59 | 673306 | 504 | 30 | 8329 | 679680 | 2 | 1 | 1 (6) | 511 |
| 50 | UL | LOW | CC1 | 40 | 15 | 216 | 3320.01 | 621334 | 3300.57 | 620038 | 0 | 30 | 7711 | 620352 | 2 | 0 | 1 (6) | 6 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 50 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (40+10) | DL |  | CC2 | 10 | 15 | 52 | 3394.98 Note 5 | 626332 | 3390.30 | 626020 | 0 | 30 | 7773 | 626304 | 8 | 1 | 0 (2) | 3 |
|  |  | HIGH | CC1 | 40 | 15 | 216 | 4120.02 Note 4 | 674668 | 4009.86 | 667324 | 504 | 30 | 8267 | 673728 | 8 | 3 | 1 (6) | 513 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 50 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 10 | 15 | 52 | 4194.99 | 679666 | 4099.59 | 673306 | 504 | 30 | 8329 | 679680 | 2 | 1 | 1 (6) | 511 |
| 60 | UL | LOW | CC1 | 40 | 15 | 216 | 3320.01 | 621334 | 3300.57 | 620038 | 0 | 30 | 7711 | 620352 | 2 | 0 | 1 (6) | 6 |
| (0) | & |  | MaxFsBW = 100 MHz, maxWgap = 40 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (40+20) | DL |  | CC2 | 20 | 15 | 106 | 3390.00 | 626000 | 3380.46 | 625364 | 0 | 30 | 7766 | 625632 | 4 | 0 | 0 (2) | 2 |
|  |  | HIGH | CC1 | 40 | 15 | 216 | 4119.99 | 674666 | 4009.83 | 667322 | 504 | 30 | 8267 | 673728 | 10 | 3 | 1 (6) | 513 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 40 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 20 | 15 | 106 | 4189.98 Note 5 | 679332 | 4089.72 | 672648 | 504 | 30 | 8322 | 679008 | 0 | 0 | 1 (6) | 510 |
| 60 | UL | LOW | CC1 | 50 | 15 | 270 | 3325.02 Note 4 | 621668 | 3300.72 | 620048 | 0 | 30 | 7711 | 620352 | 4 | 3 | 0 (2) | 5 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 40 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (50+10) | DL |  | CC2 | 10 | 15 | 52 | 3395.01 | 626334 | 3390.33 | 626022 | 0 | 30 | 7773 | 626304 | 6 | 1 | 0 (2) | 3 |
|  |  | HIGH | CC1 | 50 | 15 | 270 | 4125.00 | 675000 | 4009.98 | 667332 | 504 | 30 | 8267 | 673728 | 0 | 3 | 1 (6) | 513 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 40 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 10 | 15 | 52 | 4194.99 | 679666 | 4099.59 | 673306 | 504 | 30 | 8329 | 679680 | 2 | 1 | 1 (6) | 511 |
| 70 | UL | LOW | CC1 | 50 | 15 | 270 | 3325.02 Note 4 | 621668 | 3300.72 | 620048 | 0 | 30 | 7711 | 620352 | 4 | 3 | 0 (2) | 5 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 30 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (50+20) | DL |  | CC2 | 20 | 15 | 106 | 3390.00 Note 5 | 626000 | 3380.46 | 625364 | 0 | 30 | 7766 | 625632 | 4 | 0 | 0 (2) | 2 |
|  |  | HIGH | CC1 | 50 | 15 | 270 | 4125.00 Note 4 | 675000 | 4009.98 | 667332 | 504 | 30 | 8267 | 673728 | 0 | 3 | 1 (6) | 513 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 30 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 20 | 15 | 106 | 4189.98 Note 5 | 679332 | 4089.72 | 672648 | 504 | 30 | 8322 | 679008 | 0 | 0 | 1 (6) | 510 |
| 80 | UL | LOW | CC1 | 40 | 15 | 216 | 3320.01 | 621334 | 3300.57 | 620038 | 0 | 30 | 7711 | 620352 | 2 | 0 | 1 (6) | 6 |
| (0) | & |  | MaxFsBW = 100 MHz, maxWgap = 20 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (40+40) | DL |  | CC2 | 40 | 15 | 216 | 3379.98 Note 5 | 625332 | 3360.54 | 624036 | 0 | 30 | 7753 | 624384 | 0 | 3 | 1 (6) | 9 |
|  |  | HIGH | CC1 | 40 | 15 | 216 | 4120.02 Note 4 | 674668 | 4009.86 | 667324 | 504 | 30 | 8267 | 673728 | 8 | 3 | 1 (6) | 513 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 20 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 40 | 15 | 216 | 4179.99 | 678666 | 4069.83 | 671322 | 504 | 30 | 8308 | 677664 | 6 | 2 | 0 (2) | 508 |
| 80 | UL | LOW | CC1 | 50 | 15 | 270 | 3325.02 Note 4 | 621668 | 3300.72 | 620048 | 0 | 30 | 7711 | 620352 | 4 | 3 | 0 (2) | 5 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 20 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (50+30) | DL |  | CC2 | 30 | 15 | 160 | 3384.99 Note 5 | 625666 | 3370.59 | 624706 | 0 | 30 | 7760 | 625056 | 2 | 3 | 1 (6) | 9 |
|  |  | HIGH | CC1 | 50 | 15 | 270 | 4125.03 Note 4 | 675002 | 4010.01 | 667334 | 504 | 30 | 8267 | 673728 | 10 | 2 | 1 (6) | 512 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 20 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 30 | 15 | 160 | 4185.00 | 679000 | 4079.88 | 671992 | 504 | 30 | 8315 | 678336 | 8 | 2 | 0 (2) | 508 |
| 90 | UL | LOW | CC1 | 50 | 15 | 270 | 3325.02 Note 4 | 621668 | 3300.72 | 620048 | 0 | 30 | 7711 | 620352 | 4 | 3 | 0 (2) | 5 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 10 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (50+40) | DL |  | CC2 | 40 | 15 | 216 | 3380.01 | 625334 | 3360.57 | 624038 | 0 | 30 | 7753 | 624384 | 10 | 2 | 1 (6) | 8 |
|  |  | HIGH | CC1 | 50 | 15 | 270 | 4125.00 | 675000 | 4009.98 | 667332 | 504 | 30 | 8267 | 673728 | 0 | 3 | 1 (6) | 513 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 10 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 40 | 15 | 216 | 4179.99 | 678666 | 4069.83 | 671322 | 504 | 30 | 8308 | 677664 | 6 | 2 | 0 (2) | 508 |
| 100 | UL | LOW | CC1 | 50 | 15 | 270 | 3325.02 Note 4 | 621668 | 3300.72 | 620048 | 0 | 30 | 7711 | 620352 | 4 | 3 | 0 (2) | 5 |
| (1) | & |  | MaxFsBW = 200 MHz, maxWgap = 100 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (50+50) | DL |  | CC2 | 50 | 15 | 270 | 3474.99 Note 5 | 631666 | 3450.69 | 630046 | 0 | 30 | 7815 | 630336 | 2 | 2 | 0 (2) | 4 |
|  |  | HIGH | CC1 | 50 | 15 | 270 | 4025.01 Note 4 | 668334 | 3909.99 | 660666 | 504 | 30 | 8197 | 667008 | 6 | 2 | 0 (2) | 508 |
|  |  |  | MaxFsBW = 200 MHz, maxWgap = 100 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 50 | 15 | 270 | 4174.98 Note 5 | 678332 | 4059.96 | 670664 | 504 | 30 | 8301 | 676992 | 4 | 1 | 0 (2) | 507 |
| Note 1: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-3 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 4: Centre frequency shifted 15 kHz to align with the synchronisation raster (According to R1-1809879). Note 5: Centre frequency shifted -15 kHz to align with the synchronisation raster (According to R1-1809879). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.5.77.1-4: NR Intra-Band non-contiguous CA configuration CA_n77(2A) with UL CA (PCC=CC1 and SCC=CC2), SCS 30kHz

| Aggregated BW [MHz] (BCS) (CBW combination) | Range |  | CC Note 1 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Carrier centre [MHz] Note 1 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 3 | offsetToPointA (SIB1) [PRBs] Note 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | UL | LOW | CC1 | 10 | 30 | 24 | 3305.01 | 620334 | 3300.69 | 620046 | 0 | 30 | 7711 | 620352 | 18 | 0 | 2 (2) | 4 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 80 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (10+10) | DL |  | CC2 | 10 | 30 | 24 | 3394.98 Note 5 | 626332 | 3390.66 | 626044 | 0 | 30 | 7773 | 626304 | 20 | 0 | 0 (0) | 0 |
|  |  | HIGH | CC1 | 10 | 30 | 24 | 4105.02 Note 4 | 673668 | 3919.26 | 661284 | 504 | 30 | 8266 | 673632 | 12 | 0 | 0 (0) | 1008 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 80 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 10 | 30 | 24 | 4194.99 | 679666 | 4009.23 | 667282 | 504 | 30 | 8329 | 679680 | 14 | 0 | 2 (2) | 1012 |
| 30 | UL | LOW | CC1 | 20 | 30 | 51 | 3310.02 Note 4 | 620668 | 3300.84 | 620056 | 0 | 30 | 7711 | 620352 | 8 | 0 | 2 (2) | 4 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 70 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (20+10) | DL |  | CC2 | 10 | 30 | 24 | 3395.01 | 626334 | 3390.69 | 626046 | 0 | 30 | 7773 | 626304 | 18 | 0 | 0 (0) | 0 |
|  |  | HIGH | CC1 | 20 | 30 | 51 | 4110.00 | 674000 | 3919.38 | 661292 | 504 | 30 | 8266 | 673632 | 4 | 0 | 0 (0) | 1008 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 70 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 10 | 30 | 24 | 4194.99 | 679666 | 4009.23 | 667282 | 504 | 30 | 8329 | 679680 | 14 | 0 | 2 (2) | 1012 |
| 40 | UL | LOW | CC1 | 20 | 30 | 51 | 3310.02 Note 4 | 620668 | 3300.84 | 620056 | 0 | 30 | 7711 | 620352 | 8 | 0 | 2 (2) | 4 |
| (0) | & |  | MaxFsBW = 100 MHz, maxWgap = 60 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (20+20) | DL |  | CC2 | 20 | 30 | 51 | 3390.00 Note 5 | 626000 | 3380.82 | 625388 | 0 | 30 | 7766 | 625632 | 4 | 0 | 0 (0) | 0 |
|  |  | HIGH | CC1 | 20 | 30 | 51 | 4110.00 Note 4 | 674000 | 3919.38 | 661292 | 504 | 30 | 8266 | 673632 | 4 | 0 | 0 (0) | 1008 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 60 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 20 | 30 | 51 | 4189.98 Note 5 | 679332 | 3999.36 | 666624 | 504 | 30 | 8322 | 679008 | 0 | 0 | 2 (2) | 1012 |
| 40 | UL | LOW | CC1 | 30 | 30 | 78 | 3315.00 | 621000 | 3300.96 | 620064 | 0 | 30 | 7711 | 620352 | 0 | 0 | 2 (2) | 4 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 60 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (30+10) | DL |  | CC2 | 10 | 30 | 24 | 3394.98 Note 5 | 626332 | 3390.66 | 626044 | 0 | 30 | 7773 | 626304 | 20 | 0 | 0 (0) | 0 |
|  |  | HIGH | CC1 | 30 | 30 | 78 | 4115.01 Note 4 | 674334 | 3919.53 | 661302 | 504 | 30 | 8267 | 673728 | 18 | 0 | 3 (3) | 1014 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 60 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 10 | 30 | 24 | 4194.99 | 679666 | 4009.23 | 667282 | 504 | 30 | 8329 | 679680 | 14 | 0 | 2 (2) | 1012 |
| 50 | UL | LOW | CC1 | 40 | 30 | 106 | 3320.01 | 621334 | 3300.93 | 620062 | 0 | 30 | 7711 | 620352 | 2 | 0 | 2 (2) | 4 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 50 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (40+10) | DL |  | CC2 | 10 | 30 | 24 | 3394.98 Note 5 | 626332 | 3390.66 | 626044 | 0 | 30 | 7773 | 626304 | 20 | 0 | 0 (0) | 0 |
|  |  | HIGH | CC1 | 40 | 30 | 106 | 4120.02 Note 4 | 674668 | 3919.50 | 661300 | 504 | 30 | 8267 | 673728 | 20 | 0 | 3 (3) | 1014 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 50 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 10 | 30 | 24 | 4194.99 | 679666 | 4009.23 | 667282 | 504 | 30 | 8329 | 679680 | 14 | 0 | 2 (2) | 1012 |
| 60 | UL | LOW | CC1 | 40 | 30 | 106 | 3320.01 | 621334 | 3300.93 | 620062 | 0 | 30 | 7711 | 620352 | 2 | 0 | 2 (2) | 4 |
| (0) | & |  | MaxFsBW = 100 MHz, maxWgap = 40 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (40+20) | DL |  | CC2 | 20 | 30 | 51 | 3390.00 | 626000 | 3380.82 | 625388 | 0 | 30 | 7766 | 625632 | 4 | 0 | 0 (0) | 0 |
|  |  | HIGH | CC1 | 40 | 30 | 106 | 4119.99 | 674666 | 3919.47 | 661298 | 504 | 30 | 8267 | 673728 | 22 | 0 | 3 (3) | 1014 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 40 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 20 | 30 | 51 | 4189.98 Note 5 | 679332 | 3999.36 | 666624 | 504 | 30 | 8322 | 679008 | 0 | 0 | 2 (2) | 1012 |
| 60 | UL | LOW | CC1 | 50 | 30 | 133 | 3325.02 Note 4 | 621668 | 3301.08 | 620072 | 0 | 30 | 7711 | 620352 | 16 | 0 | 1 (1) | 2 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 40 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (50+10) | DL |  | CC2 | 10 | 30 | 24 | 3395.01 | 626334 | 3390.69 | 626046 | 0 | 30 | 7773 | 626304 | 18 | 0 | 0 (0) | 0 |
|  |  | HIGH | CC1 | 50 | 30 | 133 | 4125.00 | 675000 | 3919.62 | 661308 | 504 | 30 | 8267 | 673728 | 12 | 0 | 3 (3) | 1014 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 40 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 10 | 30 | 24 | 4194.99 | 679666 | 4009.23 | 667282 | 504 | 30 | 8329 | 679680 | 14 | 0 | 2 (2) | 1012 |
| 70 | UL | LOW | CC1 | 60 | 30 | 162 | 3330.00 | 622000 | 3300.84 | 620056 | 0 | 30 | 7711 | 620352 | 8 | 0 | 2 (2) | 4 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 30 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (60+10) | DL |  | CC2 | 10 | 30 | 24 | 3394.98 Note 5 | 626332 | 3390.66 | 626044 | 0 | 30 | 7773 | 626304 | 20 | 0 | 0 (0) | 0 |
|  |  | HIGH | CC1 | 60 | 30 | 162 | 4130.01 Note 4 | 675334 | 3919.41 | 661294 | 504 | 30 | 8266 | 673632 | 2 | 0 | 0 (0) | 1008 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 30 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 10 | 30 | 24 | 4194.99 | 679666 | 4009.23 | 667282 | 504 | 30 | 8329 | 679680 | 14 | 0 | 2 (2) | 1012 |
| 80 | UL | LOW | CC1 | 40 | 30 | 106 | 3320.01 | 621334 | 3300.93 | 620062 | 0 | 30 | 7711 | 620352 | 2 | 0 | 2 (2) | 4 |
| (0) | & |  | MaxFsBW = 100 MHz, maxWgap = 20 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (40+40) | DL |  | CC2 | 40 | 30 | 106 | 3379.98 Note 5 | 625332 | 3360.90 | 624060 | 0 | 30 | 7753 | 624384 | 12 | 0 | 3 (3) | 6 |
|  |  | HIGH | CC1 | 40 | 30 | 106 | 4120.02 Note 4 | 674668 | 3919.50 | 661300 | 504 | 30 | 8267 | 673728 | 20 | 0 | 3 (3) | 1014 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 20 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 40 | 30 | 106 | 4179.99 | 678666 | 3979.47 | 665298 | 504 | 30 | 8308 | 677664 | 6 | 0 | 1 (1) | 1010 |
| 80 | UL | LOW | CC1 | 70 | 30 | 189 | 3335.01 | 622334 | 3300.99 | 620066 | 0 | 30 | 7711 | 620352 | 22 | 0 | 1 (1) | 2 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 20 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (70+10) | DL |  | CC2 | 10 | 30 | 24 | 3394.98 Note 5 | 626332 | 3390.66 | 626044 | 0 | 30 | 7773 | 626304 | 20 | 0 | 0 (0) | 0 |
|  |  | HIGH | CC1 | 70 | 30 | 189 | 4135.02 Note 4 | 675668 | 3919.56 | 661304 | 504 | 30 | 8267 | 673728 | 16 | 0 | 3 (3) | 1014 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 20 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 10 | 30 | 24 | 4194.99 | 679666 | 4009.23 | 667282 | 504 | 30 | 8329 | 679680 | 14 | 0 | 2 (2) | 1012 |
| 90 | UL | LOW | CC1 | 80 | 30 | 217 | 3340.02 Note 4 | 622668 | 3300.96 | 620064 | 0 | 30 | 7711 | 620352 | 0 | 0 | 2 (2) | 4 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 10 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (80+10) | DL |  | CC2 | 10 | 30 | 24 | 3395.01 | 626334 | 3390.69 | 626046 | 0 | 30 | 7773 | 626304 | 18 | 0 | 0 (0) | 0 |
|  |  | HIGH | CC1 | 80 | 30 | 217 | 4140.00 | 676000 | 3919.50 | 661300 | 504 | 30 | 8267 | 673728 | 20 | 0 | 3 (3) | 1014 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 10 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 10 | 30 | 24 | 4194.99 | 679666 | 4009.23 | 667282 | 504 | 30 | 8329 | 679680 | 14 | 0 | 2 (2) | 1012 |
| 100 | UL | LOW | CC1 | 80 | 30 | 217 | 3340.02 Note 4 | 622668 | 3300.96 | 620064 | 0 | 30 | 7711 | 620352 | 0 | 0 | 2 (2) | 4 |
| (0) | & |  | MaxFsBW = 200 MHz, maxWgap = 100 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (80+20) | DL |  | CC2 | 20 | 30 | 51 | 3489.99 Note 5 | 632666 | 3480.81 | 632054 | 0 | 30 | 7836 | 632352 | 10 | 0 | 2 (2) | 4 |
|  |  | HIGH | CC1 | 80 | 30 | 217 | 4040.01 Note 4 | 669334 | 3819.51 | 654634 | 504 | 30 | 8197 | 667008 | 14 | 0 | 1 (1) | 1010 |
|  |  |  | MaxFsBW = 200 MHz, maxWgap = 100 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 20 | 30 | 51 | 4189.98 Note 5 | 679332 | 3999.36 | 666624 | 504 | 30 | 8322 | 679008 | 0 | 0 | 2 (2) | 1012 |
| 100 | UL | LOW | CC1 | 90 | 30 | 245 | 3345.00 | 623000 | 3300.90 | 620060 | 0 | 30 | 7711 | 620352 | 4 | 0 | 2 (2) | 4 |
| (1) | & |  | MaxFsBW = 200 MHz, maxWgap = 100 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (90+10) | DL |  | CC2 | 10 | 30 | 24 | 3494.97 Note 5 | 632998 | 3490.65 | 632710 | 0 | 30 | 7843 | 633024 | 2 | 0 | 3 (3) | 6 |
|  |  | HIGH | CC1 | 90 | 30 | 245 | 4045.02 Note 4 | 669668 | 3819.48 | 654632 | 504 | 30 | 8197 | 667008 | 16 | 0 | 1 (1) | 1010 |
|  |  |  | MaxFsBW = 200 MHz, maxWgap = 100 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 10 | 30 | 24 | 4194.99 | 679666 | 4009.23 | 667282 | 504 | 30 | 8329 | 679680 | 14 | 0 | 2 (2) | 1012 |
| 110 | UL | LOW | CC1 | 100 | 30 | 273 | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 | 7711 | 620352 | 6 | 0 | 2 (2) | 4 |
| (1) | & |  | MaxFsBW = 200 MHz, maxWgap = 90 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (100+10) | DL |  | CC2 | 10 | 30 | 24 | 3495.00 | 633000 | 3490.68 | 632712 | 0 | 30 | 7843 | 633024 | 0 | 0 | 3 (3) | 6 |
|  |  | HIGH | CC1 | 100 | 30 | 273 | 4050.00 | 670000 | 3819.42 | 654628 | 504 | 30 | 8197 | 667008 | 20 | 0 | 1 (1) | 1010 |
|  |  |  | MaxFsBW = 200 MHz, maxWgap = 90 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 10 | 30 | 24 | 4194.99 | 679666 | 4009.23 | 667282 | 504 | 30 | 8329 | 679680 | 14 | 0 | 2 (2) | 1012 |
| 120 | UL | LOW | CC1 | 100 | 30 | 273 | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 | 7711 | 620352 | 6 | 0 | 2 (2) | 4 |
| (0,1) | & |  | MaxFsBW = 200 MHz, maxWgap = 80 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (100+20) | DL |  | CC2 | 20 | 30 | 51 | 3489.99 Note 5 | 632666 | 3480.81 | 632054 | 0 | 30 | 7836 | 632352 | 10 | 0 | 2 (2) | 4 |
|  |  | HIGH | CC1 | 100 | 30 | 273 | 4050.00 Note 4 | 670000 | 3819.42 | 654628 | 504 | 30 | 8197 | 667008 | 20 | 0 | 1 (1) | 1010 |
|  |  |  | MaxFsBW = 200 MHz, maxWgap = 80 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 20 | 30 | 51 | 4189.98 Note 5 | 679332 | 3999.36 | 666624 | 504 | 30 | 8322 | 679008 | 0 | 0 | 2 (2) | 1012 |
| 130 | UL | LOW | CC1 | 100 | 30 | 273 | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 | 7711 | 620352 | 6 | 0 | 2 (2) | 4 |
| (1) | & |  | MaxFsBW = 200 MHz, maxWgap = 70 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (100+30) | DL |  | CC2 | 30 | 30 | 78 | 3484.98 Note 5 | 632332 | 3470.94 | 631396 | 0 | 30 | 7829 | 631680 | 20 | 0 | 1 (1) | 2 |
|  |  | HIGH | CC1 | 100 | 30 | 273 | 4050.03 Note 4 | 670002 | 3819.45 | 654630 | 504 | 30 | 8197 | 667008 | 18 | 0 | 1 (1) | 1010 |
|  |  |  | MaxFsBW = 200 MHz, maxWgap = 70 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 30 | 30 | 78 | 4185.00 | 679000 | 3989.52 | 665968 | 504 | 30 | 8315 | 678336 | 8 | 0 | 1 (1) | 1010 |
| 140 | UL | LOW | CC1 | 100 | 30 | 273 | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 | 7711 | 620352 | 6 | 0 | 2 (2) | 4 |
| (0,1) | & |  | MaxFsBW = 200 MHz, maxWgap = 60 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (100+40) | DL |  | CC2 | 40 | 30 | 106 | 3480.00 | 632000 | 3460.92 | 630728 | 0 | 30 | 7822 | 631008 | 16 | 0 | 1 (1) | 2 |
|  |  | HIGH | CC1 | 100 | 30 | 273 | 4050.00 | 670000 | 3819.42 | 654628 | 504 | 30 | 8197 | 667008 | 20 | 0 | 1 (1) | 1010 |
|  |  |  | MaxFsBW = 200 MHz, maxWgap = 60 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 40 | 30 | 106 | 4179.99 | 678666 | 3979.47 | 665298 | 504 | 30 | 8308 | 677664 | 6 | 0 | 1 (1) | 1010 |
| 150 | UL | LOW | CC1 | 100 | 30 | 273 | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 | 7711 | 620352 | 6 | 0 | 2 (2) | 4 |
| (1) | & |  | MaxFsBW = 200 MHz, maxWgap = 50 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (100+50) | DL |  | CC2 | 50 | 30 | 133 | 3474.99 Note 5 | 631666 | 3451.05 | 630070 | 0 | 30 | 7815 | 630336 | 2 | 0 | 1 (1) | 2 |
|  |  | HIGH | CC1 | 100 | 30 | 273 | 4050.00 Note 4 | 670000 | 3819.42 | 654628 | 504 | 30 | 8197 | 667008 | 20 | 0 | 1 (1) | 1010 |
|  |  |  | MaxFsBW = 200 MHz, maxWgap = 50 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 50 | 30 | 133 | 4174.98 Note 5 | 678332 | 3969.60 | 664640 | 504 | 30 | 8301 | 676992 | 16 | 0 | 0 (0) | 1008 |
| 160 | UL | LOW | CC1 | 80 | 30 | 217 | 3340.02 Note 4 | 622668 | 3300.96 | 620064 | 0 | 30 | 7711 | 620352 | 0 | 0 | 2 (2) | 4 |
| (0) | & |  | MaxFsBW = 200 MHz, maxWgap = 40 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (80+80) | DL |  | CC2 | 80 | 30 | 217 | 3459.99 Note 5 | 630666 | 3420.93 | 628062 | 0 | 30 | 7794 | 628320 | 18 | 0 | 0 (0) | 0 |
|  |  | HIGH | CC1 | 80 | 30 | 217 | 4040.01 Note 4 | 669334 | 3819.51 | 654634 | 504 | 30 | 8197 | 667008 | 14 | 0 | 1 (1) | 1010 |
|  |  |  | MaxFsBW = 200 MHz, maxWgap = 40 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 80 | 30 | 217 | 4159.98 Note 5 | 677332 | 3939.48 | 662632 | 504 | 30 | 8280 | 674976 | 8 | 0 | 0 (0) | 1008 |
| 160 | UL | LOW | CC1 | 100 | 30 | 273 | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 | 7711 | 620352 | 6 | 0 | 2 (2) | 4 |
| (1) | & |  | MaxFsBW = 200 MHz, maxWgap = 40 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (100+60) | DL |  | CC2 | 60 | 30 | 162 | 3469.98 Note 5 | 631332 | 3440.82 | 629388 | 0 | 30 | 7808 | 629664 | 12 | 0 | 1 (1) | 2 |
|  |  | HIGH | CC1 | 100 | 30 | 273 | 4050.03 Note 4 | 670002 | 3819.45 | 654630 | 504 | 30 | 8197 | 667008 | 18 | 0 | 1 (1) | 1010 |
|  |  |  | MaxFsBW = 200 MHz, maxWgap = 40 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 60 | 30 | 162 | 4170.00 | 678000 | 3959.40 | 663960 | 504 | 30 | 8294 | 676320 | 0 | 0 | 1 (1) | 1010 |
| 170 | UL | LOW | CC1 | 100 | 30 | 273 | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 | 7711 | 620352 | 6 | 0 | 2 (2) | 4 |
| (1) | & |  | MaxFsBW = 200 MHz, maxWgap = 30 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (100+70) | DL |  | CC2 | 70 | 30 | 189 | 3465.00 | 631000 | 3430.98 | 628732 | 0 | 30 | 7801 | 628992 | 20 | 0 | 0 (0) | 0 |
|  |  | HIGH | CC1 | 100 | 30 | 273 | 4050.00 | 670000 | 3819.42 | 654628 | 504 | 30 | 8197 | 667008 | 20 | 0 | 1 (1) | 1010 |
|  |  |  | MaxFsBW = 200 MHz, maxWgap = 30 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 70 | 30 | 189 | 4164.99 | 677666 | 3949.53 | 663302 | 504 | 30 | 8287 | 675648 | 10 | 0 | 0 (0) | 1008 |
| 180 | UL | LOW | CC1 | 100 | 30 | 273 | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 | 7711 | 620352 | 6 | 0 | 2 (2) | 4 |
| (0,1) | & |  | MaxFsBW = 200 MHz, maxWgap = 20 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (100+80) | DL |  | CC2 | 80 | 30 | 217 | 3459.99 Note 5 | 630666 | 3420.93 | 628062 | 0 | 30 | 7794 | 628320 | 18 | 0 | 0 (0) | 0 |
|  |  | HIGH | CC1 | 100 | 30 | 273 | 4050.00 Note 4 | 670000 | 3819.42 | 654628 | 504 | 30 | 8197 | 667008 | 20 | 0 | 1 (1) | 1010 |
|  |  |  | MaxFsBW = 200 MHz, maxWgap = 20 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 80 | 30 | 217 | 4159.98 Note 5 | 677332 | 3939.48 | 662632 | 504 | 30 | 8280 | 674976 | 8 | 0 | 0 (0) | 1008 |
| 190 | UL | LOW | CC1 | 100 | 30 | 273 | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 | 7711 | 620352 | 6 | 0 | 2 (2) | 4 |
| (1) | & |  | MaxFsBW = 200 MHz, maxWgap = 10 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (100+90) | DL |  | CC2 | 90 | 30 | 245 | 3454.98 Note 5 | 630332 | 3410.88 | 627392 | 0 | 30 | 7787 | 627648 | 16 | 0 | 0 (0) | 0 |
|  |  | HIGH | CC1 | 100 | 30 | 273 | 4050.03 Note 4 | 670002 | 3819.45 | 654630 | 504 | 30 | 8197 | 667008 | 18 | 0 | 1 (1) | 1010 |
|  |  |  | MaxFsBW = 200 MHz, maxWgap = 10 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 90 | 30 | 245 | 4155.00 | 677000 | 3929.46 | 661964 | 504 | 30 | 8273 | 674304 | 4 | 0 | 0 (0) | 1008 |
| 200 | UL | LOW | CC1 | 100 | 30 | 273 | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 | 7711 | 620352 | 6 | 0 | 2 (2) | 4 |
| (0,1) | & |  | MaxFsBW = 200 MHz, maxWgap = 0 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (100+100) | DL |  | CC2 | 100 | 30 | 273 | 3450.00 | 630000 | 3400.86 | 626724 | 0 | 30 | 7780 | 626976 | 12 | 0 | 0 (0) | 0 |
|  |  | HIGH | CC1 | 100 | 30 | 273 | 4050.00 | 670000 | 3819.42 | 654628 | 504 | 30 | 8197 | 667008 | 20 | 0 | 1 (1) | 1010 |
|  |  |  | MaxFsBW = 200 MHz, maxWgap = 0 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 100 | 30 | 273 | 4149.99 | 676666 | 3919.41 | 661294 | 504 | 30 | 8266 | 673632 | 2 | 0 | 0 (0) | 1008 |
| Note 1: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-3 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 4: Centre frequency shifted 15 kHz to align with the synchronisation raster (According to R1-1809879). Note 5: Centre frequency shifted -15 kHz to align with the synchronisation raster (According to R1-1809879). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.5.78	CA_n78(xA)

4.3.1.1.5.78.1	CA_n78(2A)

Editor’s note:	Test frequencies for CA_n78(2A) with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.5.78.1-1: NR Intra-Band non-contiguous CA configuration CA_n78(2A) without UL CA, SCS=15 kHz, Max Wgap

| CBW combination | SB | Bandwidth [MHz] | Range | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- |
| CA_n78(2A); n78A(10-50MHz) + n78A(10-50MHz) |  |  |  |  |  |
| CBW1+CBW2 | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.78-1: Low range for CBW = 10, 20, 25, 30, 40 OR 50 MHz |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.78-1: High range for CBW = 10, 20, 25, 30, 40 OR 50 MHz |
| Note 1: CA_n78(2A) is specified in [7] 38.101-1 with and without uplink CA. PCC is configured on SB1 unless otherwise stated. This table is applicable for the case of without UL CA but test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |

Table 4.3.1.1.5.78.1-1A: NR Intra-Band non-contiguous CA configuration CA_n78(2A) with UL CA, SCS=15 kHz, Max Wgap

FFS

Table 4.3.1.1.5.78.1-2: NR Intra-Band non-contiguous CA configuration CA_n78(2A) without UL CA, SCS=30 kHz, Max Wgap

| CBW combination | SB | Bandwidth [MHz] | Range | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- |
| CA_n78(2A); n78A(10-100MHz) + n78A(10-100MHz) |  |  |  |  |  |
| CBW1+CBW2 | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.78-2: Low range for CBW = 10, 20, 25, 30, 40, 50, 60, 70, 80, 90 OR 100 MHz |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.78-2: High range for CBW = 10, 20, 25, 30, 40, 50, 60, 70, 80, 90 OR 100 MHz |
| Note 1: CA_n78(2A) is specified in [7] 38.101-1 with and without uplink CA. PCC is configured on SB1 unless otherwise stated. This table is applicable for the case of without UL CA but test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |

Table 4.3.1.1.5.78.1-2A: NR Intra-Band non-contiguous CA configuration CA_n78(2A) with UL CA, SCS=30 kHz, Max Wgap

FFS

<< Unchanged clauses omitted>>

Table 6.2.3.4-3: Test frequencies for NR CA Intra-Band Non-Contiguous configurations with FR1

| NR CA configuration | SB | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) | offsetToPointA (SIB1) [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n48(2A) | SB1 | 10+10 | 24+24 | Downlink | Low | Same values as for Low and High ranges in Table 4.3.1.1.5.48.1-2 for CBW combination 10+10 and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 |  |  | & Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66(2A) | SB1 | 10+10 | 52+52 | Downlink | Low | Same values as for Low and High ranges in Table 4.3.1.1.5.66.1-1 for CBW combination 10+10 and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 |  |  | & Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n71(2A) | SB1, | 10+10 | 52+52 | Downlink | Low | Same values as for Low and High ranges in Table 4.3.1.1.5.71.1-1 for CBW combination 10+10 and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 |  |  | & Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77(2A) | SB1, | 20+20 | 106+106 | Downlink | Low | Same values as for Low and High ranges in Table 4.3.1.1.5.77.1-1 for CBW combination 20+20 and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 |  |  | & Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78(2A) | SB1 | 50+50 | 133+ | Downlink | Low | Same values as for Low and High ranges in Table 4.3.1.1.5.78.1-2 for CBW combination 50+50 and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 |  | 133 | & Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |

##### << End of changes>>